τexo=kexo×θ0-θankle			               (1)

where kexo was rotational stiffness of the exoskeleton (Nm rad-1), was the onset angle (rad), and θankle was the real-time ankle joint angle (rad). A goniometer (5 kHz, 250Hz Biometrics, Newport, UK) between the foot and shank segment of the exoskeleton measured real-time sagittal plane ankle joint angle, and the onset angle (θ0) was set to the value recorded at contact of the foot with the treadmill surface. We determined gait state and heel strike by sampling vertical ground reaction force from the instrumented treadmill (Bertec, Columbus, OH).