Pretreatment: 50 mg of samples were applied to extraction procedure, and extracted with 800 μL of methanol. 10 μL of internal standard(2.8 mg/mL, DL-o-Chlorophenylalanine) was then added. And all samples were grinded to fine powder using Grinding Mill at 65 HZ for 90 s. The samples after grinding were vortexed for 30 s, and centrifuged at 12000 rpm and 4 °C for 15 min. 200 μL of supernatant was transferred to vial for concentration by centrifuging.

Derivatization: The samples were then derivatized by shaking them with 30 μL of methoxyamine hydrochloride (20 mg/mL) in pyridine for 90 min at 37 °C and then incubating them for 16 h at room temperature. The samples were then trimethylsilylated by adding 30 μL of BSTFA and incubating them for 60 min at 70 °C, and then prepared for GC-MS analysis.

Analysis platform: Agilent6890A/ 5973C GC-MS.

Column: Agilent J&W Scientific, DB-5ms (30 m × 0.25mm × 0.25μm).

Parameters: Injector temperature: 280 °C; Ion source temperature: 230 °C; Quadrupole rod temperature: 150 °C; Helium (high purity) >99.999%; Injection Mode: Splitless; Injection volume: 1.0 μL.

Temperature raising procedure: The column temperature was held at 70 °C for 2 min, increased by 10 °C min-1 to 200 °C, then increased by 5 °C min-1 to 280 °C and held there for 6 min. The column effluent was full scanned in the mass range 50-550 m/z.

The data was performed feature extraction and preprocessed with XCMS in R software, and then normalized and edited into two-dimensional data matrix by excel 2010 software, including Retention time (RT), Mass-to-charge ratio (MZ), Observations (samples) and peak intensity.