# Supplementary materials

Occurrence and removal of illicit drugs in different wastewater treatment plants with different treatment techniques

Yanghui Deng1,2, Changsheng Guo1, Heng Zhang1, Xingxing Yin1, Like Chen1, Daishe Wu2, Jian Xu1*

1. State Key Laboratory of Environmental Criteria and Risk Assessment, State Environmental Protection Key Laboratory of Ecological Effect and Risk Assessment of Chemicals, Chinese Research Academy of Environmental Sciences, Beijing 100012, China

2. Key Laboratory of Poyang Lake Environment and Resource Utilization, Ministry of Education, School of Resources Environmental and Chemical Engineering, Nanchang University, Nanchang 330031, China

# *: To whom all correspondence should be addressed. Tel: 86-10-84916028; Fax: 86-10-84916028; E-mail: xujian@craes.org.cn

# Table S1 12 illicit drugs analyte ions and MS parameters

| Target | retention time(min) | precursor ion(m/z) | declustering potential(V) | quota ion |  | qualitative ion |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | collision voltage(V) | product Ion(m/z) | collision voltage(V) | product Ion(m/z) |
| AMP | 1.83 | 136.1 | 14 | 14 | 91 | 9 | 119.1 |
| METH | 1.96 | 150.1 | 22 | 16 | 91 | 10 | 119.1 |
| MC | 1.64 | 164.1 | 20 | 18 | 131 | 22 | 105 |
| MDA | 1.91 | 180 | 22 | 20 | 105 | 16 | 135.4 |
| MDMA | 2.01 | 194.1 | 22 | 12 | 163 | 24 | 105 |
| NK | 2.2 | 224 | 28 | 24 | 125 | 50 | 89.2 |
| KET | 2.27 | 232 | 16 | 24 | 125 | 16 | 179 |
| BE | 2.25 | 290 | 20 | 20 | 168.1 | 28 | 105 |
| COC | 2.71 | 304.1 | 6 | 18 | 182.1 | 28 | 82.1 |
| MTD | 3.93 | 310.1 | 4 | 24 | 105 | 48 | 77.1 |
| HER | 2.61 | 370 | 34 | 48 | 165.1 | 26 | 268.1 |
| COD | 1.74 | 300 | 28 | 26 | 215.1 | 35 | 165.1 |

Table S2 Validation parameters - retention time (Rt), relative retention time (Rel. Rt), linearity range, correlation coefficient obtained from calibration curve, instrumental limits of detection (LODS/N) and instrumental limits of quantification (LOQS/N) , instrumental precision.

| analytes | Rt | Rel. Rt | Recovery rates (%) | Linearity range (µg/L) | Calibration curve | R2 | LODS/N (µg/L) | LOQS/N (µg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (min) | (min) |  |  |  |  |  |  |
| AMP | 1.86 | 0.08 | 90% | 0.1-10 | Y=19828.7x-554.71 | 0.9995 | 0.01 | 0.015 |
| METH | 2.04 | 0.08 | 95% | 0.1-10 | Y=40079.6x-265.875 | 0.9987 | 0.005 | 0.01 |
| MC | 1.67 | 0.08 | 98% | 0.1-10 | Y=36872.3x-1050.63 | 0.997 | 0.01 | 0.02 |
| MDA | 1.93 | 0.07 | 101% | 0.1-10 | Y=11955.7x-417.15 | 0.9992 | 0.02 | 0.025 |
| MDMA | 2.04 | 0.08 | 97% | 0.1-10 | Y=47352.2x-1614.27 | 0.9985 | 0.005 | 0.01 |
| NK | 2.22 | 0.07 | 85% | 0.1-10 | Y=36217.3x-663.94 | 0.999 | 0.01 | 0.02 |
| KET | 2.29 | 0.07 | 105% | 0.1-10 | Y=44710.4x+3013.45 | 0.9997 | 0.02 | 0.05 |
| BE | 2.27 | 0.07 | 110% | 0.1-10 | Y=23465.0x-597.23 | 0.9988 | 0.005 | 0.01 |
| COD | 1.76 | 0.07 | 90% | 0.5-10 | Y=2682.0x-131.27 | 0.9978 | 0.01 | 0.05 |
| COC | 2.74 | 0.07 | 88% | 0.1-10 | Y=21003.5x-10.74 | 0.997 | 0.025 | 0.02 |
| MET | 3.99 | 0.08 | 85% | 0.1-10 | Y=27369.3x-4660.57 | 0.9988 | 0.01 | 0.02 |
| HR | 2.64 | 0.07 | 98% | 0.1-10 | Y=803.6x-55.84 | 0.995 | 0.02 | 0.05 |

# Table S3 Full text abbreviations of professional terms

| # methamphetamine | # METH |
| --- | --- |
| # 3,4-methylenedioxymethamphetamine | # MDMA |
| # benzoylecgonine | # BE |
| # cocaine | # COC |
| # amphetamine | # AMP |
| # methadone | # MET |
| # ketamine | # KET |
| # codeine | # COD |
| # norketamine | # NK |
| # methcathinone | # MC |
| # heroin | # HR |
| # 3,4-methylenedioxyamphetamine | # MDA |
| # methamphetamine-d8 | # METH-d8 |
| # methanol | # MeOH |
| # acetonitrile | # ACN |
| # formic acid | # FA |
| # ammonium hydroxide | # NH3H2O |
| # the solid phase extraction | # SPE |
| # ultra-performance liquid chromatography equipped with tandem mass spectrometry | # UPLC-MS/MS |
| # electrospray ionization | # ESI+ |
| # multiple-reaction monitoring | # MRM |
| # limit of detection | # LOD |
| # wastewater based epidemiology | # WBE |
| # ethylenediaminetetraacetic acid disodium salt | # EDTA-2Na |
| # anoxic oxic | # A/O |
| # limit of detection | # LOD |
| # anaerobic anoxicoxic | # A2/O |
| # anoxic oxic | # A/O |
| # sequencing batch reactor activated sludge | # SBR |
| # wastewater treatment plants | # WWTP |
| # contaminants of emerging concern | # CECs |