Optimization of the miRNA extraction

Isolation kit

While the miRNeasy Serum/Plasma Kit with 200μl of plasma input was found to be the most efficient, the highest absolute yields were achieved with NucleoSpin miRNA Plasma Kit and miRCURY RNA Isolation Kit with an input of 900μl of plasma in both cases. These three abovementioned isolation procedures were further improved using vacuum concentrator (SpeedVac) and glycogen addition (2μg and 10μg; Roche and Invitrogen) when merging two isolations (miRNeasy 400μl; NucleoSpin and miRCURY 1800μl overall input volume) with default elution volumes (miRNeasy 2x14μl; NucleoSpin and miRCURY 2x20μl).

Glycogen and vacuum concentrator

In case of glycogen, both tested trademarks reached comparable results and higher yields were ever achieved with 2μg of glycogen in isolation then with 10μg. While the total miRNA concentrations were increased with addition of 2μg of glycogen as measured with fluorometer, the increments differed between concrete miRNAs as measured with qPCR. Therefore, the glycogen utilization was excluded, because it seemed to affect the overall miRNAs profile.

Regarding the SpeedVac, miRNA concentrations always exactly corresponded to the increases in plasma input volumes as measured with fluorometer. However, inconsistent results were reported in quantification of individual miRNA assays by qPCR. In some assays, samples concentrated with SpeedVac have improved as expected – ct values decreased approximately by one cycle. Nevertheless, in other assays, the ct values of concentrated samples increased in some cases even by two or three cycles. This apparently indicates elevated levels of PCR inhibitors in samples utilized by vacuum concentrator, which as a result decreases the efficiency of PCR amplification with a various impact depending on concrete miRNA concentration; i.e. higher impact on the less-represented miRNAs.

Finally, we compared miRNAs isolated using NucleoSpin and miRCURY, with (1800μl input; 2x20μl elution volumes) and without (900μl input; 20μl elution volumes) using the SpeedVac directly on the miRNA array strip. Indeed, the results of both samples adjusted with SpeedVac showed flag indicating an inhibition of the hybridization procedure, so the application of the SpeedVac was also excluded. Since the sample isolated with NucleoSpin kit achieved slightly better results (higher overall fluorescence signal) then sample isolated with Exiqon kit, it was also finally selected for isolating of clinical samples included to our study.