Gait Asymmetry: When gait becomes less automatic due to sarcopenia and cognitive aging, left-right step coordination may require more effort, especially among frail individuals (17,34,44). Further, studies showed that no strong association between gait variability and asymmetry exists, suggesting that asymmetry reflects an independent measure of gait impairments due to distinct pathological causes (17,44). Here, step asymmetry was obtained from the autocorrelation function of the vertical acceleration signal (17,44), represented by a sequence of autocorrelation coefficients over increasing time lags.

| Asymmetry1= Ad1Ad2, | Asymmetry2 = ¦Ad1-Ad2¦max⁡(Ad1,Ad2) | (1) |
| --- | --- | --- |

where Ad1 and Ad2 are the prominence of the first and the second peaks respectively after the central (zero lag) peak (45).

Gait Irregularity: Results from supervised gait studies showed that irregularity measures can describe predictability of walking cycles, which can be influenced by both neurological and neuromuscular diseases (46–49). Further, within in-lab settings, it has been demonstrated that gait irregularity can differ between non-frail and pre-frail older adults (21). We used Sample Entropy (SampEn) assessment defined as equation (2), where A was defined as the number of matches in the filtered acceleration signal length m+1 (distance function smaller than tolerance r: dXm+1i,Xm+1(j)<r) and B as the number of matches of length m: (dXmi,Xm(j)<r) (50–54).

|  | SampEn=-logi=1N-mAii=1N-mBi=-logA/B | (2) |
| --- | --- | --- |

The time-delay of the signal was calculated using mutual information method for all the continuous walks (55), and the average time-delay of all the continuous walks was used to calculate the SamplEn for each volunteer. We used embedding dimension m=3, and tolerance r=0.2 times the standard deviation of the signal, which are commonly used to compute sample entropy of gait signal (50–54).