Supporting materials

Carbon and nitrogen Isotope fractionation of the photo-degradation of herbicides in aqueous solution by nitrates under ultraviolet radiation

Haiyan Yua, Jinling Liua, Changxu Hana, Han Fanga, Xingquan Shua, Yongfeng Liua, Yuwei Panb, Limin Maac

a College of Environmental Science and Engineering, Tongji University, Shanghai 200092, China

b College of Biology and the Environment, Nanjing Forestry University, Nanjing 210037, China

c Shanghai Institute of Pollution Control and Ecological Security, Shanghai 200092, China

| Table S1 Pseudo-first order constant (kobs) of IUP and MN under different light conditions. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Hg lamp |  |  | Xe lamp |  |  |
|  | kobs (min-1) | t1/2 (min-1) | r² | kobs (min-1) | t1/2 (h-1) | r² |
| IUP | 0.0319 | 21.7 | 0.9976 | 0.0029 | 239 | 0.9899 |
| MN | 0.1621 | 4.28 | 0.9973 | 0.0854 | 8.12 | 0.9995 |

| Table S2 Pseudo-first order constant (kobs) of IUP and MN under different initial concentrations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Initial concentration | Degradation kinetics equation | kobs (min-1) | t1/2(min-1) | r² |
| IUP | 6 | y = -0.0654x + 0.2517 | 0.065 | 10.6 | 0.97 |
|  | 12 | y = -0.0493x + 0.1943 | 0.049 | 14.1 | 0.99 |
|  | 25 | y = -0.0199x + 0.0297 | 0.020 | 34.8 | 0.99 |
| MN | 6 | y = -0.4365x - 0.0802 | 0.437 | 1.59 | 0.97 |
|  | 12 | y = -0.2711x - 0.1679 | 0.271 | 2.56 | 0.98 |
|  | 25 | y = -0.17x - 0.0679 | 0.170 | 4.08 | 0.99 |

| Table S3 Pseudo-first order constant (kobs) of IUP and MN under different concentrations of NO3− |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| substrate concentration | NO3− concentration | IUP |  |  |  | MN |  |  |
|  |  | r2 | kobs (min-1) | t1/2(min-1) |  | r2 | kobs (min-1) | t1/2(min-1) |
| 8 | 0.5 | 0.99 | 0.0065 | 106.64 |  | 0.97 | 0.213 | 32.54 |
|  | 1 | 0.95 | 0.0068 | 101.93 |  | 0.96 | 0.0254 | 27.29 |
|  | 5 | 0.95 | 0.0222 | 31.22 |  | 0.99 | 0.0272 | 25.48 |
|  | 0 | 0.97 | 0.0018 | 385.01 |  | 0.99 | 0.0068 | 101.93 |
| 20 | 5 | 0.99 | 0.0442 | 15.68 |  | 0.99 | 0.0347 | 19.98 |
|  | 10 | 0.98 | 0.0313 | 22.36 |  | 0.99 | 0.0318 | 21.80 |
|  | 20 | 0.95 | 0.0239 | 29.00 |  | 0.99 | 0.0289 | 23.98 |
|  | 0 | 0.93 | 0.00009 | 7701.3 |  | 0.99 | 0.0058 | 119.51 |

| Table S4 Pseudo-first order constant (kobs) of IUP and MN under different initial in the presence of NO3− |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Substrate concentration(mg/L) | r2 | kobs (min-1) | t1/2(min-1) |
| IUP | 8 | 0.95 | 0.0222 | 31.22 |
|  | 15 | 0.95 | 0.019 | 36.48 |
|  | 20 | 0.98 | 0.008 | 86.64 |
| MN | 8 | 0.99 | 0.0272 | 25.48 |
|  | 15 | 0.99 | 0.0228 | 30.40 |
|  | 20 | 0.99 | 0.0201 | 34.45 |

Corresponding author at: College of Environmental Science and Engineering, Tongji University, Shanghai 200092, China

E-mail address: lmma@tongji.edu.cn