# ACT-seq Appendices

# Appendix A: Expression and Purification of PA-Tnp

# REQUIRED REAGENTS

# Lysis Buffer

# 50 mM Tris-HCl pH 8.0

# 300 mM NaCl

# 20 mM Imidazole

# 0.1% Triton X-100

# 10 μg/mL Pepstatin A (calbiochem cat# 516481)

# 10 μg/mL Leupeptin Hemisulfate (calbiochem cat# 108975)

# 10 μg/mL Chymostatin (calbiochem cat# 230790)

# 6 μg/mL Antipain dihydrochloride (Sigma cat# A6191)

# 1 mg/mL Lysozyme

# Elution Buffer

# 50 mM Tris-HCl pH 8.0

# 1 M NaCl

# 250 mM Imidazole

# 0.1% Triton X-100

# Materials:

PA-Tnp Expression Construct (Addgene acc# 121137)

BL21-Gold DE3 Competent Cells (Agilent cat# 230132)

Ni-NTA Agarose Beads (Qiagen cat# 1018244)

# PROCEDURE

- Inoculate 10 mL of LB containing 100 g/mL ampicillin with a glycerol stock of BL21 (DE3) Gold transformed with the PA-Tnp T7 expression construct. Shake at 37 °C overnight.

- Transfer the 10 mL of culture into 1.2 L prewarmed medium (2x600 mL) and continue shaking at 37 °C until the cultures have an OD of 600 to 0.5.

- Cool on ice-water to room temperature. Add 0.4 mL of 1 M IPTG to final concentration 0.3 mM. Shake at room temperature for 4 hours.

- Harvest bacteria by centrifugation at 3700 rpm at 4 °C for 15 min. Suspend the cell pellets in 30 mL of Lysis Buffer and incubate on ice for 30 minutes.

- Lyse the cells by sonication with a microtip on a Misonix S-4000 sonicator (or equivalent) using a total process time of 105” with 15” on, 30” off, and 90% for amplitude.

- Spin the lysates at 15,000 rpm for 10 min at 4 °C. Transfer the supernatant to a new tube and add 5 mM 2-ME (2-mecapotoethenal), 1 mM PMSF and 1 M NaCl.

- Mix with 0.8 mL of 50% Ni-NTA agarose bead slurry and rotate for 1 hour at 4 °C.

- Transfer the beads to a 1 mL syringe with glass wool on the bottom. Wash the beads with 20 mL wash buffer. Elute the bound PA-Tnp using 3 mL of Elution Buffer by loading 0.5 mL of Elution Buffer six times and collecting the eluates into six 1.5 mL tubes. Add a final concentration of 1 mM dithiothreitol and 50% glycerol. Check the purified proteins by SDS-PAGE.

- Store protein in 15 to 50 L aliquots at -80 °C. Once removed for use, store at -20 °C to avoid freeze-thaw. Aliquots of enzyme remain good at -20 °C for several weeks.

# Appendix B: Oligonucleotide Complex Barcodes

# The complex barcode indexes were obtained by annealing the pMENTS oligo to the i5 and i7 oligos listed below. The following steps were performed to anneal the oligos:

- Mix 500 l of 100 M pMENTS with 500 l of 100 M i5 or i7.

- Heat to 99 °C for 5 min. Turn off the heat source and allow the solutions to cool to room temperature over the course of two hours.

- Store the resulting 50 M oligos at -20 °C.

| # Oligo Name | # Sequence |
| --- | --- |
| # pMENTS | # 5Phos-CTGTCTCTTATACACATCT |
| # i5 | # TCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| # i7 | # GTCTCGTGGGCTCGGAGATGTGTATAAGAGACAG |

# Appendix C: Oligonucleotide Library Barcodes

# These barcodes are used at the PCR step. Each sample/replicate needs a unique pair of barcodes in order to be distinguished after the samples are mixed. The template for these oligos are given below, with the red bases indicating the variable regions that distinguish the oligos:

# 5’ library barcode template:

# AATGATACGGCGACCACCGAGATCTACACNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG

# 3’ library barcode template:

# CAAGCAGAAGACGGCATACGAGATNNNNNNNNGTCTCGTGGGCTCGGAGATGTGTATAAGAGACAG

# For the sequences that are used in the variable regions, see the “Nextera DNA Indexes” list in the Illumina Adapter Sequences Document (available online from Illumina). The 5’ library barcodes use Illumina’s i5 variable sequences, and the 3’ library barcodes use Illumina’s i7 variable sequences.