One-century sedimentary record, sources, and ecological risk of polycyclic aromatic hydrocarbons in Dianchi Lake, China

Table S1 Normalized distribution coefficient of TOC (KTOC) (Li et al., 2019；Neff et al., 2005)

| PAHS | Nap | Acy | Ace | Flo | Phe | Ant | Flu | Pyr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log KTOC | 3.37 | 4.07 | 3.92 | 4.18 | 4.46 | 4.54 | 5.22 | 5.18 |
| PAHS | BaA | Chr | BbF | BkF | BaP | DahA | InP | BghiP |
| Log KTOC | 5.91 | 5.86 | 5.8 | 6 | 6.04 | 6.75 | 7 | 6.5 |

Table S2 The concentration ranges of each PAH

| PAHs | Minimum (ng/g) | Maximum (ng/g) | Average (ng/g) |
| --- | --- | --- | --- |
| Nap | 75 | 224 | 121 |
| Acy | 2.4 | 5.7 | 3.3 |
| Ace | 0.7 | 32 | 12.7 |
| Flo | 2.3 | 50 | 29.3 |
| Phe | 118 | 359 | 178 |
| Ant | 0.8 | 22 | 5.2 |
| Flu | 19 | 140 | 47 |
| Pyr | 9 | 132 | 56 |
| BaA | 7 | 32 | 15 |
| Chr | 3 | 55 | 9.6 |
| BbF | 4.8 | 17 | 10.5 |
| BkF | 11 | 88 | 34 |
| BaP | 4.5 | 66 | 17 |
| DBA | 8.5 | 14 | 11 |
| InP | 10 | 16 | 14 |
| Bghip | 5.6 | 9.9 | 7.5 |
| ΣPAH16 | 368 | 990 | 572 |

Note：ND, the concentration of individual PAH compound was not detected.

Table S3 The toxicity value of ∑PAH16 in pore water (μg/L) (Neff et al., 2005)

| PAHs | Acute toxicity | Chronic toxicity |
| --- | --- | --- |
| Nap | 4870 | 970 |
| Acy | 1181 | 180 |
| Ace | 1360 | 270 |
| Flo | 730 | 150 |
| Phe | 367 | 55 |
| Ant | 300 | 60 |
| Flu | 55 | 11 |
| Pyr | 61 | 12 |
| BaA | 9.8 | 2.0 |
| Chr | 11 | 2.2 |
| BbF | 14 | 2.9 |
| BkF | 8.6 | 1.7 |
| BaP | 7.6 | 1.5 |
| DBA | 1.3 | 0.25 |
| InP | 0.64 | 0.13 |
| BghiP | 2.4 | 0.49 |

Table S4 Health Risk Assessment Equivalent Factor of seven PAH congeners

| PAHs单体 | TEQ | MEQ |
| --- | --- | --- |
| BaA | 0.1 | 0.08 |
| Chr | 0.01 | 0.02 |
| BbF | 0.1 | 0.25 |
| BkF | 0.1 | 0.11 |
| BaP | 1 | 1 |
| InP | 0.1 | 0.31 |
| DBA | 1 | 0.29 |

Table S5 Designation and abbreviation

| Designation | Abbreviation | Rings |
| --- | --- | --- |
| Polycyclic aromatic hydrocarbon | PAH |  |
| ANT | anthracene | 2 |
| Ace | acenaphthene | 3 |
| Acy | acenaphthylene | 3 |
| Flo | fluorine | 3 |
| Phe | phenanthrene | 3 |
| Ant | anthracene | 3 |
| Flu | fluoranthene | 4 |
| Pyr | pyrene | 4 |
| BaA | benz(a)anthracene | 4 |
| Chr | chrysene | 4 |
| BbF | benzo(b)fluoranthene | 5 |
| BkF | benzo(k)fluoranthene | 5 |
| BaP | benzo(a)pyrene | 5 |
| DahA | dibenz(a,h)anthracene | 5 |
| Inp | indeno(1,2,3-cd)pyrene | 6 |
| BghiP | benzo(g,h,i)perylene | 6 |
| Low molecular weight PAHs （2-3 Ring） | LMW |  |
| High molecular weight PAHs （4-6 Ring） | HMW |  |

References

Li, Y., Wang, G.M., Wang, J.X., Jia, Z., Zhou, Y., Wang, C., Li, Y., Zhou, S., 2019. Determination of influencing factors on historical concentration variations of PAHs in West Taihu Lake, China. Environ. Pollut. 249, 573-580.

Neff, J.M., Stout, S.A., Gunster, D.G., 2005. Ecological Risk Assessment of Polycyclic Aromatic Hydrocarbons in Sediments: Identifying Sources and Ecological Hazard. Int. environ. Assess. Manag. 1, 22-33.