## Supplementary Information

Table 1 Data monitoring of the frequency and diameter of gas bubbles in the Nirano mud field (N1-N18). Refer to Figure 1 of the gryphons and mud pools.

| Location | Bubble diameter min (m) | Bubble diamters max (m) | Frequency min (s-1) | Frequency min (s-1) | ML (m a.s.l.) | Average gas flow rate (m3/sec) |
| --- | --- | --- | --- | --- | --- | --- |
| N1 | 0.15 | 0.20 | 0.33 | 0.50 | 206.45 | 0.0012 |
| N2 | 0.15 | 0.20 | 0.20 | 0.50 | 207.44 | 0.0010 |
| N3 | 0.00 | 0.05 | 0.50 | 1.00 | 207.59 | 0.0000 |
| N4 | No bubbles |  |  |  | 207.84 | 0.0000 |
| N5 | No bubbles |  |  |  | 207.60 | 0.0000 |
| N6 | 0.02 | 0.03 | 0.10 | 0.50 | 207.58 | 0.0000 |
| N7 | 0.02 | 0.03 | 0.05 | 0.10 | 207.73 | 0.0000 |
| N8 | 0.05 | 0.10 | 0.10 | 0.50 | 207.41 | 0.0001 |
| N9 | 0.02 | 0.03 | 0.20 | 0.50 | 212.76 | 0.0000 |
| N10 | 0.05 | 0.10 | 0.50 | 1.00 | 208.68 | 0.0002 |
| N11 | 0.04 | 0.05 | 0.00 | 0.01 | 207.96 | 0.0000 |
| N12 | 0.03 | 0.05 | 0.33 | 1.00 | 206.32 | 0.0000 |
| N13 | 0.02 | 0.03 | 0.01 | 0.02 | 203.58 | 0.0000 |
| N14 | 0.10 | 0.20 | 0.50 | 1.00 | 210.02 | 0.0013 |
| N15 | 0.02 | 0.03 | 0.17 | 0.20 | 224.13 | 0.0000 |
| N16 | 0.08 | 0.25 | 0.20 | 0.33 | 223.45 | 0.0006 |
| N17 | 0.01 | 0.08 | 0.50 | 1.00 | 222.49 | 0.0000 |
| N18 | 0.01 | 0.05 | 0.50 | 1.00 | 221.36 | 0.0000 |