Table 9. Recommended benchmarking metrics and reporting standards for future studies on soft pneumatic grippers.

| Metric | Current Reporting Rate | Recommended Minimum Standard | Priority |
| --- | --- | --- | --- |
| Cycle life (actuation cycles) | ~15% | ≥1,000 cycles | High |
| Grasping success rate (standard objects) | 17% | ≥10 objects, ≥50 trials | High |
| Energy consumption per grasp | <10% | J/grasp or Wh/cycle | High |
| Standardized object set testing | ~25% | YCB or similar benchmark set | High |
| Statistical analysis (mean ± SD) | ~30% | Include variability | Medium |