Table S1. Summary of key numerical performance outcomes from 47 experimental studies on soft pneumatic grippers published between 2016 and 2025. The table presents actuation pressure, bending/deflection performance, grasping and pull-off forces, payload capacity, stiffness modulation, and other critical metrics across twelve design categories to enable systematic comparison of innovations and performance benchmarks.

| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| --- | --- | --- | --- | --- | --- | --- |
| Hiremath et al. (2025) | Discrete Chamber (TPU 3D-printed) | TPU (MEX 3D printing) | 1–5 bar | Max deflection: 72 mm Bending: 17–72 mm | Held 1.29–13.26 g | Excellent adaptability & compliance |
| Buonamici et al. (2025) | Anthropomorphic PneuNet | TPU (FFF 3D printing) | Up to 3 bar | Bending: 68.7° at 3 bar Transversal force: 17.16 N | 100% success on several fruits/vegetables | Strong potential for assistive/rehab use |
| El Bana et al. (2020) | Silicone PneuNet | Dragon Skin 20 | Low pressure | High bending radius at low pressure | Fast actuation | Good for fragile objects |
| Arumathanthri et al. (2018) | Multi-chamber | Natural rubber | Up to 230 kPa | Bending: 25° at 230 kPa Max lift: 215 g | Easier grasp on low-friction objects | Effective for delicate food handling |
| Jiang et al. (2025) | Vision-controlled | Dragon Skin 10 | — | Bending: 60.6° (vs 90° target) Cost reduction: >60% | 100% fabrication yield | Practical framework for agricultural robotics |
| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| Bhat et al. (2025) | Human finger-inspired | LSR (cast) | — | Bending & force increased with pressure | Mimicked natural finger motion | Strong potential for prosthetics & rehab |
| Pedro et al. (2018) | Biomimetic (Sea lamprey) | Elastomer | 0.05–0.15 kgf/cm² | Load: 0.048 kg to >1 kg Max force: 4.353 N | Good force control | Simple, effective wide-range grasping |
| Zhang et al. (2023) | Fabric-reinforced contractile | TPU-coated nylon fabric | 50 kPa | Tip force: 45 N Pull-off: >150 N Payload: >7 kg | 400× dead-weight ratio | Excellent load-bearing & tunable workspace |
| Yang et al. (2020) | Twining plant-inspired | Ecoflex + Dragon Skin 10 | 0.67 MPa | Twining angle: 540° Detection: 1 mm objects | High repeatability in confined spaces | Accurate sensing + grasping in tight spaces |
| Antonelli et al. (2024) | Seahorse tail-inspired | TPU + R PRO20 silicone | 1.8 bar | Extension: 154.5 mm Push force: 11.9 N | Activation: 74.9 ms | Effective leg module for multi-legged robots |
| Ali et al. (2019) | Fin Ray effect | NinjaFlex | — | Payload: up to 3 kg | High deformation & conformal wrapping | Effective for varied shapes |
| Kitamura et al. (2023) | Fin Ray (variable cross-beam) | Elastic50A resin | — | Peak force: up to 20.8 N | Optimal angles depend on object shape | Cross-beam angle selection is object-dependent |
| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| Müller et al. | Heavy-duty Fin Ray | TPU-95A | — | Pull-off force: ~40 N | Validated via simulation | Low-cost validation method for heavy-duty use |
| Hao et al. (2016) | Reconfigurable (variable length) | Dragon Skin 30 | 30–70 kPa | Pull-off force: 13.5 N (3× increase) Opening: 160 mm | Optimal finger length exists per object size | Tunable length significantly improves performance |
| Mutlu et al. (2017) | Variable stiffness (bellows) | NinjaFlex (FFF) | Positive pressure | Stiffness increase: 30% at 2 N | Higher load carrying in stiffened state | Effective variable compliance |
| Zhao et al. (2023) | Modular variable stiffness | Silicone + Velcro + particles | 90 kPa | Bending: 140° Load capacity: 710 → 1313 g | Good model-experiment agreement | Wide controllable stiffness range |
| Al Abeach et al. (2017) | Antagonistic variable stiffness | Silicone | 100–400 kPa | Stiffness ↑ 163% Contractor force: 200 N | PID accuracy: ±0.5 mm | Effective stiffness modulation + positioning |
| Li et al. (2017) | Passive particle jamming | Silicone + particles | 20–80 kPa | Stiffness ↑ >6× Load: 369 → 1673 g | Optimized with smaller particles & thinner walls | Simple & effective stiffness enhancement |
| Zhuo et al. (2020) | Pneumatic-thermal hybrid | Ecoflex + organohydrogels | 25 kPa | Modulus: 1.29 → 0.10 MPa Cycle life: >2000 | Self-healing: 88% strength recovery | Stiffness-matching for fragile objects |
| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| Xiang et al. (2022) | Electroadhesive + LMPA variable stiffness | Ecoflex 00-30 + LMPA | 60 kPa | Stiffness: 0.143 → 0.016 N/mm Force: 10.74 N (rigid) | Successfully handled fragile/irregular objects | Combined electroadhesion + variable stiffness |
| Han et al. (2025) | Metamaterial-integrated | — | 100 kPa | Conformity error: 1.80 → 0.13 mm (planar) | Neural network inverse design | Significantly improved surface conformity |
| Tawk et al. (2022) | Metamaterial pads | TPU (3D printed) | 150 kPa | Contact pressure ↓ 4.3–8.6× Tip force: 1.94 N | Stable grasping in 2/3/4-finger configs | Improved conformability & stability |
| Gerez et al. (2020) | Soft-rigid hybrid (telescopic) | Vytaflex 40 + PLA | — | Elongation: 5.3× (20→100+ mm) Clench: 59.9 N | Resisted >40 N pull-off | Versatile for real-world use |
| Zhu et al. (2023) | Parallel hybrid (rigid + soft) | — | — | Payload: 0.1 g – 27 kg 94% span + 4 cm offset | High compliance + stability | Wide force range with compliance |
| Peng et al. (2024) | Soft + external rigid supports | Ecoflex + stainless steel | 27.3 kPa | Lifting force: 47 N (↑150%) Payload: up to 5 kg | Outperformed previous SPAs at lower pressure | Enhanced lifting while maintaining adaptability |
| Chen et al. (2020) | High-voltage hybrid | — | 160 kPa | Fingertip force: 51 N Payload ratio: 21.3:1 | Operated up to 492.3 kV AC | Reliable under high-voltage conditions |
| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| Netzev et al. | Compliant cell stacks | TPC (FDM 3D printing) | — | 120/120 successful trials (turbine blade) | High success across industrial parts | Effective for industrial benchmarking |
| Zhang et al. (2022) | Bistable CFRP | Silicone E615 + CFRP | 0.099–0.171 MPa | Payload: 77–105 g Energy: 0.0207 J/g | Fast snap-through (0.47 s) | Energy-efficient bistable grasping |
| Nguyen et al. (2020) | Tristable | Bi-stable springs | — | Pull force ↑ 186–711% | Adaptive pinch & wrap grasps | Improved reliability for small/flat objects |
| Wang et al. (2024) | Origami-based | Ecoflex 00-30 | ~6 psi | Bending: nearly 360° Lift: up to 60 g | Gesture recognition: up to 95% | Large range of motion + sensing |
| Wang et al. (2025) | FlexiRay (visual-tactile) | TPU + multi-layer sensing | — | Force accuracy: 0.17 N Texture recognition: 96.7% | High sensing coverage during deformation | High-resolution multimodal perception |
| Sithiwichankit et al. (2023) | Pincer with stiffness sensing | Ecoflex 00-30 + conductive PLA | — | Stiffness estimation error: 15% | <5% signal variation after 1000 cycles | Direct stiffness sensing during grasping |
| Goh et al. (2022) | Triboelectric AI-assisted | Ecoflex + porous Eco-EGaIn | — | Classification accuracy: 91.7% | 63% higher sensor output with porous layer | Self-powered real-time feedback |
| Zou et al. (2024) | Retrofit fluidic sensing | Multiple actuators | — | Resolution: 0.02–0.04 kPa/mm | Millimeter-level accuracy | Sensing without embedded sensors |
| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| Huang et al. (2025) | Electroadhesion-enhanced dual | — | — | Max mass: 1230 g (↑13.8%) | Handled fragile, irregular & large objects | Hybrid pneumatic + electroadhesion synergy |
| Zhou et al. (2025) | Everything-Grasping (EG) | Granular jamming + surface tension | — | Synergistic force: 6.14 N Range: 0.2 mm² – 62,000 mm² | Robust cross-scale & underwater performance | Universal single-architecture grasping |
| Mirvakili et al. (2021) | Solar-thermal phase-change | EcoFlex 00-50 | ~85 kPa | Lift: up to 0.5 kg (25× fluid mass) | Worked under sunlight | Compressor-free untethered actuation |
| Kim et al. (2020) | Tendon-driven origami pump | EcoFlex 00-30 | 33.56 kPa | Bending: 107.95° | No external compressor needed | Compact, lightweight air source |
| Zhang et al. (2023) (electromagnetic) | Electromagnetic variable pressure | — | Tunable sinusoidal | Continuous adjustable pressure waveform | Good airtightness | Suitable for pneumatic soft robots |
| Zhang et al. (2022) FPSED | Energy-harvesting high-speed | Silicone + Kevlar | — | Harvesting efficiency: 27.86–31.82% | Captured objects at 4 m/s | Reliable high-speed capture |
| Zhang et al. (2025) | High-speed oyster handling | — | 40 kPa (optimum) | Success rate: 100% | No damage/misalignment | Stable high-speed aquatic handling |
| Kultongkham et al. (2021) | Tomato harvesting | Platinum silicone | — | Max force: 1.62 N (< bio-yield 2.57 N) | Successful force-controlled harvesting | Safe for delicate produce |
| Study (Year) | Category | Material / Fabrication | Actuation Pressure | Key Performance Metrics | Notable Numerical Outcome | Main Finding |
| Wang et al. (2016) (lunch box) | Lunch box packing | Fluidic elastomer | 40–60 kPa | Successful lifting at 40 kPa (Designs 1 & 3) | Handled variable shapes & softness | Practical for automated packing |
| Krahn et al. (2017) | Friction + suction | ABS + membrane | Low chamber pressure | Grip strength: 10.1 N at 7.47 kPa | Gentle yet secure grasping | Low power + effective for fragile objects |
| Bryan et al. (2019) | Humanoid + bioinspired suction | Ecoflex + Mold Star | — | Max payload: 270 g | Successfully manipulated varied shapes | Versatile on industrial robot arm |
| Low et al. (2016) | Surgical tissue manipulation | Silicone (rod-cast) | — | Max compressive force: 0.79 N | Significantly gentler than rigid forceps | Safe for delicate surgical use |