Table S1. Mathematical formulations, parameter metrics, and weight factors (Wi) assigned to individual water quality variables for WQI determination

| Parameter | Permissible standard (Si, WHO) | Assigned weight (wi) | Relative weight factor (Wi) | Parameter sub-index formula(qi) |
| --- | --- | --- | --- | --- |
| pH | 6.5-8.5 | 4 | 0.125 | qpH=(pH-7)(8.5-7)X100 |
| EC (µS cm-1) | 1000 | 4 | 0.125 | qi=CiSix100 |
| Calcium (Ca2+, mg L-1) | 75 | 2 | 0.0625 | qi=CiSix100 |
| Magnesium Mg2+, mg L-1) | 30 | 2 | 0.0625 | qi=CiSix100 |
| Sodium (Na+, mg L-1) | 200 | 3 | 0.09375 | qi=CiSix100 |
| Chloride (Cl-, mg L-1) | 250 | 3 | 0.09375 | qi=CiSix100 |
| Sulphate (SO42-, mg L-1) | 250 | 4 | 0.125 | qi=CiSix100 |
| Nitrate (NO3-, mg L-1) | 45 | 5 | 0.15625 | qi=CiSix100 |
| Phosphate (PO43-, mg L-1) | 0.1 | 5 | 0.15625 | qi=CiSix100 |
| Total | - | wi=32 | Wi=1.000 | WQI=Wi×qi |

Table S2. Standardized metric boundaries for WQI status and consumer utility classifications

| WQI Range | Water quality status | Consumer utility and safety verdict |
| --- | --- | --- |
| <50 | Excellent water quality | Safe; completely clear, unpolluted baseline water table profiles |
| 50.1-100.0 | Good water quality | Acceptable; slight mineral adjustments, safe for general domestic use |
| 100.1-200.0 | Poor water quality | Restricted; entry of untreated municipal runoff, demands treatment |
| 200.1-300.0 | Very poor water quality | High Hazard; severe degradation, strictly restricted from intake channels |
| >300 | Unsuitable for consumption | Toxic Matrix; extreme industrial loading, posing immediate health threats. |

Table S3. Hydrochemical parameters across sampling seasons and sites

| Season | Site ID | pH | EC (µS/cm) | Ca (mg/L) | Mg (mg/L) | Na (mg/L) | K (mg/L) | HCO₃ (mg/L) | CO₃ (mg/L) | Cl (mg/L) | SO₄ (mg/L) | NO₃ (mg/L) | CBE (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Mons | S1 | 7.14 | 1575.00 | 132.31 | 34.05 | 738.76 | 11.49 | 402.73 | 0.00 | 836.62 | 206.43 | 92.57 | 7.49 |
| Post-Mons | S1 | 5.86 | 2570.00 | 128.26 | 46.17 | 92.82 | 22.22 | 122.00 | 0.00 | 320.00 | 180.00 | 192.59 | -9.41 |
| Pre-Mons | S2 | 7.23 | 465.00 | 71.17 | 72.96 | 59.91 | 7.51 | 386.08 | 0.00 | 113.44 | 170.60 | 132.25 | -10.37 |
| Post-Mons | S2 | 7.34 | 374.00 | 65.00 | 15.00 | 51.50 | 3.81 | 69.50 | 0.00 | 65.23 | 109.49 | 113.22 | -1.94 |
| Pre-Mons | S3 | 7.47 | 2000.00 | 104.10 | 94.85 | 759.23 | 31.98 | 866.44 | 0.00 | 921.70 | 230.73 | 81.00 | 0.56 |
| Post-Mons | S3 | 7.94 | 1154.00 | 96.19 | 38.88 | 375.10 | 24.18 | 305.02 | 0.00 | 553.02 | 184.19 | 79.29 | -1.55 |
| Pre-Mons | S4 | 7.47 | 119.90 | 31.01 | 80.00 | 11.10 | 3.01 | 219.67 | 0.00 | 42.54 | 59.47 | 85.62 | 7.90 |
| Post-Mons | S4 | 7.10 | 158.10 | 28.06 | 29.16 | 12.64 | 6.56 | 26.85 | 0.00 | 85.08 | 97.35 | 3.80 | -4.35 |
| Pre-Mons | S5 | 7.72 | 693.00 | 119.21 | 89.98 | 79.58 | 53.49 | 524.74 | 0.00 | 184.34 | 196.72 | 82.65 | -2.80 |
| Post-Mons | S5 | 7.67 | 451.00 | 112.22 | 38.88 | 55.22 | 45.57 | 355.27 | 0.00 | 141.80 | 132.57 | 4.51 | -1.15 |
| Pre-Mons | S6 | 5.74 | 4450.00 | 83.78 | 107.01 | 761.53 | 5.59 | 573.59 | 0.00 | 920.00 | 548.94 | 166.97 | -3.38 |
| Post-Mons | S6 | 5.40 | 1712.00 | 84.17 | 38.88 | 904.80 | 23.62 | 597.94 | 0.00 | 1063.50 | 215.16 | 85.84 | 1.81 |
| Pre-Mons | S7 | 7.40 | 611.00 | 87.67 | 72.96 | 97.06 | 70.65 | 707.81 | 0.00 | 155.98 | 3.60 | 87.61 | -3.19 |
| Post-Mons | S7 | 8.14 | 514.00 | 90.17 | 29.16 | 71.10 | 46.70 | 444.38 | 0.00 | 170.16 | 12.94 | 6.60 | -5.39 |
| Pre-Mons | S8 | 8.81 | 283.00 | 39.07 | 29.18 | 93.15 | 8.56 | 390.00 | 0.00 | 56.72 | 52.79 | 75.70 | -8.93 |
| Post-Mons | S8 | 8.47 | 291.00 | 53.00 | 21.87 | 31.10 | 0.81 | 231.00 | 0.00 | 42.54 | 61.52 | 0.07 | -3.72 |
| Pre-Mons | S9 | 8.23 | 518.00 | 48.09 | 58.37 | 150.42 | 12.12 | 526.24 | 0.00 | 99.26 | 188.82 | 84.63 | -8.66 |
| Post-Mons | S9 | 7.84 | 171.70 | 64.00 | 14.58 | 40.13 | 0.69 | 227.16 | 0.00 | 56.72 | 82.17 | 7.81 | -7.54 |
| Pre-Mons | S10 | 8.10 | 421.00 | 43.01 | 51.07 | 104.65 | 12.94 | 486.20 | 0.00 | 56.72 | 117.16 | 80.66 | -8.47 |
| Post-Mons | S10 | 8.19 | 281.00 | 75.00 | 29.16 | 25.00 | 4.19 | 292.60 | 0.00 | 42.54 | 116.17 | 2.01 | -7.03 |
| Pre-Mons | S11 | 7.12 | 2550.00 | 111.23 | 68.10 | 1066.74 | 11.42 | 427.14 | 0.00 | 1432.18 | 281.74 | 106.45 | 2.52 |
| Post-Mons | S11 | 6.61 | 2420.00 | 108.22 | 46.17 | 95.60 | 21.76 | 113.26 | 0.00 | 300.00 | 190.00 | 143.78 | -8.79 |
| Pre-Mons | S12 | 7.59 | 94.80 | 35.13 | 24.32 | 71.76 | 2.27 | 170.86 | 0.00 | 56.72 | 9.07 | 83.64 | 7.73 |
| Post-Mons | S12 | 8.07 | 97.00 | 65.00 | 12.15 | 12.92 | 1.23 | 125.26 | 0.00 | 56.72 | 76.70 | 1.23 | -4.28 |
| Pre-Mons | S13 | 7.65 | 98.30 | 39.61 | 21.89 | 71.76 | 3.01 | 207.47 | 0.00 | 42.54 | 63.72 | 85.62 | -2.33 |
| Post-Mons | S13 | 7.94 | 110.20 | 61.00 | 12.15 | 6.41 | 2.11 | 170.86 | 0.00 | 28.36 | 64.55 | 1.07 | -6.26 |
| Pre-Mons | S14 | 7.53 | 103.00 | 31.01 | 29.18 | 70.61 | 2.35 | 170.86 | 0.00 | 42.54 | 78.90 | 86.61 | 0.28 |
| Post-Mons | S14 | 7.96 | 92.70 | 71.00 | 12.15 | 5.73 | 1.04 | 183.06 | 0.00 | 28.36 | 48.16 | 0.33 | 0.11 |
| Pre-Mons | S15 | 7.76 | 102.60 | 37.01 | 29.18 | 49.22 | 2.93 | 109.84 | 0.00 | 42.54 | 91.66 | 88.60 | 0.98 |
| Post-Mons | S15 | 7.05 | 146.00 | 53.00 | 9.72 | 16.55 | 1.43 | 73.22 | 0.00 | 14.18 | 71.23 | 3.52 | 14.46 |

Table S4. Standard default values, exposure parameters, and toxicological reference metrics utilized for the US EPA human health risk assessment equations across variable demographic segments.

| Parameter | Parameter definition and unit | Demographic segment | Assigned value | Reference source |
| --- | --- | --- | --- | --- |
| CPW | Contaminant concentration in water (mg L-1) | All segments | Site-specific data | Field analysis |
| IR | Human ingestion rate (L day-1) | Adults (Men/Women) | 2.50 | US EPA (1989; 2001) |
|  |  | Children | 0.78 |  |
| ED | Exposure duration (years) | Men | 64 | This study/ Adimalla (2019) |
|  |  | Women | 67 |  |
|  |  | Children | 12 |  |
| EF | Exposure frequency (days year-1) | All segments | 365 | US EPA (1989; 2001) |
| ABW | Average body weight (kg) | Men | 65 | This study/ Adimalla (2019) |
|  |  | Women | 55 |  |
|  |  | Children | 15 |  |
| AET | Average exposure time (days) | Men | 23,360 | Calculated (ED X EF) |
|  |  | Women | 24,455 |  |
|  |  | Children | 4,380 |  |
| TC | Contact Duration (hours day-1) | All segments | 0.4 | This study/ Adimalla (2019) |
| Ki | Dermal Adsorption Parameter (cm h-1) | All segments | 0.001 | This study/ Adimalla (2019 |
| EV | Bathing Frequency (times day-1) | All segments | 1 | This study/ Adimalla (2019 |
| SSA | Skin Surface Area for Contact (cm2) | Adults (Men/Women) Children | 16,600 | This study/ Adimalla (2019 |
|  |  |  | 12,000 |  |
| CF | Unit Conversion Factor (dimensionless) | All segments | 0.001 | Liu et al. (2019); Wu & Sun (2016) |
| RfD | Toxicological Reference Dose (mg kg-1 day-1) | All segments | 1.6 | US EPA IRIS (1989) |

References

United States Environmental Protection Agency (US EPA). (1989). Risk Assessment Guidance for Superfund, Volume I: Human Health Evaluation Manual (Part A). EPA/540/1-89/002. Washington, DC.

United States Environmental Protection Agency (US EPA). (2001). Risk Assessment Guidance for Superfund: Volume III – Part A, Process for Conducting Probabilistic Risk Assessment. EPA 540-R-02-002. Washington, DC.

United States Environmental Protection Agency Integrated Risk Information System (US EPA IRIS). (1989). Nitrate-nitrogen toxicological reference benchmark parameters. Washington, DC.

Adimalla, N., & Qian, H. (2019). Groundwater quality evaluation using water quality index (WQI) for drinking purposes and human health risk (HHR) assessment in an agricultural region of Nanganur, south India. Ecotoxicology and Environmental Safety, 176, 153–161. https://doi.org/10.1016/j.ecoenv.2019.03.066

Liu, J., Jin, D., Wang, T., Gao, M., Yang, J., & Wang, Q. (2019). Hydrogeochemical processes and quality assessment of shallow groundwater in Chenqi coalfield, Inner Mongolia, China. Environmental Earth Sciences, 78(12), 347. https://doi.org/10.1007/s12665-019-8355-4

Wu, J. and Sun, Z., 2016. Evaluation of shallow groundwater contamination and associated human health risk in an alluvial plain impacted by agricultural and industrial activities, mid-west China. Exposure and health, 8(3), pp.311-329. https://doi.org/10.1007/s12403-015-0170-x