Life cycle assessment of PVC waste: a case study of PVC automotive floor mats from a circular economy perspective

Marcos Vinícius Maceno1 (Orcid: 0009-0000-7002-3767); João Victor Encide Salla1 (Orcid: 0000-0003-2693-7742); Tiago Barreto Rocha2 (Orcid: 0000-0002-6445-8315); Jane Maria Faulstich de Paiva1 (Orcid: 0000-0001-8924-6194); Virgínia Aparecida da Silva Moris1 (Orcid: 0000-0002-6374-1792)

1 Federal University of São Carlos (UFSCar) - campus Sorocaba, Sorocaba, Brazil

2 ACV Brasil

* Marcos Vinícius Maceno: mmaceno@estudante.ufscar.br; mmaceno1987@gmail.com

Hello, the supplementary materials are listed below

Thanks a lot!

Marcos

Table S1 - Electricity consumption of equipment for the production of automotive PVC floor mats

| Equipment / Process | Voltage | Power (KW) | Estimated time (seconds) | Energy consumption (KWh) |
| --- | --- | --- | --- | --- |
| Grinder | 220 V | 37.5 | 73.0 | 0.760 |
| Extruder | 220 V | 80.0 | 80.0 | 1.778 |
| Molding | 220 V | 2.25 | 80.0 | 0.050 |
| Deburring | Without consumption |  |  |  |
| Insertion (traceability code) | 220 V | 0.25 | 40.0 | 0.003 |
| Drilling | 220 V | 0.25 | 40.0 | 0.003 |
| Insertion (safety lock) | 220 V | 0.25 | 40.0 | 0.003 |
| Inspection | 220 V | 0.50 | 40.0 | 0.006 |
| Packaging | Without consumption |  |  |  |
| Shipping | Without consumption |  |  |  |

Source: Authors.

Table S2 LCA results for all scenarios

| Impact Category | Unit | Scenario |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i | ii | iii | iv | v | vi | vii | viii |
| Acidification: terrestrial (AT) | kg SO2-Eq | 0.031739 | 0.021311 | 0.005675 | 0.031739 | 0.034890 | 0.029644 | 0.031580 | 0.005516 |
| Climate change (CC) | kg CO2-Eq | 11.929502 | 8.026034 | 2.173031 | 11.929502 | 18.814262 | 17.331697 | 11.668432 | 1.911961 |
| Ecotoxicity: freshwater (EF) | kg 1,4-DCB-Eq | 0.703796 | 0.495609 | 0.183383 | 0.703796 | 0.809872 | 0.765316 | 0.592744 | 0.072330 |
| Ecotoxicity: marine (EM) | kg 1,4-DCB-Eq | 0.993048 | 0.697742 | 0.254876 | 0.993048 | 1.150217 | 1.082877 | 0.841515 | 0.103343 |
| Ecotoxicity: terrestrial (ET) | kg 1,4-DCB-Eq | 99.739646 | 70.567298 | 26.834465 | 99.739646 | 113.603676 | 101.069072 | 99.191554 | 26.286373 |
| Energy resources: non-renewable, fossil (ER) | kg oil-Eq | 4.566486 | 2.910749 | 0.427652 | 4.566486 | 4.797858 | 4.475505 | 4.547321 | 0.408488 |
| Eutrophication: freshwater (Eut. Fre.) | kg P-Eq | 0.003002 | 0.001927 | 0.000315 | 0.003002 | 0.003326 | 0.003139 | 0.002998 | 0.000311 |
| Eutrophication: marine (Eut. Mar.) | kg N-Eq | 0.002926 | 0.002739 | 0.002458 | 0.002926 | 0.001989 | 0.001913 | 0.001880 | 0.001413 |
| Human toxicity: carcinogenic (HT - Car.) | kg 1,4-DCB-Eq | 2.246685 | 1.485192 | 0.343124 | 2.246685 | 2.991266 | 2.837461 | 2.231676 | 0.328116 |
| Human toxicity: non-carcinogenic (HT - non-car.) | kg 1,4-DCB-Eq | 14.375736 | 10.074495 | 3.623705 | 14.375736 | 15.376934 | 14.554670 | 12.170854 | 1.418823 |
| Ionising radiation (IR) | kBq Co-60-Eq | 0.490728 | 0.335655 | 0.103373 | 0.490728 | 0.528882 | 0.423218 | 0.490015 | 0.102660 |
| Land use (LU) | m2*a crop-Eq | 0.851227 | 0.796338 | 0.714177 | 0.851227 | 0.876053 | 0.786034 | 0.841501 | 0.704451 |
| Material resources: metals/minerals (MR) | kg Cu-Eq | 0.366354 | 0.231948 | 0.030368 | 0.366354 | 0.437185 | 0.417174 | 0.365644 | 0.029658 |
| Ozone depletion (OD) | kg CFC-11-Eq | 0.000009 | 0.000007 | 0.000004 | 0.000009 | 0.000010 | 0.000007 | 0.000009 | 0.000004 |
| Particulate matter formation (PMF) | kg PM2.5-Eq | 0.015494 | 0.010310 | 0.002536 | 0.015494 | 0.016875 | 0.014642 | 0.015418 | 0.002460 |
| Photochemical oxidant formation: human health (POF-HH) | kg NOx-Eq | 0.023866 | 0.015990 | 0.004180 | 0.023866 | 0.026931 | 0.024333 | 0.023593 | 0.003908 |
| Photochemical oxidant formation: terrestrial ecos. (POF-T) | kg NOx-Eq | 0.025913 | 0.017310 | 0.004409 | 0.025913 | 0.029074 | 0.026361 | 0.025628 | 0.004124 |
| Water use (WU) | m3 | 0.137845 | 0.118564 | 0.089827 | 0.137845 | 0.343141 | 0.218824 | 0.137747 | 0.089729 |

Source: Authors.