Supplementary Table S7. PET/CT acquisition and reconstruction protocol by center.

| Center | Scanner | 18F-FDG dose and uptake time | Scan range | CT parameters | PET reconstruction |
| --- | --- | --- | --- | --- | --- |
| Jieyang People's Hospital | Philips Ingenuity TF PET/CT | 0.1 mCi/kg; uptake time 50-60 min | From cranial vertex to proximal femur | 120 kV; 142 mA; slice thickness 4 mm | TOF reconstruction |
| Shenzhen People's Hospital | GE Discovery 710 Clarity PET/CT | Routine weight-based 18F-FDG administration; uptake time approximately 50-60 min | Whole-body tumor PET/CT coverage for staging, from head/neck region to proximal femur or mid-thigh according to institutional protocol | Low-dose CT for attenuation correction and anatomical localization according to institutional protocol | TOF-capable clinical PET reconstruction on the GE Discovery 710 Clarity platform |
| The First Affiliated Hospital of Shantou University | Discovery MI PET/CT | 4.5-5.5 mCi adjusted by body weight; uptake time approximately 60 min | From head to mid-thigh | Low-dose CT: 120 kVp, 30-60 mAs, slice thickness 1-3 mm; diagnostic CT, when acquired, used automatic tube-current modulation and 0.625-mm slice thickness | VPHD/VPFX reconstruction; 6 iterations, 16 subsets, 192 x 192 matrix, no smoothing filter |

Center-specific protocols are summarized to document scanner and acquisition heterogeneity. All images entered the same downstream preprocessing and radiomic feature-extraction workflow.