Supporting information for

Synthesis of typical sulfonamide antibiotics with [14C]- and [13C]-labelling on phenyl ring for environmental studies

Xuan Wu1,2, Yao Yao1, Lianhong Wang1, Dashun Zhou2, Feifei Sun1*, Jianqiu Chen2, Philippe Francois-Xavier Corvini1,3, Rong Ji1*

1. State Key Laboratory of Pollution Control and Resource Reuse, School of the Environment, Nanjing University, Nanjing 210023, China.

2. School of Engineering, China Pharmaceutical University, Nanjing 211198, China.

3. Institute for Ecopreneurship, School of Life Sciences, University of Applied Sciences and Arts Northwestern Switzerland, Gründenstrasse 40, Muttenz CH-4132, Switzerland

Correspondence authors:

*Feifei Sun: Tel.: + 86 25 89680589, Fax: + 86 25 89680589. E-mail: sff@nju.edu.cn;

*Rong Ji: Tel.: + 86 25 89680581, Fax: + 86 25 89680581. E-mail: ji@nju.edu.cn;

SI.1. Flash column chromatography

Flash column chromatography for purification of 2a, 3a and 8a was performed on a medium pressure preparative chromatograph system (CHEETAH TMMP100; Agela, Tianjin, China) equipped with a UV detector and a fraction collector, using prepacked silica gel column (12 g, 40–60 μm; Agela, Tianjin, China) at a flow rate of 8 mL/min. UV detection was set at 230 nm and 254 nm. Fractions of 30 mL were collected.

# SI.2. Thin-layer chromatography

Thin-layer chromatography (TLC) was conducted on plates of silica gel 60 F254. Analytical TLC plates (0.25 mm, 3 cm × 10 cm, Huanghai, Shandong, China) were used for purity analysis using petroleum ether: ethyl acetate (1:4 / v: v), containing 0.2% CH3COOH as eluent. Purification of [14C]-labelled compounds 4a, 5a, 6a, and 7a was achieved by preparative TLC plates (1 mm, 20 × 20 cm; Huanghai, Shandong, China). Autoradiography of TLC plates was obtained on an imaging scanner (Typhoon Trio+; GE Healthcare, U.S.).

# SI.3. Liquid scintillation counting

Liquid scintillation counting (LSC) was performed with a Beckman LS6500 (LS6500; Beckman Counter, USA) scintillation counter and Gold Star scintillation cocktail (Meridian Biotechnologies Ltd, UK). Counts were corrected for background activity by using blank controls. Counting efficiency and color quenching were corrected using the external standard ratio method.

# SI.4. High performance liquid chromatography

High performance liquid chromatography (HPLC) for purity determination and identification of products was conducted on Agilent HPLC 1100 system (Agilent Technology, USA). Chromatographic separation was achieved by injecting the sample (10 μL) into an Eclipse XDB-C18 column (250 mm × 4.6 mm, 5 μm; Agilent) at 30 °C with a guard column (4.6 mm × 12.5 mm, 5 μm; Agilent). The flow-rate was set at 1 mL/min. The mobile phase for analyses of 5a and 10a consisted of H2O (A) containing 0.3% (v/v) CH3COOH and CH3OH (B). The elution gradient for 5a analysis started with 35% B for 15 min, linearly increased to 100% B within 5 min, and maintained at 100% B for 5 min, then linearly decrease to 35% B within 5 min, and finally remained at 35% B for 5 min. The elution gradient for 10a analysis started with 20% B for 15 min, linearly increased to 100% B within 5 min, and maintained at 100% B for 5 min, then linearly decrease to 20% B within 5 min, and finally remained at 20% B for 5 min. The mobile phase for analysis of 7a consisted of H2O (A) containing 0.3% (v/v) CH3COOH and CH3CN (C), running with a gradient starting with 26% C for 10 min, linear increase to 100% C within 10 min, and 100% C for 5 min, linear decrease to 26% C within 5 min, and finally 26% C for 5 min. The detection wavelength was 270 nm. The eluent fractions were collected every minute and determined for radioactivity by LSC. The mobile phase for purity determination of [13C]-intermediates and [13C]-SAs consisted of H2O (60%) and CH3CN (40%). The purity was calculated according to area normalization method at 270 nm.

# SI.5. Nuclear magnetic resonance

1H and 13C nuclear magnetic resonance (NMR) spectra were recorded on AVANCE III HD-500 spectrometer (Bruker, Germany). Samples were dissolved in CDCl3 or DMSO-d6 containing 1% tetramethylsilane as internal standard.

# SI.6. Liquid chromatography−mass spectrometry

Q-TOF-MS/MS analysis was performed using an Agilent 1260 series HPLC system (Agilent Technology, USA) coupled to a Q-TOF-MS (triple TOF 5600 system; AB SCIEX, USA). An XBridge C18 column (100 mm × 2.1 mm, 3.5 μm; Waters) was thermostatically controlled at 30 °C. The mobile phase for identification of synthesized unlabelled 5, 7, and 10 was a mixture of Milli-Q water (50%) containing 0.1% (v/v) HCOOH and methanol (50%) running isocratically at a flow rate of 0.2 mL/min. The full scan was operated in positive ESI ionization mode with a declustering potential of 80 V and a collision energy of 10 V. The collision energy of product scan was 25 V, and collision energy spread was 15 V. The full scan and product ion scan were operated with the mass range of m/z 80–800. The mobile phase for identification of synthesized unlabelled 3 was acetonitrile (100%). And the mobile phase for other intermediates was methanol (100%) running isocratically at a flow rate of 0.2 mL/min. The full scan was operated in negative ESI ionization mode with a declustering potential of −80 V and a collision energy of −10 V. The collision energy of product scan was −35 V, and the collision energy spread was 15 V. The full scan and product ion scan were operated with the mass range of m/z 80–600. The optimal parameters for Q-TOF were as follows: a nebulizer gas (N2) of 55 psi, a heater gas (N2) of 55 psi, a curtain gas of 35 psi, a turbo spray temperature of 550 °C, an ionspray voltage in ESI+ of 5.5 kV and an ionspray voltage in ESI− of −4.5 kV. The software PeakView (AB SCIEX, Foster City, CA, USA) was applied to mass spectrum analysis.

# Table S1 Purification methods of [l4C]-labelled intermediates and SAs

| Compound | Purification conditions |
| --- | --- |
| 2a | Flash column chromatography; Eluent: petroleum ether / ethyl acetate; Gradient elution program: volume fraction of ethyl acetate; 0~0%, 5 min; 20~20%, 5 min; 20~30%, 15 min; 30~30%, 5 min; 50~50%, 5 min; 100~100%, 20 min; Fraction collection: 23~32 min. |
| 3a | Flash column chromatography; Eluent: n-hexane / ethyl acetate; Gradient elution program: volume fraction of ethyl acetate; 0~0%, 5 min; 20~20%, 5 min; 20~30%, 10 min; 30~30%, 10 min; 40~40%, 10 min; 100~100%, 30 min; Fraction collection: 26~40 min. |
| 4a, 6a | Preparative TLC; Eluent: petroleum ether / ethyl acetate (1:4 / v: v) containing 0.2% CH3COOH. |
| 9a | Recrystallization in water |
| 5a, 7a | Preparative TLC; Eluent: petroleum ether / ethyl acetate (1:4 / v: v) containing 0.4% CH3COOH. |
| 10a | Recrystallization in methanol |
| 8a | Flash column chromatography; Eluent: petroleum ether / ethyl acetate; Gradient elution program: volume fraction of ethyl acetate; 0~0%, 10 min; 0~50%, 5 min; 50~50%, 10 min; 50~70%, 10 min; 70~70%, 10 min; 80~80%, 30 min; 80~100%, 10 min; 100~100%, 20 min; Fraction collection: 28~55 min. |

Table S2 The data of 1H-NMR, 13C-NMR, and LC-MS/MS of synthesized unlabelled intermediates and SAs

| Compound | MS fragments (m/z) | Chemical shifts (ppm)a |  |
| --- | --- | --- | --- |
|  |  | 1H NMR | 13C NMR |
| Unlabelled 2 | [C8H8NO]−, [M−H]−, calculated: 134.0611, found: 134.0617, MS/MS: 92.0516 [M−C2H3O]−. | 7.68 (1H, s, NHCO), 7.50 (2H, d, H-2, H-6), 7.30 (2H, t, H-3, H-5), 7.10 (1H, t, H-3), 2.16 (3H, s, H-8). | /b |
| Unlabelled 3 | [C8H7ClNO3S]−, [M−H]−, calculated: 231.9841, found: 231.9825, MS/MS: 168.0194 [M−SO2H], 126.0107 [M−C2H3O3S]. | 10.09 (1H, s, NHCO), 8.97 (4H, s, H-2, H-3, H-5, H-6), 2.03 (3H, s, H-8). | / |
| Unlabelled 4 | [C12H12N3O4S], [M−H], calculated: 294.0554, found: 294.0580, MS/MS: 198.0245 [M−C4H5N2O], 134.0618 [M−C4H5N2O3S], 92.0514 [M−C6H7N2O4S]. | 11.32 (s, 1H, SO2NH), 10.36 (1H, s, NHCO), 7.76 (4H, q, H-2, H-3, H-5, H-6), 6.12 (1H, s, H-8), 2.29 (3H, s, H-10), 2.07 (3H, s, H-12). | / |
| Unlabelled 5 | [C10H12N3O3S]+, [M+H]+, calculated: 254.0594, found: 254.0595, MS/MS: 156.0108 [M−C4H5N2O]+, 108.0448 [M−C4H5N2O2S]+, 92.0538 [M−C4H5N2O3S]+. | 10.92 (1H, s, NH), 7.46 (2H, d, H-3, H-5), 6.57 (2H, d, H-2, H-6), 6.08 (3H, d, H-8, NH2), 2.28 (3H, s, H-10). | 170.03 (C-9), 158.09 (C-7), 153.39 (C-1), 128.97 (C-3, C-5), 124.27 (C-4), 112.74 (C-2, C-6), 95.41 (C-8), 12.12 (C-10). |
| Unlabelled 6 | [C13H14N4O4S], [M−H], calculated: 321.0663, found: 321.0696, MS/MS: 134.0618 [M−C5H6N3O3S], 122.0366 [M−C8H10NO3S], 92.0507 [M−C7H8N3O4S]. | 11.97 (1H, s, SO2NH), 10.34 (1H, s, NHCO), 8.40 (1H, d, H-10), 7.85 (2H, d, H-3, H-5), 7.74 (2H, d, H-2, H-6), 6.32 (1H, d, H-8), 3.83 (3H, s, H-11), 2.06 (3H, d, H-13). | / |
| Unlabelled 7 | [C11H13N4O3S]+, [M+H]+, calculated: 281.0703, found: 281.0703, MS/MS: 215.0920 [M−HSO2]+, 156.0102 [M−C5H6N3O]+, 126.0662 [M−C6H4NO2S]+, 108.0442 [M−C5H6N3O2S]+. | 11.36 (1H, s, NH), 8.39 (1H, d, H-10), 7.54 (2H, d, H-3, H-5), 6.58 (2H, d, H-2, H-6), 6.30 (1H, d, H-8), 6.10 (2H, s, NH2), 3.82 (3H, s, H-11). | 169.85 (C-9), 158.83 (C-7), 157.52 (C-10), 153.41 (C-1), 129.29 (C-3, C-5), 124.16 (C-4), 112.62 (C-2, C-6), 90.69 (C-8), 53.94 (C-11). |
| Unlabelled 8 | [C8H10N2O3S], [M−H], calculated: 213.0339, found: 231.0345, MS/MS: 171.0236 [M−C2H3O], 134.0611 [M−H2NO2S], 92.0515 [M−C2H4NO3S]. | 10.27 (1H, s, NH), 7.81–7.64 (4H, m, H-2, H-3, H-5, H-6), 7.24 (2H, s, NH2), 2.08 (3H, d, H-8). | / |
| Unlabelled 9 | [C12H12N4O3S], [M−H], calculated: 291.0557, found: 291.0593, MS/MS: 227.0960 [M−C3HN2], 184.0765 [M−C5H4N2O], 134.0620 [M−C4H4N3O2S], 92.0262 [M−C6H6N3O3S]. | 11.68 (1H, s, SO2NH), 10.32 (1H, s, NHCO), 8.49 (2H, d, H-8, H-10), 7.91 (2H, d, H-3, H-5), 7.73 (2H, d, H-2, H-6), 7.03 (1H, t, H-9), 2.06 (3H, d, H-13). | / |
| Unlabelled 10 | [C10H11N4O2S]+, [M+H]+, calculated: 251.0597, found: 251.0584, MS/MS: 156.0112 [M−C4H4N3]+, 108.0442 [M−C4H4N3OS]+, 92.0502 [M−C4H4N3O2S]+. | 11.27 (1H, s, NH), 8.48 (2H, d, H-8, H-10), 7.69–7.55 (2H, H-3, H-5), 7.00 (1H, t, H-9), 6.63-6.47 (2H, m, H-2, H-6), 6.01 (2H, s, NH2). | 158.37 (C-7), 157.31 (C-8, C-10), 153.17 (C-1), 130.02 (C-3, C-5), 124.97 (C-4), 115.64 (C-9), 112.35 (C-2, C-6). |

a Positions of the numbered C-atoms are given in the corresponding structures.

b “/” indicates that 13C NMR analysis was not performed.