A Comparative Life Cycle Assessment of Single-Family House and Multi-Storey Apartment Building in Turkey

Sedat Gulcimen1, Tolgahan Varışlı1, Mhd. Ghaith Khidrah, Nigmet Uzal1*

1Department of Civil Engineering, Abdullah Gul University, Kayseri, 38080, Turkey

Supplementary Materials

Table S1. The inventory table for pre-operation phase

| Items | Unit | Amount for Villa | Amount for Apartmant |
| --- | --- | --- | --- |
| Concrete (C25) | m3 | 180 | 3200 |
| Cement | ton | 2 | 140 |
| Lime | ton | 1.80 | 115 |
| Sand | ton | 10.5 | 400 |
| Steel | ton | 11 | 245 |
| Roof Tile | ton | 6 | 31.50 |
| Wood | ton | 7 | 40 |
| Gypsum | ton | 7.5 | 95 |
| Glass | ton | 1 | 25 |
| Brick | ton | 31 | 65 |
| Polystyrene | ton | 0.7 | 5.3 |
| Paint | kg | 340 | 6500 |
| Parquet | m2 | 120 | 3000 |
| Ceramics | ton | 1.3 | 40 |
| Electric Cables | m | 600 | 50000 |
| Water Pipes (polyethylene) | m | 210 | 5000 |
| Pipes for Natural gas (polyethylene) | m | 80 | 300 |
| Marble (for kitchen and balcony) | ton | 0.5 | 50 |
| Rockwool | ton | 0.18 | 55 |
| Window Frame (PVC) | m2 | 10 | 170 |
| Inner Doors | m2 | 20 | 455 |
| Outer Door | m2 | 2 | 56 |
| Electricity | kWh | 420 | 14400 |
| Fuel | kg | 360 | 450 |
| Water | m3 | 210 | 12500 |
| Solid Waste | ton | 6 | 40 |
| Concrete mixer | tkm | 43200 | 460800 |
| Truck | tkm | 8500 | 90000 |

Table S2. The inventory table for operation phase

| Items | Unit | Amount for Villa | Amount for Apartmant |
| --- | --- | --- | --- |
| Electricity | kwh | 918000 | 15422400 |
| Water | m3 | 20440 | 343392 |
| Natural Gas | m3 | 45250 | 1267000 |
| Solid Waste | ton | 105.85 | 1778.28 |
| Wastewater | m3 | 17155 | 288204 |