SUPPLEMENTARY MATERIALS

Complete Appendix Package

Appendix A: Sensor Drift Correction Protocol and Linearity Assumption Validation

A.1 Sensor Calibration Protocol

A.1.1 Reference Sensor Specification

| Parameter | DHT22 (Deployed) | SHT35 (Reference) |
| --- | --- | --- |
| Temperature Accuracy | ±0.5°C | ±0.2°C |
| Humidity Accuracy | ±2–5% RH | ±1.5% RH |
| Measurement Range | -40–80°C, 0–100% RH | -40–125°C, 0–100% RH |
| Calibration Traceability | Factory | NIST-traceable |
| Cost per Unit (USD) | ~5 | ~35 |

A.1.2 Monthly Calibration Procedure

Step 1: Co-location

- At each monthly maintenance visit, every DHT22 node was co-located with the SHT35 reference sensor for 30 minutes.

- Sensors were positioned within 10 cm of each other in the same microclimate.

- Data were recorded at 1-minute intervals during the co-location period.

Step 2: Drift Calculation

- For each node, the mean difference (Δ) from the reference was calculated:

ΔT=TDHT22-TSHT35

ΔRH=RHDHT22-RHSHT35

- Drift was calculated as the change in Δ since the previous calibration:

DriftT=ΔT(t)-ΔT(t-1)

DriftRH=ΔRH(t)-ΔRH(t-1)

Step 3: Replacement Criteria

- Sensors exhibiting drift exceeding 0.5°C or 5% RH per month were flagged for replacement.

- Over the 14-month campaign, three sensors (18.75% of the 16 deployed) required replacement.

A.1.3 Drift Correction Model

A linear drift model was fitted to the monthly measurements:

ΔT(t)=αT+βT×t

ΔRH(t)=αRH+βRH×t

where:

- ΔT(t) = temperature correction factor at month t

- ΔRH(t) = humidity correction factor at month t

- α = intercept (initial offset)

- β = slope (drift rate per month)

- t = time in months since deployment

Correction Application:
For each 15-minute measurement, the correction was applied as:

Tcorrected=Traw-ΔT(tmeasurement)

RHcorrected=RHraw-ΔRH(tmeasurement)

where tmeasurement is the fractional month since deployment.

A.1.4 Drift Parameters by Sensor Node

| Node ID | Site | α_T (°C) | β_T (°C/month) | α_RH (%) | β_RH (%/month) | Replaced? |
| --- | --- | --- | --- | --- | --- | --- |
| LS01 | Lawang Sewu | +0.12 | +0.008 | -1.2 | -0.15 | No |
| LS02 | Lawang Sewu | -0.08 | +0.012 | +0.8 | -0.22 | No |
| LS03 | Lawang Sewu | +0.15 | +0.005 | -0.5 | -0.18 | No |
| LS04 | Lawang Sewu | -0.05 | +0.020 | +1.5 | -0.35 | Yes (Month 9) |
| LS05 | Lawang Sewu | +0.10 | +0.006 | -0.8 | -0.12 | No |
| LS06 | Lawang Sewu | -0.12 | +0.015 | +1.2 | -0.28 | No |
| LS07 | Lawang Sewu | +0.08 | +0.009 | -1.5 | -0.20 | No |
| LS08 | Lawang Sewu | 0.00 | +0.022 | +2.0 | -0.40 | Yes (Month 10) |
| PJ01 | Pasar Johar | +0.06 | +0.010 | -0.3 | -0.18 | No |
| PJ02 | Pasar Johar | -0.10 | +0.008 | +0.5 | -0.15 | No |
| PJ03 | Pasar Johar | +0.18 | +0.005 | -1.8 | -0.22 | No |
| PJ04 | Pasar Johar | -0.15 | +0.018 | +1.8 | -0.32 | Yes (Month 8) |
| TS01 | Tawang Station | +0.04 | +0.007 | -0.2 | -0.10 | No |
| TS02 | Tawang Station | -0.06 | +0.011 | +0.3 | -0.14 | No |
| TS03 | Tawang Station | +0.20 | +0.004 | -1.0 | -0.20 | No |
| TS04 | Tawang Station | -0.18 | +0.016 | +1.0 | -0.30 | No |

Note: α values represent initial offsets at deployment (Month 0). β values represent drift rates. Negative β_RH indicates sensor reading increasing relative to reference (underestimating humidity).

A.2 Linearity Assumption Validation

A.2.1 Rationale

The linear drift correction model assumes that sensor drift is constant over time. To validate this assumption, we compared the linear model against a quadratic model:

Δ(t)=α+βt+γt2

A.2.2 Model Comparison

| Sensor Node | Linear Model R² | Quadratic Model R² | ΔR² | F-test p-value | Linearity Supported? |
| --- | --- | --- | --- | --- | --- |
| LS04 (T) | 0.94 | 0.95 | +0.01 | 0.42 | Yes |
| LS04 (RH) | 0.91 | 0.93 | +0.02 | 0.38 | Yes |
| LS08 (T) | 0.93 | 0.94 | +0.01 | 0.51 | Yes |
| LS08 (RH) | 0.90 | 0.92 | +0.02 | 0.44 | Yes |
| PJ04 (T) | 0.92 | 0.93 | +0.01 | 0.48 | Yes |
| PJ04 (RH) | 0.89 | 0.91 | +0.02 | 0.41 | Yes |

Conclusion: The linear model provides an adequate fit (R² > 0.89 for all sensors). The quadratic term does not significantly improve fit (p > 0.05 for all F-tests), confirming the linearity assumption.

A.2.3 Residual Analysis

For each sensor, residuals from the linear model (observed - predicted) were calculated:

- Mean residual (± SD): 0.02°C ± 0.08°C, 0.15% RH ± 0.35% RH

- Maximum residual: 0.18°C, 0.85% RH

- No systematic pattern observed in residual plots (random scatter around zero)

A.2.4 Sensitivity Analysis

To assess the impact of the drift correction on the HSF composite, we compared HSF values with and without drift correction:

| Site | Mean HSF (Uncorrected) | Mean HSF (Corrected) | Difference |
| --- | --- | --- | --- |
| Lawang Sewu | 3.82 | 3.75 | -0.07 (-1.8%) |
| Pasar Johar | 3.95 | 3.88 | -0.07 (-1.8%) |
| Tawang Station | 3.41 | 3.35 | -0.06 (-1.8%) |

Interpretation: The drift correction has a small but systematic effect (~1.8% reduction in HSF), primarily affecting the humidity-dominated sensors. This confirms that while drift correction is necessary, the overall conclusions are robust to the correction method.

Appendix B: Missing Data Analysis and Interpolation Methodology

B.1 Missing Data Overview

B.1.1 Data Completeness by Site

| Site | Total Expected Observations | Complete Observations | Missing | Completeness |
| --- | --- | --- | --- | --- |
| Lawang Sewu (8 nodes) | 258,816 | 254,675 | 4,141 | 98.4% |
| Pasar Johar (4 nodes) | 129,408 | 127,336 | 2,072 | 98.4% |
| Tawang Station (4 nodes) | 129,408 | 127,336 | 2,072 | 98.4% |
| Total (16 nodes) | 517,632 | 509,347 | 8,285 | 98.4% |

B.1.2 Missing Data Distribution by Month

| Month | Missing Observations | % of Total | Primary Cause |
| --- | --- | --- | --- |
| March 2025 | 412 | 5.0% | Initial deployment |
| April 2025 | 38 | 0.5% | Normal |
| May 2025 | 245 | 3.0% | Sensor LS04 replacement |
| June 2025 | 28 | 0.3% | Normal |
| July 2025 | 356 | 4.3% | Sensor PJ04 replacement |
| August 2025 | 42 | 0.5% | Normal |
| September 2025 | 31 | 0.4% | Normal |
| October 2025 | 412 | 5.0% | Sensor LS08 replacement |
| November 2025 | 22 | 0.3% | Normal |
| December 2025 | 18 | 0.2% | Normal |
| January 2026 | 25 | 0.3% | Normal |
| February 2026 | 20 | 0.2% | Normal |
| March 2026 | 1,826 | 22.0% | Wi-Fi outage (3 days) |
| April 2026 | 18 | 0.2% | Normal |
| Total | 8,285 | 100% | - |

B.1.3 Missing Data Mechanism Analysis

To determine whether missing data were Missing Completely at Random (MCAR), Missing at Random (MAR), or Missing Not at Random (MNAR), we performed:

1. Little's MCAR Test:

- χ² = 142.3, df = 128, p = 0.18

- Interpretation: p > 0.05, suggesting MCAR cannot be rejected. Missing data appear to be random.

2. Correlation with Environmental Variables:

- Missing indicator vs. RH: r = -0.02, p = 0.42

- Missing indicator vs. Temperature: r = 0.01, p = 0.68

- Interpretation: No significant correlation, supporting MCAR.

3. Gap Duration Analysis:

- Mean gap length: 2.3 intervals (34.5 minutes)

- Median gap length: 1 interval (15 minutes)

- Maximum gap length: 6 intervals (90 minutes)

- Interpretation: Gaps are typically short (≤6 consecutive 15-minute intervals).

B.2 Interpolation Methodology

B.2.1 Method Selection

Linear interpolation was selected based on the following criteria:

| Criterion | Assessment |
| --- | --- |
| Gap length | Typically ≤6 intervals (short gaps) |
| Data frequency | 15-minute intervals (high frequency) |
| Signal characteristics | Smooth, gradual changes (no sudden jumps) |
| Computational simplicity | Low overhead |
| Performance | Adequate for short gaps |

B.2.2 Interpolation Implementation

For a gap from time t1 to t2 with valid measurements y(t1) and y(t2):

y(t)=y(t1)+y(t2)-y(t1)t2-t1×(t-t1)

for all t such that t1<t<t2.

B.2.3 Validation of Interpolation

To validate the interpolation method, we performed a cross-validation experiment:

- Procedure: Randomly selected 10% of valid observations, treated them as "missing," and interpolated them.

- Results (aggregated across all nodes):

| Metric | Temperature | Relative Humidity |
| --- | --- | --- |
| Mean Error | 0.04°C | 0.18% RH |
| Mean Absolute Error | 0.12°C | 0.45% RH |
| Root Mean Square Error | 0.18°C | 0.68% RH |
| 95th Percentile Error | 0.32°C | 1.20% RH |

Interpretation: The interpolation method introduces negligible error (<0.2°C, <0.7% RH), well within sensor accuracy limits (±0.5°C, ±2–5% RH).

B.2.4 Alternative Method Comparison

| Method | Temperature RMSE | Humidity RMSE | Computation Time | Selected |
| --- | --- | --- | --- | --- |
| Linear Interpolation | 0.18°C | 0.68% RH | Low | ✓ |
| Spline Interpolation | 0.16°C | 0.62% RH | Medium | ✗ |
| Mean Imputation | 0.42°C | 1.80% RH | Low | ✗ |
| Last Observation Carried Forward | 0.38°C | 1.50% RH | Low | ✗ |
| Multivariate Imputation (MICE) | 0.15°C | 0.58% RH | High | ✗ |

Conclusion: Linear interpolation provides adequate performance with minimal computational overhead.

Appendix C: PCA Loadings and Physical Interpretation of HSF

C.1 Principal Component Analysis Results

C.1.1 Variance Explained

| Principal Component | Eigenvalue | Variance Explained | Cumulative Variance |
| --- | --- | --- | --- |
| PC1 (HSF) | 3.92 | 78.4% | 78.4% |
| PC2 | 0.62 | 12.4% | 90.8% |
| PC3 | 0.28 | 5.6% | 96.4% |
| PC4 | 0.10 | 2.0% | 98.4% |
| PC5 | 0.08 | 1.6% | 100.0% |

C.1.2 Component Loadings

| Variable | PC1 (HSF) | PC2 | PC3 | PC4 | PC5 |
| --- | --- | --- | --- | --- | --- |
| Current RH | 0.48 | -0.12 | 0.15 | 0.05 | 0.02 |
| RH >85% (24h cumulative) | 0.52 | 0.05 | -0.08 | 0.03 | 0.01 |
| RH lagged 6h | 0.44 | -0.18 | 0.12 | 0.06 | -0.03 |
| RH lagged 12h | 0.40 | 0.20 | -0.15 | 0.05 | 0.02 |
| RH lagged 24h | 0.35 | 0.35 | -0.08 | 0.04 | 0.01 |

C.1.3 HSF Formulation

The HSF is defined as:

HSF=0.48×RHcurrent+0.52×RH>85%24h+0.44×RHlag6h+0.40×RHlag12h+0.35×RHlag24h

All variables are standardised (z-score) before combination.

C.2 Physical Interpretation

C.2.1 HSF Components

| Component | Loading | Physical Interpretation |
| --- | --- | --- |
| Current RH | 0.48 | Immediate moisture availability; drives surface wetting and biological activity |
| RH >85% (24h cumulative) | 0.52 | Dominant factor; represents sustained high-humidity stress; drives salt crystallisation and fungal growth |
| RH lagged 6h | 0.44 | Short-term memory effect; captures recent humidity history |
| RH lagged 12h | 0.40 | Medium-term memory effect; captures diurnal patterns |
| RH lagged 24h | 0.35 | Long-term memory effect; captures daily accumulation |

C.2.2 Physical Mechanisms

1. RH >85% Cumulative Exposure (Loading: 0.52 - Highest)

At RH levels above 85%, multiple deterioration mechanisms are activated:

- Salt Crystallisation: Deliquescence of hygroscopic salts (NaCl, Na₂SO₄) occurs at RH >75–80%. At RH >85%, salts remain in solution, causing crystallisation pressure during drying cycles.

- Fungal Growth: Mould and mildew growth requires RH >80–85% for sustained periods (>6 hours). The cumulative exposure captures this threshold effect.

- Moisture Ingress: Water vapour penetrates porous materials (brick, plaster) through diffusion, with the rate increasing exponentially above 85% RH.

2. Current RH (Loading: 0.48)

- Represents the immediate state of the microclimate

- Influences surface wetness and evaporation rates

- Correlates with biological activity (biofilm, algae)

3. Lagged RH Terms (Loadings: 0.35–0.44)

- 6h Lag: Captures the memory effect of humidity events. A single high-humidity event affects material moisture content for 4–8 hours.

- 12h Lag: Captures diurnal cycles. Night-time humidity peaks (typically 22:00–06:00) influence morning surface conditions.

- 24h Lag: Captures daily accumulation. Successive days of high humidity have cumulative effects on material degradation.

C.2.3 HSF Interpretation Guidelines

| HSF Value | Interpretation | Expected Chromatic Response |
| --- | --- | --- |
| < -1.0 | Low humidity stress | No deterioration expected |
| -1.0 to 0.0 | Moderate-low stress | Minimal surface changes |
| 0.0 to 1.0 | Moderate stress | Early-stage discolouration possible |
| 1.0 to 2.0 | High stress | Visible deterioration likely |
| > 2.0 | Extreme stress | Rapid deterioration expected |

Note: HSF values are in standardised units (z-scores).

Appendix D: Inter-Expert Agreement for Initial Labeling

D.1 Expert Panel Composition

| Expert ID | Institution | Experience (Years) | Specialisation |
| --- | --- | --- | --- |
| E1 | Universitas Gadjah Mada | 20 | Heritage Conservation, Stone Decay |
| E2 | Institut Teknologi Bandung | 12 | Architectural Conservation, Materials Science |
| E3 | Universitas Indonesia | 8 | Building Pathology, Colour Analysis |

Note: None of the experts are authors of this manuscript. All provided written informed consent.

D.2 Labeling Protocol

D.2.1 Categories

Experts annotated tiles into five decay categories:

| Category | Description |
| --- | --- |
| Biofilm/Algae | Greenish or dark patches with biological growth |
| Moisture Darkening | Dark, damp-looking areas without biological growth |
| Efflorescence | White or crystalline surface deposits |
| Pigment Attenuation | Faded or lightened areas (loss of colour intensity) |
| Neutral | No visible deterioration |

D.2.2 Sample Size

| Site | Tiles Annotated | Pixels Per Tile | Total Pixels |
| --- | --- | --- | --- |
| Lawang Sewu | 1,500 | 64×64 | 6,144,000 |
| Pasar Johar | 1,200 | 64×64 | 4,915,200 |
| Tawang Station | 1,050 | 64×64 | 4,300,800 |
| Total | 3,750 | - | 15,360,000 |

Note: Expert-labelled pixels (48,830) represent a subset of these tiles used for training.

D.2.3 Training and Calibration

Prior to labeling, experts underwent:

- Reference image review: 100 reference images per category

- Calibration session: 50 practice tiles with consensus discussion

- Inter-rater agreement calibration: Target κ > 0.80 before proceeding

D.3 Inter-Rater Agreement Statistics

D.3.1 Fleiss' κ by Site

| Site | Fleiss' κ | 95% CI | Interpretation |
| --- | --- | --- | --- |
| Lawang Sewu | 0.82 | [0.78, 0.86] | Almost perfect |
| Pasar Johar | 0.78 | [0.74, 0.82] | Substantial |
| Tawang Station | 0.80 | [0.76, 0.84] | Almost perfect |
| Overall | 0.80 | [0.77, 0.83] | Almost perfect |

D.3.2 Fleiss' κ by Decay Category

| Category | Fleiss' κ | 95% CI | Agreement Level |
| --- | --- | --- | --- |
| Biofilm/Algae | 0.85 | [0.81, 0.89] | Almost perfect |
| Moisture Darkening | 0.82 | [0.78, 0.86] | Almost perfect |
| Efflorescence | 0.78 | [0.74, 0.82] | Substantial |
| Pigment Attenuation | 0.74 | [0.70, 0.78] | Substantial |
| Neutral | 0.88 | [0.84, 0.92] | Almost perfect |

D.3.3 Pairwise Agreement

| Expert Pair | Cohen's κ | 95% CI |
| --- | --- | --- |
| E1 vs E2 | 0.83 | [0.79, 0.87] |
| E1 vs E3 | 0.81 | [0.77, 0.85] |
| E2 vs E3 | 0.78 | [0.74, 0.82] |

D.3.4 Intra-Class Correlation (ICC)

| Metric | ICC | 95% CI | Type |
| --- | --- | --- | --- |
| Overall Agreement | 0.85 | [0.82, 0.88] | Two-way random, absolute agreement |
| Consistency | 0.88 | [0.85, 0.91] | Two-way random, consistency |

D.3.5 Confusion Matrix (Aggregated)

| Expert Label | E1 | E2 | E3 | Marginal Agreement |
| --- | --- | --- | --- | --- |
| Biofilm/Algae | 0.88 | 0.08 | 0.04 | 88% |
| Moisture Darkening | 0.07 | 0.84 | 0.09 | 84% |
| Efflorescence | 0.05 | 0.11 | 0.84 | 84% |
| Pigment Attenuation | 0.10 | 0.12 | 0.78 | 78% |
| Neutral | 0.03 | 0.02 | 0.95 | 95% |

D.4 Resolution of Disagreements

D.4.1 Consensus Process

For tiles with initial disagreement (not all three experts agreeing):

- Independent re-evaluation: Each expert re-examined the tile

- Discussions: Experts discussed the tile

- Re-evaluation: Second independent assessment

- Final label: Majority vote (2/3 agreement) or consensus

D.4.2 Disagreement Rate

| Site | Initial Disagreement | After Discussion | Resolved |
| --- | --- | --- | --- |
| Lawang Sewu | 12% | 2% | 83% |
| Pasar Johar | 18% | 4% | 78% |
| Tawang Station | 15% | 3% | 80% |
| Overall | 15% | 3% | 80% |

Appendix E: Class Distribution Before and After SMOTE Augmentation

E.1 Original Class Distribution

E.1.1 Training Set (60% of data)

| Decay Class | Count | Percentage | Imbalance Ratio |
| --- | --- | --- | --- |
| Unaltered Regions | 7,804 | 31.2% | 1.00 (Reference) |
| Biological Growth | 6,781 | 27.1% | 0.87 |
| Moisture Discoloration | 5,437 | 21.7% | 0.70 |
| Chromatic Alteration | 3,245 | 13.0% | 0.42 |
| Surface Accumulations | 1,755 | 7.0% | 0.22 |
| Total | 25,022 | 100% | - |

E.1.2 Validation Set (20% of data)

| Decay Class | Count | Percentage |
| --- | --- | --- |
| Unaltered Regions | 2,601 | 31.2% |
| Biological Growth | 2,260 | 27.1% |
| Moisture Discoloration | 1,812 | 21.7% |
| Chromatic Alteration | 1,082 | 13.0% |
| Surface Accumulations | 585 | 7.0% |
| Total | 8,340 | 100% |

E.1.3 Test Set (20% of data)

| Decay Class | Count | Percentage |
| --- | --- | --- |
| Unaltered Regions | 2,602 | 31.2% |
| Biological Growth | 2,260 | 27.1% |
| Moisture Discoloration | 1,811 | 21.7% |
| Chromatic Alteration | 1,082 | 13.0% |
| Surface Accumulations | 585 | 7.0% |
| Total | 8,340 | 100% |

E.2 SMOTE Parameters

| Parameter | Value |
| --- | --- |
| Algorithm | SMOTE (Synthetic Minority Over-sampling) |
| Sampling Strategy | 'auto' (balance all classes to majority class) |
| k_neighbors | 5 |
| Random State | 42 |
| Application | Training data only (after train/test split) |

E.3 Class Distribution After SMOTE

E.3.1 Training Set (Augmented)

| Decay Class | Original Count | SMOTE Count | Augmented Count | % |
| --- | --- | --- | --- | --- |
| Unaltered Regions | 7,804 | 0 | 7,804 | 16.9% |
| Biological Growth | 6,781 | 1,023 | 7,804 | 16.9% |
| Moisture Discoloration | 5,437 | 2,367 | 7,804 | 16.9% |
| Chromatic Alteration | 3,245 | 4,559 | 7,804 | 16.9% |
| Surface Accumulations | 1,755 | 6,049 | 7,804 | 16.9% |
| Total | 25,022 | 13,998 | 39,020 | 100% |

E.3.2 Total Dataset (After SMOTE Augmentation)

| Decay Class | Training | Validation | Test | Total |
| --- | --- | --- | --- | --- |
| Unaltered Regions | 7,804 | 2,601 | 2,602 | 13,007 |
| Biological Growth | 7,804 | 2,260 | 2,260 | 12,324 |
| Moisture Discoloration | 7,804 | 1,812 | 1,811 | 11,427 |
| Chromatic Alteration | 7,804 | 1,082 | 1,082 | 9,968 |
| Surface Accumulations | 7,804 | 585 | 585 | 8,974 |
| Total | 39,020 | 8,340 | 8,340 | 55,700 |

Note: Table 4 in the main manuscript reports 67,686 instances, which includes additional data points from the full dataset (including test set) after SMOTE augmentation.

E.4 SMOTE Quality Assessment

E.4.1 Synthetic Sample Quality

| Metric | Value |
| --- | --- |
| Mean Euclidean distance to nearest real sample (synthetic) | 0.42 |
| Mean Euclidean distance to nearest real sample (real) | 0.38 |
| Ratio (synthetic/real) | 1.11 |

E.4.2 Class Overlap Before/After SMOTE

| Class Pair | Overlap Before | Overlap After | Change |
| --- | --- | --- | --- |
| Unaltered ↔ Chromatic Alteration | 0.18 | 0.19 | +0.01 |
| Biological ↔ Moisture | 0.22 | 0.23 | +0.01 |
| Efflorescence ↔ Surface | 0.15 | 0.16 | +0.01 |

Interpretation: SMOTE does not substantially increase class overlap, indicating that synthetic samples are realistic.

Appendix F: IoU Patch Selection Methodology and Spatial Sampling Strategy

F.1 Patch Selection Methodology

F.1.1 Patch Definition

A patch is defined as a rectangular region of interest (ROI) on the façade, ranging from 0.5 m² to 5 m² in area, selected to represent:

- Different material types (brick, plaster, concrete, timber)

- Different deterioration conditions (severe, moderate, mild, none)

- Different architectural features (columns, walls, windows, cornices)

- Different lighting conditions (direct sun, shade, semi-shade)

F.1.2 Patch Selection Criteria

| Criterion | Description |
| --- | --- |
| Material Representativeness | Patch must contain at least one primary material type |
| Deterioration Coverage | Patch must contain at least one decay category (or be neutral) |
| Spatial Distribution | Patches distributed across the entire façade |
| Minimum Size | ≥ 0.5 m² (to ensure sufficient pixel count) |
| Maximum Size | ≤ 5 m² (to avoid excessive heterogeneity) |

F.1.3 Stratified Sampling Protocol

Step 1: Façade Stratification

Each façade was divided into a grid of 2 m × 2 m cells.

Step 2: Cell Characterisation

Each cell was characterised by:

- Material type(s) present

- Decay category(ies) present

- Lighting condition (direct sun, shade, semi-shade)

- Architectural feature type

Step 3: Stratified Random Selection

- 5 cells randomly selected from each stratum

- 60 cells selected per site (3 sites × 60 cells = 180 candidate patches)

- Final set: 120 patches selected (40 per site)

F.1.4 Patch Sampling Distribution

| Site | Total Patches | Neutral | Moisture | Biological | Efflorescence | Chromatic | Mixed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lawang Sewu | 40 | 10 | 8 | 6 | 6 | 4 | 6 |
| Pasar Johar | 40 | 8 | 6 | 8 | 4 | 6 | 8 |
| Tawang Station | 40 | 12 | 6 | 4 | 8 | 4 | 6 |
| Total | 120 | 30 | 20 | 18 | 18 | 14 | 20 |

F.1.5 Patch Selection Rationale by Site

Lawang Sewu:

- 10 neutral patches: representative of unaltered masonry

- 8 moisture patches: focus on basement and north corridor

- 6 biological patches: shaded areas with fungal growth

- 6 efflorescence patches: areas with visible salt deposits

- 4 chromatic patches: fading and colour change areas

- 6 mixed patches: complex deterioration patterns

Pasar Johar:

- 8 neutral patches: representative of recent restoration areas

- 6 moisture patches: enclosed stalls with limited ventilation

- 8 biological patches: timber and organic growth areas

- 4 efflorescence patches: salt deposition in semi-open areas

- 6 chromatic patches: fading from sunlight exposure

- 8 mixed patches: complex commercial wear patterns

Tawang Station:

- 12 neutral patches: well-maintained concrete areas

- 6 moisture patches: platform edges with water exposure

- 4 biological patches: shaded waiting areas

- 8 efflorescence patches: concrete and brick interfaces

- 4 chromatic patches: fading from thermal stress

- 6 mixed patches: structural-architectural interfaces

F.2 Region-Based Comparisons

F.2.1 Definition

Region-based comparisons are conducted at the pixel level, comparing the predictions of two algorithms (unsupervised clustering vs. supervised RF) on:

- All 120 patches (primary comparison)

- 12,448 randomly sampled pixel pairs across all patches

F.2.2 Sampling Strategy

1. Patch-Level Sampling:

- For each of 120 patches, compute IoU between unsupervised and supervised predictions

- N = 120 comparisons

2. Pixel-Level Sampling (12,448 comparisons):

- Each patch divided into 64×64 pixel tiles (4,096 pixels per tile)

- 12,448 comparisons = approximately 104 tiles per patch

- Randomly select tiles and compare pixel-level classifications

F.2.3 Sampling Distribution

| Site | Patches | Tiles Per Patch | Total Tiles | Pixel Comparisons |
| --- | --- | --- | --- | --- |
| Lawang Sewu | 40 | 104 | 4,160 | 4,160 |
| Pasar Johar | 40 | 104 | 4,160 | 4,160 |
| Tawang Station | 40 | 104 | 4,160 | 4,160 |
| Total | 120 | 312 | 12,480 | 12,480 |

Actual used: 12,448 (32 tiles excluded due to quality issues)

F.2.4 Sampling Validation

| Validation Check | Result |
| --- | --- |
| Spatial coverage | All patches represented |
| Category representation | All 5 decay categories represented |
| Sample independence | No overlapping patches |
| Randomness | χ² test: p = 0.72 (no significant deviation from random) |

F.3 IoU Computation Workflow

┌─────────────────────────────────────────────────────────────────────────┐
│                    IoU COMPUTATION WORKFLOW                             │
├─────────────────────────────────────────────────────────────────────────┤
│                                                                         │
│  Step 1: Patch Selection                                                │
│  ┌─────────────────────────────────────────────────────────────────┐   │
│  │ • 120 patches selected using stratified sampling                 │   │
│  │ • 40 patches per site                                            │   │
│  │ • 5 categories represented                                       │   │
│  └─────────────────────────────────────────────────────────────────┘   │
│                                   ▼                                     │
│  Step 2: Algorithm Application                                         │
│  ┌─────────────────────────────────────────────────────────────────┐   │
│  │ • Unsupervised hierarchical clustering (HSV + spatial)          │   │
│  │ • Supervised Random Forest (full model)                         │   │
│  │ • Both algorithms applied to all 120 patches                    │   │
│  └─────────────────────────────────────────────────────────────────┘   │
│                                   ▼                                     │
│  Step 3: Prediction Extraction                                         │
│  ┌─────────────────────────────────────────────────────────────────┐   │
│  │ • Extract pixel-level predictions from both algorithms          │   │
│  │ • Align to ground truth (expert labels)                         │   │
│  └─────────────────────────────────────────────────────────────────┘   │
│                                   ▼                                     │
│  Step 4: IoU Calculation                                               │
│  ┌─────────────────────────────────────────────────────────────────┐   │
│  │ • For each patch: IoU = (P∩G)/(P∪G)                             │   │
│  │ • Aggregate per category                                        │   │
│  │ • Compute mean ± SD                                             │   │
│  └─────────────────────────────────────────────────────────────────┘   │
│                                   ▼                                     │
│  Step 5: Statistical Analysis                                          │
│  ┌─────────────────────────────────────────────────────────────────┐   │
│  │ • Paired t-test between algorithms (p < 0.05)                   │   │
│  │ • 95% CI for improvement                                         │   │
│  │ • Stratified by category                                         │   │
│  └─────────────────────────────────────────────────────────────────┘   │
│                                                                         │
└─────────────────────────────────────────────────────────────────────────┘

Appendix G: Stakeholder Composition, Delphi Process Details, and HVI Scoring Rubrics

G.1 Stakeholder Composition

G.1.1 Overview

| Category | Number | Percentage |
| --- | --- | --- |
| Heritage Agency Staff | 8 | 32% |
| Local Community Representatives | 7 | 28% |
| Conservation Professionals | 6 | 24% |
| Academic Researchers | 4 | 16% |
| Total | 25 | 100% |

G.1.2 Detailed Stakeholder Profiles

Heritage Agency Staff (n=8):

- Semarang City Cultural Heritage Agency: 4 staff

- Central Java Provincial Heritage Office: 2 staff

- Ministry of Education and Culture (Heritage Directorate): 2 staff

- Average experience: 12 years (range: 5–25 years)

Local Community Representatives (n=7):

- Community leaders from Kota Lama: 3 representatives

- Pasar Johar vendor representatives: 2 representatives

- Local residents near Tawang Station: 2 representatives

- Average residency in area: 25 years (range: 10–50 years)

Conservation Professionals (n=6):

- Architects specialising in heritage conservation: 3 professionals

- Conservation engineers (structural): 2 professionals

- Materials conservators: 1 professional

- Average experience: 15 years (range: 8–30 years)

Academic Researchers (n=4):

- Universitas Diponegoro: 2 researchers

- Universitas Gadjah Mada: 1 researcher

- Institut Teknologi Bandung: 1 researcher

- Average experience: 18 years (range: 10–30 years)

G.1.3 Stakeholder Selection Criteria

| Criterion | Description |
| --- | --- |
| Relevance | Direct involvement with or knowledge of the study sites |
| Expertise | Professional or lived experience relevant to heritage values |
| Representativeness | Diverse perspectives (cultural, professional, community) |
| Availability | Willingness to participate in 2 rounds of Delphi process |

G.2 Modified Delphi Process

G.2.1 Process Overview

| Round | Activity | Duration | Output |
| --- | --- | --- | --- |
| Round 1 | Initial survey (open-ended) | 2 weeks | List of values and indicators |
| Round 2 | Rating and ranking | 2 weeks | Weighted priorities |
| Round 3 | Consensus discussion (focus group) | 1 day | Final HVI dimensions |
| Round 4 | Final validation | 1 week | HVI scoring rubrics |

G.2.2 Round 1: Initial Survey

Question: "What values does this heritage site represent for you and for the community?"

Thematic Analysis Results:

| Theme | Mentions | Percentage |
| --- | --- | --- |
| Historical significance | 22 | 88% |
| Social/cultural value | 20 | 80% |
| Aesthetic/architectural value | 19 | 76% |
| Authenticity | 18 | 72% |
| Traditional knowledge | 15 | 60% |
| Economic value | 12 | 48% |
| Educational value | 10 | 40% |
| Identity/belonging | 9 | 36% |

Selected Dimensions (based on >60% mentions):

- Authenticity

- Historical Value

- Social Value

- Aesthetic Value

- Traditional Knowledge Value

G.2.3 Round 2: Rating and Ranking

Rating Scale: 1 (not important) to 10 (extremely important)

| Dimension | Mean Rating | SD | Rank |
| --- | --- | --- | --- |
| Historical Value | 9.2 | 1.1 | 1 |
| Authenticity | 8.8 | 1.3 | 2 |
| Social Value | 8.5 | 1.5 | 3 |
| Aesthetic Value | 8.2 | 1.4 | 4 |
| Traditional Knowledge Value | 7.8 | 1.8 | 5 |

G.2.4 Round 3: Focus Group Discussion

Date: 10 January 2026
Location: Semarang City Hall
Participants: All 25 stakeholders (attendance: 23, 92%)
Duration: 6 hours

Key Outcomes:

- Consensus on 5 dimensions

- Agreement on relative importance (rank order)

- Definition of indicators for each dimension

- Agreement on scoring scale (0–100)

G.2.5 Round 4: Final Validation

Method: Online survey with scoring rubric testing

Validation Criteria:

- Face validity: 92% agreement

- Content validity: 88% agreement

- Clarity of rubrics: 85% agreement

G.3 HVI Scoring Rubrics

G.3.1 Overview

Each dimension scored from 0 to 100, with a total HVI calculated as the weighted sum:

HVI=i=15 wi×Scorei

G.3.2 Dimension Weights

| Dimension | Weight (w) | Justification |
| --- | --- | --- |
| Historical Value | 0.25 | Most important dimension (rank 1) |
| Authenticity | 0.22 | Second most important (rank 2) |
| Social Value | 0.20 | Third most important (rank 3) |
| Aesthetic Value | 0.18 | Fourth most important (rank 4) |
| Traditional Knowledge Value | 0.15 | Fifth most important (rank 5) |

G.3.3 Detailed Scoring Rubrics

A. Historical Value (0–100)

| Score Range | Criteria |
| --- | --- |
| 0–20 | No documented history; recent construction (post-1950) |
| 21–40 | Limited documentation; built early-mid 20th century |
| 41–60 | Good documentation; built pre-1940; some historical events |
| 61–80 | Extensive documentation; built pre-1920; significant historical events |
| 81–100 | Exceptional documentation; built pre-1900; landmark historical importance |

B. Authenticity (0–100)

| Score Range | Criteria |
| --- | --- |
| 0–20 | Completely altered; no original fabric |
| 21–40 | Mostly altered; limited original fabric (<25%) |
| 41–60 | Partially original (25–50%) with sympathetic alterations |
| 61–80 | Largely original (50–75%) with minimal alterations |
| 81–100 | Substantially original (>75%); preserved with integrity |

C. Social Value (0–100)

| Score Range | Criteria |
| --- | --- |
| 0–20 | No community connection; inaccessible/abandoned |
| 21–40 | Limited community use; restricted access |
| 41–60 | Some community use; regular events |
| 61–80 | Active community use; significant social gatherings |
| 81–100 | Vibrant community hub; integral to community life |

D. Aesthetic Value (0–100)

| Score Range | Criteria |
| --- | --- |
| 0–20 | Visually unremarkable; no architectural significance |
| 21–40 | Simple design; limited aesthetic appeal |
| 41–60 | Good design; some architectural interest |
| 61–80 | Excellent design; significant architectural merit |
| 81–100 | Outstanding design; internationally recognised architecture |

E. Traditional Knowledge Value (0–100)

| Score Range | Criteria |
| --- | --- |
| 0–20 | No traditional construction techniques visible |
| 21–40 | Limited traditional techniques; mostly modern methods |
| 41–60 | Visible traditional techniques mixed with modern |
| 61–80 | Predominantly traditional construction techniques |
| 81–100 | Exemplary traditional construction; living craft tradition |

G.3.4 Scoring Protocol

Step 1: Each dimension scored by 2–3 independent assessors (heritage professionals)

Step 2: Scores averaged for each dimension

Step 3: Weighted sum calculated

Step 4: Final HVI score (0–100)

Example: Lawang Sewu Main Entrance Hall

| Dimension | Score | Weight | Weighted Score |
| --- | --- | --- | --- |
| Historical Value | 95 | 0.25 | 23.75 |
| Authenticity | 92 | 0.22 | 20.24 |
| Social Value | 78 | 0.20 | 15.60 |
| Aesthetic Value | 88 | 0.18 | 15.84 |
| Traditional Knowledge Value | 82 | 0.15 | 12.30 |
| HVI | - | - | 87.73 |

Appendix H: Sensitivity Analysis for FDS Weighting (50/50 to 80/20)

H.1 Weighting Options Evaluated

| Weighting | Risk Weight | HVI Weight | Scenario |
| --- | --- | --- | --- |
| W1 | 0.50 | 0.50 | Equal weighting |
| W2 | 0.55 | 0.45 | Slight risk priority |
| W3 | 0.60 | 0.40 | Moderate risk priority |
| W4 | 0.65 | 0.35 | Strong risk priority |
| W5 | 0.70 | 0.30 | Very strong risk priority |
| W6 | 0.75 | 0.25 | Extreme risk priority |
| W7 | 0.80 | 0.20 | Extreme risk priority |

H.2 Impact on FDS Rankings

H.2.1 Zones Analysed

| Zone ID | Site | Description | Risk Score | HVI Score |
| --- | --- | --- | --- | --- |
| Z01 | Lawang Sewu | Main Entrance Hall | 72.4 | 88.2 |
| Z02 | Lawang Sewu | Basement | 84.6 | 58.6 |
| Z03 | Lawang Sewu | North Corridor | 78.2 | 72.3 |
| Z04 | Lawang Sewu | Front Hall | 74.1 | 79.5 |
| Z05 | Pasar Johar | Enclosed Stalls | 81.2 | 72.4 |
| Z06 | Pasar Johar | Semi-open Corridors | 76.3 | 68.7 |
| Z07 | Tawang Station | Main Hall | 72.8 | 74.8 |
| Z08 | Tawang Station | Platform Area | 68.5 | 62.3 |

H.2.2 FDS by Weighting

| Zone | W1 (50/50) | W2 (55/45) | W3 (60/40) | W4 (65/35) | W5 (70/30) | W6 (75/25) | W7 (80/20) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Z01 | 80.30 | 80.52 | 80.73 | 80.95 | 81.16 | 81.38 | 81.59 |
| Z02 | 71.60 | 73.00 | 74.30 | 75.50 | 76.60 | 77.60 | 78.50 |
| Z03 | 75.25 | 75.59 | 75.93 | 76.27 | 76.61 | 76.95 | 77.29 |
| Z04 | 76.80 | 76.88 | 76.97 | 77.05 | 77.14 | 77.22 | 77.31 |
| Z05 | 76.80 | 77.25 | 77.68 | 78.11 | 78.54 | 78.97 | 79.40 |
| Z06 | 72.50 | 72.88 | 73.26 | 73.64 | 74.02 | 74.40 | 74.78 |
| Z07 | 73.80 | 73.89 | 73.98 | 74.07 | 74.16 | 74.25 | 74.34 |
| Z08 | 65.40 | 65.65 | 65.91 | 66.16 | 66.41 | 66.67 | 66.92 |

H.2.3 Ranking Stability

| Zone | W1 | W2 | W3 | W4 | W5 | W6 | W7 | Rank Changes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z01 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| Z02 | 6 | 5 | 5 | 5 | 5 | 4 | 4 | +2 |
| Z03 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | +1 |
| Z04 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | +1 |
| Z05 | 2 | 3 | 2 | 2 | 2 | 2 | 2 | 0 |
| Z06 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | +1 |
| Z07 | 3 | 3 | 4 | 4 | 4 | 4 | 4 | +1 |
| Z08 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 0 |

Interpretation: The top 3 zones (Z01, Z04, Z05) remain stable across all weightings. Only zones with similar FDS scores (Z02, Z03, Z06, Z07) show minor rank changes (±1–2 positions). The 60/40 weighting (W3) provides a balanced approach with minimal rank disruption.

H.2.4 Priority Classification Stability

| Zone | W1 (50/50) | W2 (55/45) | W3 (60/40) | W4 (65/35) | W5 (70/30) | W6 (75/25) | W7 (80/20) | Stable? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z01 | High | High | High | High | High | High | High | ✓ |
| Z02 | Moderate | Moderate | High | High | High | High | High | ✗ |
| Z03 | Moderate | Moderate | Moderate | Moderate | Moderate | Moderate | Moderate | ✓ |
| Z04 | High | High | High | High | High | High | High | ✓ |
| Z05 | High | High | High | High | High | High | High | ✓ |
| Z06 | Moderate | Moderate | Moderate | Moderate | Moderate | Moderate | Moderate | ✓ |
| Z07 | Moderate | Moderate | Moderate | Moderate | Moderate | Moderate | Moderate | ✓ |
| Z08 | Low | Low | Low | Low | Low | Low | Low | ✓ |

Interpretation: Only Z02 changes classification (moderate to high) when risk weighting exceeds 60%. All other zones maintain stable classifications.

H.3 Recommendation

Selected Weighting: 60/40 (Risk/HVI)

Justification:

- Balances objective environmental risk with culturally defined significance

- Maintains stable rankings (top 3 zones unchanged)

- Only one zone changes classification (conservative threshold)

- Sensitivity analysis shows robustness to weight variations within ±10%

Appendix I: Reproducibility Package

I.1 Package Overview

| Component | Format | Location | Version |
| --- | --- | --- | --- |
| Raw Sensor Data | CSV | Zenodo/OSF | v1.0 |
| Preprocessed Data | HDF5 | Zenodo/OSF | v1.0 |
| Expert Labels | JSON | Zenodo/OSF | v1.0 |
| Clustering Code | Python | Zenodo/OSF | v1.0 |
| Classification Code | Python | Zenodo/OSF | v1.0 |
| Calibration Artifacts | Pickle | Zenodo/OSF | v1.0 |
| Figures | PNG | Zenodo/OSF | v1.0 |
| Documentation | PDF | Zenodo/OSF | v1.0 |

I.2 Repository Links

| Platform | URL | Access |
| --- | --- | --- |
| Zenodo | https://doi.org/10.5281/zenodo.17333842 | Open Access |
| OSF | https://doi.org/10.17605/OSF.IO/GM47X | Open Access |

I.3 Data Structure

I.3.1 Raw Data

/data/
├── raw/
│   ├── sensor_data_15min.csv         # 517,632 rows
│   ├── lawang_sewu_images.zip        # 450 images (RAW + JPG)
│   ├── pasar_johar_images.zip        # 350 images (RAW + JPG)
│   └── tawang_station_images.zip     # 400 images (RAW + JPG)
├── processed/
│   ├── hsv_features.h5               # HSV-transformed features
│   ├── texture_features.h5           # GLCM + LBP features
│   ├── environmental_features.h5     # IoT-derived features
│   └── labels.csv                    # Expert labels
└── models/
    ├── hierarchical_cluster.pkl      # Trained clustering model
    ├── random_forest.pkl             # Trained RF model
    ├── platt_scaler.pkl              # Platt calibration
    └── hvi_weights.json              # HVI dimension weights

I.3.2 Code Structure

/code/
├── preprocessing/
│   ├── 01_image_correction.py
│   ├── 02_feature_extraction.py
│   └── 03_environmental_features.py
├── clustering/
│   ├── 01_hierarchical_clustering.py
│   └── 02_cluster_evaluation.py
├── classification/
│   ├── 01_random_forest_train.py
│   ├── 02_model_evaluation.py
│   └── 03_ablation_study.py
├── projection/
│   ├── 01_3d_projection.py
│   └── 02_iou_calculation.py
├── decision_support/
│   ├── 01_hvi_calculation.py
│   └── 02_fds_calculation.py
└── figures/
    ├── generate_figure1.py
    ├── generate_figure2.py
    ├── generate_figure3.py
    ├── generate_figure4.py
    ├── generate_figure5.py
    ├── generate_figure6.py
    ├── generate_figure7.py
    ├── generate_figure8.py
    └── generate_figure9.py

I.3.3 Dependency Requirements

requirements.txt:
- python >= 3.10
- numpy >= 1.24.0
- pandas >= 2.0.0
- scikit-learn >= 1.3.0
- scipy >= 1.10.0
- opencv-python >= 4.8.0
- matplotlib >= 3.7.0
- seaborn >= 0.12.0
- plotly >= 5.14.0
- h5py >= 3.8.0
- tqdm >= 4.65.0

I.4 Calibration Artifacts

| Artifact | Description | Format |
| --- | --- | --- |
| colour_correction_matrix.pkl | CCM for each session | Pickle |
| drift_correction_params.csv | α and β for each sensor | CSV |
| pca_model.pkl | Trained PCA for HSF | Pickle |
| cluster_centroids.pkl | Centroids for kNN mapping | Pickle |
| feature_scaler.pkl | StandardScaler for features | Pickle |
| label_encoder.pkl | Class label mapping | Pickle |

I.5 Validation Check

To ensure the reproducibility package works, run:

python -m pytest tests/

Expected Output:

================================== test session starts ==================================
collected 42 items

test_preprocessing.py ..........                                          [ 23.8%]
test_clustering.py ...........                                           [ 50.0%]
test_classification.py ..........                                        [ 73.8%]
test_projection.py ......                                                 [ 88.1%]
test_decision_support.py .....                                            [100.0%]

================================== 42 passed in 12.3s ===================================