| Journal: | Life Cycle Assessment of Coconut Residue Valorization via Pyrolysis – Supplementary Information |
| --- | --- |
| Article Type: | Original Research Paper |
| Authors: | Alessandra da Rocha Duailibe Monteiro – Department of Chemical and Petroleum Engineering, Federal Fluminense University (UFF), Rua Passo da Pátria, 152, São Domingos, Niterói - Rio de Janeiro State, Brazil. ZIP CODE 24210-240 ORCID ID: 0000-0002-3114-4133 Mariana da Silva Teixeira – Department of Chemical and Petroleum Engineering, Federal Fluminense University (UFF), Rua Passo da Pátria, 152, São Domingos, Niterói – Rio de Janeiro State, Brazil. ZIP CODE 24210-240 ORCID ID: 0009-0005-8082-656X |
| Corresponding Author: | Alessandra da Rocha Duailibe Monteiro Phone Number : +55 21 992631295 alessandra_duailibe@id.uff.br |

This supplementary information presents the input and output data obtained in the OpenLCA software for Process 1 (Table A.1), present in all the scenarios analyzed in this work, and for Process 7 (Table A.2), considered in scenario 2. Additionally, the sensitivity analysis data for the modified Process 7 (Table A.3), used in scenario 4.1, and Process 7 modified for scenario 4.2 (Table A.4) are presented.

The "Sourcing Process" column indicates the process of the Ecoinvent 3.6 database. selected as the producer of the corresponding chain. When there is no associated supply process, the flow is characterized as elementary flow. In addition, for the products of the process itself, the description "Process Product" was inserted in the same column.

Table A.1 – Process 1: Cultivation and Consumption of Coconut. (Page 2)

(continues)

| Inputs |  |  |  |
| --- | --- | --- | --- |
| Flows | Value | Unit | Sourcing Process |
| Ammonium Sulfate, as N | 0.00041 | kg | market for ammonium sulfate, as N ¦ ammonium sulfate, as N ¦ Cutoff, U |
| Carbon Dioxide, in the air | 0.20444 | kg | – |
| Energy, gross calorific value, in biomass | 2.64358 | MJ | – |
| Orchard Establishment | 0.00088 | item(s) | market for establishing orchard ¦ establishing orchard ¦ Cutoff, U |
| Fruit tree seedling, for planting | 0.00088 | item(s) | market for fruit tree seedling, for planting ¦ fruit tree seedling, for planting ¦ Cutoff, U |
| Glyphosate | 0.00013 | kg | market for glyphosate ¦ glyphosate ¦ Cutoff, U |
| Irrigation | 0.06049 | m3 | market for irrigation ¦ irrigation ¦ Cutoff, U |
| Liquid manure spreading, by vacuum tank | 0.00004 | m3 | market for liquid manure spreading, by vacuum tanker ¦ liquid manure spreading, by vacuum tanker ¦ Cutoff, U |
| Liquid manure, bovine | 0.06361 | kg | market for manure, liquid, cattle ¦ manure, liquid, cattle ¦ Cutoff, U |
| Liquid manure, pig | 0,01191 | kg | market for manure, liquid, swine ¦ manure, liquid, swine ¦ Cutoff, U |
| nitrogen fertilizer, such as N | 0,00027 | kg | market for nitrogen fertiliser, as N ¦ nitrogen fertiliser, as N ¦ Cutoff, U |
| Occupation, permanent crop, irrigated | 0.26396 | m2.a | – |
| Packaging, for fertilizers | 0.00859 | kg | market for packaging, for fertilisers ¦ packaging, for fertilisers ¦ Cutoff, U |
| Packaging, for pesticides | 0.00030 | kg | market for packaging, for pesticides ¦ packaging, for pesticides ¦ Cutoff, U |

Table A.1 – Process 1: Cultivation and Consumption of Coconut. (Page 3)

(continues)

| Inputs |  |  |  |
| --- | --- | --- | --- |
| Flows | Value | Unit | Sourcing Process |
| Phosphate fertilizer, such as P₂O₅ | 0.00095 | kg | market for phosphate fertiliser, as P2O5 ¦ phosphate fertiliser, as P2O5 ¦ Cutoff, U |
| Tree planting | 0.00008 | item(s) | market for planting tree ¦ planting tree ¦ Cutoff, U |
| Potassium chloride, as K₂O | 0.00115 | kg | market for potassium chloride, as K2O ¦ potassium chloride, as K2O ¦ Cutoff, U |
| Potassium fertilizer, such as K₂O | 0.00052 | kg | market for potassium fertiliser, as K2O ¦ potassium fertiliser, as K2O ¦ Cutoff, U |
| Potassium sulfate, such as K₂O | 0.00005 | kg | market for potassium sulfate, as K2O ¦ potassium sulfate, as K2O ¦ Cutoff, U |
| Bird dung, dry | 0.00985 | kg | market for poultry manure, dried ¦ poultry manure, dried ¦ Cutoff, U |
| Bird manure, fresh | 0.00139 | kg | market for poultry manure, fresh ¦ poultry manure, fresh ¦ Cutoff, U |
| Pyrethroid compound | 0.00002 | kg | market for pyrethroid-compound ¦ pyrethroid-compound ¦ Cutoff, U |
| Loading and spreading of solid manure by loader and hydraulic distributor | 0.04411 | kg | market for solid manure loading and spreading, by hydraulic loader and spreader ¦ solid manure loading and spreading, by hydraulic loader and spreader ¦ Cutoff, U |
| Processing, permanent crop, irrigated | 0.00439 | m2 | – |
| Processing, permanent crop, irrigated | 0.00439 | m2 | – |
| Processing, for permanent crops, irrigated | 0.00439 | m2 | – |
| Transport, tractor and trailer, agricultural | 0.00031 | t.km | market for transport, tractor and trailer, agricultural ¦ transport, tractor and trailer, agricultural ¦ Cutoff, U |
| Espalier system, softwood poles, impregnated with tar | 0.00003 | has | market for trellis system, wooden poles, soft wood, tar impregnated ¦ trellis system, wooden poles, soft wood, tar impregnated ¦ Cutoff, U |
| Urea, as N | 0.00095 | kg | market for urea, as N ¦ urea, as N ¦ Cutoff, U |
| Transportation, freight, 16–32 metric ton truck, EURO4 | 27.4 | kg.km | transport, freight, lorry 16-32 metric ton, EURO4 ¦ transport, freight, lorry 16-32 metric ton, EURO4 ¦ Cutoff, U - RoW |

Table A.1 – Process 1: Cultivation and Consumption of Coconut. (Page 3)

| Outputs |  |  |  |
| --- | --- | --- | --- |
| Flows | Value | Unit | Sourcing Process |
| Coconut Shell - Post Consumption | 0.10000 | kg | Process Product |
| Hulled Coconut | 0.15641 | kg | Process Product |
| Ammonia | 0.00040 | kg | – |
| Cadmium | 9.0618E-8 | kg | – |
| Cadmium, ion | 2.1313E-09 | kg | – |
| Carbon dioxide, fossil | 0.00162 | kg | – |
| Carbon, soil or biomass dioxide | 0.01614 | kg | – |
| Chrome | 2.0038E-08 | kg | – |
| Chromium, ion | 5.7483E-07 | kg | – |
| Copper | -4.6341E-08 | kg | – |
| Copper, Ion | 3.0099E-07 | kg | – |
| Cypermethrin | 0.00002 | kg | – |
| Nitrous oxide (dinitrogen monoxide) | 0.00006 | kg | – |
| Glyphosate | 0.00013 | kg | – |
| Lead | 1.5399E-07 | kg | – |
| Mercury | 3.4085E-09 | kg | – |
| Nickel | 1.5197E-07 | kg | – |
| Nickel, ion | 1.0048E-07 | kg | – |
| Nitrate | 0.00694 | kg | – |
| Nitrogen oxides | 0.00007 | kg | – |
| Phosphate | 2.7542E-05 | kg | – |
| Phosphorus | 3.9036E-06 | kg | – |
| Wood residue, untreated | 0.00877 | kg | market group for waste wood, untreated ¦ waste wood, untreated ¦ Cutoff, U |
| Water | 0.06049 | m3 | – |
| Zinc | 2.1616E-06 | kg | – |
| Zinc, Ion | 9.0864E-07 | kg | – |

Table A.2 – Process 7: Sanitary Landfill.

| Inputs |  |  |  |
| --- | --- | --- | --- |
| Flows | Value | Unit | Sourcing Process |
| Coconut Shell - Consumed | 0.10000 | kg | Process 1: Cultivation and Consumption of Coconut |
| Electricity - High Voltage | 0.01641 | kWh | market for electricity, high voltage ¦ electricity, high voltage ¦ Cutoff, U - BR-Southern grid |
| Clay | 0.03313 | kg | market for clay ¦ clay ¦ Cutoff, U - RoW |
| Diesel | 2.4000E-5 | kg | diesel production, petroleum refinery operation ¦ diesel ¦ Cutoff, U - BR |
| Outputs |  |  |  |
| Flows | Value | Unit | Sourcing Process |
| Biogas (Biomethane) | 0.0014 | kg | Process Product |
| Biomethane | - 0.0014 | kg | Process created (biomethane) |
| Methane | 0.004765 | kg | – |
| Hydrogen | 0.000009 | kg | – |
| Carbon Dioxide, from soil or biomass stock | 0.003617 | kg | – |
| Nitrogen | 0.001896 | kg | – |
| Oxygen | 0.000243 | kg | – |
| Nitrogen Monoxide | 1030 | ug | – |
| Carbon Monoxide, Soil or Biomass Stock | 1200 | ug | – |
| Particulates, unspecified | 388.0 | ug | – |
| Dioxinas / Furano | 10.900 | pg | – |
| Leachate Emissions |  |  |  |
| Flows | Value | Unit | Sourcing Process |
| Biological Oxygen Demand (BOD5) | 3.8200E-09 | t | – |
| Chemical Oxygen Demand (COD) | 9.1200E-09 | t | – |
| Suspended Solids, unspecified | 1.7700E-09 | t | – |
| Ammonia | 2.0200E-09 | t | – |
| Nitrogen, Organically Bound | 1.5000E-10 | t | – |
| Phosphorus | 6.1900E-11 | t | – |

Table A.3 – Process 7 – Scenario 4.1: Landfill with Electricity Generation.

| Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Flows |  | Value | Unit | Sourcing Process |
| Coconut Shell - Consumed |  | 0.10000 | kg | Process 1 |
| Electricity - High Voltage |  | 0.00091 | kWh | market for electricity, high voltage ¦ electricity, high voltage ¦ Cutoff, U - BR-Southern grid |
| Electricity - High Voltage |  | -0.00091 | kWh | market for electricity, high voltage ¦ electricity, high voltage ¦ Cutoff, U - BR-Southern grid |
| Clay |  | 0.03313 | kg | market for clay ¦ clay ¦ Cutoff, U - RoW |
| Diesel |  | 2.4000E-5 | kg | diesel production, petroleum refinery operation ¦ diesel ¦ Cutoff, U - BR |
| Outputs |  |  |  |  |
| Flows |  | Value | Unit | Sourcing Process |
| Biogas (Biomethane) |  | 0.0014 | kg | Process Product |
| Biomethane |  | - 0.0014 | kg | Auxiliary Process (Biomethane) |
| Emissions to air |  |  |  |  |
| Flows |  | Value | Unit | Sourcing Process |
| Methane |  | 0.00399 | kg | – |
| Hydrogen |  | 0.000008 | kg | – |
| Carbon Dioxide, from soil or biomass stock |  | 0.00303 | kg | – |
| Nitrogen |  | 0.00159 | kg | – |
| Oxygen |  | 0.00020 | kg | – |
| Nitrogen Monoxide |  | 94630 | ug | – |
| Carbon Monoxide, Soil or Biomass Stock |  | 63000 | ug | – |
| Particulates, unspecified |  | 2258 | ug | – |
| Dioxinas / Furano |  | 10.900 | pg | – |
| Leachate emissions |  |  |  |  |
| Flows |  | Value | Unit | Sourcing Process |
| Biological Oxygen Demand (BOD5) |  | 3.8200E-09 | t | – |
| Chemical Oxygen Demand (COD) |  | 9.1200E-09 | t | – |
| Suspended Solids, unspecified | 1.7700E-09 |  | t | – |
| Ammonia | 2.0200E-09 |  | t | – |
| Nitrogen, Organically Bound | 1.5000E-10 |  | t | – |
| Phosphorus | 6.1900E-11 |  | t | – |

Table A.4 – Process 7 - Scenario 4.2: Landfill with 100% reuse.

| Inputs |  |  |  |
| --- | --- | --- | --- |
| Flows | Value | Unit | Sourcing Process |
| Coconut Shell - Consumed | 0.10000 | kg | Process 1 |
| Electricity - High Voltage | 0.00177 | kWh | market for electricity, high voltage ¦ electricity, high voltage ¦ Cutoff, U - BR-Southern grid |
| Electricity - High Voltage | -0.00177 | kWh | market for electricity, high voltage ¦ electricity, high voltage ¦ Cutoff, U - BR-Southern grid |
| Clay | 0.03313 | kg | market for clay ¦ clay ¦ Cutoff, U - RoW |
| Diesel | 2.4000E-5 | kg | diesel production, petroleum refinery operation ¦ diesel ¦ Cutoff, U - BR |
| Outputs |  |  |  |
| Flows | Value | Unit | Sourcing Process |
| Biogas (Biomethane) | 0.0028 | kg | Process Product |
| Biomethane | - 0.0028 | kg | Auxiliary Process (biomethane) |
| Emissions to air |  |  |  |
| Flows | Value | Unit | Sourcing Process |
| Methane | 0.00228 | kg | – |
| Hydrogen | 0.000004 | kg | – |
| Carbon Dioxide, from soil or biomass stock | 0.00173 | kg | – |
| Nitrogen | 0.00091 | kg | – |
| Oxygen | 0.00012 | kg | – |
| Nitrogen Monoxide | 18926 | ug | – |
| Carbon Monoxide, Soil or Biomass Stock | 13860 | ug | – |
| Particulates, unspecified | 4516 | ug | – |
| Dioxinas / Furano | 21.80 | pg | – |
| Leachate emissions |  |  |  |
| Flows | Value | Unit | Sourcing Process |
| Biological Oxygen Demand (BOD5) | 3.8200E-09 | t | – |
| Chemical Oxygen Demand (COD) | 9.1200E-09 | t | – |
| Suspended Solids, unspecified | 1.7700E-09 | t | – |
| Ammonia | 2.0200E-09 | t | – |
| Nitrogen, Organically Bound | 1.5000E-10 | t | – |
| Phosphorus | 6.1900E-11 | t | – |