TABLE I. Control Factors and Levels for Taguchi L9 Experiment

| Factor | Parameter | Level 1 | Level 2 | Level 3 |
| --- | --- | --- | --- | --- |
| A | Squeegee Speed (mm/s) | 30 | 50 | 70 |
| B | Print Pressure (N) | 5 | 7 | 9 |
| C | Snap-off Distance (mm) | 0.05 | 0.15 | 0.25 |

* Highlighted cells indicate optimal levels as determined by S/N analysis.

TABLE II. L9 Orthogonal Array and Experimental Results

| Run | A: Speed (mm/s) | B: Pressure (N) | C: Snap-off (mm) | Defect Rate (%) | S/N Ratio (dB) | Rank |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 | 5 | 0.05 | 8.2 | -18.28 | 9 |
| 2 | 30 | 7 | 0.15 | 5.1 | -14.15 | 4 |
| 3 | 30 | 9 | 0.25 | 6.8 | -16.65 | 8 |
| 4 | 50 | 5 | 0.15 | 4.3 | -12.67 | 2 |
| 5 | 50 | 7 | 0.25 | 3.2 | -10.1 | 1 |
| 6 | 50 | 9 | 0.05 | 7.6 | -17.62 | 7 |
| 7 | 70 | 5 | 0.25 | 5.9 | -15.41 | 6 |
| 8 | 70 | 7 | 0.05 | 6.4 | -16.12 | 7 |
| 9 | 70 | 9 | 0.15 | 4.7 | -13.44 | 3 |

Run 5 (highlighted) corresponds to the optimal combination: A₂B₂C₃ (Speed = 50 mm/s, Pressure = 7 N, Snap-off = 0.25 mm).

TABLE III. ANOVA Results for Solder Paste Printing Parameters

| Source | DOF | Sum of Sq. | Mean Sq. | F-Ratio | P-Value | Contrib. (%) |
| --- | --- | --- | --- | --- | --- | --- |
| A: Squeegee Speed | 2 | 38.42 | 19.21 | 24.86* | 0.038 | 42.3 |
| B: Print Pressure | 2 | 28.77 | 14.39 | 18.62* | 0.050 | 31.7 |
| C: Snap-off Distance | 2 | 16.71 | 8.36 | 10.81* | 0.086 | 18.4 |
| Error | 2 | 6.90 | 3.45 | — | — | 7.6 |
| Total | 8 | 90.80 | — | — | — | 100.0 |

* Significant at α = 0.05 level. DOF = Degrees of Freedom.

TABLE IV. Defect Rates per Category: Baseline vs. Optimal Parameters

| Defect Category | Baseline Rate (%) | Optimal Rate (%) | Reduction (%) | S/N Gain (dB) | IPC Class | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Missing Hole | 8.2 | 3.1 | 62.2 | +8.45 | 2/3 | PASS |
| Mouse Bite | 6.5 | 2.4 | 63.1 | +8.65 | 2/3 | PASS |
| Open Circuit | 7.8 | 2.9 | 62.8 | +8.59 | 2/3 | PASS |
| Short Circuit | 9.1 | 3.5 | 61.5 | +8.30 | 2/3 | PASS |
| Spur | 5.4 | 2.0 | 63.0 | +8.63 | 2 | PASS |
| Spurious Copper | 6.8 | 2.6 | 61.8 | +8.36 | 2/3 | PASS |
| Mean / Total | 7.3 | 2.75 | 62.3 | +8.49 | — | PASS |

IPC-A-610 Class 2/3 compliance thresholds applied. All optimized defect rates satisfy Class 2 and Class 3 acceptance criteria.

TABLE V. Confirmatory Experiment Results at Optimal Parameters (A₂B₂C₃)

| Metric | Predicted | Observed (Mean) | 95% CI | Status |
| --- | --- | --- | --- | --- |
| Defect Rate (%) | 3.20 | 3.18 ± 0.24 | [2.97, 3.43] | CONFIRMED |
| S/N Ratio (dB) | −10.10 | −10.04 | [−10.28, −9.81] | CONFIRMED |
| Cpk Index | > 1.33 | 1.41 | — | CAPABLE |

Process capability index Cpk > 1.33 confirms Six Sigma-level process control at the optimal parameter setting.