Table 1: Experimental parameter variation domains and levels studied.

| Parameters | Current Density (A) | Contact Area (cm²) | Electrolysis Time (min) | Magnetic Stirring (rpm) |
| --- | --- | --- | --- | --- |
| Variables and Concept | X1 | X2 | X3 | X4 |
| Low Level (-1)‎ | 0.25 | 150 | 30 | 100 |
| Mid-Level (0) | 0.425 | 225 | 45 | 200 |
| High Level (-1)‎ | 0.6 | 300 | 60 | 300 |

Table 2 : Experiment design matrix.

| Variable experiment | X1 (A) | X2(cm2) | X3(min) | X4(rpm) | Y1(%) | Y2(%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 0 | 0 | 93.34 | 88.90 |
| 2 | 0 | -1 | -1 | 0 | 90.19 | 85.90 |
| 3 | -1 | -1 | 0 | 0 | 89.36 | 84.15 |
| 4 | 1 | 0 | 0 | -1 | 95.53 | 90.98 |
| 5 | 0 | -1 | 0 | 1 | 93.71 | 89.25 |
| 6 | 0 | 1 | 0 | -1 | 90.55 | 86.24 |
| 7 | 1 | 0 | 0 | 1 | 94 | 89.53 |
| 8 | 0 | 0 | 0 | 0 | 91.51 | 87.16 |
| 9 | 0 | 0 | -1 | -1 | 89.23 | 84.98 |
| 10 | -1 | 0 | -1 | 0 | 89.10 | 84.70 |
| 11 | 1 | 0 | -1 | 0 | 91.57 | 87.21 |
| 12 | 0 | -1 | 1 | 0 | 89.83 | 85.56 |
| 13 | 1 | 1 | 0 | 0 | 94.39 | 89.89 |
| 14 | 0 | -1 | 0 | -1 | 91.15 | 86.74 |
| 15 | 0 | 1 | -1 | 0 | 93.52 | 89.07 |
| 16 | 0 | 1 | 1 | 0 | 91.75 | 87.38 |
| 17 | 0 | 0 | 1 | -1 | 89.33 | 85.08 |
| 18 | -1 | 0 | 0 | -1 | 90.19 | 85.90 |
| 19 | 0 | 0 | -1 | 1 | 92.92 | 88.50 |
| 20 | -1 | 0 | 1 | 0 | 92.81 | 88.39 |
| 21 | 0 | 0 | 0 | 0 | 91.50 | 87.11 |
| 22 | 0 | 0 | 0 | 0 | 91.57 | 87.18 |
| 23 | 0 | 0 | 1 | 1 | 90.38 | 86.08 |
| 24 | 0 | 1 | 0 | 1 | 92.97 | 88.55 |
| 25 | -1 | 0 | 0 | 1 | 88.32 | 84.11 |
| 26 | 1 | 0 | 1 | 0 | 96.02 | 91.44 |
| 27 | -1 | 1 | 0 | 0 | 91.75 | 87.38 |
| 28 | 0 | 0 | 0 | 0 | 90.41 | 86.5 |
| 29 | 0 | 0 | 0 | 0 | 90.35 | 85.9 |
| 30 | 0 | -1 | 0 | 1 | 92.06 | 88.3 |
| 31 | 0 | 1 | -1 | 0 | 89.6 | 85.8 |

Table 3 : Wastewater characteristics.

| Parameter | Average Value | Maximum |
| --- | --- | --- |
| pH | 7.52 | 7.8 |
| Temperature (°C) | 15.42 | 18.2 |
| Conductivity (µS/cm) | 1.398 | 1.8 |
| COD (mg O₂/L) | 386.70 | 488 |
| BOD₅ (mg O₂/L) | 215 | 284.4 |
| TSS (mg/L) | 278.7 | 300 |
| Turbidity (NTU) | 202.29 | 550 |
| NO₃⁻ (mg/L) | 4.22 | 6.5 |
| NO2- (mg/L) | 4.46 | 5.8 |

Table 4: Definition of the influencing factors and responses of the experimental design.

| X | X1 | X2 | X3 | X4 | Y1 | Y2 |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Current (A) | Contact Area (cm²) | Electrolysis Time (min) | Stirring Speed (rpm) | Turbidity Removal (Efficiency (%) | Chemical Oxygen Demand Removal Efficiency (%) |

Table 5: Regression parameters and determination coefficients for turbidity yield.

| S | R-sq | R-sq (adj) | R-sq (pred) |
| --- | --- | --- | --- |
| 0.399916 | 97.72% | 95.72% | 92.67% |

Table 6: Analysis of Variance (ANOVA) Results for the Response Surface Model of Turbidity.

| Source | DF | Adj SS | Adj MS | F-value | P- value |
| --- | --- | --- | --- | --- | --- |
| Model | 14 | 109.626 | 7.8304 | 48.96 | 0.000 |
| Linear | 4 | 105.983 | 26.4958 | 165.67 | ‎0‎‏.‏‎000‎ |
| X1(A) | 1 | 61.953 | 61.9531 | 387.37 | ‎0‎‏.‏‎000‎ |
| X2(cm2) | 1 | 24.929 | 24.9288 | 155.87 | ‎0‎‏.‏‎000‎ |
| X3 (min) | 1 | 16.203 | 16.2033 | 101.31 | ‎0‎‏.‏‎000‎ |
| X4 (rpm) | 1 | 2.898 | 2.8982 | 18.12 | 0.001 |
| Square | 4 | 0.726 | 0.1815 | 1.14 | 0.375 |
| X1(A)*X1(A) | 1 | 0.142 | 0.1422 | 0.89 | 0.360 |
| X2(cm²) *X2(cm²) | 1 | 0.373 | 0.3733 | 2.33 | 0.146 |
| X3(min)*X3(min) | 1 | 0.158 | 0.1577 | 0.99 | 0.336 |
| X4(rpm)*X4(rpm) | 1 | 0.268 | 0.2677 | 1.67 | 0.214 |
| Two-Factor Interaction | 6 | 2.917 | 0.4861 | 3.04 | 0.035 |
| X1(A)*X2(cm2) | 1 | 0.008 | 0.0081 | 0,05 | 0.825 |
| X1(A)*X3(min) | 1 | 2.031 | 2.0306 | 12,70 | 0.003 |
| X1(A)*X4(rpm) | 1 | 0.109 | 0.1089 | 0,68 | 0.421 |
| X2(cm²) *X3(min) | 1 | 0.286 | 0.2862 | 1.79 | 0.200 |
| X2 (cm²) * X4 (rpm) | 1 | 0.152 | 0.1521 | 0.95 | 0.344 |
| X3(min)*X4(rpm) | 1 | 0.331 | 0.3306 | 2.07 | 0.170 |
| Error | 16 | 2.559 | 0.1599 |  |  |
| Lack of Fit | 10 | 1.079 | 0.1079 | 0.44 | 0.881 |
| Pure Error | 6 | 1.480 | 0.2467 |  |  |
| Total | 30 | 112.185 |  |  |  |

Table 7: Regression parameters and coefficients of determination for COD yield.

| S | R-sq | R-sq (adj) | R-sq (pred) |
| --- | --- | --- | --- |
| 0.224210 | 99.25% | 98.60% | 96.87% |

Table 8: Analysis of Variance (ANOVA) Results for the Response Surface Model of COD.

| Source | DF | Adj SS | Adj MS | F-value | P- value |
| --- | --- | --- | --- | --- | --- |
| Model | 14 | 107.147 | 7.6533 | 152.24 | 0.000 |
| Linear | 4 | 104.099 | 26.0248 | 517.70 | 0.000 |
| X1(A) | 1 | 59.189 | 59.1890 | 1177.41 | 0.000 |
| X2(cm2) | 1 | 24.584 | 24.5835 | 489.03 | 0.000 |
| X3 (min) | 1 | 16.884 | 16,8840 | 335.86 | 0.000 |
| X4 (rpm) | 1 | 3.443 | 3.4428 | 68.49 | 0.000 |
| Square | 4 | 1.200 | 0.2999 | 5.97 | 0.004 |
| X1(A)*X1(A) | 1 | 0.262 | 0.2621 | 5.21 | 0.036 |
| X2(cm²) *X2(cm²) | 1 | 0.537 | 0.5366 | 10.67 | 0.005 |
| X3(min)*X3(min) | 1 | 0.443 | 0.4431 | 8.81 | 0.009 |
| X4(rpm)*X4(rpm) | 1 | 0.327 | 0.3273 | 6.51 | 0.021 |
| Two-Factor Interaction | 6 | 1.848 | 0.3079 | 6.13 | 0.002 |
| X1(A)*X2(cm2) | 1 | 0.104 | 0.1040 | 2.07 | 0.170 |
| X1(A)*X3(min) | 1 | 1.249 | 1.2488 | 24.84 | 0.000 |
| X1(A)*X4(rpm) | 1 | 0.305 | 0.3053 | 6.07 | 0.025 |
| X2(cm²) *X3(min) | 1 | 0.074 | 0.0743 | 1.48 | 0.242 |
| X2 (cm²) * X4 (rpm) | 1 | 0.018 | 0.0176 | 0.35 | 0.563 |
| X3(min)*X4(rpm) | 1 | 0.098 | 0.0977 | 1.94 | 0.182 |
| Error | 16 | 0.804 | 0.0503 |  |  |
| Lack of Fit | 10 | 0.520 | 0.0520 | 1.10 | 0.476 |
| Pure Error | 6 | 0.284 | 0.0474 |  |  |
| Total | 30 | 107.951 |  |  |  |