Supporting Information

Fossil Carbon Transformation in Municipal Wastewater Treatment and Its Implications for Greenhouse Gas Accounting

Haiyan Lia,*, Yutong Chonga, Xinyue Hea, Wenqi Yanga, Yanzhi Wanga, Juanjuan Chena, b, Lu Lua,*

a State Key Laboratory of Urban-Rural Water Resource and Environment, Shenzhen Key Laboratory of Organic Pollution Prevention and Control, School of Eco-Environment, Harbin Institute of Technology Shenzhen, Shenzhen, 518055, China

b Guangdong Branch, Beijing General Municipal Engineering Design & Research Institute Co., Ltd, Guangzhou, 510075, China

*Correspondance to: Haiyan Li (lihaiyan2021@hit.edu.cn), Lu Lu (lulu@hit.edu.cn)

S1. Wastewater Treatment Plant: Basic conditions and nutrient removal

The experiments were conducted in five selected MWWTPs, P1 to P5, operating three process configurations: modified AAO (P1, P4), AAO-type oxidation ditch (P2, P5), and Unitank (a modification of SBR, and a combined alternating activated sludge process; P3). The flow rates and influent and effluent characteristics are summarized in Table S1.

Table S1. Flow rates and influent and effluent characteristics of the MWWTPs

| WWTPs | Sampling site | Flow rate | COD | SS | TOC | TN | NH3-N | TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | m3/d | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
| P1 (AAO) | Influent | 204658 | 230 | 152 | 63.74 | 48.8 | 39.2 | 5.35 |
|  | Primary effluent |  | 155 | - | 26.74 | 23.3 | 21.9 | 0.48 |
|  | Effluent |  | 13 | 5 | 9.36 | 7.35 | 0.50 | 0.22 |
| P2 (OD) | Influent | 47000 | 448 | 125 | 66.96 | 32.5 | 30.3 | 10.2 |
|  | Primary effluent |  | 371 | - | 58.34 | 24.8 | 24.2 | 6.95 |
|  | Effluent |  | 12 | 4 | 9.23 | 6.94 | 0.39 | 0.12 |
| P3 (Unitank) | Influent | 166500 | 376 | 130 | 61.54 | 28.2 | 23.1 | 2.67 |
|  | Primary effluent |  | 256 | - | 58.80 | 21.7 | 19.8 | 0.36 |
|  | Effluent |  | 11 | 4 | 7.76 | 7.36 | 0.34 | 0.09 |
| P4 (AAO) | Influent | 56480 | 173 | 182 | - | 35.2 | 19.1 | 2.12 |
|  | Effluent |  | 6.7 | 0.21 | - | 4.3 | 0.47 | 0.11 |
| P5 (OD) | Influent | 62311 | 181 | 104.5 | - | 29.3 | 24.8 | 2.73 |
|  | Effluent |  | 13.5 | 4.5 | - | 6.7 | 0.20 | 0.20 |

S2. Monitoring and experimental procedures

Trace gas analyzers (dimensions: 0.8 m × 0.4 m × 0.1 m) were connected to the gas collection pipelines of MWWTPs to measure real-time GHG concentrations (in ppm/ppb). The monitoring setup is illustrated in Figure S1. Due to fluctuations in air supply, the exhaust gas velocity varied dynamically. To improve the accuracy of emission calculations, a Testo 480 multifunctional analyzer, equipped with temperature, humidity, and pressure sensors, as well as an anemometer probe, was used alongside the gas concentration measurements. During operation, the probe was vertically inserted into the central zone of the exhaust outlet, ensuring proper alignment with the airflow to optimize sensor performance. By continuously recording wind speed and combining it with the known cross-sectional area of the outlet, dynamic gas flow rates were determined throughout the monitoring period.

Figure S1. Monitoring configurations for GHG emissions at the five MWWTPs: (a) P1; (b) P2; (c) P3; (d) P4; (e) P5. Each panel provides an overview of the site-specific gas collection and monitoring setup.

As the ventilation fans draw in ambient air to transport off-gas from the treatment units, the background GHG concentration in the plant area was used as the baseline. When calculating actual GHG emissions, this background concentration was subtracted to isolate process-related emissions. This approach enabled accurate estimation of GHG emissions by integrating real-time gas concentrations and flow rates, as detailed in Equations (S1)–(S4).

|  |  | (S1) |
| --- | --- | --- |
|  |  | (S2) |
|  |  | (S3) |
|  |  | (S4) |

Where E is the instantaneous emission rate of the target GHG (CO2, CH4, and N2O) at the exhaust outlet (g s-1), M is the molecular mass of the GHG (g mol-1), c and c' are the measured GHG concentrations at the exhaust outlet and the in-plant background, respectively (ppm for CO2, ppb for CH4 and N2O), V0 is the molar volume of an ideal gas at standard conditions (22.4 L mol-1), T0 and P0 are the standard temperature (273.15 K) and pressure (101.325 kPa), T and P are the gas temperature (K) and pressure (kPa) at the exhaust outlet, Q is the volumetric flow rate at the outlet (m3 s-1); S is the cross-sectional area of the outlet (m2), v is the measured air velocity at the outlet (m/s), and D is the inner diameter of the exhaust pipeline (m).

Figure S2 illustrates the procedures for the collection and processing of both liquid and solid samples.

Figure S2. Collection and processing of liquid and solid samples: (a) 24-hours liquid sample collection; (b) Preparation of mixed liquid and sludge samples; (c) Drying of sludge samples.

S3. Monte Carlo Simulation results: determination of emission factors

Based on our field measurements and literature data, fossil CO2 emissions were estimated for each MWWTP. Using the chemical oxygen demand (COD) removal of each plant as the activity data, we calculated the fossil CO2 emission factor (EF), defined as the amount of fossil CO2 emitted per unit mass of COD removed (kg f- CO2/kg CODremoved; Table S2).

Table S2. Fossil CO2 EFs of MWWTPs from this study and published literature

| No. | Location | Process type | Emission factor (kg f-CO2/kg COD removed) | Resource |
| --- | --- | --- | --- | --- |
| 1 | Queensland, Australia | ASP | 0.0300 0.0179 | 1 |
| 2 | Southern California, USA | MLE ASP | 0.0283 | 2 |
| 3 | Beijing, China | AO | 0.1924 0.0326 | 3 |
| 4 | Beijing, China | SBR | 0.1639 0.0284 |  |
| 5 | Guangdong, China (P1) | AAO | 0.0722 0.0051 | This study |
| 6 | Guangdong, China (P2) | OD | 0.0028 0.0005 |  |
| 7 | Guangdong, China (P3) | Unitank | 0.0033 0.0008 |  |
| 8 | Guangdong, China (P4) | AAO | 0.1650 0.0391 |  |
| 9 | Guangdong, China (P5) | OD | 0.0352 0.0165 |  |

Following Monte Carlo simulation, the distribution of fossil CO2 EFs for different treatment technologies is presented in Figure S3. The simulated EF values for several representative wastewater treatment processes are summarized in Table S3. A comprehensive set of fossil CO2 EFs for various treatment technologies is provided in Table S4, which are further used to estimate total fossil CO2 emissions from MWWTPs across China.

Figure S3. Monte Carlo simulation results for fossil CO2 emission factors across different wastewater treatment technologies.

Table S3. Fossil CO2 EFs derived from Monte Carlo simulation

| Process type | Emission factor (kg f-CO2/kg COD removed) |
| --- | --- |
| AAO | 0.1005 0.0194 |
| OD | 0.0179 0.0083 |
| SBR | 0.0820 0.0142 |
| AO | 0.0959 0.0114 |
| ASP | 0.0217 0.0045 |

Table S4. Fossil CO2 EFs for various wastewater treatment technologies to estimate nationwide fossil CO2 emissions from MWWTPs in China

| No. | Treatment technology | Emission factor (kg fossil CO2/kg CODremoved) | No. | Treatment technology | Emission factor (kg fossil CO2/kg CODremoved) |
| --- | --- | --- | --- | --- | --- |
| 1 | AAO | 0.1005 | 11 | Biological contact oxidation | 0.0636 |  |
| 2 | Reverse AAO | 0.1005 | 12 | Biofilm | 0.0636 |  |
| 3 | AO | 0.0959 | 13 | Aerobic biological treatment | 0.0636 |  |
| 4 | SBR | 0.0820 | 14 | Anaerobic hydrolysis | 0.0636 |  |
| 5 | OD | 0.0179 | 15 | Anaerobic biological treatment | 0.0636 |  |
| 6 | Membrane bio-reactor | 0.0636 | 16 | Biological treatment | 0.0636 |  |
| 7 | Activated sludge | 0.0217 | 17 | Stabilization pond | 0.0636 |  |
| 8 | Biological aerated filter | 0.0636 | 18 | Constructed wetland | 0.0636 |  |
| 9 | Rotating biological contactor | 0.0636 | 19 | Others | 0.0636 |  |
| 10 | Biofilter | 0.0636 | 20 | Unrecognized | 0.0636 |  |

S4. Geographical distribution of fossil CO2 emissions from MWWTPs in China

Figure S4 presents the geographical distribution of fossil CO2 emissions from 8,703 MWWTPs across China in 2020. The size of each point represents the magnitude of fossil CO2 emissions, while the color indicates the specific wastewater treatment process used. The spatial distribution of MWWTPs is highly uneven, with a clear concentration in East China, an area characterized by high population density and significant contributions to national GDP. Notably, the point sizes reveal substantial variation in fossil CO2 emissions among MWWTPs, reflecting differences in treatment scale, technology, and influent characteristics across regions.

Figure S4. Geographic distribution of fossil CO2 emissions from 8,703 MWWTPs across China in 2020. Each dot represents a MWWTP, with dot size indicating the magnitude of fossil CO2 emissions and dot color denoting the type of biological treatment technology employed.

References

1.	Law, Y., Jacobsen, G. E., Smith, A. M., Yuan, Z. & Lant, P. Fossil organic carbon in wastewater and its fate in treatment plants. Water Res. 47, 5270–5281 (2013).

2.	Tseng, L. Y. et al. Identification of preferential paths of fossil carbon within water resource recovery facilities via radiocarbon analysis. Environ. Sci. Technol. 50, 12166–12178 (2016).

3.	Bao, Z., Sun, S. & Sun, D. Assessment of greenhouse gas emission from A/O and SBR wastewater treatment plants in Beijing, China. Int. Biodeterior. Biodegrad. 108, 108–114 (2016).