ADDITIONAL FILE 1

Table S1: Additional example datasets and parameters to consider as constraints or exclusions across Steps 1-3 (in either energy or value mapping, depending on the restrictions, stakeholder preferences, and policy context of a given project area). These datasets and thresholds are derived in part from (Ryberg et al. 2020), however the suggested thresholds outlined here serve as a preliminary guide and should be further adjusted to the regional or local context as appropriate.

| Detailed exclusions | Excludes | Low | Typical | High | Unit |
| --- | --- | --- | --- | --- | --- |
| ECONOMIC |  |  |  |  |  |
| Resource |  |  |  |  |  |
| Wind speed | values below | 4 | 5 | 7 | m/s |
| Global Horizontal Irradiance (GHI) | values below | none | 3 | 5 | kWh/m2 per day |
| Road access | distances above | 45 | 10 | 1 | km |
| Electric grid connection | distances above | 45 | 10 | 1 | km |
| Land value | cost above | * | * | * | * |
| CURRENT INFRASTRUCTURE |  |  |  |  |  |
| Settlements/Buildings | feature + buffer | 0 | 800 | 1000 | m |
| Urban | feature + buffer | 0 | 1 | 3 | km |
| Rural | feature + buffer | 240 | 500 | 2000 | m |
| Roads | feature + buffer | 50 | 150 | 500 | m |
| Major roads | feature + buffer | 50 | 200 | 500 | m |
| Minor roads | feature + buffer | 50 | 100 | 500 | m |
| Airports | feature + buffer | 0 | 5 | 8 | km |
| Large & Commercial airports | feature + buffer | 0 | 5 | 25 | km |
| Airfields | feature + buffer | 0 | 3 | 8 | km |
| Railways | feature + buffer | 50 | 150 | 500 | m |
| Powerlines | feature + buffer | 100 | 200 | 240 | m |
| Mining/industrial areas | feature + buffer | 0 | 100 | 500 | m |
| Power plants | feature + buffer | 100 | 150 | 200 | m |
| PV solar farm | feature + buffer | 0 |  |  | m |
| Wind turbines | feature + buffer | 0 | 300 | 700 | m |
| Gas lines | feature + buffer | 100 | 150 | 300 | m |
| Radio/cell towers (wind only) | feature + buffer | 400 | 500 | 600 | m |
| BIOPHYSICAL |  |  |  |  |  |
| Slope |  |  |  |  |  |
| PV | values above | 1 | 10 | 30 | degrees |
| Wind | values above | 1 | 10 | 30 | degrees |
| Water | feature + buffer | 0 | 300 | 3000 | m |
| Lakes | feature + buffer | 100 | 400 | 4000 | m |
| Rivers | feature + buffer | 100 | 200 | 400 | m |
| Sea coastlines | feature + buffer | 0 | 1 | 3 | km |
| Elevation (wind only) | values above | 1500 | 2000 | 3000 | m |
| Soils |  |  |  |  |  |
| Sandy | feature + buffer | 0 | 1 | 4 | km |
| Landcover |  |  |  |  |  |
| Artificial surfaces (wind & large-scale solar) | feature + buffer | 0 | 0 | 500 | m |
| Woodlands | feature + buffer | 0 | 300 | 1000 | m |
| Wetlands | feature + buffer | 0 | 1 | 3 | km |
| Croplands (PV only) | feature + buffer | 0 | 50 | 240 | m |
| Rock and ice | feature + buffer |  |  |  | m |
| REGULATORY (OTHER) |  |  |  |  |  |
| Protected areas | feature + buffer | 0 | 1 | 3 | km |
| Local zoning restrictions | feature + buffer | Context-dependent |  |  |  |
| Military zones | feature + buffer | Context-dependent |  |  |  |
| ENVIRONMENTAL (see Guidance Framework Step 2 and Table 2 for more detail) |  |  |  |  |  |
| Critical habitat | feature + buffer | Context-dependent |  |  |  |
| Critical migration corridors | feature + buffer | Context-dependent |  |  |  |
| Natura 2000 sites | feature + buffer | Context and often policy-dependent |  |  |  |
| Key Biodiversity Areas | feature + buffer | Context-dependent |  |  |  |
| SOCIAL (see Step 3 and Table 2 for more detail) |  |  |  |  |  |
| Cultural and historic sites | feature + buffer | Context-dependent |  |  |  |
| Spiritual sites | feature + buffer | Context-dependent |  |  |  |
| Recreational areas | feature + buffer | Context-dependent |  |  |  |
| Tourist zones | feature + buffer | Context and often policy-dependent |  |  |  |
| Visually aesthetic landscapes | feature + buffer | Context-dependent |  |  |  |
| Indigenous Peoples and Local Communities (IPLC) lands | feature + buffer | Context and often policy-dependent |  |  |  |
| Population centers or settlements | feature + buffer | Context and often policy-dependent |  |  |  |
| Population density | values above | Context and often policy-dependent |  |  |  |

Table S2: A summary of broad categories for environmental and social elements, types of data to consider and examples of available datasets, and fine-scale indicators or methodological approaches. Note that several data types, such as vistas/landscape aesthetics, may overlap across contributed value groups and are not necessarily mutually exclusive.

| Value elements | Filters | Broad categories | Rationale | Data type(s) |  | Examples and data availability |
| --- | --- | --- | --- | --- | --- | --- |
| ENVIRONMENTAL | Coarse | Vegetation or Landcover | Landcover datasets serve as a valuable proxy for the coarse filter representation of biodiversity in a place. Advances in remote sensing technologies have enabled more frequent updates and improved consistency and accuracy of landcover datasets across large landscape and through time. Landcover datasets capture a wide range of habitats and important natural ecological processes, such as fire regimes and hydrological cycles, that are critical for maintaining long-term ecosystem health and the diversity it supports. | - Land Cover classifications |  | Global Dynamic Land Cover (Copernicus) – an annual global land cover dataset and fractional layers provide land cover information at different classification levels. https://land.copernicus.eu/en/products/global-dynamic-land-cover GLAD Global Land Cover and Land Use Change 2000-2020 - 30m resolution dataset with a focus on quantifying changes in forest extent, cropland, developed lands, surface water, and perennial snow and ice extent (Potapov et al. 2022). https://glad.umd.edu/dataset/GLCLUC2020 Global Forest Change – Forest Cover change from 2000-2023 and percent tree cover dataset (Hansen et al. 2013). https://storage.googleapis.com/earthenginepartners-hansen/GFC-2023-v1.11/download.html Dynamic World – A near real-time 10m land cover dataset tracking 9 land use and cover types (water, trees, grass, crops, shrub & scrub, flooded vegetation, build-up area, bare ground, and snow & ice) (Brown et al. 2022). https://dynamicworld.app/ Habitat maps, Important Plant Areas (IPA) |
|  |  | Ecosystems | Ecosystems are characterized by interactions of living organisms with their abiotic environments and are often represented by proxy datasets such as known aggregation areas or critical habitat areas for important biodiversity assets. Land cover datasets such as those above can also be manipulated to represent important ecological processes e.g., movement of organisms at different trophic levels. | - Protected areas - Important biodiversity areas - Landscape condition |  | Condition: The “Forest Integrity Index” represents forest condition based on observed and inferred human pressures and forest connectivity loss (Grantham et al. 2020). “Free-flowing rivers” provides spatial data on the distribution, condition and connectivity of river reaches (Grill et al. 2019). NATURA 2000 – Protected sites in the European Union designated under the Habitats Directive – Sites of Community Importance (SCIs), and Special Areas of Conservation (SACs). Note, the conservation and access to these areas have social implications given the benefits people derive from wildlife and ecosystem function. https://www.eea.europa.eu/data-and-maps/data/natura-14 Similarly, other protected area datasets or areas of biodiversity significance: World Database on Protected Areas Key Biodiversity Areas (KBA) Important Bird Areas (IBA), flyways Global Hydrology Network |
|  | Fine | Species data | A fine-filter approach focuses on species-level biodiversity elements that are not adequately represented by a coarse-filter approach alone. This may, for example, be because of a species’ special conservation status or specific habitat needs. | - Species occurrences - Species distribution models - Area of Habitat maps | IUCN RedList Species ranges Spatial datasets of the known ranges of more than 166,000 species across several taxonomic groups. These maps often have a coarse spatial resolution and have been criticized for missing local and regional contexts. Attempts to improve the utility of these range maps include the production of Area of Habitat (AOH) maps (Lumbierres et al. 2022), which refine ranges by removing areas, i.e., habitats and elevation gradients, considered unsuitable for species. https://www.iucnredlist.org/resources/spatial-data-download Global Biodiversity Information Facility (GBIF): GBIF is an international network that aggregates species occurrence and distribution data from numerous institutions. Although GBIF datasets require expertise to access, manage and analyze, they remain a critical dataset for understanding and tracking biodiversity change over time. https://www.gbif.org/ Community science datasets – collections of species-level locational and photographic data gathered by the wider public. These data are typically made freely available to support scientific research and policymaking. Although community science datasets can vary in accuracy and reliability and often suffer from sampling bias, they nevertheless represent valuable information that might not otherwise have been collected. Well-known community science datasets include eBird (https://ebird.org/home) and iNaturalist (https://www.inaturalist.org/). |  |
| SOCIAL | Coarse | Cultural | People and communities have connections to places and their contributions or meaning. The cultural values associated with certain landscapes can play an important role in shaping their identities, perceptions, and quality of life. | - Historical significance - Spiritual/religious sites - Recreational areas - Community congregation - Scenic areas - Vistas/landscape aesthetics* |  | Cultural Gems (European Commission Joint Research Center): An interactive web platform to monitor cultural and creative sites in European cities that compiles locations deemed “cultural and creative spaces” reported by individuals, universities, public and private organizations, as well as OpenStreetMaps. https://knowledge4policy.ec.europa.eu/projects-activities/cultural-gems_en Social Media content: Social media content, filtered as relevant, can provide information on the importance of specific places and landscapes. For example, the concentration of photos at specific locations may be further filtered with keywords to identify places of greater aesthetic or recreational value. Methods for data downloading and transformation vary by social media platforms and values of interest. One peer-reviewed example includes a continental-scale dataset for Europe on landscape values (van Zanten et al. 2016), based on the concentration of filtered photos from FlickR, Instagram, or Panoramia for a given time span. Several studies have applied a similar method at smaller scales (Figueroa-Alfaro and Tang 2017, Yoshimura and Hiura 2017, Langemeyer et al. 2018, Havinga et al. 2021). National and subnational datasets, depending on availability: Important Tourism Areas Hunting Areas Locations of community centers National Landscape Character Assessments Landscape aesthetics: Visibility analyses/viewsheds, integrating social media content (see above example) |
|  |  | Economic | Demand for large amounts of land for renewable energy expansion can threaten people’s land rights and access to resources. Economic conditions and demographic distribution provide insight on areas with vulnerable populations, potential risks related to land and resource rights, or opportunities for community engagement and/or benefits sharing. | - Job opportunities - Income - Population density - Remoteness, proximity to population centers - Land tenure - Historically marginalized or vulnerable populations - Planned settlements |  | Population Density: NASA’s “Gridded Population of the World” (CIESIN 2018) spatially represents the number of persons per square kilometer (at 5 years intervals between 2000- 2020). https://www.earthdata.nasa.gov/data/projects/gpw European Commission’s Global Human Settlement layer (GHSL) (Schiavina et al. 2023) comprises the distribution of residential populations (number of people/cell) that is further disaggregated and informed by information on built-up areas (also see other GHSL layers). https://publications.jrc.ec.europa.eu/repository/handle/JRC133256 Road networks: The Global Roads Open Access Data Set (CIESIN 2013) joins road networks between settlements across countries (individual source data varies and ranges from 1980-2010). https://www.earthdata.nasa.gov/data/catalog/sedac-ciesin-sedac-groads-v1-1.0 Global Roads Inventory Project (GRIP) dataset (Meijer et al. 2018) provides current and future road infrastructure. www.globio.info Urbanization: Probability of urban expansion by 2030 (Seto et al. 2012, 2016) at a global scale, representing forecasts of urban land cover change between 2000 to 2030. https://data.nasa.gov/dataset/global-grid-of-probabilities-of-urban-expansion-to-2030 IPLC Lands: Global extent of Indigenous Peoples’ lands based on management and/or tenure rights (Garnett et al. 2018). Locations and size of areas designated as “Territories and areas conserved by Indigenous Peoples and communities” (ICCAs) (UNEP-WCMC 2024), which are reported by communities and documented in the voluntary ICCA registry. https://www.iccaregistry.org/ Vulnerability: NASA’s Global Gridded Relative Deprivation Index (GRDI) (CIESIN 2022) that combines six sociodemographic components to estimate relative levels of deprivation and poverty, including data on built-up areas, child-dependency, infant mortality, the human development index, and intensity of nighttime lights. https://map.unbiodiversitylab.org/ Global, high-resolution estimates of UN Human Development Index (HDI) aggregated for first-level administrative units (e.g., states/provinces) (Sherman et al. 2026) https://hdr.undp.org/data-center/human-development-index#/indicies/HDI National and subnational datasets, depending on availability: Household income Unemployment rate Percent of household income spent on energy costs |
|  |  | Ecological | People often rely on resources from their surrounding environment, through provisioning (i.e., material benefits extracted) and regulating (i.e., material, and nonmaterial benefits from ecosystem processes) ecosystem services. | - Nature parks - Freshwater access - Agricultural or grazing lands, food security - Ecosystem services |  | Agriculture: NASA-funded “Global Food-and-Water Security-support Analysis Data” (GFSAD30) (Oliphant et al. 2022) provides cropland extent, including permanent plantations, lands cultivated and harvested for food, feed, and fiber at least once in a year, and fallows. The dataset also includes crop types (8 globally predominant types), irrigated versus rainfed cropland, cropping intensity, and change in cropland between 2000-2025. Ecosystem Services: “Critical Natural Assets” (Chaplin-Kramer et al. 2023) consists of a global spatial dataset of 14 measures of NCPs, including locally optimized layers that are relevant to RE considerations (e.g., livestock fodder production, access to nature, coral reef tourism). Co$tingNature (Mulligan 2017) is a web-based tool with several spatial datasets documenting natural capital and ecosystem services, including an aggregated measure of “relative total realized bundled services index” that accounts for 16 ecosystem services (e.g., domestic and commercial timber, non-wood forest products, water provisioning, aesthetic quality services, wildlife services and disservices). https://www.policysupport.org/costingnature World Database of Protected Areas (WDPA) (UNEP-WCMC 2024): A global database of protected areas (marine and terrestrial), which also includes locations of UNESCO World Heritage sites. https://www.protectedplanet.net/en/thematic-areas/wdpa?tab=WDPA |
|  | Fine | Participatory approaches to collect this information: Survey-based techniques, focus group discussions, systematic surveys - may be used to assign weights to “coarse” social indicators or identify which social data to include in a suitability analysis Participatory mapping, public participation GIS – data reported by community members in a spatial manner may be used to identify hotspots and clusters of social value, preferences, and other participant feedback. Whenever possible, within the scope of a project, applying a “fine filter” lens for social data can help capture local level information on place-based values and a community’s preferences or priorities. This information is often washed out or inadequately represented in larger scale datasets that serve as proxies. |  |  |  |  |

References

Brown, C. F., S. P. Brumby, B. Guzder-Williams, T. Birch, S. B. Hyde, J. Mazzariello, W. Czerwinski, V. J. Pasquarella, R. Haertel, S. Ilyushchenko, K. Schwehr, M. Weisse, F. Stolle, C. Hanson, O. Guinan, R. Moore, and A. M. Tait. 2022. Dynamic World, Near real-time global 10 m land use land cover mapping. Scientific Data 9:251.

Chaplin-Kramer, R., R. A. Neugarten, R. P. Sharp, P. M. Collins, S. Polasky, D. Hole, R. Schuster, M. Strimas-Mackey, M. Mulligan, C. Brandon, S. Diaz, E. Fluet-Chouinard, L. J. Gorenflo, J. A. Johnson, C. M. Kennedy, P. W. Keys, K. Longley-Wood, P. B. McIntyre, M. Noon, U. Pascual, C. Reidy Liermann, P. R. Roehrdanz, G. Schmidt-Traub, M. R. Shaw, M. Spalding, W. R. Turner, A. Soesbergen, and R. A. Watson. 2023. Mapping the planet’s critical natural assets. Nature Ecology & Evolution 7:51–61.

Figueroa-Alfaro, R. W., and Z. Tang. 2017. Evaluating the aesthetic value of cultural ecosystem services by mapping geo-tagged photographs from social media data on Panoramio and Flickr. Journal of Environmental Planning and Management 60:266–281.

Garnett, S. T., N. D. Burgess, J. E. Fa, Á. Fernández-Llamazares, Z. Molnár, C. J. Robinson, J. E. M. Watson, K. K. Zander, B. Austin, E. S. Brondizio, N. F. Collier, T. Duncan, E. Ellis, H. Geyle, M. V. Jackson, H. Jonas, P. Malmer, B. McGowan, A. Sivongxay, and I. Leiper. 2018. A spatial overview of the global importance of Indigenous lands for conservation. Nature Sustainability 1:369–374.

Grantham, H. S., A. Duncan, T. D. Evans, K. R. Jones, H. L. Beyer, R. Schuster, J. Walston, J. C. Ray, J. G. Robinson, M. Callow, T. Clements, H. M. Costa, A. DeGemmis, P. R. Elsen, J. Ervin, P. Franco, E. Goldman, S. Goetz, A. Hansen, E. Hofsvang, P. Jantz, S. Jupiter, A. Kang, P. Langhammer, W. F. Laurance, S. Lieberman, M. Linkie, Y. Malhi, S. Maxwell, M. Mendez, R. Mittermeier, N. J. Murray, H. Possingham, J. Radachowsky, S. Saatchi, C. Samper, J. Silverman, A. Shapiro, B. Strassburg, T. Stevens, E. Stokes, R. Taylor, T. Tear, R. Tizard, O. Venter, P. Visconti, S. Wang, and J. E. M. Watson. 2020. Anthropogenic modification of forests means only 40% of remaining fore sts have high ecosystem integrity. Nature Communications 11:5978.

Grill, G., B. Lehner, M. Thieme, B. Geenen, D. Tickner, F. Antonelli, S. Babu, P. Borrelli, L. Cheng, H. Crochetiere, H. Ehalt Macedo, R. Filgueiras, M. Goichot, J. Higgins, Z. Hogan, B. Lip, M. E. McClain, J. Meng, M. Mulligan, C. Nilsson, J. D. Olden, J. J. Opperman, P. Petry, C. Reidy Liermann, L. Sáenz, S. Salinas-Rodríguez, P. Schelle, R. J. P. Schmitt, J. Snider, F. Tan, K. Tockner, P. H. Valdujo, A. Soesbergen, and C. Zarfl. 2019. Mapping the world’s free-flowing rivers. Nature 569:215–221.

Hansen, M. C., P. V. Potapov, R. Moore, M. Hancher, S. A. Turubanova, A. Tyukavina, D. Thau, S. V. Stehman, S. J. Goetz, T. R. Loveland, A. Kommareddy, A. Egorov, L. Chini, C. O. Justice, and J. R. G. Townshend. 2013. High-Resolution Global Maps of 21st-Century Forest Cover Change. Science 342:850–853.

Havinga, I., D. Marcos, P. W. Bogaart, L. Hein, and D. Tuia. 2021. Social media and deep learning capture the aesthetic quality of the landscape. Scientific Reports 11:20000.

Langemeyer, J., F. Calcagni, and F. Baró. 2018. Mapping the intangible: Using geolocated social media data to examine landscape aesthetics. Land Use Policy 77:542–552.

Lumbierres, M., P. R. Dahal, C. D. Soria, M. Di Marco, S. H. M. Butchart, P. F. Donald, and C. Rondinini. 2022. Area of Habitat maps for the world’s terrestrial birds and mammals. Scientific Data 9:749.

Meijer, J. R., M. A. J. Huijbregts, K. C. G. J. Schotten, and A. M. Schipper. 2018. Global patterns of current and future road infrastructure. Environmental Research Letters 13:064006.

Mulligan, M. 2017. Co$tingNature Policy Support System [Version 3]. Kings College London.

Oliphant, A., P. Thenkabail, and P. Teluguntla. 2022. Global food-security-support-analysis data at 30-m resolution (GFSAD30 ) cropland-extent products. U.S. Geological Survey (USGS).

Potapov, P., M. C. Hansen, A. Pickens, A. Hernandez-Serna, A. Tyukavina, S. Turubanova, V. Zalles, X. Li, A. Khan, F. Stolle, N. Harris, X.-P. Song, A. Baggett, I. Kommareddy, and A. Kommareddy. 2022. The Global 2000-2020 Land Cover and Land Use Change Dataset Derived From the Landsat Archive: First Results. Frontiers in Remote Sensing 3.

Ryberg, D. S., Z. Tulemat, D. Stolten, and M. Robinius. 2020. Uniformly constrained land eligibility for onshore European wind power. Renewable Energy 146:921–931.

Schiavina, M., M. Melchiorri, M. Pesaresi, P. Politis, F. S. M. Carneiro, L. Maffenini, P. Florio, D. Ehrlich, K. Goch, A. Carioli, J. Uhl, P. Tommasi, and T. Kemper. 2023. GHSL Data Package 2023. https://publications.jrc.ec.europa.eu/repository/handle/JRC133256.

Seto, K. C., B. Güneralp, and L. R. Hutyra. 2012. Global forecasts of urban expansion to 2030 and direct impacts on biodiversity and carbon pools. Proceedings of the National Academy of Sciences 109:16083–16088.

Seto, K. C., B. Güneralp, and L. R. Hutyra. 2016. Global Grid of Probabilities of Urban Expansion to 2030. Palisades, New York: NASA Socioeconomic Data and Applications Center ( SEDAC).

Sherman, L., J. Proctor, H. Druckenmiller, H. Tapia, and S. Hsiang. 2026. Global high-resolution estimates of the UN Human Development Index using satellite imagery and machine learning. Nature Communications 17:1315.

Yoshimura, N., and T. Hiura. 2017. Demand and supply of cultural ecosystem services: Use of geotagged pho tos to map the aesthetic value of landscapes in Hokkaido. Ecosystem Services 24:68–78.

van Zanten, B. T., D. B. Van Berkel, R. K. Meentemeyer, J. W. Smith, K. F. Tieskens, and P. H. Verburg. 2016. Continental-scale quantification of landscape values using social media data. Proceedings of the National Academy of Sciences 113:12974–12979.