Supplementary information: Assessing Low-Carbon Steel Pathways in Europe: A Spatially and Temporally Explicit TEA-LCA Framework

Johannes Kern1, Carsten Gondorf2, Ali Abdelshafy3, and Grit Walther1

1RWTH Aachen University, Chair of Operations Management; Kackertstraße 7, 52072 Aachen (Germany)

2RWTH Aachen University, Department for Industrial Furnaces and Heat Engineering; Kopernikusstraße 10, 52074 Aachen (Germany)

3Delft University of Technology; Energy and Industry Group, Faculty of Technology Policy and Management; Jaffalaan5, 2628 BX Delft (Netherlands)

S1. Hydrogen and renewable electricity modelling

S1.1 Renewable electricity potential and spatial data processing

A Geographical Information System (GIS) framework was developed to generate spatially resolved input data for the quantitative analysis. The availability of wind and solar energy was assessed by integrating raster-based datasets representing renewable resource potentials.

For solar energy, photovoltaic yield data (PVOUT), defined as the electricity output per installed kilowatt-peak capacity (kWh/kWp/day), were obtained from the 1. For wind energy, capacity factor (CF) data for IEC Class 1 turbines were derived from the 2. These datasets provide spatially explicit information on renewable resource availability based on long-term meteorological observations.

The raster datasets were converted into vector-based grid maps with a spatial resolution of 0.1° × 0.1°. Each grid cell represents the average value of all raster pixels contained within its boundaries.

To ensure realistic deployment conditions, areas unsuitable for renewable energy generation, such as urban areas, water bodies, protected areas, and sealed surfaces, were excluded using land-use data derived from OpenStreetMap (OSM). The remaining available area per grid cell is used to determine the installable renewable generation capacity.

Solar capacity factors are calculated as:

CFPV=PVOUT⋅3658760

Wind capacity factors are directly obtained from the dataset:

CFWind=CFdataset

S1.2 Cost calculation framework

The techno-economic calculations follow the framework proposed by 3, which combines capital cost annualization with energy production estimates to derive levelized costs.

The capital recovery factor (CRF) is calculated as:

CRF=i(1+i)n1+i)n-1

Annual energy production (AEP) is defined as:

AEP=Prated⋅CF⋅8760

The levelized cost of electricity (LCOE) is calculated as:

LCOE=CAPEX⋅CRF+OPEXfixAEP+OPEXvar

S1.3 Electricity cost assumptions

Future PV CAPEX developments are modeled using a one-factor learning curve:

CAPEXt=CAPEX2025⋅QtQ2025-b

with:

b=-log⁡2(1-LR)

A learning rate of 18.72% is applied based on 4, which was explicitly developed for renewable electricity technologies. All techno-economic assumptions fir both solar and wind electricity costs are summarized in Table S1 and Table S2.

Table S1 Solar PV assumptions

| Parameter | 2025 | 2030 | 2040 | 2050 | Unit | Source |
| --- | --- | --- | --- | --- | --- | --- |
| CAPEX | 665 | 551 | 479 | 439 | €/kW | 4,5 |
| Fixed OPEX | 1.5 | 1.5 | 1.5 | 1.5 | % CAPEX/a | 6,5 |
| Variable OPEX | 0 | 0 | 0 | 0 | €/kWh | 6,5 |
| Lifetime | 30 | 30 | 30 | 30 | years | 6,5 |
| Interest rate | 3.0 | 3.0 | 3.0 | 3.0 | % | 6,5 |

Table S2 Wind assumptions (IEC Class 1)

| Parameter | 2025 | 2030 | 2040 | 2050 | Unit | Source |
| --- | --- | --- | --- | --- | --- | --- |
| CAPEX | 1034 | 988 | 840 | 782 | €/kW | 7 |
| Fixed OPEX | 21.2 | 20.0 | 18.8 | 18.0 | €/kW/a | 7 |
| Variable OPEX | 0.008 | 0.008 | 0.008 | 0.008 | €/kWh | 5 |
| Lifetime | 25 | 25 | 25 | 25 | years | 5 |
| Interest rate | 3.5 | 3.5 | 3.5 | 3.5 | % | 5 |

S1.4 Hydrogen production modelling

Hydrogen production is modeled via electrolysis using renewable electricity from either solar PV or wind.

According to 8, the electricity demand per kilogram of hydrogen is:

EH2,t=33.3ηt

Annual hydrogen production per kW:

H2,t=8760⋅CFEH2,t

The levelized cost of hydrogen (LCOH) is calculated as:

LCOHt=CAPEXt⋅CRF+OPEXfix,tH2,t+LCOEt⋅EH2,t+Cwater+Cstack,t

S1.5 Electrolyser assumptions

Table S3 Electrolyser assumptions

| Parameter | 2025 | 2030 | 2040 | 2050 | Unit | Source |
| --- | --- | --- | --- | --- | --- | --- |
| CAPEX | 787.5 | 625 | 537.5 | 450 | €/kW | 9,10,11 |
| Efficiency | 65 | 70 | 75 | 80 | % | 9,10 |
| Electricity demand | 51.23 | 47.57 | 44.40 | 41.63 | kWh/kg H₂ | Own calculations |
| Stack lifetime | 40,000 | 60,000 | 100,000 | 140,000 | h | 9 |
| Stack replacement | 20 | 18 | 15 | 12 | % CAPEX | Assumption |
| Fixed OPEX | 6.25 | 5.5 | 5.0 | 4.5 | % CAPEX/a | 12 |
| Lifetime | 25 | 25 | 25 | 25 | years | 9 |
| Interest rate | 7.0 | 7.0 | 7.0 | 7.0 | % | 9 |
| Water cost | 0.03 | 0.03 | 0.03 | 0.03 | €/kg H₂ | Abdelshafy et al. (2024) |

S1.5 Output variables

The model generates the following spatially resolved output variables for each grid cell and year:

Table S4 Variable description with unit.

| Output variable | Description | Unit |
| --- | --- | --- |
| CFPV | Solar PV capacity factor | – |
| CFWind | Wind capacity factor | – |
| LCOEPV,t | Solar PV electricity cost | €/MWh |
| LCOEWind,t | Wind electricity cost | €/MWh |
| LCOHPV,t | Hydrogen cost using PV electricity | €/kg H₂ |
| LCOHWind,t | Hydrogen cost using wind electricity | €/kg H₂ |
| BestTechtLCOE | Least-cost electricity technology | PV / Wind |
| BestTechtLCOH | Least-cost hydrogen technology | PV / Wind |
| H2,t | Annual hydrogen production | t H₂/year |

S1.6 Model validation

The calculated country-level hydrogen production costs were validated against the Global Hydrogen Cost Tool developed by 13. For each country, the median LCOH value of all 0.1° × 0.1° grid cells was determined from our spatial model. These values were compared with the country-specific LCOH reported in the reference dataset for the baseline scenario and low-temperature electrolysis. The resulting country-level comparison for 2025, 2030, 2040, and 2050 is provided in Table S4.

Table S5 Comparison of model-derived median country-level LCOH in €/kg H2 values with minimum country-level LCOH values reported by 13.

|  | 2025 |  |  | 2030 |  |  | 2040 |  |  | 2050 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country | Our model | Validation | MEA (%) | Our model | Validation | MEA (%) | Our model | Validation | MEA (%) | Our model | Validation | MEA (%) |
| Austria | 4,89 | 5,11 | -4,39 | 3,54 | 4,27 | -16,93 | 2,71 | 3,41 | -20,66 | 2,29 | 2,73 | -16,21 |
| Belarus | 4,19 | 6,19 | -32,25 | 3,33 | 5,20 | -35,98 | 2,65 | 4,15 | -36,14 | 2,30 | 3,32 | -30,70 |
| Belgium | 3,31 | 3,77 | -12,25 | 2,63 | 3,37 | -21,99 | 2,10 | 2,85 | -26,42 | 1,83 | 2,45 | -25,31 |
| Bulgaria | 4,56 | 4,69 | -2,58 | 3,26 | 3,91 | -16,48 | 2,48 | 3,13 | -20,81 | 2,08 | 2,50 | -17,02 |
| Croatia | 4,85 | 4,97 | -2,48 | 3,46 | 4,15 | -16,61 | 2,62 | 3,31 | -20,97 | 2,20 | 2,65 | -17,24 |
| Denmark | 2,57 | 3,00 | -14,34 | 2,04 | 2,65 | -23,09 | 1,63 | 2,24 | -27,16 | 1,43 | 1,92 | -25,39 |
| Estonia | 3,70 | 3,84 | -3,58 | 2,94 | 3,40 | -13,72 | 2,34 | 2,88 | -18,54 | 2,04 | 2,46 | -17,17 |
| Finland | 4,31 | 4,72 | -8,57 | 3,43 | 4,21 | -18,60 | 2,72 | 3,37 | -19,08 | 2,37 | 2,69 | -12,19 |
| France | 4,12 | 2,83 | 45,72 | 3,08 | 2,52 | 22,20 | 2,39 | 2,13 | 12,32 | 2,06 | 1,83 | 12,77 |
| Germany | 3,94 | 3,49 | 12,90 | 3,12 | 3,12 | 0,06 | 2,49 | 2,64 | -5,79 | 2,16 | 2,27 | -4,75 |
| Greece | 4,15 | 3,31 | 25,62 | 2,97 | 2,75 | 8,09 | 2,25 | 2,20 | 2,37 | 1,89 | 1,76 | 7,32 |
| Hungary | 4,80 | 5,01 | -4,22 | 3,47 | 4,19 | -17,06 | 2,63 | 3,35 | -21,25 | 2,21 | 2,68 | -17,46 |
| Ireland | 2,48 | 3,89 | -36,30 | 1,97 | 3,44 | -42,73 | 1,58 | 2,91 | -45,70 | 1,38 | 2,45 | -43,62 |
| Italy | 4,34 | 3,47 | 25,09 | 3,10 | 2,88 | 7,67 | 2,36 | 2,31 | 2,04 | 1,98 | 1,85 | 6,96 |
| Moldova | 4,07 | 5,84 | -30,31 | 3,23 | 4,91 | -34,25 | 2,56 | 3,92 | -34,56 | 2,18 | 3,13 | -30,37 |
| Netherlands | 2,88 | 2,86 | 0,69 | 2,29 | 2,55 | -10,01 | 1,83 | 2,15 | -14,94 | 1,60 | 1,84 | -13,24 |
| Norway | 3,64 | 2,71 | 34,45 | 2,89 | 2,42 | 19,55 | 2,30 | 2,04 | 12,70 | 2,00 | 1,75 | 14,24 |
| Poland | 3,77 | 3,57 | 5,74 | 2,99 | 3,20 | -6,35 | 2,39 | 2,70 | -11,81 | 2,08 | 2,33 | -10,82 |
| Portugal | 3,88 | 3,51 | 10,47 | 2,78 | 2,92 | -4,63 | 2,11 | 2,34 | -9,46 | 1,78 | 1,87 | -4,89 |
| Romania | 4,78 | 4,88 | -2,14 | 3,44 | 4,08 | -15,69 | 2,61 | 3,26 | -19,79 | 2,19 | 2,61 | -15,89 |
| Slovakia | 5,08 | 5,23 | -2,88 | 3,65 | 4,36 | -16,31 | 2,77 | 3,49 | -20,54 | 2,33 | 2,79 | -16,63 |
| Slovenia | 4,97 | 4,84 | 2,69 | 3,55 | 4,04 | -12,10 | 2,69 | 3,23 | -16,68 | 2,25 | 2,58 | -12,76 |
| Spain | 3,79 | 3,25 | 16,39 | 2,72 | 2,70 | 0,59 | 2,06 | 2,16 | -4,59 | 1,73 | 1,73 | 0,16 |
| Sweden | 4,03 | 3,52 | 14,25 | 3,20 | 3,15 | 1,47 | 2,55 | 2,66 | -4,48 | 2,21 | 2,29 | -3,39 |
| Switzerland | 4,96 | 5,15 | -3,66 | 3,55 | 4,30 | -17,30 | 2,69 | 3,43 | -21,56 | 2,27 | 2,75 | -17,48 |
| Ukraine | 3,69 | 5,39 | -31,58 | 2,93 | 4,79 | -38,90 | 2,33 | 3,82 | -38,92 | 2,03 | 3,06 | -33,58 |
| United Kingdom | 2,70 | 3,71 | -27,22 | 2,15 | 3,31 | -35,10 | 1,72 | 2,80 | -38,66 | 1,50 | 2,40 | -37,42 |

S2. Producer price index

Table S6 Producer price index in Germany from 2000 to 2025

| Producer Price Index (Germany) | 14 |
| --- | --- |
| Jahr | PPI |
| 2000 | 69.9 |
| 2001 | 72 |
| 2002 | 71.5 |
| 2003 | 72.8 |
| 2004 | 73.9 |
| 2005 | 77.2 |
| 2006 | 81.4 |
| 2007 | 82.4 |
| 2008 | 86.9 |
| 2009 | 83.3 |
| 2010 | 84.6 |
| 2011 | 89 |
| 2012 | 90.4 |
| 2013 | 90.4 |
| 2014 | 89.5 |
| 2015 | 87.9 |
| 2016 | 86.4 |
| 2017 | 88.8 |
| 2018 | 91.1 |
| 2019 | 92.1 |
| 2020 | 91.2 |
| 2021 | 100 |
| 2022 | 129.8 |
| 2023 | 130.1 |
| 2024 | 127.7 |
| 2025 | 126.2 |

S3. Carbon Capture and Storage

Table S7 Techno-economic assumptions for CCS

|  | CO2 avoindance cost [€/tCO2] | Source |
| --- | --- | --- |
| CO2 capture and conditioning | 52.31 | 15,16 |
| CO2 transport via pipeline | 0.01-0.02/km | 17 |
| CO2 storage | 18.8 | 16 |

S4. Substitution of fossil carbon sources with biochar

Table S8  Lower heating value (LHV) of different biochars, coke and hard coal

|  | Lower heating value (MJ/kg) | Sources |
| --- | --- | --- |
| Eucalyptus-based biochar | 25.7 | 18 |
| Poplar-based biochar | 25.41 | 19 |
| Coke | 28 | 20 |
| Hard coal | 29 | 20 |

Table S9  Process-specific substitution ratios of biochar for fossil carbon sources (coke and hard coal) applied in this study

|  | Coke | Hard coal |
| --- | --- | --- |
| Sintering | 50% | - |
| Pelletizing | - | 50% |
| Coke production | - | 5% |
| Blast furnace (BF) | 0% | 100% |
| Electric arc furnace (EAF) | - | 100% |

References

1. Global Solar Atlas. Map and data downloads. https://globalsolaratlas.info/download/world (2025).

2. Global Wind Atlas. GIS files & API access. https://globalwindatlas.info/en/download/gis-files (2025).

3. Rhodes, J., King, C., Gulen, G., Olmstead, S., Dyer, J., Hebner, R., Beach, F., Edgar, T. & Webber, M. A geographically resolved method to estimate levelized power plant costs with environmental externalities. Energy Policy 102, 491–499 (2017).

4. Dai, Y., Shu, Y. & Zeng, Y. Use of learning curve to explore the competition between renewables and conventional fossil fuel power technologies in China. Energy Strategy Reviews 52, 101352 (2024).

5. Kost, C., Shammugam, S., Fluri, V., Peper, D., Memar, A. D. & Schlegl, T. Levelized Cost of Electricity – Renewable Energy Technologies. Fraunhofer Institute for Solar Energy Systems ISE (2021).

6. Erichsen, G., Ball, C., Kather, A. & Kuckshinrichs, W. VEREKON Cost Parameters for 2050 in its Energy System Model. Hamburg University of Technology (TUHH) (2019).

7. Satymov, R., Bogdanov, D. & Breyer, C. Global-local analysis of cost-optimal onshore wind turbine configurations considering wind classes and hub heights. Energy 256, 124629 (2022).

8. Andersson, J. & Grönkvist, S. Large-scale storage of hydrogen. International Journal of Hydrogen Energy 44, 11901–11919 (2019).

9. Abdelshafy, A., Lambert, M. & Walther, G. Mena region as a potential hydrogen supplier for the European market: Analysing a prospective route between Kingdom of Saudi Arabia and Germany. OIES Paper ET31 (The Oxford Institute for Energy Studies, 2024). https://www.oxfordenergy.org/publications/mena-region-as-a-potential-hydrogen-supplier-for-the-european-market-analysing-a-prospective-route-between-kingdom-of-saudi-arabia-and-germany/

10. International Renewable Energy Agency. Green hydrogen cost reduction: Scaling up electrolysers to meet the 1.5°C climate goal (IRENA, 2020).

11. Reksten, A. H., Thomassen, M. S., Møller-Holst, S. & Sundseth, K. Projecting the future cost of PEM and alkaline water electrolysers; a CAPEX model including electrolyser plant size and technology development. International Journal of Hydrogen Energy 47, 38106–38113 (2022).

12. Ali Khan, M. H., Daiyan, R., Han, Z., Hablutzel, M., Haque, N., Amal, R. & MacGill, I. Designing optimal integrated electricity supply configurations for renewable hydrogen generation in Australia. iScience 24, 102539 (2021).

13. Institute of Energy Economics at the University of Cologne (EWI). Global H2 Cost Tool V3 (2021). https://www.ewi.uni-koeln.de/en/tools/schaetzung-der-langfristigen-globalen-versorgungskosten-fuer-kohlenstoffarmen-wasserstoff/

14. Statistisches Bundesamt (Destatis). Erzeugerpreisindex gewerblicher Produkte: Deutschland, Jahre (2026).

15. Roussanaly, S. et al. [Note: exact title/journal for this CO2 capture and conditioning cost benchmark could not be confidently verified via literature search — requires manual check] (2020).

16. Subraveti, S. G., Roussanaly, S., Anantharaman, R., Riboldi, L. & Rajendran, A. How much can novel solid sorbents reduce the cost of post-combustion CO2 capture? A techno-economic investigation on the cost limits of pressure–vacuum swing adsorption. Applied Energy 306, 117955 (2022).

17. Oeuvray, P., Burger, J., Roussanaly, S., Mazzotti, M. & Becattini, V. Multi-criteria assessment of inland and offshore carbon dioxide transport options. Journal of Cleaner Production 443, 140781 (2024).

18. Wallikhani, A. H., Asakereh, A. & Farrokhian Firouzi, A. Biochar and bioenergy production by pyrolysis of Conocarpus and Eucalyptus wastes: a case study, Khuzestan province, Iran. International Journal of Environmental Science and Technology 19, 5839–5848 (2022).

19. Kulas, D., Winjobi, O., Zhou, W. & Shonnard, D. Effects of Coproduct Uses on Environmental and Economic Sustainability of Hydrocarbon Biofuel from One- and Two-Step Pyrolysis of Poplar. ACS Sustainable Chemistry & Engineering 6, 5969–5980 (2018).

20. Meng, F., Rong, G., Zhao, R., Chen, B., Xu, X., Qiu, H., Cao, X. & Zhao, L. Incorporating biochar into fuels system of iron and steel industry: carbon emission reduction potential and economic analysis. Applied Energy 356, 122377 (2024).

Corresponding author: johannes.kern@om.rwth-aachen.de