Title: Influence of Vegetation Type and Hydrology on Short-Term Nutrient Retention in Wetland Mesocosms

Journal: Wetlands

Authors: Spencer G. Womble1,2, Justin N. Murdock2, Shrijana Duwadi1,3, Robert Brown1

Affiliations: 1Tennessee Technological University, School of Environmental Studies, Cookeville, Tennessee, USA

2Tennessee Technological University, Center for the Management, Utilization, and Protection of Water Resources, Tennessee Technological University, Cookeville, Tennessee, USA

Corresponding Author Email: swomble@tntech.edu

Supplemental Equations Additional equations used to convert MIMS signal data into an areal flux rate.

SUP Equation 1:

Gasi conc.µM=Gasi:Arn* Gasi solubility*(Gasi:Ar solubility)Average(Gasi:Ar of the standards)

where Gasi conc. is the concentration of a gas in a sample reported as µM. Gasi:Arn is the gas:Ar ratio (µM for the nth sample. Gasi solubility and Gasi:Ar solubility are the solubilities of Gasi and the Gasi:Ar ratio (µM corrected for the temperature and barometric pressure at which the sample was collected, respectively. Average(Gasi:Ar of the standards) is the average Gasi:Ar concentration (µM ) of the triplicate standards.

SUP Equation 2:

Elemental Gasi conc. µM= Gasi conc. µM*# of atoms

where Elemental Gasi conc. is the concentration of N or O atoms in a sample for each gas compound for N2 and N2O and O2, respectively. Gasi conc. is the concentration µM  of a gas in each sampled.  # of atoms is the number of N or O atoms in each respective gas compound.

SUP Equation 3:

Gasi conc. mg L-1=Elemental Gasi conc. (µM )  *the molecular mass1000

where Gasi conc. is the concentration in mg L-1 of each gas as N or O. Elemental Gasi conc. is the number of N or O atoms in a sample. Dividing by 1,000 converts the values from µM L-1 to mg L-1.

SUP Equation 3:

Aeral Flux mg m-2 h-1=([Core]out-Core]in*Core flowSoil Area

where [Core]out and [Core]in are outflow and inflow concentrations (mg L-1) of N2, O2, and N2O from the soil cores. Coreflow is the flow rate of water through a core (L h-1). Soil Area is the surface area of an individual core (m2). Positive flux rates for N2, O2, and N2O indicate a net gain in the water column (i.e., production), and negative fluxes indicate a net loss (removal) of N2, O2, and N2O from the water within a core.

Nitrous oxide yield, which represents the proportion of N gas produced as N2O (reported as a percentage), for each core was also calculated (SUP Equation 4). Calculating N2O yield can be used to evaluate denitrification efficiency as a high N2O yield can indicate incomplete denitrification (failure of NO3- to be converted completely to N2). Further, examining N2O yield also helps evaluate potential tradeoffs in nutrient removal and greenhouse gas emissions by comparing how increases in N2 production affect N2O production.

SUP Equation 4:

N2O Yield= (N2O (mg L-1)+N2(mg L-1))N2 (mg L-1)*100