LC/MS/MS Method

10 µL of CsD (200 ng/mL) solution used as internal standard was added into 50 µL cell lysate. After vortex mixing for 30 s, 200 µL of acetonitrile were added to the mixture and vortex-mixed for 1 min. Samples were then centrifuged at 14,500 × g for 10 min and supernatant was obtained. CsA concentration in the supernatant was measured using an Agilent 6460A (CA, USA) triple quadrupole LC-MS/MS system with an Agilent 1200 series combined LC system. The Agilent source parameters were a capillary voltage of 4500 V, gas temperature was 180 ℃, gas flow was 13 L/min, nebulizer gas pressure was 18 psi. The autosampler temperature was set at 10 ℃ and the column oven temperature was set at 60 ℃. Analyses were performed with a 3 μm 100 × 2.1 mm Thermo HyPURITY C18 analytical column (MA, USA). The mobile phase consists of 90% methanol and 10% 10 mM ammonium formate buffer (pH 3.5), with a flow of 0.3 mL/min and a run time of 3.0 min. The injection volume was 5 μL.