SUPPORTING INFORMATION

Fig.S1 Structure and selected chemical properties of Sulfamethoxazole (SMX) [1-3]

Fig.S2 Schematic representation of batch reactor (a) and water distribution systems (b is the illustration of WDS model, c and d are real pictures)

Table S1 Characteristics of tap water in Hangzhou, Zhejiang, China

| Water | pH | Temperature (℃) | Dissolved oxygen (DO, mg/L) | Conductivity (μs/cm) | Total organic carbon (TOC, mg/L) | Residual chlorine concentration (mg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| Tap water | 7.11-7.49 | 18.15-21.35 | 8.20-9.63 | 563-607 | 3.087-4.265 | 0.03-0.08 |

Table S2 Sequences of primers for PCR and qPCR.

| Target gene | Primer pairs | 5′→3′sequence |
| --- | --- | --- |
| sul 1 | sul 1 F | CGCACCGGAAACATCGCTGCAC |
|  | sul 1 R | TGAAGTTCCGCCGCAAGGCTCG |
| sul 2 | sul 2 F | TCCGGTGGAGGCCGGTATCTGG |
|  | sul 2 R | CGGGAATGCCATCTGCCTTGAG |
| intI1 | IntI1-F | GGCTTCGTGATCICCTGCTT |
|  | IntI1-R | CATTCCTGGCCGTGGTTCT |
| 16S rDNA | V3-FW(341F) | CCTACGGGAGGCAGCAG |
|  | V3-RV(518R) | ATTACCGCGGCTGCTGG |

The qPCRs were conducted in 96-well plates with a final volume of 20 μL mixture, containing 10μL SYBR Premix Ex Taq (TaKaRa, Bio, Shiga, Japan), 7.2 μL DNA-free water, 2 μL template DNA and 0.2 μM forward and reverse primers. The temperature procedure was as the following: initial denaturation step for 180 s at 95 °C and then 35 cycles of amplification containing denaturation for 30 s at 94 °C, annealing for 30 s at 55 °C and extension for 60 s at 72 °C, with a final extension step for 600 s at 72 °C.

Table S3 First-order rate constants (k1) and goodness of fit (R2) of SMX degradation at different chlorine concentration.

| FC(mg/L) | Batch reactor |  |  | WDS |  |
| --- | --- | --- | --- | --- | --- |
|  | k1(min-1) | R2 |  | k1(min-1) | R2 |
| 0.4 | 0.07394 | 0.9934 |  | 0.07087 | 0.9746 |
| 0.6 | 0.15341 | 0.99709 |  | 0.09473 | 0.98961 |
| 0.8 | 0.26667 | 0.99944 |  | 0.17617 | 0.99431 |
| 1.0 | 0.34836 | 0.9995 |  | 0.24479 | 0.95029 |

Fig. S3 The sediment existed in the stainless steel, PE pipe and ductile iron pipe through the visible pipe section.

Fig. S4 Second order kinetics (k2) plot of SMX degradation in batch reactor(a) and WDS(b).

Table S4 First-order rate constants (k) and goodness of fit (R2) of SMX degradation at different pH.

| Batch reactor |  |  |  | WDS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| pH | k(min-1) | R2 |  | pH | k(min-1) | R2 |
| 6.3 | 0.117 | 0.999 |  | − | − | − |
| 6.6 | 0.132 | 0.974 |  | 6.6 | 0.097 | 0.989 |
| 6.9 | 0.139 | 0.999 |  | 7.0 | 0.107 | 0.973 |
| 7.2 | 0.110 | 0.999 |  | 7.4 | 0.085 | 0.988 |
| 7.5 | 0.092 | 0.992 |  | 7.8 | 0.058 | 0.979 |
| 7.8 | 0.085 | 0.994 |  | 8.2 | 0.045 | 0.970 |
| 8.1 | 0.064 | 0.993 |  | − | − | − |
| 8.4 | 0.050 | 0.999 |  | − | − | − |
| 8.7 | 0.038 | 0.993 |  | − | − | − |
| 9.0 | 0.027 | 0.970 |  | − | − | − |

Fig. S5 Mole fraction of HClO and SMX at different pH

Table S5 First-order rate constants (k), goodness of fit (R2) and Reynolds number (Re) of SMX degradation at different flow velocities in WDS (ductile iron pipe).

| Flow rate(m/s) | WDS |  |  |
| --- | --- | --- | --- |
|  | k(min-1) | R2 | Re |
| 0.7 | 0.129 | 0.986 | 117318 |
| 1.0 | 0.190 | 0.995 | 167597 |
| 1.3 | 0.211 | 0.984 | 217877 |
| 1.5 | 0.201 | 0.994 | 251396 |

Table S6 First-order rate constants (k), goodness of fit (R2) of SMX degradation in different pipe materials

| Pipe materials | WDS |  |
| --- | --- | --- |
|  | k(min-1) | R2 |
| Polyethylene pipe (PE) | 0.139 | 0.992 |
| Stainless steel (SS) | 0.358 | 0.976 |
| Ductile iron (DI) | 0.191 | 0.996 |

REFERENCES

[1] M.C. Dodd, C.H. Huang, Transformation of the antibacterial agent sulfamethoxazole in reactions with chlorine: Kinetics mechanisms, and pathways, Environmental Science & Technology 38(21) (2004) 5607-5615. https://doi.org/10.1021/es035225z.

[2] A. Bialk-Bielinska, S. Stolte, J. Arning, U. Uebers, A. Boschen, P. Stepnowski, M. Matzke, Ecotoxicity evaluation of selected sulfonamides, Chemosphere 85(6) (2011) 928-933. https://doi.org/10.1016/j.chemosphere.2011.06.058.

[3] Y. Yao, Y. Zhang, B. Gao, R.J. Chen, F. Wu, Removal of sulfamethoxazole (SMX) and sulfapyridine (SPY) from aqueous solutions by biochars derived from anaerobically digested bagasse, Environ. Sci. Pollut. Res. 25(26) (2018) 25659-25667. https://doi.org/10.1007/s11356-017-8849-0.