Supplemental Information

The Hidden Material Footprint and Circularity Limits of Spacecraft Expansion

Moisés Gómez, Xianlai Zeng*, and Jinhui Li

State Key Laboratory of Iron and Steel Industry Environmental Protection, School of Environment, Tsinghua University, Beijing 100084, China

* Correspondence: xlzeng@tsinghua.edu.cn

This document includes:

Supplementary Text

Figs. S1 to S20

Tables S1 to S45

Other Supplemental Information for this manuscript includes the following:

Dataset as Excel

Supplementary Text

S1. Global spacecraft growth and future projections	5

S2. Spacecraft end-of-life and environmental challenges	6

S3. Material competition and bottleneck effects	7

S4. Resource competition in a mineral-constrained energy transition	7

S5. Circular governance of spacecraft materials	8

S6. Examples of spacecraft classification	9

List of Figures

Figure S1. Spacecraft mass by year: (A) total mass, (B) average mass, and (C) by orbit.	10

Figure S2. Number of satellites launches: (A) by year, and (B) by orbit.	11

Figure S3. Distribution of electronics mass fractions used in the Monte Carlo simulation (N=10,000). Values were sampled from a triangular distribution (min=10%, mode=20%, max=30%).	25

Figure S4. Monte Carlo (N=10,000) distribution of within-electronics metal composition (wt.%).	30

Figure S5. Specific power-to-weight ratio of solar arrays in spacecraft. The larger red circles represent missions located farther from Earth, corresponding to the red tones on the distance scale. Figure obtained from NASA61.	37

Figure S6.  Distribution of solar-panel mass fractions derived from Monte Carlo sampling (N=10,000). Fractions follow a triangular distribution (20-30%, mode = 25%) to capture differences in spacecraft architectures, orbit profiles, and mission power requirements.	38

Figure S7.  Monte Carlo distribution of spacecraft specific power (W/kg) based on 10,000 simulations.	38

Figure S8.  Sensitivity of cumulative solar-panel metal demand to spacecraft specific power (1957-2023).	46

Figure S9. Probability distribution of battery subsystem mass fractions sampled using a triangular distribution (min=5%, mode=12.5%, max=20%) in a Monte Carlo framework (N=10,000).	49

Figure S10. Ag-Zn battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Ag-Zn battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.	50

Figure S11. Ni-Cd battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Ni-Cd battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.	52

Figure S12. Ni-H2 battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Ni-H2 battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.	53

Figure S13. Li-ion battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Li-ion battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.	56

Figure S14. Histogram of structural mass fractions for spacecraft structure (triangular: 10-15%, mode = 12.5%). These values were sampled 10,000 times to propagate uncertainty in the structural mass budget across spacecraft classes.	63

Figure S15. Distribution of solar-array specific power values (W/kg) sampled in the Monte Carlo simulation (N=10,000). The triangular distribution (20-100 W/kg; mode=30 W/kg) reflects the performance range of crystalline-silicon and multi-junction space solar arrays as reported by NASA (Fig S5).	64

Figure S16.  Metals used in space stations (ISS & TSS).	66

Figure S17.  Histogram of simulated micro-class spacecraft dry masses (N=10,000), based on a triangular distribution with bounds of 0.01–10 kg and a most-likely value of 5 kg.	80

Figure S18.  Histogram of simulated small-class spacecraft dry masses (N=10,000), based on a triangular distribution with bounds of 10–500 kg and a most-likely value of 250 kg.	80

Figure S19.  Histogram of simulated medium-class spacecraft dry masses (N=10,000), based on a triangular distribution with bounds of 500–1,000 kg and a most-likely value of 750 kg.	81

Figure S20.  Histogram of simulated large-class spacecraft dry masses (N = 10,000), based on a triangular distribution with bounds of 1,000–6,000 kg and a most-likely value of 3,500 kg	81

List of Tables

Table S1. Spacecraft launches per year, orbit and average and cumulative mass.	12

Table S2. Total spacecrafts launched by nations from 1957 to 2023.	14

Table S3. Total weight of spacecraft by nations from 1957 to 2023.	16

Table S4. Spacecrafts launched each from 1957 to 2023 by orbit.	19

Table S5. Cumulative metal demand for electronics over 1957-2023. Includes P05, P50, P95 (t) and relative uncertainty (%), defined as (P95-P05)/P50×100.	21

Table S6. Cumulative metal demand for solar panels over 1957-2023. Cumulative totals incorporate uncertainty from subsystem mass fractions, solar W/kg distribution, technology shares, and Dirichlet-sampled composition.	21

Table S7. Cumulative metal demand for batteries (1957-2023), with uncertainty propagated across all chemistry transitions and composition distributions.	22

Table S8. Cumulative structural metal demand (1957-2023).	22

Table S9. Global cumulative metal demand from all spacecraft subsystems combined (1957-2023), with 90% confidence intervals and relative uncertainties.	22

Table S10. Global cumulative metal demand from all spacecraft subsystems combined (1957-2023) and their volume (data of Figure 4).	23

Table S11. Metal intensity in mobile phones’ PCBs.53	24

Table S12. Metal intensity in other PCBs.	25

Table S13. Annual metal demand attributable to spacecraft electronics (1957-2023). Values represent 5th, 50th, and 95th percentiles (kg/year) derived from 10,000 Monte Carlo iterations. Part I.	31

Table S14. Annual metal demand attributable to spacecraft electronics (1957-2023). Values represent 5th, 50th, and 95th percentiles (kg/year) derived from 10,000 Monte Carlo iterations. Part II.	33

Table S15. Metal intensity in solar panels.62	37

Table S16. Solar panels distribution in spacecrafts and its evolution.	39

Table S17. Annual metal demand for spacecraft solar panels (1957-2023). Includes uncertainty from subsystem mass fraction (triangular distribution), solar specific power (20-100 W/kg), and Dirichlet-sampled compositional shares. Values represent P05, P50, and P95 (kg/y). Part I.	41

Table S18. Annual metal demand for spacecraft solar panels (1957-2023). Includes uncertainty from subsystem mass fraction (triangular distribution), solar specific power (20-100 W/kg), and Dirichlet-sampled compositional shares. Values represent P05, P50, and P95 (kg/y). Part I.	43

Table S19. Cumulative metal demand in spacecraft solar panels (1957–2023) under varying specific-power assumptions (20-100 W/kg).	46

Table S20. Cumulative Metal intensity in Ag-Zn batteries.	47

Table S21. Metal intensity in Ni-Cd batteries.	47

Table S22. Metal intensity in Ni-H2 batteries.	48

Table S23. Metal intensity in Li-ion batteries.	48

Table S24. Evolution of battery technologies over time.	57

Table S25. Annual metal demand from all battery chemistries combined (Ag-Zn, Ni-Cd, Ni-H2, Li-ion), weighted by historical adoption. Values are reported as P05, P50, and P95 (kg/y). Part I.	58

Table S26. Annual metal demand from all battery chemistries combined (Ag-Zn, Ni-Cd, Ni-H2, Li-ion), weighted by historical adoption. Values are reported as P05, P50, and P95 (kg/y). Part II.	60

Table S27. Annual structural metal demand across spacecraft (aluminum). Values represent P05-P50-P95 ranges (kg/y).	64

Table S28. Metals used in space stations (ISS & TSS).	66

Table S29. Total metals used in spacecrafts (1957-2023), mining extraction and reserves in 2024.	67

Table S30. Metal reserves. Information obtained from USGS.77	68

Table S31. Annual number of spacecrafts by category (micro, small, medium, and large) and year.	69

Table S32. Annual percentage distribution of spacecraft by category (micro, small, medium, and large) by year.	71

Table S33. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2010-2023.	73

Table S34. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2006-2023.	74

Table S35. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2008-2023.	75

Table S36. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2012-2023.	76

Table S37. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2014-2023.	77

Table S38. Classification of spacecraft by category, function, mass, orbital5,26,50,78.	78

Table S39. Classification of spacecraft by mass (kg).	79

Table S40. Summary statistics of Monte Carlo-derived dry mass distributions for each spacecraft class (N=10,000 samples per class). Mean, median, standard deviation (SD), and percentiles (P05, P25, P75, P95).	79

Table S41. Energy footprint of metals in mining (MJ/kg).	82

Table S42. CO2 footprint of metals in mining (kg CO2-eq/kg)81.	82

Table S43. Water stress (m3/t)	83

Table S44. Rock-to-metal ratio (waste rock + tailings + processing loss).83,84	84

Table S45. ISO 3166-1 alpha-3 country codes corresponding to the countries listed in this study.	85

Supplementary Text

S1. Global spacecraft growth and future projections

Our results show that 18,305 spacecraft were launched between 1957 and 2023, with a cumulative launched mass of 23.7 kt. Independent tallies for 2025 corroborate the scale of activity: the United Nations Office for Outer Space Affairs (UNOOSA) online index lists ~21,289 objects launched into outer space (accessed 20 August 2025);1 the European Space Agency (ESA)’s DISCOSweb live dashboard reports ~22,740 satellites placed into Earth orbit as of 23 July 2025;2 and the U.S. Space Force (USSF) Space-Track catalog records ~22,055 payloads (in orbit plus decayed) as of 20 August 2025.3 Based on our results (Figure 4A), we estimate a cumulative total of ~20,700 ± 3,480 spacecraft launched by 2025. Differences across sources are expected, in part because international registration can lag: the UN Registration Convention requires States to furnish launch and orbital data “as soon as practicable,”4 not on a fixed timetable, and empirical audits document delays from months to years, with occasional cases exceeding a decade.5,6 Recent legal analyses likewise note persistent compliance and process gaps, particularly for rideshare and multi-party missions.7 Counts are further complicated by limited public disclosure for military and defense-related payloads.6 Nevertheless, the close convergence of independent tallies supports the robustness of our dataset compilation and results.

In the near future, the 2030 estimate from our model (25,500 ± 4,700 units) is consistent with the Base scenario of McKinsey & Company, which forecasts approximately 27,000 satellites by that year.8 In comparison, the U.S. Government Accountability Office (GAO) reported approximately 5,500 satellites in orbit as of 2022, with projections for an additional 58,000 launches by 2030, mostly due to commercial operators, low costs, and ambitious mega-constellation deployment plans filed with the US Federal Communications Commission.9 This would result in a total of about 63,500 satellites, predominantly in LEO. Our projection is therefore conservative relative to this higher-growth scenario. We found no primary source projecting satellite numbers to 2040. Notably, we found that several news and even scientific articles state that over 60,000 satellites are expected by 2040; however, this figure originates from the US GAO estimate for 2030 and has been repeatedly misattributed and misused.10,11 Our projections for the future satellite population are consistent with announced deployment plans from major constellation programs, including Starlink (~42,000 satellites),12 Amazon Kuiper (~3,200),13 OneWeb (~6,400),14 and Guowang (~13,000).15

Projections in this study indicate that cumulative spacecraft will reach 112,000 ± 3,800 by 2050, which is about a sixfold increase over the next 25 years. The corresponding cumulative launched mass rises twice, from 23.7 kt (through 2023) to approximately 46 kt by 2050. To our knowledge, no prior peer-reviewed study has quantified the embodied materials of the global spacecraft fleet from the start of the Space Age through mid-century. Most related work instead quantifies the orbital-debris environment and its composition.16 Operational debris models (NASA ORDEM; ESA MASTER) likewise project debris populations and fluxes through 2050 to support risk assessment, but they do not quantify materials embodied in active spacecraft.17,18 While essential for design and safety, these strands do not provide a sector-wide accounting of materials or their evolution by size class and subsystem. By assembling a historical dataset (1957–2023) and projecting to 2050, our analysis fills this gap and reframes spacecraft not only as orbital assets but as a measurable and policy-relevant material system. This level of historical and forward-looking material accounting, capturing size-class evolution, subsystem composition, and technology transitions, provides a degree of resolution not achieved in previous global assessments. Quantifying the embedded materials and metals in this way is a critical step toward understanding the resource implications of space activities, identifying which materials face the greatest pressure, and estimating the scale of resources that are irreversibly lost to space. These insights, in turn, can inform the design of circular-economy strategies, end-of-life management frameworks, and governance interventions aimed at aligning space-sector growth with long-term planetary sustainability.

S2. Spacecraft end-of-life and environmental challenges

Regarding the end-of-life (EoL) of satellites in orbit, there are typically three possible outcomes: controlled or uncontrolled atmospheric reentries, post-mission disposal into graveyard orbits, and non-disposal.19-21 Controlled re-entry is commonly employed for satellites in LEO, using remaining onboard fuel to decelerate the spacecraft and guide it back into Earth’s atmosphere, where it typically disintegrates due to aerodynamic heating. However, larger spacecraft may not fully burn up during re-entry, and surviving debris can reach the Earth's surface, often targeted toward remote regions such as the designated area in the South Pacific Ocean.22 Uncontrolled re-entries occur without active guidance, typically due to fuel exhaustion, system failures, or loss of contact. In some cases, surviving macroscopic fragments can fall anywhere within the latitude band defined by the orbital inclination of the re-entering object, which may include inhabited regions, posing risks to people and property on the ground.21-23 For spacecraft operating in higher orbits, such as MEO and GEO, deorbiting is generally infeasible due to the substantial fuel requirements and associated costs. Instead, a post-mission disposal (PMD) strategy is implemented, whereby spacecraft are maneuvered into a graveyard orbit—a stable orbital region located approximately 36,050 km above the Earth's surface (about 300 km beyond GEO).19,22 While graveyard orbits are widely associated with GEO spacecraft, similar strategies are also recommended and increasingly applied to MEO satellites to minimize collision risks and prevent the generation of space debris.24 The third scenario occurs when a satellite cannot perform either deorbiting or PMD due to system failure, collision, or other anomalies. In this case, the satellite transitions into space debris, remaining indefinitely in orbit and contributing to the growing population of inactive objects. This scenario is of particular concern as it not only reflects a total loss of mission control but also increases the risk of cascade collisions (e.g., Kessler Syndrome).25,26 As the orbital environment becomes increasingly congested, minimizing non-disposed objects is critical for the sustainability of future space operations.

The EoL outcomes for spacecraft (graveyard orbits, uncontrolled abandonment, or atmospheric reentry) result in the permanent loss of their materials. Our analysis shows that spacecraft have already consumed ~23.7 kt of materials between 1957 and 2023, with cumulative demand projected to reach ~46 kt by 2050. Currently, approximately 45% of all spacecraft have either been destroyed upon reentry or stranded in graveyard orbits,2 ensuring that aluminum, copper, lithium, cobalt, REEs, and other metals are irreversibly dissipated into the stratosphere or locked beyond retrieval. When a spacecraft disintegrates upon reentry into the stratosphere, it disperses a multitude of chemicals and metals.27,28 Recent studies indicate that about 10% of stratospheric aerosol particles contain aluminum, copper, lithium, and other metals originating from the burn-up of satellites and rocket stages during re-entry,27 and rocket emissions in the upper atmosphere exhibit nearly 500 times the warming potential of surface or aviation sources.29 With the rapid rise of SmallSat launches, as seen in the last years (Figure 1A–B), it is projected that up to half of stratospheric sulfuric acid particles could incorporate metals within decades, though the implications for stratospheric processes remain uncertain.27 Rocket exhaust further contributes to ozone depletion, circulation changes, ecosystem toxicity, human health risks, and degradation of wildlife and vegetation.30 A recent life-cycle analysis of annual global space activities shows that launch emissions, spacecraft material production, and orbital operations currently produce minimal but measurable impacts on climate forcing, ozone chemistry, and upper-atmospheric heating, these effects are expected to become significant as space activities continue to scale.31 Such atmospheric changes could trigger unforeseen consequences for life on Earth. It is therefore essential to monitor, characterize, quantify spacecraft (as undertaken in this study), and to investigate their effect in greater detail.

S3. Material competition and bottleneck effects

Translating spacecraft metal demand into clean-energy-equivalent deployment further reveals that resource competition is governed not by total material volumes, but by the availability of specific bottleneck elements. Despite the relatively modest aggregate metal demand of spacecraft, the corresponding material requirements could alternatively support on the order of ~13,000 electric vehicles, ~1.4 GW of solar PV capacity, or ~40–45 MW of wind capacity, depending on the technology considered. In all cases, deployment potential is constrained by a single limiting material, lithium for electric vehicles, silicon for solar PV, and zinc or rare earth elements for wind, highlighting that system-level outcomes are dictated by the scarcest components rather than the total metal mix.

This bottleneck-driven behavior has important implications for the energy transition. As emphasized by the IEA, future clean-energy deployment is increasingly dependent on a narrow set of critical minerals, many of which exhibit supply concentration, by-product dependency, and long project development timelines32,33. In such systems, even relatively small additional demands such as those associated with spacecraft expansion, can exert disproportionate marginal pressure on supply chains, particularly for materials like lithium, rare earth elements, and nickel. While spacecraft are unlikely to become a dominant driver of global metal demand, their growing reliance on these strategically important elements suggests that cross-sectoral competition may intensify under sustainability-aligned pathways.

More broadly, these findings emphasize that the sustainability of emerging technologies cannot be assessed solely on the basis of total material consumption. Instead, attention must be directed toward material specificity, supply chain constraints, and the environmental intensity of individual elements. In this context, the expansion of the space sector represents not only a question of scale, but also of composition, where the choice and sourcing of critical materials will play a central role in shaping its long-term environmental and resource implications.

S4. Resource competition in a mineral-constrained energy transition

The transition to low-carbon energy systems is widely recognized as mineral-intensive, requiring unprecedented quantities of metals for renewable energy, electrification, and energy storage technologies.33-36 Crucially, the materials required for spacecraft overlap significantly with those needed for the energy transition. Copper is essential for both spacecraft electronics and electrical grids; lithium, cobalt, and nickel underpin batteries in both satellites and electric vehicles; and rare earth elements are critical for advanced electronics and wind turbines. This overlap creates a shared demand base across sectors that are simultaneously expanding.

While spacecraft demand alone is unlikely to constrain global supply, it becomes significant when considered within a resource-constrained system characterized by rapid demand growth and limited supply flexibility. Critical mineral supply chains are highly concentrated, with a small number of countries dominating production and processing, increasing vulnerability to geopolitical disruptions.32,36,37 Recent analyses show that space infrastructures are tightly coupled to extractive systems, reinforcing existing global inequalities and systemic vulnerabilities.38 At the same time, satellite manufacturing is increasingly exposed to geopolitical concentration, with critical refining and processing capacities, particularly for REEs, gallium, germanium, and specialty alloys, controlled by a limited number of countries.39 Environmental constraints may further limit the expansion of mining, especially in regions with high ecological sensitivity or stringent regulatory frameworks.40

In this context, spacecraft materials constitute an additional demand on mineral systems already under pressure from the low-carbon transition.33,34,41 The importance of this interaction lies not in its absolute magnitude, but in its systemic character: even relatively small increments in demand can exacerbate supply bottlenecks when multiple sectors depend on the same constrained resources. This is particularly relevant for by-product metals such as indium and gallium, where supply is inherently limited by the production of host metals.42 Moreover, the temporal alignment of demand is critical. Both spacecraft deployment and energy-transition technologies are projected to scale rapidly over the same period (2025–2050), increasing the likelihood of synchronized demand peaks. Under sustainability-oriented scenarios that constrain mining expansion, this overlap could intensify supply risks and potentially delay decarbonization efforts.35 These findings reinforce the need to integrate the space sector into critical mineral governance frameworks, which have thus far focused primarily on terrestrial technologies. Failure to account for space-related demand risks underestimating future resource pressures and misaligning policy responses.

S5. Circular governance of spacecraft materials

Efforts to manage the growing burden of space waste have increasingly focused on in-orbit servicing (IOS), including refueling, repairs, debris removal, and recycling. Europe’s Clean Space initiative is targeting its first in-orbit servicing mission for 2028,43 while China’s Shijian-21 has already demonstrated debris towing.44 In contrast, NASA’s OSAM-1 refueling mission was cancelled in 2024 due to cost overruns.45 While such initiatives can extend spacecraft lifespans and reduce debris generation, they do not address the deeper challenge of irreversible material loss from Earth to space. Recent work on the circular space economy highlights that IOS, by itself, is insufficient for closing material loops: current missions are designed primarily for orbital safety rather than material recovery; no operational pathway exists to return materials to Earth; and the energy and logistical requirements of on-orbit recycling remain prohibitive.46 As a result, even an ideal IOS ecosystem would only slow, rather than prevent, the depletion of critical metals embedded in spacecraft.

Our results indicate that by 2050, the cumulative material embodied in spacecraft could more than double relative to today, with no established pathways for recovery or reuse. This future fleet will require several thousand tonnes of metals, dominated by aluminum, copper, and nickel, alongside smaller yet strategically critical quantities of cobalt, lithium, gallium, indium, REEs, and others. Recent assessments reveal that orbital debris already contains metallic stocks of comparable magnitude, particularly aluminium, titanium, steel alloys, and copper from derelict satellites and upper stages.16 Emerging demonstrations of in-space recycling and additive manufacturing show the technical feasibility of converting debris into functional structural components, creating opportunities to partially substitute virgin materials and reduce dependence on launch-intensive supply chains.47,48 However, debris reservoirs are highly uneven: while they hold substantial volumes of high-mass structural metals, they contain only trace amounts of critical and energy-transition materials, which remain scarcely represented in legacy spacecraft. Consequently, in-orbit recycling could realistically contribute to meeting demand for aluminium, steel, titanium, and copper, but cannot offset requirements for the full suite of metals needed for next-generation space systems. A forward-looking governance strategy must therefore integrate material-efficient spacecraft design, targeted debris harvesting, and responsible terrestrial supply-chain management to reduce long-term resource pressures as orbital infrastructure expands.

Addressing the challenge of EoL spacecraft and space debris will require a shift from short-term mitigation toward circular-economy strategies that prioritize reduce, reuse, and recycling at scale.46 Without such measures, the space sector risks replicating the extractive, linear trajectory of e-waste management on Earth— locking away or dissipating vast quantities of critical metals at a time when they are urgently needed for the global energy transition. IOS initiatives, including refueling, repairs, and debris removal, mark an important step,43,46,49 but durable solutions demand deeper knowledge of spacecraft material composition, standardized data infrastructures, and internationally enforceable regulation. The implications extend well beyond aerospace: safeguarding orbital environments intersects directly with climate and energy policy, as unaddressed material losses could intensify pressure on terrestrial supply chains and complicate decarbonization efforts. Embedding material stewardship into space governance is therefore essential not only for sustainable expansion into orbit but also for securing the critical resources required for life on Earth.

S6. Examples of spacecraft classification

Spacecraft design is inherently mission-specific, with mass, dimensions, orbit type, and configuration tailored to fulfill each mission’s objectives. Over the decades, spacecraft mass have drastically changed from small satellites in its origins such as Sputniks (84 kg), Vanguards (1.5 kg), Explorers (14 kg) to the International Space Station (450 t), the biggest spacecraft ever launched, assembled over a decade by 15 nations.50 In recent years, small satellites (SmallSats), spacecraft with masses <500 kg have revolutionized the spacecrafts’ industry.51 Among the SmallSats, CubeSats, satellites built in standard units of 10 cm × 10 cm × 10 cm cubes (called 1U) with a typically weigh of 1—10 kg, have been a disruptive innovation in the satellite industry.52 Originally conceived for educational purposes in 1999, the CubeSat market has experienced exponential growth in recent years due to their compactness, rapid development, modular design, versatility, and cost-effectiveness. Since 2020, over 90% of all launched satellites have been SmallSats, with an average of 1,732 SmallSats and 107 larger satellites during the 2020-2022 period.51 The notable rise in SmallSat numbers can be primarily attributed to the deployment of satellite constellations, a group or network of artificial satellites working together in coordination to achieve specific objectives. Examples include the Global Positioning System (GPS) for navigation, the Iridium constellation for global satellite phone communication, and constellations offering broadband internet services such as Starlink and OneWeb. These constellations have collectively launched 7,57812 and more than 63014 satellites by May 2025, respectively, contributing significantly to the surge in satellite deployments. These figures are expected to increase rapidly with the integration of satellites from Project Kuiper and Starlink in the coming years. Project Kuiper, a competitor to Starlink, envisions the deployment of 3,236 satellites, with the first 27 production satellites launched into space in April 2025.13 Meanwhile, Starlink has ambitious plans to launch 12,000 satellites soon, with the potential to extend this number to a staggering 42,000 satellites.12 This rapid proliferation of satellites is evident, yet the materials embodied in these spacecraft and their broader implications for resource demand and sustainability, remain largely unexamined.

B

A

C

Figure S1. Spacecraft mass by year: (A) total mass, (B) average mass, and (C) by orbit.

A

B

Figure S2. Number of satellites launches: (A) by year, and (B) by orbit.

Table S1. Spacecraft launches per year, orbit and average and cumulative mass.

| Year | LEO | MEO | GEO | HEO | Total annual | Cumulative | Average of mass |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | t | t | t | t | t | kt | Kg |
| 1957 | 0.59 | 0.00 | 0.00 | 0.00 | 0.59 | 0.00 | 197.8 |
| 1958 | 6.75 | 0.01 | 0.00 | 1.20 | 7.96 | 0.01 | 284.2 |
| 1959 | 7.13 | 0.00 | 0.00 | 1.88 | 9.01 | 0.02 | 346.5 |
| 1960 | 41.95 | 0.00 | 0.00 | 2.41 | 44.35 | 0.06 | 1056.0 |
| 1961 | 56.19 | 3.60 | 0.00 | 12.85 | 72.64 | 0.13 | 1274.3 |
| 1962 | 86.63 | 3.80 | 0.00 | 6.24 | 96.67 | 0.23 | 1017.6 |
| 1963 | 58.15 | 6.00 | 0.14 | 5.04 | 69.33 | 0.30 | 779.0 |
| 1964 | 168.67 | 0.00 | 0.07 | 6.11 | 174.85 | 0.48 | 1494.4 |
| 1965 | 220.53 | 0.14 | 0.06 | 14.21 | 234.94 | 0.71 | 1350.2 |
| 1966 | 273.56 | 0.87 | 0.48 | 7.77 | 282.69 | 0.99 | 1734.3 |
| 1967 | 186.11 | 1.59 | 0.81 | 50.01 | 238.52 | 1.23 | 1454.4 |
| 1968 | 263.43 | 1.99 | 0.45 | 94.72 | 360.60 | 1.59 | 2388.0 |
| 1969 | 278.02 | 2.05 | 1.49 | 372.11 | 653.67 | 2.25 | 4539.4 |
| 1970 | 269.96 | 1.65 | 2.65 | 60.51 | 334.77 | 2.58 | 2461.6 |
| 1971 | 309.96 | 1.40 | 4.25 | 200.67 | 516.28 | 3.10 | 3148.0 |
| 1972 | 303.91 | 0.81 | 4.30 | 186.20 | 495.22 | 3.59 | 3588.5 |
| 1973 | 471.53 | 0.35 | 6.56 | 102.25 | 580.69 | 4.17 | 3977.3 |
| 1974 | 369.16 | 0.00 | 13.84 | 11.27 | 394.27 | 4.57 | 2836.5 |
| 1975 | 353.17 | 1.40 | 14.85 | 44.46 | 413.88 | 4.98 | 2554.8 |
| 1976 | 383.84 | 0.41 | 14.62 | 7.10 | 405.97 | 5.39 | 2521.6 |
| 1977 | 376.99 | 1.13 | 18.10 | 4.09 | 400.31 | 5.79 | 2839.1 |
| 1978 | 374.18 | 4.69 | 27.50 | 8.30 | 414.66 | 6.20 | 2468.2 |
| 1979 | 371.72 | 0.34 | 16.44 | 2.53 | 391.04 | 6.59 | 3008.0 |
| 1980 | 374.76 | 0.76 | 29.84 | 2.84 | 408.19 | 7.00 | 2776.8 |
| 1981 | 376.87 | 5.07 | 30.22 | 16.61 | 428.77 | 7.43 | 2663.1 |
| 1982 | 412.38 | 1.42 | 40.05 | 4.94 | 458.78 | 7.89 | 2903.7 |
| 1983 | 363.72 | 7.24 | 35.53 | 17.37 | 423.86 | 8.31 | 2682.6 |
| 1984 | 375.49 | 6.32 | 54.99 | 18.74 | 455.53 | 8.77 | 2711.5 |
| 1985 | 359.19 | 4.54 | 80.62 | 11.96 | 456.30 | 9.22 | 2834.1 |
| 1986 | 355.13 | 1.42 | 37.17 | 9.88 | 403.60 | 9.63 | 2745.6 |
| 1987 | 475.76 | 2.83 | 58.20 | 2.44 | 539.23 | 10.17 | 3964.9 |
| 1988 | 477.43 | 4.25 | 42.79 | 16.30 | 540.77 | 10.71 | 3890.4 |
| 1989 | 304.86 | 17.92 | 70.52 | 13.78 | 407.08 | 11.12 | 3107.5 |
| 1990 | 309.74 | 21.00 | 67.24 | 10.39 | 408.37 | 11.52 | 2505.4 |
| 1991 | 256.03 | 3.28 | 48.78 | 8.28 | 316.37 | 11.84 | 2292.6 |
| 1992 | 216.59 | 14.14 | 59.57 | 8.52 | 298.82 | 12.14 | 2316.4 |
| 1993 | 164.28 | 12.47 | 55.68 | 8.28 | 240.71 | 12.38 | 2208.4 |
| 1994 | 159.27 | 6.24 | 107.55 | 11.99 | 285.05 | 12.66 | 2375.4 |
| 1995 | 158.19 | 4.30 | 84.64 | 13.34 | 260.47 | 12.92 | 2604.7 |
| 1996 | 123.39 | 5.69 | 72.91 | 23.62 | 225.61 | 13.15 | 2278.9 |
| 1997 | 161.62 | 24.06 | 95.02 | 15.58 | 296.28 | 13.45 | 1863.4 |
| 1998 | 152.58 | 4.02 | 88.36 | 5.12 | 250.07 | 13.70 | 1359.1 |
| 1999 | 131.28 | 6.53 | 78.29 | 9.96 | 226.07 | 13.92 | 1603.3 |
| 2000 | 159.23 | 10.75 | 111.77 | 14.69 | 296.44 | 14.22 | 2390.7 |
| 2001 | 151.27 | 14.61 | 67.37 | 5.57 | 238.82 | 14.46 | 2745.0 |
| 2002 | 150.86 | 1.99 | 104.92 | 5.11 | 262.87 | 14.72 | 2796.5 |
| 2003 | 82.72 | 8.93 | 79.04 | 8.39 | 179.08 | 14.90 | 1946.5 |
| 2004 | 91.55 | 16.33 | 63.48 | 7.57 | 178.93 | 15.08 | 2323.8 |
| 2005 | 134.47 | 6.88 | 81.95 | 5.21 | 228.50 | 15.31 | 2967.5 |
| 2006 | 114.20 | 8.30 | 98.58 | 6.68 | 227.76 | 15.54 | 1997.9 |
| 2007 | 142.79 | 14.36 | 89.54 | 2.25 | 248.94 | 15.78 | 2091.9 |
| 2008 | 130.72 | 11.51 | 112.45 | 5.38 | 260.05 | 16.04 | 2476.7 |
| 2009 | 149.31 | 5.71 | 110.84 | 9.22 | 275.09 | 16.32 | 2166.1 |
| 2010 | 160.95 | 15.47 | 111.27 | 9.39 | 297.08 | 16.62 | 2320.9 |
| 2011 | 203.03 | 14.71 | 114.76 | 14.80 | 347.30 | 16.96 | 2535.0 |
| 2012 | 168.52 | 8.64 | 160.73 | 2.00 | 339.89 | 17.30 | 2445.3 |
| 2013 | 194.39 | 10.79 | 121.61 | 4.29 | 331.07 | 17.63 | 1554.3 |
| 2014 | 202.60 | 13.55 | 104.49 | 9.07 | 329.69 | 17.96 | 1106.4 |
| 2015 | 192.77 | 9.12 | 141.96 | 0.63 | 344.48 | 18.31 | 1275.9 |
| 2016 | 185.28 | 13.52 | 131.12 | 1.53 | 331.44 | 18.64 | 1479.7 |
| 2017 | 234.62 | 7.11 | 150.45 | 12.26 | 404.44 | 19.04 | 846.1 |
| 2018 | 225.47 | 30.05 | 112.62 | 11.63 | 379.77 | 19.42 | 820.2 |
| 2019 | 265.81 | 15.48 | 117.34 | 4.33 | 402.96 | 19.83 | 804.3 |
| 2020 | 449.51 | 8.93 | 92.26 | 10.25 | 560.95 | 20.39 | 435.2 |
| 2021 | 651.05 | 3.76 | 110.10 | 17.87 | 782.78 | 21.17 | 415.5 |
| 2022 | 957.37 | 12.13 | 102.00 | 6.20 | 1077.71 | 22.25 | 425.3 |
| 2023 | 1318.70 | 12.83 | 90.53 | 17.39 | 1439.45 | 23.688 | 499.1 |
| TOTAL | 17657.86 | 461.135 | 3945.75 | 1623.572 | 23688.32 |  |  |
| % | 74.54 | 1.95 | 16.66 | 6.85 | 100.00 |  |  |

Table S2. Total spacecrafts launched by nations from 1957 to 2023.

| Nation | Quantity | % |
| --- | --- | --- |
| USA | 10160 | 55.4 |
| Russia | 3868 | 21.1 |
| China | 1086 | 5.9 |
| UK | 763 | 4.2 |
| International | 604 | 3.3 |
| Japan | 336 | 1.8 |
| India | 151 | 0.8 |
| Germany | 128 | 0.7 |
| France | 105 | 0.6 |
| Canada | 102 | 0.6 |
| Italy | 85 | 0.5 |
| Spain | 78 | 0.4 |
| Argentina | 71 | 0.4 |
| South Korea | 66 | 0.4 |
| Israel | 56 | 0.3 |
| Luxembourg | 55 | 0.3 |
| Australia | 48 | 0.3 |
| Saudi Arabia | 32 | 0.2 |
| Brazil | 31 | 0.2 |
| Turkey | 27 | 0.1 |
| Switzerland | 26 | 0.1 |
| Singapore | 25 | 0.1 |
| Indonesia | 24 | 0.1 |
| UAE | 23 | 0.1 |
| Taiwan | 21 | 0.1 |
| Sweden | 19 | 0.1 |
| Iran | 19 | 0.1 |
| Netherlands | 19 | 0.1 |
| Norway | 18 | 0.1 |
| Thailand | 16 | 0.1 |
| Mexico | 16 | 0.1 |
| Denmark | 16 | 0.1 |
| Poland | 14 | 0.1 |
| South Africa | 11 | 0.1 |
| Egypt | 11 | 0.1 |
| Finland | 10 | 0.1 |
| Malaysia | 10 | 0.1 |
| Ukraine | 10 | 0.1 |
| Belgium | 9 | 0.0 |
| Chile | 9 | 0.0 |
| Kazakhstan | 9 | 0.0 |
| North Korea | 8 | 0.0 |
| Greece | 8 | 0.0 |
| Bulgaria | 7 | 0.0 |
| Vietnam | 7 | 0.0 |
| Hungary | 7 | 0.0 |
| Lithuania | 7 | 0.0 |
| Philippines | 7 | 0.0 |
| Czechoslovakia | 6 | 0.0 |
| Czech Republic | 6 | 0.0 |
| Pakistan | 6 | 0.0 |
| Algeria | 5 | 0.0 |
| Nigeria | 5 | 0.0 |
| Romania | 4 | 0.0 |
| New Zealand | 4 | 0.0 |
| Peru | 4 | 0.0 |
| Estonia | 4 | 0.0 |
| Belarus | 4 | 0.0 |
| Ecuador | 3 | 0.0 |
| Austria | 3 | 0.0 |
| Slovenia | 3 | 0.0 |
| Colombia | 3 | 0.0 |
| Venezuela | 3 | 0.0 |
| Oman | 2 | 0.0 |
| Angola | 2 | 0.0 |
| Morocco | 2 | 0.0 |
| Kuwait | 2 | 0.0 |
| Ethiopia | 2 | 0.0 |
| Nepal | 2 | 0.0 |
| Latvia | 1 | 0.0 |
| Uruguay | 1 | 0.0 |
| Turkmenistan | 1 | 0.0 |
| Moldova | 1 | 0.0 |
| Portugal | 1 | 0.0 |
| Monaco | 1 | 0.0 |
| Tunisia | 1 | 0.0 |
| Armenia | 1 | 0.0 |
| Rwanda | 1 | 0.0 |
| Djibouti | 1 | 0.0 |
| Qatar | 1 | 0.0 |
| Myanmar | 1 | 0.0 |
| Azerbaijan | 1 | 0.0 |
| Slovakia | 1 | 0.0 |
| Ireland | 1 | 0.0 |
| Jordan | 1 | 0.0 |
| Bangladesh | 1 | 0.0 |
| Costa Rica | 1 | 0.0 |
| Mauritius | 1 | 0.0 |
| Guatemala | 1 | 0.0 |
| Bolivia | 1 | 0.0 |
| Kenya | 1 | 0.0 |
| Vatican | 1 | 0.0 |
| Sudan | 1 | 0.0 |
| Laos | 1 | 0.0 |
| Total | 18338 | 100 |

Table S3. Total weight of spacecraft by nations from 1957 to 2023.

| Nation | kg | t | % |
| --- | --- | --- | --- |
| Russia | 12307600.0 | 12307.6 | 52.0 |
| USA | 7031395.2 | 7031.4 | 29.7 |
| China | 1483917.8 | 1483.9 | 6.3 |
| International | 1046228.1 | 1046.2 | 4.4 |
| Japan | 456462.7 | 456.5 | 1.9 |
| UK | 164329.7 | 164.3 | 0.7 |
| India | 150774.9 | 150.8 | 0.6 |
| Luxembourg | 130288.0 | 130.3 | 0.6 |
| Italy | 82935.0 | 82.9 | 0.4 |
| Canada | 76896.5 | 76.9 | 0.3 |
| France | 68548.5 | 68.5 | 0.3 |
| Spain | 53099.0 | 53.1 | 0.2 |
| Indonesia | 52028.0 | 52.0 | 0.2 |
| Saudi Arabia | 51511.0 | 51.5 | 0.2 |
| Brazil | 50735.8 | 50.7 | 0.2 |
| South Korea | 45547.4 | 45.5 | 0.2 |
| Germany | 42986.7 | 43.0 | 0.2 |
| Mexico | 39097.6 | 39.1 | 0.2 |
| UAE | 35946.0 | 35.9 | 0.2 |
| Australia | 33283.7 | 33.3 | 0.1 |
| Netherlands | 32252.0 | 32.3 | 0.1 |
| Turkey | 30397.2 | 30.4 | 0.1 |
| Israel | 26319.2 | 26.3 | 0.1 |
| Malaysia | 19165.3 | 19.2 | 0.1 |
| Egypt | 18994.0 | 19.0 | 0.1 |
| Argentina | 16731.8 | 16.7 | 0.1 |
| Thailand | 15094.0 | 15.1 | 0.1 |
| Sweden | 14668.0 | 14.7 | 0.1 |
| Nigeria | 10800.0 | 10.8 | 0.0 |
| Norway | 8651.8 | 8.7 | 0.0 |
| Ukraine | 8363.0 | 8.4 | 0.0 |
| Venezuela | 6929.0 | 6.9 | 0.0 |
| Belarus | 6448.0 | 6.4 | 0.0 |
| Pakistan | 5842.0 | 5.8 | 0.0 |
| Vietnam | 5777.0 | 5.8 | 0.0 |
| Algeria | 5664.0 | 5.7 | 0.0 |
| Qatar | 5300.0 | 5.3 | 0.0 |
| Kazakhstan | 5189.5 | 5.2 | 0.0 |
| Bolivia | 5100.0 | 5.1 | 0.0 |
| Laos | 4000.0 | 4.0 | 0.0 |
| Taiwan | 3907.6 | 3.9 | 0.0 |
| Bulgaria | 3700.0 | 3.7 | 0.0 |
| Bangladesh | 3700.0 | 3.7 | 0.0 |
| Angola | 3611.0 | 3.6 | 0.0 |
| Azerbaijan | 3275.0 | 3.3 | 0.0 |
| Greece | 3255.8 | 3.3 | 0.0 |
| Singapore | 2439.3 | 2.4 | 0.0 |
| Morocco | 2220.0 | 2.2 | 0.0 |
| Philippines | 1910.0 | 1.9 | 0.0 |
| North Korea | 1430.0 | 1.4 | 0.0 |
| Iran | 960.0 | 1.0 | 0.0 |
| Peru | 432.1 | 0.4 | 0.0 |
| South Africa | 339.0 | 0.3 | 0.0 |
| Czechoslovakia | 281.0 | 0.3 | 0.0 |
| Belgium | 257.0 | 0.3 | 0.0 |
| Chile | 243.0 | 0.2 | 0.0 |
| Finland | 166.5 | 0.2 | 0.0 |
| Switzerland | 120.0 | 0.1 | 0.0 |
| Denmark | 97.0 | 0.1 | 0.0 |
| Lithuania | 84.0 | 0.1 | 0.0 |
| Slovenia | 75.0 | 0.1 | 0.0 |
| Sudan | 75.0 | 0.1 | 0.0 |
| Ethiopia | 71.0 | 0.1 | 0.0 |
| Portugal | 50.0 | 0.1 | 0.0 |
| Myanmar | 50.0 | 0.1 | 0.0 |
| Poland | 42.0 | 0.0 | 0.0 |
| New Zealand | 23.3 | 0.0 | 0.0 |
| Latvia | 15.0 | 0.0 | 0.0 |
| Austria | 14.0 | 0.0 | 0.0 |
| Colombia | 10.0 | 0.0 | 0.0 |
| Czech Republic | 9.0 | 0.0 | 0.0 |
| Hungary | 8.3 | 0.0 | 0.0 |
| Estonia | 6.0 | 0.0 | 0.0 |
| Ecuador | 6.0 | 0.0 | 0.0 |
| Oman | 6.0 | 0.0 | 0.0 |
| Kuwait | 3.0 | 0.0 | 0.0 |
| Rwanda | 3.0 | 0.0 | 0.0 |
| Vatican | 3.0 | 0.0 | 0.0 |
| Tunisia | 2.9 | 0.0 | 0.0 |
| Uruguay | 2.0 | 0.0 | 0.0 |
| Ireland | 2.0 | 0.0 | 0.0 |
| Kenya | 2.0 | 0.0 | 0.0 |
| Romania | 1.8 | 0.0 | 0.0 |
| Turkmenistan | 1.0 | 0.0 | 0.0 |
| Moldova | 1.0 | 0.0 | 0.0 |
| Monaco | 1.0 | 0.0 | 0.0 |
| Armenia | 1.0 | 0.0 | 0.0 |
| Djibouti | 1.0 | 0.0 | 0.0 |
| Slovakia | 1.0 | 0.0 | 0.0 |
| Jordan | 1.0 | 0.0 | 0.0 |
| Costa Rica | 1.0 | 0.0 | 0.0 |
| Mauritius | 1.0 | 0.0 | 0.0 |
| Guatemala | 1.0 | 0.0 | 0.0 |
| Nepal | 0.5 | 0.0 | 0.0 |
| Total | 23688216.2 | 23688.2 | 100.0 |

Table S4. Spacecrafts launched each from 1957 to 2023 by orbit.

| Year | GEO | HEO | LEO | MEO | Total |
| --- | --- | --- | --- | --- | --- |
| 2023 | 27 | 7 | 2921 | 8 | 2963 |
| 2022 | 29 | 13 | 2504 | 8 | 2554 |
| 2021 | 31 | 6 | 1847 | 5 | 1889 |
| 2020 | 24 | 4 | 1277 | 7 | 1312 |
| 2019 | 27 | 3 | 464 | 16 | 510 |
| 2018 | 26 | 10 | 417 | 31 | 484 |
| 2017 | 37 | 4 | 440 | 8 | 489 |
| 2016 | 34 | 1 | 180 | 12 | 227 |
| 2015 | 37 | 2 | 225 | 8 | 272 |
| 2014 | 31 | 7 | 255 | 9 | 302 |
| 2013 | 27 | 4 | 175 | 10 | 216 |
| 2012 | 40 | 1 | 92 | 9 | 142 |
| 2011 | 30 | 8 | 89 | 13 | 140 |
| 2010 | 33 | 8 | 78 | 11 | 130 |
| 2009 | 31 | 5 | 89 | 5 | 130 |
| 2008 | 32 | 3 | 61 | 9 | 105 |
| 2007 | 27 | 8 | 76 | 8 | 119 |
| 2006 | 32 | 3 | 74 | 5 | 114 |
| 2005 | 22 | 4 | 47 | 5 | 78 |
| 2004 | 20 | 4 | 46 | 7 | 77 |
| 2003 | 27 | 9 | 51 | 5 | 92 |
| 2002 | 33 | 2 | 55 | 4 | 94 |
| 2001 | 23 | 5 | 51 | 8 | 87 |
| 2000 | 40 | 7 | 69 | 9 | 125 |
| 1999 | 28 | 5 | 106 | 2 | 141 |
| 1998 | 33 | 6 | 143 | 2 | 184 |
| 1997 | 38 | 6 | 110 | 5 | 159 |
| 1996 | 33 | 14 | 47 | 6 | 100 |
| 1995 | 32 | 5 | 60 | 4 | 101 |
| 1994 | 36 | 5 | 74 | 6 | 121 |
| 1993 | 25 | 5 | 71 | 9 | 110 |
| 1992 | 30 | 5 | 87 | 9 | 131 |
| 1991 | 27 | 5 | 105 | 4 | 141 |
| 1990 | 39 | 7 | 105 | 14 | 165 |
| 1989 | 26 | 8 | 87 | 14 | 135 |
| 1988 | 29 | 10 | 99 | 3 | 141 |
| 1987 | 20 | 2 | 115 | 2 | 139 |
| 1986 | 21 | 6 | 121 | 1 | 149 |
| 1985 | 37 | 9 | 111 | 4 | 161 |
| 1984 | 37 | 11 | 112 | 9 | 169 |
| 1983 | 25 | 8 | 120 | 5 | 158 |
| 1982 | 28 | 3 | 127 | 1 | 159 |
| 1981 | 21 | 7 | 128 | 5 | 161 |
| 1980 | 16 | 3 | 127 | 1 | 147 |
| 1979 | 11 | 2 | 116 | 1 | 130 |
| 1978 | 22 | 8 | 133 | 5 | 168 |
| 1977 | 19 | 4 | 117 | 2 | 142 |
| 1976 | 14 | 4 | 142 | 1 | 161 |
| 1975 | 13 | 7 | 141 | 2 | 163 |
| 1974 | 11 | 3 | 125 |  | 139 |
| 1973 | 6 | 11 | 128 | 1 | 146 |
| 1972 | 4 | 11 | 121 | 2 | 138 |
| 1971 | 5 | 15 | 144 | 2 | 166 |
| 1970 | 7 | 9 | 121 | 2 | 139 |
| 1969 | 5 | 25 | 115 | 4 | 149 |
| 1968 | 2 | 7 | 134 | 14 | 157 |
| 1967 | 4 | 15 | 133 | 20 | 172 |
| 1966 | 2 | 9 | 139 | 19 | 169 |
| 1965 | 1 | 13 | 157 | 4 | 175 |
| 1964 | 1 | 9 | 113 |  | 123 |
| 1963 | 2 | 5 | 80 | 5 | 92 |
| 1962 |  | 9 | 86 | 2 | 97 |
| 1961 |  | 2 | 53 | 2 | 57 |
| 1960 |  | 7 | 35 |  | 42 |
| 1959 |  | 7 | 19 |  | 26 |
| 1958 |  | 7 | 15 | 6 | 28 |
| 1957 |  |  | 3 |  | 3 |
| Total | 1430 | 447 | 16008 | 420 | 18305 |

Table S5. Cumulative metal demand for electronics over 1957-2023. Includes P05, P50, P95 (t) and relative uncertainty (%), defined as (P95-P05)/P50×100.

| Metal | Cum_P05 (t) | Cum_P50 (t) | Cum_P95 (t) | Rel_Unc_% |
| --- | --- | --- | --- | --- |
| Ag | 3.38 | 3.89 | 4.46 | 27.80 |
| Al | 70.37 | 72.99 | 75.79 | 7.43 |
| Au | 1.48 | 1.83 | 2.23 | 40.61 |
| Cu | 806.97 | 823.97 | 841.13 | 4.15 |
| Fe | 36.68 | 38.47 | 40.35 | 9.55 |
| Ni | 56.82 | 59.20 | 61.66 | 8.19 |
| Pd | 0.17 | 0.29 | 0.46 | 101.98 |
| Pt | 0.04 | 0.10 | 0.22 | 170.33 |
| REEs | 31.17 | 32.81 | 34.52 | 10.23 |
| Sn | 42.41 | 44.37 | 46.48 | 9.16 |
| Ti | 9.96 | 10.85 | 11.82 | 17.17 |
| Zn | 13.72 | 14.79 | 15.89 | 14.68 |

Table S6. Cumulative metal demand for solar panels over 1957-2023. Cumulative totals incorporate uncertainty from subsystem mass fractions, solar W/kg distribution, technology shares, and Dirichlet-sampled composition.

| Metal | Cum_P05 (t) | Cum_P50 (t) | Cum_P95 (t) | Rel_Unc_% |
| --- | --- | --- | --- | --- |
| Ag | 1.20 | 2.69 | 5.79 | 170.72 |
| Al | 15.74 | 28.64 | 51.88 | 126.19 |
| As | 20.93 | 37.39 | 66.72 | 122.46 |
| Cu | 117.66 | 210.40 | 373.08 | 121.40 |
| Ga | 3.95 | 7.39 | 13.51 | 129.48 |
| In | 1.01 | 2.07 | 4.17 | 152.96 |
| Pb | 7.26 | 13.60 | 24.88 | 129.52 |
| Si | 629.90 | 1126.94 | 1990.69 | 120.75 |
| Sn | 7.05 | 13.06 | 23.87 | 128.83 |
| Zn | 0.00 | 0.00 | 0.03 | 5983.67 |

Table S7. Cumulative metal demand for batteries (1957-2023), with uncertainty propagated across all chemistry transitions and composition distributions.

| Metal | Cum_P05 (t) | Cum_P50 (t) | Cum_P95 (t) | Rel_Unc_% |
| --- | --- | --- | --- | --- |
| Ag | 41.55 | 51.42 | 62.69 | 41.11 |
| Al | 66.69 | 101.06 | 150.42 | 82.84 |
| Cd | 314.03 | 351.39 | 391.13 | 21.94 |
| Co | 50.47 | 75.76 | 115.27 | 85.53 |
| Cu | 53.59 | 84.38 | 130.31 | 90.93 |
| Fe | 84.14 | 123.45 | 177.11 | 75.31 |
| Li | 8.51 | 20.63 | 46.80 | 185.66 |
| Ni | 808.63 | 876.73 | 952.14 | 16.37 |
| Zn | 156.84 | 180.76 | 205.59 | 26.97 |

Table S8. Cumulative structural metal demand (1957-2023).

| Metal | Cum_P05 (t) | Cum_P50 (t) | Cum_P95 (t) | Rel_Unc_% |
| --- | --- | --- | --- | --- |
| Al | 2904.21 | 2960.49 | 3017.79 | 3.84 |

Table S9. Global cumulative metal demand from all spacecraft subsystems combined (1957-2023), with 90% confidence intervals and relative uncertainties.

| Metal | Cum_P05 (t) | Cum_P50 (t) | Cum_P95 (t) | Rel_Unc_% |
| --- | --- | --- | --- | --- |
| Ag | 48.14 | 58.29 | 69.94 | 37.39 |
| Al | 3094.24 | 3167.59 | 3243.14 | 4.70 |
| As | 20.93 | 37.39 | 66.72 | 122.46 |
| Au | 1.48 | 1.83 | 2.23 | 40.61 |
| Cd | 314.03 | 351.39 | 391.13 | 21.94 |
| Co | 50.47 | 75.76 | 115.27 | 85.53 |
| Cu | 1018.62 | 1123.16 | 1288.76 | 24.05 |
| Fe | 122.73 | 161.82 | 215.50 | 57.33 |
| Ga | 3.95 | 7.39 | 13.51 | 129.48 |
| In | 1.01 | 2.07 | 4.17 | 152.96 |
| Li | 8.51 | 20.63 | 46.80 | 185.66 |
| Ni | 867.75 | 935.98 | 1011.44 | 15.35 |
| Pb | 7.26 | 13.60 | 24.88 | 129.52 |
| Pd | 0.17 | 0.29 | 0.46 | 101.98 |
| Pt | 0.04 | 0.10 | 0.22 | 170.33 |
| REEs | 31.17 | 32.81 | 34.52 | 10.23 |
| Si | 629.90 | 1126.94 | 1990.69 | 120.75 |
| Sn | 50.94 | 57.47 | 68.46 | 30.48 |
| Ti | 9.96 | 10.85 | 11.82 | 17.17 |
| Zn | 171.72 | 195.58 | 220.32 | 24.85 |
| Total | 6453.03 | 7380.91 | 8819.98 | 32.07 |

Table S10. Global cumulative metal demand from all spacecraft subsystems combined (1957-2023) and their volume (data of Figure 4).

| Metals ordered based on their respective weight |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cum_P05 | Cum_P50 | Cum_P95 | Density | Volume | Cube edge |
|  | t | t | t | Kg/m3 | m3 | m |
| Al | 3136.36 | 3211.27 | 3288.50 | 2700.0 | 1189.36 | 10.60 |
| Si | 629.90 | 1126.94 | 1990.69 | 2330.0 | 483.67 | 7.85 |
| Cu | 1501.58 | 1616.29 | 1792.17 | 8850.0 | 182.63 | 5.67 |
| Ni | 901.75 | 971.41 | 1048.34 | 8908.0 | 109.05 | 4.78 |
| Cd | 314.03 | 351.39 | 391.13 | 8650.0 | 40.62 | 3.44 |
| Zn | 179.93 | 204.43 | 229.83 | 7133.0 | 28.66 | 3.06 |
| Fe | 144.68 | 184.85 | 239.65 | 7800.0 | 23.70 | 2.87 |
| Co | 50.47 | 75.76 | 115.27 | 8860.0 | 8.55 | 2.04 |
| Ag | 50.17 | 60.61 | 72.61 | 10490.0 | 5.78 | 1.79 |
| Sn | 76.32 | 84.02 | 96.27 | 7260.0 | 11.57 | 2.26 |
| As | 20.93 | 37.39 | 66.72 | 5778.0 | 6.47 | 1.86 |
| REEs | 49.82 | 52.44 | 55.19 | 9066.0 | 5.78 | 1.80 |
| Li | 8.51 | 20.63 | 46.80 | 535.0 | 38.56 | 3.38 |
| Pb | 7.26 | 13.60 | 24.88 | 11400.0 | 1.19 | 1.06 |
| Ti | 15.91 | 17.34 | 18.89 | 4540.0 | 3.82 | 1.56 |
| Ga | 3.95 | 7.39 | 13.51 | 5907.0 | 1.25 | 1.08 |
| In | 1.01 | 2.07 | 4.17 | 7310.0 | 0.28 | 0.66 |
| Au | 2.37 | 2.92 | 3.56 | 19300.0 | 0.15 | 0.53 |
| Pd | 0.27 | 0.46 | 0.74 | 12023.0 | 0.04 | 0.34 |
| Pt | 0.07 | 0.16 | 0.35 | 21500.0 | 0.01 | 0.20 |
| Total | 7095.31 | 8041.37 | 9499.28 | - | 2141.15 | 56.83 |

*: The density for the REE mixture was approximated using the density of erbium, which accounts for more than 80% of the total REE content in electronics (Table S11).

Electronics in spacecraft

Table S11. Metal intensity in mobile phones’ PCBs.53

|  | G1 | G2 | G3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Metals in wt.% |  |  | Average |  | SD |
| Cu | 21.8 | 29.5 | 32.2 | 27.8 |  | 5.40 |
| Al | 2.2 | 2.9 | 2.3 | 2.5 |  | 0.38 |
| Ni | 1.1 | 1.7 | 3.2 | 2.0 |  | 1.08 |
| Sn | 0.2 | 1.2 | 3.1 | 1.5 |  | 1.47 |
| Ti | 0.3 | 0.3 | 0.5 | 0.4 |  | 0.12 |
| Zn | 0.2 | 0.5 | 0.8 | 0.5 |  | 0.30 |
| Fe | 0.5 | 0.9 | 2.5 | 1.3 |  | 1.06 |
|  | Metals in ppm |  |  |  |  |  |
| Ag | 1095.1 | 1246.7 | 1611.1 | 1317.6 |  | 265.21 |
| Au | 502.1 | 588.2 | 765.4 | 618.6 |  | 134.25 |
| Pd | 106.7 | 109.1 | 85.8 | 100.5 |  | 12.82 |
| Pt | 26.4 | 29.4 | 59.2 | 38.3 |  | 18.13 |
|  | Metals in ppm |  |  | ppm | % |  |
| Sc | 3.8 | 4.9 | 5.1 | 4.6 | 0.04 |  |
| La | 439.5 | 61 | 4.7 | 168.4 | 1.52 |  |
| Ce | 9.9 | 2.5 | <DL | 6.2 | 0.06 |  |
| Pr | 130 | 114.9 | 64.8 | 103.2 | 0.93 |  |
| Nd | 1657.5 | 1627.9 | 982.6 | 1422.7 | 12.83 |  |
| Sm | 20.8 | 18.4 | 12.1 | 17.1 | 0.15 |  |
| Eu | 2.3 | 2.1 | 0.7 | 1.7 | 0.02 |  |
| Y | 28.7 | 17.1 | 20.1 | 22.0 | 0.20 |  |
| Gd | 9.5 | 56.3 | 45.9 | 37.2 | 0.34 |  |
| Tb | 10 | 50.3 | 3.7 | 21.3 | 0.19 |  |
| Dy | 46.4 | 91.1 | 125.7 | 87.7 | 0.79 |  |
| Ho | 13.5 | 4.2 | <DL | 8.9 | 0.08 |  |
| Er | 7857.8 | 7977.6 | 11731.1 | 9188.8 | 82.85 |  |
| Tm | <DL | <DL | <DL | - | - |  |
| Yb | 0.6 | 0.5 | 1.4 | 0.8 | 0.01 |  |
| Lu | 0.4 | 0.1 | 1 | 0.5 | 0.00 |  |
| Total REEs |  |  |  | 11091.2 | 100.0 | 265.21 |

Table S12. Metal intensity in other PCBs.

| Element (wt.%) | References |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 54 | 55 |  | 56 | 57 | 58 | 59 | 60 | 53 |
|  | 2007 | 2011 | 2011 | 2015 | 2017 | 2019 | 2019 | 2020 | 2023 |
| Al | 1.83 | 0.31 | 0.99 | - | - | 0.17 | - | 1.6 | 2.47 |
| Ba/ppm | 0.11 | - | - | - | - | - | - | - | 2,300 |
| Cd | <0.56 | - | - | - | - | - | - | - | 0.0034 |
| Cu | 32.31 | 39.56 | 38.33 | 40.8 | 41.8 | 57.47 | 47.9 | 30 | 27.93 |
| Fe | 0.54 | 1.42 | 6.53 | 0.28 | 0.66 | 25.23 | 0.5 | 4.7 | 1.3 |
| Mn | 0.65 | - | - | - | - | 0.24 | - | - | 0.63 |
| Ni | 0.69 | 3.42 | 1.67 | 0.39 | 1.93 | 5.1 | 0.8 | 2.1 | 2.03 |
| Pb | 0.25 | 1.17 | 1.26 | 1.36 | 0.19 | 0.85 | - | 1.3 | 5.11 |
| Sn | - | 2.09 | 3.11 | 1.6 | 4.57 | 3.49 | 2 | - | 1.51 |
| Zn | 0.96 | 3.43 | 0.97 | 0.41 | - | 4.9 | - | 0.9 | 0.52 |
| Ag | 0.41 | 0.06 | 0.06 | 0.11 | 0.2 | 0.15 | 0.13 | - | 1.32 |
| Au | 0.28 | 0.06 | 0.1 | 0.0065 | 0.12 | 0.12 | 0.1 | 0.1 | 0.62 |

Figure S3. Distribution of electronics mass fractions used in the Monte Carlo simulation (N=10,000). Values were sampled from a triangular distribution (min=10%, mode=20%, max=30%).

Metal mass shares for electronics were sampled using Dirichlet distributions parameterized by literature-derived mean concentrations and standard deviations (Tables S11-S12). The Dirichlet formulation preserves the constant-sum constraint (100 wt.%) across all iterations while allowing realistic covariance among metals. Histograms show the resulting uncertainty ranges for Cu, Al, Fe, Au, Ag, Pd, Sn, Pb, Ni, Ta, and rare earth elements. Median values, interquartile ranges, and full simulation envelopes are displayed for each metal.

Figure S4. Monte Carlo (N=10,000) distribution of within-electronics metal composition (wt.%).

Table S13. Annual metal demand attributable to spacecraft electronics (1957-2023). Values represent 5th, 50th, and 95th percentiles (kg/year) derived from 10,000 Monte Carlo iterations. Part I.

| Year | Ag_P05 | Ag_P50 | Ag_P95 | Al_P05 | Al_P50 | Al_P95 | Au_P05 | Au_P50 | Au_P95 | Cu_P05 | Cu_P50 | Cu_P95 | Fe_P05 | Fe_P50 | Fe_P95 | Ni_P05 | Ni_P50 | Ni_P95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 0.04 | 0.14 | 0.34 | 2.20 | 2.88 | 3.70 | 0.01 | 0.06 | 0.20 | 28.05 | 32.75 | 37.70 | 1.07 | 1.51 | 2.08 | 1.75 | 2.33 | 3.04 |
| 1958 | 0.57 | 1.86 | 4.44 | 29.69 | 38.87 | 49.90 | 0.10 | 0.75 | 2.68 | 377.56 | 441.75 | 508.76 | 14.23 | 20.34 | 27.97 | 23.59 | 31.45 | 41.13 |
| 1959 | 0.61 | 2.08 | 5.04 | 33.64 | 44.01 | 56.52 | 0.11 | 0.85 | 2.96 | 428.20 | 500.46 | 574.54 | 16.13 | 23.03 | 31.80 | 26.67 | 35.58 | 46.58 |
| 1960 | 3.00 | 10.34 | 24.62 | 165.08 | 215.95 | 278.59 | 0.56 | 4.20 | 14.83 | 2108.24 | 2458.31 | 2829.86 | 79.63 | 113.44 | 155.68 | 131.20 | 174.83 | 229.72 |
| 1961 | 4.97 | 16.93 | 40.15 | 272.70 | 355.61 | 454.54 | 0.88 | 6.79 | 24.19 | 3446.70 | 4029.10 | 4637.69 | 131.08 | 185.32 | 257.07 | 214.64 | 286.87 | 374.16 |
| 1962 | 6.76 | 22.70 | 54.18 | 362.13 | 471.92 | 606.67 | 1.17 | 9.06 | 32.73 | 4594.95 | 5368.77 | 6184.42 | 173.79 | 246.89 | 340.25 | 285.23 | 381.33 | 502.78 |
| 1963 | 4.83 | 16.33 | 39.22 | 258.92 | 339.99 | 432.53 | 0.86 | 6.59 | 23.39 | 3292.35 | 3852.69 | 4425.76 | 124.76 | 176.87 | 243.65 | 204.26 | 273.36 | 359.09 |
| 1964 | 12.15 | 40.36 | 97.90 | 655.54 | 854.94 | 1097.95 | 2.10 | 16.32 | 58.48 | 8351.60 | 9744.21 | 11169.62 | 315.95 | 446.75 | 615.93 | 518.21 | 691.97 | 905.00 |
| 1965 | 15.93 | 54.20 | 130.16 | 878.14 | 1147.44 | 1481.42 | 2.93 | 22.54 | 80.53 | 11166.40 | 13049.53 | 14987.24 | 424.48 | 601.37 | 824.49 | 697.23 | 928.45 | 1219.45 |
| 1966 | 19.87 | 65.30 | 159.73 | 1053.85 | 1372.63 | 1759.42 | 3.48 | 26.76 | 94.12 | 13414.82 | 15649.85 | 17948.22 | 506.54 | 718.71 | 992.27 | 835.15 | 1112.76 | 1460.47 |
| 1967 | 16.60 | 54.96 | 134.50 | 888.03 | 1163.76 | 1493.28 | 2.99 | 22.90 | 81.17 | 11361.20 | 13257.13 | 15241.24 | 432.88 | 611.03 | 839.84 | 701.38 | 943.98 | 1247.35 |
| 1968 | 25.26 | 85.24 | 203.92 | 1345.09 | 1755.26 | 2251.46 | 4.78 | 34.41 | 120.07 | 17123.29 | 20004.33 | 23034.89 | 648.86 | 920.62 | 1269.69 | 1062.45 | 1424.11 | 1862.93 |
| 1969 | 45.37 | 152.42 | 367.00 | 2446.12 | 3199.32 | 4094.87 | 8.45 | 61.49 | 219.69 | 30972.80 | 36254.16 | 41742.15 | 1171.90 | 1675.30 | 2310.53 | 1935.99 | 2583.02 | 3391.83 |
| 1970 | 23.03 | 78.68 | 192.77 | 1256.49 | 1638.69 | 2097.51 | 4.16 | 32.20 | 112.21 | 15948.47 | 18627.10 | 21378.89 | 608.52 | 856.73 | 1176.18 | 990.02 | 1325.62 | 1738.54 |
| 1971 | 34.86 | 119.43 | 290.61 | 1932.52 | 2521.22 | 3236.76 | 6.23 | 50.70 | 178.17 | 24551.10 | 28690.62 | 32969.23 | 929.98 | 1315.00 | 1815.97 | 1524.99 | 2032.51 | 2677.80 |
| 1972 | 33.20 | 116.25 | 279.87 | 1840.69 | 2426.50 | 3113.75 | 5.88 | 46.22 | 163.18 | 23509.04 | 27515.06 | 31585.99 | 894.02 | 1266.11 | 1742.16 | 1462.47 | 1961.86 | 2562.77 |
| 1973 | 41.38 | 136.79 | 326.98 | 2157.77 | 2832.69 | 3640.79 | 7.12 | 53.99 | 188.86 | 27681.68 | 32257.28 | 36998.88 | 1038.60 | 1481.16 | 2043.87 | 1714.81 | 2291.59 | 3014.22 |
| 1974 | 26.44 | 91.63 | 220.30 | 1470.86 | 1926.99 | 2473.84 | 4.81 | 37.55 | 130.17 | 18839.12 | 21931.00 | 25162.96 | 714.40 | 1010.76 | 1388.52 | 1167.54 | 1563.60 | 2032.64 |
| 1975 | 29.50 | 96.58 | 230.93 | 1549.13 | 2023.05 | 2593.23 | 4.86 | 39.21 | 138.58 | 19743.01 | 23029.60 | 26449.95 | 756.33 | 1061.14 | 1478.52 | 1228.35 | 1645.78 | 2147.69 |
| 1976 | 28.28 | 93.57 | 223.55 | 1514.95 | 1984.21 | 2548.74 | 4.67 | 38.65 | 136.84 | 19298.08 | 22580.85 | 25972.71 | 727.98 | 1041.95 | 1434.80 | 1189.68 | 1602.13 | 2107.51 |
| 1977 | 26.58 | 92.87 | 222.26 | 1493.61 | 1958.75 | 2502.01 | 4.48 | 37.74 | 135.69 | 19048.73 | 22212.34 | 25546.78 | 719.22 | 1020.08 | 1407.12 | 1186.27 | 1578.78 | 2081.39 |
| 1978 | 27.76 | 96.12 | 231.43 | 1543.80 | 2025.44 | 2602.29 | 5.12 | 39.26 | 137.95 | 19682.11 | 23025.73 | 26412.01 | 749.02 | 1061.42 | 1462.42 | 1225.03 | 1644.50 | 2153.55 |
| 1979 | 26.89 | 90.40 | 218.47 | 1465.99 | 1908.77 | 2451.68 | 4.42 | 36.80 | 128.85 | 18609.56 | 21743.35 | 24977.12 | 709.47 | 999.70 | 1386.96 | 1160.69 | 1548.32 | 2036.71 |
| 1980 | 28.30 | 94.09 | 229.59 | 1525.29 | 1994.07 | 2566.45 | 5.09 | 39.25 | 138.86 | 19411.27 | 22724.97 | 26114.85 | 735.81 | 1044.65 | 1443.20 | 1207.46 | 1613.24 | 2123.23 |
| 1981 | 28.60 | 99.24 | 241.18 | 1601.47 | 2097.10 | 2688.83 | 5.32 | 40.59 | 139.63 | 20372.44 | 23803.31 | 27373.62 | 777.34 | 1097.53 | 1509.31 | 1257.63 | 1692.14 | 2222.83 |
| 1982 | 31.45 | 106.81 | 260.37 | 1714.30 | 2238.14 | 2895.55 | 5.52 | 44.14 | 155.42 | 21865.80 | 25500.52 | 29302.97 | 822.38 | 1177.07 | 1614.16 | 1363.07 | 1822.44 | 2385.04 |
| 1983 | 29.14 | 98.15 | 235.56 | 1590.38 | 2065.18 | 2663.98 | 5.37 | 39.87 | 137.03 | 20151.96 | 23550.75 | 27031.23 | 764.06 | 1088.88 | 1498.03 | 1253.88 | 1674.08 | 2194.60 |
| 1984 | 31.87 | 107.13 | 255.66 | 1691.22 | 2219.12 | 2861.80 | 5.82 | 42.63 | 151.98 | 21606.43 | 25300.22 | 29040.35 | 815.32 | 1164.40 | 1608.33 | 1339.95 | 1801.93 | 2379.61 |
| 1985 | 31.47 | 105.75 | 255.51 | 1706.54 | 2228.35 | 2854.86 | 5.62 | 42.60 | 152.37 | 21757.79 | 25355.13 | 29093.80 | 819.23 | 1168.53 | 1610.06 | 1346.71 | 1805.00 | 2361.14 |
| 1986 | 27.45 | 93.31 | 224.92 | 1498.26 | 1972.27 | 2532.20 | 5.37 | 39.02 | 136.38 | 19186.87 | 22385.18 | 25719.22 | 730.21 | 1029.24 | 1428.84 | 1194.79 | 1592.88 | 2097.91 |
| 1987 | 37.84 | 126.11 | 306.90 | 2015.15 | 2635.11 | 3373.40 | 6.53 | 50.80 | 180.76 | 25729.97 | 29962.55 | 34438.75 | 968.58 | 1379.33 | 1903.66 | 1595.98 | 2134.85 | 2796.46 |
| 1988 | 36.55 | 125.48 | 303.86 | 2021.12 | 2632.17 | 3386.25 | 6.87 | 50.93 | 180.49 | 25671.30 | 30035.09 | 34485.37 | 975.12 | 1386.77 | 1909.25 | 1598.03 | 2139.59 | 2786.81 |
| 1989 | 28.44 | 96.00 | 227.86 | 1518.64 | 1986.11 | 2555.55 | 5.12 | 38.23 | 135.44 | 19339.77 | 22617.10 | 25989.58 | 737.28 | 1041.08 | 1439.93 | 1199.85 | 1610.59 | 2107.51 |
| 1990 | 28.68 | 94.75 | 229.92 | 1534.57 | 1994.31 | 2554.98 | 4.76 | 38.32 | 135.14 | 19449.61 | 22683.37 | 26090.89 | 737.37 | 1043.61 | 1446.12 | 1204.05 | 1610.55 | 2123.22 |
| 1991 | 22.20 | 73.50 | 175.44 | 1186.23 | 1547.81 | 1987.77 | 3.56 | 30.22 | 106.83 | 15067.02 | 17581.58 | 20209.62 | 575.08 | 811.01 | 1119.14 | 936.65 | 1250.64 | 1641.30 |
| 1992 | 20.95 | 69.59 | 168.55 | 1115.91 | 1460.81 | 1877.16 | 3.55 | 29.04 | 98.85 | 14220.68 | 16601.39 | 19095.43 | 536.73 | 761.60 | 1050.74 | 882.97 | 1183.22 | 1548.52 |
| 1993 | 16.23 | 56.06 | 134.72 | 904.34 | 1177.27 | 1514.36 | 2.86 | 22.73 | 79.32 | 11427.85 | 13377.14 | 15385.49 | 433.32 | 617.67 | 849.14 | 715.32 | 953.16 | 1248.63 |
| 1994 | 19.35 | 66.51 | 157.07 | 1060.85 | 1393.14 | 1795.11 | 3.63 | 27.15 | 97.25 | 13526.23 | 15835.23 | 18216.43 | 512.25 | 728.18 | 1005.42 | 839.20 | 1123.87 | 1477.32 |
| 1995 | 17.49 | 60.46 | 143.56 | 975.69 | 1272.91 | 1634.18 | 3.28 | 25.25 | 85.72 | 12399.79 | 14457.56 | 16653.86 | 473.80 | 669.64 | 923.75 | 764.61 | 1029.95 | 1342.57 |
| 1996 | 15.48 | 52.80 | 126.18 | 846.66 | 1103.18 | 1418.06 | 2.66 | 21.46 | 76.13 | 10761.03 | 12544.74 | 14395.81 | 407.40 | 579.44 | 796.83 | 669.06 | 892.50 | 1170.51 |
| 1997 | 20.39 | 68.51 | 166.92 | 1117.31 | 1446.12 | 1862.19 | 3.57 | 28.04 | 99.70 | 14058.73 | 16471.49 | 18881.68 | 535.78 | 759.97 | 1049.00 | 877.43 | 1172.31 | 1535.80 |
| 1998 | 17.24 | 58.00 | 140.27 | 929.62 | 1220.13 | 1573.39 | 3.07 | 23.66 | 84.44 | 11871.57 | 13882.42 | 15968.11 | 448.66 | 635.52 | 881.37 | 738.91 | 990.95 | 1296.54 |
| 1999 | 15.31 | 52.55 | 126.59 | 844.74 | 1103.38 | 1415.59 | 2.71 | 21.29 | 74.61 | 10734.46 | 12585.55 | 14486.42 | 407.73 | 577.87 | 798.86 | 666.86 | 891.57 | 1173.31 |
| 2000 | 20.08 | 68.86 | 164.55 | 1110.03 | 1445.35 | 1847.23 | 4.00 | 28.32 | 98.37 | 14059.22 | 16449.55 | 18917.92 | 530.85 | 757.57 | 1044.75 | 878.74 | 1173.44 | 1538.95 |
| 2001 | 16.65 | 55.62 | 133.04 | 896.24 | 1166.97 | 1490.91 | 2.88 | 22.45 | 79.27 | 11379.59 | 13255.34 | 15259.51 | 431.18 | 611.87 | 848.81 | 702.13 | 944.15 | 1234.60 |
| 2002 | 18.14 | 61.40 | 147.02 | 984.75 | 1282.04 | 1651.99 | 3.27 | 25.20 | 87.63 | 12517.27 | 14615.86 | 16761.94 | 474.88 | 674.30 | 930.24 | 779.33 | 1038.82 | 1359.45 |
| 2003 | 12.58 | 41.60 | 99.16 | 670.42 | 876.40 | 1131.93 | 2.32 | 16.88 | 58.94 | 8507.40 | 9956.33 | 11456.59 | 322.82 | 459.65 | 635.66 | 529.23 | 708.86 | 929.60 |
| 2004 | 12.74 | 41.64 | 100.54 | 665.95 | 871.98 | 1121.79 | 2.27 | 16.94 | 59.00 | 8503.15 | 9939.52 | 11418.74 | 320.49 | 457.32 | 629.43 | 530.36 | 706.65 | 920.84 |
| 2005 | 15.76 | 53.44 | 127.87 | 857.91 | 1117.57 | 1438.01 | 2.72 | 21.52 | 76.38 | 10879.55 | 12667.22 | 14601.87 | 412.11 | 584.11 | 806.75 | 675.89 | 904.74 | 1188.42 |
| 2006 | 15.67 | 52.70 | 127.95 | 845.39 | 1109.77 | 1433.44 | 2.80 | 21.91 | 74.50 | 10825.88 | 12659.63 | 14555.87 | 408.31 | 580.48 | 802.43 | 672.61 | 900.64 | 1180.42 |
| 2007 | 16.63 | 58.06 | 139.50 | 928.73 | 1214.40 | 1552.80 | 2.98 | 24.01 | 83.60 | 11831.57 | 13799.15 | 15929.14 | 448.26 | 634.93 | 875.54 | 736.09 | 983.55 | 1294.17 |
| 2008 | 18.04 | 60.07 | 145.83 | 971.63 | 1273.24 | 1630.46 | 3.18 | 24.58 | 86.13 | 12363.13 | 14429.13 | 16593.20 | 466.80 | 664.96 | 918.48 | 764.67 | 1029.75 | 1343.35 |
| 2009 | 19.07 | 63.82 | 152.77 | 1030.59 | 1345.24 | 1728.25 | 3.31 | 26.07 | 91.56 | 13084.93 | 15297.01 | 17566.76 | 495.16 | 701.97 | 970.70 | 813.05 | 1086.23 | 1438.57 |
| 2010 | 20.11 | 69.12 | 167.66 | 1112.50 | 1451.26 | 1867.06 | 3.72 | 27.88 | 99.24 | 14144.71 | 16497.98 | 18934.89 | 539.28 | 758.56 | 1039.01 | 873.43 | 1177.23 | 1541.82 |
| 2011 | 23.74 | 80.36 | 197.43 | 1289.13 | 1697.30 | 2184.64 | 4.29 | 32.33 | 115.29 | 16511.18 | 19297.37 | 22128.87 | 624.25 | 886.96 | 1221.30 | 1029.39 | 1370.89 | 1792.91 |
| 2012 | 23.27 | 78.37 | 188.31 | 1270.96 | 1654.97 | 2125.43 | 4.16 | 32.83 | 113.41 | 16112.04 | 18915.25 | 21707.01 | 613.48 | 868.93 | 1195.44 | 1007.67 | 1343.17 | 1766.41 |
| 2013 | 22.23 | 77.53 | 187.18 | 1241.48 | 1620.25 | 2082.27 | 3.82 | 31.42 | 109.53 | 15718.22 | 18386.51 | 21116.75 | 594.65 | 849.54 | 1168.73 | 982.36 | 1308.02 | 1722.37 |
| 2014 | 22.40 | 77.44 | 184.71 | 1226.43 | 1613.64 | 2064.46 | 4.15 | 31.44 | 110.36 | 15697.35 | 18329.33 | 21060.08 | 596.32 | 844.69 | 1160.19 | 978.03 | 1303.06 | 1705.82 |
| 2015 | 23.80 | 79.38 | 193.53 | 1287.22 | 1682.00 | 2165.70 | 4.22 | 32.62 | 114.95 | 16376.26 | 19130.63 | 21970.49 | 619.73 | 880.05 | 1214.72 | 1020.66 | 1359.85 | 1792.70 |
| 2016 | 22.52 | 78.41 | 188.09 | 1235.37 | 1612.17 | 2074.33 | 4.29 | 31.99 | 112.38 | 15741.67 | 18393.19 | 21122.94 | 599.57 | 848.51 | 1162.74 | 978.18 | 1306.43 | 1717.05 |
| 2017 | 27.82 | 94.41 | 228.40 | 1508.70 | 1977.76 | 2531.95 | 4.84 | 38.69 | 134.99 | 19195.48 | 22456.28 | 25784.88 | 725.42 | 1039.97 | 1420.16 | 1191.89 | 1593.82 | 2095.96 |
| 2018 | 26.53 | 87.18 | 212.43 | 1416.01 | 1852.84 | 2384.51 | 4.87 | 36.53 | 127.94 | 18059.69 | 21091.16 | 24193.97 | 681.25 | 967.34 | 1335.33 | 1119.04 | 1496.46 | 1965.19 |
| 2019 | 28.61 | 94.64 | 227.84 | 1507.55 | 1973.01 | 2527.87 | 5.14 | 38.50 | 134.93 | 19141.08 | 22396.84 | 25713.69 | 723.48 | 1032.79 | 1426.17 | 1188.77 | 1590.41 | 2083.14 |
| 2020 | 39.27 | 131.25 | 313.85 | 2089.64 | 2730.89 | 3513.04 | 7.25 | 53.62 | 186.26 | 26688.82 | 31188.48 | 35817.36 | 1011.81 | 1436.68 | 1986.17 | 1658.84 | 2217.84 | 2894.57 |
| 2021 | 55.47 | 183.60 | 437.93 | 2915.63 | 3811.77 | 4901.21 | 9.37 | 73.72 | 261.20 | 37209.68 | 43450.41 | 49834.86 | 1417.90 | 1996.79 | 2748.87 | 2308.50 | 3093.24 | 4045.47 |
| 2022 | 74.87 | 249.77 | 603.71 | 4018.34 | 5242.54 | 6748.72 | 13.03 | 100.39 | 356.66 | 51336.86 | 59729.48 | 68961.57 | 1938.39 | 2752.19 | 3797.78 | 3179.50 | 4253.02 | 5583.53 |
| 2023 | 102.08 | 334.61 | 814.13 | 5383.26 | 7035.40 | 9032.95 | 18.39 | 139.72 | 496.67 | 68410.75 | 80003.06 | 91872.33 | 2621.80 | 3682.70 | 5071.74 | 4214.24 | 5688.22 | 7507.41 |
| Total | 1534.31 | 5178.67 | 12478.27 | 83095.81 | 108618.93 | 139531.10 | 273.29 | 2110.12 | 7424.44 | 1058072.39 | 1235842.89 | 1420159.02 | 40098.69 | 56907.36 | 78472.33 | 65738.37 | 87940.46 | 115344 |

Table S14. Annual metal demand attributable to spacecraft electronics (1957-2023). Values represent 5th, 50th, and 95th percentiles (kg/year) derived from 10,000 Monte Carlo iterations. Part II.

| Year | Pd_P05 | Pd_P50 | Pd_P95 | Pt_P05 | Pt_P50 | Pt_P95 | REEs_P05 | REEs_P50 | REEs_P95 | Sn_P05 | Sn_P50 | Sn_P95 | Ti_P05 | Ti_P50 | Ti_P95 | Zn_P05 | Zn_P50 | Zn_P95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 0.00 | 0.00 | 0.06 | 0.00 | 0.00 | 0.03 | 0.87 | 1.28 | 1.80 | 1.26 | 1.74 | 2.36 | 0.21 | 0.41 | 0.72 | 0.33 | 0.57 | 0.93 |
| 1958 | 0.00 | 0.02 | 0.83 | 0.00 | 0.00 | 0.38 | 11.91 | 17.35 | 24.51 | 16.89 | 23.44 | 31.70 | 2.87 | 5.50 | 9.57 | 4.35 | 7.62 | 12.46 |
| 1959 | 0.00 | 0.02 | 0.87 | 0.00 | 0.00 | 0.42 | 13.46 | 19.55 | 27.59 | 19.21 | 26.71 | 36.16 | 3.20 | 6.35 | 10.95 | 4.88 | 8.76 | 14.02 |
| 1960 | 0.00 | 0.12 | 4.64 | 0.00 | 0.00 | 1.90 | 66.46 | 96.73 | 135.55 | 94.06 | 130.72 | 177.02 | 16.08 | 31.04 | 54.21 | 24.84 | 42.80 | 69.19 |
| 1961 | 0.00 | 0.19 | 6.97 | 0.00 | 0.00 | 3.21 | 108.57 | 158.57 | 222.56 | 154.00 | 213.92 | 290.19 | 26.23 | 50.36 | 87.68 | 40.23 | 69.79 | 112.30 |
| 1962 | 0.00 | 0.26 | 9.99 | 0.00 | 0.00 | 3.94 | 142.87 | 210.20 | 294.93 | 205.55 | 286.29 | 387.78 | 35.18 | 67.56 | 118.10 | 53.79 | 93.50 | 152.35 |
| 1963 | 0.00 | 0.19 | 7.09 | 0.00 | 0.00 | 3.00 | 104.65 | 151.14 | 212.44 | 148.20 | 205.91 | 279.19 | 24.78 | 48.33 | 83.97 | 38.47 | 66.55 | 108.57 |
| 1964 | 0.00 | 0.44 | 17.87 | 0.00 | 0.00 | 8.03 | 262.87 | 380.23 | 534.81 | 374.16 | 516.64 | 700.13 | 63.02 | 122.08 | 213.10 | 96.98 | 167.93 | 274.02 |
| 1965 | 0.00 | 0.74 | 24.49 | 0.00 | 0.00 | 10.82 | 349.49 | 511.73 | 721.24 | 502.44 | 695.30 | 946.76 | 84.00 | 164.58 | 285.53 | 129.03 | 228.67 | 367.71 |
| 1966 | 0.00 | 0.76 | 28.43 | 0.00 | 0.01 | 12.62 | 422.80 | 612.83 | 863.95 | 603.70 | 832.43 | 1127.01 | 102.97 | 198.06 | 348.83 | 153.92 | 271.00 | 438.42 |
| 1967 | 0.00 | 0.60 | 24.96 | 0.00 | 0.00 | 10.55 | 355.55 | 519.09 | 734.11 | 504.91 | 707.13 | 953.14 | 84.81 | 166.39 | 291.63 | 131.59 | 231.93 | 373.14 |
| 1968 | 0.00 | 0.94 | 36.15 | 0.00 | 0.01 | 14.85 | 538.29 | 786.25 | 1099.35 | 766.13 | 1062.77 | 1441.31 | 131.65 | 253.98 | 441.48 | 199.95 | 344.63 | 564.47 |
| 1969 | 0.00 | 1.83 | 68.16 | 0.00 | 0.01 | 27.38 | 969.37 | 1417.93 | 1997.44 | 1387.04 | 1927.32 | 2597.03 | 232.29 | 455.51 | 791.76 | 364.86 | 632.46 | 1025.59 |
| 1970 | 0.00 | 0.86 | 34.69 | 0.00 | 0.01 | 15.07 | 504.46 | 731.09 | 1036.86 | 716.92 | 986.78 | 1343.31 | 120.51 | 233.38 | 408.77 | 182.07 | 323.39 | 521.55 |
| 1971 | 0.00 | 1.35 | 50.64 | 0.00 | 0.01 | 23.84 | 768.72 | 1117.67 | 1575.39 | 1104.63 | 1528.04 | 2063.81 | 188.48 | 363.13 | 629.80 | 283.71 | 496.72 | 812.73 |
| 1972 | 0.00 | 1.32 | 50.85 | 0.00 | 0.01 | 21.49 | 736.29 | 1079.66 | 1519.84 | 1053.61 | 1464.50 | 1977.99 | 178.32 | 348.68 | 608.35 | 271.96 | 478.34 | 777.68 |
| 1973 | 0.00 | 1.50 | 56.34 | 0.00 | 0.01 | 25.79 | 869.87 | 1265.29 | 1778.85 | 1235.38 | 1713.46 | 2316.69 | 209.62 | 405.49 | 706.12 | 323.72 | 564.51 | 907.72 |
| 1974 | 0.00 | 0.99 | 40.29 | 0.00 | 0.01 | 17.91 | 592.80 | 855.60 | 1206.53 | 837.62 | 1161.17 | 1582.88 | 141.37 | 276.31 | 479.55 | 214.66 | 380.18 | 621.10 |
| 1975 | 0.00 | 1.09 | 42.11 | 0.00 | 0.01 | 18.27 | 619.48 | 901.09 | 1278.72 | 885.94 | 1227.65 | 1657.20 | 150.94 | 291.54 | 501.43 | 226.58 | 400.26 | 650.57 |
| 1976 | 0.00 | 1.15 | 42.94 | 0.00 | 0.01 | 18.08 | 607.81 | 888.56 | 1242.42 | 869.10 | 1203.32 | 1628.68 | 145.59 | 282.54 | 495.27 | 224.94 | 394.23 | 633.07 |
| 1977 | 0.00 | 1.10 | 40.33 | 0.00 | 0.01 | 17.66 | 599.46 | 869.05 | 1220.69 | 847.53 | 1182.68 | 1599.75 | 144.91 | 279.68 | 483.26 | 218.30 | 384.23 | 621.06 |
| 1978 | 0.00 | 1.12 | 40.02 | 0.00 | 0.01 | 18.72 | 618.46 | 904.89 | 1271.67 | 883.73 | 1223.26 | 1650.59 | 148.54 | 292.53 | 505.34 | 229.05 | 399.21 | 648.07 |
| 1979 | 0.00 | 1.01 | 38.38 | 0.00 | 0.00 | 17.23 | 584.48 | 850.94 | 1199.71 | 830.41 | 1154.78 | 1558.82 | 140.61 | 272.96 | 470.35 | 216.67 | 375.06 | 610.02 |
| 1980 | 0.00 | 1.06 | 40.64 | 0.00 | 0.01 | 17.93 | 608.02 | 889.20 | 1254.95 | 869.26 | 1209.37 | 1632.70 | 148.63 | 284.87 | 499.04 | 226.02 | 394.90 | 638.03 |
| 1981 | 0.00 | 1.13 | 42.59 | 0.00 | 0.01 | 19.69 | 640.53 | 931.88 | 1311.30 | 913.58 | 1268.06 | 1712.58 | 153.15 | 301.84 | 518.09 | 234.03 | 412.42 | 672.86 |
| 1982 | 0.00 | 1.25 | 46.08 | 0.00 | 0.01 | 21.34 | 689.03 | 998.89 | 1405.71 | 982.33 | 1361.42 | 1835.02 | 166.40 | 320.84 | 555.96 | 250.21 | 443.53 | 718.92 |
| 1983 | 0.00 | 1.15 | 42.00 | 0.00 | 0.01 | 19.32 | 631.73 | 927.09 | 1301.18 | 902.55 | 1252.35 | 1693.95 | 151.48 | 296.44 | 517.46 | 233.39 | 407.06 | 661.21 |
| 1984 | 0.00 | 1.31 | 43.40 | 0.00 | 0.01 | 21.31 | 685.66 | 987.84 | 1402.68 | 971.26 | 1346.69 | 1825.99 | 164.26 | 319.99 | 558.86 | 256.22 | 444.34 | 708.94 |
| 1985 | 0.00 | 1.36 | 45.85 | 0.00 | 0.01 | 20.28 | 679.65 | 995.79 | 1395.84 | 969.32 | 1342.31 | 1814.06 | 165.34 | 319.73 | 554.07 | 251.82 | 439.03 | 709.30 |
| 1986 | 0.00 | 1.05 | 40.43 | 0.00 | 0.01 | 18.14 | 599.80 | 879.74 | 1235.30 | 862.26 | 1192.17 | 1618.57 | 145.81 | 281.23 | 493.21 | 224.12 | 389.86 | 629.70 |
| 1987 | 0.00 | 1.49 | 53.54 | 0.00 | 0.01 | 24.66 | 805.83 | 1176.73 | 1662.09 | 1154.30 | 1597.97 | 2159.82 | 194.58 | 378.95 | 655.73 | 292.73 | 521.66 | 841.34 |
| 1988 | 0.00 | 1.64 | 52.14 | 0.00 | 0.01 | 22.36 | 803.48 | 1175.23 | 1643.76 | 1144.70 | 1597.40 | 2179.11 | 196.62 | 379.01 | 662.96 | 298.14 | 524.74 | 850.97 |
| 1989 | 0.00 | 1.06 | 41.29 | 0.00 | 0.01 | 17.37 | 607.61 | 888.19 | 1252.19 | 868.14 | 1206.91 | 1636.15 | 147.67 | 285.86 | 499.69 | 225.65 | 393.71 | 641.67 |
| 1990 | 0.00 | 1.08 | 41.81 | 0.00 | 0.01 | 17.12 | 609.45 | 886.27 | 1248.41 | 869.28 | 1206.09 | 1624.17 | 147.56 | 283.43 | 494.74 | 224.09 | 392.93 | 638.21 |
| 1991 | 0.00 | 0.81 | 31.80 | 0.00 | 0.00 | 14.99 | 469.19 | 686.54 | 974.23 | 670.37 | 936.22 | 1281.00 | 114.55 | 222.05 | 386.11 | 174.13 | 304.56 | 491.69 |
| 1992 | 0.00 | 0.79 | 29.68 | 0.00 | 0.00 | 13.45 | 447.63 | 654.39 | 918.02 | 635.18 | 883.35 | 1201.14 | 107.92 | 208.42 | 364.89 | 168.32 | 288.78 | 468.27 |
| 1993 | 0.00 | 0.69 | 24.75 | 0.00 | 0.00 | 11.13 | 357.33 | 526.41 | 746.91 | 513.35 | 711.22 | 968.21 | 85.94 | 167.34 | 290.84 | 132.32 | 232.74 | 375.83 |
| 1994 | 0.00 | 0.81 | 28.06 | 0.00 | 0.00 | 12.15 | 424.22 | 621.26 | 882.97 | 608.52 | 838.31 | 1140.73 | 102.19 | 199.72 | 347.66 | 157.31 | 275.97 | 446.55 |
| 1995 | 0.00 | 0.67 | 25.27 | 0.00 | 0.00 | 11.96 | 387.18 | 566.32 | 800.44 | 549.31 | 767.71 | 1046.22 | 94.96 | 182.82 | 317.76 | 142.77 | 250.94 | 407.35 |
| 1996 | 0.00 | 0.63 | 22.07 | 0.00 | 0.00 | 10.16 | 337.95 | 492.07 | 692.35 | 481.40 | 668.92 | 903.02 | 82.86 | 158.92 | 277.15 | 126.33 | 219.13 | 357.07 |
| 1997 | 0.00 | 0.76 | 28.87 | 0.00 | 0.00 | 13.83 | 442.96 | 645.85 | 908.03 | 626.28 | 872.72 | 1185.93 | 108.61 | 205.12 | 353.44 | 161.57 | 286.37 | 461.01 |
| 1998 | 0.00 | 0.62 | 23.97 | 0.00 | 0.00 | 11.16 | 376.54 | 544.20 | 767.82 | 531.84 | 736.66 | 989.65 | 89.96 | 174.66 | 301.04 | 136.02 | 240.56 | 391.66 |
| 1999 | 0.00 | 0.59 | 22.49 | 0.00 | 0.00 | 9.74 | 337.74 | 492.77 | 694.14 | 484.51 | 669.43 | 904.40 | 82.01 | 158.57 | 274.62 | 125.55 | 218.55 | 357.23 |
| 2000 | 0.00 | 0.78 | 28.95 | 0.00 | 0.00 | 13.17 | 442.66 | 647.38 | 911.71 | 625.98 | 872.25 | 1190.80 | 105.61 | 208.01 | 358.04 | 162.52 | 287.13 | 460.18 |
| 2001 | 0.00 | 0.67 | 23.77 | 0.00 | 0.00 | 11.18 | 356.08 | 519.94 | 733.68 | 504.24 | 707.77 | 955.60 | 86.85 | 167.30 | 294.40 | 132.98 | 229.99 | 375.54 |
| 2002 | 0.00 | 0.68 | 26.77 | 0.00 | 0.00 | 11.38 | 391.91 | 570.74 | 799.53 | 560.94 | 780.71 | 1055.84 | 95.79 | 183.08 | 319.48 | 144.40 | 252.49 | 409.58 |
| 2003 | 0.00 | 0.48 | 17.08 | 0.00 | 0.00 | 8.04 | 267.60 | 389.61 | 549.11 | 383.23 | 530.54 | 722.48 | 65.25 | 125.66 | 217.77 | 97.98 | 173.35 | 281.09 |
| 2004 | 0.00 | 0.48 | 17.85 | 0.00 | 0.00 | 7.17 | 267.74 | 388.62 | 548.73 | 380.77 | 527.09 | 708.30 | 64.96 | 126.05 | 218.97 | 98.77 | 173.21 | 280.35 |
| 2005 | 0.00 | 0.63 | 23.36 | 0.00 | 0.00 | 10.11 | 341.88 | 497.88 | 697.65 | 487.05 | 674.51 | 910.38 | 83.33 | 159.68 | 278.60 | 125.22 | 220.59 | 354.62 |
| 2006 | 0.00 | 0.57 | 23.42 | 0.00 | 0.00 | 10.17 | 339.58 | 497.83 | 696.13 | 490.78 | 675.19 | 906.39 | 82.16 | 159.57 | 277.98 | 124.99 | 219.15 | 354.75 |
| 2007 | 0.00 | 0.62 | 24.49 | 0.00 | 0.00 | 10.89 | 369.76 | 539.28 | 769.21 | 532.61 | 736.60 | 1005.96 | 88.80 | 174.16 | 305.10 | 137.43 | 240.15 | 389.22 |
| 2008 | 0.00 | 0.65 | 25.54 | 0.00 | 0.00 | 10.64 | 391.13 | 566.60 | 794.98 | 553.51 | 769.35 | 1037.80 | 94.48 | 182.10 | 313.36 | 142.03 | 249.58 | 406.52 |
| 2009 | 0.00 | 0.73 | 27.60 | 0.00 | 0.00 | 12.37 | 412.86 | 599.91 | 844.31 | 589.39 | 813.86 | 1099.45 | 100.95 | 193.76 | 339.00 | 154.52 | 266.24 | 426.65 |
| 2010 | 0.00 | 0.83 | 30.63 | 0.00 | 0.01 | 14.19 | 445.91 | 647.31 | 911.26 | 638.39 | 881.41 | 1190.77 | 106.79 | 208.77 | 362.10 | 164.59 | 285.93 | 461.66 |
| 2011 | 0.00 | 0.91 | 35.59 | 0.00 | 0.01 | 15.28 | 523.12 | 756.92 | 1063.57 | 747.39 | 1027.29 | 1390.46 | 126.40 | 243.76 | 424.42 | 191.63 | 334.90 | 540.48 |
| 2012 | 0.00 | 0.94 | 34.42 | 0.00 | 0.00 | 14.04 | 507.96 | 742.30 | 1049.15 | 712.11 | 1004.38 | 1362.21 | 123.04 | 239.62 | 406.57 | 187.56 | 328.30 | 531.21 |
| 2013 | 0.00 | 0.93 | 33.85 | 0.00 | 0.00 | 14.86 | 490.01 | 718.86 | 1013.67 | 702.08 | 979.46 | 1316.24 | 121.43 | 234.30 | 405.89 | 180.90 | 320.19 | 516.58 |
| 2014 | 0.00 | 0.83 | 34.56 | 0.00 | 0.00 | 14.41 | 495.81 | 718.30 | 1019.36 | 704.10 | 976.12 | 1312.75 | 120.65 | 232.76 | 403.67 | 183.83 | 320.47 | 514.29 |
| 2015 | 0.00 | 0.95 | 35.55 | 0.00 | 0.00 | 16.10 | 510.95 | 749.32 | 1056.88 | 733.32 | 1023.29 | 1386.64 | 124.57 | 240.04 | 415.59 | 189.45 | 332.86 | 539.67 |
| 2016 | 0.00 | 0.84 | 33.36 | 0.00 | 0.00 | 15.18 | 492.95 | 716.11 | 1018.13 | 706.34 | 975.33 | 1328.29 | 119.97 | 230.15 | 405.63 | 181.30 | 317.94 | 513.04 |
| 2017 | 0.00 | 1.07 | 40.53 | 0.00 | 0.00 | 16.36 | 599.94 | 879.33 | 1238.89 | 861.75 | 1197.29 | 1620.62 | 143.28 | 284.39 | 491.60 | 220.60 | 391.18 | 631.26 |
| 2018 | 0.00 | 0.96 | 36.26 | 0.00 | 0.01 | 18.57 | 566.30 | 823.84 | 1166.48 | 813.96 | 1119.44 | 1525.99 | 136.60 | 265.77 | 462.21 | 208.56 | 366.45 | 593.40 |
| 2019 | 0.00 | 1.06 | 41.76 | 0.00 | 0.01 | 17.66 | 604.26 | 875.16 | 1231.41 | 859.08 | 1190.97 | 1617.60 | 146.34 | 282.03 | 488.14 | 220.18 | 388.48 | 630.18 |
| 2020 | 0.00 | 1.54 | 56.63 | 0.00 | 0.01 | 26.10 | 844.66 | 1219.97 | 1732.83 | 1197.51 | 1653.62 | 2230.55 | 200.71 | 393.81 | 677.86 | 308.74 | 541.43 | 875.97 |
| 2021 | 0.00 | 2.28 | 79.49 | 0.00 | 0.01 | 34.47 | 1179.92 | 1696.09 | 2406.33 | 1662.68 | 2311.59 | 3158.68 | 283.37 | 548.10 | 947.13 | 432.67 | 751.32 | 1218.60 |
| 2022 | 0.00 | 2.48 | 105.18 | 0.00 | 0.01 | 49.34 | 1625.47 | 2356.27 | 3315.14 | 2287.05 | 3178.07 | 4290.18 | 389.87 | 753.01 | 1299.43 | 598.53 | 1040.89 | 1687.62 |
| 2023 | 0.00 | 3.80 | 150.27 | 0.00 | 0.02 | 63.82 | 2157.29 | 3129.17 | 4433.59 | 3061.59 | 4244.68 | 5756.35 | 521.15 | 1002.78 | 1734.74 | 792.20 | 1380.29 | 2220.41 |
| Total | 0.00 | 59.49 | 2230.48 | 0.00 | 0.33 | 990.96 | 33258.99 | 48430.94 | 68267.43 | 47384.44 | 65747.34 | 89025.88 | 8037.90 | 15582.06 | 27062.05 | 12265.01 | 21476.34 | 34777.01 |

Solar panels in spacecraft

| The power subsystem typically accounts for one-third of the total spacecraft weight, with batteries making up one-third of the power system's weight. This system typically includes the solar array, batteries, control circuitry for managing battery charging and discharging, as well as power filters, converters, and switches responsible for distributing power to various spacecraft subsystems. |  |
| --- | --- |
| Reference: NASA | https://ntrs.nasa.gov/api/citations/19960054145/downloads/19960054145.pdf |
| Generally, solar panels can account for a significant portion of the total satellite weight, typically ranging from around 20-30%. |  |
| Reference: NASA | https://www.lpi.usra.edu/opag/meetings/aug2015/presentations/day-2/11_beauchamp.pdf |
| Power-to-weight ratio of solar panels for spacecraft (W/kg), 110 W/Kg in 1972 |  |
| The power-to-weight ratio for solar panels used in spacecraft typically ranges from about 50 watts per kilogram (W/kg) to over 300 W/kg for high-efficiency solar cells. |  |
| Reference: NASA | https://ntrs.nasa.gov/api/citations/19720022383/downloads/19720022383.pdf |
| Space missions using solar arrays, regardless of spacecraft mass, are strongly clustered around ~30 W/kg. |  |
| The average mission power consumption of 1000 W and a medium value of 600 W. |  |
| Reference: NASA | https://www.nasa.gov/smallsat-institute/sst-soa/power-subsystems/ |

Figure S5. Specific power-to-weight ratio of solar arrays in spacecraft. The larger red circles represent missions located farther from Earth, corresponding to the red tones on the distance scale. Figure obtained from NASA61.

Table S15. Metal intensity in solar panels.62

| Crystalline silicon |  | Multijunction |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Material | kg/MW | III-V |  | Material | kg/MW |
| Al | 184 | In | 15 | In | 15 |
| Ag | 21 | Ga | 52 | Ga | 52 |
| Cu | 946 | Al | 19 | Al | 27 |
| Pb | 62 | As | 262 | As | 262 |
| Sn | 59 | TCO |  | Zn | 0.04 |
| Si | 3430 | Al | 3 | Si | 4595 |
| Efficiency (%) | 17 | Zn | 0.04 | Cu | 574 |
|  |  | Si bottom cell |  | Sn | 36 |
|  |  | Si | 4595 | Pb | 37 |
|  |  | Al | 5 | Efficiency (%) | 28 |
|  |  | Cu | 574 |  |  |
|  |  | Sn | 36 |  |  |
|  |  | Pb | 37 |  |  |

Figure S6.  Distribution of solar-panel mass fractions derived from Monte Carlo sampling (N=10,000). Fractions follow a triangular distribution (20-30%, mode = 25%) to capture differences in spacecraft architectures, orbit profiles, and mission power requirements.

Figure S7.  Monte Carlo distribution of spacecraft specific power (W/kg) based on 10,000 simulations.

Histogram of simulated specific power values derived from sampled spacecraft masses and power-system parameters.

Table S16. Solar panels distribution in spacecrafts and its evolution.

| Year | Panels mass | Capacity | c-Si | M-junction | c-Si | M-junction |
| --- | --- | --- | --- | --- | --- | --- |
|  | t | MW | % | % | MW | MW |
| 1957 | 0.15 | 0.00 | 100 | 0 | 0.00 | 0.00 |
| 1958 | 1.99 | 0.06 | 100 | 0 | 0.06 | 0.00 |
| 1959 | 2.25 | 0.07 | 100 | 0 | 0.07 | 0.00 |
| 1960 | 11.09 | 0.33 | 100 | 0 | 0.33 | 0.00 |
| 1961 | 18.16 | 0.54 | 100 | 0 | 0.54 | 0.00 |
| 1962 | 24.17 | 0.73 | 100 | 0 | 0.73 | 0.00 |
| 1963 | 17.33 | 0.52 | 100 | 0 | 0.52 | 0.00 |
| 1964 | 43.71 | 1.31 | 100 | 0 | 1.31 | 0.00 |
| 1965 | 58.73 | 1.76 | 100 | 0 | 1.76 | 0.00 |
| 1966 | 70.67 | 2.12 | 100 | 0 | 2.12 | 0.00 |
| 1967 | 59.63 | 1.79 | 100 | 0 | 1.79 | 0.00 |
| 1968 | 90.15 | 2.70 | 100 | 0 | 2.70 | 0.00 |
| 1969 | 163.42 | 4.90 | 100 | 0 | 4.90 | 0.00 |
| 1970 | 83.69 | 2.51 | 100 | 0 | 2.51 | 0.00 |
| 1971 | 129.07 | 3.87 | 100 | 0 | 3.87 | 0.00 |
| 1972 | 123.80 | 3.71 | 100 | 0 | 3.71 | 0.00 |
| 1973 | 145.17 | 4.36 | 100 | 0 | 4.36 | 0.00 |
| 1974 | 98.57 | 2.96 | 100 | 0 | 2.96 | 0.00 |
| 1975 | 103.47 | 3.10 | 100 | 0 | 3.10 | 0.00 |
| 1976 | 101.49 | 3.04 | 100 | 0 | 3.04 | 0.00 |
| 1977 | 100.08 | 3.00 | 100 | 0 | 3.00 | 0.00 |
| 1978 | 103.67 | 3.11 | 100 | 0 | 3.11 | 0.00 |
| 1979 | 97.76 | 2.93 | 100 | 0 | 2.93 | 0.00 |
| 1980 | 102.05 | 3.06 | 100 | 0 | 3.06 | 0.00 |
| 1981 | 107.19 | 3.22 | 100 | 0 | 3.22 | 0.00 |
| 1982 | 114.70 | 3.44 | 100 | 0 | 3.44 | 0.00 |
| 1983 | 105.96 | 3.18 | 100 | 0 | 3.18 | 0.00 |
| 1984 | 113.88 | 3.42 | 100 | 0 | 3.42 | 0.00 |
| 1985 | 114.07 | 3.42 | 100 | 0 | 3.42 | 0.00 |
| 1986 | 100.90 | 3.03 | 100 | 0 | 3.03 | 0.00 |
| 1987 | 134.81 | 4.04 | 90 | 10 | 3.64 | 0.40 |
| 1988 | 135.19 | 4.06 | 80 | 20 | 3.24 | 0.81 |
| 1989 | 101.77 | 3.05 | 70 | 30 | 2.14 | 0.92 |
| 1990 | 102.09 | 3.06 | 60 | 40 | 1.84 | 1.23 |
| 1991 | 79.09 | 2.37 | 50 | 50 | 1.19 | 1.19 |
| 1992 | 74.71 | 2.24 | 40 | 60 | 0.90 | 1.34 |
| 1993 | 60.18 | 1.81 | 30 | 70 | 0.54 | 1.26 |
| 1994 | 71.26 | 2.14 | 20 | 80 | 0.43 | 1.71 |
| 1995 | 65.12 | 1.95 | 10 | 90 | 0.20 | 1.76 |
| 1996 | 56.40 | 1.69 | 0 | 100 | 0.00 | 1.69 |
| 1997 | 74.07 | 2.22 | 0 | 100 | 0.00 | 2.22 |
| 1998 | 62.52 | 1.88 | 0 | 100 | 0.00 | 1.88 |
| 1999 | 56.52 | 1.70 | 0 | 100 | 0.00 | 1.70 |
| 2000 | 74.11 | 2.22 | 0 | 100 | 0.00 | 2.22 |
| 2001 | 59.70 | 1.79 | 0 | 100 | 0.00 | 1.79 |
| 2002 | 65.72 | 1.97 | 0 | 100 | 0.00 | 1.97 |
| 2003 | 44.77 | 1.34 | 0 | 100 | 0.00 | 1.34 |
| 2004 | 44.73 | 1.34 | 0 | 100 | 0.00 | 1.34 |
| 2005 | 57.12 | 1.71 | 0 | 100 | 0.00 | 1.71 |
| 2006 | 56.94 | 1.71 | 0 | 100 | 0.00 | 1.71 |
| 2007 | 62.23 | 1.87 | 0 | 100 | 0.00 | 1.87 |
| 2008 | 65.01 | 1.95 | 0 | 100 | 0.00 | 1.95 |
| 2009 | 68.77 | 2.06 | 0 | 100 | 0.00 | 2.06 |
| 2010 | 74.27 | 2.23 | 0 | 100 | 0.00 | 2.23 |
| 2011 | 86.82 | 2.60 | 0 | 100 | 0.00 | 2.60 |
| 2012 | 84.97 | 2.55 | 0 | 100 | 0.00 | 2.55 |
| 2013 | 82.77 | 2.48 | 0 | 100 | 0.00 | 2.48 |
| 2014 | 82.42 | 2.47 | 0 | 100 | 0.00 | 2.47 |
| 2015 | 86.12 | 2.58 | 0 | 100 | 0.00 | 2.58 |
| 2016 | 82.86 | 2.49 | 0 | 100 | 0.00 | 2.49 |
| 2017 | 101.11 | 3.03 | 0 | 100 | 0.00 | 3.03 |
| 2018 | 94.94 | 2.85 | 0 | 100 | 0.00 | 2.85 |
| 2019 | 100.74 | 3.02 | 0 | 100 | 0.00 | 3.02 |
| 2020 | 140.24 | 4.21 | 0 | 100 | 0.00 | 4.21 |
| 2021 | 195.70 | 5.87 | 0 | 100 | 0.00 | 5.87 |
| 2022 | 269.43 | 8.08 | 0 | 100 | 0.00 | 8.08 |
| 2023 | 359.86 | 10.80 | 0 | 100 | 0.00 | 10.80 |
| Total | 5922.1 | 177.66 |  |  | 86.32 | 91.35 |

Table S17. Annual metal demand for spacecraft solar panels (1957-2023). Includes uncertainty from subsystem mass fraction (triangular distribution), solar specific power (20-100 W/kg), and Dirichlet-sampled compositional shares. Values represent P05, P50, and P95 (kg/y). Part I.

| Year | Ag_P05 | Ag_P50 | Ag_P95 | Al_P05 | Al_P50 | Al_P95 | As_P05 | As_P50 | As_P95 | Cu_P05 | Cu_P50 | Cu_P95 | Ga_P05 | Ga_P50 | Ga_P95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 0.1 | 0.1 | 0.3 | 0.7 | 1.3 | 2.3 | 0.0 | 0.0 | 0.0 | 3.6 | 6.5 | 11.8 | 0.0 | 0.0 | 0.0 |
| 1958 | 0.8 | 1.9 | 4.0 | 9.1 | 17.0 | 31.3 | 0.0 | 0.0 | 0.0 | 48.2 | 87.7 | 158.8 | 0.0 | 0.0 | 0.0 |
| 1959 | 0.9 | 2.1 | 4.6 | 10.4 | 19.3 | 35.6 | 0.0 | 0.0 | 0.0 | 55.0 | 99.9 | 178.8 | 0.0 | 0.0 | 0.0 |
| 1960 | 4.5 | 10.3 | 22.5 | 51.5 | 94.6 | 175.6 | 0.0 | 0.0 | 0.0 | 270.1 | 490.4 | 890.2 | 0.0 | 0.0 | 0.0 |
| 1961 | 7.5 | 17.0 | 37.0 | 84.4 | 155.7 | 287.2 | 0.0 | 0.0 | 0.0 | 443.3 | 807.4 | 1458.6 | 0.0 | 0.0 | 0.0 |
| 1962 | 9.9 | 22.5 | 49.2 | 111.6 | 207.4 | 383.6 | 0.0 | 0.0 | 0.0 | 590.5 | 1068.1 | 1932.3 | 0.0 | 0.0 | 0.0 |
| 1963 | 7.1 | 16.2 | 35.1 | 79.9 | 148.3 | 276.6 | 0.0 | 0.0 | 0.0 | 423.5 | 764.6 | 1387.6 | 0.0 | 0.0 | 0.0 |
| 1964 | 17.7 | 41.0 | 88.8 | 201.8 | 374.1 | 692.1 | 0.0 | 0.0 | 0.0 | 1064.1 | 1931.6 | 3493.8 | 0.0 | 0.0 | 0.0 |
| 1965 | 24.1 | 55.0 | 119.7 | 272.1 | 502.3 | 930.2 | 0.0 | 0.0 | 0.0 | 1434.2 | 2602.1 | 4685.0 | 0.0 | 0.0 | 0.0 |
| 1966 | 28.9 | 65.9 | 143.4 | 325.3 | 605.1 | 1121.8 | 0.0 | 0.0 | 0.0 | 1715.3 | 3120.3 | 5659.2 | 0.0 | 0.0 | 0.0 |
| 1967 | 24.2 | 55.9 | 120.9 | 275.4 | 509.1 | 950.2 | 0.0 | 0.0 | 0.0 | 1453.3 | 2634.2 | 4771.4 | 0.0 | 0.0 | 0.0 |
| 1968 | 36.8 | 83.9 | 184.6 | 413.7 | 770.1 | 1425.8 | 0.0 | 0.0 | 0.0 | 2196.1 | 3996.0 | 7185.5 | 0.0 | 0.0 | 0.0 |
| 1969 | 66.6 | 151.9 | 335.0 | 754.0 | 1402.7 | 2585.2 | 0.0 | 0.0 | 0.0 | 3994.9 | 7246.4 | 13045.2 | 0.0 | 0.0 | 0.0 |
| 1970 | 34.2 | 78.3 | 169.6 | 386.5 | 716.9 | 1316.7 | 0.0 | 0.0 | 0.0 | 2043.6 | 3708.6 | 6648.5 | 0.0 | 0.0 | 0.0 |
| 1971 | 52.2 | 120.9 | 263.1 | 595.8 | 1105.9 | 2052.6 | 0.0 | 0.0 | 0.0 | 3139.2 | 5706.0 | 10352.3 | 0.0 | 0.0 | 0.0 |
| 1972 | 50.8 | 115.2 | 252.3 | 571.7 | 1062.7 | 1963.5 | 0.0 | 0.0 | 0.0 | 3019.0 | 5479.2 | 9835.6 | 0.0 | 0.0 | 0.0 |
| 1973 | 59.5 | 136.1 | 294.1 | 663.8 | 1241.9 | 2311.4 | 0.0 | 0.0 | 0.0 | 3537.4 | 6428.3 | 11590.6 | 0.0 | 0.0 | 0.0 |
| 1974 | 39.9 | 92.4 | 200.0 | 450.4 | 844.4 | 1559.2 | 0.0 | 0.0 | 0.0 | 2383.5 | 4357.0 | 7842.8 | 0.0 | 0.0 | 0.0 |
| 1975 | 42.5 | 96.9 | 211.4 | 474.2 | 886.1 | 1651.6 | 0.0 | 0.0 | 0.0 | 2518.6 | 4578.9 | 8254.8 | 0.0 | 0.0 | 0.0 |
| 1976 | 41.1 | 94.7 | 206.3 | 466.6 | 867.9 | 1600.4 | 0.0 | 0.0 | 0.0 | 2468.1 | 4477.4 | 8057.3 | 0.0 | 0.0 | 0.0 |
| 1977 | 40.3 | 93.7 | 204.2 | 459.8 | 853.3 | 1584.4 | 0.0 | 0.0 | 0.0 | 2435.1 | 4420.9 | 7992.1 | 0.0 | 0.0 | 0.0 |
| 1978 | 42.5 | 97.1 | 211.5 | 476.4 | 888.0 | 1653.8 | 0.0 | 0.0 | 0.0 | 2534.9 | 4609.7 | 8323.2 | 0.0 | 0.0 | 0.0 |
| 1979 | 40.2 | 91.7 | 199.8 | 449.3 | 835.8 | 1550.7 | 0.0 | 0.0 | 0.0 | 2367.8 | 4334.1 | 7817.4 | 0.0 | 0.0 | 0.0 |
| 1980 | 41.7 | 95.2 | 206.6 | 471.1 | 875.5 | 1611.1 | 0.0 | 0.0 | 0.0 | 2491.5 | 4516.1 | 8122.6 | 0.0 | 0.0 | 0.0 |
| 1981 | 44.0 | 100.5 | 217.3 | 493.4 | 920.1 | 1695.3 | 0.0 | 0.0 | 0.0 | 2609.7 | 4745.2 | 8545.3 | 0.0 | 0.0 | 0.0 |
| 1982 | 46.7 | 107.2 | 232.3 | 530.0 | 984.2 | 1816.0 | 0.0 | 0.0 | 0.0 | 2791.3 | 5079.1 | 9150.5 | 0.0 | 0.0 | 0.0 |
| 1983 | 43.3 | 98.9 | 215.6 | 487.8 | 907.7 | 1680.5 | 0.0 | 0.0 | 0.0 | 2573.7 | 4674.3 | 8467.6 | 0.0 | 0.0 | 0.0 |
| 1984 | 46.5 | 106.9 | 232.6 | 522.6 | 973.9 | 1799.7 | 0.0 | 0.0 | 0.0 | 2781.2 | 5033.0 | 9117.7 | 0.0 | 0.0 | 0.0 |
| 1985 | 46.8 | 107.0 | 232.0 | 528.7 | 982.8 | 1815.2 | 0.0 | 0.0 | 0.0 | 2778.5 | 5057.2 | 9104.1 | 0.0 | 0.0 | 0.0 |
| 1986 | 41.5 | 94.5 | 204.4 | 465.5 | 865.1 | 1604.3 | 0.0 | 0.0 | 0.0 | 2440.6 | 4478.1 | 8041.5 | 0.0 | 0.0 | 0.0 |
| 1987 | 49.6 | 113.4 | 247.4 | 569.3 | 1055.5 | 1942.9 | 90.7 | 165.1 | 299.3 | 3161.5 | 5754.2 | 10288.7 | 17.3 | 32.5 | 60.5 |
| 1988 | 44.1 | 100.7 | 220.2 | 512.4 | 959.2 | 1778.5 | 182.0 | 331.5 | 597.9 | 3033.3 | 5526.5 | 9937.9 | 34.3 | 65.4 | 122.0 |
| 1989 | 29.0 | 66.6 | 145.0 | 348.6 | 647.6 | 1199.4 | 203.9 | 374.9 | 679.9 | 2178.7 | 3978.0 | 7150.2 | 38.8 | 74.2 | 137.5 |
| 1990 | 25.1 | 57.5 | 125.5 | 311.3 | 576.1 | 1059.9 | 274.7 | 499.5 | 913.3 | 2112.7 | 3807.6 | 6887.2 | 52.2 | 98.6 | 184.4 |
| 1991 | 16.1 | 37.0 | 80.8 | 210.0 | 388.0 | 713.3 | 266.4 | 487.1 | 869.9 | 1552.8 | 2819.4 | 5004.2 | 50.5 | 95.8 | 177.1 |
| 1992 | 12.1 | 27.9 | 60.7 | 169.4 | 313.6 | 572.7 | 300.9 | 549.9 | 999.8 | 1392.0 | 2538.3 | 4564.0 | 56.8 | 108.8 | 200.9 |
| 1993 | 7.5 | 16.9 | 36.9 | 112.3 | 208.2 | 380.6 | 283.2 | 517.4 | 931.5 | 1062.9 | 1940.4 | 3463.9 | 53.9 | 102.3 | 187.4 |
| 1994 | 5.9 | 13.3 | 29.1 | 104.6 | 193.6 | 356.2 | 385.8 | 700.3 | 1266.4 | 1190.8 | 2171.0 | 3899.9 | 72.6 | 138.5 | 255.8 |
| 1995 | 2.7 | 6.1 | 13.3 | 69.0 | 129.4 | 241.7 | 391.7 | 717.7 | 1308.7 | 1017.0 | 1860.2 | 3364.1 | 74.4 | 141.5 | 264.0 |
| 1996 | 0.0 | 0.0 | 0.0 | 36.0 | 70.1 | 135.4 | 379.3 | 694.8 | 1249.2 | 832.5 | 1525.6 | 2736.0 | 72.0 | 136.7 | 252.2 |
| 1997 | 0.0 | 0.0 | 0.0 | 47.1 | 92.0 | 177.7 | 497.7 | 907.0 | 1643.0 | 1089.3 | 1992.1 | 3582.6 | 94.6 | 178.6 | 334.2 |
| 1998 | 0.0 | 0.0 | 0.0 | 40.1 | 77.5 | 151.1 | 422.3 | 765.2 | 1391.5 | 927.2 | 1684.3 | 3035.4 | 79.8 | 150.6 | 281.2 |
| 1999 | 0.0 | 0.0 | 0.0 | 36.1 | 70.1 | 136.2 | 380.6 | 691.5 | 1256.1 | 824.8 | 1517.8 | 2750.4 | 72.3 | 137.2 | 254.4 |
| 2000 | 0.0 | 0.0 | 0.0 | 47.0 | 92.1 | 179.0 | 498.9 | 904.4 | 1641.3 | 1097.3 | 1984.7 | 3576.9 | 94.9 | 178.8 | 332.1 |
| 2001 | 0.0 | 0.0 | 0.0 | 38.0 | 74.1 | 144.0 | 402.0 | 728.8 | 1330.0 | 881.2 | 1603.8 | 2904.5 | 76.2 | 144.5 | 268.7 |
| 2002 | 0.0 | 0.0 | 0.0 | 41.8 | 81.3 | 157.6 | 443.1 | 801.8 | 1455.0 | 971.1 | 1762.9 | 3198.5 | 83.8 | 158.5 | 294.9 |
| 2003 | 0.0 | 0.0 | 0.0 | 28.3 | 55.7 | 108.1 | 299.7 | 549.9 | 994.5 | 659.4 | 1208.3 | 2182.0 | 57.7 | 108.6 | 200.1 |
| 2004 | 0.0 | 0.0 | 0.0 | 28.3 | 55.6 | 107.7 | 300.5 | 547.7 | 988.8 | 659.6 | 1203.8 | 2164.5 | 57.4 | 108.4 | 200.4 |
| 2005 | 0.0 | 0.0 | 0.0 | 36.3 | 70.7 | 136.3 | 383.3 | 697.9 | 1268.5 | 844.7 | 1534.2 | 2777.5 | 72.5 | 138.1 | 257.6 |
| 2006 | 0.0 | 0.0 | 0.0 | 36.3 | 70.6 | 138.1 | 382.0 | 699.1 | 1264.1 | 837.7 | 1532.9 | 2764.2 | 72.1 | 137.5 | 258.1 |
| 2007 | 0.0 | 0.0 | 0.0 | 39.7 | 77.2 | 149.9 | 420.0 | 762.5 | 1379.8 | 919.4 | 1679.5 | 3006.1 | 79.1 | 150.4 | 280.6 |
| 2008 | 0.0 | 0.0 | 0.0 | 41.7 | 80.9 | 157.0 | 437.7 | 795.8 | 1447.6 | 964.0 | 1744.2 | 3172.8 | 83.7 | 156.7 | 292.4 |
| 2009 | 0.0 | 0.0 | 0.0 | 43.6 | 85.1 | 165.5 | 466.6 | 842.2 | 1522.3 | 1019.7 | 1850.3 | 3325.9 | 88.3 | 166.6 | 307.8 |
| 2010 | 0.0 | 0.0 | 0.0 | 47.1 | 92.3 | 179.0 | 497.5 | 910.1 | 1646.3 | 1093.9 | 2001.7 | 3622.8 | 93.9 | 179.5 | 334.4 |
| 2011 | 0.0 | 0.0 | 0.0 | 55.0 | 107.3 | 208.4 | 583.3 | 1057.7 | 1914.8 | 1278.9 | 2327.8 | 4185.9 | 111.4 | 209.1 | 387.2 |
| 2012 | 0.0 | 0.0 | 0.0 | 54.3 | 105.7 | 205.6 | 574.6 | 1041.4 | 1882.9 | 1264.3 | 2287.1 | 4125.5 | 109.3 | 205.7 | 384.0 |
| 2013 | 0.0 | 0.0 | 0.0 | 52.5 | 102.8 | 198.5 | 555.9 | 1013.4 | 1829.6 | 1226.0 | 2222.1 | 3998.9 | 106.1 | 200.3 | 370.1 |
| 2014 | 0.0 | 0.0 | 0.0 | 52.2 | 102.4 | 199.1 | 557.7 | 1009.6 | 1832.0 | 1221.3 | 2226.0 | 4004.5 | 105.8 | 198.6 | 369.4 |
| 2015 | 0.0 | 0.0 | 0.0 | 55.2 | 107.1 | 208.1 | 580.7 | 1053.9 | 1917.7 | 1273.2 | 2316.8 | 4214.0 | 110.2 | 208.2 | 389.1 |
| 2016 | 0.0 | 0.0 | 0.0 | 52.5 | 103.2 | 199.0 | 560.8 | 1015.9 | 1838.0 | 1228.2 | 2232.5 | 4034.3 | 105.8 | 200.4 | 371.9 |
| 2017 | 0.0 | 0.0 | 0.0 | 64.8 | 125.6 | 244.0 | 680.8 | 1235.1 | 2242.6 | 1494.0 | 2705.4 | 4887.7 | 128.9 | 243.5 | 451.9 |
| 2018 | 0.0 | 0.0 | 0.0 | 60.7 | 117.9 | 228.7 | 639.2 | 1164.2 | 2103.8 | 1397.4 | 2557.0 | 4607.3 | 120.8 | 229.0 | 425.6 |
| 2019 | 0.0 | 0.0 | 0.0 | 64.4 | 125.2 | 241.9 | 679.3 | 1233.6 | 2234.2 | 1489.3 | 2701.2 | 4893.8 | 128.6 | 243.8 | 456.6 |
| 2020 | 0.0 | 0.0 | 0.0 | 89.7 | 173.6 | 338.7 | 940.7 | 1722.3 | 3109.2 | 2070.8 | 3763.3 | 6809.6 | 178.7 | 338.8 | 632.0 |
| 2021 | 0.0 | 0.0 | 0.0 | 123.5 | 242.5 | 469.9 | 1310.5 | 2395.1 | 4361.6 | 2881.1 | 5262.8 | 9489.6 | 249.5 | 474.6 | 880.3 |
| 2022 | 0.0 | 0.0 | 0.0 | 171.5 | 334.1 | 647.2 | 1811.5 | 3298.3 | 5980.1 | 3986.1 | 7248.0 | 13040.1 | 346.1 | 648.7 | 1208.1 |
| 2023 | 0.0 | 0.0 | 0.0 | 229.2 | 445.8 | 865.7 | 2421.8 | 4401.7 | 8015.6 | 5325.2 | 9676.3 | 17516.5 | 459.6 | 872.4 | 1613.7 |
| Total | 1174.6 | 2690.5 | 5857.0 | 15203.6 | 28429.2 | 52886.5 | 20487.3 | 37284.2 | 67606.9 | 115065.2 | 209286.2 | 377289.8 | 3890.1 | 7361.9 | 13678.7 |

Table S18. Annual metal demand for spacecraft solar panels (1957-2023). Includes uncertainty from subsystem mass fraction (triangular distribution), solar specific power (20-100 W/kg), and Dirichlet-sampled compositional shares. Values represent P05, P50, and P95 (kg/y). Part I.

| Year | In_P05 | In_P50 | In_P95 | Pb_P05 | Pb_P50 | Pb_P95 | Si_P05 | Si_P50 | Si_P95 | Sn_P05 | Sn_P50 | Sn_P95 | Zn_P05 | Zn_P50 | Zn_P95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 0.0 | 0.0 | 0.0 | 0.2 | 0.4 | 0.8 | 13.1 | 23.8 | 42.6 | 0.2 | 0.4 | 0.8 | 0.0 | 0.0 | 0.0 |
| 1958 | 0.0 | 0.0 | 0.0 | 2.9 | 5.7 | 11.0 | 176.7 | 320.4 | 576.3 | 2.7 | 5.4 | 10.5 | 0.0 | 0.0 | 0.0 |
| 1959 | 0.0 | 0.0 | 0.0 | 3.3 | 6.4 | 12.5 | 200.0 | 362.7 | 650.7 | 3.1 | 6.1 | 11.9 | 0.0 | 0.0 | 0.0 |
| 1960 | 0.0 | 0.0 | 0.0 | 16.1 | 31.7 | 62.0 | 984.0 | 1784.1 | 3216.8 | 15.2 | 30.1 | 58.9 | 0.0 | 0.0 | 0.0 |
| 1961 | 0.0 | 0.0 | 0.0 | 26.2 | 52.1 | 100.9 | 1610.9 | 2935.2 | 5285.4 | 25.0 | 49.4 | 96.0 | 0.0 | 0.0 | 0.0 |
| 1962 | 0.0 | 0.0 | 0.0 | 35.1 | 69.1 | 133.9 | 2157.3 | 3884.9 | 7005.4 | 33.0 | 65.3 | 128.0 | 0.0 | 0.0 | 0.0 |
| 1963 | 0.0 | 0.0 | 0.0 | 25.1 | 49.7 | 96.6 | 1539.7 | 2782.9 | 5016.0 | 23.9 | 47.0 | 92.0 | 0.0 | 0.0 | 0.0 |
| 1964 | 0.0 | 0.0 | 0.0 | 63.1 | 125.2 | 242.8 | 3856.1 | 7042.9 | 12635.1 | 60.1 | 119.1 | 229.8 | 0.0 | 0.0 | 0.0 |
| 1965 | 0.0 | 0.0 | 0.0 | 85.0 | 168.0 | 323.8 | 5244.8 | 9443.6 | 16975.1 | 81.0 | 159.2 | 311.9 | 0.0 | 0.0 | 0.0 |
| 1966 | 0.0 | 0.0 | 0.0 | 101.6 | 202.9 | 390.6 | 6249.3 | 11373.9 | 20464.1 | 97.2 | 191.6 | 373.0 | 0.0 | 0.0 | 0.0 |
| 1967 | 0.0 | 0.0 | 0.0 | 86.5 | 170.2 | 329.4 | 5304.2 | 9617.9 | 17257.8 | 81.8 | 161.6 | 316.7 | 0.0 | 0.0 | 0.0 |
| 1968 | 0.0 | 0.0 | 0.0 | 129.5 | 258.0 | 499.8 | 8002.2 | 14503.7 | 26082.7 | 123.6 | 244.2 | 476.8 | 0.0 | 0.0 | 0.0 |
| 1969 | 0.0 | 0.0 | 0.0 | 236.4 | 468.4 | 907.3 | 14524.9 | 26355.7 | 47280.0 | 224.5 | 443.9 | 862.4 | 0.0 | 0.0 | 0.0 |
| 1970 | 0.0 | 0.0 | 0.0 | 120.0 | 239.6 | 458.8 | 7424.7 | 13476.5 | 24179.8 | 114.9 | 228.1 | 439.9 | 0.0 | 0.0 | 0.0 |
| 1971 | 0.0 | 0.0 | 0.0 | 186.6 | 368.4 | 715.7 | 11476.3 | 20774.1 | 37469.6 | 178.7 | 350.6 | 681.8 | 0.0 | 0.0 | 0.0 |
| 1972 | 0.0 | 0.0 | 0.0 | 178.8 | 355.1 | 689.2 | 10980.4 | 19919.1 | 35800.0 | 170.1 | 335.7 | 655.3 | 0.0 | 0.0 | 0.0 |
| 1973 | 0.0 | 0.0 | 0.0 | 208.7 | 416.1 | 801.7 | 12802.7 | 23349.0 | 41940.3 | 199.7 | 393.3 | 772.6 | 0.0 | 0.0 | 0.0 |
| 1974 | 0.0 | 0.0 | 0.0 | 141.5 | 282.3 | 546.8 | 8746.8 | 15866.9 | 28393.6 | 134.5 | 268.2 | 519.4 | 0.0 | 0.0 | 0.0 |
| 1975 | 0.0 | 0.0 | 0.0 | 149.3 | 295.6 | 576.3 | 9145.1 | 16647.5 | 30049.6 | 142.3 | 279.8 | 546.2 | 0.0 | 0.0 | 0.0 |
| 1976 | 0.0 | 0.0 | 0.0 | 147.1 | 289.8 | 561.7 | 9001.4 | 16247.7 | 29134.0 | 139.5 | 275.9 | 535.1 | 0.0 | 0.0 | 0.0 |
| 1977 | 0.0 | 0.0 | 0.0 | 143.9 | 286.5 | 555.3 | 8881.2 | 16106.0 | 28861.1 | 136.2 | 270.9 | 528.6 | 0.0 | 0.0 | 0.0 |
| 1978 | 0.0 | 0.0 | 0.0 | 150.0 | 297.1 | 575.9 | 9221.6 | 16735.6 | 29915.6 | 142.7 | 281.4 | 549.5 | 0.0 | 0.0 | 0.0 |
| 1979 | 0.0 | 0.0 | 0.0 | 140.9 | 280.3 | 544.8 | 8628.2 | 15784.8 | 28193.9 | 133.6 | 265.3 | 513.8 | 0.0 | 0.0 | 0.0 |
| 1980 | 0.0 | 0.0 | 0.0 | 147.4 | 291.7 | 564.6 | 9100.2 | 16426.7 | 29375.1 | 140.2 | 275.9 | 541.5 | 0.0 | 0.0 | 0.0 |
| 1981 | 0.0 | 0.0 | 0.0 | 155.0 | 307.3 | 594.0 | 9534.5 | 17287.1 | 31018.4 | 147.4 | 292.1 | 566.5 | 0.0 | 0.0 | 0.0 |
| 1982 | 0.0 | 0.0 | 0.0 | 165.3 | 327.3 | 634.7 | 10195.6 | 18450.8 | 33136.5 | 157.1 | 312.3 | 605.5 | 0.0 | 0.0 | 0.0 |
| 1983 | 0.0 | 0.0 | 0.0 | 153.5 | 302.6 | 587.2 | 9397.2 | 16999.2 | 30717.6 | 145.2 | 286.2 | 561.1 | 0.0 | 0.0 | 0.0 |
| 1984 | 0.0 | 0.0 | 0.0 | 164.1 | 326.2 | 633.4 | 10127.3 | 18297.7 | 32992.1 | 156.3 | 309.4 | 603.3 | 0.0 | 0.0 | 0.0 |
| 1985 | 0.0 | 0.0 | 0.0 | 164.2 | 327.0 | 632.6 | 10165.3 | 18398.7 | 33051.5 | 156.3 | 310.5 | 601.9 | 0.0 | 0.0 | 0.0 |
| 1986 | 0.0 | 0.0 | 0.0 | 146.0 | 288.9 | 556.0 | 8939.6 | 16258.8 | 29181.3 | 139.5 | 275.1 | 531.8 | 0.0 | 0.0 | 0.0 |
| 1987 | 4.4 | 9.1 | 18.5 | 188.9 | 369.7 | 709.1 | 12384.2 | 22441.4 | 40145.9 | 180.2 | 352.3 | 678.1 | 0.0 | 0.0 | 0.1 |
| 1988 | 8.8 | 18.3 | 37.3 | 182.3 | 356.6 | 684.5 | 12803.3 | 23255.0 | 41603.1 | 175.9 | 339.5 | 649.0 | 0.0 | 0.0 | 0.3 |
| 1989 | 10.0 | 20.7 | 42.2 | 133.6 | 257.5 | 486.5 | 9933.3 | 18071.7 | 32399.0 | 127.8 | 246.1 | 466.6 | 0.0 | 0.0 | 0.3 |
| 1990 | 13.5 | 27.6 | 56.4 | 129.4 | 246.7 | 465.0 | 10318.9 | 18617.8 | 33635.0 | 125.0 | 235.4 | 444.1 | 0.0 | 0.0 | 0.4 |
| 1991 | 13.0 | 26.8 | 54.6 | 95.8 | 182.1 | 338.2 | 8213.5 | 14919.2 | 26521.3 | 92.2 | 174.9 | 322.7 | 0.0 | 0.0 | 0.4 |
| 1992 | 14.8 | 30.4 | 61.8 | 86.4 | 163.7 | 302.5 | 7971.6 | 14543.0 | 26040.5 | 83.7 | 157.3 | 290.9 | 0.0 | 0.0 | 0.4 |
| 1993 | 13.8 | 28.6 | 58.4 | 66.2 | 124.4 | 230.2 | 6602.8 | 12014.6 | 21432.5 | 64.4 | 120.6 | 221.6 | 0.0 | 0.0 | 0.4 |
| 1994 | 18.8 | 38.7 | 78.6 | 74.5 | 139.1 | 258.4 | 8047.2 | 14626.9 | 26211.8 | 71.8 | 134.5 | 249.7 | 0.0 | 0.0 | 0.5 |
| 1995 | 19.2 | 39.7 | 81.6 | 62.5 | 119.0 | 223.6 | 7482.9 | 13654.5 | 24713.0 | 61.1 | 116.3 | 216.4 | 0.0 | 0.0 | 0.6 |
| 1996 | 18.5 | 38.2 | 77.6 | 50.5 | 96.6 | 182.3 | 6721.2 | 12227.2 | 21827.8 | 49.0 | 94.0 | 177.1 | 0.0 | 0.0 | 0.5 |
| 1997 | 24.1 | 50.2 | 103.0 | 66.5 | 126.8 | 240.8 | 8813.4 | 15972.4 | 28736.6 | 64.7 | 123.5 | 233.6 | 0.0 | 0.0 | 0.7 |
| 1998 | 20.5 | 42.3 | 86.3 | 56.2 | 106.9 | 203.0 | 7438.1 | 13463.8 | 24182.3 | 54.7 | 103.8 | 197.0 | 0.0 | 0.0 | 0.6 |
| 1999 | 18.4 | 38.4 | 79.0 | 50.5 | 96.5 | 182.7 | 6699.9 | 12215.3 | 21971.3 | 49.0 | 94.1 | 177.8 | 0.0 | 0.0 | 0.5 |
| 2000 | 24.1 | 50.3 | 101.6 | 66.5 | 127.1 | 240.0 | 8834.1 | 15915.5 | 28536.4 | 64.8 | 123.6 | 233.8 | 0.0 | 0.0 | 0.7 |
| 2001 | 19.6 | 40.5 | 82.7 | 53.7 | 102.1 | 193.8 | 7105.7 | 12839.9 | 23155.7 | 52.1 | 99.5 | 187.9 | 0.0 | 0.0 | 0.6 |
| 2002 | 21.5 | 44.5 | 91.2 | 58.8 | 112.2 | 211.2 | 7838.6 | 14138.1 | 25432.1 | 56.9 | 109.3 | 206.6 | 0.0 | 0.0 | 0.6 |
| 2003 | 14.7 | 30.4 | 61.9 | 40.1 | 76.8 | 145.7 | 5279.4 | 9668.7 | 17321.2 | 38.9 | 74.5 | 140.5 | 0.0 | 0.0 | 0.4 |
| 2004 | 14.6 | 30.3 | 62.0 | 40.0 | 76.4 | 144.9 | 5311.5 | 9615.8 | 17329.6 | 39.0 | 74.6 | 140.1 | 0.0 | 0.0 | 0.4 |
| 2005 | 18.8 | 38.7 | 78.7 | 51.1 | 97.6 | 184.7 | 6778.6 | 12309.8 | 22204.4 | 49.8 | 95.4 | 179.6 | 0.0 | 0.0 | 0.6 |
| 2006 | 18.5 | 38.6 | 78.6 | 50.7 | 97.5 | 185.3 | 6739.1 | 12271.2 | 22061.5 | 49.6 | 95.0 | 179.0 | 0.0 | 0.0 | 0.6 |
| 2007 | 20.4 | 42.3 | 86.0 | 55.7 | 106.8 | 201.7 | 7389.0 | 13455.8 | 24142.1 | 54.7 | 104.0 | 195.5 | 0.0 | 0.0 | 0.6 |
| 2008 | 21.5 | 44.0 | 90.7 | 58.6 | 111.7 | 210.8 | 7776.0 | 14028.3 | 25272.0 | 56.9 | 108.5 | 205.0 | 0.0 | 0.0 | 0.6 |
| 2009 | 22.5 | 46.6 | 94.8 | 61.4 | 117.3 | 222.0 | 8235.6 | 14796.1 | 26630.9 | 60.1 | 114.6 | 215.4 | 0.0 | 0.0 | 0.6 |
| 2010 | 24.4 | 50.4 | 103.3 | 66.1 | 126.7 | 241.6 | 8799.5 | 16039.9 | 28819.7 | 64.3 | 123.1 | 233.7 | 0.0 | 0.0 | 0.7 |
| 2011 | 28.5 | 59.1 | 119.3 | 77.7 | 148.1 | 280.8 | 10298.0 | 18683.7 | 33654.0 | 75.4 | 144.2 | 272.5 | 0.0 | 0.0 | 0.8 |
| 2012 | 27.9 | 57.6 | 117.9 | 76.3 | 145.9 | 275.5 | 10134.9 | 18296.8 | 33034.6 | 74.6 | 141.2 | 268.3 | 0.0 | 0.0 | 0.8 |
| 2013 | 27.1 | 56.3 | 114.4 | 73.9 | 141.6 | 267.3 | 9866.2 | 17807.5 | 31921.5 | 72.5 | 137.7 | 259.0 | 0.0 | 0.0 | 0.8 |
| 2014 | 27.1 | 55.9 | 114.1 | 74.2 | 141.4 | 267.3 | 9835.8 | 17812.9 | 31939.1 | 71.9 | 137.3 | 259.9 | 0.0 | 0.0 | 0.8 |
| 2015 | 28.2 | 58.2 | 119.7 | 77.0 | 147.2 | 281.9 | 10239.6 | 18521.4 | 33695.9 | 75.2 | 143.5 | 273.5 | 0.0 | 0.0 | 0.8 |
| 2016 | 27.3 | 56.0 | 114.6 | 73.8 | 141.9 | 269.4 | 9882.1 | 17870.1 | 32220.3 | 72.5 | 138.0 | 260.1 | 0.0 | 0.0 | 0.8 |
| 2017 | 33.0 | 68.6 | 139.6 | 90.1 | 173.6 | 325.5 | 12032.1 | 21728.9 | 39139.8 | 88.2 | 168.7 | 316.9 | 0.0 | 0.0 | 1.0 |
| 2018 | 31.1 | 64.6 | 130.6 | 84.5 | 162.9 | 307.2 | 11260.6 | 20457.7 | 36779.3 | 82.7 | 158.0 | 297.9 | 0.0 | 0.0 | 0.9 |
| 2019 | 33.1 | 68.3 | 139.4 | 90.4 | 172.5 | 328.6 | 12004.1 | 21691.0 | 39110.6 | 87.9 | 167.3 | 317.8 | 0.0 | 0.0 | 1.0 |
| 2020 | 45.7 | 95.2 | 193.7 | 125.2 | 240.5 | 457.3 | 16639.3 | 30177.0 | 54369.0 | 122.6 | 234.0 | 441.6 | 0.0 | 0.0 | 1.4 |
| 2021 | 64.4 | 133.1 | 271.4 | 174.6 | 334.7 | 637.4 | 23208.6 | 42177.1 | 75922.6 | 170.7 | 325.5 | 612.4 | 0.0 | 0.0 | 1.9 |
| 2022 | 88.3 | 183.0 | 373.6 | 242.5 | 461.6 | 873.6 | 31974.5 | 57949.8 | 104199.7 | 235.6 | 448.1 | 846.6 | 0.0 | 0.0 | 2.6 |
| 2023 | 118.4 | 245.0 | 496.3 | 320.9 | 617.2 | 1165.8 | 42932.2 | 77483.8 | 139562.7 | 315.7 | 598.6 | 1134.1 | 0.0 | 0.0 | 3.4 |
| Total | 998.6 | 2066.6 | 4211.5 | 6900.6 | 13456.5 | 25766.4 | 617456.8 | 1119217.3 | 2011772.4 | 6637.7 | 12890.6 | 24725.0 | 0.0 | 0.0 | 29.4 |

Sensitivity analysis for solar panels

Table S19 reports cumulative masses (kg) of all modeled metals (Ag, Al, As, Cu, Ga, In, Pb, Si, Sn, Zn) across four specific-power scenarios (20, 50, 70, 100 W/kg). Metal intensities were derived from Monte Carlo simulations (N=10,000) using Dirichlet-sampled mass fractions. Changes in cumulative demand reflect the sensitivity of total panel area—and thus material use—to specific-power assumptions. Values represent deterministic recalculations using historical spacecraft power requirements; no additional uncertainty sampling was applied in this step.

Cumulative masses of metals contained in solar-panel assemblies were recalculated across a range of assumed spacecraft specific power values (20-100 W/kg). Higher specific power implies larger solar-array areas to meet historical power budgets, thereby increasing total material demand. Aluminum shows the dominant sensitivity due to its role in panel frames and structural support, while metals such as Cu, Pb, Si, Sn, and Zn exhibit moderate increases. Critical metals with low baseline shares (Ag, Ga, In, As) show comparatively small absolute changes. Results reflect cumulative deployments from 1957-2023, using metal‐intensity coefficients drawn from the Monte Carlo Dirichlet composition model.

Table S19. Cumulative metal demand in spacecraft solar panels (1957–2023) under varying specific-power assumptions (20-100 W/kg).

| Scenario | Ag | Al | As | Cu | Ga | In | Pb | Si | Sn | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W/kg | t |  |  |  |  |  |  |  |  |  |
| 20 | 1.21 | 12.23 | 15.96 | 89.39 | 3.17 | 0.91 | 5.82 | 477.20 | 5.59 | 0.00 |
| 30 | 1.81 | 18.35 | 23.93 | 134.09 | 4.75 | 1.37 | 8.73 | 715.80 | 8.38 | 0.00 |
| 40 | 2.42 | 24.46 | 31.91 | 178.78 | 6.33 | 1.83 | 11.64 | 954.40 | 11.17 | 0.00 |
| 50 | 3.02 | 30.58 | 39.89 | 223.48 | 7.92 | 2.28 | 14.55 | 1193.00 | 13.97 | 0.01 |
| 60 | 3.63 | 36.70 | 47.87 | 268.18 | 9.50 | 2.74 | 17.46 | 1431.60 | 16.76 | 0.01 |
| 80 | 4.83 | 48.93 | 63.82 | 357.57 | 12.67 | 3.65 | 23.28 | 1908.80 | 22.35 | 0.01 |
| 100 | 6.04 | 61.16 | 79.78 | 446.96 | 15.83 | 4.57 | 29.10 | 2386.00 | 27.94 | 0.01 |

Figure S8.  Sensitivity of cumulative solar-panel metal demand to spacecraft specific power (1957-2023).

Batteries in spacecraft

| Data: Batteries account for 15% of the dry mass of a typical communications spacecraft and nearly half the mass of the entire payload. |  |  |  |
| --- | --- | --- | --- |
| Reference: ESA | https://www.esa.int/esapub/bulletin/bullet90/b90dudle.htm |  |  |
| Data: Typically, one-third of the total weight of the spacecraft is taken by the power subsystem, with the batteries weighing one-third of the power system (this is nearly 11% of the total spacecraft's weight) |  |  |  |
| Reference: NASA | https://ntrs.nasa.gov/api/citations/19960054145/downloads/19960054145.pdf |  |  |
| Data: Batteries and battery parts alone can comprise 10-20% of the satellite’s total weight. |  |  |  |
| Reference: SatMagazine |  | http://www.satmagazine.com/story.php?number=464410536 |  |
| General information about spacecraft's batteries |  |  |  |
| https://blog.satsearch.co/2021-06-23-satellite-batteries-for-cubesats-nanosats-and-other-form-factors |  |  |  |

Table S20. Cumulative Metal intensity in Ag-Zn batteries.

|  | Ag-Zn (wt. %) |  |
| --- | --- | --- |
|  | Ref.63 | Data selected |
| Ag | 20-30 | 20 |
| Zn | 70-80 | 70 |
| Electrolyte | <5 |  |

Table S21. Metal intensity in Ni-Cd batteries.

|  | Ni-Cd (wt. %) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Ref.64 | Ref.65 | Ref.66 | Ref.67 | Data selected |
| Ni | 51 | 52 | 61.3 | 44.5 | 52.2 |
| Cd | 25 | 53.5 | 21.5 | 30 | 32.5 |
| Co | 0.9 | 3 | 0.5 | 3.3 | 1.9 |

Table S22. Metal intensity in Ni-H2 batteries.

|  | Ni-H2 (wt. %) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ref.a | Ref.b | Ref.c | Data selected |
| Ni (Ni[OH]2) | 30-40 | 30-40 |  | 35 |
| Hydrogen-Absorbing Alloy (e.g., AB5 or AB2 alloy) | 50-60 |  |  | 50 |
| Electrolyte | 26 |  | 26 | 26 |

Nickel–hydrogen cell technology started to become obsolete in the 2000s with the introduction of lithium-ion battery technology that had significantly better specific energy, energy density, lower self-discharge rates, and higher columbic efficiency68.

References for Table S22:

| a | https://ntrs.nasa.gov/api/citations/19950020757/downloads/19950020757.pdf |
| --- | --- |
| b | https://ntrs.nasa.gov/api/citations/19950010446/downloads/19950010446.pdf |
| c | https://www.powerstream.com/BatteryFAQ-nickel-hydrogen.html |

Table S23. Metal intensity in Li-ion batteries.

|  | Li-ion (wt.%)/Reference |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 69 | 67 | 70 |  |  |  |  | 71 | 72 | 73 | 74 | 75 | Data selected | SD |
| Li | 2-3 | 2-3 | 1.9 | 1.4 | 1.3 | 2 | 1.2 | 1.91 | 6.0 | 2.6 | 4.2 | 2.5 | 2.5 | 1.5 |
| Co | 22-28 | 22-26 | 2.3 | 0 | 8.5 | 17.3 | 0 | 0.002 | 7.4 | 6.4 | 18.1 | 1.8 | 15.8 | 9.1 |
| Ni | <1 | 11 | 12 | 0 | 15 | 1 | 0 | 0.1 | 17.6 | 7.7 | 12.3 | 6.4 | 7.6 | 6.5 |
| Fe | 5-6 | <1 | <1 | <1 | 9 | 17 | 43 | 11.3 | - | - | 0.4 | 0.6 | 13.6 | 15.8 |
| Cu | <1 | 2-12 | 13 | 14 | 17 | 7 | 8 | 0.16 | 17.2 | 7.9 | 2.1 | 8.1 | 9.4 | 5.8 |
| Al | <1 | 1-3 | 23 | 22 | 23 | 5 | 7 | 0.25 | 12.5 | - | 2.8 | 4.8 | 11.2 | 9.2 |

Figure S9. Probability distribution of battery subsystem mass fractions sampled using a triangular distribution (min=5%, mode=12.5%, max=20%) in a Monte Carlo framework (N=10,000).

This reflects historical and modern variation in on-board energy storage demands.

Figure S10. Ag-Zn battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Ag-Zn battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.

Figure S11. Ni-Cd battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Ni-Cd battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.

Figure S12. Ni-H2 battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Ni-H2 battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.

Figure S13. Li-ion battery metal composition (Monte Carlo, N=10,000). Monte Carlo distributions of Li-ion battery metal shares generated using a Dirichlet model calibrated to literature mean compositions.

Table S24. Evolution of battery technologies over time.

| Year | Ag-Zn | Ni-Cd | Ni-H2 | Li-ion | Year | Ag-Zn | Ni-Cd | Ni-H2 | Li-ion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| % |  |  |  |  |  |  |  |  |  |
| 1957 | 100 | 0 | 0 | 0 | 1996 | 0 | 0 | 100 | 0 |
| 1958 | 100 | 0 | 0 | 0 | 1997 | 0 | 0 | 100 | 0 |
| 1959 | 100 | 0 | 0 | 0 | 1998 | 0 | 0 | 100 | 0 |
| 1960 | 100 | 0 | 0 | 0 | 1999 | 0 | 0 | 100 | 0 |
| 1961 | 100 | 0 | 0 | 0 | 2000 | 0 | 0 | 100 | 0 |
| 1962 | 90 | 10 | 0 | 0 | 2001 | 0 | 0 | 100 | 0 |
| 1963 | 80 | 20 | 0 | 0 | 2002 | 0 | 0 | 90 | 10 |
| 1964 | 70 | 30 | 0 | 0 | 2003 | 0 | 0 | 80 | 20 |
| 1965 | 60 | 40 | 0 | 0 | 2004 | 0 | 0 | 70 | 30 |
| 1966 | 50 | 50 | 0 | 0 | 2005 | 0 | 0 | 60 | 40 |
| 1967 | 40 | 60 | 0 | 0 | 2006 | 0 | 0 | 50 | 50 |
| 1968 | 30 | 70 | 0 | 0 | 2007 | 0 | 0 | 40 | 60 |
| 1969 | 20 | 80 | 0 | 0 | 2008 | 0 | 0 | 30 | 70 |
| 1970 | 10 | 90 | 0 | 0 | 2009 | 0 | 0 | 20 | 80 |
| 1971 | 0 | 100 | 0 | 0 | 2010 | 0 | 0 | 10 | 90 |
| 1972 | 0 | 100 | 0 | 0 | 2011 | 0 | 0 | 0 | 100 |
| 1973 | 0 | 100 | 0 | 0 | 2012 | 0 | 0 | 0 | 100 |
| 1974 | 0 | 100 | 0 | 0 | 2013 | 0 | 0 | 0 | 100 |
| 1975 | 0 | 100 | 0 | 0 | 2014 | 0 | 0 | 0 | 100 |
| 1976 | 0 | 100 | 0 | 0 | 2015 | 0 | 0 | 0 | 100 |
| 1977 | 0 | 100 | 0 | 0 | 2016 | 0 | 0 | 0 | 100 |
| 1978 | 0 | 100 | 0 | 0 | 2017 | 0 | 0 | 0 | 100 |
| 1979 | 0 | 100 | 0 | 0 | 2018 | 0 | 0 | 0 | 100 |
| 1980 | 0 | 100 | 0 | 0 | 2019 | 0 | 0 | 0 | 100 |
| 1981 | 0 | 100 | 0 | 0 | 2020 | 0 | 0 | 0 | 100 |
| 1982 | 0 | 90 | 10 | 0 | 2021 | 0 | 0 | 0 | 100 |
| 1983 | 0 | 80 | 20 | 0 | 2022 | 0 | 0 | 0 | 100 |
| 1984 | 0 | 70 | 30 | 0 | 2023 | 0 | 0 | 0 | 100 |
| 1985 | 0 | 60 | 40 | 0 |  |  |  |  |  |
| 1986 | 0 | 50 | 50 | 0 |  |  |  |  |  |
| 1987 | 0 | 40 | 60 | 0 |  |  |  |  |  |
| 1988 | 0 | 30 | 70 | 0 |  |  |  |  |  |
| 1989 | 0 | 20 | 80 | 0 |  |  |  |  |  |
| 1990 | 0 | 10 | 90 | 0 |  |  |  |  |  |
| 1991 | 0 | 0 | 100 | 0 |  |  |  |  |  |
| 1992 | 0 | 0 | 100 | 0 |  |  |  |  |  |
| 1993 | 0 | 0 | 100 | 0 |  |  |  |  |  |
| 1994 | 0 | 0 | 100 | 0 |  |  |  |  |  |
| 1995 | 0 | 0 | 100 | 0 |  |  |  |  |  |

Table S25. Annual metal demand from all battery chemistries combined (Ag-Zn, Ni-Cd, Ni-H2, Li-ion), weighted by historical adoption. Values are reported as P05, P50, and P95 (kg/y). Part I.

| Year | Ag_P05 | Ag_P50 | Ag_P95 | Al_P05 | Al_P50 | Al_P95 | Cd_P05 | Cd_P50 | Cd_P95 | Co_P05 | Co_P50 | Co_P95 | Cu_P05 | Cu_P50 | Cu_P95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 6.28 | 13.88 | 25.86 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1958 | 84.88 | 186.70 | 352.71 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1959 | 96.64 | 211.71 | 402.82 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1960 | 469.62 | 1051.20 | 1990.57 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1961 | 788.89 | 1704.74 | 3231.25 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1962 | 1048.93 | 2265.91 | 4274.48 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1963 | 746.79 | 1651.92 | 3046.25 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1964 | 1868.85 | 4068.31 | 7724.53 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1965 | 2288.45 | 5004.05 | 9413.46 | 0.00 | 0.00 | 0.00 | 505.91 | 933.26 | 1508.28 | 2.63 | 37.66 | 175.14 | 0.00 | 0.00 | 0.00 |
| 1966 | 2446.36 | 5279.17 | 9914.85 | 0.00 | 0.00 | 0.00 | 1209.53 | 2223.93 | 3616.15 | 6.06 | 90.22 | 413.78 | 0.00 | 0.00 | 0.00 |
| 1967 | 1793.18 | 3939.37 | 7384.93 | 0.00 | 0.00 | 0.00 | 1527.08 | 2825.30 | 4583.62 | 7.95 | 115.52 | 533.20 | 0.00 | 0.00 | 0.00 |
| 1968 | 2323.85 | 5054.92 | 9489.88 | 0.00 | 0.00 | 0.00 | 3040.27 | 5710.59 | 9250.92 | 15.65 | 225.33 | 1050.04 | 0.00 | 0.00 | 0.00 |
| 1969 | 3483.85 | 7665.13 | 14524.14 | 0.00 | 0.00 | 0.00 | 6977.20 | 12874.57 | 20813.53 | 34.30 | 518.10 | 2413.54 | 0.00 | 0.00 | 0.00 |
| 1970 | 1441.92 | 3133.40 | 5956.15 | 0.00 | 0.00 | 0.00 | 4257.20 | 7910.31 | 12866.15 | 22.43 | 320.32 | 1484.83 | 0.00 | 0.00 | 0.00 |
| 1971 | 1683.00 | 3624.10 | 6850.79 | 0.00 | 0.00 | 0.00 | 7664.53 | 14165.24 | 23024.68 | 37.77 | 564.65 | 2642.37 | 0.00 | 0.00 | 0.00 |
| 1972 | 1044.20 | 2351.95 | 4388.57 | 0.00 | 0.00 | 0.00 | 8479.72 | 15552.87 | 25419.53 | 43.04 | 625.07 | 2935.47 | 0.00 | 0.00 | 0.00 |
| 1973 | 613.39 | 1374.07 | 2602.94 | 0.00 | 0.00 | 0.00 | 11132.43 | 20542.10 | 33072.13 | 58.01 | 839.84 | 3819.83 | 0.00 | 0.00 | 0.00 |
| 1974 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8532.06 | 15569.18 | 25285.23 | 44.67 | 629.63 | 2898.80 | 0.00 | 0.00 | 0.00 |
| 1975 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8824.64 | 16347.52 | 26235.01 | 45.45 | 671.95 | 3021.62 | 0.00 | 0.00 | 0.00 |
| 1976 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8701.12 | 15993.06 | 26003.85 | 46.40 | 646.26 | 2984.91 | 0.00 | 0.00 | 0.00 |
| 1977 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8423.06 | 15706.66 | 25540.01 | 43.14 | 629.40 | 2958.32 | 0.00 | 0.00 | 0.00 |
| 1978 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8893.72 | 16371.10 | 26452.90 | 44.79 | 660.39 | 3034.69 | 0.00 | 0.00 | 0.00 |
| 1979 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8318.95 | 15271.54 | 24651.03 | 41.35 | 605.94 | 2898.83 | 0.00 | 0.00 | 0.00 |
| 1980 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8730.14 | 16042.66 | 25973.41 | 44.50 | 644.41 | 3033.52 | 0.00 | 0.00 | 0.00 |
| 1981 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 9129.91 | 16957.13 | 27292.84 | 49.22 | 652.50 | 3073.61 | 0.00 | 0.00 | 0.00 |
| 1982 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 9852.56 | 17927.86 | 29065.62 | 48.65 | 739.62 | 3328.86 | 0.00 | 0.00 | 0.00 |
| 1983 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8942.34 | 16705.75 | 26909.16 | 48.16 | 664.67 | 3237.30 | 0.00 | 0.00 | 0.00 |
| 1984 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 9726.31 | 18031.56 | 29006.05 | 53.53 | 726.66 | 3382.97 | 0.00 | 0.00 | 0.00 |
| 1985 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8930.29 | 16327.68 | 26309.08 | 44.27 | 662.45 | 3012.43 | 0.00 | 0.00 | 0.00 |
| 1986 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 6937.57 | 12766.57 | 20581.55 | 33.98 | 501.84 | 2344.92 | 0.00 | 0.00 | 0.00 |
| 1987 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8093.73 | 14926.76 | 24101.87 | 39.66 | 600.96 | 2732.57 | 0.00 | 0.00 | 0.00 |
| 1988 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 6882.22 | 12817.83 | 20858.88 | 34.84 | 513.53 | 2373.07 | 0.00 | 0.00 | 0.00 |
| 1989 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4358.93 | 8063.29 | 12941.91 | 22.91 | 320.58 | 1492.33 | 0.00 | 0.00 | 0.00 |
| 1990 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 3537.62 | 6444.78 | 10512.99 | 19.12 | 260.03 | 1221.47 | 0.00 | 0.00 | 0.00 |
| 1991 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 2035.18 | 3744.82 | 6042.56 | 10.41 | 148.06 | 685.24 | 0.00 | 0.00 | 0.00 |
| 1992 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1299.92 | 2360.98 | 3816.90 | 7.18 | 96.47 | 453.93 | 0.00 | 0.00 | 0.00 |
| 1993 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 510.01 | 945.12 | 1529.52 | 2.54 | 38.06 | 174.16 | 0.00 | 0.00 | 0.00 |
| 1994 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1995 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1996 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1997 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1998 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1999 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2000 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2001 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2002 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2003 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2004 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2005 | 0.00 | 0.00 | 0.00 | 37.17 | 252.50 | 814.79 | 0.00 | 0.00 | 0.00 | 5.49 | 115.70 | 572.37 | 21.87 | 198.28 | 742.81 |
| 2006 | 0.00 | 0.00 | 0.00 | 75.25 | 508.69 | 1626.18 | 0.00 | 0.00 | 0.00 | 10.55 | 226.95 | 1130.70 | 44.40 | 408.03 | 1487.41 |
| 2007 | 0.00 | 0.00 | 0.00 | 123.75 | 817.16 | 2742.03 | 0.00 | 0.00 | 0.00 | 16.72 | 368.40 | 1919.40 | 72.52 | 649.67 | 2366.35 |
| 2008 | 0.00 | 0.00 | 0.00 | 170.14 | 1168.46 | 3709.53 | 0.00 | 0.00 | 0.00 | 24.00 | 507.27 | 2633.70 | 106.43 | 934.53 | 3345.08 |
| 2009 | 0.00 | 0.00 | 0.00 | 227.72 | 1535.11 | 4974.03 | 0.00 | 0.00 | 0.00 | 31.58 | 673.73 | 3426.15 | 131.18 | 1225.69 | 4413.61 |
| 2010 | 0.00 | 0.00 | 0.00 | 286.98 | 1994.20 | 6416.61 | 0.00 | 0.00 | 0.00 | 41.82 | 885.54 | 4532.93 | 184.45 | 1565.67 | 5749.47 |
| 2011 | 0.00 | 0.00 | 0.00 | 399.25 | 2646.83 | 8825.89 | 0.00 | 0.00 | 0.00 | 53.06 | 1204.78 | 6042.93 | 232.43 | 2164.17 | 7854.06 |
| 2012 | 0.00 | 0.00 | 0.00 | 435.99 | 3018.79 | 9900.64 | 0.00 | 0.00 | 0.00 | 64.09 | 1340.68 | 6698.77 | 260.34 | 2501.25 | 8989.51 |
| 2013 | 0.00 | 0.00 | 0.00 | 499.38 | 3324.12 | 10674.29 | 0.00 | 0.00 | 0.00 | 66.99 | 1477.20 | 7525.75 | 304.90 | 2676.30 | 9637.82 |
| 2014 | 0.00 | 0.00 | 0.00 | 570.77 | 3594.82 | 11866.69 | 0.00 | 0.00 | 0.00 | 76.29 | 1604.69 | 8165.66 | 334.12 | 2945.17 | 10620.26 |
| 2015 | 0.00 | 0.00 | 0.00 | 521.91 | 3777.60 | 12542.53 | 0.00 | 0.00 | 0.00 | 85.63 | 1748.31 | 8467.05 | 363.99 | 3143.42 | 11242.58 |
| 2016 | 0.00 | 0.00 | 0.00 | 552.55 | 3766.43 | 12203.34 | 0.00 | 0.00 | 0.00 | 78.74 | 1621.81 | 8273.37 | 344.45 | 2934.00 | 10823.67 |
| 2017 | 0.00 | 0.00 | 0.00 | 658.12 | 4579.20 | 14803.17 | 0.00 | 0.00 | 0.00 | 86.95 | 1958.81 | 10226.67 | 393.12 | 3484.78 | 12965.46 |
| 2018 | 0.00 | 0.00 | 0.00 | 632.22 | 4235.97 | 13840.10 | 0.00 | 0.00 | 0.00 | 83.40 | 1852.41 | 9340.16 | 366.55 | 3343.12 | 12385.93 |
| 2019 | 0.00 | 0.00 | 0.00 | 636.74 | 4409.90 | 14784.22 | 0.00 | 0.00 | 0.00 | 95.50 | 2041.00 | 10227.45 | 409.81 | 3552.62 | 13384.08 |
| 2020 | 0.00 | 0.00 | 0.00 | 922.37 | 6197.22 | 20536.51 | 0.00 | 0.00 | 0.00 | 128.54 | 2739.16 | 13689.51 | 576.38 | 4948.10 | 17974.84 |
| 2021 | 0.00 | 0.00 | 0.00 | 1302.94 | 8654.94 | 28624.46 | 0.00 | 0.00 | 0.00 | 169.22 | 3893.58 | 19194.49 | 799.93 | 6881.36 | 25043.70 |
| 2022 | 0.00 | 0.00 | 0.00 | 1770.24 | 12041.88 | 38893.70 | 0.00 | 0.00 | 0.00 | 296.63 | 5409.27 | 26788.33 | 1086.68 | 9651.44 | 35598.78 |
| 2023 | 0.00 | 0.00 | 0.00 | 2257.68 | 15909.41 | 52466.19 | 0.00 | 0.00 | 0.00 | 333.43 | 7111.14 | 36847.45 | 1418.58 | 12556.92 | 47174.95 |
| Total | 22229.1 | 48580.5 | 91574.2 | 12081.2 | 82433.2 | 270244.9 | 185454.2 | 342060.0 | 553265.3 | 2701.2 | 50530.5 | 249514.6 | 7452.1 | 65764.5 | 241800.4 |

Table S26. Annual metal demand from all battery chemistries combined (Ag-Zn, Ni-Cd, Ni-H2, Li-ion), weighted by historical adoption. Values are reported as P05, P50, and P95 (kg/y). Part II.

| Year | Fe_P05 | Fe_P50 | Fe_P95 | Li_P05 | Li_P50 | Li_P95 | Ni_P05 | Ni_P50 | Ni_P95 | Zn_P05 | Zn_P50 | Zn_P95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 29.95 | 50.83 | 75.35 |
| 1958 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 399.15 | 688.73 | 1025.78 |
| 1959 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 446.71 | 780.15 | 1151.41 |
| 1960 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 2191.05 | 3833.51 | 5702.53 |
| 1961 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 3617.92 | 6286.84 | 9288.56 |
| 1962 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4872.39 | 8393.21 | 12334.45 |
| 1963 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 3472.68 | 6020.24 | 8833.14 |
| 1964 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8686.51 | 15138.95 | 22373.13 |
| 1965 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 862.43 | 1521.78 | 2281.73 | 10574.71 | 18454.19 | 27095.38 |
| 1966 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 2064.23 | 3623.26 | 5459.52 | 11239.07 | 19587.60 | 28793.56 |
| 1967 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 2595.30 | 4620.61 | 6932.55 | 8333.45 | 14548.98 | 21477.74 |
| 1968 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5289.98 | 9273.04 | 13995.41 | 10768.51 | 18781.70 | 27844.12 |
| 1969 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 11974.99 | 21071.15 | 31928.15 | 16302.30 | 28374.17 | 42016.10 |
| 1970 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 7330.42 | 12843.02 | 19434.89 | 6663.06 | 11529.69 | 17101.27 |
| 1971 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 13159.29 | 23200.38 | 34989.42 | 7705.13 | 13443.07 | 19779.42 |
| 1972 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14489.26 | 25485.97 | 38647.39 | 4941.69 | 8523.44 | 12730.64 |
| 1973 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 19111.09 | 33443.09 | 50658.68 | 2905.34 | 5006.29 | 7444.58 |
| 1974 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14454.74 | 25382.85 | 38447.49 | 0.00 | 0.00 | 0.00 |
| 1975 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 15144.14 | 26729.39 | 40350.34 | 0.00 | 0.00 | 0.00 |
| 1976 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14934.30 | 26051.03 | 39225.67 | 0.00 | 0.00 | 0.00 |
| 1977 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14576.31 | 25562.45 | 38715.74 | 0.00 | 0.00 | 0.00 |
| 1978 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 15130.21 | 26632.16 | 40246.81 | 0.00 | 0.00 | 0.00 |
| 1979 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14410.60 | 25127.77 | 37988.00 | 0.00 | 0.00 | 0.00 |
| 1980 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14890.07 | 26329.72 | 39616.16 | 0.00 | 0.00 | 0.00 |
| 1981 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 15875.91 | 27581.72 | 41900.73 | 0.00 | 0.00 | 0.00 |
| 1982 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 16814.18 | 29548.97 | 44689.28 | 0.00 | 0.00 | 0.00 |
| 1983 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 15506.78 | 27325.62 | 40976.01 | 0.00 | 0.00 | 0.00 |
| 1984 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 16804.60 | 29437.33 | 44358.32 | 0.00 | 0.00 | 0.00 |
| 1985 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 16422.42 | 28370.12 | 42757.29 | 0.00 | 0.00 | 0.00 |
| 1986 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 13923.16 | 24348.77 | 35803.81 | 0.00 | 0.00 | 0.00 |
| 1987 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 18017.70 | 31366.11 | 46673.00 | 0.00 | 0.00 | 0.00 |
| 1988 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 17531.30 | 30301.47 | 45198.29 | 0.00 | 0.00 | 0.00 |
| 1989 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 12767.19 | 21982.45 | 32583.62 | 0.00 | 0.00 | 0.00 |
| 1990 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 12256.83 | 21236.93 | 31499.72 | 0.00 | 0.00 | 0.00 |
| 1991 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8997.07 | 15645.62 | 23827.84 | 0.00 | 0.00 | 0.00 |
| 1992 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 8036.59 | 14107.53 | 21584.50 | 0.00 | 0.00 | 0.00 |
| 1993 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5927.97 | 10745.57 | 16931.41 | 0.00 | 0.00 | 0.00 |
| 1994 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 6522.08 | 12135.48 | 19667.88 | 0.00 | 0.00 | 0.00 |
| 1995 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5988.51 | 11074.03 | 17975.96 | 0.00 | 0.00 | 0.00 |
| 1996 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5066.64 | 9498.80 | 15506.75 | 0.00 | 0.00 | 0.00 |
| 1997 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 6795.02 | 12605.14 | 20160.92 | 0.00 | 0.00 | 0.00 |
| 1998 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5733.97 | 10631.86 | 17309.66 | 0.00 | 0.00 | 0.00 |
| 1999 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5222.71 | 9608.06 | 15598.21 | 0.00 | 0.00 | 0.00 |
| 2000 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 6807.40 | 12565.13 | 20520.65 | 0.00 | 0.00 | 0.00 |
| 2001 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5467.78 | 10106.15 | 16411.12 | 0.00 | 0.00 | 0.00 |
| 2002 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 6033.80 | 11203.22 | 18136.96 | 0.00 | 0.00 | 0.00 |
| 2003 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4099.81 | 7645.76 | 12353.67 | 0.00 | 0.00 | 0.00 |
| 2004 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4040.85 | 7610.11 | 12244.32 | 0.00 | 0.00 | 0.00 |
| 2005 | 60.54 | 318.11 | 951.14 | 0.02 | 20.87 | 311.43 | 4830.14 | 8896.59 | 14556.14 | 0.00 | 0.00 | 0.00 |
| 2006 | 127.82 | 644.65 | 1870.67 | 0.05 | 40.84 | 651.66 | 4416.86 | 8282.69 | 13231.18 | 0.00 | 0.00 | 0.00 |
| 2007 | 200.28 | 1040.43 | 3086.22 | 0.05 | 64.52 | 1051.24 | 4364.72 | 8094.36 | 12998.04 | 0.00 | 0.00 | 0.00 |
| 2008 | 267.02 | 1463.86 | 4202.81 | 0.07 | 86.82 | 1382.00 | 4065.72 | 7586.42 | 12443.65 | 0.00 | 0.00 | 0.00 |
| 2009 | 343.57 | 1913.51 | 5620.58 | 0.11 | 124.72 | 1919.60 | 3735.59 | 6957.63 | 11853.55 | 0.00 | 0.00 | 0.00 |
| 2010 | 475.03 | 2528.49 | 7362.65 | 0.12 | 158.48 | 2512.68 | 3376.76 | 6526.21 | 11682.58 | 0.00 | 0.00 | 0.00 |
| 2011 | 647.97 | 3447.34 | 9827.69 | 0.15 | 216.17 | 3343.13 | 3206.34 | 6355.73 | 12391.02 | 0.00 | 0.00 | 0.00 |
| 2012 | 745.52 | 3833.37 | 11292.37 | 0.18 | 263.26 | 3755.05 | 2255.19 | 4850.86 | 11126.51 | 0.00 | 0.00 | 0.00 |
| 2013 | 784.33 | 4247.02 | 12512.36 | 0.25 | 256.01 | 4129.57 | 1304.03 | 3393.18 | 10128.22 | 0.00 | 0.00 | 0.00 |
| 2014 | 867.54 | 4593.11 | 13768.32 | 0.30 | 292.81 | 4522.17 | 155.62 | 2216.95 | 9500.36 | 0.00 | 0.00 | 0.00 |
| 2015 | 895.51 | 4883.41 | 14415.01 | 0.25 | 308.74 | 4610.49 | 175.82 | 2273.76 | 10008.01 | 0.00 | 0.00 | 0.00 |
| 2016 | 870.70 | 4541.30 | 13526.09 | 0.27 | 295.45 | 4649.91 | 164.36 | 2163.95 | 9318.08 | 0.00 | 0.00 | 0.00 |
| 2017 | 1020.22 | 5689.18 | 16592.96 | 0.28 | 342.10 | 5514.50 | 207.75 | 2654.70 | 11271.76 | 0.00 | 0.00 | 0.00 |
| 2018 | 994.74 | 5333.90 | 15774.78 | 0.29 | 334.15 | 5426.30 | 186.76 | 2430.62 | 10527.35 | 0.00 | 0.00 | 0.00 |
| 2019 | 1092.48 | 5634.80 | 16698.15 | 0.31 | 364.21 | 5548.23 | 201.56 | 2690.03 | 11316.49 | 0.00 | 0.00 | 0.00 |
| 2020 | 1455.54 | 7951.16 | 22891.96 | 0.40 | 490.55 | 7705.81 | 285.88 | 3672.13 | 15316.23 | 0.00 | 0.00 | 0.00 |
| 2021 | 2065.99 | 10760.97 | 31535.29 | 0.55 | 689.84 | 10684.65 | 412.55 | 5251.44 | 22016.72 | 0.00 | 0.00 | 0.00 |
| 2022 | 3002.53 | 15240.73 | 45146.08 | 0.96 | 974.40 | 14355.67 | 548.66 | 7185.88 | 30222.96 | 0.00 | 0.00 | 0.00 |
| 2023 | 3893.23 | 20265.75 | 59826.09 | 0.92 | 1316.38 | 20849.50 | 714.77 | 9643.29 | 40794.44 | 0.00 | 0.00 | 0.00 |
| Total | 19810.6 | 104331.1 | 306901.2 | 5.5 | 6640.3 | 102923.6 | 455686.7 | 844706.0 | 1414291.2 | 103149.6 | 179441.6 | 265067.1 |

Structure in spacecraft

Typically, the structure will have a mass of 10 to 25% (10-15% used in this study for aluminum-based structure with an average of 12.5% used in this study) of a spacecraft's total weight without propellants (dry weight). The spacecraft structure was assumed to be constructed primarily of aluminum

Reference: Spacecraft Systems Design and Engineering76

Examples: Structure weight of CubeSats

Material: aluminum alloy 6061 or 7075

Reference: NASA

https://www.nasa.gov/smallsat-institute/sst-soa/structures-materials-and-mechanisms/

Other satellites

Material: aluminum alloys

Reference: Spacecraft bus design and sizing. Delft University of Technology

https://pure.tudelft.nl/ws/portalfiles/portal/247423772/Reader_1222_-_Spacecraft_Design_v2.3-1_total_incl_errata.pdf

Figure S14. Histogram of structural mass fractions for spacecraft structure (triangular: 10-15%, mode = 12.5%). These values were sampled 10,000 times to propagate uncertainty in the structural mass budget across spacecraft classes.

Figure S15. Distribution of solar-array specific power values (W/kg) sampled in the Monte Carlo simulation (N=10,000). The triangular distribution (20-100 W/kg; mode=30 W/kg) reflects the performance range of crystalline-silicon and multi-junction space solar arrays as reported by NASA (Fig S5).

Table S27. Annual structural metal demand across spacecraft (aluminum). Values represent P05-P50-P95 ranges (kg/y).

| Year | Al_P05 (kg) | Al_P50 (kg) | Al_P95 (kg) | Cumulative (t) |
| --- | --- | --- | --- | --- |
| 1957 | 63.8 | 73.7 | 84.0 | 0.000 |
| 1958 | 856.7 | 994.3 | 1133.0 | 0.001 |
| 1959 | 972.3 | 1125.4 | 1280.0 | 0.002 |
| 1960 | 4788.4 | 5540.8 | 6316.1 | 0.008 |
| 1961 | 7838.7 | 9077.9 | 10329.5 | 0.017 |
| 1962 | 10425.7 | 12076.8 | 13733.9 | 0.029 |
| 1963 | 7471.8 | 8669.3 | 9853.8 | 0.038 |
| 1964 | 18827.7 | 21818.2 | 24862.0 | 0.059 |
| 1965 | 25250.3 | 29355.0 | 33475.1 | 0.089 |
| 1966 | 30543.0 | 35379.4 | 40185.1 | 0.124 |
| 1967 | 25736.2 | 29813.0 | 33979.9 | 0.154 |
| 1968 | 38904.4 | 45120.4 | 51203.0 | 0.199 |
| 1969 | 70657.0 | 81667.9 | 92753.1 | 0.281 |
| 1970 | 36225.4 | 41816.9 | 47579.7 | 0.323 |
| 1971 | 55664.5 | 64588.6 | 73440.7 | 0.387 |
| 1972 | 53474.7 | 61925.5 | 70387.6 | 0.449 |
| 1973 | 62534.6 | 72622.7 | 82576.6 | 0.522 |
| 1974 | 42438.4 | 49273.3 | 55913.1 | 0.571 |
| 1975 | 44657.2 | 51647.2 | 58753.4 | 0.623 |
| 1976 | 43761.7 | 50774.7 | 57577.7 | 0.673 |
| 1977 | 43255.3 | 50018.3 | 56882.5 | 0.723 |
| 1978 | 44663.4 | 51749.8 | 58871.7 | 0.775 |
| 1979 | 42301.7 | 48946.7 | 55586.3 | 0.824 |
| 1980 | 44186.2 | 51098.5 | 57932.9 | 0.875 |
| 1981 | 46315.1 | 53567.7 | 60924.1 | 0.929 |
| 1982 | 49491.0 | 57323.6 | 65170.3 | 0.986 |
| 1983 | 45837.3 | 52848.0 | 60329.1 | 1.039 |
| 1984 | 49021.1 | 56990.3 | 64830.8 | 1.096 |
| 1985 | 49133.7 | 57013.1 | 64799.6 | 1.153 |
| 1986 | 43626.4 | 50437.7 | 57427.1 | 1.203 |
| 1987 | 57923.5 | 67477.9 | 76627.9 | 1.271 |
| 1988 | 58437.0 | 67655.8 | 76822.7 | 1.338 |
| 1989 | 44012.8 | 50925.1 | 57933.2 | 1.389 |
| 1990 | 44039.2 | 50971.0 | 58060.5 | 1.440 |
| 1991 | 34152.2 | 39496.2 | 44868.8 | 1.480 |
| 1992 | 32252.7 | 37227.0 | 42319.0 | 1.517 |
| 1993 | 25977.8 | 30062.0 | 34218.0 | 1.547 |
| 1994 | 30816.0 | 35614.9 | 40521.9 | 1.583 |
| 1995 | 28161.0 | 32555.8 | 37005.4 | 1.615 |
| 1996 | 24344.7 | 28214.8 | 32076.4 | 1.644 |
| 1997 | 31925.0 | 37002.8 | 42033.8 | 1.681 |
| 1998 | 27037.3 | 31286.6 | 35567.0 | 1.712 |
| 1999 | 24437.4 | 28247.7 | 32091.9 | 1.740 |
| 2000 | 31970.2 | 37056.7 | 42154.1 | 1.777 |
| 2001 | 25770.5 | 29798.5 | 33900.1 | 1.807 |
| 2002 | 28350.6 | 32874.4 | 37353.0 | 1.840 |
| 2003 | 19292.8 | 22344.3 | 25459.5 | 1.862 |
| 2004 | 19265.4 | 22343.4 | 25462.4 | 1.885 |
| 2005 | 24641.1 | 28528.5 | 32414.8 | 1.913 |
| 2006 | 24549.8 | 28487.3 | 32372.8 | 1.942 |
| 2007 | 26849.0 | 31079.8 | 35352.3 | 1.973 |
| 2008 | 28098.8 | 32457.3 | 36985.4 | 2.005 |
| 2009 | 29750.0 | 34408.1 | 39143.5 | 2.039 |
| 2010 | 31984.1 | 37143.3 | 42167.7 | 2.077 |
| 2011 | 37464.5 | 43425.6 | 49421.6 | 2.120 |
| 2012 | 36743.4 | 42525.2 | 48361.2 | 2.163 |
| 2013 | 35630.8 | 41315.8 | 47038.6 | 2.204 |
| 2014 | 35621.0 | 41273.8 | 46867.4 | 2.245 |
| 2015 | 37087.9 | 42953.8 | 48838.4 | 2.288 |
| 2016 | 35675.3 | 41287.8 | 47003.7 | 2.329 |
| 2017 | 43518.5 | 50493.4 | 57495.5 | 2.380 |
| 2018 | 40927.3 | 47476.8 | 54023.5 | 2.427 |
| 2019 | 43627.7 | 50402.6 | 57257.0 | 2.478 |
| 2020 | 60625.3 | 70143.0 | 79692.5 | 2.548 |
| 2021 | 84671.0 | 97815.5 | 111282.1 | 2.646 |
| 2022 | 116283.4 | 134791.1 | 153263.7 | 2.781 |
| 2023 | 155705.8 | 180073.5 | 204818.8 | 2.961 |
| Total | 2556544.0 | 2960591.6 | 3366447.6 | - |

Metal footprint for ISS & TSS

Figure S16.  Metals used in space stations (ISS & TSS).

Table S28. Metals used in space stations (ISS & TSS).

| Metal | P05 (kg) | P50 (kg) | P95 (kg) | P05 (kg) | P50 (kg) | P95 (kg) |
| --- | --- | --- | --- | --- | --- | --- |
|  | International Space Station |  |  | Tiangong Space Station |  |  |
| Ag | 18.42 | 60.54 | 148.64 | 4.43 | 14.55 | 34.97 |
| Al | 50108.37 | 59268.90 | 70890.18 | 12016.34 | 14134.96 | 16958.29 |
| As | 709.77 | 1288.94 | 2306.82 | 168.88 | 307.27 | 549.15 |
| Au | 3.29 | 25.08 | 86.30 | 0.76 | 5.96 | 21.10 |
| Co | 94.80 | 2109.64 | 10342.90 | 22.71 | 484.80 | 2483.33 |
| Cu | 16851.84 | 21557.62 | 31448.16 | 4013.95 | 5132.72 | 7489.09 |
| Fe | 1769.26 | 6594.45 | 18012.79 | 428.20 | 1566.77 | 4326.34 |
| Ga | 135.92 | 255.63 | 471.42 | 32.18 | 60.43 | 111.88 |
| In | 34.98 | 72.28 | 144.46 | 8.16 | 17.21 | 34.51 |
| Li | 0.29 | 378.23 | 5749.21 | 0.07 | 88.41 | 1322.25 |
| Ni | 1240.78 | 3789.55 | 12812.49 | 293.25 | 919.91 | 2991.86 |
| Pb | 94.06 | 179.27 | 340.35 | 22.48 | 42.82 | 80.58 |
| Pd | 0.00 | 0.77 | 27.09 | 0.00 | 0.17 | 6.17 |
| Pt | 0.00 | 0.00 | 11.32 | 0.00 | 0.00 | 2.67 |
| REEs | 390.92 | 571.21 | 808.85 | 93.36 | 136.57 | 193.38 |
| Si | 12544.00 | 22698.46 | 40380.63 | 2989.35 | 5399.62 | 9642.63 |
| Sn | 720.89 | 964.28 | 1271.66 | 171.43 | 230.10 | 304.55 |
| Ti | 93.14 | 183.99 | 322.83 | 22.67 | 43.72 | 75.68 |
| Zn | 142.78 | 253.91 | 409.40 | 33.96 | 60.37 | 98.59 |

Metal footprint of spacecraft

Table S29. Total metals used in spacecrafts (1957-2023), mining extraction and reserves in 2024.

|  | Spacecraft | Mining 202477 | Reserves 202477 |
| --- | --- | --- | --- |
|  | t | kt | Mt |
| Cu | 4106.23 | 23000 | 1000 |
| Al | 6576.36 | 72000 | 79 |
| Fe | 744.50 | 1600000 | 87000 |
| Ni | 1362.07 | 3700 | 130 |
| Sn | 183.84 | 300 | 4.3 |
| Ti | 35.89 | 18300 | 1440 |
| Zn | 227.99 | 12000 | 220 |
| Ag | 67.54 | 25 | 0.61 |
| Au | 6.15 | 3.3 | 0.059 |
| Pd | 1.01 | 0.19 | 0.071 0.071 |
| Pt | 0.35 | 0.17 |  |
| REEs | 111.94 | 390 | 110 |
| In | 16.53 | 1.108 | - |
| Ga | 57.51 | 0.76 | 0.0011 |
| As | 289.90 | 58 | - |
| Pb | 49.26 | 4300 | 95 |
| Si | 5555.53 | 4600 | - |
| Cd | 351.39 | 24 | - |
| Li | 118.23 | 240 | 28 |
| Co | 293.49 | 290 | 11 |

Table S30. Metal reserves. Information obtained from USGS.77

| Metal | Observations |
| --- | --- |
| Cu | The most recent U.S. Geological Survey assessment of global copper resources indicated that, as of 2015, identified resources contained 2.1 billion tons of copper and undiscovered resources contained an estimated 3.5 billion tons. |
| Al | Global resources of bauxite are estimated to be between 55 billion and 75 billion tons and are sufficient to meet world demand for metal well into the future. |
| Fe | World resources are estimated to be greater than 800 billion tons of crude ore containing more than 230 billion tons of iron. |
| Ni | Globally, nickel resources have been estimated to contain more than 350 million tons of nickel, with 54% in laterites and 35% in magmatic sulfide deposits |
| Sn | World resources, principally in western Africa, southeastern Asia, Australia, Bolivia, Brazil, Indonesia, and Russia, are extensive and, if developed, could sustain recent annual production rates well into the future. |
| Ti | Ilmenite accounts for about 90% of the world’s consumption of titanium minerals. World resources of anatase, ilmenite, and rutile total more than 2 billion tons |
|  | Ilmenite (FeTiO3): 8600000, 690000000 (reserves) |
| Zn | Identified zinc resources of the world are about 1.9 billion tons. |
| Ag | Although silver was a principal product at several mines, silver was primarily obtained as a byproduct from lead-zinc, copper, and gold mines, in descending order of silver production. |
| Au | Nearly one-quarter of the gold in undiscovered resources was estimated to be contained in porphyry copper deposits. |
| PGM | World resources of PGMs are estimated to total more than 100 million kilograms. Platinum group metals (Palladium, platinum, iridium, osmium, rhodium, and ruthenium) for reserves |
| REEs | Rare earth oxides (REOs) for mining and reserves. Rare earths are relatively abundant in the Earth’s crust, but minable concentrations are less common than for most other mineral commodities. |
| In | Indium is most recovered from the zinc-sulfide ore mineral sphalerite. The indium content of zinc deposits from which it is recovered ranges from less than 1 part per million to 100 parts per million. |
| Ga | Gallium occurs in very small concentrations in ores of other metals. Most gallium is produced as a byproduct of processing bauxite, and the remainder is produced from zinc-processing residues. The average gallium content of bauxite is 50 parts per million. |
| As | World reserves data were unavailable but were estimated to be more than 20 times world production. Arsenic may be obtained from copper, gold, and lead smelter flue dust, as well as from roasting arsenopyrite, the most abundant ore mineral of arsenic. |
| Pb | Identified world lead resources total more than 2 billion tons. |
| Si | World and domestic resources for making silicon metal and alloys are abundant and, in most producing countries, adequate to supply world requirements for many decades. |
|  | Si metal used. Ferrosilicon (an alloy of iron and silicon with a typical silicon content by weight of 15–90%): 5200000 t produced in 2023 globally. |
| Cd | Cadmium is generally recovered from zinc ores and concentrates. Quantitative estimates of reserves were not available. The cadmium content of typical zinc ores averages about 0.03%. |
| Li | Owing to continuing exploration, measured and indicated lithium resources have increased substantially worldwide and total about 105 million tons. |
| Co | Identified world terrestrial cobalt resources are about 25 million tons. |

Table S31. Annual number of spacecrafts by category (micro, small, medium, and large) and year.

| Year | Micro | Small | Medium | Large | Total |
| --- | --- | --- | --- | --- | --- |
| 1957 | 1 | 1 | 1 | 0 | 3 |
| 1958 | 14 | 11 | 0 | 3 | 28 |
| 1959 | 6 | 11 | 9 | 0 | 26 |
| 1960 | 1 | 19 | 13 | 9 | 42 |
| 1961 | 2 | 21 | 12 | 22 | 57 |
| 1962 | 8 | 39 | 4 | 44 | 95 |
| 1963 | 10 | 42 | 8 | 29 | 89 |
| 1964 | 4 | 64 | 13 | 36 | 117 |
| 1965 | 6 | 95 | 15 | 58 | 174 |
| 1966 | 4 | 78 | 22 | 59 | 163 |
| 1967 | 2 | 88 | 23 | 51 | 164 |
| 1968 | 3 | 79 | 19 | 50 | 151 |
| 1969 | 2 | 52 | 16 | 74 | 144 |
| 1970 | 0 | 58 | 14 | 64 | 136 |
| 1971 | 4 | 75 | 15 | 70 | 164 |
| 1972 | 0 | 54 | 18 | 66 | 138 |
| 1973 | 0 | 56 | 16 | 74 | 146 |
| 1974 | 0 | 57 | 13 | 69 | 139 |
| 1975 | 1 | 59 | 20 | 82 | 162 |
| 1976 | 0 | 53 | 30 | 78 | 161 |
| 1977 | 0 | 34 | 29 | 78 | 141 |
| 1978 | 0 | 55 | 35 | 78 | 168 |
| 1979 | 0 | 34 | 18 | 78 | 130 |
| 1980 | 0 | 42 | 24 | 81 | 147 |
| 1981 | 0 | 52 | 17 | 92 | 161 |
| 1982 | 0 | 36 | 25 | 97 | 158 |
| 1983 | 0 | 36 | 28 | 94 | 158 |
| 1984 | 0 | 39 | 25 | 104 | 168 |
| 1985 | 0 | 34 | 22 | 105 | 161 |
| 1986 | 0 | 44 | 17 | 86 | 147 |
| 1987 | 0 | 32 | 19 | 85 | 136 |
| 1988 | 0 | 28 | 14 | 98 | 140 |
| 1989 | 0 | 33 | 12 | 86 | 131 |
| 1990 | 3 | 43 | 18 | 99 | 163 |
| 1991 | 0 | 55 | 14 | 69 | 138 |
| 1992 | 7 | 33 | 11 | 78 | 129 |
| 1993 | 1 | 32 | 11 | 65 | 109 |
| 1994 | 8 | 29 | 9 | 74 | 120 |
| 1995 | 7 | 21 | 8 | 64 | 100 |
| 1996 | 0 | 22 | 17 | 60 | 99 |
| 1997 | 1 | 33 | 61 | 64 | 159 |
| 1998 | 3 | 74 | 55 | 52 | 184 |
| 1999 | 2 | 79 | 8 | 52 | 141 |
| 2000 | 13 | 34 | 8 | 69 | 124 |
| 2001 | 2 | 25 | 6 | 54 | 87 |
| 2002 | 4 | 21 | 13 | 56 | 94 |
| 2003 | 7 | 23 | 10 | 52 | 92 |
| 2004 | 1 | 22 | 5 | 49 | 77 |
| 2005 | 4 | 17 | 6 | 50 | 77 |
| 2006 | 22 | 22 | 14 | 56 | 114 |
| 2007 | 9 | 40 | 10 | 60 | 119 |
| 2008 | 12 | 24 | 8 | 61 | 105 |
| 2009 | 16 | 40 | 10 | 61 | 127 |
| 2010 | 23 | 16 | 12 | 77 | 128 |
| 2011 | 13 | 27 | 24 | 73 | 137 |
| 2012 | 26 | 29 | 11 | 75 | 141 |
| 2013 | 91 | 38 | 17 | 67 | 213 |
| 2014 | 144 | 55 | 18 | 81 | 298 |
| 2015 | 133 | 46 | 12 | 79 | 270 |
| 2016 | 91 | 43 | 17 | 73 | 224 |
| 2017 | 297 | 46 | 59 | 76 | 478 |
| 2018 | 227 | 92 | 55 | 89 | 463 |
| 2019 | 191 | 206 | 24 | 80 | 501 |
| 2020 | 164 | 1051 | 7 | 67 | 1289 |
| 2021 | 326 | 1444 | 19 | 95 | 1884 |
| 2022 | 330 | 2074 | 32 | 99 | 2535 |
| 2023 | 353 | 2358 | 91 | 82 | 2884 |
| Total | 2599 | 9725 | 1266 | 4458 | 18048 |

Table S32. Annual percentage distribution of spacecraft by category (micro, small, medium, and large) by year.

| Year | Micro | Small | Medium | Large |
| --- | --- | --- | --- | --- |
| 1957 | 33.3 | 33.3 | 33.3 | 0.0 |
| 1958 | 50.0 | 39.3 | 0.0 | 10.7 |
| 1959 | 23.1 | 42.3 | 34.6 | 0.0 |
| 1960 | 2.4 | 45.2 | 31.0 | 21.4 |
| 1961 | 3.5 | 36.8 | 21.1 | 38.6 |
| 1962 | 8.4 | 41.1 | 4.2 | 46.3 |
| 1963 | 11.2 | 47.2 | 9.0 | 32.6 |
| 1964 | 3.4 | 54.7 | 11.1 | 30.8 |
| 1965 | 3.4 | 54.6 | 8.6 | 33.3 |
| 1966 | 2.5 | 47.9 | 13.5 | 36.2 |
| 1967 | 1.2 | 53.7 | 14.0 | 31.1 |
| 1968 | 2.0 | 52.3 | 12.6 | 33.1 |
| 1969 | 1.4 | 36.1 | 11.1 | 51.4 |
| 1970 | 0.0 | 42.6 | 10.3 | 47.1 |
| 1971 | 2.4 | 45.7 | 9.1 | 42.7 |
| 1972 | 0.0 | 39.1 | 13.0 | 47.8 |
| 1973 | 0.0 | 38.4 | 11.0 | 50.7 |
| 1974 | 0.0 | 41.0 | 9.4 | 49.6 |
| 1975 | 0.6 | 36.4 | 12.3 | 50.6 |
| 1976 | 0.0 | 32.9 | 18.6 | 48.4 |
| 1977 | 0.0 | 24.1 | 20.6 | 55.3 |
| 1978 | 0.0 | 32.7 | 20.8 | 46.4 |
| 1979 | 0.0 | 26.2 | 13.8 | 60.0 |
| 1980 | 0.0 | 28.6 | 16.3 | 55.1 |
| 1981 | 0.0 | 32.3 | 10.6 | 57.1 |
| 1982 | 0.0 | 22.8 | 15.8 | 61.4 |
| 1983 | 0.0 | 22.8 | 17.7 | 59.5 |
| 1984 | 0.0 | 23.2 | 14.9 | 61.9 |
| 1985 | 0.0 | 21.1 | 13.7 | 65.2 |
| 1986 | 0.0 | 29.9 | 11.6 | 58.5 |
| 1987 | 0.0 | 23.5 | 14.0 | 62.5 |
| 1988 | 0.0 | 20.0 | 10.0 | 70.0 |
| 1989 | 0.0 | 25.2 | 9.2 | 65.6 |
| 1990 | 1.8 | 26.4 | 11.0 | 60.7 |
| 1991 | 0.0 | 39.9 | 10.1 | 50.0 |
| 1992 | 5.4 | 25.6 | 8.5 | 60.5 |
| 1993 | 0.9 | 29.4 | 10.1 | 59.6 |
| 1994 | 6.7 | 24.2 | 7.5 | 61.7 |
| 1995 | 7.0 | 21.0 | 8.0 | 64.0 |
| 1996 | 0.0 | 22.2 | 17.2 | 60.6 |
| 1997 | 0.6 | 20.8 | 38.4 | 40.3 |
| 1998 | 1.6 | 40.2 | 29.9 | 28.3 |
| 1999 | 1.4 | 56.0 | 5.7 | 36.9 |
| 2000 | 10.5 | 27.4 | 6.5 | 55.6 |
| 2001 | 2.3 | 28.7 | 6.9 | 62.1 |
| 2002 | 4.3 | 22.3 | 13.8 | 59.6 |
| 2003 | 7.6 | 25.0 | 10.9 | 56.5 |
| 2004 | 1.3 | 28.6 | 6.5 | 63.6 |
| 2005 | 5.2 | 22.1 | 7.8 | 64.9 |
| 2006 | 19.3 | 19.3 | 12.3 | 49.1 |
| 2007 | 7.6 | 33.6 | 8.4 | 50.4 |
| 2008 | 11.4 | 22.9 | 7.6 | 58.1 |
| 2009 | 12.6 | 31.5 | 7.9 | 48.0 |
| 2010 | 18.0 | 12.5 | 9.4 | 60.2 |
| 2011 | 9.5 | 19.7 | 17.5 | 53.3 |
| 2012 | 18.4 | 20.6 | 7.8 | 53.2 |
| 2013 | 42.7 | 17.8 | 8.0 | 31.5 |
| 2014 | 48.3 | 18.5 | 6.0 | 27.2 |
| 2015 | 49.3 | 17.0 | 4.4 | 29.3 |
| 2016 | 40.6 | 19.2 | 7.6 | 32.6 |
| 2017 | 62.1 | 9.6 | 12.3 | 15.9 |
| 2018 | 49.0 | 19.9 | 11.9 | 19.2 |
| 2019 | 38.1 | 41.1 | 4.8 | 16.0 |
| 2020 | 12.7 | 81.5 | 0.5 | 5.2 |
| 2021 | 17.3 | 76.6 | 1.0 | 5.0 |
| 2022 | 13.0 | 81.8 | 1.3 | 3.9 |
| 2023 | 12.2 | 81.8 | 3.2 | 2.8 |
| Total | 14.4 | 53.9 | 7.0 | 24.7 |

Table S33. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2010-2023.

| Year | Micro | Small | Medium | Large | Spacecraft | Micro | Small | Medium | Large |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Share of spacecraft (%) |  |  |  | Estimated spacecraft (units) |  |  |  |  |
| 2024 | 0.21361 | 0.75125 | 0.01223 | 0.02292 | 1096 | 234 | 824 | 13 | 25 |
| 2025 | 0.20335 | 0.76995 | 0.01000 | 0.01670 | 1185 | 241 | 912 | 12 | 20 |
| 2026 | 0.19449 | 0.78508 | 0.00822 | 0.01221 | 1280 | 249 | 1005 | 11 | 16 |
| 2027 | 0.18679 | 0.79748 | 0.00678 | 0.00895 | 1384 | 258 | 1103 | 9 | 12 |
| 2028 | 0.18003 | 0.80777 | 0.00561 | 0.00658 | 1495 | 269 | 1208 | 8 | 10 |
| 2029 | 0.17403 | 0.81646 | 0.00466 | 0.00485 | 1616 | 281 | 1319 | 8 | 8 |
| 2030 | 0.16861 | 0.82392 | 0.00388 | 0.00359 | 1746 | 294 | 1438 | 7 | 6 |
| 2031 | 0.16366 | 0.83045 | 0.00324 | 0.00265 | 1886 | 309 | 1566 | 6 | 5 |
| 2032 | 0.15908 | 0.83626 | 0.00270 | 0.00197 | 2038 | 324 | 1704 | 6 | 4 |
| 2033 | 0.15478 | 0.84151 | 0.00226 | 0.00146 | 2202 | 341 | 1853 | 5 | 3 |
| 2034 | 0.15071 | 0.84632 | 0.00189 | 0.00108 | 2379 | 358 | 2013 | 4 | 3 |
| 2035 | 0.14682 | 0.85080 | 0.00158 | 0.00080 | 2570 | 377 | 2186 | 4 | 2 |
| 2036 | 0.14308 | 0.85500 | 0.00132 | 0.00060 | 2776 | 397 | 2374 | 4 | 2 |
| 2037 | 0.13947 | 0.85898 | 0.00111 | 0.00044 | 2999 | 418 | 2576 | 3 | 1 |
| 2038 | 0.13596 | 0.86278 | 0.00093 | 0.00033 | 3239 | 440 | 2795 | 3 | 1 |
| 2039 | 0.13255 | 0.86643 | 0.00078 | 0.00024 | 3499 | 464 | 3032 | 3 | 1 |
| 2040 | 0.12922 | 0.86995 | 0.00065 | 0.00018 | 3779 | 488 | 3288 | 2 | 1 |
| 2041 | 0.12596 | 0.87335 | 0.00055 | 0.00014 | 4082 | 514 | 3565 | 2 | 1 |
| 2042 | 0.12278 | 0.87666 | 0.00046 | 0.00010 | 4409 | 541 | 3865 | 2 | 0 |
| 2043 | 0.11965 | 0.87989 | 0.00038 | 0.00007 | 4761 | 570 | 4189 | 2 | 0 |
| 2044 | 0.11660 | 0.88303 | 0.00032 | 0.00006 | 5142 | 600 | 4540 | 2 | 0 |
| 2045 | 0.11360 | 0.88609 | 0.00027 | 0.00004 | 5553 | 631 | 4920 | 2 | 0 |
| 2046 | 0.11066 | 0.88908 | 0.00023 | 0.00003 | 5996 | 664 | 5331 | 1 | 0 |
| 2047 | 0.10778 | 0.89201 | 0.00019 | 0.00002 | 6475 | 698 | 5775 | 1 | 0 |
| 2048 | 0.10495 | 0.89487 | 0.00016 | 0.00002 | 6991 | 734 | 6256 | 1 | 0 |
| 2049 | 0.10219 | 0.89767 | 0.00013 | 0.00001 | 7549 | 771 | 6776 | 1 | 0 |
| 2050 | 0.09947 | 0.90040 | 0.00011 | 0.00001 | 8150 | 811 | 7338 | 1 | 0 |

Table S34. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2006-2023.

| Year | Micro | Small | Medium | Large | Spacecraft | Micro | Small | Medium | Large |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Share of spacecraft (%) |  |  |  | Estimated spacecraft (units) |  |  |  |  |
| 2024 | 0.30033 | 0.64669 | 0.01729 | 0.03569 | 1096 | 329 | 709 | 19 | 39 |
| 2025 | 0.29973 | 0.65761 | 0.01485 | 0.02781 | 1185 | 355 | 779 | 18 | 33 |
| 2026 | 0.29936 | 0.66621 | 0.01277 | 0.02165 | 1280 | 383 | 853 | 16 | 28 |
| 2027 | 0.29931 | 0.67283 | 0.01099 | 0.01687 | 1384 | 414 | 931 | 15 | 23 |
| 2028 | 0.29962 | 0.67775 | 0.00948 | 0.01315 | 1495 | 448 | 1013 | 14 | 20 |
| 2029 | 0.30032 | 0.68123 | 0.00819 | 0.01026 | 1616 | 485 | 1101 | 13 | 17 |
| 2030 | 0.30142 | 0.68349 | 0.00708 | 0.00801 | 1746 | 526 | 1193 | 12 | 14 |
| 2031 | 0.30290 | 0.68470 | 0.00613 | 0.00626 | 1886 | 571 | 1292 | 12 | 12 |
| 2032 | 0.30474 | 0.68504 | 0.00532 | 0.00490 | 2038 | 621 | 1396 | 11 | 10 |
| 2033 | 0.30691 | 0.68463 | 0.00462 | 0.00384 | 2202 | 676 | 1507 | 10 | 8 |
| 2034 | 0.30939 | 0.68358 | 0.00402 | 0.00302 | 2379 | 736 | 1626 | 10 | 7 |
| 2035 | 0.31213 | 0.68200 | 0.00350 | 0.00237 | 2570 | 802 | 1753 | 9 | 6 |
| 2036 | 0.31511 | 0.67998 | 0.00305 | 0.00186 | 2776 | 875 | 1888 | 8 | 5 |
| 2037 | 0.31830 | 0.67757 | 0.00266 | 0.00146 | 2999 | 955 | 2032 | 8 | 4 |
| 2038 | 0.32166 | 0.67486 | 0.00232 | 0.00115 | 3239 | 1042 | 2186 | 8 | 4 |
| 2039 | 0.32518 | 0.67188 | 0.00203 | 0.00091 | 3499 | 1138 | 2351 | 7 | 3 |
| 2040 | 0.32882 | 0.66869 | 0.00178 | 0.00072 | 3779 | 1243 | 2527 | 7 | 3 |
| 2041 | 0.33255 | 0.66533 | 0.00155 | 0.00057 | 4082 | 1357 | 2716 | 6 | 2 |
| 2042 | 0.33636 | 0.66183 | 0.00136 | 0.00045 | 4409 | 1483 | 2918 | 6 | 2 |
| 2043 | 0.34024 | 0.65822 | 0.00119 | 0.00035 | 4761 | 1620 | 3134 | 6 | 2 |
| 2044 | 0.34415 | 0.65453 | 0.00104 | 0.00028 | 5142 | 1770 | 3366 | 5 | 1 |
| 2045 | 0.34808 | 0.65078 | 0.00091 | 0.00022 | 5553 | 1933 | 3614 | 5 | 1 |
| 2046 | 0.35203 | 0.64700 | 0.00080 | 0.00017 | 5996 | 2111 | 3879 | 5 | 1 |
| 2047 | 0.35597 | 0.64319 | 0.00070 | 0.00014 | 6475 | 2305 | 4164 | 5 | 1 |
| 2048 | 0.35990 | 0.63937 | 0.00062 | 0.00011 | 6991 | 2516 | 4470 | 4 | 1 |
| 2049 | 0.36381 | 0.63556 | 0.00054 | 0.00009 | 7549 | 2746 | 4798 | 4 | 1 |
| 2050 | 0.36768 | 0.63177 | 0.00047 | 0.00007 | 8150 | 2997 | 5149 | 4 | 1 |

Table S35. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2008-2023.

| Year | Micro | Small | Medium | Large | Spacecraft | Micro | Small | Medium | Large |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Share of spacecraft (%) |  |  |  | Estimated spacecraft (units) |  |  |  |  |
| 2024 | 0.26823 | 0.68876 | 0.01525 | 0.02776 | 1096 | 294 | 755 | 17 | 30 |
| 2025 | 0.26404 | 0.70227 | 0.01289 | 0.02080 | 1185 | 313 | 832 | 15 | 25 |
| 2026 | 0.26059 | 0.71289 | 0.01091 | 0.01561 | 1280 | 334 | 913 | 14 | 20 |
| 2027 | 0.25789 | 0.72112 | 0.00927 | 0.01173 | 1384 | 357 | 998 | 13 | 16 |
| 2028 | 0.25589 | 0.72739 | 0.00789 | 0.00883 | 1495 | 383 | 1088 | 12 | 13 |
| 2029 | 0.25454 | 0.73207 | 0.00673 | 0.00666 | 1616 | 411 | 1183 | 11 | 11 |
| 2030 | 0.25378 | 0.73544 | 0.00576 | 0.00503 | 1746 | 443 | 1284 | 10 | 9 |
| 2031 | 0.25352 | 0.73774 | 0.00493 | 0.00381 | 1886 | 478 | 1392 | 9 | 7 |
| 2032 | 0.25371 | 0.73917 | 0.00423 | 0.00289 | 2038 | 517 | 1506 | 9 | 6 |
| 2033 | 0.25429 | 0.73989 | 0.00364 | 0.00219 | 2202 | 560 | 1629 | 8 | 5 |
| 2034 | 0.25518 | 0.74002 | 0.00313 | 0.00166 | 2379 | 607 | 1760 | 7 | 4 |
| 2035 | 0.25635 | 0.73968 | 0.00270 | 0.00127 | 2570 | 659 | 1901 | 7 | 3 |
| 2036 | 0.25775 | 0.73896 | 0.00233 | 0.00096 | 2776 | 716 | 2051 | 6 | 3 |
| 2037 | 0.25934 | 0.73791 | 0.00201 | 0.00073 | 2999 | 778 | 2213 | 6 | 2 |
| 2038 | 0.26110 | 0.73661 | 0.00173 | 0.00056 | 3239 | 846 | 2386 | 6 | 2 |
| 2039 | 0.26298 | 0.73509 | 0.00150 | 0.00043 | 3499 | 920 | 2572 | 5 | 1 |
| 2040 | 0.26498 | 0.73340 | 0.00129 | 0.00033 | 3779 | 1001 | 2772 | 5 | 1 |
| 2041 | 0.26706 | 0.73157 | 0.00112 | 0.00025 | 4082 | 1090 | 2986 | 5 | 1 |
| 2042 | 0.26922 | 0.72962 | 0.00097 | 0.00019 | 4409 | 1187 | 3217 | 4 | 1 |
| 2043 | 0.27144 | 0.72758 | 0.00084 | 0.00014 | 4761 | 1292 | 3464 | 4 | 1 |
| 2044 | 0.27371 | 0.72546 | 0.00072 | 0.00011 | 5142 | 1407 | 3730 | 4 | 1 |
| 2045 | 0.27601 | 0.72327 | 0.00063 | 0.00008 | 5553 | 1533 | 4016 | 3 | 0 |
| 2046 | 0.27835 | 0.72104 | 0.00054 | 0.00006 | 5996 | 1669 | 4323 | 3 | 0 |
| 2047 | 0.28071 | 0.71877 | 0.00047 | 0.00005 | 6475 | 1818 | 4654 | 3 | 0 |
| 2048 | 0.28309 | 0.71647 | 0.00041 | 0.00004 | 6991 | 1979 | 5009 | 3 | 0 |
| 2049 | 0.28548 | 0.71414 | 0.00035 | 0.00003 | 7549 | 2155 | 5391 | 3 | 0 |
| 2050 | 0.28788 | 0.71179 | 0.00030 | 0.00002 | 8150 | 2346 | 5801 | 2 | 0 |

Table S36. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2012-2023.

| Year | Micro | Small | Medium | Large | Spacecraft | Micro | Small | Medium | Large |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Share of spacecraft (%) |  |  |  | Estimated spacecraft (units) |  |  |  |  |
| 2024 | 0.14584 | 0.81909 | 0.01232 | 0.02274 | 1096 | 160 | 898 | 14 | 25 |
| 2025 | 0.12754 | 0.84573 | 0.01014 | 0.01660 | 1185 | 151 | 1002 | 12 | 20 |
| 2026 | 0.11184 | 0.86763 | 0.00838 | 0.01215 | 1280 | 143 | 1111 | 11 | 16 |
| 2027 | 0.09826 | 0.88586 | 0.00696 | 0.00892 | 1384 | 136 | 1226 | 10 | 12 |
| 2028 | 0.08642 | 0.90123 | 0.00579 | 0.00656 | 1495 | 129 | 1348 | 9 | 10 |
| 2029 | 0.07604 | 0.91430 | 0.00482 | 0.00484 | 1616 | 123 | 1477 | 8 | 8 |
| 2030 | 0.06689 | 0.92552 | 0.00403 | 0.00357 | 1746 | 117 | 1616 | 7 | 6 |
| 2031 | 0.05882 | 0.93519 | 0.00336 | 0.00263 | 1886 | 111 | 1764 | 6 | 5 |
| 2032 | 0.05167 | 0.94358 | 0.00281 | 0.00194 | 2038 | 105 | 1923 | 6 | 4 |
| 2033 | 0.04536 | 0.95087 | 0.00235 | 0.00143 | 2202 | 100 | 2094 | 5 | 3 |
| 2034 | 0.03977 | 0.95721 | 0.00196 | 0.00106 | 2379 | 95 | 2277 | 5 | 3 |
| 2035 | 0.03484 | 0.96274 | 0.00164 | 0.00078 | 2570 | 90 | 2474 | 4 | 2 |
| 2036 | 0.03049 | 0.96757 | 0.00137 | 0.00057 | 2776 | 85 | 2686 | 4 | 2 |
| 2037 | 0.02666 | 0.97178 | 0.00114 | 0.00042 | 2999 | 80 | 2914 | 3 | 1 |
| 2038 | 0.02329 | 0.97545 | 0.00095 | 0.00031 | 3239 | 75 | 3160 | 3 | 1 |
| 2039 | 0.02033 | 0.97865 | 0.00079 | 0.00023 | 3499 | 71 | 3424 | 3 | 1 |
| 2040 | 0.01774 | 0.98143 | 0.00066 | 0.00017 | 3779 | 67 | 3709 | 2 | 1 |
| 2041 | 0.01546 | 0.98386 | 0.00055 | 0.00012 | 4082 | 63 | 4016 | 2 | 1 |
| 2042 | 0.01347 | 0.98598 | 0.00046 | 0.00009 | 4409 | 59 | 4347 | 2 | 0 |
| 2043 | 0.01173 | 0.98782 | 0.00038 | 0.00007 | 4761 | 56 | 4703 | 2 | 0 |
| 2044 | 0.01021 | 0.98942 | 0.00032 | 0.00005 | 5142 | 53 | 5087 | 2 | 0 |
| 2045 | 0.00888 | 0.99081 | 0.00026 | 0.00004 | 5553 | 49 | 5502 | 1 | 0 |
| 2046 | 0.00773 | 0.99203 | 0.00022 | 0.00003 | 5996 | 46 | 5948 | 1 | 0 |
| 2047 | 0.00672 | 0.99308 | 0.00018 | 0.00002 | 6475 | 44 | 6430 | 1 | 0 |
| 2048 | 0.00584 | 0.99399 | 0.00015 | 0.00001 | 6991 | 41 | 6949 | 1 | 0 |
| 2049 | 0.00508 | 0.99479 | 0.00013 | 0.00001 | 7549 | 38 | 7509 | 1 | 0 |
| 2050 | 0.00441 | 0.99548 | 0.00011 | 0.00001 | 8150 | 36 | 8113 | 1 | 0 |

Table S37. Projected shares and counts of spacecraft by size category for the period 2024-2050, based on a calibration window from 2014-2023.

| Year | Micro | Small | Medium | Large | Spacecraft | Micro | Small | Medium | Large |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Share of spacecraft (%) |  |  |  | Estimated spacecraft (units) |  |  |  |  |
| 2024 | 0.09304 | 0.87485 | 0.01075 | 0.02136 | 1096 | 102 | 959 | 12 | 23 |
| 2025 | 0.07258 | 0.90343 | 0.00860 | 0.01540 | 1185 | 86 | 1070 | 10 | 18 |
| 2026 | 0.05659 | 0.92537 | 0.00691 | 0.01113 | 1280 | 72 | 1185 | 9 | 14 |
| 2027 | 0.04408 | 0.94230 | 0.00556 | 0.00806 | 1384 | 61 | 1304 | 8 | 11 |
| 2028 | 0.03427 | 0.95541 | 0.00448 | 0.00584 | 1495 | 51 | 1429 | 7 | 9 |
| 2029 | 0.02659 | 0.96556 | 0.00361 | 0.00423 | 1616 | 43 | 1560 | 6 | 7 |
| 2030 | 0.02060 | 0.97343 | 0.00291 | 0.00306 | 1746 | 36 | 1699 | 5 | 5 |
| 2031 | 0.01592 | 0.97952 | 0.00235 | 0.00222 | 1886 | 30 | 1848 | 4 | 4 |
| 2032 | 0.01229 | 0.98422 | 0.00189 | 0.00160 | 2038 | 25 | 2006 | 4 | 3 |
| 2033 | 0.00947 | 0.98785 | 0.00152 | 0.00116 | 2202 | 21 | 2175 | 3 | 3 |
| 2034 | 0.00729 | 0.99065 | 0.00122 | 0.00084 | 2379 | 17 | 2357 | 3 | 2 |
| 2035 | 0.00561 | 0.99280 | 0.00098 | 0.00060 | 2570 | 14 | 2551 | 3 | 2 |
| 2036 | 0.00431 | 0.99446 | 0.00079 | 0.00043 | 2776 | 12 | 2761 | 2 | 1 |
| 2037 | 0.00331 | 0.99574 | 0.00063 | 0.00031 | 2999 | 10 | 2986 | 2 | 1 |
| 2038 | 0.00254 | 0.99672 | 0.00051 | 0.00023 | 3239 | 8 | 3229 | 2 | 1 |
| 2039 | 0.00195 | 0.99747 | 0.00041 | 0.00016 | 3499 | 7 | 3490 | 1 | 1 |
| 2040 | 0.00150 | 0.99805 | 0.00033 | 0.00012 | 3779 | 6 | 3772 | 1 | 0 |
| 2041 | 0.00115 | 0.99850 | 0.00026 | 0.00008 | 4082 | 5 | 4076 | 1 | 0 |
| 2042 | 0.00088 | 0.99884 | 0.00021 | 0.00006 | 4409 | 4 | 4404 | 1 | 0 |
| 2043 | 0.00068 | 0.99911 | 0.00017 | 0.00004 | 4761 | 3 | 4757 | 1 | 0 |
| 2044 | 0.00052 | 0.99931 | 0.00014 | 0.00003 | 5142 | 3 | 5138 | 1 | 0 |
| 2045 | 0.00040 | 0.99947 | 0.00011 | 0.00002 | 5553 | 2 | 5550 | 1 | 0 |
| 2046 | 0.00031 | 0.99959 | 0.00009 | 0.00002 | 5996 | 2 | 5994 | 1 | 0 |
| 2047 | 0.00023 | 0.99968 | 0.00007 | 0.00001 | 6475 | 2 | 6473 | 0 | 0 |
| 2048 | 0.00018 | 0.99976 | 0.00006 | 0.00001 | 6991 | 1 | 6989 | 0 | 0 |
| 2049 | 0.00014 | 0.99981 | 0.00005 | 0.00001 | 7549 | 1 | 7547 | 0 | 0 |
| 2050 | 0.00011 | 0.99985 | 0.00004 | 0.00000 | 8150 | 1 | 8149 | 0 | 0 |

Table S38. Classification of spacecraft by category, function, mass, orbital5,26,50,78.

| Category | Sub-type/Function | Typical mass (kg) | Design lifetime (years) | Typical orbit | Representative examples |
| --- | --- | --- | --- | --- | --- |
| Communication satellites | Broadcast, telecommunication, internet constellations | 1000–6000 | 10–20 | GEO / MEO | Intelsat, Starlink, OneWeb |
| Earth observation satellites | Optical, radar, climate and environmental monitoring | 500-3000 | 5-10 | LEO / SSO | Landsat-8, Sentinel-2, Gaofen-1 |
| Navigation satellites | Positioning, navigation, timing (PNT) | 1000-2000 | 10-15 | MEO | GPS IIF, Galileo, BeiDou |
| Scientific & astronomy satellites | Space and Earth science, astrophysics, planetary missions | 500-10000 | 5-20 | LEO / HEO / Deep Space | Hubble, James Webb, Gaia |
| Technology demonstration satellites | Testing new sensors, materials, or propulsion | 1-500 | 1-5 | LEO | Surrey TechSat, CubeSat missions |
| Military & reconnaissance satellites | Surveillance, early warning, secure communications | 1000-10000 | 5-15 | LEO / GEO | NRO satellites, Yaogan series |
| Crewed spacecraft | Transport and habitation of astronauts | 7000-20000 | <10 (per vehicle) | LEO / Lunar | Soyuz, Crew Dragon, Orion |
| Space probes | Interplanetary exploration | 100-5000 | 10-30 | Heliocentric / Planetary | Voyager, Cassini, New Horizons |
| Space stations | Long-term human operations | >100000 | 10-30 | LEO | ISS, Tiangong |
| Small satellites | Low-cost, compact platforms | <500 | 1-5 | LEO | Planet Dove, ICEYE, Swarm |
| • Minisatellite |  | 100-500 | 2-7 | LEO | Astroscale ELSA-d |
| • Microsatellite |  | 10-100 | 1-5 | LEO | PlanetScope |
| • Nanosatellite/ CubeSat |  | 1-10 | 0.5-3 | LEO | 1U-12U CubeSats (universities, startups) |
| • Picosatellite/ Femtosatellite |  | <1 | <1 | LEO | ThinSat, KickSat (ChipSat prototypes) |

Table S39. Classification of spacecraft by mass (kg).

| Large | Medium | Small | Mini | Micro | Nano | Pico | Femto | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ≥1000 | 500-1000 | 100-500 | - | 10-100 | 1-10 | 0.1-1 | <0.1 | 5 |
| ≥1000 | ≥500 & <1000 | <500 | ≥100 & <500 | ≥10 & <100 | ≥1 & <10 | ≥0.1 & <1 | ≤0.1 | 78 |
| ≥1000 | 500-1000 | - | 100-500 | 10-100 | 1-10 | 0.01-1 | 0.01-0.01 | 79 |
| >1000 | 500-1000 | - | 100-500 | 10-100 | 1-10 | <1 | - | 80 |

Table S40. Summary statistics of Monte Carlo-derived dry mass distributions for each spacecraft class (N=10,000 samples per class). Mean, median, standard deviation (SD), and percentiles (P05, P25, P75, P95).

| Class | Mean | Median | SD | P05 | P25 | P75 | P95 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Micro | 5.0 | 5.0 | 2.0 | 1.6 | 3.5 | 6.4 | 8.4 |
| Small | 253.9 | 253.8 | 99.7 | 88.5 | 181.6 | 325.3 | 422.4 |
| Medium | 749.8 | 749.9 | 100.6 | 582.1 | 676.6 | 822.4 | 917.0 |
| Large | 3515.9 | 3514.6 | 1018.8 | 1797.8 | 2785.9 | 4240.1 | 5208.9 |

Figure S17.  Histogram of simulated micro-class spacecraft dry masses (N=10,000), based on a triangular distribution with bounds of 0.01–10 kg and a most-likely value of 5 kg.

Figure S18.  Histogram of simulated small-class spacecraft dry masses (N=10,000), based on a triangular distribution with bounds of 10–500 kg and a most-likely value of 250 kg.

Figure S19.  Histogram of simulated medium-class spacecraft dry masses (N=10,000), based on a triangular distribution with bounds of 500–1,000 kg and a most-likely value of 750 kg.

Figure S20.  Histogram of simulated large-class spacecraft dry masses (N = 10,000), based on a triangular distribution with bounds of 1,000–6,000 kg and a most-likely value of 3,500 kg

Table S41. Energy footprint of metals in mining (MJ/kg).

|  | 42 | 81 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Element |  |  | Mean | SD | Samples |
| Ag | 3280 | 11246 | 1745 | 2171 | 2 |
| Al | 131 | 74 | 171 | 65 | 17 |
| As | 5 | 75 | 40 | 35 | 1 |
| Au | 208000 | 243437 | 315000 | 101823 | 2 |
| Cd | 53 | 14171 | 7112 | 7059 | 1 |
| Co | 128 | 156 | 129 | 170 | 12 |
| Cu | 53.7 | 23 | 58 | 39 | 20 |
| Fe | 23.1 | 24 | 19 | 7 | 8 |
| Ga | 3030 | 786 | 1908 | 1122 | 1 |
| In | 1720 | 27716 | 14718 | 12998 | 1 |
| Li | 125 | 69 | 89 | 26 | 9 |
| Ni | 111 | 145 | 156 | 68 | 15 |
| Pb | 18.9 | 10 | 25 | 6 | 8 |
| Pd | 72700 | 122789 | 188350 | 163554 | 2 |
| Pt | 243000 | 76135 | 315000 | 101823 | 2 |
| REEs | 3665 | 4060 | 3862.5 | 197.5 | 1 |
| Si | - | 1396 | 5242 | 3039 | 5 |
| Sn | 321 | 64 | 192.5 | 128.5 | 1 |
| Ti | 115 | 321 | 280 | 143 | 3 |
| Zn | 52.9 | 19 | 48 | 12 | 12 |

Table S42. CO2 footprint of metals in mining (kg CO2-eq/kg)81.

| Element | HES | LES | Mean | SD | Samples |
| --- | --- | --- | --- | --- | --- |
| Ag | 1489 | 432 | 283 | 301 | 10 |
| Al | 10.1 | 2.3 | 16 | 9 | 23 |
| As | 9.9 | 2.3 | 0.3 | 4 | 1 |
| Au | 26337 | 10191 | 29222 | 21825 | 32 |
| Cd | 1898 | 354 | 3 | 823 | 1 |
| Co | 20.5 | 6.4 | 14 | 12 | 11 |
| Cu | 3.2 | 0.6 | 5.5 | 9 | 46 |
| Fe | 2.5 | 1.9 | 3.9 | 7 | 11 |
| Ga | 63.4 | 55.5 | 205 | 69 | 1 |
| In | 3713 | 692 | 102 | 1582 | 1 |
| Li | 6.4 | 4.4 | 7 | 7 | 24 |
| Ni | 18.1 | 5.6 | 13 | 5 | 26 |
| Pb | 1.1 | 0.5 | 2 | 1 | 10 |
| Pd | 12971 | 4204 | 15128 | 11049 | 5 |
| Pt | 9350 | 2121 | 27488 | 8844 | 5 |
| REEs | 542 | 116 | 221 | 181 |  |
| Si | 128.8 | 60 | 461 | 238 | 5 |
| Sn | 7.6 | 2.4 | 17 | 6 | 1 |
| Ti | 36.6 | 23 | 27 | 16 | 3 |
| Zn | 2.6 | 0.5 | 4.1 | 1 | 15 |

HES: high emission scenario (based mostly on fossil fuels)

LES: low emission scenario (utilizing a combination of renewable energy sources)

Table S43. Water stress (m3/t)

| Element | average | SD | Comments | Reference |
| --- | --- | --- | --- | --- |
| Ag | 1713 | 671 | Water consumption for silver concentrate production (average) | Ref. A |
| Al | 15.26 | 4.0 | Water scarcity global aluminium including China and global aluminium excluding China | Ref. B |
| As* | 11.93 | - | As is commonly obtained as a by-product of smelting of copper, lead, cobalt, and gold ores, often as As trioxide recovered from flue dust | - |
| Au | 265861 | 122990.2 | Water consumption for refined gold metal production (average) | Ref. A |
| Cd** | 43.2 |  | Cadmium is commonly recovered as a by-product of zinc refining | - |
| Co | 208.4 | 118.5 | Water consumption for cobalt concentrate production (average) | Ref. A |
| Cu | 43.2 | 26.5 | Water consumption for copper concentrate production (average) | Ref. A |
| Fe | 0.447 | 0.1 | Water consumption for iron ore fines production (average) | Ref. A |
| Ga | 6800 | - | Water consumption for primary gallium metal production (4N purity) | Ref. D |
| In | 3114.29 | - | ProBas dataset for 1 t indium | Ref. E |
| Li | 211 | 107.8 | Production from salars (lithium carbonate, Li2CO3, battery grade) | Ref. F |
| Ni | 193.8 | 61.5 | Nickel metal production | Ref. A |
| Pb | 6.597 | 2.8 | Leacd concentrate | Ref. A |
| Pd | 210713 | 108995.3 | Palladium metal production | Ref. A |
| Pt | 313496 | 134724.9 | Platinium metal | Ref. A |
| REEs | 11170 | 13049.5 | Average from monazite-based rare-earth production; range 3,803–29,902 kg/kg depending on element | Ref. G |
| Si | 3120 | - | Best available proxy for upstream/primary silicon refining. 62,300 m3 water withdrawal/t Si | Ref. H |
| Sn | 312 | - | Water use for refined tin ingot (m3 world eq) | Ref. I |
| Ti | 30.1 | - | For titanium sponge production | Ref. J |
| Zn | 11.93 | 5.3 | Zinc concentrate | Ref. A |

*Arsenic water consumption was estimated at ~43.2 m³/t As, using copper concentrate production as the proxy parent system. This proxy was selected because arsenic is generally recovered as a by-product of nonferrous smelting, especially from copper smelter flue dust, where arsenic trioxide is volatilized during smelting and subsequently recovered. Because no direct open arsenic-specific water-consumption inventory was identified, the average specific water consumption of copper concentrates (43.2 m3/t metal-equivalent) reported by Meißner (2021) was used as the best available real-industry proxy.82 This estimate should therefore be interpreted as the water consumption intensity of the dominant copper production route associated with arsenic recovery, not as a direct measured inventory for pure arsenic production

** Cadmium water consumption was estimated at ~11.93 m3/t Cd, using zinc concentrate production as the proxy parent system. This proxy was selected because cadmium is produced predominantly as a by-product of zinc processing; approximately 80% of cadmium production is associated with zinc production, and about 3 kg Cd are produced per tonne of Zn. Since no direct open cadmium-specific water-consumption inventory was identified, the average specific water consumption of zinc concentrates (11.93 m3/t metal-equivalent) reported by Meißner (2021), based on a comparative review of LCA datasets and industrial sources, was adopted as the best available real-industry proxy.82 This estimate should therefore be interpreted as the water consumption intensity of the dominant zinc production route that generates cadmium, rather than a fully allocated cadmium-only process inventory.

Table S44. Rock-to-metal ratio (waste rock + tailings + processing loss).83,84

| Element | RMR | SD |
| --- | --- | --- |
| Ag | 22378 | 840000 |
| Al | 7.14 | 4.5 |
| As* | 513 | - |
| Au | 3046349 | 8670000 |
| Cd* | 71 | - |
| Co | 859 | 3080 |
| Cu | 513 | 944 |
| Fe | 9.23 | 13 |
| Ga | 15604 | 6350 |
| In | 1253310 | 1130000 |
| Li | 1634 | 2550 |
| Ni | 250 | 342 |
| Pb** | 50 | - |
| Pd | 688473 | 1050000 |
| Pt | 834932 | 1180000 |
| REEs | 980 | - |
| Si | 3 | - |
| Sn | 2231 | 1600 |
| Ti | 99 | 54 |
| Zn | 71 | 58.5 |

*: Arsenic and cadmium were assigned provisional route-based rock-to-metal proxies equal to those of their dominant parent production systems, namely copper (513 t rock/t metal) and zinc (71 t rock/t metal), respectively. These values should be interpreted as production-route proxies rather than Cd- or As-specific rock-to-metal ratios, because the RMR framework allocates burdens based on operation-specific revenue shares, recoveries, and mined material flows.

**: In the case of lead, data was obtained from: https://thebreakthrough.org/issues/energy/updated-mining-footprints-and-raw-material-needs-for-clean-energy

Table S45. ISO 3166-1 alpha-3 country codes corresponding to the countries listed in this study.

| Country | Code | Country | Code | Country | Code |
| --- | --- | --- | --- | --- | --- |
| USA | USA | Russia | RUS | China | CHN |
| UK | GBR | International | INT* | Japan | JPN |
| India | IND | Germany | DEU | France | FRA |
| Canada | CAN | Italy | ITA | Spain | ESP |
| Argentina | ARG | South Korea | KOR | Israel | ISR |
| Luxembourg | LUX | Australia | AUS | Saudi Arabia | SAU |
| Brazil | BRA | Turkey | TUR | Switzerland | CHE |
| Singapore | SGP | Indonesia | IDN | UAE | ARE |
| Sweden | SWE | Iran | IRN | Netherlands | NLD |
| Norway | NOR | Thailand | THA | Mexico | MEX |
| Denmark | DNK | Poland | POL | South Africa | ZAF |
| Egypt | EGY | Finland | FIN | Malaysia | MYS |
| Ukraine | UKR | Belgium | BEL | Chile | CHL |
| Kazakhstan | KAZ | North Korea | PRK | Greece | GRC |
| Bulgaria | BGR | Vietnam | VNM | Hungary | HUN |
| Lithuania | LTU | Philippines | PHL | Czechoslovakia | CSK** |
| Czech Republic | CZE | Pakistan | PAK | Algeria | DZA |
| Nigeria | NGA | Romania | ROU | New Zealand | NZL |
| Peru | PER | Estonia | EST | Belarus | BLR |
| Ecuador | ECU | Austria | AUT | Slovenia | SVN |
| Colombia | COL | Venezuela | VEN | Oman | OMN |
| Angola | AGO | Morocco | MAR | Kuwait | KWT |
| Ethiopia | ETH | Nepal | NPL | Latvia | LVA |
| Uruguay | URY | Turkmenistan | TKM | Moldova | MDA |
| Portugal | PRT | Monaco | MCO | Tunisia | TUN |
| Armenia | ARM | Rwanda | RWA | Djibouti | DJI |
| Qatar | QAT | Myanmar | MMR | Azerbaijan | AZE |
| Slovakia | SVK | Ireland | IRL | Jordan | JOR |
| Bangladesh | BGD | Costa Rica | CRI | Mauritius | MUS |
| Guatemala | GTM | Bolivia | BOL | Kenya | KEN |
| Vatican | VAT | Sudan | SDN | Laos | LAO |

*INT = non-official placeholder for international entities.

**CSK = historical ISO code (Czechoslovakia).

These standardized codes are widely used for data harmonization and international comparisons. “International” (INT) does not correspond to an official ISO designation and is used here as a placeholder to represent multinational or non-country-specific activities. “Czechoslovakia” (CSK) refers to a historical entity and is included using its former ISO code (ISO 3166-3) for consistency with legacy data.

References:

1.	Affairs, U.N.O.f.O.S. (2026). United Nations Register of Objects Launched into Outer Space. https://www.unoosa.org/oosa/en/spaceobjectregister/index.html.

2.	European Space Agency (ESA) (2025). Space Environment Statistics. https://sdup.esoc.esa.int/discosweb/statistics/.

3.	United States Space Force (USSF) (2025). Space-Track: Box Score (dashboard). https://www.space-track.org/#boxscore.

4.	United Nations Office for Outer Space Affairs (1974). Resolution adopted by the general assembly. https://www.unoosa.org/oosa/en/ourwork/spacelaw/treaties/registration-convention.html.

5.	Le May, S., Carter, B.A., Gehly, S., Flegel, S., and Jah, M. (2020). Representing and querying space object registration data using graph databases. Acta Astronautica 173, 392-403. https://doi.org/10.1016/j.actaastro.2020.04.056.

6.	Jakhu, R.S., Jasani, B., and McDowell, J.C. (2018). Critical issues related to registration of space objects and transparency of space activities. Acta Astronautica 143, 406-420. https://doi.org/10.1016/j.actaastro.2017.11.042.

7.	Masson-Zwaan, T., Martinez, P., Letizia, F., Melograna, C., Reynders, M., Rovetto, R., Skinner, M.A., Stanciu-Manolescu, M., Strah, M., Volynskaya, O., and Wang, G. (2024). The need to improve registration practices in the context of space traffic management. Acta Astronautica 223, 242-248. https://doi.org/10.1016/j.actaastro.2024.06.052.

8.	Chris Daehnick, J.G., and Ilan Rozenkopf (2023). Space launch: Are we heading for oversupply or a shortfall? https://www.mckinsey.com/industries/aerospace-and-defense/our-insights/space-launch-are-we-heading-for-oversupply-or-a-shortfall.

9.	U.S. Government Accountability Office (2022). Large Constellations of Satellites: Mitigating environmental and other effects. https://www.gao.gov/assets/gao-22-105166.pdf.

10.	Maloney, C.M., Portmann, R.W., Ross, M.N., and Rosenlof, K.H. (2025). Investigating the Potential Atmospheric Accumulation and Radiative Impact of the Coming Increase in Satellite Reentry Frequency. Journal of Geophysical Research: Atmospheres 130, e2024JD042442. https://doi.org/10.1029/2024JD042442.

11.	Ravilious, K. (2025). Dying satellites can drive climate change and ozone depletion, study finds. https://www.theguardian.com/news/2025/may/01/dying-satellites-can-drive-climate-change-and-ozone-depletion-study-finds.

12.	X, S. (2025). Starlink Mission. https://www.spacex.com/launches/.

13.	Amazon (2025). Project Kuiper. https://www.aboutamazon.com/what-we-do/devices-services/project-kuiper.

14.	Eutelsat Group (2025). Our Story. https://oneweb.net/.

15.	Jones, A. (2025). China launches third batch of Guowang megaconstellation satellites. https://spacenews.com/china-launches-third-batch-of-guowang-megaconstellation-satellites/.

16.	Hayashi, F., Kioka, A., Ishii, T., and Nakamura, T. (2025). Unveiling the resource potential of space debris: A forecast of valuable metals to 2050. Waste Management 193, 376-385. https://doi.org/10.1016/j.wasman.2024.12.019.

17.	NASA (2025). NASA Orbital Debris Engineering Model. https://orbitaldebris.jsc.nasa.gov/modeling/ordem.html.

18.	ESA (2025). Analysis and prediction. https://www.esa.int/Space_Safety/Space_Debris/Analysis_and_prediction.

19.	NASA (2022). Where Do Old Satellites Go When They Die? https://spaceplace.nasa.gov/spacecraft-graveyard/en/.

20.	Witze, A. (2018). The quest to conquer Earth's space junk problem. Nature 561, 24+.

21.	Park, S.-H., Mischler, S., and Leyland, P. (2021). Re-entry analysis of critical components and materials for design-for-demise techniques. Advances in Space Research 68, 1-24. https://doi.org/10.1016/j.asr.2021.03.019.

22.	Federal Aviation Administration (FAA) (2021). Risk Associated with Reentry Disposal of Satellites from Proposed Large Constellations in Low Earth Orbit. https://www.faa.gov/sites/faa.gov/files/Report_to_Congress_Reentry_Disposal_of_Satellites.pdf.

23.	Pardini, C., and Anselmo, L. (2019). Uncontrolled re-entries of spacecraft and rocket bodies: A statistical overview over the last decade. Journal of Space Safety Engineering 6, 30-47. https://doi.org/10.1016/j.jsse.2019.02.001.

24.	Skoulidou, D.K., Rosengren, A.J., Tsiganis, K., and Voyatzis, G. (2019). Medium Earth Orbit dynamical survey and its use in passive debris removal. Advances in Space Research 63, 3646-3674. https://doi.org/10.1016/j.asr.2019.02.015.

25.	Lawler, S. (2023). Bright satellites are disrupting astronomy research worldwide. Nature 623, 917-918. 10.1038/d41586-023-03610.

26.	European Space Agency (ESA) Space Debris Office (2025). ESA’s Annual Space Environment Report. Technical Report GEN-DB-LOG-00288-OPS-SD.

27.	Murphy, D.M., Abou-Ghanem, M., Cziczo, D.J., Froyd, K.D., Jacquot, J., Lawler, M.J., Maloney, C., Plane, J.M.C., Ross, M.N., Schill, G.P., and Shen, X. (2023). Metals from spacecraft reentry in stratospheric aerosol particles. Proceedings of the National Academy of Sciences 120, e2313374120. https://doi.org/10.1073/pnas.2313374120.

28.	Williams, A., Boley, A., Rotola, G., and Green, R. (2024). Sustainable skies and the Earth–space environment. Nature Sustainability 7, 228-231. 10.1038/s41893-024-01308-8.

29.	Ryan, R.G., Marais, E.A., Balhatchet, C.J., and Eastham, S.D. (2022). Impact of Rocket Launch and Space Debris Air Pollutant Emissions on Stratospheric Ozone and Global Climate. Earth's Future 10, e2021EF002612. https://doi.org/10.1029/2021EF002612.

30.	Dallas, J.A., Raval, S., Alvarez Gaitan, J.P., Saydam, S., and Dempster, A.G. (2020). The environmental impact of emissions from space launches: A comprehensive review. Journal of Cleaner Production 255, 120209. https://doi.org/10.1016/j.jclepro.2020.120209.

31.	Wilson, A.R., Vasile, M., Maddock, C.A., and Baker, K.J. (2022). Ecospheric life cycle impacts of annual global space activities. Science of The Total Environment 834, 155305. https://doi.org/10.1016/j.scitotenv.2022.155305.

32.	IEA (2025). Global Critical Minerals Outlook 2025. IEA. https://www.iea.org/reports/global-critical-minerals-outlook-2025.

33.	IEA (2021). The Role of Critical Minerals in Clean Energy Transitions. https://www.iea.org/reports/the-role-of-critical-minerals-in-clean-energy-transitions.

34.	Kirsten Hund, D.L.P., Thao P. Fabregas, Tim Laing, John Drexhage (2020). Minerals for Climate Action: The Mineral Intensity of the Clean Energy Transition. Climate Smart Mining Initiative - The World Bank Group. https://pubdocs.worldbank.org/en/961711588875536384/Minerals-for-Climate-Action-The-Mineral-Intensity-of-the-Clean-Energy-Transition.pdf.

35.	IEA (2025). World Energy Outlook 2025. https://www.iea.org/reports/world-energy-outlook-2025.

36.	Gómez, M., Fishman, T., Zhu, X., Li, J., and Zeng, X. (2026). Scaling wind power: addressing critical metals challenges and opportunities in the global energy transition to 2050. Resources, Conservation and Recycling 231, 108924. https://doi.org/10.1016/j.resconrec.2026.108924.

37.	IEA (2023). Critical Minerals Market Review 2023. https://www.iea.org/reports/critical-minerals-market-review-2023.

38.	Klinger, J.M., Armstrong, E., and Richaud, R. (2024). Wasting and wanting: An extractive supply chain approach to outer space geographies. Environment and Planning D: Society and Space 43, 283-304. 10.1177/02637758241274629.

39.	Wischer G, A.G., Bazilian MD (2024). Mineral Supply Chains and Space Assets: Mitigating Manufacturing Dependencies. Æther: A Journal of Strategic Airpower & Spacepower 2.

40.	Quan, Y., and Tan-Soo, J.-S. (2026). Deforestation-induced emissions from mining energy transition minerals. Nature Climate Change 16, 52-57. 10.1038/s41558-025-02520-w.

41.	Sprecher, B., and Kleijn, R. (2021). Tackling material constraints on the exponential growth of the energy transition. One Earth 4, 335-338. 10.1016/j.oneear.2021.02.020.

42.	Nuss, P., and Eckelman, M.J. (2014). Life Cycle Assessment of Metals: A Scientific Synthesis. PLOS ONE 9, e101298. 10.1371/journal.pone.0101298.

43.	ESA (2023). ESA moves ahead with In-Orbit Servicing missions. https://www.esa.int/Enabling_Support/Preparing_for_the_Future/Discovery_and_Preparation/ESA_moves_ahead_with_In-Orbit_Servicing_missions2.

44.	Jones, A. (2022). China’s Shijian-21 towed dead satellite to a high graveyard orbit. https://spacenews.com/chinas-shijian-21-spacecraft-docked-with-and-towed-a-dead-satellite/.

45.	Clark, S. (2024). NASA cancels a multibillion-dollar satellite servicing demo mission. https://arstechnica.com/space/2024/03/nasa-cancels-a-multibillion-dollar-satellite-servicing-demo-mission/.

46.	Yang, Z., Liu, L., Xing, L., Amara, A., and Xuan, J. (2025). Resource and material efficiency in the circular space economy. Chem Circularity. https://doi.org/10.1016/j.wasman.2024.12.019.

47.	Baker, C.R., Zhu, N., Ravindranath, P.K., Pawelski, J.W., Ritter, C.L., Swinney, R.M., Matthews, W.S., Fleck, T.J., Jordon, J.B., and Allison, P.G. (2025). Sustainable in-space manufacturing by upcycling metal space debris via a vertically integrated processing paradigm. npj Advanced Manufacturing 2, 29. 10.1038/s44334-025-00042-z.

48.	Buitrago-Leiva, J.N., Camps, A., and Moncada Niño, A. (2024). Considerations for Eco-LeanSat Satellite Manufacturing and Recycling. Sustainability 16, 4933. 10.3390/su16124933.

49.	Opromolla, R., Grishko, D., Auburn, J., Bevilacqua, R., Buinhas, L., Cassady, J., Jäger, M., Jankovic, M., Rodriguez, J., Perino, M.A., and Bastida-Virgili, B. (2024). Future in-orbit servicing operations in the space traffic management context. Acta Astronautica 220, 469-477. https://doi.org/10.1016/j.actaastro.2024.05.007.

50.	Krebs, G.D. (2024). Chronology of Space Launches. https://space.skyrocket.de/directories/chronology.htm.

51.	Swartwout, M. (2023). The Internet Ruins Everything: The Sixth Age of Small Satellites. 4-11 March 2023. pp. 1-8.

52.	Shkolnik, E.L. (2018). On the verge of an astronomy CubeSat revolution. Nature Astronomy 2, 374-378. 10.1038/s41550-018-0438-8.

53.	Gómez, M., Grimes, S., Qian, Y., Feng, Y., and Fowler, G. (2023). Critical and strategic metals in mobile phones: A detailed characterisation of multigenerational waste mobile phones and the economic drivers for recovery of metal value. Journal of Cleaner Production, 138099. https://doi.org/10.1016/j.jclepro.2023.138099.

54.	Hall, W.J., and Williams, P.T. (2007). Separation and recovery of materials from scrap printed circuit boards. Resources, Conservation and Recycling 51, 691-709. https://doi.org/10.1016/j.resconrec.2006.11.010.

55.	Kasper, A.C., Berselli, G.B.T., Freitas, B.D., Tenório, J.A.S., Bernardes, A.M., and Veit, H.M. (2011). Printed wiring boards for mobile phones: Characterization and recycling of copper. Waste Management 31, 2536-2545. https://doi.org/10.1016/j.wasman.2011.08.013.

56.	Xiu, F.-R., Qi, Y., and Zhang, F.-S. (2015). Leaching of Au, Ag, and Pd from waste printed circuit boards of mobile phone by iodide lixiviant after supercritical water pre-treatment. Waste Management 41, 134-141. https://doi.org/10.1016/j.wasman.2015.02.020.

57.	Tan, Q., Dong, Q., Liu, L., Song, Q., Liang, Y., and Li, J. (2017). Potential recycling availability and capacity assessment on typical metals in waste mobile phones: A current research study in China. Journal of Cleaner Production 148, 509-517. https://doi.org/10.1016/j.jclepro.2017.02.036.

58.	Fontana, D., Pietrantonio, M., Pucciarmati, S., Rao, C., and Forte, F. (2019). A comprehensive characterization of End-of-Life mobile phones for secondary material resources identification. Waste Management 99, 22-30. https://doi.org/10.1016/j.wasman.2019.08.011.

59.	Park, H.S., and Kim, Y.J. (2019). A novel process of extracting precious metals from waste printed circuit boards: Utilization of gold concentrate as a fluxing material. Journal of Hazardous Materials 365, 659-664. https://doi.org/10.1016/j.jhazmat.2018.11.051.

60.	Jeon, S., Tabelin, C.B., Park, I., Nagata, Y., Ito, M., and Hiroyoshi, N. (2020). Ammonium thiosulfate extraction of gold from printed circuit boards (PCBs) of end-of-life mobile phones and its recovery from pregnant leach solution by cementation. Hydrometallurgy 191, 105214. https://doi.org/10.1016/j.hydromet.2019.105214.

61.	Yost, B., and Weston, S. (2024). State-of-the-Art of Small Spacecraft Technology. NASA. http://www.nasa.gov/smallsat-institute/sst-soa.

62.	Gervais, E., Shammugam, S., Friedrich, L., and Schlegl, T. (2021). Raw material needs for the large-scale deployment of photovoltaics – Effects of innovation-driven roadmaps on material constraints until 2050. Renew Sust Energ Rev 137, 110589. https://doi.org/10.1016/j.rser.2020.110589.

63.	Torabi, F., and Ahmadi, P. (2020). Chapter 7 - Zinc–silver oxide batteries. In Simulation of Battery Systems, F. Torabi, and P. Ahmadi, eds. (Academic Press), pp. 217-262. https://doi.org/10.1016/B978-0-12-816212-5.00011-8.

64.	Nogueira, C.A., and Margarido, F. (2004). Leaching behaviour of electrode materials of spent nickel–cadmium batteries in sulphuric acid media. Hydrometallurgy 72, 111-118. https://doi.org/10.1016/S0304-386X(03)00123-3.

65.	Yang, C.-C. (2003). Recovery of heavy metals from spent Ni–Cd batteries by a potentiostatic electrodeposition technique. Journal of Power Sources 115, 352-359. https://doi.org/10.1016/S0378-7753(03)00015-6.

66.	Reddy, B.R., Priya, D.N., and Park, K.H. (2006). Separation and recovery of cadmium(II), cobalt(II) and nickel(II) from sulphate leach liquors of spent Ni–Cd batteries using phosphorus based extractants. Separation and Purification Technology 50, 161-166. https://doi.org/10.1016/j.seppur.2005.11.020.

67.	Vassura, I., Morselli, L., Bernardi, E., and Passarini, F. (2009). Chemical characterisation of spent rechargeable batteries. Waste Management 29, 2332-2335. https://doi.org/10.1016/j.wasman.2009.03.033.

68.	Burke, K. (2012). 4.02 - Current Perspective on Hydrogen and Fuel Cells. In Comprehensive Renewable Energy, A. Sayigh, ed. (Elsevier), pp. 29-63. https://doi.org/10.1016/B978-0-08-087872-0.00402-9.

69.	Shin, S.M., Kim, N.H., Sohn, J.S., Yang, D.H., and Kim, Y.H. (2005). Development of a metal recovery process from Li-ion battery wastes. Hydrometallurgy 79, 172-181. https://doi.org/10.1016/j.hydromet.2005.06.004.

70.	Winslow, K.M., Laux, S.J., and Townsend, T.G. (2018). A review on the growing concern and potential management strategies of waste lithium-ion batteries. Resources, Conservation and Recycling 129, 263-277. https://doi.org/10.1016/j.resconrec.2017.11.001.

71.	Müller, M., Obuz, H.E., Keber, S., Tekmanli, F., Mettke, L.N., and Yagmurlu, B. (2024). Concepts for the Sustainable Hydrometallurgical Processing of End-of-Life Lithium Iron Phosphate (LFP) Batteries. Sustainability 16. 10.3390/su162411267.

72.	Guimarães, L.F., Tenório, J.A., Vaccari, M., Espinosa, D.C., and Botelho Junior, A.B. (2024). Characterization of Lithium-Ion Batteries from Recycling Perspective towards Circular Economy. Minerals 14. 10.3390/min14090878.

73.	Wilke, C., Kaas, A., and Peuker, U.A. (2024). Influence of the cell type on yield and composition of black mass deriving from a mechanical recycling process of automotive Lithium-ion batteries. Next Sustainability 4, 100050. https://doi.org/10.1016/j.nxsust.2024.100050.

74.	van de Ven, J.J.M.M., Yang, Y., and Abrahami, S.T. (2024). A closer look at lithium-ion batteries in E-waste and the potential for a universal hydrometallurgical recycling process. Scientific Reports 14, 16661. 10.1038/s41598-024-67507-7.

75.	Wei, X., Guo, Z., Zhao, Y., Sun, Y., Hankin, A., and Titirici, M. (2025). Recovery of graphite from industrial lithium-ion battery black mass. RSC Sustainability 3, 264-274. 10.1039/D4SU00427B.

76.	Pisacane, V.L. (2003). Spacecraft Systems Design and Engineering. In Encyclopedia of Physical Science and Technology (Third Edition), R.A. Meyers, ed. (Academic Press), pp. 463-483. https://doi.org/10.1016/B0-12-227410-5/00888-7.

77.	United States Geological Survey (USGS) (2025). Statistics and Information. https://www.usgs.gov/search?keywords=Statistics+and+Information.

78.	Di Mauro, G., Lawn, M., and Bevilacqua, R. (2017). Survey on Guidance Navigation and Control Requirements for Spacecraft Formation-Flying Missions. Journal of Guidance, Control, and Dynamics 41, 581-602. 10.2514/1.G002868.

79.	Stevan M. Spremo, A.R.C., and Tina L. Panontin (2017). Small Spacecraft Overview. NASA. https://ntrs.nasa.gov/citations/20190031730.

80.	Spacecraft Mass.  (2008). In Spacecraft Structures, J.J. Wijker, ed. (Springer Berlin Heidelberg), pp. 215-228. 10.1007/978-3-540-75553-1_14.

81.	Torrubia, J., Valero, A., and Valero, A. (2023). Energy and carbon footprint of metals through physical allocation. Implications for energy transition. Resources, Conservation and Recycling 199, 107281. https://doi.org/10.1016/j.resconrec.2023.107281.

82.	Meißner, S. (2021). The Impact of Metal Mining on Global Water Stress and Regional Carrying Capacities—A GIS-Based Water Impact Assessment. Resources 10, 120. 10.3390/resources10120120.

83.	Nassar, N.T., Lederer, G.W., Padilla, A.J., Gambogi, J., Cordier, D.J., Brainard, J.L., Lessard, J.D., and Charab, R. (2023). Rock-to-metal ratios of the rare earth elements. Journal of Cleaner Production 405, 136958. https://doi.org/10.1016/j.jclepro.2023.136958.

84.	Nassar, N.T., Lederer, G.W., Brainard, J.L., Padilla, A.J., and Lessard, J.D. (2022). Rock-to-Metal Ratio: A Foundational Metric for Understanding Mine Wastes. Environmental Science & Technology 56, 6710-6721. https://doi.org/10.1021/acs.est.1c07875.