Electronic supporting information (ESI)

Reversible hydrogen storage with aromatics: boosting LOHC effectiveness through oxygen functionality

Sergey P. Verevkina,* Arina V. Elbakari,b Artemiy A. Samarov,c Sergey V. Vostrikov,b Anton V. Eremin,d Riko Siewerta

a Institute of Technical Thermodynamics, University of Rostock, 18059 Rostock, Germany

b Engineering and Technology Department, Samara State Technical University, 443100 Samara, Russia

c Department of Chemical Thermodynamics and Kinetics, Saint Petersburg State University, Peterhof, 198504 Saint Petersburg, Russia.

d Department of Industrial Heat Power Engineering, Samara State Technical University, 443100 Samara, Russia

Table S1

Calculation of the liquid phase reaction enthalpies, ∆rHmo(liq), the total Gibbs free energies of reactions, ∆rGmo(liq), and the equilibrium temperatures, Teq, of the hydrogenation (in kJ·mol-1)

| HL compound | ∆fHmo(liq)HLa 298K | HR compound | ∆fHmo(liq)HRb 298 K | ∆rHmo(liq)/H2 c 298 K | ∆rGmo(liq)d 500 K | Teq/K e |
| --- | --- | --- | --- | --- | --- | --- |
| benzene | 49.0±0.9 | cyclohexane | -156.4±0.8 | -68.5 | -21.1 | 569 |
| toluene | 12.0±1.1 | methyl-cyclohexane | -190.2±1.0 | -67.4 | -16.7 | 555 |
| naphthalene | 95.2±2.1 | decalin | -230.6±1.0 | -65.2 | -2.9 | 535 |
| indane | 11.5±1.6 | 6H-indane | -176.2±1.8 | -62.6 | -0.1 | 511 |
| phenol | -150.5±1.3 | cyclohexanol | -352.0±0.7 | -67.2 | -9.0 | 534 |
| methoxy-benzene | -116.9±0.7 | methoxy-cyclohexane | -309.8±3.6 | -64.3 | -10.1 | 542 |
| furan | -62.3±0.7 | tetrahydrofuran | -216.2±0.7 | -77.0 | -17.0 | 657 |
| cyclopentanone | -236.2±1.1 | cyclopentanol | -300.1±1.6 | -63.9 | 8.2 | 442 |
| cyclohexanone | -271.2±2.1 | cyclohexanol | -352.0±0.7 | -80.8 | -3.3 | 519 |

a Liquid-phase enthalpies of formation of hydrogen-lean (HL) counterparts of the LOHC systems from Table S15.

b Liquid-phase enthalpies of formation of hydrogen-rich (HR) counterparts of the LOHC systems from Table S15.

c Reaction enthalpies per 1 mole of H2.

d The total Gibbs free energies of hydrogenation reactions.

e The equilibrium temperature of hydrogenation reactions.

General aspects of quantum-chemical calculations

The conformational analysis was performed with a computer code called CREST (conformer-rotamer ensemble sampling tool). The software package uses a metadynamics simulation-based screening procedure that is used to generate molecular conformations in the gas phase. A history-dependent bias potential is applied, where the collective variables for the metadynamics are the previous structures on the potential energy surface, expressed as the root-mean-square deviation of the atoms between them, which are calculated according to the quaternion algorithm. Conformations are generated at the GFNn-xTB level in the iMTD-GC workflow. More details are provided in the Ref. [1]. The obtained structures were optimised with the B3LYP/6-31g(d,p) method [2]. It turned out that the energetics of the conformers were quite close to each other within a few kJ·mol-1. The most stable conformers for the aromatic compounds and their hydrogenated products are shown in Table S2.

Table S2. Structures of the most stable conformers of compounds dtudied in this work (in kJ·mol-1)

| Side view | Top view | ∆fHmo(g)a |
| --- | --- | --- |
|  |  | -68.2 |
| 1-indanone [83-33-0] |  |  |
|  |  | -249.2 |
| 6H-1-indanone [29927-85-3] |  |  |
|  |  | -297.8 |
| 6H-1-indanol [53927-31-4] |  |  |
|  |  | -56.9 |
| 2-indanone [615-13-4] |  |  |
|  |  | -252.1 |
| 6H-2-indanone [6088-72-8] |  |  |
|  |  | -296.0 |
| 6H-2-indanol [59078-17-0] |  |  |
|  |  | -97.8 |
| 2-Me-1-indanone [17496-14-9] |  |  |
|  |  | -277.5 |
| 6H-2-Me-1-indanone [2308731-82-8] |  |  |
|  |  | -326.8 |
| cis-6H-2-Me-1-indanol [16149-29-4] |  |  |
|  |  | -97.8 |
| 3-Me-1-indanone [6072-57-7] |  |  |
|  |  | -281.3 |
| 6H-3-Me-1-indanone [64558-16-3] |  |  |
|  |  | -329.5 |
| 6H-3-Me-1-indanol [857773-69-4] |  |  |
|  |  | -101.3 |
| 6-Me-1-indanone [24623-20-9] |  |  |
|  |  | -281.0 |
| 6H-6-Me-1-indanone [1824616-45-6] |  |  |
|  |  | -329.6 |
| 6H-6-Me-1-indanol |  |  |
|  |  | -225.1 |
| 4-methoxy-1-indanone [13336-31-7] |  |  |
|  |  | -397.6 |
| 6H-4-methoxy-1-indanone [1823221-69-7] |  |  |
|  |  | -446.8 |
| 6H-4-methoxy-1-indanol [1823625-45-1] |  |  |
|  |  | -225.8 |
| 5-methoxy-1-indanone [5111-70-6] |  |  |
|  |  | -395.2 |
| 6H-5-methoxy-1-indanone |  |  |
|  |  | -444,6 |
|  |  | -444,9 |
| 6H-5-methoxy-1-indanol |  |  |
|  |  | -224.1 |
| 6-methoxy-1-indanone [13623-25-1] |  |  |
|  |  | -396.1 |
| 6H-6-methoxy-1-indanone |  |  |
|  |  | -445.0 |
| 6H-6-methoxy-1-indanol [13336-33-9] |  |  |
|  |  | -370.3 |
| 5,6-dimethoxy-1-indanone [2107-69-9] |  |  |
|  |  | -537.8 |
| 6H-5,6-dimethoxy-1-indanone [1195675-41-2] |  |  |
|  |  | -588.3 |
| 6H-5,6-dimethoxy-1-indanol |  |  |
|  |  | -174.9 |
| 1,3-indanedione [606-23-5] |  |  |
|  |  | -347.8 |
| 6H-1,3-indanedione [16385-56-1] |  |  |
|  |  | -461.7 |
| 6H-1,3-indanediol |  |  |
|  |  | -222.8 |
| 6,7-dihydro-4-benzofuranone [16806-93-2] |  |  |
|  |  | -345.9 |
| 6H-4-benzofuranone [101581-29-7] |  |  |
|  |  | -409.1 |
| 8H-4-benzofuranol [99767-96-1] |  |  |
|  |  | -111.3 |
| 1-indanol [6351-10-6] |  |  |
|  |  | -297.8 |
| 6H-1-indanol [53927-31-4] |  |  |
|  |  | -114.9 |
| 2-indanol [4254-29-9] |  |  |
|  |  | -296.0 |
| 6H-2-indanol [59078-17-0] |  |  |

a Calculated using G3MP2 method (see Tables S3 to S6).

The Gaussian 16 series software [3] was used for QC calculations. The total H298-enthalpies and of the most stable conformers were calculated using the G3MP2 [4] .The G3MP2 method was chosen as the optimal approach combining high accuracy of energy calculation with reasonable computational cost. This method has been systematically used in our laboratory for calculations of C-, H-, O-containing molecules, and in our experience G3MP2 provides the theoretical enthalpies of formation in very good agreement with the reliable experimental values. The H298-values were finally converted to the theoretical   ∆fHmo(g, 298 K)QC values and discussed. The well-established assumption “rigid rotator-harmonic oscillator” was used for the quantum-chemical calculations. The Smo(g, 298 K)QC values were calculated according to Gibbs-Helmholtz equation, using H298 and G298 from the output file. The general details of the calculation procedure have been published elsewhere [5].

Table S3

Results of quantum-chemical calculations with G3MP2 method for LOHC pairs (at T = 298 K, p° = 0.1 MPa, all data are in kJ·mol-1)

| Compound, CAS | ∆fHmo(g)ATa | ∆fHmo(g)corrb | Smo(g) c |
| --- | --- | --- | --- |
| aromatic ketones [from Table S4] |  |  |  |
| 1-indanone, 83-33-0 | -68.2 | -62.9 | 367.8 |
| 2-indanone, 615-13-4 | -56.9 | -52.0 | 366.3 |
| 2-Me-1-indanone, 17496-14-9 | -97.8 | -91.4 | 395.0 |
| 3-Me-1-indanone, 6072-57-7 | -97.8 | -91.4 | 399.1 |
| 5-Me-1-indanone, 4593-38-8 | -101.9 | -95.3 | 410.0 |
| 6-Me-1-indanone, 24623-20-9 | -101.3 | -94.8 | 409.3 |
| 4-methoxy-1-indanone, 13336-31-7 | -225.1 | -213.9 | 422.5 |
| 5-methoxy-1-indanone, 5111-70-6 | -225.8 | -214.6 | 423.5 |
| 6-methoxy-1-indanone, 13623-25-1 | -224.1 | -212.9 | 423.4 |
| 5,6-dimethoxy-1-indanone, 2107-69-9 | -370.3 | -353.6 | 477.2 |
| 1,3-indandione, 606-23-5 | -174.9 | -165.6 | 377.6 |
| aliphatic ketones [from Table S5] |  |  |  |
| 6H-1-indanone, [29927-85-3] | -249.2 | -248.4 | 383.5 |
| 6H-2-indanone, [6088-72-8] | -252.1 | -251.4 | 383.0 |
| 6H-2-Me-1-indanone [2308731-82-8] | -277.5 | -276.7 | 413.7 |
| 6H-3-Me-1-indanone [64558-16-3] | -281.3 | -280.5 | 413.0 |
| 6H-5-Me-1-indanone, [92015-40-2] | -280.8 | -280.0 | 412.8 |
| 6H-6-Me-1-indanone [1824616-45-6] | -281.0 | -280.2 | 412.8 |
| 6H-4-methoxy-1-indanone [1823221-69-7] | -397.6 | -396.9 | 444.3 |
| 6H-5-methoxy-1-indanone | -395.2 | -394.5 | 445.2 |
| 6H-6-methoxy-1-indanone | -396.1 | -395.4 | 444.3 |
| 6H-5,6-dimethoxy-1-indanone [1195675-41-2] | -538.7 | -538.1 | 503.9 |
| 6H-1,3-indandione [16385-56-1] | -347.8 | -347.1 | 402.2 |
| 6H-4-benzofuranone [101581-29-7] | -345.9 | -345.2 | 385.4 |
| 6,7-dihydro-4(5H)-benzofuranone, 16806-93-2 | -222.6 | -221.8 | 365.7 |
| 6,7-Dihydro-5(4H)-benzofuranone | -202.9 | -202.1 |  |
| 4,5-Dihydro-6(7H)-benzofuranone [169378-54-5] | -202.7 | -201.9 |  |
| 5,6-Dihydro-7(4H)-benzofuranone [108153-93-1] | -212.0 | -211.2 |  |
| alcohols [from Table S6] |  |  |  |
| 1-indanol, 6351-10-6 | -111.3 | -107.3 | 369.3 |
| 2-indanol, 4254-29-9 | -114.9 | -110.8 | 368.2 |
| 6H-1-indanol [53927-31-4] | -297.8 | -296.2 | 395.2 |
| 6H-2-indanol [59078-17-0] | -296.0 | -294.3 | 398.6 |
| 6H-2-Me-1-indanol [16149-29-4] | -326.8 | -325.5 | 422.1 |
| 6H-3-Me-1-indanol [857773-69-4] | -329.5 | -328.3 | 425.7 |
| 6H-5-Me-1-indanol | -329.5 | -328.3 | 424.5 |
| 6H-6-Me-1-indanol | -329.6 | -328.4 | 424.5 |
| 6H-4-methoxy-1-indanol [1823625-45-1] | -446.8 | -447.1 | 455.8 |
| 6H-5-methoxy-1-indanol | -444.6 | -444.9 | 457.0 |
| 6H-6-methoxy-1-indanol [13336-33-9] | -445.0 | -445.3 | 456.2 |
| 6H-5,6-methoxy-1-indanol | -588.3 | -590.5 | 515.5 |
| 6H-1,3-indanediol | -461.7 | -462.2 | 420.5 |
| 6H-4-benzofuranol [99767-96-1] | -409.1 | -408.9 | 390.5 |
| 6H-1-indanol [53927-31-4] | -297.8 | -296.2 | 395.2 |
| 6H-2-indanol [59078-17-0] | -296.0 | -294.3 | 398.6 |
| 8H-4-benzofuranol [99767-96-1] | -409.1 | -408.9 | 390.5 |
| 1,2-dimethoxy-cyclohexane d | -416.6 | -410.7 |  |

a Calculated by G3MP2 and atomization reaction with the expanded uncertainties of ±4.1 kJ·mol-1 [4].

b Corrected atomization results from in Tables S4-S6.

c Calculated by G3MP2 according to Gibbs-Helmholtz equation, using H298 and G298 from the output file.

d Calculated in this work by G4 and atomization reaction and corrected according to equation:

∆fHmo(g)corr/ kJ.mol-1 = 0.9868 × ∆fHmo(g, AT)G4 + 0.45 	with R2 = 0.9989, developed in our previous work [6].

Table S4

Results of quantum-chemical calculations with G3MP2 method for aromatic ketones (at T = 298 K, p° = 0.1 MPa, all data are in kJ·mol-1)

| CAS | Compound | ∆fHmo(g)ATa | ∆fHmo(g)expb | ∆fHmo(g)corrc | d |
| --- | --- | --- | --- | --- | --- |
| 83-33-0 | 1-indanone | -68.2 | -63.9±1.1 | -62.9 | -1.0 |
| 615-13-4 | 2-indanone | -56.9 | -54.6±3.1 | -52.0 | -2.6 |
| 17496-14-9 | 2-Methyl-1-indanone | -97.8 |  | -91.4 | 91.4 |
| 6072-57-7 | 3-Methyl-1-indanone | -97.8 | -91.2±3.2 | -91.4 | 0.2 |
| 4593-38-8 | 5-Methyl-1-indanone | -101.9 |  | -95.3 |  |
| 24623-20-9 | 6-Methyl-1-indanone | -101.3 | -95.1±2.8 | -94.8 | -0.3 |
| 13336-31-7 | 4-Methoxy-1-indanone | -225.1 | -217.6±3.4 | -213.9 | -3.7 |
| 5111-70-6 | 5-Methoxy-1-indanone | -225.8 |  | -214.6 |  |
| 13623-25-1 | 6-Methoxy-1-indanone | -224.1 | -216.0±3.4 | -212.9 | -3.1 |
| 2107-69-9 | 5,6-dimethoxy-1-indanone | -370.3 | -356.7±3.4 | -353.6 | -3.1 |
| 606-23-5 | 1,3-indandione | -174.9 | -166.9±2.6 | -165.6 | -1.3 |
| 529-34-0 | 1-tetralone [7] | -102.8 | -95.3±0.8 | -96.2 | 0.9 |
| 530-93-8 | 2-tetralone [8] | -91.5 | -82.0±4.1 | -85.3 | 3.3 |
| 1078-19-9 | 6-Methoxy-1-tetralon [9] | -261.4 | -243.0±2.8 | -248.8 | 5.8 |
| 33892-75-0 | 5-Methoxy-1-tetralon [9] | -257.4 | -244.8±1.9 | -245.0 | 0.2 |
| 6836-19-7 | 7-Methoxy-1-tetralon [9] | -259.0 | -242.3±2.6 | -246.5 | 4.2 |

a Calculated by G3MP2 and atomization reaction with the expanded uncertainties of ±4.1 kJ·mol-1 [4].

b The experimental data as compiled from the original literature (see Table 1 and the references inside this table) and evaluated using the on-line data base [10].

c Calculated according to equation:

∆fHmo(g)corr/ kJ.mol-1 = 0.9622 × ∆fHmo(g, AT)G3MP2 + 2.7 	with R2 = 0.999,

derived from the correlations of the results given in columns 3 and 4 of this table.

d Difference between column 4 and column 5.

Table S5

Results of quantum-chemical calculations with G3MP2 method for aliphatic ketones (at T = 298 K, p° = 0.1 MPa, all data are in kJ·mol-1)

| CAS | Compound | ∆fHmo(g)ATa | ∆fHmo(g)expb | ∆fHmo(g)corrc | d |
| --- | --- | --- | --- | --- | --- |
| 108-94-1 | cyclopentanone | -192.1 | -193 | -191.3 | -1.7 |
| 502-42-1 | cyclohexanone | -228.7 | -226.1 | -227.9 | 1.8 |
| 78-93-3 | methyl ethyl ketone | -237.8 | -238.7 | -237.0 | -1.7 |
| 96-22-0 | diethyl ketone | -260.0 | -257.9 | -259.3 | 1.4 |
| 107-87-9 | methyl propyl ketone | -258.3 | -259.0 | -257.6 | -1.4 |
| 563-80-4 | isopropyl methyl ketone | -262.2 | -262.5 | -261.5 | -1.0 |
| 591-78-6 | butyl methyl ketone | -279.7 | -279.8 | -279.0 | -0.8 |
| 565-80-0 | diisopropyl ketone | -311.2 | -311.3 | -310.5 | -0.8 |
| 502-56-7 | dibutyl_ketone | -343.8 | -344.9 | -343.1 | -1.8 |
| 111-13-7 | 2-octanone | -322.4 | -321.6 | -321.7 | 0.1 |
| 821-55-6 | methyl heptyl ketone | -343.8 | -340.7 | -343.1 | 2.4 |
| 927-49-1 | dipentyl ketone | -387.7 | -387.4 | -387.0 | -0.4 |
|  |  |  |  | 0.9 | -0.9 |
| 5689 04 3 | cis-6H-indan-2-one [Table S16] | -252.1 | -249.5 | -251.4 | 1.9 |
| 16484-17-6 | trans-6H-indan-2-one [Table S16] | -252.1 | -249.0 | -251.4 | 2.4 |
| 13351-29-6 | cis-6H-3a-methyl-indan-2-one [Table S16] | -286.4 | -287.0 | -285.7 | -1.3 |
| 20379-99-1 | trans-6H-3a-methyl-indan-2-one [Table S16] | -277.7 | -275.2 | -277.0 | 1.8 |
|  |  |  |  | 0.9 |  |
| 29927-85-3 | 6H-1-indanone | -249.2 |  | -248.4 |  |
| 6088-72-8 | 6H-2-indanone | -252.1 |  | -251.4 |  |
| 2308731-82-8 | 6H-2-Me-1-indanone | -277.5 |  | -276.8 |  |
| 64558-16-3 | 6H-3-Me-1-indanone | -281.3 |  | -280.6 |  |
| 92015-40-2 | 6H-5-Me-1-indanone | -280.8 |  | -280.1 |  |
| 1824616-45-6 | 6H-6-Me-1-indanone | -281.0 |  | -280.3 |  |
| 1823221-69-7 | 6H-4-methoxy-1-indanone | -397.6 |  | -396.9 |  |
|  | 6H-5-methoxy-1-indanone | -395.2 |  | -394.5 |  |
|  | 6H-6-methoxy-1-indanone | -396.1 |  | -395.4 |  |
| 1195675-41-2 | 6H-5,6-methoxy-1-indanone | -538.7 |  | -538.1 |  |
| 16385-56-1 | 6H-1,3-indandione | -347.8 |  | -347.1 |  |
| 101581-29-7 | 6H-4-benzofuranone | -345.9 |  | -345.2 |  |
| 16806-93-2 | 6,7-dihydro-4(5H)-benzofuranone | -222.8 |  | -222.0 |  |
|  | 6,7-dihydro-5(4H)-benzofuranone | -202.9 |  | -202.1 |  |
| 169378-54-5 | 4,5-dihydro-6(7H)-benzofuranone | -202.7 |  | -201.9 |  |
| 108153-93-1 | 5,6-dihydro-7(4H)-benzofuranone | -212.0 |  | -211.2 |  |

a Calculated by G3MP2 and atomization reaction with the expanded uncertainties of ±4.1 kJ·mol-1 [4].

b The experimental data as compiled from the Ref. [11] (see Table 1 and the references inside this table) and evaluated using the on-line data base [10].

c Calculated according to equation:

∆fHmo(g)corr/ kJ.mol-1 = 1.0007 × ∆fHmo(g, AT)G3MP2 + 0.99 	with R2 = 0.999,

derived from the correlations of the results given in columns 3 and 4 of this table.

d Difference between column 4 and column 5.

Table S6

Results of quantum-chemical calculations with G3MP2 method for alcohols (at T = 298 K, p° = 0.1 MPa, all data are in kJ·mol-1)

| CAS | Compound | ∆fHmo(g)ATa | ∆fHmo(g)expb | ∆fHmo(g)corrc | d |
| --- | --- | --- | --- | --- | --- |
| 6351-10-6 | 1-indanol [12] | -111.3 | -107.7 | -107.3 | -0.4 |
| 4254-29-9 | 2-indanol [12] | -114.9 | -109.5 | -110.8 | 1.3 |
| 1470-94-6 | 5-indanol [12] | -120.1 | -117.6 | -116.2 | -1.4 |
| 939-19-5 | 3-hydroxycoumarin [13] | -367.6 | -367.7 | -367.0 | -0.7 |
| 99-93-4 | 4-hydroxy-acetophenone [14] | -267.9 | -264.6 | -266.0 | 1.4 |
| 108-93-0 | cyclohexanol [Table S15] | -290.8 | -290.2 | -289.2 | -1.0 |

a Calculated by G3MP2 and atomization reaction with the expanded uncertainties of ±4.1 kJ·mol-1 [4].

b The experimental data as compiled from the original literature (the references inside this table) and evaluated using the on-line data base [10].

c Calculated according to equation:

∆fHmo(g)corr/ kJ.mol-1 = 1.0133 × ∆fHmo(g, AT)G3MP2 + 5.6 	with R2 = 0.9999,

derived from the correlations of the results given in columns 3 and 4 of this table.

d Difference between column 4 and column 5.

Table S7

Compilation of quantum-chemical calculations with G3MP2/B3LYP and G3MP2 methods for ketones (at T = 298 K, p° = 0.1 MPa, all data are in kJ·mol-1)

|  | G3MP2/B3LYP a | G3MP2/B3LYP b | G3MP2c |  |
| --- | --- | --- | --- | --- |
| Compound | ∆fHmo(g)corr | ∆fHmo(g)WBR | ∆fHmo(g)corr | ∆fHmo(g)QCd |
| 1-indanone | -63.5 | - | -62.9 | -63.3±2.9 |
| 2-methyl-1-indanone | -92.7 | -94.4±4.2 [15] | -91.4 | -92.8±2.4 |
| 3-methyl-1-indanone | -92.6 | -94.3±4.2 [15] | -91.4 | -92.8±2.4 |
| 6-methyl-1-indanone | -92.2 | -96.3±2.4 [16] | -94.8 | -95.2±1.9 |
| 4-methoxy-1-indanone | -218.9 | 219.1±3.3 [15] | -213.9 | -217.6±2.2 |
| 5-methoxy-1-indanone | -220.0 | -220.3±3.3 [15] | -214.6 | -218.7±2.2 |
| 6-Methoxy-1-indanone | -212.6 | -215.3±2.4 [16] | -212.9 | -214.5±1.9 |
| 5,6-dimethoxy-1-indanone | -355.8 | -356.1±2.4 [16] | -353.6 | -355.6±1.9 |
| 6,7-dihydro-4(5H)-benzofuranone | - | -217.9±2.5 [17] | -221.8 | -219.0±2.1 |
| 6,7-dihydro-5(4H)-benzofuranone | - | -197.9±2.5 [17] | -202.1 | -199.0±2.1 |
| 4,7-dihydro-6(7H)-benzofuranone | - | -197.9±2.5 [17] | -201.9 | -199.0±2.1 |
| 5,6-dihydro-7(4H)-benzofuranone | - | -207.3±2.5 [17] | -211.2 | -208.4±2.1 |

a From Table S8 with the expanded uncertainties of ±4.1 kJ·mol-1 [4].

b Results from [15, 16] were calculated with sets of well-balanced reactions (WBR)

c From Table S3 with the expanded uncertainties of ±4.1 kJ·mol-1 [4].

d Weighted average value calculated from the results given in columns 2 to 4.

Table S8

Results of quantum-chemical calculations with G3MP2/B3LYP methods (at T = 298 K, p° = 0.1 MPa, all data are in kJ·mol-1)

| CAS | Compound | ∆fHmo(g)AT a | ∆fHmo(g)exp b | ∆fHmo(g)corr c | d |
| --- | --- | --- | --- | --- | --- |
| 71-43-2 | benzene | 77.0 [15] | 82.9 [18] | 82.2 | 0.7 |
| 108-88-3 | toluene | 44.6 [15] | 50.1 [18] | 50.3 | -0.2 |
| 100-66-3 | methoxy-benzene | -77.3 [15] | -70.3 [19] | -69.9 | -0.4 |
| 83-33-0 | 1-indanone | -70.8 [15] | -63.9 | -63.5 | -0.4 |
| 17496-14-9 | 2-methyl-1-indanone | -100.4 [15] |  | -92.7 |  |
| 6072-57-7 | 3-methyl-1-indanone | -100.3 [15] | -91.2 | -92.6 | 1.4 |
| 24623-20-9 | 6-methyl-1-indanone | -99.9 [16] | -95.1 | -92.2 | -2.9 |
| 13336-31-7 | 4-methoxy-1-indanone | -228.4 [15] | -217.6 | -218.9 | 1.3 |
| 5111-70-6 | 5-methoxy-1-indanone | -229.6 [15] | -215.7 | -220.0 | 4.3 |
| 13623-25-1 | 6-methoxy-1-indanone | -222.1 [16] | -216.0 | -212.6 | -3.4 |
| 2107-69-9 | 5,6-dimethoxy-1-indanone | -367.3 [16] | -356.7 | -355.8 | -0.9 |

a Calculated in the literature by G3MP2/B3LYP and atomization reaction (expanded uncertainties of ±4.1 kJ·mol-1 [4]).

b The experimental data as compiled from the original literature (Table 1 and the references inside this table) and evaluated using the on-line data base [10].

c Calculated according to equation: ∆fHmo(g)corr/ kJ.mol-1 = 0.9858 × ∆fHmo(g, AT)G3MP2/B3LYP + 6.3 	with R2 = 0.9997,

derived from the correlations of the results given in columns 3 and 4 of this table.

d Difference between column 4 and column 5.

Temperature adjustments of vaporization enthalpies

The vapor pressures, pi, at different temperatures for HL and HR compounds available in the literature were approximated uniformly using the three-parameter equation [20]:

R ln(pi/pref)=a+ bT+∆lgCp,mo lnTT0				(S1),

where a and b are two adjustable parameters, and the third parameter ∆lgCp,mo is fixed. This parameter ∆l,crgCp,mo = Cp,mo(g) – Cp,mo(liq or cr) is the difference between the molar heat capacities of the gaseous Cp,mo(g) and the liquid/crystal phase Cp,mo(liq or cr), respectively.

The ∆l,crgCp,mo-values used in Eq. (S1) are usually estimated according to empirical equations developed by Acree and Chickos [21]:

∆lgCp,mo298 K=-0.26×Cp,moliq, 298 K-10.58 	(S2)

∆crgCp,mo298 K=-0.15×Cp,mocr, 298 K-0.75 	(S3)

where Cp,mo(cr, 298 K) or Cp,mo(liq, 298 K) values are of experimental origin or they can be also estimated according to the group-additivity procedure by Acree and Chickos [21]. The empirical coefficients in Eqs. (S2) and (S3) were originally derived from experimental data available for various organic compounds. It has turned out that the ∆lgCp,mo–values calculated using the conventional method of Acree and Chickos [21] are systematically and significantly overestimated when the procedure is applied e.g. to aromatic and aliphatic ethers [6, 22]. Therefore, the empirical coefficients in Eq. (S2) were re-evaluated using the reliable vapor pressures from the literature and the specific for each family equations were derived:

for aliphatic cyclic hydrocarbons [23]:

∆lgCp,mo298 K=-0.8×∆lgCp,mo(Eq. 2) 	(S4)

for aromatic ketones [24]:

∆lgCp,mo298 K=-0.26×Cp,moliq, 298 K+9.6 	(S5)

for aliphatic cyclic ketones [25]:

∆lgCp,mo298 K=-0.26×Cp,moliq, 298 K+5.6 	(S6)

for aromatic alcohols [19]:

∆lgCp,mo298 K=-0.26×Cp,moliq, 298 K-19.0 	(S7)

for aliphatic cyclic alcohols [25]:

∆lgCp,mo298 K=-0.26×Cp,moliq, 298 K-51.0 	(S8)

for dimethoxy alkanes [26]:

∆lgCp,mo298 K=-0.26×Cp,moliq, 298 K-51.0 	(S9)

Table S9

Compilation of data on molar heat capacities Cp,mo(liq) and heat capacity differences ∆lgCp,mo (in J.K-1.mol-1) at T = 298 K

| Compounds | Cp,mo(liq) a | -∆lgCp,mo b |
| --- | --- | --- |
| 1-indanone | 216.8 | 46.8 [Eq. S5] |
| 2-indanone | 216.8 | 46.8 [Eq. S5] |
| 2-Me-1-indanone | 243.6 | 53.8 [Eq. S5] |
| 3-Me-1-indanone | 243.6 | 53.8 [Eq. S5] |
| 5-Me-1-indanone | 245.2 | 54.2 [Eq. S5] |
| 6-Me-1-indanone | 245.2 | 54.2 [Eq. S5] |
| 4-MeO-1-indanone | 268.4 | 60.2 [Eq. S5] |
| 5-MeO-1-indanone | 268.4 | 60.2 [Eq. S5] |
| 6-MeO-1-indanone | 268.4 | 60.2 [Eq. S5] |
| 5,6-diMeO-1-indanone | 320.0 | 73.6 [Eq. S5] |
| 1,3-indandione | 239.3 | 52.6 [Eq. S5] |
| 6,7-dihydro-4-benzofuranone | 199.3 | 46.2 [Eq. 2] |
| 1-indanol | 244.9 | 74.6 [Eq. S7] |
| 2-indanol | 244.9 | 74.6 [Eq. S7] |
| 6H-1-indanone | 235.8 | 55.7 [Eq. S6] |
| 6H-2-indanone | 235.8 | 55.7 [Eq. S6] |
| 6H-2-Me-1-indanone | 262.6 | 62.7 [Eq. S6] |
| 6H-3-Me-1-indanone | 262.6 | 62.7 [Eq. S6] |
| 6H-5-Me-1-indanone | 262.6 |  |
| 6H-6-Me-1-indanone | 262.6 |  |
| 6H-4-methoxy-1-indanone | 285.8 |  |
| 6H-5-methoxy-1-indanone | 285.8 |  |
| 6H-6-methoxy-1-indanone | 285.8 |  |
| 6H-5,6-methoxy-1-indanone | 335.8 |  |
| 6H-1,3-indandione | 258.3 |  |
| 6H-4-benzofuranone | 233.5 |  |
| 6H-1-indanol | 263.9 | 119.6 [Eq. S8] |
| 6H-2-indanol | 263.9 | 119.6 [Eq. S8] |
| 6H-2-Me-1-indanol | 290.7 |  |
| 6H-3-Me-1-indanol | 290.7 |  |
| 6H-5-Me-1-indanol | 290.7 |  |
| 6H-6-Me-1-indanol | 290.7 |  |
| 6H-4-methoxy-1-indanol | 313.9 |  |
| 6H-5-methoxy-1-indanol | 313.9 |  |
| 6H-6-methoxy-1-indanol | 313.9 |  |
| 6H-5,6-methoxy-1-indanol | 363.9 |  |
| 6H-1,3-indanediol | 314.5 |  |
| 8H-4-benzofuranol [99767-96-1] | 261.6 |  |
| 1,2-dimethoxy-cyclohexane [19752-95-5] | 253.0 | 58.4 [Eq. S9] |
| 2,6-dimethylphenol | 256.0 | 85.6 [Eq. S7] |
| 2,4,6-trimethylphenol | 297.4 | 96.3 [Eq. S7] |
| naphthalene | 205.0 | 63.0 [Eq. S2] |
| decalin | 238.8 | 55.7 [27] |

a Calculated according to the group-additivity procedure developed by Acree and Chickos [21].

b Calculated according to the empirical procedures developed by Acree and Chickos [21] and modified in this work (see text above).

Table S10

Compilation of data on molar heat capacities Cp,mo(cr) and heat capacity differences ∆crgCp,mo (in J.K-1.mol-1) at T = 298 K

| Compounds | Cp,mo(cr) a | -∆crgCp,mo b |
| --- | --- | --- |
| 1-indanone | 171.8 [7] | 26.4 |
| 2-indanone | 170.7 | 26.4 |
| 6-Me-1-indanone | 198.3 | 30.5 |
| 4-MeO-1-indanone | 216.8 | 33.3 |
| 5-MeO-1-indanone | 216.8 | 33.3 |
| 5,6-diMeO-1-indanone | 262.9 | 40.2 |
| 1,3-indandione | 180.6 | 27.8 |
| 1-indanol | 170.6 | 26.3 |
| 2-indanol | 170.6 | 26.3 |
| 2,6-dimethylphenol | 174.4 | 26.9 |
| 2,4,6-trimethylphenol | 220.0 | 33.7 |

a Calculated according to the group-additivity procedure developed by Acree and Chickos [21] or taken from the literature.

b Calculated according to Eq. (S3).

Calculations of vaporisation enthalpies from the boiling points at reduced pressures (BP-method)

The experimental boiling points (BP) at various reduced pressures available in the literature [28, 29] may be helpful to support these new results. As a rule, these boiling points come from the distillation of reaction mixtures after synthesis and are used to identify the compound. Usually, the temperatures are given in the range of a few degrees and the pressures are measured with non-calibrated manometers. However, in our previous work, e.g. on methyl-biphenyls [30] or alkoxy-benzenes [31] we have shown that reasonable trends can generally be obtained from even such coarse data. For example, in Fig. S1 reliable vapour pressures available for ethoxy-benzene were compared with the boiling points (BP) at reduced pressures found in the literature [28, 29] and evaluated in our previous study [31]. The apparent agreement of vapour pressures shown in Fig. S1 has supported the usefulness of BP as supplemental information.

The accuracy of this data is questionable as it comes from the distillation of a compound after its synthesis and not from special physico-chemical studies. However, the numerous data on boiling temperatures at standard pressure as well as at reduced pressures provide at least a reliable level of the experimental vapour pressures and trend of the dependence of the vapour pressure with temperature [30, 31]. This type of data is especially important and helps to reconcile the results in a case where the conventional methods show significant disagreement.

Fig. S1. The temperature dependence of vapour pressures over the liquid sample of ethoxy-benzene:

○ – [32]; ● – [33];  - boiling points at reduced pressures compiled in Ref. [31].

Table S11

The vapour pressures p, and standard molar vaporisation enthalpies and entropies obtained by the approximation of boiling points at different pressures available in the literature [28, 29, 34]

| T/ Ka | p/ Pa | ∆lgHmo/a kJmol-1 | ∆lgSmo/ JK-1mol-1 |
| --- | --- | --- | --- |

1-indanone [874-35-1]

∆lgHmo(298 K) = (65.51.4) kJ.mol-1

∆lgSmo(298 K) = (133.01.5) JK-1mol-1

∆lgGmo(298 K) = (25.90.4) kJ.mol-1

ln (p/pref)=275.5R-79489.0RT-46.8RlnT298 ; pref = 1 Pa

| 332 | 40 | 64.0 | 127.6 |
| --- | --- | --- | --- |
| 338 | 67 | 63.7 | 127.6 |
| 338 | 67 | 63.7 | 127.6 |
| 348 | 133 | 63.2 | 126.6 |
| 357 | 200 | 62.8 | 124.2 |
| 357 | 200 | 62.8 | 124.2 |
| 358 | 200 | 62.7 | 123.6 |
| 358 | 200 | 62.7 | 123.6 |
| 360 | 267 | 62.6 | 124.7 |
| 361 | 267 | 62.6 | 124.1 |
| 370 | 400 | 62.2 | 122.1 |
| 390 | 1333 | 61.2 | 121.1 |
| 391 | 1333 | 61.2 | 120.6 |
| 392 | 1333 | 61.1 | 120.1 |
| 392 | 1333 | 61.1 | 120.1 |
| 392 | 1333 | 61.1 | 120.1 |
| 393 | 1333 | 61.1 | 119.6 |
| 393 | 1600 | 61.1 | 121.1 |
| 398 | 2000 | 60.9 | 120.4 |
| 398 | 2000 | 60.9 | 120.4 |
| 399 | 1600 | 60.8 | 118.0 |
| 399 | 2266 | 60.8 | 120.9 |
| 399 | 1600 | 60.8 | 118.0 |
| 515 | 101325 | 55.4 | 107.7 |
| 516 | 101325 | 55.3 | 107.4 |
| 516 | 101325 | 55.3 | 107.4 |
| 517 | 101325 | 55.3 | 107.1 |
| 518 | 101325 | 55.2 | 106.8 |

2-indanone [615-13-4]

∆lgHmo(298 K) = (63.12.6) kJ.mol-1

∆lgSmo(298 K) = (128.27.8) JK-1mol-1

∆lgGmo(298 K) = (24.80.8) kJ.mol-1

ln (p/pref)=270.7R-77001.1RT-46.8RlnT298 ; pref = 1 Pa

| 339 | 67 | 61.1 | 119.5 |
| --- | --- | --- | --- |
| 357 | 267 | 60.3 | 119.6 |
| 362 | 400 | 60.1 | 120.0 |
| 383 | 1333 | 59.1 | 118.3 |
| 385 | 1333 | 59.0 | 117.3 |
| 522 | 101325 | 52.6 | 100.8 |

2-Me-indanone [17496-14-9]

∆lgHmo(298 K) = (64.81.5) kJ.mol-1

∆lgSmo(298 K) = (134.22.1) JK-1mol-1

∆lgGmo(298 K) = (24.80.4) kJ.mol-1

ln (p/pref)=283.7R-80885.4RT-53.8RlnT298 ; pref = 1 Pa

| 361 | 400 | 61.5 | 124.4 |
| --- | --- | --- | --- |
| 363 | 400 | 61.4 | 123.1 |
| 363 | 400 | 61.4 | 123.1 |
| 366 | 533 | 61.2 | 123.7 |
| 368 | 533 | 61.1 | 122.5 |
| 373 | 800 | 60.8 | 122.9 |
| 376 | 933 | 60.7 | 122.5 |
| 377 | 933 | 60.6 | 121.9 |
| 381 | 1200 | 60.4 | 121.7 |
| 384 | 1200 | 60.2 | 120.1 |
| 385 | 1467 | 60.2 | 121.2 |
| 385 | 1200 | 60.2 | 119.5 |
| 386 | 1467 | 60.1 | 120.6 |
| 388 | 1333 | 60.0 | 118.8 |
| 391 | 1600 | 59.8 | 118.7 |
| 392 | 2133 | 59.8 | 120.5 |
| 393 | 2000 | 59.7 | 119.5 |
| 393 | 2133 | 59.7 | 120.0 |
| 398 | 2400 | 59.5 | 118.4 |
| 398 | 2400 | 59.5 | 118.4 |
| 399 | 2400 | 59.4 | 117.9 |
| 401 | 2533 | 59.3 | 117.3 |
| 402 | 2533 | 59.3 | 116.8 |
| 443 | 15599 | 57.1 | 113.3 |
| 503 | 97325 | 53.8 | 106.8 |
| 507 | 97325 | 53.6 | 105.5 |
| 508 | 97325 | 53.6 | 105.2 |

3-Me-indanone [6072-57-7]

∆lgHmo(298 K) = (67.21.8) kJ.mol-1

∆lgSmo(298 K) = (139.44.4) JK-1mol-1

∆lgGmo(298 K) = (25.70.4) kJ.mol-1

ln (p/pref)=288.9R-83255.6RT-53.8RlnT298 ; pref = 1 Pa

| 386.2 | 1199.9 | -68.92 | 62.5 | 125.0 |
| --- | --- | --- | --- | --- |
| 386.2 | 1199.9 | -68.92 | 62.5 | 125.0 |
| 389.2 | 1333.2 | -140.69 | 62.3 | 124.2 |
| 391.2 | 1599.9 | -26.45 | 62.2 | 124.7 |
| 392.2 | 1599.9 | -107.71 | 62.2 | 124.1 |
| 392.2 | 1599.9 | -107.71 | 62.2 | 124.1 |
| 393.2 | 1599.9 | -192.52 | 62.1 | 123.6 |
| 396.2 | 1866.5 | -202.84 | 61.9 | 123.3 |
| 398.2 | 2666.4 | 392.26 | 61.8 | 125.2 |
| 399.2 | 2133.2 | -249.93 | 61.8 | 122.8 |
| 401.2 | 2666.4 | 51.79 | 61.7 | 123.6 |
| 499.9 | 101324.7 | 14259.21 | 56.4 | 112.9 |

6-Me-indanone [24623-20-9]

∆lgHmo(298 K) = (63.92.7) kJ.mol-1

∆lgSmo(298 K) = (126.07.5) JK-1mol-1

∆lgGmo(298 K) = (26.30.7) kJ.mol-1

ln (p/pref)=275.9R-80054.2RT-54.2RlnT298 ; pref = 1 Pa

| 343 | 53 | 61.5 | 116.5 |
| --- | --- | --- | --- |
| 385 | 667 | 59.2 | 112.1 |
| 397 | 1467 | 58.5 | 112.3 |
| 399 | 1467 | 58.4 | 111.3 |
| 549 | 101325 | 50.3 | 91.9 |

4-MeO-indanone [13336-31-7]

∆lgHmo(298 K) = (74.62.0) kJ.mol-1

∆lgSmo(298 K) = (140.23.0) JK-1mol-1

∆lgGmo(298 K) = (32.80.4) kJ.mol-1

ln (p/pref)=296.1R-92521.0RT-60.2RlnT298 ; pref = 1 Pa

| 393 | 200 | 68.9 | 123.6 |
| --- | --- | --- | --- |
| 576 | 101325 | 57.8 | 100.5 |

5-MeO-indanone [5111-70-6]

∆lgHmo(298 K) = (75.23.4) kJ.mol-1

∆lgSmo(298 K) = (138.58.1) JK-1mol-1

∆lgGmo(298 K) = (33.90.4) kJ.mol-1

ln (p/pref)=294.4R-93118.9RT-60.2RlnT298 ; pref = 1 Pa

| 418 | 533 | 68.0 | 119.1 |
| --- | --- | --- | --- |
| 423 | 533 | 67.7 | 116.4 |
| 592 | 101325 | 57.5 | 97.2 |

5,6-diMeO-indanone [2107-69-9]

∆lgHmo(298 K) = (93.63.2) kJ.mol-1

∆lgSmo(298 K) = (162.51.7) JK-1mol-1

∆lgGmo(298 K) = (45.20.4) kJ.mol-1

ln (p/pref)=331.8R-115555.2RT-73.6RlnT298 ; pref = 1 Pa

| 411 | 27 | 85.3 | 139.1 |
| --- | --- | --- | --- |
| 412 | 27 | 85.2 | 138.4 |
| 645 | 101325 | 68.1 | 105.7 |

1-indanol [6351-10-6]

∆lgHmo(298 K) = (69.12.4) kJ.mol-1

∆lgSmo(298 K) = (141.42.4) JK-1mol-1

∆lgGmo(298 K) = (26.90.2) kJ.mol-1

ln (p/pref)=311.8R-91298.7RT-74.6RlnT298 ; pref = 1 Pa

| 392 | 1200 | 62.1 | 121.5 |
| --- | --- | --- | --- |
| 393 | 1200 | 62.0 | 120.9 |
| 396 | 1333 | 61.8 | 120.3 |
| 401 | 1600 | 61.4 | 118.7 |
| 483 | 33331 | 55.3 | 105.3 |
| 523 | 101325 | 52.3 | 100.1 |
| 528 | 101325 | 51.9 | 98.4 |

2-indanol [4254-29-9]

∆lgHmo(298 K) = (68.62.7) kJ.mol-1

∆lgSmo(298 K) = (141.35.3) JK-1mol-1

∆lgGmo(298 K) = (26.40.5) kJ.mol-1

ln (p/pref)=311.7R-90799.8RT-74.6RlnT298 ; pref = 1 Pa

| 372 | 400 | 63.1 | 123.9 |
| --- | --- | --- | --- |
| 375 | 533 | 62.8 | 124.0 |
| 393 | 1600 | 61.5 | 122.1 |
| 396 | 1600 | 61.3 | 120.3 |
| 523 | 101325 | 51.8 | 99.1 |

6H-1-indanone [29927-85-3]

∆lgHmo(298 K) = (58.61.7) kJ.mol-1

∆lgSmo(298 K) = (128.45.1) JK-1mol-1

∆lgGmo(298 K) = (20.30.4) kJ.mol-1

ln (p/pref)=279.8R-75157.9RT-55.7RlnT298 ; pref = 1 Pa

| 305 | 40 | 58.2 | 125.7 |
| --- | --- | --- | --- |
| 305 | 40 | 58.2 | 125.7 |
| 306 | 40 | 58.1 | 124.9 |
| 330 | 267 | 56.8 | 122.8 |
| 331 | 267 | 56.7 | 122.1 |
| 345 | 800 | 55.9 | 122.0 |
| 345 | 800 | 55.9 | 122.0 |
| 346 | 800 | 55.9 | 121.4 |
| 346 | 800 | 55.9 | 121.4 |
| 363 | 1600 | 54.9 | 117.0 |
| 363 | 2133 | 54.9 | 119.4 |
| 364 | 1600 | 54.9 | 116.4 |
| 368 | 2666 | 54.7 | 118.4 |
| 489 | 101058 | 47.9 | 98.1 |

cis-6H-2-indanone [5689-05-3]

∆lgHmo(298 K) = (56.81.6) kJ.mol-1

∆lgSmo(298 K) = (120.62.9) JK-1mol-1

∆lgGmo(298 K) = (20.80.3) kJ.mol-1

ln (p/pref)=272.0R-73400.1RT-55.7RlnT298 ; pref = 1 Pa

| 357 | 800 | 53.5 | 109.8 |
| --- | --- | --- | --- |
| 369 | 1600 | 52.8 | 108.8 |
| 369 | 1600 | 52.8 | 108.8 |
| 370 | 1600 | 52.8 | 108.3 |
| 370 | 1600 | 52.8 | 108.3 |
| 378 | 2666 | 52.3 | 108.3 |
| 381 | 3066 | 52.2 | 108.0 |
| 382 | 3066 | 52.1 | 107.5 |
| 383 | 2666 | 52.1 | 105.8 |
| 383 | 3066 | 52.1 | 107.0 |
| 383 | 2666 | 52.1 | 105.8 |
| 400 | 5999 | 51.1 | 104.4 |
| 400 | 5999 | 51.1 | 104.4 |
| 401 | 5999 | 51.1 | 103.9 |
| 401 | 5999 | 51.1 | 103.9 |
| 498 | 101325 | 45.7 | 91.8 |

trans-6H-2-indanone [16484-17-6]

∆lgHmo(298 K) = (56.21.5) kJ.mol-1

∆lgSmo(298 K) = (120.84.8) JK-1mol-1

∆lgGmo(298 K) = (20.10.2) kJ.mol-1

ln (p/pref)=272.2R-72765.4RT-55.7RlnT298 ; pref = 1 Pa

| 333 | 267 | 54.2 | 113.5 |
| --- | --- | --- | --- |
| 356 | 933 | 52.9 | 109.8 |
| 361 | 1333 | 52.7 | 110.0 |
| 361 | 1333 | 52.7 | 110.0 |
| 362 | 1467 | 52.6 | 110.2 |
| 362 | 1333 | 52.6 | 109.4 |
| 362 | 1333 | 52.6 | 109.4 |
| 363 | 1467 | 52.5 | 109.6 |
| 363 | 1467 | 52.5 | 109.6 |
| 364 | 1733 | 52.5 | 110.5 |
| 365 | 1733 | 52.4 | 109.9 |
| 365 | 1733 | 52.4 | 109.9 |
| 365 | 1733 | 52.4 | 109.9 |
| 371 | 2533 | 52.1 | 109.9 |
| 371 | 2533 | 52.1 | 109.9 |
| 376 | 2800 | 51.8 | 108.1 |
| 382 | 3333 | 51.5 | 106.5 |
| 386 | 4666 | 51.3 | 107.3 |
| 491 | 101325 | 45.4 | 92.6 |
| 491 | 101325 | 45.4 | 92.6 |

6H-2-Me-1-indanone [2308731-82-8]

∆lgHmo(298 K) = (59.11.4) kJ.mol-1

∆lgSmo(298 K) = (125.21.5) JK-1mol-1

∆lgGmo(298 K) = (21.80.2) kJ.mol-1

ln (p/pref)=283.6R-77811.6RT-62.7RlnT298 ; pref = 1 Pa

| 328 | 133 | 57.2 | 119.5 |
| --- | --- | --- | --- |
| 329 | 133 | 57.2 | 118.8 |
| 501 | 101325 | 46.4 | 92.6 |

6H-3-Me-1-indanone [64558-16-3]

∆lgHmo(298 K) = (60.31.8) kJ.mol-1

∆lgSmo(298 K) = (126.43.3) JK-1mol-1

∆lgGmo(298 K) = (22.60.3) kJ.mol-1

ln (p/pref)=284.8R-78958.0RT-62.7RlnT298 ; pref = 1 Pa

| 332 | 133 | 58.1 | 120.1 |
| --- | --- | --- | --- |
| 360 | 667 | 56.4 | 115.0 |
| 363 | 667 | 56.2 | 113.2 |
| 506 | 101325 | 47.2 | 93.4 |

trans-6H-6-Me-1-indanone [24623-20-9]

∆lgHmo(298 K) = (61.61.9) kJ.mol-1

∆lgSmo(298 K) = (120.93.2) JK-1mol-1

∆lgGmo(298 K) = (25.50.4) kJ.mol-1

ln (p/pref)=270.8R-77707.6RT-54.2RlnT298 ; pref = 1 Pa

| 343 | 80 | 59.1 | 113.1 |
| --- | --- | --- | --- |
| 385 | 667 | 56.8 | 106.0 |
| 397 | 1467 | 56.2 | 106.4 |
| 399 | 1467 | 56.1 | 105.5 |
| 549 | 101325 | 48.0 | 87.6 |

6H-1-indanol [53927-31-4]

∆lgHmo(298 K) = (72.22.5) kJ.mol-1

∆lgSmo(298 K) = (159.23.5) JK-1mol-1

∆lgGmo(298 K) = (24.70.5) kJ.mol-1

ln (p/pref)=374.5R-107837.6RT-119.6RlnT298 ; pref = 1 Pa

| 325 | 53 | 69.0 | 149.5 |
| --- | --- | --- | --- |
| 328 | 53 | 68.6 | 146.5 |
| 380 | 1600 | 62.4 | 129.8 |
| 382 | 2000 | 62.2 | 130.5 |
| 383 | 2000 | 62.1 | 129.8 |
| 496 | 101325 | 48.5 | 97.9 |

6H-2-indanol [59078-17-1]

∆lgHmo(298 K) = (73.23.4) kJ.mol-1

∆lgSmo(298 K) = (160.23.3) JK-1mol-1

∆lgGmo(298 K) = (25.50.3) kJ.mol-1

ln (p/pref)=375.5R-108883.8RT-119.6RlnT298 ; pref = 1 Pa

| 379 | 1333 | 63.6 | 132.2 |
| --- | --- | --- | --- |
| 380 | 1467 | 63.4 | 131.8 |
| 381 | 1467 | 63.3 | 131.1 |
| 382 | 1467 | 63.3 | 130.7 |
| 382 | 1600 | 63.3 | 131.4 |
| 383 | 1467 | 63.1 | 130.0 |
| 383 | 1600 | 63.1 | 130.3 |
| 384 | 1467 | 63.0 | 129.2 |
| 403 | 4000 | 60.7 | 123.8 |
| 404 | 4000 | 60.6 | 123.2 |
| 498 | 101325 | 49.3 | 99.2 |

6H-3-Me-indanol [857773-69-4]

∆lgHmo(298 K) = (76.94.0) kJ.mol-1

∆lgSmo(298 K) = (161.21.7) JK-1mol-1

∆lgGmo(298 K) = (28.90.2) kJ.mol-1

ln (p/pref)=383.5R-114669.8RT-126.0RlnT298 ; pref = 1 Pa

| 382 | 533 | 66.3 | 130.1 |
| --- | --- | --- | --- |
| 383 | 533 | 66.2 | 129.3 |
| 539 | 101325 | 46.4 | 86.3 |

1,2-di-MeO-cyclohexane [19752-95-5]

∆lgHmo(298 K) = (64.11.7) kJ.mol-1

∆lgSmo(298 K) = (159.66.3) JK-1mol-1

∆lgGmo(298 K) = (16.60.6) kJ.mol-1

ln (p/pref)=313.7R-81541.1RT-58.4RlnT298 ; pref = 1 Pa

| 302 | 200 | 63.9 | 159.9 |
| --- | --- | --- | --- |
| 326 | 1333 | 62.5 | 155.8 |
| 336 | 3066 | 61.9 | 155.3 |
| 338 | 2000 | 61.8 | 150.3 |
| 339 | 2000 | 61.7 | 149.6 |
| 340 | 3066 | 61.7 | 152.5 |
| 340 | 2800 | 61.7 | 151.7 |
| 341 | 2800 | 61.6 | 151.0 |
| 348 | 4266 | 61.2 | 149.7 |
| 348 | 4533 | 61.2 | 150.2 |
| 348 | 4266 | 61.2 | 149.7 |
| 348 | 4533 | 61.2 | 150.2 |
| 405 | 101325 | 57.9 | 143.0 |
| 408 | 101325 | 57.7 | 141.6 |

a Uncertainties in this table are expressed as the expanded uncertainty (0.95 level of confidence. k = 2). Uncertainties of the vaporisation enthalpies include uncertainties from the fitting equation (Eq. S1) and uncertainties from temperature adjustment to T = 298 K. Uncertainties in the temperature adjustment of vaporisation enthalpies to the reference temperature T = 298 K are estimated to account with 20 % to the total adjustment.

Table S12

Correlation of vaporisation enthalpies, ∆lgHmo(298 K)/ kJ.mol-1 of auxiliary compounds with their Kovats indices (Jx)

| Compound | Jx a | ∆lgHmo(298 K)exp b | ∆lgHmo(298 K)calc c | d |
| --- | --- | --- | --- | --- |
| indane | 1030 | 49.0±0.5 [Table S15] | 49.6 | -0.6 |
| 2,6-dimethylphenol | 1105 | 57.9±1.1 [Table S13] | 56.9 | 1.0 |
| 2,4,6-trimethyl-phenol | 1205 | 66.1±1.3 [Table S13] | 66.6 | -0.5 |
| 1-indanole | 1230 |  | 69.0 |  |
| 2-indanole | 1225 |  | 68.5 |  |

a Kovats indices on the non-polar column DB-5 [35].

b Experimental data evaluated in Tables S13 and S15.

c Calculated using equation ∆lgHmo(298 K) = -50.2 + 0.0969×Jx with R² = 0.9888,

with the assessed expanded uncertainty of ±1.0 kJ.mol-1 (0.95 level of confidence, k = 2).

d Difference between column 3 and 4 in this table.

Table S13

Compilation of enthalpies of vaporization, ∆lgHmo(298 K), enthalpies of fusion, ∆crlHmo(298 K), and sublimation enthalpies, ∆crgHmo(298 K), and of aromatic compounds (in kJ.mol-1)a

|  | ∆lgHmo(298 K) | ∆crlHmo(298 K)b | ∆crgHmo(298 K) |
| --- | --- | --- | --- |
| 1-indanone | 63.1±0.7 [Table 2] | 17.4±0.5 | 80.5±0.9 d |
| 2-indanone | 62.9±1.3 [Table 2] | 15.6±0.5 | 78.5±1.4 |
| 6-Me-1-indanone | 66.1±1.9 c | 18.9±1.0 | 85.0±1.6 [16] |
| 4-MeO-1-indanone | 75.1±2.7 c | 23.2±1.2 | 98.3±2.4 [15] |
| 5-MeO-1-indanone | 77.9±1.4 [Table 2] | 23.4±1.2 | 101.3±0.7 [Table 2] |
| 6-MeO-1-indanone | 76.7±2.4 c | 23.2±1.2 | 99.9±2.1 [16] |
| 5,6-MeO-1-indanone | 94.8±2.8 c | 21.4±1.4 | 116.2±2.4 [16] |
| 1,3-indandione | 76.6±1.4 [Table 2] | 19.2±1.3 | 95.8±1.9 d |
| 1-indanole | 70.2±1.5 c | 17.1±1.1 | 87.3±1.0 [12] |
| 2-indanole | 71.5±1.6 c | 17.2±1.2 | 88.7±1.0 [12] |
| 2,6-dimethylphenol | 57.9±1.1 c | 17.7±1.1 | 75.6±0.2 [36] |
| 2,4,6-trimethylphenol | 66.1±1.3 c | 16.7±1.3 | 82.8±0.3 [37] |

a Uncertainties are presented as expanded uncertainties (0.95 level of confidence with k=2).

b The experimental enthalpies of fusion ∆crlHmo measured at Tfus were adjusted to 298 K in Table S14.

c The difference of columns 4 and 3 in this table.

d The sum of columns 2 and 3 in this table.

Estimation of fusion enthalpies according to the Walden´s Rule

In 1908 Paul Walden found [38] that the ratio according to Eq. (S10) can be considered as a constant (Walden´s Constant) [39, 40]:

WC = ∆crlHmo Tfus = ∆crlSmo = 56.5 J·K-1·mol-1	(S10)

This observation was supported by experimental results from 35 compounds (mostly substituted benzenes like nitrobenzene, aniline, dimethylaniline, diphenylmethane, diphenylamine, acetophenone, para-propenylanisole, but also aliphatic esters, anhydrides, etc.). The prerequisite for this constancy is that the compounds did not associate in the liquid state. Eq. (S10) is known as Walden's rule for thermochemistry [41]. From a practical point of view and based on our experience, Eq. (S10) can be easily adapted for calculations within a range of similarly shaped molecules. We have already observed similarity of Walden´s Constants for R-acetanilides with R = alkyl, F, Cl, Br, NO2, NH2, OH, OCH3 [42] and for the for R-substituted benzamides [43]. We have found that for these series the WC for each series deviates from the “classic” value 56.5 J·K-1·mol-1 by about ±10 J·K-1·mol-1. Such a “modified” Walden's Constant helps not only in evaluating the consistency of the experimental fusion data within a set of similarly structured compounds, but also the Walden's rule serves as a valuable tool for estimating missing fusion enthalpies of interesting compounds, provided that their fusion temperatures are available. Moreover, the “modified” Walden's Rule often helps to evaluate available phase transition data.

Table S14

Adjustment of experimental enthalpies of fusion, ∆crlHmo, of aromatics from melting points, Tfus, to T = 298 K a

| Compound | Tfus b | ∆crlHmo(Tfus) c | WC d | ∆crlHmo(298 K) e |
| --- | --- | --- | --- | --- |
|  | K | kJmol-1 | JK-1mol-1 | kJmol-1 |
| 1-indanone | 312.9 | 17.8±0.2 [7] |  |  |
|  | 314.1 | 17.6±0.2 [44] |  |  |
|  | 313.5 | 17.7±0.2 (average) | 56.5 | 17.4±0.5 |
| 2-indanone | 330.0 | 16.9±0.2 [44] |  | 15.6±0.5 |
| 6-Me-1-indanone | 335.1 | 19.7±0.3 [16] | 58.8 | 18.9±1.0 |
| 4-MeO-1-indanone | 379.0 [29] | (379.0×67.0) = (25.4±1.0)f |  | 23.2±1.2 |
| 5-MeO-1-indanone | 383.0 [29] | (383.0×67.0) = (25.7±1.0)f |  | 23.4±1.2 |
| 6-MeO-1-indanone | 380.6 | 25.5±0.6 [16] | 67.0 | 23.3±1.2 |
| 5,6-diMeO-1-indanone | 392.3 | 24.5±0.4 [16] | 62.5 | 21.4±1.4 |
| 1,3-indandione | 401.5 | 21.8±0.5 [44] |  | 19.2±1.3 |
| 1-indanole | 328.0 [29] | (328.0×56.5) = (18.5±1.0)f |  | 17.1±1.1 |
| 2-indanole | 343.0 [29] | (343.0×56.5) = (19.4±1.0)f |  | 17.2±1.2 |
| 2,6-dimethylphenol | 318.9 | 18.9±0.2 [45] | 59.2 | 17.7±1.1 |
| 2,4,6-trimethylphenol | 344.0 [29] | (344.0×56.5) = (19.4±1.0)f |  | 16.7±1.3 |

a Uncertainties are presented as expanded uncertainties (0.95 level of confidence with k=2).

b Experimental melting temperatures from references cited inside of this table.

c Enthalpies of fusion ∆crlHmo measured at Tfus .

d Walden´s Constants calculated according to Eq. (S10).

e Enthalpies of fusion ∆crlHmo measured at Tfus were adjusted to T = 298 K with help of the following equation [21]:

∆crlHmo(298 K)/(J·mol-1) = ∆crlHmo(Tfus/K) – (∆crgCp,mo-∆lgCp,mo)×[(Tfus/K) – 298 K],

where ∆lgCp,mo and ∆crgCp,mo were taken from Tables S9 and S10. Uncertainties in the temperature adjustment of fusion enthalpies from Tfus to the reference temperature are estimated to account with 30 % to the total adjustment [21].

f Calculated according to the equation: ∆crlHmo(Tfus/K)  = WC × Tfus with the Walden's Constant (WC = 56.5 J·K-1·mol-1) [38, 39]

Table S15

Thermochemical data at T = 298 K (p° = 0.1 MPa) for auxiliary compounds (in kJ·mol-1)

| Compounds | ∆fHmo(liq) | ∆lgHmo | ∆fHmo(g) |
| --- | --- | --- | --- |
| cyclopentane [287-92-3] [11] | -105.1±0.8 | 28.7±0.2 | -76.4±0.8 |
| methyl-cyclopentane [96-37-7] [11] | -137.9±0.8 | 31.8±0.2 | -106.2±0.8 |
| cyclohexane [110-82-7] [11] | -156.4±0.8 | 33.0±0.2 | -123.4±0.8 |
| methyl-cyclohexane [108-87-2] [11] | -190.2±1.0 | 35.4±0.2 | -154.8±1.1 |
| naphthalene [18] | 95.2±2.1 | 55.4±1.4 | 150.6±1.5 |
| trans-decalin [11] | -230.6±1.0 | 48.4±0.3 [27] | -182.2±1.1 |
| furan 110-00-9 [11] | -62.3±0.7 | 27.4±0.2 | -34.9±0.7 |
| tetrahydro-furan 109-99-9 [11] | -216.2±0.8 | 32.0±0.2 | -184.2±0.8 |
| cyclopentanone [120-92-3] [46] | -236.2±1.1 | 42.8±0.2 | -193.4±1.1 |
| cyclohexanone 108-94-1 [11] | -271.2±2.1 | 45.1±0.2 | -226.1±2.1 |
| cyclopentanol 96-41-3 [11] | -300.1±1.6 | 57.6±0.3 | -242.6±1.7 |
| cyclohexanol [108-93-0] [47, 48] | -352.0±0.7 | 61.8±0.6 | -290.2±0.9 |
| 2-Me-cyclopentanone [1120-72-5] [11] | -268.6±3.6 | 45.1±0.7 | -223.5±3.5 |
| 3-Me-cyclopentanone [1757-42-2] [11] | -269.9±3.6 | 45.8±0.8 | -224.1±3.5] |
| cis-2-Me-cyclopentanol [25144-05-2] [11] | -329.1±3.9 | 58.3±1.7 | -270.8±3.5 |
| 1,2-dimethoxy-ethane [26] | -379.6±0.6 | 41.9±0.6 | -337.7±0.8 |
| 1,2-dimethoxy-benzene [49] | -290.3±2.3 | 64.5±0.3 | -206.0±3.1 |
| 1,2-dimethoxy-cyclohexane [Tables S3 and S11] | -474.8±4.0 | 64.1±2.0 | -410.7±3.5 |
| 8H-benzofuran [13054-97-2] [6] | -283.1±4.3 | 48.8±1.2 | -234.3±4.1 |
| benzene [71-43-2] [18] | 49.0±0.9 | 33.9±0.1 | 82.9±0.9 |
| methyl-benzene [108-88-3] [18] | 12.0±1.1 | 38.1±0.1 | 50.1±1.1 |
| methyl-cyclohexane [108-87-2] [11] | -190.2±1.0 | 35.4±0.2 | -154.8±1.1 |
| phenol [108-95-2] [19] | -150.5±1.3 | 58.0±0.6 | -92.5±1.2 |
| methoxy-benzene [100-66-3] [19] | -116.9±0.7 | 46.4±0.2 | -70.3±0.7 |
| methoxy-cyclohexane [931-56-6] [50] | -309.8±3.6 | 43.0±0.5 | -266.8±3.5 |
| indane [496-11-7] [11] | 11.5±1.6 | 49.0±0.5 | 60.5±1.7 |
| trans-6H-indane [3296-50-2] [11] | -176.2±1.8 | 44.8±1.3 | -131.4±2.2 |

Table S16

Evaluation of the available thermochemical data at T = 298 K (p° = 0.1 MPa) for auxiliary bicyclic aliphatic ketones (in kJ·mol-1)

| Compound | ∆fHmo(liq)exp b | ∆lgHmoc | ∆fHmo(g)expd | ∆fHmo(g)QCe | f |
| --- | --- | --- | --- | --- | --- |
| cis-6H-1-indanone [2826-65-5] | (-328.0±5.0) [51] | 58.8±0.9 | (-269.2±5.1) | -248.4 | -20.8 |
| trans-6H-1-indanone [16783-22-5] | (-349.0±5.0) [51] | 58.8±0.9 | (-290.2±5.1) | -248.4 | -41.8 |
| cis-6H-2-indanone [5689-04-3] | -307.1±1.3 [52] | 57.6±0.3 | -249.5±1.3 | -251.4 | 1.9 |
| trans-6H-2-indanone [16484-17-6] | -305.3±1.6 [52] | 56.3±0.3 | -249.0±1.6 | -251.4 | 2.4 |
| cis-3a-Me-2-indanone [13351-29-6](cr) | -347.9±3.4 [53] | 60.9±0.2 [53] | -287.0±3.4 | -285.7 | -1.3 |
| trans-3a-Me-2-indanone [20379-99-1] | -333.5±1.6 [53] | 58.3±0.2 [53] | -275.2±3.2 | -277.7 | 1.8 |

a Uncertainties correspond to expanded uncertainties of the mean (0.95 level of confidence).

b Experimental values from the literature. Numbers in parentheses were considered questionable. The recommended values are printed in bold.

c The evaluated values from Table 2.

d The sum of columns 3 and 4.

e The evaluated values from Tables S3 and S8.

f Difference between columns 4 and 5.

Basics of the group-additivity concept (“centerpiece” approach)

Group additivity (GA) methods are successfully used to predict both the gas-phase enthalpies of formation and vaporisation enthalpies. The idea behind conventional GA methods is to split the experimental enthalpies of molecules into relatively small groups in order to obtain well-defined numerical contributions for them. The prediction then proceeds as construction of a framework of desired model molecule from the appropriate number and type of these contributions. Comprehensive systems of group contributions (or increments) are developed, covering the main classes of organic compound. Admittedly, the applicability of GA in the case of cyclic molecules is limited. To overcome this limitation, various ring corrections are implemented in the GA parameterization, however, each correction term is specific only to a particular unsubstituted ring (e.g. for cyclobutane, cyclopentane, cyclohexane, etc.). However, any type of substitution significantly affects ring strain due to intense interactions of the substituent(s) with the ring. The idea of the “centerpiece” approach is to select a potentially large "centerpiece" molecule that has a reliable enthalpy and that can generally mimic the structure of the molecule of interest. Then the necessary groups or blocks are attached to the "centerpiece", resulting in the construction of the desired molecule. A prerequisite for this method is that reliable experimental data are available for the selected “centerpiece” molecule.

Fig. S2 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 2-Me-1-indanone using the “centerpiece approach” based on 1-indanone and the structure-property difference in the pair 2-Me-cyclopentanone/cyclopentanone. The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 and Table S15.

Fig. S3 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 3-Me-1-indanone using the “centerpiece approach” based on 1-indanone and the structure-property difference in the pair 3-Me-cyclopentanone/cyclopentanone. The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 and Table S15.

Fig. S4 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 5-Me-1-indanone (and 6-Me-1-indanone) using the “centerpiece approach” based on 1-indanone and the structure-property difference in the pair toluene/benzene. The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 and Table S15.

Fig. S5 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 5-MeO-1-indanone (as well as 4-MeO-1-indanone and 6-MeO-1-indanone) using the “centerpiece approach” based on 1-indanone and the structure-property difference in the pair methoxybenzene/benzene. The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 and Table S15.

Fig. S6 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 5,6-dimethoxy-1-indanone using the “centerpiece approach” based on 1-indanone and the structure-property difference in the pair 1,2-dimethoxybenzene/benzene. The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 and Table S15.

Fig. S7 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 1,3-indanedione using the “centerpiece approach” based on 1-indanone and the structure-property difference in the pair cyclopentanone/cyclopentane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 andTable S15.

Fig. S8 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-1-indanone (or 6H-2-indanone) using the “centerpiece approach” based on indane and the structure-property difference in the pair cyclopentanone/cyclopentane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S9 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-1-indanone (or 6H-2-indanone) using the “centerpiece approach” based on 1-indanone (or 2-indanone) and the structure-property difference in the pair indane/6H-indane (first line). The numerical values of enthalpies of vaporisation required for calculations are given in Table 2 and Table S15.

Fig. S10 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-3Me-1-indanone (or 6H-2Me-1-indanone) using the “centerpiece approach” based on 6H-1-indanone and the structure-property difference in the pair Me-cyclopentane/cyclopentane. The numerical values of enthalpies of vaporisation required for calculations are given in Tables 3 and S15.

Fig. S11 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-5Me-1-indanone (or 6H-6Me-1-indanone) using the “centerpiece approach” based on 6H-1-indanone and the structure-property difference in the pair Me-cyclohexane/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S12 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-4MeO-1-indanone (or 6H-5MeO-1-indanone and 6H-6MeO-1-indanone) using the “centerpiece approach” based on 6H-1-indanone and the structure-property difference in the pair methoxy-cyclohexane/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and S15.

Fig. S13 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-5,6-diMeO-1-indanone using the “centerpiece approach” based on 6H-1-indanone and the structure-property difference in the pair 1,2-dimethoxy-cyclohexane/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S14 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-1,3-indanedione (using the “centerpiece approach” based on 6H-1-indanone and the structure-property difference in the pair cyclopentanone/cyclopentane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S15 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-4-benzofuranone using the “centerpiece approach” based on 8H-benzofuran and the structure-property difference in the pair cyclohexanone/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S16 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-1-indanol (or 6H-2-indanol) using the “centerpiece approach” based on 6H-indane and the structure-property difference in the pair cyclopentanol/cyclopentane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S17 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-2Me-1-indanol using the “centerpiece approach” based on 6H-1-indanol and the structure-property difference in the pair 2-Me-cyclopentanol/cyclopentanol. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S18 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-3Me-1-indanol using the “centerpiece approach” based on 6H-1-indanol and the structure-property difference in the pair 3-Me-cyclopentanol/cyclopentanol. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S19 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-5Me-1-indanol (or 6H-6Me-1-indanol) using the “centerpiece approach” based on 6H-1-indanol and the structure-property difference in the pair Me-cyclohexanol/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S20 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-4MeO-1-indanol (or 6H-5MeO-1-indanol and 6H-6MeO-1-indanol) using the “centerpiece approach” based on 6H-1-indanol and the structure-property difference in the pair methoxy-cyclohexane/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S21 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-5,6-diMeO-1-indanol using the “centerpiece approach” based on 6H-1-indanol and the structure-property difference in the pair 1,2-dimethoxy-cyclohexane/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Fig. S22 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 6H-1,3-indanediol (using the “centerpiece approach” based on 6H-1-indanol and the structure-property difference in the pair 1cyclopentanol/cyclopentane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and S15.

Fig. S23 Calculating the enthalpy of vaporisation, ∆lgHmo(298 K)/ kJmol-1 of 8H-4-benzofuranol using the “centerpiece approach” based on 8H-benzofuran and the structure-property difference in the pair cyclohexanol/cyclohexane. The numerical values of enthalpies of vaporisation required for calculations are given in Table 3 and Table S15.

Table S17 Calculation of the liquid phase reaction enthalpies, ∆rHmo(liq), the equilibrium temperatures, Teq, and the total Gibbs free energies of reactions, ∆rGmo(liq), of the partial hydrogenation of the hydrogen-lean compounds shown in Fig. 1 at T = 298.15 K (p°=0.1 MPa, in kJ·mol-1)

| HL Compound | ∆fHmo(liq) | HR Compound | ∆fHmo(liq) |  | ∆rHmo(liq) | ∆rHmoa | Teq | ∆rGmo(liq)b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-indanone, 83-33-0 | -126.0 | 6H-1-indanone, [29927-85-3] | -307.2 | R1 | -181.2 | -60.4 | 477 | 12.5 |
| 2-indanone, 615-13-4 | -114.9 | 6H-2-indanone, [6088-72-8] | -307.6 | R2 | -192.7 | -64.2 | 525 | -5.2 |
| 2-Me-1-indanone, 17496-14-9 | -156.5 | 6H-2-Me-1-indanone [2308731-82-8] | -337.1 | R3 | -180.6 | -60.2 | 496 | 5.8 |
| 3-Me-1-indanone, 6072-57-7 | -157.2 | 6H-3-Me-1-indanone [64558-16-3] | -341.7 | R4 | -184.5 | -61.5 | 505 | 1.9 |
| 5-Me-1-indanone, 4593-38-8 | -160.7 | 6H-5-Me-1-indanone, [92015-40-2] | -341.2 | R5 | -180.5 | -60.2 | 482 | 10.5 |
| 6-Me-1-indanone, 24623-20-9 | -160.2 | 6H-6-Me-1-indanone [1824616-45-6] | -341.5 | R6 | -181.3 | -60.4 | 473 | 14.4 |
| 4-methoxy-1-indanone, 13336-31-7 | -288.8 | 6H-4-methoxy-1-indanone [1823221-69-7] | -465.7 | R7 | -176.9 | -59.0 | 486 | 8.9 |
| 5-methoxy-1-indanone, 5111-70-6 | -292.1 | 6H-5-methoxy-1-indanone | -463.3 | R8 | -171.2 | -57.1 | 467 | 15.9 |
| 6-methoxy-1-indanone, 13623-25-1 | -289.5 | 6H-6-methoxy-1-indanone | -464.2 | R9 | -174.6 | -58.2 | 479 | 11.6 |
| 5,6-methoxy-1-indanone, 2107-69-9 | -447.9 | 6H-5,6-methoxy-1-indanone [1195675-41-2] | -628.0 | R10 | -180.1 | -60.0 | 521 | -3.4 |
| 1,3-indandione, 606-23-5 | -242.2 | 6H-1,3-indandione [16385-56-1] | -420.0 | R11 | -177.8 | -59.3 | 465 | 17.3 |
| 6,7-dihydro-4-benzofuranone, 16806-93-2 | -291.2 | 6H-4-benzofuranone [101581-29-7] | -406.1 | R12 | -114.7 | -57.5 | 496 | 2.3 |
| 1-indanol, 6351-10-6 | -176.6 | 6H-1-indanol [53927-31-4] | -369.5 | R13 | -193.0 | -64.3 | 503 | 2.8 |
| 2-indanol, 4254-29-9 | -179.8 | 6H-2-indanol [59078-17-0] | -367.9 | R14 | -188.1 | -62.7 | 494 | 6.0 |

a Reaction enthalpies per 1 mole of H2.

a Gibbs free reaction enthalpies at 500 K.

Table S18

Calculation of the liquid phase reaction enthalpies, ∆rHmo(liq), the equilibrium temperatures, Teq, and the total Gibbs free energies of reactions, ∆rGmo(liq), of the aliphatic cyclic  ketones (shown in Fig. 1) to the aliphatic alcohols (shown in Fig. 1)  at T = 298.15 K (p°=0.1 MPa, in kJ·mol-1)

| HL Compound | ∆fHmo(liq) | HR Compound | ∆fHmo(liq) |  | ∆rHmo(liq)a | Teq | ∆rGmo(liq)b |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6H-1-indanone, [29927-85-3] | -307.2 | 6H-1-indanol [53927-31-4] | -369.5 | R1´ | -62.3 | 416 | 12.6 |
| 6H-2-indanone, [6088-72-8] | -307.6 | 6H-2-indanol [59078-17-0] | -367.9 | R2´ | -60.3 | 388 | 17.3 |
| 6H-2-Me-1-indanone [2308731-82-8] | -337.1 | 6H-2-Me-1-indanol [16149-29-4] | -399.5 | R3´ | -62.4 | 396 | 16.4 |
| 6H-3-Me-1-indanone [64558-16-3] | -341.7 | 6H-3-Me-1-indanol [857773-69-4] | -404.6 | R4´ | -62.8 | 412 | 13.5 |
| 6H-5-Me-1-indanone, [92015-40-2] | -341.2 | 6H-5-Me-1-indanol | -404.0 | R5´ | -62.8 | 394 | 16.9 |
| 6H-6-Me-1-indanone [1824616-45-6] | -341.5 | 6H-6-Me-1-indanol | -404.1 | R6´ | -62.7 | 393 | 17.1 |
| 6H-4-methoxy-1-indanone [1823221-69-7] | -465.7 | 6H-4-methoxy-1-indanol [1823625-45-1] | -530.4 | R7´ | -64.7 | 418 | 12.7 |
| 6H-5-methoxy-1-indanone | -463.3 | 6H-5-methoxy-1-indanol | -528.2 | R8´ | -64.9 | 420 | 12.5 |
| 6H-6-methoxy-1-indanone | -464.2 | 6H-6-methoxy-1-indanol [13336-33-9] | -528.6 | R9´ | -64.4 | 416 | 13.0 |
| 6H-5,6-methoxy-1-indanone [1195675-41-2] | -628.0 | 6H-5,6-methoxy-1-indanol | -694.8 | R10´ | -66.8 | 432 | 10.6 |
| 6H-1,3-indandione [16385-56-1] | -420.0 | 6H-1,3-indandiol | -564.4 | R11´ | -72.2 | 467 | 10.4 |
| 6H-4-benzofuranone [101581-29-7] | -406.1 | 8H-4-benzofuranol [99767-96-1] | -486.5 | R12´ | -80.4 | 540 | -6.0 |

a Reaction enthalpies per 1 mole of H2.

b Gibbs free reaction energies at 500 K.

Table S19 Calculation of the liquid phase reaction enthalpies, ∆rHmo(liq), the equilibrium temperatures, Teq, and the total Gibbs free energies of reactions, ∆rGmo(liq), ), of the full hydrogenation of the hydrogen-lean compounds (shown in Fig. 1) to the aliphatic alcohols (shown in Fig. 1) at T = 298.15 K (p°=0.1 MPa, in kJ·mol-1)

| HL Compound | ∆fHmo(liq) | HR Compound | ∆fHmo(liq) |  | ∆rHmo(liq) | ∆rHmoa | Teq | ∆rGmo(liq)b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-indanone, 83-33-0 | -126.0 | 6H-1-indanol [53927-31-4] | -369.5 | R1´´ | -243.5 | -60.9 | 460 | 25.1 |
| 2-indanone, 615-13-4 | -114.9 | 6H-2-indanol [59078-17-0] | -367.9 | R2´´ | -253.0 | -63.3 | 484 | 12.1 |
| 2-Me-1-indanone, 17496-14-9 | -156.5 | 6H-2-Me-1-indanol [16149-29-4] | -399.5 | R3´´ | -242.9 | -60.8 | 465 | 22.2 |
| 3-Me-1-indanone, 6072-57-7 | -157.2 | 6H-3-Me-1-indanol [857773-69-4] | -404.6 | R4´´ | -247.3 | -61.8 | 478 | 15.5 |
| 5-Me-1-indanone, 4593-38-8 | -160.7 | 6H-5-Me-1-indanol | -404.0 | R5´´ | -243.3 | -60.8 | 456 | 27.4 |
| 6-Me-1-indanone, 24623-20-9 | -160.2 | 6H-6-Me-1-indanol | -404.1 | R6´´ | -243.9 | -61.0 | 449 | 31.5 |
| 4-methoxy-1-indanone, 13336-31-7 | -288.8 | 6H-4-methoxy-1-indanol [1823625-45-1] | -530.4 | R7´´ | -241.6 | -60.4 | 466 | 21.6 |
| 5-methoxy-1-indanone, 5111-70-6 | -292.1 | 6H-5-methoxy-1-indanol | -528.2 | R8´´ | -236.1 | -59.0 | 453 | 28.4 |
| 6-methoxy-1-indanone, 13623-25-1 | -289.5 | 6H-6-methoxy-1-indanol [13336-33-9] | -528.6 | R9´´ | -239.1 | -59.8 | 460 | 24.6 |
| 5,6-methoxy-1-indanone, 2107-69-9 | -447.9 | 6H-5,6-methoxy-1-indanol | -694.8 | R10´´ | -246.9 | -61.7 | 494 | 7.2 |
| 1,3-indandione, 606-23-5 | -242.2 | 6H-1,3-indandiol | -564.4 | R11´´ | -322.2 | -64.4 | 466 | 27.7 |
| 6,7-dihydro-4-benzofuranone, 16806-93-2 | -291.2 | 8H-4-benzofuranol [99767-96-1] | -486.5 | R12´´ | -195.3 | -65.1 | 513 | -3.6 |

a Reaction enthalpies per 1 mole of H2.

b Gibbs free reaction energies at 500 K.

Table S20

Calculation of the liquid phase reaction entropies, ∆rSmo(liq), of the hydrogenation of compounds shown in Fig. 1, at T = 298 K (p°=0.1 MPa, in J·K-1·mol-1)

|  | HL compound | Smo(liq)HLa | HR compound | Smo(liq)HRb | ∆rSmo(liq)c | ∆rSmo(liq)/H2 d |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | 1-indanone | 242.7 | 6H-1-indanone | 255.1 | -379.7 | -126.6 |
| R2 | 2-indanone | 238.1 | 6H-2-indanone | 262.3 | -367.9 | -122.6 |
| R3 | 2-Me-1-indanone | 260.8 | 6H-2-Me-1-indanone | 288.5 | -364.4 | -121.5 |
| R4 | 3-Me-1-indanone | 259.7 | 6H-3-Me-1-indanone | 286.6 | -365.2 | -121.7 |
| R5 | 5-Me-1-indanone | 274.0 | 6H-5-Me-1-indanone | 291.9 | -374.2 | -124.7 |
| R6 | 6-Me-1-indanone | 283.3 | 6H-6-Me-1-indanone | 291.9 | -383.5 | -127.8 |
| R7 | 4-MeO-1-indanone | 282.3 | 6H-4-methoxy-1-indanone | 310.7 | -363.7 | -121.2 |
| R8 | 5-MeO-1-indanone | 285.0 | 6H-5-methoxy-1-indanone | 310.7 | -366.4 | -122.1 |
| R9 | 6-MeO-1-indanone | 283.4 | 6H-6-methoxy-1-indanone | 310.7 | -364.8 | -121.6 |
| R10 | 5,6-diMeO-1-indanone | 314.7 | 6H-5,6-methoxy-1-indanone | 361.4 | -345.4 | -115.1 |
| R11 | 1,3-indandione | 251.5 | 6H-1,3-indandione | 261.1 | -382.5 | -127.5 |
| R12 | 6,7-dihydro-4-benzofuranone | 179.9 | 6H-4-benzofuranone | 209.5 | -231.8 | -115.9 |
| R13 | 1-indanol | 227.9 | 6H-1-indanol | 236.0 | -384.0 | -128.0 |
| R14 | 2-indanol | 226.9 | 6H-2-indanol | 238.4 | -380.6 | -126.9 |
| R1'' | 1-indanone | 242.7 | 6H-1-indanol | 236.0 | -529.5 | -132.4 |
| R2'' | 2-indanone | 238.1 | 6H-2-indanol | 238.4 | -522.5 | -130.6 |
| R3'' | 2-Me-1-indanone | 260.8 | 6H-2-Me-1-indanol | 260.9 | -522.7 | -130.7 |
| R4'' | 3-Me-1-indanone | 259.7 | 6H-3-Me-1-indanol | 264.5 | -518.0 | -129.5 |
| R5'' | 5-Me-1-indanone | 274.0 | 6H-5-Me-1-indanol | 263.3 | -533.5 | -133.4 |
| R6'' | 6-Me-1-indanone | 283.3 | 6H-6-Me-1-indanol | 263.3 | -542.8 | -135.7 |
| R7'' | 4-MeO-1-indanone | 282.3 | 6H-4-methoxy-1-indanol | 286.7 | -518.4 | -129.6 |
| R8'' | 5-MeO-1-indanone | 285.0 | 6H-5-methoxy-1-indanol | 286.7 | -521.1 | -130.3 |
| R9'' | 6-MeO-1-indanone | 283.4 | 6H-6-methoxy-1-indanol | 286.7 | -519.5 | -129.9 |
| R10'' | 5,6-diMeO-1-indanone | 314.7 | 6H-5,6-methoxy-1-indanol | 337.4 | -500.1 | -125.0 |
| R11'' | 1,3-indandione | 251.5 | 6H-1,3-indanediol | 213.1 | -691.9 | -138.4 |
| R12'' | 6,7-dihydro-4-benzofuranone | 179.9 | 8H-4-benzofuranol | 191.4 | -380.6 | -126.9 |
|  |  |  |  |  | average | -126.8 |
| R1' | 6H-1-indanone | 255.1 | 6H-1-indanol | 236.0 | -149.8 | -149.8 |
| R2' | 6H-2-indanone | 262.3 | 6H-2-indanol | 238.4 | -154.6 | -154.6 |
| R3' | 6H-2-Me-1-indanone | 287.8 | 6H-2-Me-1-indanol | 260.9 | -157.6 | -157.6 |
| R4' | 6H-3-Me-1-indanone | 286.6 | 6H-3-Me-1-indanol | 264.5 | -152.8 | -152.8 |
| R5' | 6H-5-Me-1-indanone | 291.9 | 6H-5-Me-1-indanol | 263.3 | -159.3 | -159.3 |
| R6' | 6H-6-Me-1-indanone | 291.9 | 6H-6-Me-1-indanol | 263.3 | -159.3 | -159.3 |
| R7' | 6H-4-methoxy-1-indanone | 310.7 | 6H-4-methoxy-1-indanol | 286.7 | -154.7 | -154.7 |
| R8' | 6H-5-methoxy-1-indanone | 310.7 | 6H-5-methoxy-1-indanol | 286.7 | -154.7 | -154.7 |
| R9' | 6H-6-methoxy-1-indanone | 310.7 | 6H-6-methoxy-1-indanol | 286.7 | -154.7 | -154.7 |
| R10' | 6H-5,6-methoxy-1-indanone | 361.4 | 6H-5,6-methoxy-1-indanol | 337.4 | -154.7 | -154.7 |
| R11' | 6H-1,3-indandione | 261.1 | 6H-1,3-indanediol | 213.1 | -178.7 | -154.7 |
| R12' | 6H-4-benzofuranone | 209.5 | 8H-4-benzofuranol | 191.4 | -148.8 | -148.5 |
|  |  |  |  |  | average | -154.6 |

a Liquid-phase entropies of formation of hydrogen-lean (HL) counterparts of the LOHC systems from Table 5.

b Liquid-phase entropies of formation of hydrogen-rich (HR) counterparts of the LOHC systems from Table 5.

c The total change of the reaction entropies of hydrogenation reactions of compounds shown in Fig. 1.

d Reaction entropies per 1 mole of H2.

Table S21.

Calculation of the liquid phase reaction enthalpies, ∆rHmo (at T = 298 K), the equilibrium temperatures, Teq (in K), and the Gibbs free energies of reactions, ∆rGmo, (in kJ·mol-1) of the partial or full hydrogenation of the HL and HR compounds shown in Fig. 1 (p°=0.1 MPa, in kJ·mol-1). Values in bold are related are related to the number of hydrogen molecules released in the reactions

| Reactants | ∆fHmoa (298 K) | Smob (298 K) | Cp,moc (298 K) | ∆rHmod (298 K) | ∆rSmoe (298 K) | ∆rGmof (298 K) | ∆rGmo g (500 K) | Teqh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-indanone | -126.0 | 242.7 | 216.8 |  |  |  |  |  |
| 6H-1-indanone | -307.2 | 255.1 | 235.8 |  |  |  |  |  |
| R1 |  |  |  | -181.2 | -379.7 | -67.4 | -68.0 | 477 |
|  |  |  |  | -60.4 | -126.6 | -22.7 | 4.2 |  |
| 2-indanone | -114.9 | 238.1 | 216.8 |  |  |  |  |  |
| 6H-2-indanone | -307.6 | 262.3 | 235.8 |  |  |  |  |  |
| R2 |  |  |  | -192.7 | -367.9 | -83.3 | -5.2 | 525 |
|  |  |  |  | -64.2 | -122.6 | -27.8 | -1.7 |  |
| 2-Me-1-indanone | -156.5 | 260.8 | 243.6 |  |  |  |  |  |
| 6H-2-Me-1-indanone | -337.1 | 288.5 | 262.6 |  |  |  |  |  |
| R3 |  |  |  | -180.6 | -364.4 | -71.2 | 5.4 | 496 |
|  |  |  |  | -60.2 | -121.5 | -24.0 | 1.8 |  |
| 3-Me-1-indanone | -157.2 | 259.7 | 243.6 |  |  |  |  |  |
| 6H-3-Me-1-indanone | -341.7 | 286.6 | 262.6 |  |  |  |  |  |
| R4 |  |  |  | -184.5 | -365.2 | -75.6 | 1.9 | 505 |
|  |  |  |  | -61.5 | -121.7 | -25.2 | 0.6 |  |
| 5-Me-1-indanone | -160.7 | 274.0 | 245.2 |  |  |  |  |  |
| 6H-5-Me-1-indanone | -341.2 | 291.9 | 262.6 |  |  |  |  |  |
| R5 |  |  |  | -180.5 | -374.2 | -68.9 | 10.5 | 482 |
|  |  |  |  | -60.2 | -124.7 | -23.0 | 3.5 |  |
| 6-Me-1-indanone | -160.2 | 283.3 | 245.2 |  |  |  |  |  |
| 6H-6-Me-1-indanone | -341.5 | 291.9 | 262.6 |  |  |  |  |  |
| R6 |  |  |  | -181.3 | -383.5 | -67.0 | 14.4 | 473 |
|  |  |  |  | -60.4 | -127.8 | -22.3 | 4.8 |  |
| 4-MeO-1-indanone | -288.8 | 282.3 | 268.4 |  |  |  |  |  |
| 6H-4-MeO-1-indanone | -465.7 | 310.7 | 285.8 |  |  |  |  |  |
| R7 |  |  |  | -176.9 | -363.7 | -68.5 | 8.9 | 486 |
|  |  |  |  | -59.0 | -121.2 | -22.8 | 3.0 |  |
| 5-MeO-1-indanone | -292.1 | 285.0 | 268.4 |  |  |  |  |  |
| 6H-5-MeO-1-indanone | -463.3 | 310.7 | 285.8 |  |  |  |  |  |
| R8 |  |  |  | -171.2 | -366.4 | -62.0 | 15.9 | 467 |
|  |  |  |  | -57.1 | -122.1 | -20.7 | 5.3 |  |
| 6-MeO-1-indanone | -289.6 | 283.4 | 268.4 |  |  |  |  |  |
| 6H-6-MeO-1-indanone | -464.2 | 310.7 | 285.8 |  |  |  |  |  |
| R9 |  |  |  | -174.7 | -364.8 | -65.9 | 11.6 | 479 |
|  |  |  |  | -58.2 | -121.6 | -22.0 | 3.9 |  |
| 5,6-diMeO-1-indanone | -447.9 | 314.7 | 320.0 |  |  |  |  |  |
| 6H-5,6-di-MeO-1-indanone | -628.0 | 361.4 | 335.8 |  |  |  |  |  |
| R10 |  |  |  | -180.1 | -345.4 | -77.1 | -3.4 | 521 |
|  |  |  |  | -60.0 | -115.1 | -25.7 | -1.1 |  |
| 1,3-indandione | -242.2 | 251.5 | 239.3 |  |  |  |  |  |
| 6H-1,3-indandione | -420. | 261.1 | 258.3 |  |  |  |  |  |
| R11 |  |  |  | -177.8 | -382.5 | -63.8 | 17.3 | 465 |
|  |  |  |  | -59.3 | -127.5 | -21.3 | 5.8 |  |
| 6,7-dihydro-4-benzofuranone | -291.2 | 179.9 | 199.3 |  |  |  |  |  |
| 6H-4-benzofuranone | -406.1 | 209.5 | 233.5 |  |  |  |  |  |
| R12 |  |  |  | -114.9 | -231.8 | -45.8 | 2.3 | 496 |
|  |  |  |  | -57.5 | -115.9 | -22.9 | 1.2 |  |
| 1-indanol | -176.5 | 227.9 | 244.9 |  |  |  |  |  |
| 6H-1-indanol | -369.5 | 236.0 | 263.9 |  |  |  |  |  |
| R13 |  |  |  | -193.0 | -384.0 | -78.5 | 2.8 | 503 |
|  |  |  |  | -64.3 | -128.0 | -26.2 | 0.9 |  |
| 2-indanol | -179.8 | 226.9 | 244.9 |  |  |  |  |  |
| 6H-2-indanol | -367.9 | 238.4 | 263.9 |  |  |  |  |  |
| R14 |  |  |  | -188.1 | -380.6 | -74.6 | 6.0 | 494 |
|  |  |  |  | -62.7 | -126.9 | -24.9 | 2.0 |  |

a From Table 4 (main text).

b From Table 5 (main text).

c From Table S9.

d Calculated according to Eq. (1) from standard molar enthalpies of formation of reactants from column 2.

e Calculated according to Eq. (3) from standard molar entropies of reactants from column 3.

f Calculated according to Eq. (4) from results given in column 5 and 6 and referenced to 298 K.

g Calculated according to Eq. (4) from results given in column 5 and 6 and adjusted to 500 K.

h Calculated according to Eq. (2).

Table S22

Calculation of the liquid phase reaction enthalpies, ∆rHmo (at T = 298 K), the equilibrium temperatures, Teq (in K), and the Gibbs free energies of reactions, ∆rGmo, (in kJ·mol-1), of the aliphatic cyclic ketones (shown in Fig. 1) to the aliphatic alcohols (shown in Fig. 1) (p°=0.1 MPa, in kJ·mol-1). Values in bold are related are related to the number of hydrogen molecules released in the reactions

| Reactants | ∆fHmoa (298 K) | Smob (298 K) | Cp,moc (298 K) | ∆rHmod (298 K) | ∆rSmoe (298 K) | ∆rGmof (298 K) | ∆rGmo g (500 K) | Teqh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6H-1-indanone | -307.2 | 255.1 | 235.8 |  |  |  |  |  |
| 6H-1-indanol | -369.5 | 236.0 | 263.9 |  |  |  |  |  |
| R1´ |  |  |  | -62.3 | -149.8 | -17.6 | 12.6 | 416 |
| 6H-2-indanone | -307.6 | 262.3 | 235.8 |  |  |  |  |  |
| 6H-2-indanol | -367.9 | 238.4 | 263.9 |  |  |  |  |  |
| R2´ |  |  |  | -60.0 | -154.6 | -13.9 | 17.3 | 388 |
| 6H-2-Me-1-indanone | -337.1 | 288.5 | 262.6 |  |  |  |  |  |
| 6H-2-Me-1-indanol | -399.4 | 260.9 | 290.7 |  |  |  |  |  |
| R3´ |  |  |  | -62.4 | -158.3 | -15.2 | 16.8 | 394 |
| 6H-3-Me-1-indanone | -341.7 | 286.6 | 262.6 |  |  |  |  |  |
| 6H-3-Me-1-indanol | -404.6 | 264.5 | 290.7 |  |  |  |  |  |
| R4´ |  |  |  | -62.8 | -152.8 | -17.3 | 13.5 | 412 |
| 6H-5-Me-1-indanone | -341.2 | 291.9 | 262.6 |  |  |  |  |  |
| 6H-5-Me-1-indanol | -404.0 | 263.3 | 290.7 |  |  |  |  |  |
| R5´ |  |  |  | -62.8 | -159.3 | -15.3 | 16.9 | 394 |
| 6H-6-Me-1-indanone | -341.5 | 291.9 | 262.6 |  |  |  |  |  |
| 6H-6-Me-1-indanol | -404.1 | 263.3 | 290.7 |  |  |  |  |  |
| R6´ |  |  |  | -62.7 | -159.3 | -15.1 | 17.1 | 393 |
| 6H-4-MeO-1-indanone | -465.7 | 310.7 | 285.8 |  |  |  |  |  |
| 6H-4-MeO-1-indanol | -530.4 | 286.7 | 313.9 |  |  |  |  |  |
| R7´ |  |  |  | -64.7 | -154.7 | -18.6 | 12.7 | 418 |
| 6H-5-MeO-1-indanone | -463.3 | 310.7 | 285.8 |  |  |  |  |  |
| 6H-5-MeO-1-indanol | -528.2 | 286.7 | 313.9 |  |  |  |  |  |
| R8´ |  |  |  | -64.9 | -154.7 | 18.8 | 12.5 | 420 |
| 6H-6-MeO-1-indanone | -464.2 | 310.7 | 285.8 |  |  |  |  |  |
| 6H-6-MeO-1-indanone | -528.6 | 286.7 | 313.9 |  |  |  |  |  |
| R9´ |  |  |  | -64.4 | -154.7 | -18.3 | 13.0 | 416 |
| 6H-5,6-diMeO-1-indanone | -628.0 | 361.4 | 335.8 |  |  |  |  |  |
| 6H-5,6-di-MeO-1-indanone | -694.8 | 337.4 | 363.9 |  |  |  |  |  |
| R10´ |  |  |  | -66.8 | -154.7 | -20.7 | 10.6 | 432 |
| 6H-1,3-indandione | -420.0 | 261.1 | 258.3 |  |  |  |  |  |
| 6H-1,3-indanediol | -564.4 | 213.1 | 314.5 |  |  |  |  |  |
| R11´ |  |  |  | -144.4 | -309.4 | -52.2 | 10.4 | 467 |
| 6H-4-benzofuranone | -406.1 | 209.5 | 233.5 |  |  |  |  |  |
| 8H-4-benzofuranol | -486.5 | 191.4 | 261.5 |  |  |  |  |  |
| R12´ |  |  |  | -80.4 | -148.8 | -36.0 | -6.0 | 540 |

a From Table 4 (main text).

b From Table 5 (main text).

c From Table S9.

d Calculated according to Eq. (1) from standard molar enthalpies of formation of reactants from column 2.

e Calculated according to Eq. (3) from standard molar entropies of reactants from column 3.

f Calculated according to Eq. (4) from results given in column 5 and 6 and referenced to 298 K.

g Calculated according to Eq. (4) from results given in column 5 and 6 and adjusted to 500 K.

h Calculated according to Eq. (2).

Table S23

Calculation of the liquid phase reaction enthalpies, ∆rHmo (at T = 298 K), the equilibrium temperatures, Teq (in K), and the Gibbs free energies of reactions, ∆rGmo, (in kJ·mol-1) of the full hydrogenation of the hydrogen-lean compounds (shown in Fig. 1) to the aliphatic alcohols (shown in Fig. 1) (p°=0.1 MPa, in kJ·mol-1). Values in bold are related are related to the number of hydrogen molecules released in the reactions

| Reactants | ∆fHmoa (298 K) | Smob (298 K) | Cp,moc (298 K) | ∆rHmod (298 K) | ∆rSmoe (298 K) | ∆rGmof (298 K) | ∆rGmo g (500 K) | Teqh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-indanone | -126.0 | 242.7 | 216.8 |  |  |  |  |  |
| 6H-1-indanol | -369.5 | 236.0 | 263.9 |  |  |  |  |  |
| R1´´ |  |  |  | -243.5 | -529.5 | -85.6 | 25.1 | 460 |
|  |  |  |  | -60.9 | -132.4 | -21.4 | 6.3 |  |
| 2-indanone | -114.9 | 238.1 | 216.8 |  |  |  |  |  |
| 6H-2-indanol | -367.9 | 238.4 | 263.9 |  |  |  |  |  |
| R2´´ |  |  |  | -253.0 | -522.5 | -97.2 | 12.1 | 484 |
|  |  |  |  | -63.3 | -130.6 | -24.3 | 3.0 |  |
| 2-Me-1-indanone | -156.5 | 260.8 | 243.6 |  |  |  |  |  |
| 6H-2-Me-1-indanol | -399.4 | 260.9 | 290.7 |  |  |  |  |  |
| R3´´ |  |  |  | -243.0 | -522.7 | -87.2 | 22.2 | 465 |
|  |  |  |  | -60.8 | -130.7 | -21.8 | 5.6 |  |
| 3-Me-1-indanone | -157.2 | 259.7 | 243.6 |  |  |  |  |  |
| 6H-3-Me-1-indanol | -404.6 | 264.5 | 290.7 |  |  |  |  |  |
| R4´´ |  |  |  | -274.4 | -518.0 | -93.0 | 15.5 | 478 |
|  |  |  |  | -61.8 | -129.5 | -23.2 | 3.9 |  |
| 5-Me-1-indanone | -160.7 | 274.0 | 245.2 |  |  |  |  |  |
| 6H-5-Me-1-indanol | -404.0 | 263.3 | 290.7 |  |  |  |  |  |
| R5´´ |  |  |  | -243.3 | -533.3 | -84.2 | 27.4 | 456 |
|  |  |  |  | -60.8 | -133.4 | -21.1 | 6.3 |  |
| 6-Me-1-indanone | -160.2 | 283.3 | 245.2 |  |  |  |  |  |
| 6H-6-Me-1-indanol | -404.1 | 263.3 | 290.7 |  |  |  |  |  |
| R6´´ |  |  |  | -243.9 | -542.8 | -82.1 | 31.5 | 449 |
|  |  |  |  | -61.0 | -135.7 | -20.5 | 7.9 |  |
| 4-MeO-1-indanone | -288.8 | 282.3 | 268.4 |  |  |  |  |  |
| 6H-4-MeO-1-indanol | -530.4 | 286.7 | 313.9 |  |  |  |  |  |
| R7´´ |  |  |  | -241.6 | -518.4 | -7.0 | 21.6 | 466 |
|  |  |  |  | -60.4 | -129.6 | 21.8 | 5.4 |  |
| 5-MeO-1-indanone | -292.1 | 285.0 | 268.4 |  |  |  |  |  |
| 6H-5-MeO-1-indanone | -528.2 | 286.7 | 313.9 |  |  |  |  |  |
| R8´´ |  |  |  | -236.1 | -521.1 | -80.7 | 28.4 | 453 |
|  |  |  |  | -59.0 | -130.3 | -20.2 | 7.1 |  |
| 6-MeO-1-indanone | -289.5 | 283.4 | 268.4 |  |  |  |  |  |
| 6H-6-MeO-1-indanol | -528.6 | 286.7 | 313.9 |  |  |  |  |  |
| R9´´ |  |  |  | -239.1 | -519.5 | -84.2 | 24.6 | 460 |
|  |  |  |  | -59.8 | -129.9 | -21.1 | 6.2 |  |
| 5,6-diMeO-1-indanone | -447.9 | 314.7 | 320.0 |  |  |  |  |  |
| 6H-5,6-di-MeO-1-indanol | -694.8 | 337.4 | 363.9 |  |  |  |  |  |
| R10´´ |  |  |  | -246.9 | -500.1 | -97.8 | 7.2 | 494 |
|  |  |  |  | -61.7 | -125.0 | -24.4 | 1.8 |  |
| 1,3-indandione | -242.2 | 251.5 | 239.3 |  |  |  |  |  |
| 6H-1,3-indanediol | -564.4 | 213.1 | 314.5 |  |  |  |  |  |
| R11´´ |  |  |  | -322.2 | -691.9 | -115.9 | 27.7 | 466 |
|  |  |  |  | -64.4 | -138.4 | -23.2 | 5.5 |  |
| 6,7-dihydro-4-benzofuranone | -291.2 | 179.9 | 199.3 |  |  |  |  |  |
| 8H-4-benzofuranol | -486.5 | 191.4 | 261.6 |  |  |  |  |  |
| R12´´ |  |  |  | -195.3 | -380.6 | -81.8 | -3.6 | 513 |
|  |  |  |  | -65.1 | -126.9 | -27.3 | -1.2 |  |
| Auxiliary (see Table S1) |  |  |  |  |  |  |  |  |
| cyclopentanone | -236.2 | 220.3 | 150.0 |  |  |  |  |  |
| cyclopentanol | -300.1 | 206.3 | 184.1 |  |  |  |  |  |
|  |  |  |  | -63.9 | -144.7 | -20.8 | 8.2 | 442 |
| cyclohexanone | -271.2 | 229.0 | 177.2 |  |  |  |  |  |
| cyclohexanol | -352.0 | 203.9 | 213.6 |  |  |  |  |  |
|  |  |  |  | -80.8 | -155.8 | -34.3 | -3.3 | 519 |
| furan | -62.3 | 176.7 | 114.6 |  |  |  |  |  |
| tetrahydrofuran | -216.2 | 203.9 | 123.9 |  |  |  |  |  |
|  |  |  |  | -153.9 | -234.2 | -48.3 | -34.1 | 657 |
|  |  |  |  | -77.0 | -117.1 | -42.0 | -17.0 |  |

a From Table 4 (main text).

b From Table 5 (main text).

c From Table S9.

d Calculated according to Eq. (1) from standard molar enthalpies of formation of reactants from column 2.

e Calculated according to Eq. (3) from standard molar entropies of reactants from column 3.

f Calculated according to Eq. (4) from results given in column 5 and 6 and referenced to 298 K.

g Calculated according to Eq. (4) from results given in column 5 and 6 and adjusted to 500 K.

h Calculated according to Eq. (2).

References

1. 	Petersson GA, Bennett A, Tensfeldt TG, et al (1988) A complete basis set model chemistry. I. The total energies of closed‐shell atoms and hydrides of the first‐row elements. J Chem Phys 89:2193–2218. https://doi.org/10.1063/1.455064

2. 	Pracht P, Bohle F, Grimme S (2020) Automated exploration of the low-energy chemical space with fast quantum chemical methods. Phys Chem Chem Phys 22:7169–7192. https://doi.org/10.1039/C9CP06869D

3. 	Frisch MJ, Trucks GW, Schlegel HB, et al (2016) Gaussian 16, Revision C.01 Gaussian, Inc., Wallingford CT

4. 	Curtiss LA, Raghavachari K, Redfern PC, Pople JA (2000) Gaussian-3 theory using scaled energies. J Chem Phys 112:1125–1132. https://doi.org/10.1063/1.480668

5. 	Verevkin SP, Emel’yanenko VN, Notario R, et al (2012) Rediscovering the Wheel. Thermochemical Analysis of Energetics of the Aromatic Diazines. J Phys Chem Lett 3:3454–3459. https://doi.org/10.1021/jz301524c

6. 	Siewert R, Samarov AA, Vostrikov S V, et al (2025) Hydrogenation of Aromatic Ethers and Lactones: Does the Oxygen Functionality Really Improve the Thermodynamics of Reversible Hydrogen Storage in the Related LOHC Systems? Oxygen 5:18. https://doi.org/10.3390/oxygen5030018

7. 	Verevkin SP (1998) Thermochemistry of aromatic ketones. Experimental enthalpies of formation and structural effects. Thermochim Acta 310:229–235. https://doi.org/10.1016/s0040-6031(97)00231-1

8. 	Matos MAR, Sousa CCS, Morais VMF (2008) Experimental and theoretical thermochemistry of β-tetralone. J Chem Thermodyn 40:1552–1557. https://doi.org/10.1016/j.jct.2008.06.019

9. 	Matos MAR, Sousa CCS, Morais VMF (2009) Thermochemical study of some methoxytetralones. J Chem Thermodyn 41:69–73. https://doi.org/10.1016/j.jct.2008.07.021

10. 	Database Quantum chemical calculations. Available online: https://http://gaussdb.samgtu.ru:8000/homepage/ (accessed on 15.09.2025)

11. 	Pedley JB, Naylor RD, Kirby SP (1986) Thermochemical data of organic compounds, Chapman and Hall, New York. 1–792

12. 	Silva ALR, Gama PMV, Ribeiro da Silva MDMC (2019) Influence of the functional groups −NH 2 , −OCH 3 , and −OH on the thermochemistry of indanes. Can J Chem 97:788–794. https://doi.org/10.1139/cjc-2019-0257

13. 	Sousa CCS, Morais VMF, Matos MAR (2010) Energetics of the isomers: 3- and 4-hydroxycoumarin. J Chem Thermodyn 42:1372–1378. https://doi.org/10.1016/j.jct.2010.06.003

14. 	Andreeva I V., Varfolomeev MA, Verevkin SP (2020) Thermochemistry of di-substituted benzenes: Ortho-, meta-, and para-hydroxyacetophenones. J Chem Thermodyn 140:105893. https://doi.org/10.1016/j.jct.2019.105893

15. 	Silva ALR, León GP, Ribeiro da Silva MDMC (2023) Energetic Effects in Methyl- and Methoxy-Substituted Indanones: A Synergistic Experimental and Computational Study. Appl Sci 13:10262. https://doi.org/10.3390/app131810262

16. 	Silva ALR, Lima ACMO, Ribeiro da Silva MDMC (2018) Energetic characterization of indanone derivatives involved in biomass degradation. J Therm Anal Calorim 134:1267–1276. https://doi.org/10.1007/s10973-018-7533-z

17. 	Sousa CCS, Morais VMF, Matos MAR (2013) Experimental and computational thermochemistry of 6,7-dihydro-4(5H)-benzofuranone. J Chem Thermodyn 56:83–88. https://doi.org/10.1016/j.jct.2012.07.008

18. 	Roux MV, Temprado M, Chickos JS, Nagano Y (2008) Critically evaluated thermochemical properties of polycyclic aromatic hydrocarbons. J Phys Chem Ref Data 37:1855–1996. https://doi.org/10.1063/1.2955570

19. 	Verevkin SP (2022) Weaving a web of reliable thermochemistry around lignin building blocks: phenol, benzaldehyde, and anisole. J Therm Anal Calorim 147:6073–6085. https://doi.org/10.1007/s10973-021-10924-x

20. 	Verevkin SP, Emel’yanenko VN (2008) Transpiration method: Vapor pressures and enthalpies of vaporization of some low-boiling esters. Fluid Phase Equilib 266:64–75. https://doi.org/10.1016/j.fluid.2008.02.001

21. 	Acree W, Chickos JS (2017) Phase transition enthalpy measurements of organic and organometallic compounds and ionic liquids. sublimation, vaporization, and fusion enthalpies from 1880 to 2015. part 2. C11-C192. J Phys Chem Ref Data 46:013104. https://doi.org/10.1063/1.4970519

22. 	Verevkin SP, Samarov AA, Vostrikov S V. (2024) Does the Oxygen Functionality Really Improve the Thermodynamics of Reversible Hydrogen Storage with Liquid Organic Hydrogen Carriers? Oxygen 4:266–284. https://doi.org/10.3390/oxygen4030015

23. 	Verevkin SP, Samarov AA, Vostrikov S V., et al (2023) Comprehensive Thermodynamic Study of Alkyl-Cyclohexanes as Liquid Organic Hydrogen Carriers Motifs. Hydrogen 4:42–59. https://doi.org/10.3390/hydrogen4010004

24. 	Steele W V, Chirico RD, Knipmeyer SE, Nguyen A (1996) Vapor Pressure of Acetophenone, (±)-1,2-Butanediol, (±)-1,3-Butanediol, Diethylene Glycol Monopropyl Ether, 1,3-Dimethyladamantane, 2-Ethoxyethyl Acetate, Ethyl Octyl Sulfide, and Pentyl Acetate. J Chem Eng Data 41:1255–1268. https://doi.org/10.1021/je9601117

25. 	Ambrose D, Ghiassee N. (1987) Vapour pressures and critical temperatures and critical pressures of C5 and C6 cyclic alcohols and ketones. J Chem Thermodyn 19:903–909. https://doi.org/10.1016/0021-9614(87)90036-X

26. 	Steele W V., Chirico RD, Knipmeyer SE, et al (1996) Thermodynamic Properties and Ideal-Gas Enthalpies of Formation for Butyl Vinyl Ether, 1,2-Dimethoxyethane, Methyl Glycolate, Bicyclo[2.2.1]hept-2-ene, 5-Vinylbicyclo[2.2.1]hept-2-ene, trans -Azobenzene, Butyl Acrylate, Di- tert -butyl Ether, and Hexane-1,. J Chem Eng Data 41:1285–1302. https://doi.org/10.1021/je960117w

27. 	Mokbel I, Rauzy E, Loiseleur H, et al (1995) Vapor pressures of 12 alkylcyclohexanes, cyclopentane, butylcyclopentane and trans-decahydronaphthalene down to 0.5 Pa. Experimental results, correlation and prediction by an equation of state. Fluid Phase Equilib 108:103–120. https://doi.org/10.1016/0378-3812(95)02707-L

28. 	SciFinder - Chemical Abstracts Service (2023) Available online: http://scifinder.cas.org/ (accessed on 15.05.2026)

29. 	Reaxys (2023) Available online: https://www.reaxys.com/ (accessed on 15.05.2026)

30. 	Samarov AA, Verevkin SP (2022) Comprehensive thermodynamic study of alkyl-biphenyls as a promising liquid organic hydrogen carriers. J Chem Thermodyn 174:106872. https://doi.org/10.1016/j.jct.2022.106872

31. 	Verevkin SP, Samarov AA, Vostrikov S V (2024) Thermodynamics of reversible hydrogen storage: Does alkoxy-substitution of naphthalene yield functional advantages for LOHC systems? J Chem Phys 160:154709. https://doi.org/10.1063/5.0200047

32. 	Dreisbach RR, Martin RA (1949) Physical Data on Some Organic Compounds. Ind Eng Chem 41:2875–2878. https://doi.org/10.1021/ie50480a053

33. 	Collerson RR, Counsell JF, Handley R, et al (1965) 677. Thermodynamic properties of organic oxygen compounds. Part XV. Purification and vapour pressures of some ketones and ethers. J Chem Soc 3697. https://doi.org/10.1039/jr9650003697

34. 	(2021) Guidechem Chemical Network. Available online: https://www.guidechem.com (accessed on 15.05.2026)

35. 	Lai W (1995) Temperature-programmed retention indices for g.c. and g.c.-m.s. analysis of coal- and petroleum-derived liquid fuels. Fuel 74:1436–1451. https://doi.org/10.1016/0016-2361(95)00108-H

36. 	Andon RJL, Biddiscombe DP, Cox JD, et al (1960) 1009. Thermodynamic properties of organic oxygen compounds. Part I. Preparation and physical properties of pure phenol, cresols, and xylenols. J Chem Soc 5246. https://doi.org/10.1039/jr9600005246

37. 	Verevkin SP (1999) Thermochemistry of phenols: buttress effects in sterically hindered phenols. J Chem Thermodyn 31:1397–1416. https://doi.org/10.1006/jcht.1999.0466

38. 	Walden P (1908) Über die Schmelzwärme, spezifische Kohäsion und Molekulargrösse bei der Schmelztemperatur. Zeitschrift für Elektrotechnik und Elektrochemie 14:713–724. https://doi.org/10.1002/bbpc.19080144302

39. 	Abdelaziz A, Zaitsau DH, Kuratieva N V., et al (2019) Melting of nucleobases. Getting the cutting edge of “Walden’s Rule.” Phys Chem Chem Phys 21:12787–12797. https://doi.org/10.1039/C9CP00716D

40. 	Verevkin SP, Konnova ME, Turovtsev V V., et al (2020) Weaving a Network of Reliable Thermochemistry around Lignin Building Blocks: Methoxy-Phenols and Methoxy-Benzaldehydes. Ind Eng Chem Res 59:22626–22639. https://doi.org/10.1021/acs.iecr.0c04281

41. 	Chickos JS, Hesse DG, Liebman JF (1992) Estimating Enthalpies of Sublimation of Hydrocarbons. In: Energetics of Organometallic Species. Springer Netherlands, Dordrecht, pp 159–169

42. 	Nagrimanov RN, Ziganshin MA, Solomonov BN, Verevkin SP (2019) Thermochemistry of drugs: experimental and theoretical study of analgesics. Struct Chem 30:247–261. https://doi.org/10.1007/s11224-018-1188-z

43. 	Verevkin SP, Emel’yanenko VN, Nagrimanov RN (2016) Nearest-Neighbor and Non-Nearest-Neighbor Interactions between Substituents in the Benzene Ring. Experimental and Theoretical Study of Functionally Substituted Benzamides. J Phys Chem A 120:9867–9877. https://doi.org/10.1021/acs.jpca.6b10332

44. 	Matos MAR, Miranda MS, Monte MJS, et al (2007) Calorimetric and Computational Study of Indanones. J Phys Chem A 111:11153–11159. https://doi.org/10.1021/jp074718d

45. 	Acree WE (1991) Thermodynamic properties of organic compounds: enthalpy of fusion and melting point temperature compilation. Thermochim Acta 189:37–56. https://doi.org/10.1016/0040-6031(91)87098-H

46. 	Wolf G (1972) Thermochemische Untersuchungen an cyclischen Ketonen. Helv Chim Acta 55:1446–1459. https://doi.org/10.1002/hlca.19720550510

47. 	Wiberg KB, Wasserman DJ, Martin EJ, Murcko MA (1985) Enthalpies of hydration of alkenes. 3. Cycloalkenes. J Am Chem Soc 107:6019–6022. https://doi.org/10.1021/ja00307a030

48. 	Verevkin SP (1998) Thermochemistry of Alcohols: Experimental Standard Molar Enthalpies of Formation and Strain of Some Alkyl and Phenyl Congested Alcohols. Struct Chem 9:375–382. https://doi.org/10.1023/A:1022471228499

49. 	Varfolomeev MA, Abaidullina DI, Solomonov BN, et al (2010) Pairwise Substitution Effects, Inter- and Intramolecular Hydrogen Bonds in Methoxyphenols and Dimethoxybenzenes. Thermochemistry, Calorimetry, and First-Principles Calculations. J Phys Chem B 114:16503–16516. https://doi.org/10.1021/jp108459r

50. 	Verevkin SP, Samarov AA, Vostrikov S V. (2023) Thermodynamics of Reversible Hydrogen Storage: Are Methoxy-Substituted Biphenyls Better through Oxygen Functionality? Hydrogen 4:862–880. https://doi.org/10.3390/hydrogen4040052

51. 	Hückel W, Friedrich H (1927) Zur Stereochemie bicyclischer Ringsysteme III: Die Stereoisomerie des Hydrindans und seiner Derivate I. Justus Liebigs Ann Chem 451:132–160. https://doi.org/10.1002/jlac.19274510106

52. 	Sellers P, Holm JL, Welch BJ, et al (1971) Enthalpies of Formation of cis- and trans-2-Hydrindanone. Acta Chem Scand 25:2189–2193. https://doi.org/10.3891/acta.chem.scand.25-2189

53. 	Sellers P, Gram F, Groth P, et al (1970) Enthalpies of Formation of cis- and trans-8-Methyl-2-hydrindanone. Acta Chem Scand 24:2453–2458. https://doi.org/10.3891/acta.chem.scand.24-2453

1. 	Petersson GA, Bennett A, Tensfeldt TG, Al‐Laham MA, Shirley WA, Mantzaris J (1988) A complete basis set model chemistry. I. The total energies of closed‐shell atoms and hydrides of the first‐row elements. J Chem Phys 89:2193–2218. https://doi.org/10.1063/1.455064

2. 	Pracht P, Bohle F, Grimme S (2020) Automated exploration of the low-energy chemical space with fast quantum chemical methods. Phys Chem Chem Phys 22:7169–7192. https://doi.org/10.1039/C9CP06869D

3. 	Frisch MJ, Trucks GW, Schlegel HB, Scuseria GE, Robb MA, Cheeseman JR, Scalmani G, Barone V, Petersson GA, Nakatsuji H, Li X, Caricato M, Marenich AV, Bloino J, Janesko BG, Gomperts R, Mennucci B, Hratchian HP, Ortiz JV, Izmaylov AF, Sonnenberg JL, Williams-Young D, Ding F, Lipparini F, Egidi F, Goings J, Peng B, Petrone A, Henderson T, Ranasinghe D, Zakrzewski VG, Gao J, Rega N, Zheng G, Liang W, Hada M, Ehara M, Toyota K, Fukuda R, Hasegawa J, Ishida M, Nakajima T, Honda Y, Kitao O, Nakai H, Vreven T, Throssell K, Montgomery JAJr, Peralta JE, Ogliaro F, Bearpark MJ, Heyd JJ, Brothers EN, Kudin KN, Staroverov VN, Keith TA, Kobayashi R, Normand J, Raghavachari K, Rendell AP, Burant JC, Iyengar SS, Tomasi J, Cossi M, Millam JM, Klene M, Adamo C, Cammi R, Ochterski JW, Martin RL, Morokuma K, Farkas O, Foresman JB, Fox DJ (2016) Gaussian 16, Revision C.01 Gaussian, Inc., Wallingford CT

4. 	Curtiss LA, Raghavachari K, Redfern PC, Pople JA (2000) Gaussian-3 theory using scaled energies. J Chem Phys 112:1125–1132. https://doi.org/10.1063/1.480668

5. 	Verevkin SP, Emel’yanenko VN, Notario R, Roux MV, Chickos JS, Liebman JF (2012) Rediscovering the Wheel. Thermochemical Analysis of Energetics of the Aromatic Diazines. J Phys Chem Lett 3:3454–3459. https://doi.org/10.1021/jz301524c

6. 	Siewert R, Samarov AA, Vostrikov SV, Müller K, Wasserscheid P, Verevkin SP (2025) Hydrogenation of Aromatic Ethers and Lactones: Does the Oxygen Functionality Really Improve the Thermodynamics of Reversible Hydrogen Storage in the Related LOHC Systems? Oxygen 5:18. https://doi.org/10.3390/oxygen5030018

7. 	Verevkin SP (1998) Thermochemistry of aromatic ketones. Experimental enthalpies of formation and structural effects. Thermochim Acta 310:229–235. https://doi.org/10.1016/s0040-6031(97)00231-1

8. 	Matos MAR, Sousa CCS, Morais VMF (2008) Experimental and theoretical thermochemistry of β-tetralone. J Chem Thermodyn 40:1552–1557. https://doi.org/10.1016/j.jct.2008.06.019

9. 	Matos MAR, Sousa CCS, Morais VMF (2009) Thermochemical study of some methoxytetralones. J Chem Thermodyn 41:69–73. https://doi.org/10.1016/j.jct.2008.07.021

10. 	Database Quantum chemical calculations. Available online: https://http://gaussdb.samgtu.ru:8000/homepage/ (accessed on 15.09.2025)

11. 	Pedley JB, Naylor RD, Kirby SP (1986) Thermochemical data of organic compounds, Chapman and Hall, New York. 1–792

12. 	Silva ALR, Gama PMV, Ribeiro da Silva MDMC (2019) Influence of the functional groups −NH 2 , −OCH 3 , and −OH on the thermochemistry of indanes. Can J Chem 97:788–794. https://doi.org/10.1139/cjc-2019-0257

13. 	Sousa CCS, Morais VMF, Matos MAR (2010) Energetics of the isomers: 3- and 4-hydroxycoumarin. J Chem Thermodyn 42:1372–1378. https://doi.org/10.1016/j.jct.2010.06.003

14. 	Andreeva IV, Varfolomeev MA, Verevkin SP (2020) Thermochemistry of di-substituted benzenes: Ortho-, meta-, and para-hydroxyacetophenones. J Chem Thermodyn 140:105893. https://doi.org/10.1016/j.jct.2019.105893

15. 	Silva ALR, León GP, Ribeiro da Silva MDMC (2023) Energetic Effects in Methyl- and Methoxy-Substituted Indanones: A Synergistic Experimental and Computational Study. Appl Sci 13:10262. https://doi.org/10.3390/app131810262

16. 	Silva ALR, Lima ACMO, Ribeiro da Silva MDMC (2018) Energetic characterization of indanone derivatives involved in biomass degradation. J Therm Anal Calorim 134:1267–1276. https://doi.org/10.1007/s10973-018-7533-z

17. 	Sousa CCS, Morais VMF, Matos MAR (2013) Experimental and computational thermochemistry of 6,7-dihydro-4(5H)-benzofuranone. J Chem Thermodyn 56:83–88. https://doi.org/10.1016/j.jct.2012.07.008

18. 	Roux MV, Temprado M, Chickos JS, Nagano Y (2008) Critically evaluated thermochemical properties of polycyclic aromatic hydrocarbons. J Phys Chem Ref Data 37:1855–1996. https://doi.org/10.1063/1.2955570

19. 	Verevkin SP (2022) Weaving a web of reliable thermochemistry around lignin building blocks: phenol, benzaldehyde, and anisole. J Therm Anal Calorim 147:6073–6085. https://doi.org/10.1007/s10973-021-10924-x

20. 	Verevkin SP, Emel’yanenko VN (2008) Transpiration method: Vapor pressures and enthalpies of vaporization of some low-boiling esters. Fluid Phase Equilib 266:64–75. https://doi.org/10.1016/j.fluid.2008.02.001

21. 	Acree W, Chickos JS (2017) Phase transition enthalpy measurements of organic and organometallic compounds and ionic liquids. sublimation, vaporization, and fusion enthalpies from 1880 to 2015. part 2. C11-C192. J Phys Chem Ref Data 46:013104. https://doi.org/10.1063/1.4970519

22. 	Verevkin SP, Samarov AA, Vostrikov S V. (2024) Does the Oxygen Functionality Really Improve the Thermodynamics of Reversible Hydrogen Storage with Liquid Organic Hydrogen Carriers? Oxygen 4:266–284. https://doi.org/10.3390/oxygen4030015

23. 	Verevkin SP, Samarov AA, Vostrikov SV, Wasserscheid P, Müller K (2023) Comprehensive Thermodynamic Study of Alkyl-Cyclohexanes as Liquid Organic Hydrogen Carriers Motifs. Hydrogen 4:42–59. https://doi.org/10.3390/hydrogen4010004

24. 	Steele W V, Chirico RD, Knipmeyer SE, Nguyen A (1996) Vapor Pressure of Acetophenone, (±)-1,2-Butanediol, (±)-1,3-Butanediol, Diethylene Glycol Monopropyl Ether, 1,3-Dimethyladamantane, 2-Ethoxyethyl Acetate, Ethyl Octyl Sulfide, and Pentyl Acetate. J Chem Eng Data 41:1255–1268. https://doi.org/10.1021/je9601117

25. 	Ambrose D, Ghiassee N. (1987) Vapour pressures and critical temperatures and critical pressures of C5 and C6 cyclic alcohols and ketones. J Chem Thermodyn 19:903–909. https://doi.org/10.1016/0021-9614(87)90036-X

26. 	Steele WV, Chirico RD, Knipmeyer SE, Nguyen A, Smith NK (1996) Thermodynamic Properties and Ideal-Gas Enthalpies of Formation for Butyl Vinyl Ether, 1,2-Dimethoxyethane, Methyl Glycolate, Bicyclo[2.2.1]hept-2-ene, 5-Vinylbicyclo[2.2.1]hept-2-ene, trans -Azobenzene, Butyl Acrylate, Di- tert -butyl Ether, and Hexane-1,. J Chem Eng Data 41:1285–1302. https://doi.org/10.1021/je960117w

27. 	Mokbel I, Rauzy E, Loiseleur H, Berro C, Jose J (1995) Vapor pressures of 12 alkylcyclohexanes, cyclopentane, butylcyclopentane and trans-decahydronaphthalene down to 0.5 Pa. Experimental results, correlation and prediction by an equation of state. Fluid Phase Equilib 108:103–120. https://doi.org/10.1016/0378-3812(95)02707-L

28. 	SciFinder - Chemical Abstracts Service (2023) Available online: http://scifinder.cas.org/ (accessed on 15.05.2026)

29. 	Reaxys (2023) Available online: https://www.reaxys.com/ (accessed on 15.05.2026)

30. 	Samarov AA, Verevkin SP (2022) Comprehensive thermodynamic study of alkyl-biphenyls as a promising liquid organic hydrogen carriers. J Chem Thermodyn 174:106872. https://doi.org/10.1016/j.jct.2022.106872

31. 	Verevkin SP, Samarov AA, Vostrikov SV (2024) Thermodynamics of reversible hydrogen storage: Does alkoxy-substitution of naphthalene yield functional advantages for LOHC systems? J Chem Phys 160:154709. https://doi.org/10.1063/5.0200047

32. 	Dreisbach RR, Martin RA (1949) Physical Data on Some Organic Compounds. Ind Eng Chem 41:2875–2878. https://doi.org/10.1021/ie50480a053

33. 	Collerson RR, Counsell JF, Handley R, Martin JF, Sprake CHS (1965) 677. Thermodynamic properties of organic oxygen compounds. Part XV. Purification and vapour pressures of some ketones and ethers. J Chem Soc 3697. https://doi.org/10.1039/jr9650003697

34. 	Guidechem Chemical Network (2021). Available online: https://www.guidechem.com (accessed on 15.05.2026)

35. 	Lai W (1995) Temperature-programmed retention indices for g.c. and g.c.-m.s. analysis of coal- and petroleum-derived liquid fuels. Fuel 74:1436–1451. https://doi.org/10.1016/0016-2361(95)00108-H

36. 	Andon RJL, Biddiscombe DP, Cox JD, Handley R, Harrop D, Herington EFG, Martin JF (1960) 1009. Thermodynamic properties of organic oxygen compounds. Part I. Preparation and physical properties of pure phenol, cresols, and xylenols. J Chem Soc 5246. https://doi.org/10.1039/jr9600005246

37. 	Verevkin SP (1999) Thermochemistry of phenols: buttress effects in sterically hindered phenols. J Chem Thermodyn 31:1397–1416. https://doi.org/10.1006/jcht.1999.0466

38. 	Walden P (1908) Über die Schmelzwärme, spezifische Kohäsion und Molekulargrösse bei der Schmelztemperatur. Zeitschrift für Elektrotechnik und Elektrochemie 14:713–724. https://doi.org/10.1002/bbpc.19080144302

39. 	Abdelaziz A, Zaitsau DH, Kuratieva NV, Verevkin SP, Schick C (2019) Melting of nucleobases. Getting the cutting edge of “Walden’s Rule.” Phys Chem Chem Phys 21:12787–12797. https://doi.org/10.1039/C9CP00716D

40. 	Verevkin SP, Konnova ME, Turovtsev VV, Riabchunova AV, Pimerzin AA (2020) Weaving a Network of Reliable Thermochemistry around Lignin Building Blocks: Methoxy-Phenols and Methoxy-Benzaldehydes. Ind Eng Chem Res 59:22626–22639. https://doi.org/10.1021/acs.iecr.0c04281

41. 	Chickos JS, Hesse DG, Liebman JF (1992) Estimating Enthalpies of Sublimation of Hydrocarbons. In: Energetics of Organometallic Species. Springer Netherlands, Dordrecht, pp 159–169

42. 	Nagrimanov RN, Ziganshin MA, Solomonov BN, Verevkin SP (2019) Thermochemistry of drugs: experimental and theoretical study of analgesics. Struct Chem 30:247–261. https://doi.org/10.1007/s11224-018-1188-z

43. 	Verevkin SP, Emel’yanenko VN, Nagrimanov RN (2016) Nearest-Neighbor and Non-Nearest-Neighbor Interactions between Substituents in the Benzene Ring. Experimental and Theoretical Study of Functionally Substituted Benzamides. J Phys Chem A 120:9867–9877. https://doi.org/10.1021/acs.jpca.6b10332

44. 	Matos MAR, Miranda MS, Monte MJS, Santos LMNBF, Morais VMF, Chickos JS, Umnahanant P, Liebman JF (2007) Calorimetric and Computational Study of Indanones. J Phys Chem A 111:11153–11159. https://doi.org/10.1021/jp074718d

45. 	Acree WE (1991) Thermodynamic properties of organic compounds: enthalpy of fusion and melting point temperature compilation. Thermochim Acta 189:37–56. https://doi.org/10.1016/0040-6031(91)87098-H

46. 	Wolf G (1972) Thermochemische Untersuchungen an cyclischen Ketonen. Helv Chim Acta 55:1446–1459. https://doi.org/10.1002/hlca.19720550510

47. 	Wiberg KB, Wasserman DJ, Martin EJ, Murcko MA (1985) Enthalpies of hydration of alkenes. 3. Cycloalkenes. J Am Chem Soc 107:6019–6022. https://doi.org/10.1021/ja00307a030

48. 	Verevkin SP (1998) Thermochemistry of Alcohols: Experimental Standard Molar Enthalpies of Formation and Strain of Some Alkyl and Phenyl Congested Alcohols. Struct Chem 9:375–382. https://doi.org/10.1023/A:1022471228499

49. 	Varfolomeev MA, Abaidullina DI, Solomonov BN, Verevkin SP, Emel’yanenko VN (2010) Pairwise Substitution Effects, Inter- and Intramolecular Hydrogen Bonds in Methoxyphenols and Dimethoxybenzenes. Thermochemistry, Calorimetry, and First-Principles Calculations. J Phys Chem B 114:16503–16516. https://doi.org/10.1021/jp108459r

50. 	Verevkin SP, Samarov AA, Vostrikov S V. (2023) Thermodynamics of Reversible Hydrogen Storage: Are Methoxy-Substituted Biphenyls Better through Oxygen Functionality? Hydrogen 4:862–880. https://doi.org/10.3390/hydrogen4040052

51. 	Hückel W, Friedrich H (1927) Zur Stereochemie bicyclischer Ringsysteme III: Die Stereoisomerie des Hydrindans und seiner Derivate I. Justus Liebigs Ann Chem 451:132–160. https://doi.org/10.1002/jlac.19274510106

52. 	Sellers P, Holm JL, Welch BJ, Prelesnik A, Zupancic I, Ehrenberg L (1971) Enthalpies of Formation of cis- and trans-2-Hydrindanone. Acta Chem Scand 25:2189–2193. https://doi.org/10.3891/acta.chem.scand.25-2189

53. 	Sellers P, Gram F, Groth P, Klewe B, Rømming C (1970) Enthalpies of Formation of cis- and trans-8-Methyl-2-hydrindanone. Acta Chem Scand 24:2453–2458. https://doi.org/10.3891/acta.chem.scand.24-2453