Sequential Fenton reaction and biodegradation for treating decabromodiphenyl ether in soil slurries

Cheng-Chun Lin, Chun-Wen Yang, Po-Hsin Yuan and Yi-Tang Chang *

Supplementary information A

Department of Microbiology

Soochow University

70, Linhsi Rd., Shinlin District

Taipei, 11102

Taiwan

*Corresponding author:

Distinguished Professor

Tel: +886-2-28819471-6862

FAX: +886-2-28831193

E-mail: ytchang@scu.edu.tw; timchang08@gmail.com

Table S1. Parameters of TCS measured during the experimental period

| Composition (%) |  |  | pH | BET-(N2) SA (m2 g−1) | SOM (%) | CEC (meq/100 g) |
| --- | --- | --- | --- | --- | --- | --- |
| Sand | Slit | Clay |  |  |  |  |
| 10 | 54 | 36 | 8.03 | 5.21 | 1.883 | 1.37 |

Table S2. Composition of iron oxides in TCS

| Free iron oxides (mg g-1) | Amorphous iron oxides (mg g-1) | Crystalline iron oxides (mg g-1)1 |
| --- | --- | --- |
| 5.84 | 2.59 | 3.25 |

1Crystalline iron oxides = Free iron oxides − Amorphous iron oxides is satisfied (Sun et al., 2024). Therefore, the following analysis focuses on free iron oxides and amorphous iron oxides.

Experimental procedure for analyzing free and amorphous iron oxides in soil

The method for analyzing free iron oxides in soil involves weighing 2.0 g of soil, adding 100 mL of 0.2 M acidic oxalic acid–ammonium oxalate solution, and shaking the mixture in the dark for 4 h. Subsequently, a few drops of 0.4% Superfloc® are added, the suspension is vigorously shaken for 10 s, allowed to stand for 30 min, centrifuged at 1164 × g for 15 min, and the resulting supernatant is filtered through Whatman No. 42 filter paper before being analyzed by atomic absorption spectrometry.

The method for analyzing amorphous iron oxides in soil involves accurately weighing 1.0 g of soil, adding 40 mL of 0.3 M sodium citrate solution and 5 mL of 1 M sodium bicarbonate solution, and then placing the mixture in an 80°C water bath for approximately 15–20 min. Subsequently, 1 g of sodium dithionite is added and stirred for approximately 15 min, followed by the addition of 10 mL of saturated sodium chloride solution, centrifugation at 3,500 ×g for 15 min, and collection of the suspension, with these steps repeated until the soil turns gray. Finally, the suspension is diluted to 100 mL and analyzed using atomic absorption spectrometry at 372 nm.

References:

Sun, Z., Liu, S., Jiang, Y., 2024. Iron composition of a typical Loess-Paleosol sequence in Northeast China. Agronomy, 14, 1333. https://doi.org/10.3390/agronomy14061333

Table S3. Removal rate of BDE‐209 following the Fenton reaction

| Sample | Initial BDE‐209 (mg kg-1) | Final BDE‐209 (mg kg-1) |
| --- | --- | --- |
| Blank | 20.21±0.86 | 19.16 ± 0.00 |
| Fenton processed | 20.21±0.86 | 11.32 ± 1.52 |

Table S4. Design of a single carbon source for the MT2 microplate

| A1 MSB | A2 BDE‐99 | A3 3′‐OH‐BDE‐7 |
| --- | --- | --- |
| B1 Hexane (negative control) | B2 BDE‐100 | B3 3′‐OH‐BDE‐28 |
| C1 Glucose (positive control) | C2 BDE‐153 | C3 3′‐OH‐BDE‐47 |
| D1 Biphenyl ether | D2 BDE‐154 | D3 6‐OH‐BDE‐180 |
| E1 BDE‐3 | E2 BDE‐209 | E3 2′‐MeO‐BDE‐3 |
| F1 BDE‐28 | F2 5Br mix1 | F3 3′‐MeO‐BDE‐7 |
| G1 BDE‐47 | G2 PBDE mix2 | G3 3′‐MeO‐BDE‐28 |
| H1 BDE‐66 | H2 2′‐OH‐BDE‐3 | H3 3′‐MeO‐BDE‐47 |

1BDE‐47, BDE‐99, BDE‐100, BDE‐85, and BDE‐119 30 ppm each

2BDE‐3, BDE‐28, BDE‐47, BDE‐66, BDE‐99, BDE‐100, BDE‐153, BDE‐154, and BDE‐209 30 ppm each.

Table S5. Colony and microscopic characteristics of isolated strains in the FB group (scale bar = 10 μm)

| Strain no. | Colony characteristics |  |  |  |  |  |  | Microscopy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Graph | Shape | Size (mm) | Surface | Pigment | Margin | Transparency | Graph | Gram stain | Shape |
| B2 |  | Oval | 2.0 | Rough | White | Undulate | Slightly translucent |  | G− | Rod |
| P5 |  | Circular | 1.0 | Smooth | Yellow | Smooth | Opaque |  | G+ | Rod |
| D1 |  | Oval | 2.0 | Smooth | White | Undulate | Slightly translucent |  | G− | Cocci |
| D2 |  | Circular | 2.0 | Smooth | Beige | Smooth | Opaque |  | G+ | Rod |
| D3 |  | Circular | 2.0 | Smooth | White | Smooth | Opaque |  | G− | Short rod |

| D4 |  | Oval | 3.0 | Smooth | White | Undulate | Slightly translucent |  | G− | Cocci |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D5 |  | Circular | 1.0 | Smooth | White | Smooth | Opaque |  | G+ | Cocci |
| D6 |  | Circular | 3.0 | Smooth | White | Smooth | Opaque |  | G− | Rod |
| D7 |  | Circular | 1.0 | Smooth | Yellow | Smooth | Opaque |  | G− | Rod |
| D8 |  | Circular | 1.0 | Smooth | Beige | Smooth | Opaque |  | G+ | Cocci |

Table S3. Colony and microscopic characteristics of isolated strains in the FB group. (Continued)

Fig. S1. Analysis of ion concentrations (Br−, NO3−) following sequential Fenton oxidation and biodegradation.

Fig. S2. Number of clones and operational taxonomic units (OTUs) in the Da‐an chemostat.

Fig. S3. Number of clones and operational taxonomic units (OTUs) in the FB and D11 groups (the first number in the legend indicates degradation time; “B” in the legend represents the FB group; “D” in the legend represents the D11 group).