Supplementary Information

# S1. Additional TG–DTG processing equations

## S1.1. Linear heating and TG normalisation

The programmed non-isothermal heating rate was defined as follows (Tu et al., 2019):

|  | β = dTdt | (S1) |
| --- | --- | --- |

Each TG curve was normalised independently to its first valid paired temperature–mass datum using the expression reported by Conesa and Domene (2011):

|  | w = mm0 | (S2) |
| --- | --- | --- |

Here, w(T) is the dimensionless mass fraction, m(T) is the instantaneous mass, and m₀ is the first valid mass of the corresponding run. Percentage mass remaining was reported as 100w.

## S1.2. DTG definitions and stage-wise mass loss

To distinguish the derivative per unit time from the derivative per unit temperature, the following operational definitions were used:

|  | DTGt(T) = -d[100w(T)]dt | (S3a) |
| --- | --- | --- |
|  | DTGT(T) = -d[100w(T)]dT | (S3b) |
|  | DTGt(T) = βDTGT(T) | (S3c) |

DTGₜ is expressed in % min⁻¹ and DTGT in % °C⁻¹. The supplied rosehip DTG data were verified as time-based rates; therefore, reported peak heights were not divided by β.

For the diagnostic profile at 20 °C min⁻¹, the mass losses in the study-defined intervals ending at 150, 400, and 700 °C, together with the residue at 700 °C, were calculated by direct subtraction of cumulative mass-loss values:

|  | ΔWI = W(150 °C) - W(T0) | (S4a) |
| --- | --- | --- |
|  | ΔWII = W(400 °C) - W(150 °C) | (S4b) |
|  | ΔWIII = W(700 °C) - W(400 °C) | (S4c) |
|  | R700 = 100% - W(700 °C) | (S4d) |

In Eqs. (S4a)–(S4d), T₀ is the first valid temperature, ΔW is the interval mass loss, and R₇₀₀ is the mass remaining at 700 °C. The analytical boundaries follow the diagnostic comparison used for fractionated food-processing residues and are not universal constituent boundaries (Soomro et al., 2026).

## S1.3. Interpolation and non-isothermal conversion rate

Characteristic mass-loss temperatures and conversion temperatures were obtained from the first pair of measurements bracketing the target by two-point linear interpolation (Wang et al., 2026; de Castro et al., 2026):

|  | T_C = T₁ + P_S - P₁P₂ - P₁(T₂ - T₁) | (S5) |
| --- | --- | --- |

The non-isothermal conversion-rate relationship was written as reported by Liu et al. (2019):

|  | dαdt = (dαdT)(dTdt) = βdαdT | (S6) |
| --- | --- | --- |

# S2. Additional DTG peak-domain and Gaussian diagnostics

## S2.1. Across-rate variability and conditional Kissinger analysis

Descriptive dispersion across the four programmed heating rates was summarised using the coefficient of variation (Mishra et al., 2019), with thermal-analysis precedent provided by Müller et al. (2020):

|  | CV(%) = 100 sxx | (S7) |
| --- | --- | --- |

The coefficient of variation was used only as a descriptive measure because heating rates were experimental conditions rather than replicate measurements.

For a persistent peak domain represented unambiguously at all four heating rates, the secondary Kissinger peak-shift diagnostic was evaluated using the following relation (Kissinger, 1956):

|  | ln(βTp2) = -EK,pR Tp + C | (S8) |
| --- | --- | --- |

The slope of ln(β/Tₚ²) against 1/Tₚ equals −E_K,p/R. The estimate was retained only when peak identity and ordering were preserved and R² ≥ 0.95; it was not treated as evidence of single-step kinetics (Vyazovkin, 2020).

## S2.2. Gaussian area, width, residuals, and information criteria

For the area-normalised Gaussian parameterisation used in the conditional DTG decomposition, the component-area identity was (Singh et al., 2023):

|  | ∫-∞∞ Gj(T) dT = Aj | (S9) |
| --- | --- | --- |

When the fitted ordinate was DTGₜ, Aⱼ retained units of %·°C·min⁻¹ and was not interpreted directly as percentage mass loss.

For a retained symmetric Gaussian component, full width at half maximum was calculated as (Li et al., 1999):

|  | FWHMj = 2(2 ln 2)1/2 σj | (S10) |
| --- | --- | --- |

Pointwise DTG residuals were defined as observed minus fitted values at the same temperature (Kim, 2019):

|  | ei = yi - ŷi | (S11) |
| --- | --- | --- |

Candidate Gaussian component numbers were compared using the finite-sample corrected Akaike information criterion and the Bayesian information criterion in the locked least-squares forms (Burnham and Anderson, 2004):

|  | AICc = n ln(RSSn) + 2k + 2k(k+1)n-k-1 | (S12) |
| --- | --- | --- |
|  | BIC = n ln(RSSn) + k ln n | (S13) |

# S3. Supporting distributed-reactivity equations

## S3.1. Model notation, active-interval normalisation, and activation-energy density

For a standalone specification of the distributed-reactivity calculations, the feedstock index, complete heating-rate set, parameter vector, model-start temperature, parameter constraints, and ordered distribution centres were defined as follows:

r∈{seed, skin, whole fruit}.

B={5,10,15,20} K min-1.

θr,K=Arj,μrj,σrj,crjj=1K.

T0=423.15 K=150∘C.

Arj>0,  σrj>0,  crj≥0,  j=1Kcrj≤1,

μr1<μr2<⋯<μrK.

These are study-defined model specifications. They identify the fitted parameter space and prevent label switching; they do not assign the formal domains to anatomical tissues or individual biomass constituents.

Within the distributed-reactivity analysis only, measured mass was normalised to the value at the 150 °C model boundary so that the observed and predicted trajectories began from unity. This active-interval adaptation follows the post-drying normalisation principle described by Várhegyi et al. (2011):

|  | mr,βobsT=Mr,βTMr,βT0. | (S14) |
| --- | --- | --- |

The activation-energy distribution within each formal reactivity domain was represented by the locked non-negative normalised Gaussian density, based on the Gaussian form reported by Mani et al. (2009) and the distributed-energy formulation of Várhegyi et al. (2011):

|  | DrjE=exp​-E-μrj22σrj20∞exp​-u-μrj22σrj2du, E≥0. 0∞DrjE dE=1. | (S15) |
| --- | --- | --- |

The fitted distributions describe formal regions of apparent thermal reactivity and were not interpreted as anatomical or chemical fractions.

## S3.2. Predicted DTG and fit diagnostics

Predicted DTG was derived from the fitted normalised-mass trajectory as a positive time-based percentage mass-loss rate (Várhegyi et al., 2011):

|  | DTGr,βT=-100 β dmr,βTdT. | (S16) |
| --- | --- | --- |

Predicted DTG was used as a derivative-based diagnostic after fitting TG and was not assigned equal weight in the primary optimisation.

Global fit quality was reported as the root-mean-square discrepancy in normalised-mass percentage points using the structurally adapted metric based on Várhegyi et al. (2011):

|  | RMSEr,Kglobal=100Jr,K​θr,K. RMSEr,Kglobal=1001Bfitβ∈Bfit​1Nr,βi=1Nr,βmr,βobsTi-mr,β​Ti;θr,K21/2. | (S17) |
| --- | --- | --- |

Signed pointwise TG residuals were retained to identify temperature-dependent underprediction and overprediction. The residual identity follows the observed-minus-predicted difference contained in the global fitting framework of Várhegyi et al. (2011):

|  | er,β,KglobalTi=100mr,βobsTi-mr,β​Ti;θr,K. | (S18) |
| --- | --- | --- |

Residuals were interpreted as deterministic model–data discrepancies rather than replicate errors.

# S4. Supporting equations for analytical-support-screened isoconversional analysis

The four activation-energy estimators are reported in the main methodology. The equations below document the secondary calculations used to quantify regression precision, agreement among methods, and pointwise regression diagnostics.

## S4.1. Regression-slope uncertainty and activation-energy precision

For each conversion level α and isoconversional method, the ordinary-least-squares standard error of the fitted slope was calculated from the regression residuals (Kim, 2019):

|  | SEmα=i=1nyα,i-yα,i2n-2i=1nxα,i-xα2 | (S19) |
| --- | --- | --- |

The slope uncertainty was propagated to the apparent activation-energy standard error using the method-specific slope factors. These factors follow Friedman Eq. (5), KAS Eq. (10), FWO Eq. (27), and Starink Eq. (29) in Starink (2003):

|  | SEEa,α=R SEmα,Friedman and KASR SEmα1.0518,FWOR SEmα1.0008,Starink | (S20) |
| --- | --- | --- |

Pointwise 95% confidence intervals were calculated as:

|  | CI95%Ea,α=Ea,α±t0.975,n-2 SEEa,α | (S21) |
| --- | --- | --- |

Here, n is the number of heating rates used in the fixed-conversion regression. For the present four-rate analysis, n = 4 and the regression degrees of freedom were n − 2 = 2. The error bars in the principal Eₐ(α) figure represent ±SE(Eₐ), whereas the wider 95% confidence intervals are reported in the supplementary tables.

## S4.2. Integral-method agreement and regression diagnostics

Agreement among KAS, FWO, and Starink at each conversion level was summarised using the integral-method mean, sample standard deviation, and coefficient of variation (Mishra et al., 2019; Müller et al., 2020):

|  | Ea,α,int=Ea,α,KAS+Ea,α,FWO+Ea,α,St3 | (S22) |
| --- | --- | --- |
|  | SDa,α,int=j=13Ea,α,j-Ea,α,int23-1 | (S23) |
|  | CVa,α,int=100SDa,α,intEa,α,int | (S24) |

The relative deviation of Friedman from the integral-method mean was calculated as:

|  | Dα,Fr=100Ea,α,Fr-Ea,α,intEa,α,int | (S25) |
| --- | --- | --- |

For descriptive cross-method reporting only, the four-method mean was calculated as:

|  | Ea,α,4=Ea,α,Fr+Ea,α,KAS+Ea,α,FWO+Ea,α,St4 | (S26) |
| --- | --- | --- |

Regression RMSE and the maximum absolute regression residual were retained as pointwise linearity diagnostics (Kim, 2019):

|  | RMSEα,M=1ni=1nyα,i,M-yα,i,M2 | (S27) |
| --- | --- | --- |
|  | emax,α,M=maxiyα,i,M-yα,i,M | (S28) |

# S5. Supporting equations for endpoint-sensitivity analysis

## S5.1. Endpoint-sensitivity equations

Fixed-conversion temperature by first-bracket interpolation

For each target conversion, the first adjacent observations satisfying α1,βTf≤α≤α2,βTf were identified, and the endpoint-conditioned fixed-conversion temperature was calculated as

| Tα,βTf=T1+α-α1,βTfα2,βTf-α1,βTfT2-T1. | (S29) |
| --- | --- |

where T1 and T2 are the temperatures bracketing the target conversion. Equation (S29) is the standard two-point linear-interpolation relation. The first forward bracket was used, and extrapolation was not permitted.

Study-defined endpoint-sensitivity diagnostics

The signed change in apparent activation energy relative to the adopted 700 °C basis was calculated as

| ΔEa,kTfα=Ea,kTfα-Ea,k700α. | (S30) |
| --- | --- |

The corresponding absolute relative deviation was calculated as

| δEa,kTfα=100Ea,kTfα-Ea,k700αEa,k700α. | (S31) |
| --- | --- |

The complete pointwise spread across the three terminal-temperature definitions was calculated as

| RE,kα=maxTf∈{600,650,700}Ea,kTfα-minTf∈{600,650,700}Ea,kTfα. | (S32) |
| --- | --- |

In Eqs. (S30)–(S32), k denotes Friedman, KAS, FWO, Starink, or the integral-method mean. Positive ΔEa values indicate estimates above the adopted 700 °C result, whereas δEa reports only the magnitude of the relative deviation and is expressed as a percentage.

Endpoint mass retention and cross-heating-rate spread

Normalized mass remaining at a selected endpoint was obtained by extending the manuscript definition of normalized residual mass:

| RTf,β=100wβTf=100mβTfm0,β. | (S33) |
| --- | --- |

The cross-heating-rate spread in endpoint mass retention was then defined as

| SRTf=maxβ∈{5,10,15,20}RTf,β-minβ∈{5,10,15,20}RTf,β. | (S34) |
| --- | --- |

Here, m0,β is the first valid mass of the corresponding run, and wβT is the normalized mass fraction already defined in the manuscript. Equation (S33) generalizes R700,β to the selected endpoints of 600 and 650 °C, and Eq. (S34) is a study-defined range descriptor over β∈{5,10,15,20} °C min⁻¹.

Principal-window endpoint summary

For feedstock f and endpoint Tf, the descriptive mean apparent activation energy over the retained conversion set was calculated as

| Eaf,AfTf=1Nfα∈AfEa,fTfα, Nf=Af. | (S35) |
| --- | --- |

where Af is the preparation-specific retained conversion set, Nf is the number of included conversion levels, and Ea,fTfα is the arithmetic mean of the KAS, FWO, and Starink estimates at the same conversion. The discrete retained sets were Aseed={0.10,0.20,0.30,0.40,0.50,0.60,0.70}, Askin={0.40,0.50,0.60}, and Awhole={0.30,0.40,0.50,0.60}. All conversion levels α=0.10,0.20,…,0.90 were nevertheless recalculated and are reported for transparency.

# S6. Supplementary information for Section 3.1.1: TG–DTG behaviour

## S6.1. Supplementary tables S1–S9

Table S1. Characteristic mass-loss temperatures and principal DTG descriptors for rosehip feedstocks at 5–20 °C min⁻¹

| Feedstock | β (°C min⁻¹) | T₅% (°C) | T₁₀% (°C) | T₅₀% (°C) | Tₘₐₓ (°C) | DTGₘₐₓ (% min⁻¹) | Resolved peaks (n) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 180.3 | 231.2 | 342.3 | 332.0 | 3.598 | 1 |
| Rosehip seed | 10 | 204.4 | 253.4 | 354.6 | 346.0 | 7.397 | 2 |
| Rosehip seed | 15 | 213.6 | 257.6 | 363.4 | 354.0 | 10.310 | 1 |
| Rosehip seed | 20 | 228.0 | 268.9 | 368.3 | 360.0 | 14.060 | 1 |
| Rosehip skin | 5 | 141.8 | 165.5 | 351.4 | 173.0 | 1.516 | 2 |
| Rosehip skin | 10 | 153.8 | 180.3 | 368.1 | 209.0 | 2.840 | 3 |
| Rosehip skin | 15 | 162.6 | 185.7 | 372.1 | 198.0 | 5.187 | 2 |
| Rosehip skin | 20 | 175.1 | 197.9 | 372.3 | 223.0 | 6.975 | 3 |
| Whole-fruit rosehip | 5 | 116.0 | 164.5 | 338.6 | 334.0 | 1.886 | 2 |
| Whole-fruit rosehip | 10 | 128.3 | 178.8 | 349.7 | 341.0 | 3.640 | 2 |
| Whole-fruit rosehip | 15 | 133.4 | 189.9 | 357.9 | 352.0 | 5.966 | 2 |
| Whole-fruit rosehip | 20 | 136.9 | 193.9 | 355.6 | 351.0 | 7.871 | 2 |

Note. T₅%, T₁₀%, and T₅₀% are temperatures at 5%, 10%, and 50% cumulative loss of the initial mass, obtained by first-bracket linear interpolation. Tₘₐₓ is the raw experimental temperature of the largest eligible DTGₜ maximum. DTGₘₐₓ is reported in % min⁻¹.

Table S2. Stage-wise mass loss and residue at 700 °C for the diagnostic comparison at 20 °C min⁻¹

| Feedstock | Stage I loss (%) | Stage II loss (%) | Stage III loss (%) | R₇₀₀ (%) | Bookkeeping closure (%) | Closure error (pp) |
| --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 2.03 | 56.85 | 18.21 | 22.91 | 100.00 | 0.000 |
| Rosehip skin | 1.78 | 50.95 | 17.30 | 29.97 | 100.00 | 0.000 |
| Whole-fruit rosehip | 5.70 | 53.27 | 20.10 | 20.93 | 100.00 | 0.000 |

Note. Stage boundaries are study-defined diagnostic intervals: first valid temperature–150 °C, 150–400 °C, and 400–700 °C. Closure was calculated as Stage I + Stage II + Stage III + R₇₀₀. Exact closure is a bookkeeping identity obtained from partitioning the same normalised curve; it is not an independent test of calibration or reproducibility.

Table S3. Resolved principal and secondary DTG peaks across feedstocks and heating rates

| Feedstock | β (°C min⁻¹) | Peak type | Peak T (°C) | Raw DTGₜ (% min⁻¹) | Smoothed DTGₜ (% min⁻¹) | Prominence (% min⁻¹) |
| --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | Principal | 332.0 | 3.598 | 3.551 | 3.449 |
| Rosehip seed | 10 | Secondary | 278.0 | 3.614 | 3.614 | 0.292 |
| Rosehip seed | 10 | Principal | 346.0 | 7.397 | 7.328 | 7.111 |
| Rosehip seed | 15 | Principal | 354.0 | 10.310 | 10.233 | 9.858 |
| Rosehip seed | 20 | Principal | 360.0 | 14.060 | 13.992 | 13.476 |
| Rosehip skin | 5 | Principal | 173.0 | 1.516 | 1.512 | 1.343 |
| Rosehip skin | 5 | Secondary | 310.0 | 1.440 | 1.432 | 0.505 |
| Rosehip skin | 10 | Principal | 211.0 | 2.836 | 2.837 | 2.584 |
| Rosehip skin | 10 | Secondary | 318.0 | 2.830 | 2.829 | 0.874 |
| Rosehip skin | 10 | Secondary | 444.0 | 0.935 | 0.931 | 0.132 |
| Rosehip skin | 15 | Principal | 198.0 | 5.187 | 5.177 | 4.789 |
| Rosehip skin | 15 | Secondary | 328.0 | 3.888 | 3.869 | 1.127 |
| Rosehip skin | 20 | Principal | 222.0 | 6.972 | 6.975 | 6.423 |
| Rosehip skin | 20 | Secondary | 335.0 | 5.857 | 5.832 | 1.841 |
| Rosehip skin | 20 | Secondary | 456.0 | 1.907 | 1.900 | 0.340 |
| Whole-fruit rosehip | 5 | Secondary | 174.0 | 0.973 | 0.971 | 0.140 |
| Whole-fruit rosehip | 5 | Principal | 333.0 | 1.884 | 1.877 | 1.667 |
| Whole-fruit rosehip | 10 | Secondary | 214.0 | 2.014 | 2.015 | 0.350 |
| Whole-fruit rosehip | 10 | Principal | 341.0 | 3.640 | 3.638 | 3.180 |
| Whole-fruit rosehip | 15 | Secondary | 222.0 | 2.964 | 2.960 | 0.529 |
| Whole-fruit rosehip | 15 | Principal | 352.0 | 5.966 | 5.953 | 5.276 |
| Whole-fruit rosehip | 20 | Secondary | 225.0 | 4.281 | 4.280 | 0.793 |
| Whole-fruit rosehip | 20 | Principal | 351.0 | 7.871 | 7.875 | 6.894 |

Note. Peak positions were identified from the Savitzky–Golay-smoothed DTGₜ signal, and the raw DTGₜ value at the detected temperature was retained as the reported peak height. A detected peak temperature can therefore differ slightly from the raw experimental Tₘₐₓ in Table S1. Principal and secondary labels are descriptive; competing and recurring peaks are classified in Table S10.

Table S4. Heating-rate shifts in characteristic and principal-peak temperatures between 5 and 20 °C min⁻¹

| Feedstock | T₅%, 5 | T₅%, 20 | ΔT₅% | T₁₀%, 5 | T₁₀%, 20 | ΔT₁₀% | T₅₀%, 5 | T₅₀%, 20 | ΔT₅₀% | Tₘₐₓ, 5 | Tₘₐₓ, 20 | ΔTₘₐₓ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 180.3 | 228.0 | 47.7 | 231.2 | 268.9 | 37.7 | 342.3 | 368.3 | 26.0 | 332.0 | 360.0 | 28.0 |
| Rosehip skin | 141.8 | 175.1 | 33.4 | 165.5 | 197.9 | 32.4 | 351.4 | 372.3 | 20.9 | 173.0 | 223.0 | 50.0 |
| Whole-fruit rosehip | 116.0 | 136.9 | 20.9 | 164.5 | 193.9 | 29.4 | 338.6 | 355.6 | 17.0 | 334.0 | 351.0 | 17.0 |

Note. All temperature entries are in °C. Δ values equal the full-precision value at 20 °C min⁻¹ minus that at 5 °C min⁻¹; displayed operands are rounded. For rosehip skin, ΔTₘₐₓ reflects a change in the automatically assigned dominant maximum and should not be interpreted as migration of one homologous reaction peak because competing peaks exchange relative dominance.

Table S5. Across-rate variation in residue at 700 °C

| Feedstock | Minimum R₇₀₀ (%) | Maximum R₇₀₀ (%) | Range (pp) |
| --- | --- | --- | --- |
| Rosehip seed | 22.91 | 26.79 | 3.88 |
| Rosehip skin | 29.36 | 33.47 | 4.11 |
| Whole-fruit rosehip | 20.93 | 29.44 | 8.51 |

Note. The predefined screening threshold for the across-rate R₇₀₀ range was 5.0 percentage points. The whole-fruit range exceeded this threshold and therefore requires replicate and sample-homogeneity verification before it is attributed solely to heating rate.

Table S6. Conversion-temperature matrix for the selected analytical interval of 150–700 °C

| Feedstock | β (°C min⁻¹) | Tα=0.10 (°C) | Tα=0.20 (°C) | Tα=0.30 (°C) | Tα=0.40 (°C) | Tα=0.50 (°C) | Tα=0.60 (°C) | Tα=0.70 (°C) | Tα=0.80 (°C) | Tα=0.90 (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 236.4 | 264.2 | 287.2 | 308.8 | 324.4 | 335.1 | 351.5 | 389.9 | 451.0 |
| Rosehip seed | 10 | 254.7 | 278.2 | 299.1 | 320.7 | 336.6 | 347.1 | 359.6 | 396.7 | 467.8 |
| Rosehip seed | 15 | 257.0 | 283.4 | 305.6 | 328.3 | 345.4 | 356.9 | 373.0 | 417.1 | 510.9 |
| Rosehip seed | 20 | 266.5 | 292.7 | 314.4 | 336.2 | 352.2 | 363.5 | 379.5 | 419.5 | 512.9 |
| Rosehip skin | 5 | 174.0 | 197.4 | 223.8 | 257.0 | 286.4 | 310.5 | 340.7 | 400.1 | 481.4 |
| Rosehip skin | 10 | 183.0 | 206.7 | 231.3 | 263.7 | 293.5 | 318.6 | 350.9 | 413.4 | 493.0 |
| Rosehip skin | 15 | 185.5 | 206.7 | 230.9 | 266.6 | 302.4 | 332.0 | 376.0 | 450.4 | 531.0 |
| Rosehip skin | 20 | 192.8 | 215.2 | 236.4 | 268.2 | 302.8 | 331.8 | 368.2 | 445.4 | 529.0 |
| Whole-fruit rosehip | 5 | 187.7 | 225.0 | 258.4 | 284.4 | 307.0 | 326.3 | 345.0 | 391.5 | 469.1 |
| Whole-fruit rosehip | 10 | 194.4 | 228.7 | 264.7 | 291.3 | 315.0 | 334.8 | 354.3 | 402.9 | 485.3 |
| Whole-fruit rosehip | 15 | 203.5 | 239.7 | 275.8 | 302.1 | 325.7 | 344.9 | 362.6 | 399.0 | 463.7 |
| Whole-fruit rosehip | 20 | 210.4 | 248.2 | 284.5 | 311.1 | 335.0 | 354.9 | 384.1 | 458.0 | 547.1 |

Note. Tα is the temperature at fractional conversion α defined by manuscript Eq. (2), with m₁₅₀ and m₇₀₀ as the rate-specific masses at 150 and 700 °C. Values were obtained from the first bracketing α(T) pair by Eq. (S5), without extrapolation. All rows increase monotonically from α = 0.10 to 0.90.

Table S7. Comparison between T₅₀% and Tα=0.50

| Feedstock | β (°C min⁻¹) | T₅₀% (°C) | Tα=0.50 (°C) | Tα=0.50 − T₅₀% (°C) |
| --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 342.3 | 324.4 | -18.0 |
| Rosehip seed | 10 | 354.6 | 336.6 | -18.1 |
| Rosehip seed | 15 | 363.4 | 345.4 | -18.1 |
| Rosehip seed | 20 | 368.3 | 352.2 | -16.1 |
| Rosehip skin | 5 | 351.4 | 286.4 | -65.0 |
| Rosehip skin | 10 | 368.1 | 293.5 | -74.6 |
| Rosehip skin | 15 | 372.1 | 302.4 | -69.7 |
| Rosehip skin | 20 | 372.3 | 302.8 | -69.5 |
| Whole-fruit rosehip | 5 | 338.6 | 307.0 | -31.6 |
| Whole-fruit rosehip | 10 | 349.7 | 315.0 | -34.7 |
| Whole-fruit rosehip | 15 | 357.9 | 325.7 | -32.2 |
| Whole-fruit rosehip | 20 | 355.6 | 335.0 | -20.6 |

Table S8. Run-level quality-control summary

| Feedstock | β | Valid end T (°C) | Initial mass gain (%) | Loss decreases (n) | Detected peaks (n) | DTG–TG derivative r | DTG integral difference (pp) | Closure error (pp) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 788 | 0.40 | 4 | 1 | 0.9999 | 2.09 | 0.000 |
| Rosehip seed | 10 | 785 | 0.30 | 3 | 2 | 0.9999 | 2.38 | 0.000 |
| Rosehip seed | 15 | 783 | 1.00 | 9 | 1 | 0.9998 | 2.47 | 0.000 |
| Rosehip seed | 20 | 781 | 1.20 | 11 | 1 | 0.9998 | 2.76 | 0.000 |
| Rosehip skin | 5 | 788 | 1.30 | 9 | 2 | 0.9996 | 1.93 | 0.000 |
| Rosehip skin | 10 | 785 | 0.80 | 7 | 3 | 0.9997 | 2.26 | 0.000 |
| Rosehip skin | 15 | 783 | 1.70 | 14 | 2 | 0.9988 | 2.47 | 0.000 |
| Rosehip skin | 20 | 781 | 1.30 | 11 | 3 | 0.9983 | 2.71 | 0.000 |
| Whole-fruit rosehip | 5 | 788 | 0.20 | 2 | 2 | 0.9997 | 1.32 | 0.000 |
| Whole-fruit rosehip | 10 | 785 | 0.20 | 2 | 2 | 0.9997 | 1.78 | 0.000 |
| Whole-fruit rosehip | 15 | 783 | 0.30 | 3 | 2 | 0.9997 | 1.92 | 0.000 |
| Whole-fruit rosehip | 20 | 781 | 0.50 | 5 | 2 | 0.9993 | 2.30 | 0.000 |

Table S9. Numerical diagnostics for the conversion coordinate and conversion-rate relation.

| Feedstock | β (°C min⁻¹) | Eq. (S6) max ¦difference¦ (min⁻¹) | Eq. (S6) vs DTG RMSE (min⁻¹) | Processed α < 0 (n) | Processed α > 1 (n) |
| --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 8.33e-16 | 0.000630 | 0 | 0 |
| Rosehip seed | 10 | 2.33e-15 | 0.001298 | 0 | 0 |
| Rosehip seed | 15 | 2.94e-15 | 0.001820 | 0 | 0 |
| Rosehip seed | 20 | 4.44e-15 | 0.002385 | 0 | 0 |
| Rosehip skin | 5 | 6.11e-16 | 0.000540 | 0 | 0 |
| Rosehip skin | 10 | 1.22e-15 | 0.001084 | 0 | 0 |
| Rosehip skin | 15 | 1.55e-15 | 0.001580 | 0 | 0 |
| Rosehip skin | 20 | 2.44e-15 | 0.002234 | 0 | 0 |
| Whole-fruit rosehip | 5 | 6.66e-16 | 0.000555 | 0 | 0 |
| Whole-fruit rosehip | 10 | 1.44e-15 | 0.001116 | 0 | 0 |
| Whole-fruit rosehip | 15 | 2.61e-15 | 0.001670 | 0 | 0 |
| Whole-fruit rosehip | 20 | 2.89e-15 | 0.002138 | 0 | 0 |

N

## S6.2. Supplementary figure S1

Figure S1. Conversion and conversion-rate profiles calculated over 150–700 °C using manuscript Eq. (2) and Supplementary Eq. (S6).

Note. Panels (a,b), (c,d), and (e,f) correspond to rosehip seed, rosehip skin, and whole-fruit rosehip, respectively. Left panels show α(T), with α = 0 at 150 °C and α = 1 at 700 °C by construction. Right panels show dα/dt in min⁻¹ calculated from Eq. (S6). The conversion-rate curves are interval-normalised kinetic descriptors and are not numerically identical to the raw DTGₜ curves in % min⁻¹.

# S7. Supplementary information for Section 3.1.2: DTG feature analysis and peak-domain metrics

## S7.1. Supplementary tables S10–S14

Table S10. Direct DTG peaks and cross-heating-rate persistence classification for rosehip feedstocks.

| Feedstock | β (°C min⁻¹) | Tₚ (°C) | Hₚ (% min⁻¹) | Prominence (% min⁻¹) | Peak identity | Classification |
| --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 332 | 3.598 | 2.944 | Persistent peak 1 | Principal persistent |
| Rosehip seed | 10 | 278 | 3.614 | 0.292 | — | Non-persistent shoulder |
| Rosehip seed | 10 | 346 | 7.397 | 6.099 | Persistent peak 1 | Principal persistent |
| Rosehip seed | 15 | 354 | 10.310 | 8.186 | Persistent peak 1 | Principal persistent |
| Rosehip seed | 20 | 360 | 14.060 | 10.871 | Persistent peak 1 | Principal persistent |
| Rosehip skin | 5 | 173 | 1.516 | 1.101 | Lower-temperature peak | Principal persistent |
| Rosehip skin | 5 | 310 | 1.440 | 0.505 | Higher-temperature peak | Secondary persistent |
| Rosehip skin | 10 | 211 | 2.836 | 2.039 | Lower-temperature peak | Principal persistent |
| Rosehip skin | 10 | 318 | 2.830 | 0.874 | Higher-temperature peak | Secondary persistent |
| Rosehip skin | 15 | 198 | 5.187 | 3.848 | Lower-temperature peak | Principal persistent |
| Rosehip skin | 15 | 328 | 3.888 | 1.127 | Higher-temperature peak | Secondary persistent |
| Rosehip skin | 20 | 222 | 6.972 | 5.384 | Lower-temperature peak | Principal persistent |
| Rosehip skin | 20 | 335 | 5.857 | 1.841 | Higher-temperature peak | Secondary persistent |
| Whole-fruit rosehip | 5 | 174 | 0.973 | 0.140 | Lower-temperature peak | Secondary persistent |
| Whole-fruit rosehip | 5 | 333 | 1.884 | 1.429 | Higher-temperature peak | Principal persistent |
| Whole-fruit rosehip | 10 | 214 | 2.014 | 0.350 | Lower-temperature peak | Secondary persistent |
| Whole-fruit rosehip | 10 | 341 | 3.640 | 2.734 | Higher-temperature peak | Principal persistent |
| Whole-fruit rosehip | 15 | 222 | 2.964 | 0.529 | Lower-temperature peak | Secondary persistent |
| Whole-fruit rosehip | 15 | 352 | 5.966 | 4.227 | Higher-temperature peak | Principal persistent |
| Whole-fruit rosehip | 20 | 225 | 4.281 | 0.793 | Lower-temperature peak | Secondary persistent |
| Whole-fruit rosehip | 20 | 351 | 7.871 | 5.916 | Higher-temperature peak | Principal persistent |

Note. Peak positions were located from the smoothed DTGₜ signal, whereas Hₚ is the raw experimental ordinate at the detected temperature. The 100–420 °C search interval excludes later minor features from persistence classification. The seed feature at 278 °C occurred only at 10 °C min⁻¹ and is retained as a non-persistent shoulder.

Table S11. Peak-height ratios and inter-peak temperature separations for feedstocks containing two persistent DTG domains.

| Feedstock | β (°C min⁻¹) | Tₚ,₁ (°C) | Hₚ,₁ (% min⁻¹) | Tₚ,₂ (°C) | Hₚ,₂ (% min⁻¹) | RH = Hₚ,₂/Hₚ,₁ (–) | ΔTₚ (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip skin | 5 | 173 | 1.516 | 310 | 1.440 | 0.950 | 137 |
| Rosehip skin | 10 | 211 | 2.836 | 318 | 2.830 | 0.998 | 107 |
| Rosehip skin | 15 | 198 | 5.187 | 328 | 3.888 | 0.750 | 130 |
| Rosehip skin | 20 | 222 | 6.972 | 335 | 5.857 | 0.840 | 113 |
| Whole-fruit rosehip | 5 | 174 | 0.973 | 333 | 1.884 | 1.937 | 159 |
| Whole-fruit rosehip | 10 | 214 | 2.014 | 341 | 3.640 | 1.807 | 127 |
| Whole-fruit rosehip | 15 | 222 | 2.964 | 352 | 5.966 | 2.013 | 130 |
| Whole-fruit rosehip | 20 | 225 | 4.281 | 351 | 7.871 | 1.839 | 126 |

Table S12. Descriptive variability of persistent peak-domain descriptors and two-peak metrics across 5–20 °C min⁻¹

| Feedstock | Peak/metric group | Descriptor | n | Mean | Sample SD | Range | CV (%) | CV basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | Persistent peak 1 | Tₚ (°C) | 4 | 348.00 | 12.11 | 28 | 1.95 | Kelvin mean |
| Rosehip seed | Persistent peak 1 | Hₚ (% min⁻¹) | 4 | 8.841 | 4.434 | 10.462 | 50.15 | Native ratio scale |
| Rosehip seed | Persistent peak 1 | Prominence (% min⁻¹) | 4 | 7.025 | 3.349 | 7.927 | 47.68 | Native ratio scale |
| Rosehip skin | Lower-temperature peak | Tₚ (°C) | 4 | 201.00 | 21.09 | 49 | 4.45 | Kelvin mean |
| Rosehip skin | Lower-temperature peak | Hₚ (% min⁻¹) | 4 | 4.128 | 2.429 | 5.456 | 58.85 | Native ratio scale |
| Rosehip skin | Lower-temperature peak | Prominence (% min⁻¹) | 4 | 3.093 | 1.906 | 4.283 | 61.62 | Native ratio scale |
| Rosehip skin | Higher-temperature peak | Tₚ (°C) | 4 | 322.75 | 11.00 | 25 | 1.85 | Kelvin mean |
| Rosehip skin | Higher-temperature peak | Hₚ (% min⁻¹) | 4 | 3.504 | 1.862 | 4.417 | 53.14 | Native ratio scale |
| Rosehip skin | Higher-temperature peak | Prominence (% min⁻¹) | 4 | 1.087 | 0.564 | 1.336 | 51.88 | Native ratio scale |
| Whole-fruit rosehip | Lower-temperature peak | Tₚ (°C) | 4 | 208.75 | 23.63 | 51 | 4.90 | Kelvin mean |
| Whole-fruit rosehip | Lower-temperature peak | Hₚ (% min⁻¹) | 4 | 2.558 | 1.407 | 3.308 | 55.02 | Native ratio scale |
| Whole-fruit rosehip | Lower-temperature peak | Prominence (% min⁻¹) | 4 | 0.453 | 0.277 | 0.653 | 61.15 | Native ratio scale |
| Whole-fruit rosehip | Higher-temperature peak | Tₚ (°C) | 4 | 344.25 | 9.00 | 19 | 1.46 | Kelvin mean |
| Whole-fruit rosehip | Higher-temperature peak | Hₚ (% min⁻¹) | 4 | 4.840 | 2.623 | 5.987 | 54.18 | Native ratio scale |
| Whole-fruit rosehip | Higher-temperature peak | Prominence (% min⁻¹) | 4 | 3.576 | 1.934 | 4.487 | 54.07 | Native ratio scale |
| Rosehip skin | Two-peak metrics | R_H (–) | 4 | 0.884 | 0.112 | 0.248 | 12.61 | Native ratio scale |
| Rosehip skin | Two-peak metrics | ΔTₚ (°C) | 4 | 121.75 | 14.08 | 30 | 11.56 | Temperature interval |
| Whole-fruit rosehip | Two-peak metrics | R_H (–) | 4 | 1.899 | 0.094 | 0.205 | 4.94 | Native ratio scale |
| Whole-fruit rosehip | Two-peak metrics | ΔTₚ (°C) | 4 | 135.50 | 15.76 | 33 | 11.63 | Temperature interval |

Table S13. Complete Kissinger peak-shift analysis for persistent DTG domains.

| Feedstock | Peak identity | Tₚ,5 (°C) | Tₚ,10 (°C) | Tₚ,15 (°C) | Tₚ,20 (°C) | E_K (kJ mol⁻¹) | R² |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | Persistent peak 1 | 332 | 346 | 354 | 360 | 147.60 | 0.9998 |
| Rosehip skin | Lower-temperature peak | 173 | 211 | 198 | 222 | 37.34 | 0.6708 |
| Rosehip skin | Higher-temperature peak | 310 | 318 | 328 | 335 | 148.56 | 0.9619 |
| Whole-fruit rosehip | Lower-temperature peak | 174 | 214 | 222 | 225 | 36.73 | 0.8654 |
| Whole-fruit rosehip | Higher-temperature peak | 333 | 341 | 352 | 351 | 192.95 | 0.9221 |

Table S14. Run-level diagnostics for the information-criterion-selected Gaussian candidates.

| Feedstock | β (°C min⁻¹) | Selected m | AICc | BIC | ¦r₁¦ | Max residual/peak | Mean residual/peak |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip skin | 5 | 3 | -1143.04 | -1105.08 | 0.9874 | 0.114 | 0.0025 |
| Rosehip skin | 10 | 3 | -897.12 | -859.16 | 0.9891 | 0.086 | 0.0024 |
| Rosehip skin | 15 | 3 | -614.73 | -576.77 | 0.9885 | 0.087 | 0.0003 |
| Rosehip skin | 20 | 3 | -238.49 | -200.53 | 0.9912 | 0.091 | 0.0033 |
| Whole-fruit rosehip | 5 | 3 | -1056.76 | -1018.80 | 0.9873 | 0.102 | 0.0028 |
| Whole-fruit rosehip | 10 | 3 | -872.24 | -834.28 | 0.9865 | 0.065 | 0.0015 |
| Whole-fruit rosehip | 15 | 3 | -504.84 | -466.88 | 0.9898 | 0.055 | 0.0030 |
| Whole-fruit rosehip | 20 | 3 | -501.53 | -463.57 | 0.9886 | 0.048 | 0.0031 |

## S7.2. Supplementary figures S2–S4

Figure S2. Complete Kissinger diagnostic set for all persistent DTG domains. Solid lines and circular markers denote the two regressions retained for formal peak-shift reporting; dashed lines and cross markers denote regressions not retained because of insufficient linearity and/or non-monotonic displacement. Non-retained apparent energies are diagnostic outputs only and are not used as principal kinetic results.

Figure S3. Descriptive coefficients of variation calculated using Eq. (S7). (a) Relative variability of persistent peak temperature, peak height, and prominence; absolute peak-temperature CVs use kelvin, while means, SDs, and ranges remain reported in °C in Table S12. (b) Variability of R_H and ΔTₚ for rosehip skin and whole-fruit rosehip. The values summarize four programmed heating rates and are not inferential measures of repeatability.

Figure S4. Gaussian-decomposition retention-decision diagnostics. Panels (a,c) show the mean AICc and BIC differences for one-, two-, and three-component candidates. Panels (b,d) show the lag-one residual correlation and maximum residual-to-peak ratio for the selected three-component models.

# S8. Supplementary information for Section 3.1.3: Analytical-support-screened isoconversional analysis

## S8.1. Supplementary tables S15–S22

Table S15. Input-data normalization and TG–DTG quality control.

| Sample | β (K min⁻¹) | TG–DTG scale | DTG unit | DTG sign | DTG correlation | m150 | m700 | Mass loss | n TG points |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 1.000000 | per_min | 1 | 0.999887 | 96.200 | 25.980 | 70.220 | 768 |
| Rosehip seed | 10 | 0.999001 | per_min | 1 | 0.999895 | 96.743 | 26.793 | 69.950 | 765 |
| Rosehip seed | 15 | 0.999001 | per_min | 1 | 0.999842 | 97.353 | 24.755 | 72.597 | 763 |
| Rosehip seed | 20 | 1.000000 | per_min | 1 | 0.999810 | 97.970 | 22.910 | 75.060 | 761 |
| Rosehip skin | 5 | 0.999001 | per_min | 1 | 0.999674 | 93.576 | 33.367 | 60.210 | 768 |
| Rosehip skin | 10 | 0.999001 | per_min | 1 | 0.999696 | 95.534 | 33.467 | 62.068 | 765 |
| Rosehip skin | 15 | 0.999001 | per_min | 1 | 0.999603 | 96.793 | 29.361 | 67.433 | 763 |
| Rosehip skin | 20 | 0.999001 | per_min | 1 | 0.999603 | 98.222 | 29.970 | 68.252 | 761 |
| Whole-fruit rosehip | 5 | 1.000000 | per_min | 1 | 0.999740 | 92.110 | 29.190 | 62.920 | 766 |
| Whole-fruit rosehip | 10 | 0.999001 | per_min | 1 | 0.999734 | 93.546 | 29.441 | 64.106 | 763 |
| Whole-fruit rosehip | 15 | 1.000000 | per_min | 1 | 0.999755 | 94.120 | 28.740 | 65.380 | 761 |
| Whole-fruit rosehip | 20 | 0.999001 | per_min | 1 | 0.999391 | 94.296 | 20.929 | 73.367 | 759 |

Table S16. Fixed-conversion temperatures and Friedman rate inputs for all heating rates.

| Sample | α | β | Tα (°C) | Tα (K) | dα/dT (K⁻¹) | βdα/dT (min⁻¹) | Relative DTG |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 0.10 | 5 | 236.424 | 509.574 | 0.0025509 | 0.0127547 | 0.2489 |
| Rosehip seed | 0.20 | 5 | 264.247 | 537.397 | 0.0045286 | 0.0226429 | 0.4419 |
| Rosehip seed | 0.30 | 5 | 287.213 | 560.363 | 0.0045868 | 0.0229340 | 0.4476 |
| Rosehip seed | 0.40 | 5 | 308.805 | 581.955 | 0.0054589 | 0.0272943 | 0.5327 |
| Rosehip seed | 0.50 | 5 | 324.397 | 597.547 | 0.0085139 | 0.0425697 | 0.8308 |
| Rosehip seed | 0.60 | 5 | 335.109 | 608.259 | 0.0098645 | 0.0493226 | 0.9626 |
| Rosehip seed | 0.70 | 5 | 351.456 | 624.606 | 0.0036649 | 0.0183245 | 0.3576 |
| Rosehip seed | 0.80 | 5 | 389.913 | 663.062 | 0.0023138 | 0.0115689 | 0.2258 |
| Rosehip seed | 0.90 | 5 | 450.971 | 724.121 | 0.0009838 | 0.0049189 | 0.0960 |
| Rosehip seed | 0.10 | 10 | 254.660 | 527.810 | 0.0030744 | 0.0307438 | 0.2910 |
| Rosehip seed | 0.20 | 10 | 278.183 | 551.333 | 0.0051609 | 0.0516086 | 0.4885 |
| Rosehip seed | 0.30 | 10 | 299.113 | 572.263 | 0.0048226 | 0.0482257 | 0.4565 |
| Rosehip seed | 0.40 | 10 | 320.651 | 593.801 | 0.0052776 | 0.0527762 | 0.4996 |
| Rosehip seed | 0.50 | 10 | 336.589 | 609.739 | 0.0084551 | 0.0845506 | 0.8004 |
| Rosehip seed | 0.60 | 10 | 347.144 | 620.294 | 0.0104898 | 0.1048976 | 0.9930 |
| Rosehip seed | 0.70 | 10 | 359.632 | 632.782 | 0.0054684 | 0.0546845 | 0.5176 |
| Rosehip seed | 0.80 | 10 | 396.707 | 669.857 | 0.0021383 | 0.0213828 | 0.2024 |
| Rosehip seed | 0.90 | 10 | 467.760 | 740.910 | 0.0008204 | 0.0082045 | 0.0777 |
| Rosehip seed | 0.10 | 15 | 257.039 | 530.189 | 0.0025444 | 0.0381653 | 0.2690 |
| Rosehip seed | 0.20 | 15 | 283.424 | 556.574 | 0.0048252 | 0.0723778 | 0.5102 |
| Rosehip seed | 0.30 | 15 | 305.628 | 578.778 | 0.0045839 | 0.0687590 | 0.4846 |
| Rosehip seed | 0.40 | 15 | 328.280 | 601.430 | 0.0049705 | 0.0745573 | 0.5255 |
| Rosehip seed | 0.50 | 15 | 345.373 | 618.523 | 0.0078725 | 0.1180881 | 0.8323 |
| Rosehip seed | 0.60 | 15 | 356.882 | 630.032 | 0.0092107 | 0.1381601 | 0.9738 |
| Rosehip seed | 0.70 | 15 | 373.036 | 646.186 | 0.0037134 | 0.0557005 | 0.3926 |
| Rosehip seed | 0.80 | 15 | 417.114 | 690.264 | 0.0019249 | 0.0288742 | 0.2035 |
| Rosehip seed | 0.90 | 15 | 510.860 | 784.010 | 0.0008131 | 0.0121967 | 0.0860 |
| Rosehip seed | 0.10 | 20 | 266.478 | 539.628 | 0.0026049 | 0.0520976 | 0.2781 |
| Rosehip seed | 0.20 | 20 | 292.691 | 565.841 | 0.0048705 | 0.0974090 | 0.5200 |
| Rosehip seed | 0.30 | 20 | 314.406 | 587.556 | 0.0047048 | 0.0940969 | 0.5023 |
| Rosehip seed | 0.40 | 20 | 336.247 | 609.397 | 0.0052958 | 0.1059165 | 0.5654 |
| Rosehip seed | 0.50 | 20 | 352.183 | 625.333 | 0.0082695 | 0.1653899 | 0.8829 |
| Rosehip seed | 0.60 | 20 | 363.524 | 636.674 | 0.0090114 | 0.1802287 | 0.9622 |
| Rosehip seed | 0.70 | 20 | 379.507 | 652.657 | 0.0040702 | 0.0814043 | 0.4346 |
| Rosehip seed | 0.80 | 20 | 419.453 | 692.603 | 0.0020917 | 0.0418337 | 0.2233 |
| Rosehip seed | 0.90 | 20 | 512.880 | 786.030 | 0.0007268 | 0.0145366 | 0.0776 |
| Rosehip skin | 0.10 | 5 | 174.024 | 447.174 | 0.0050170 | 0.0250851 | 0.9973 |
| Rosehip skin | 0.20 | 5 | 197.392 | 470.542 | 0.0041069 | 0.0205344 | 0.8164 |
| Rosehip skin | 0.30 | 5 | 223.805 | 496.955 | 0.0033845 | 0.0169226 | 0.6728 |
| Rosehip skin | 0.40 | 5 | 256.989 | 530.139 | 0.0032549 | 0.0162744 | 0.6470 |
| Rosehip skin | 0.50 | 5 | 286.386 | 559.536 | 0.0039459 | 0.0197294 | 0.7844 |
| Rosehip skin | 0.60 | 5 | 310.452 | 583.602 | 0.0047725 | 0.0238625 | 0.9487 |
| Rosehip skin | 0.70 | 5 | 340.727 | 613.877 | 0.0025749 | 0.0128746 | 0.5118 |
| Rosehip skin | 0.80 | 5 | 400.067 | 673.217 | 0.0014713 | 0.0073565 | 0.2925 |
| Rosehip skin | 0.90 | 5 | 481.383 | 754.533 | 0.0009620 | 0.0048102 | 0.1912 |
| Rosehip skin | 0.10 | 10 | 182.972 | 456.122 | 0.0042541 | 0.0425406 | 0.9307 |
| Rosehip skin | 0.20 | 10 | 206.689 | 479.839 | 0.0045284 | 0.0452835 | 0.9907 |
| Rosehip skin | 0.30 | 10 | 231.329 | 504.479 | 0.0035964 | 0.0359645 | 0.7868 |
| Rosehip skin | 0.40 | 10 | 263.747 | 536.897 | 0.0032609 | 0.0326089 | 0.7134 |
| Rosehip skin | 0.50 | 10 | 293.543 | 566.693 | 0.0038396 | 0.0383957 | 0.8400 |
| Rosehip skin | 0.60 | 10 | 318.585 | 591.735 | 0.0045503 | 0.0455026 | 0.9954 |
| Rosehip skin | 0.70 | 10 | 350.936 | 624.086 | 0.0022725 | 0.0227249 | 0.4971 |
| Rosehip skin | 0.80 | 10 | 413.425 | 686.575 | 0.0013227 | 0.0132271 | 0.2894 |
| Rosehip skin | 0.90 | 10 | 492.950 | 766.100 | 0.0010234 | 0.0102339 | 0.2239 |
| Rosehip skin | 0.10 | 15 | 185.517 | 458.667 | 0.0043800 | 0.0656996 | 0.8550 |
| Rosehip skin | 0.20 | 15 | 206.710 | 479.860 | 0.0048322 | 0.0724836 | 0.9433 |
| Rosehip skin | 0.30 | 15 | 230.917 | 504.067 | 0.0037191 | 0.0557864 | 0.7260 |
| Rosehip skin | 0.40 | 15 | 266.556 | 539.706 | 0.0027286 | 0.0409284 | 0.5326 |
| Rosehip skin | 0.50 | 15 | 302.429 | 575.579 | 0.0032141 | 0.0482116 | 0.6274 |
| Rosehip skin | 0.60 | 15 | 332.042 | 605.192 | 0.0037661 | 0.0564920 | 0.7351 |
| Rosehip skin | 0.70 | 15 | 376.000 | 649.150 | 0.0015960 | 0.0239407 | 0.3115 |
| Rosehip skin | 0.80 | 15 | 450.400 | 723.550 | 0.0014438 | 0.0216563 | 0.2818 |
| Rosehip skin | 0.90 | 15 | 531.000 | 804.150 | 0.0009174 | 0.0137615 | 0.1791 |
| Rosehip skin | 0.10 | 20 | 192.777 | 465.927 | 0.0039119 | 0.0782379 | 0.7663 |
| Rosehip skin | 0.20 | 20 | 215.164 | 488.314 | 0.0050364 | 0.1007287 | 0.9866 |
| Rosehip skin | 0.30 | 20 | 236.366 | 509.516 | 0.0043986 | 0.0879726 | 0.8617 |
| Rosehip skin | 0.40 | 20 | 268.200 | 541.350 | 0.0029242 | 0.0584836 | 0.5728 |
| Rosehip skin | 0.50 | 20 | 302.762 | 575.912 | 0.0032892 | 0.0657836 | 0.6443 |
| Rosehip skin | 0.60 | 20 | 331.757 | 604.907 | 0.0041542 | 0.0830840 | 0.8138 |
| Rosehip skin | 0.70 | 20 | 368.218 | 641.368 | 0.0017149 | 0.0342974 | 0.3359 |
| Rosehip skin | 0.80 | 20 | 445.400 | 718.550 | 0.0013236 | 0.0264725 | 0.2593 |
| Rosehip skin | 0.90 | 20 | 528.967 | 802.117 | 0.0008968 | 0.0179352 | 0.1757 |
| Whole-fruit rosehip | 0.10 | 5 | 187.659 | 460.809 | 0.0028034 | 0.0140169 | 0.4676 |
| Whole-fruit rosehip | 0.20 | 5 | 224.963 | 498.113 | 0.0026440 | 0.0132199 | 0.4410 |
| Whole-fruit rosehip | 0.30 | 5 | 258.417 | 531.567 | 0.0037855 | 0.0189276 | 0.6315 |
| Whole-fruit rosehip | 0.40 | 5 | 284.415 | 557.565 | 0.0042768 | 0.0213842 | 0.7134 |
| Whole-fruit rosehip | 0.50 | 5 | 307.032 | 580.182 | 0.0051672 | 0.0258362 | 0.8619 |
| Whole-fruit rosehip | 0.60 | 5 | 326.300 | 599.450 | 0.0056338 | 0.0281691 | 0.9398 |
| Whole-fruit rosehip | 0.70 | 5 | 345.015 | 618.165 | 0.0042972 | 0.0214859 | 0.7168 |
| Whole-fruit rosehip | 0.80 | 5 | 391.509 | 664.659 | 0.0017625 | 0.0088126 | 0.2940 |
| Whole-fruit rosehip | 0.90 | 5 | 469.133 | 742.283 | 0.0009443 | 0.0047213 | 0.1575 |
| Whole-fruit rosehip | 0.10 | 10 | 194.372 | 467.522 | 0.0030766 | 0.0307660 | 0.5424 |
| Whole-fruit rosehip | 0.20 | 10 | 228.729 | 501.879 | 0.0027752 | 0.0277524 | 0.4893 |
| Whole-fruit rosehip | 0.30 | 10 | 264.732 | 537.882 | 0.0036045 | 0.0360453 | 0.6354 |
| Whole-fruit rosehip | 0.40 | 10 | 291.338 | 564.488 | 0.0041931 | 0.0419312 | 0.7392 |
| Whole-fruit rosehip | 0.50 | 10 | 314.983 | 588.133 | 0.0047802 | 0.0478022 | 0.8427 |
| Whole-fruit rosehip | 0.60 | 10 | 334.829 | 607.979 | 0.0055474 | 0.0554742 | 0.9780 |
| Whole-fruit rosehip | 0.70 | 10 | 354.304 | 627.454 | 0.0042613 | 0.0426134 | 0.7512 |
| Whole-fruit rosehip | 0.80 | 10 | 402.860 | 676.010 | 0.0015986 | 0.0159863 | 0.2818 |
| Whole-fruit rosehip | 0.90 | 10 | 485.260 | 758.410 | 0.0009473 | 0.0094727 | 0.1670 |
| Whole-fruit rosehip | 0.10 | 15 | 203.544 | 476.694 | 0.0029214 | 0.0438207 | 0.4802 |
| Whole-fruit rosehip | 0.20 | 15 | 239.725 | 512.875 | 0.0025703 | 0.0385546 | 0.4225 |
| Whole-fruit rosehip | 0.30 | 15 | 275.850 | 549.000 | 0.0037291 | 0.0559368 | 0.6130 |
| Whole-fruit rosehip | 0.40 | 15 | 302.085 | 575.235 | 0.0041735 | 0.0626022 | 0.6860 |
| Whole-fruit rosehip | 0.50 | 15 | 325.700 | 598.850 | 0.0049202 | 0.0738024 | 0.8088 |
| Whole-fruit rosehip | 0.60 | 15 | 344.883 | 618.033 | 0.0058323 | 0.0874847 | 0.9587 |
| Whole-fruit rosehip | 0.70 | 15 | 362.594 | 635.744 | 0.0051655 | 0.0774821 | 0.8491 |
| Whole-fruit rosehip | 0.80 | 15 | 398.950 | 672.100 | 0.0019521 | 0.0292819 | 0.3209 |
| Whole-fruit rosehip | 0.90 | 15 | 463.743 | 736.893 | 0.0011506 | 0.0172587 | 0.1891 |
| Whole-fruit rosehip | 0.10 | 20 | 210.370 | 483.520 | 0.0028351 | 0.0567018 | 0.5291 |
| Whole-fruit rosehip | 0.20 | 20 | 248.165 | 521.315 | 0.0024049 | 0.0480988 | 0.4488 |
| Whole-fruit rosehip | 0.30 | 20 | 284.546 | 557.696 | 0.0037212 | 0.0744249 | 0.6944 |
| Whole-fruit rosehip | 0.40 | 20 | 311.057 | 584.207 | 0.0040964 | 0.0819289 | 0.7644 |
| Whole-fruit rosehip | 0.50 | 20 | 335.029 | 608.179 | 0.0048822 | 0.0976431 | 0.9111 |
| Whole-fruit rosehip | 0.60 | 20 | 354.905 | 628.055 | 0.0052884 | 0.1057673 | 0.9869 |
| Whole-fruit rosehip | 0.70 | 20 | 384.138 | 657.288 | 0.0019435 | 0.0388692 | 0.3627 |
| Whole-fruit rosehip | 0.80 | 20 | 458.022 | 731.172 | 0.0012284 | 0.0245679 | 0.2292 |
| Whole-fruit rosehip | 0.90 | 20 | 547.086 | 820.236 | 0.0009351 | 0.0187021 | 0.1745 |

Table S17. Activation energies and regression-derived uncertainty at each conversion

| Sample | α | Method | n | R² | Eₐ | SE(Eₐ) | CI low | CI high | RMSE | Max ¦residual¦ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 0.10 | Friedman | 4 | 0.9918 | 109.029 | 7.007 | 78.879 | 139.179 | 0.04735 | 0.08040 |
| Rosehip seed | 0.20 | Friedman | 4 | 0.9850 | 132.803 | 11.577 | 82.992 | 182.615 | 0.06677 | 0.09410 |
| Rosehip seed | 0.30 | Friedman | 4 | 0.9874 | 144.057 | 11.517 | 94.505 | 193.610 | 0.05906 | 0.06811 |
| Rosehip seed | 0.40 | Friedman | 4 | 0.9976 | 145.623 | 5.016 | 124.041 | 167.206 | 0.02436 | 0.03647 |
| Rosehip seed | 0.50 | Friedman | 4 | 0.9952 | 149.539 | 7.331 | 117.997 | 181.082 | 0.03471 | 0.05844 |
| Rosehip seed | 0.60 | Friedman | 4 | 0.9734 | 143.537 | 16.782 | 71.328 | 215.746 | 0.07897 | 0.13338 |
| Rosehip seed | 0.70 | Friedman | 4 | 0.7655 | 150.079 | 58.740 | -102.657 | 402.815 | 0.26957 | 0.44100 |
| Rosehip seed | 0.80 | Friedman | 4 | 0.8685 | 131.633 | 36.226 | -24.233 | 287.500 | 0.17070 | 0.21890 |
| Rosehip seed | 0.90 | Friedman | 4 | 0.9389 | 71.281 | 12.862 | 15.939 | 126.622 | 0.10299 | 0.14379 |
| Rosehip skin | 0.10 | Friedman | 4 | 0.9337 | 109.634 | 20.662 | 20.732 | 198.535 | 0.11319 | 0.19286 |
| Rosehip skin | 0.20 | Friedman | 4 | 0.9009 | 172.276 | 40.400 | -1.549 | 346.101 | 0.18811 | 0.32330 |
| Rosehip skin | 0.30 | Friedman | 4 | 0.9179 | 273.722 | 57.899 | 24.604 | 522.841 | 0.17416 | 0.24838 |
| Rosehip skin | 0.40 | Friedman | 4 | 0.9800 | 257.429 | 26.034 | 145.413 | 369.445 | 0.06612 | 0.09210 |
| Rosehip skin | 0.50 | Friedman | 4 | 0.9113 | 166.495 | 36.726 | 8.476 | 324.514 | 0.13172 | 0.17811 |
| Rosehip skin | 0.60 | Friedman | 4 | 0.8559 | 134.549 | 39.034 | -33.402 | 302.499 | 0.17170 | 0.22937 |
| Rosehip skin | 0.70 | Friedman | 4 | 0.6088 | 65.254 | 36.987 | -93.890 | 224.398 | 0.21942 | 0.25350 |
| Rosehip skin | 0.80 | Friedman | 4 | 0.9137 | 90.591 | 19.691 | 5.869 | 175.312 | 0.14533 | 0.16742 |
| Rosehip skin | 0.90 | Friedman | 4 | 0.8157 | 103.227 | 34.700 | -46.074 | 252.529 | 0.21141 | 0.28595 |
| Whole-fruit rosehip | 0.10 | Friedman | 4 | 0.9246 | 108.305 | 21.876 | 14.181 | 202.429 | 0.14462 | 0.21827 |
| Whole-fruit rosehip | 0.20 | Friedman | 4 | 0.8239 | 104.636 | 34.202 | -42.521 | 251.794 | 0.20510 | 0.28643 |
| Whole-fruit rosehip | 0.30 | Friedman | 4 | 0.9436 | 122.912 | 21.249 | 31.484 | 214.339 | 0.12261 | 0.17157 |
| Whole-fruit rosehip | 0.40 | Friedman | 4 | 0.9355 | 130.299 | 24.185 | 26.239 | 234.360 | 0.12842 | 0.17844 |
| Whole-fruit rosehip | 0.50 | Friedman | 4 | 0.9629 | 136.177 | 18.903 | 54.845 | 217.508 | 0.09662 | 0.12535 |
| Whole-fruit rosehip | 0.60 | Friedman | 4 | 0.9324 | 143.603 | 27.343 | 25.954 | 261.252 | 0.13249 | 0.13778 |
| Whole-fruit rosehip | 0.70 | Friedman | 4 | 0.1531 | 41.660 | 69.284 | -256.445 | 339.765 | 0.41840 | 0.62511 |
| Whole-fruit rosehip | 0.80 | Friedman | 4 | 0.2427 | 35.132 | 43.878 | -153.658 | 223.923 | 0.40288 | 0.61387 |
| Whole-fruit rosehip | 0.90 | Friedman | 4 | 0.2450 | 41.674 | 51.726 | -180.886 | 264.235 | 0.48030 | 0.68732 |
| Rosehip seed | 0.10 | KAS | 4 | 0.9518 | 98.010 | 15.597 | 30.900 | 165.119 | 0.10538 | 0.15812 |
| Rosehip seed | 0.20 | KAS | 4 | 0.9828 | 117.561 | 10.996 | 70.251 | 164.872 | 0.06341 | 0.10711 |
| Rosehip seed | 0.30 | KAS | 4 | 0.9851 | 133.108 | 11.572 | 83.318 | 182.898 | 0.05934 | 0.08712 |
| Rosehip seed | 0.40 | KAS | 4 | 0.9912 | 141.127 | 9.386 | 100.743 | 181.510 | 0.04558 | 0.05429 |
| Rosehip seed | 0.50 | KAS | 4 | 0.9963 | 145.114 | 6.229 | 118.310 | 171.917 | 0.02949 | 0.04237 |
| Rosehip seed | 0.60 | KAS | 4 | 0.9942 | 145.740 | 7.878 | 111.845 | 179.635 | 0.03707 | 0.06084 |
| Rosehip seed | 0.70 | KAS | 4 | 0.9474 | 146.095 | 24.335 | 41.391 | 250.799 | 0.11168 | 0.18259 |
| Rosehip seed | 0.80 | KAS | 4 | 0.8794 | 136.570 | 35.769 | -17.331 | 290.471 | 0.16855 | 0.24500 |
| Rosehip seed | 0.90 | KAS | 4 | 0.8746 | 74.919 | 20.064 | -11.409 | 161.246 | 0.16065 | 0.21618 |
| Rosehip skin | 0.10 | KAS | 4 | 0.9597 | 124.388 | 18.016 | 46.873 | 201.904 | 0.09869 | 0.16845 |
| Rosehip skin | 0.20 | KAS | 4 | 0.8919 | 142.215 | 35.003 | -8.389 | 292.819 | 0.16298 | 0.28043 |
| Rosehip skin | 0.30 | KAS | 4 | 0.8936 | 223.902 | 54.621 | -11.115 | 458.919 | 0.16430 | 0.26538 |
| Rosehip skin | 0.40 | KAS | 4 | 0.9850 | 278.928 | 24.338 | 174.210 | 383.646 | 0.06182 | 0.09033 |
| Rosehip skin | 0.50 | KAS | 4 | 0.9516 | 191.264 | 30.499 | 60.040 | 322.489 | 0.10938 | 0.15486 |
| Rosehip skin | 0.60 | KAS | 4 | 0.9204 | 151.469 | 31.491 | 15.974 | 286.964 | 0.13852 | 0.17862 |
| Rosehip skin | 0.70 | KAS | 4 | 0.7822 | 101.498 | 37.875 | -61.465 | 264.462 | 0.22468 | 0.27442 |
| Rosehip skin | 0.80 | KAS | 4 | 0.8276 | 81.056 | 26.155 | -31.482 | 193.594 | 0.19305 | 0.21833 |
| Rosehip skin | 0.90 | KAS | 4 | 0.8330 | 99.405 | 31.472 | -36.010 | 234.820 | 0.19175 | 0.23822 |
| Whole-fruit rosehip | 0.10 | KAS | 4 | 0.9495 | 101.068 | 16.486 | 30.133 | 172.003 | 0.10899 | 0.16337 |
| Whole-fruit rosehip | 0.20 | KAS | 4 | 0.8640 | 106.777 | 29.955 | -22.110 | 235.663 | 0.17964 | 0.25196 |
| Whole-fruit rosehip | 0.30 | KAS | 4 | 0.9179 | 113.932 | 24.098 | 10.247 | 217.617 | 0.13904 | 0.20065 |
| Whole-fruit rosehip | 0.40 | KAS | 4 | 0.9275 | 124.731 | 24.666 | 18.601 | 230.861 | 0.13097 | 0.18494 |
| Whole-fruit rosehip | 0.50 | KAS | 4 | 0.9401 | 130.378 | 23.266 | 30.272 | 230.484 | 0.11892 | 0.16527 |
| Whole-fruit rosehip | 0.60 | KAS | 4 | 0.9435 | 137.970 | 23.885 | 35.202 | 240.737 | 0.11573 | 0.14804 |
| Whole-fruit rosehip | 0.70 | KAS | 4 | 0.8220 | 101.775 | 33.489 | -42.317 | 245.866 | 0.20224 | 0.25748 |
| Whole-fruit rosehip | 0.80 | KAS | 4 | 0.4430 | 47.933 | 38.007 | -115.600 | 211.466 | 0.34898 | 0.50520 |
| Whole-fruit rosehip | 0.90 | KAS | 4 | 0.2468 | 35.777 | 44.195 | -154.380 | 225.934 | 0.41037 | 0.57954 |
| Rosehip seed | 0.10 | FWO | 4 | 0.9590 | 101.456 | 14.838 | 37.614 | 165.299 | 0.10545 | 0.15852 |
| Rosehip seed | 0.20 | FWO | 4 | 0.9852 | 120.481 | 10.430 | 75.602 | 165.359 | 0.06327 | 0.10686 |
| Rosehip seed | 0.30 | FWO | 4 | 0.9871 | 135.619 | 10.961 | 88.459 | 182.779 | 0.05912 | 0.08690 |
| Rosehip seed | 0.40 | FWO | 4 | 0.9924 | 143.586 | 8.879 | 105.384 | 181.787 | 0.04535 | 0.05403 |
| Rosehip seed | 0.50 | FWO | 4 | 0.9968 | 147.626 | 5.878 | 122.336 | 172.915 | 0.02927 | 0.04207 |
| Rosehip seed | 0.60 | FWO | 4 | 0.9950 | 148.396 | 7.448 | 116.349 | 180.443 | 0.03686 | 0.06054 |
| Rosehip seed | 0.70 | FWO | 4 | 0.9541 | 148.994 | 23.107 | 49.573 | 248.414 | 0.11154 | 0.18238 |
| Rosehip seed | 0.80 | FWO | 4 | 0.8953 | 140.566 | 33.988 | -5.672 | 286.803 | 0.16845 | 0.24479 |
| Rosehip seed | 0.90 | FWO | 4 | 0.9053 | 83.182 | 19.027 | 1.314 | 165.050 | 0.16024 | 0.21532 |
| Rosehip skin | 0.10 | FWO | 4 | 0.9642 | 125.475 | 17.107 | 51.869 | 199.081 | 0.09857 | 0.16826 |
| Rosehip skin | 0.20 | FWO | 4 | 0.9021 | 142.787 | 33.262 | -0.327 | 285.901 | 0.16290 | 0.28026 |
| Rosehip skin | 0.30 | FWO | 4 | 0.9004 | 220.826 | 51.925 | -2.590 | 444.242 | 0.16428 | 0.26531 |
| Rosehip skin | 0.40 | FWO | 4 | 0.9859 | 273.657 | 23.155 | 174.027 | 373.286 | 0.06186 | 0.09039 |
| Rosehip skin | 0.50 | FWO | 4 | 0.9559 | 190.825 | 28.986 | 66.109 | 315.542 | 0.10934 | 0.15484 |
| Rosehip skin | 0.60 | FWO | 4 | 0.9293 | 153.415 | 29.922 | 24.673 | 282.158 | 0.13843 | 0.17857 |
| Rosehip skin | 0.70 | FWO | 4 | 0.8142 | 106.479 | 35.965 | -48.267 | 261.225 | 0.22440 | 0.27419 |
| Rosehip skin | 0.80 | FWO | 4 | 0.8630 | 88.108 | 24.818 | -18.677 | 194.893 | 0.19267 | 0.21803 |
| Rosehip skin | 0.90 | FWO | 4 | 0.8647 | 106.843 | 29.882 | -21.729 | 235.416 | 0.19149 | 0.23776 |
| Whole-fruit rosehip | 0.10 | FWO | 4 | 0.9563 | 103.552 | 15.643 | 36.244 | 170.861 | 0.10877 | 0.16313 |
| Whole-fruit rosehip | 0.20 | FWO | 4 | 0.8811 | 109.572 | 28.457 | -12.868 | 232.013 | 0.17949 | 0.25186 |
| Whole-fruit rosehip | 0.30 | FWO | 4 | 0.9289 | 116.928 | 22.878 | 18.491 | 215.366 | 0.13884 | 0.20046 |
| Whole-fruit rosehip | 0.40 | FWO | 4 | 0.9369 | 127.611 | 23.415 | 26.863 | 228.359 | 0.13077 | 0.18474 |
| Whole-fruit rosehip | 0.50 | FWO | 4 | 0.9480 | 133.348 | 22.080 | 38.347 | 228.349 | 0.11870 | 0.16505 |
| Whole-fruit rosehip | 0.60 | FWO | 4 | 0.9508 | 140.875 | 22.664 | 43.362 | 238.388 | 0.11550 | 0.14781 |
| Whole-fruit rosehip | 0.70 | FWO | 4 | 0.8497 | 106.852 | 31.775 | -29.864 | 243.568 | 0.20183 | 0.25695 |
| Whole-fruit rosehip | 0.80 | FWO | 4 | 0.5517 | 56.625 | 36.096 | -98.685 | 211.935 | 0.34860 | 0.50495 |
| Whole-fruit rosehip | 0.90 | FWO | 4 | 0.3777 | 46.330 | 42.053 | -134.609 | 227.270 | 0.41071 | 0.57938 |
| Rosehip seed | 0.10 | Starink | 4 | 0.9521 | 98.279 | 15.585 | 31.222 | 165.336 | 0.10539 | 0.15813 |
| Rosehip seed | 0.20 | Starink | 4 | 0.9829 | 117.833 | 10.986 | 70.565 | 165.102 | 0.06341 | 0.10710 |
| Rosehip seed | 0.30 | Starink | 4 | 0.9852 | 133.383 | 11.561 | 83.640 | 183.126 | 0.05933 | 0.08711 |
| Rosehip seed | 0.40 | Starink | 4 | 0.9913 | 141.409 | 9.376 | 101.066 | 181.753 | 0.04557 | 0.05428 |
| Rosehip seed | 0.50 | Starink | 4 | 0.9964 | 145.404 | 6.223 | 118.630 | 172.178 | 0.02948 | 0.04235 |
| Rosehip seed | 0.60 | Starink | 4 | 0.9942 | 146.037 | 7.870 | 112.177 | 179.897 | 0.03706 | 0.06083 |
| Rosehip seed | 0.70 | Starink | 4 | 0.9477 | 146.402 | 24.314 | 41.787 | 251.018 | 0.11167 | 0.18258 |
| Rosehip seed | 0.80 | Starink | 4 | 0.8801 | 136.911 | 35.739 | -16.863 | 290.686 | 0.16854 | 0.24499 |
| Rosehip seed | 0.90 | Starink | 4 | 0.8760 | 75.361 | 20.046 | -10.889 | 161.611 | 0.16063 | 0.21615 |
| Rosehip skin | 0.10 | Starink | 4 | 0.9599 | 124.592 | 18.000 | 47.142 | 202.042 | 0.09869 | 0.16844 |
| Rosehip skin | 0.20 | Starink | 4 | 0.8924 | 142.420 | 34.974 | -8.060 | 292.900 | 0.16298 | 0.28042 |
| Rosehip skin | 0.30 | Starink | 4 | 0.8939 | 224.057 | 54.578 | -10.771 | 458.885 | 0.16430 | 0.26538 |
| Rosehip skin | 0.40 | Starink | 4 | 0.9850 | 279.061 | 24.319 | 174.424 | 383.698 | 0.06182 | 0.09033 |
| Rosehip skin | 0.50 | Starink | 4 | 0.9518 | 191.489 | 30.474 | 60.371 | 322.607 | 0.10938 | 0.15486 |
| Rosehip skin | 0.60 | Starink | 4 | 0.9208 | 151.743 | 31.465 | 16.360 | 287.127 | 0.13851 | 0.17862 |
| Rosehip skin | 0.70 | Starink | 4 | 0.7836 | 101.837 | 37.843 | -60.989 | 264.662 | 0.22467 | 0.27441 |
| Rosehip skin | 0.80 | Starink | 4 | 0.8293 | 81.455 | 26.133 | -30.984 | 193.895 | 0.19303 | 0.21831 |
| Rosehip skin | 0.90 | Starink | 4 | 0.8345 | 99.844 | 31.446 | -35.455 | 235.143 | 0.19174 | 0.23820 |
| Whole-fruit rosehip | 0.10 | Starink | 4 | 0.9498 | 101.301 | 16.472 | 30.428 | 172.174 | 0.10898 | 0.16336 |
| Whole-fruit rosehip | 0.20 | Starink | 4 | 0.8648 | 107.030 | 29.930 | -21.749 | 235.809 | 0.17963 | 0.25196 |
| Whole-fruit rosehip | 0.30 | Starink | 4 | 0.9184 | 114.203 | 24.077 | 10.607 | 217.799 | 0.13903 | 0.20065 |
| Whole-fruit rosehip | 0.40 | Starink | 4 | 0.9279 | 125.011 | 24.645 | 18.972 | 231.049 | 0.13096 | 0.18493 |
| Whole-fruit rosehip | 0.50 | Starink | 4 | 0.9405 | 130.669 | 23.246 | 30.650 | 230.687 | 0.11891 | 0.16526 |
| Whole-fruit rosehip | 0.60 | Starink | 4 | 0.9438 | 138.267 | 23.864 | 35.590 | 240.944 | 0.11572 | 0.14803 |
| Whole-fruit rosehip | 0.70 | Starink | 4 | 0.8232 | 102.117 | 33.459 | -41.847 | 246.082 | 0.20222 | 0.25746 |
| Whole-fruit rosehip | 0.80 | Starink | 4 | 0.4478 | 48.359 | 37.975 | -115.036 | 211.754 | 0.34897 | 0.50519 |
| Whole-fruit rosehip | 0.90 | Starink | 4 | 0.2522 | 36.266 | 44.161 | -153.745 | 226.277 | 0.41039 | 0.57953 |

Table S18. Linear-regression slope and intercept diagnostics

| Sample | α | Method | Slope, m | Intercept | SE(m) | m CI low | m CI high |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 0.10 | Friedman | -13113.136635 | 21.386720 | 842.783604 | -16739.341809 | -9486.931460 |
| Rosehip seed | 0.20 | Friedman | -15972.587303 | 25.978112 | 1392.394465 | -21963.577149 | -9981.597456 |
| Rosehip seed | 0.30 | Friedman | -17326.125614 | 27.193150 | 1385.140457 | -23285.903981 | -11366.347248 |
| Rosehip seed | 0.40 | Friedman | -17514.471607 | 26.517340 | 603.289665 | -20110.217532 | -14918.725683 |
| Rosehip seed | 0.50 | Friedman | -17985.454808 | 26.968118 | 881.717543 | -21779.179200 | -14191.730416 |
| Rosehip seed | 0.60 | Friedman | -17263.567118 | 25.443132 | 2018.467937 | -25948.333696 | -8578.800540 |
| Rosehip seed | 0.70 | Friedman | -18050.362524 | 25.178205 | 7064.749573 | -48447.526559 | 12346.801511 |
| Rosehip seed | 0.80 | Friedman | -15831.859449 | 19.570633 | 4356.946849 | -34578.288702 | 2914.569805 |
| Rosehip seed | 0.90 | Friedman | -8573.103973 | 6.624181 | 1546.964193 | -15229.153680 | -1917.054265 |
| Rosehip skin | 0.10 | Friedman | -13185.885208 | 25.832742 | 2485.064862 | -23878.256319 | -2493.514097 |
| Rosehip skin | 0.20 | Friedman | -20720.031504 | 40.231658 | 4858.949647 | -41626.404468 | 186.341460 |
| Rosehip skin | 0.30 | Friedman | -32921.228438 | 62.176652 | 6963.628877 | -62883.305232 | -2959.151643 |
| Rosehip skin | 0.40 | Friedman | -30961.568562 | 54.263691 | 3131.185646 | -44433.973029 | -17489.164095 |
| Rosehip skin | 0.50 | Friedman | -20024.729698 | 31.936562 | 4417.111373 | -39030.026003 | -1019.433392 |
| Rosehip skin | 0.60 | Friedman | -16182.490892 | 24.095162 | 4694.736197 | -36382.310404 | 4017.328621 |
| Rosehip skin | 0.70 | Friedman | -7848.260408 | 8.611380 | 4448.566866 | -26988.898775 | 11292.377959 |
| Rosehip skin | 0.80 | Friedman | -10895.551282 | 11.393421 | 2368.229277 | -21085.219444 | -705.883120 |
| Rosehip skin | 0.90 | Friedman | -12415.364058 | 11.337934 | 4173.435822 | -30372.209092 | 5541.480976 |
| Whole-fruit rosehip | 0.10 | Friedman | -13026.060449 | 24.162292 | 2631.053724 | -24346.570938 | -1705.549959 |
| Whole-fruit rosehip | 0.20 | Friedman | -12584.877749 | 21.204637 | 4113.500801 | -30283.843200 | 5114.087701 |
| Whole-fruit rosehip | 0.30 | Friedman | -14782.863424 | 23.988932 | 2555.688071 | -25779.101681 | -3786.625168 |
| Whole-fruit rosehip | 0.40 | Friedman | -15671.419186 | 24.411994 | 2908.801472 | -28186.981780 | -3155.856591 |
| Whole-fruit rosehip | 0.50 | Friedman | -16378.287083 | 24.681906 | 2273.458731 | -26160.190497 | -6596.383668 |
| Whole-fruit rosehip | 0.60 | Friedman | -17271.498871 | 25.380493 | 3288.652957 | -31421.430493 | -3121.567249 |
| Whole-fruit rosehip | 0.70 | Friedman | -5010.493003 | 4.698484 | 8332.949704 | -40864.281793 | 30843.295786 |
| Whole-fruit rosehip | 0.80 | Friedman | -4225.446201 | 2.142279 | 5277.274254 | -26931.724677 | 18480.832274 |
| Whole-fruit rosehip | 0.90 | Friedman | -5012.247449 | 2.055102 | 6221.245366 | -31780.105804 | 21755.610907 |
| Rosehip seed | 0.10 | KAS | 11787.847210 | -12.256734 | 1875.911389 | 3716.451953 | 19859.242466 |
| Rosehip seed | 0.20 | KAS | 14139.368748 | -15.361654 | 1322.477772 | 8449.206153 | 19829.531343 |
| Rosehip seed | 0.30 | KAS | 16009.243758 | -17.559508 | 1391.784957 | 10020.876416 | 21997.611101 |
| Rosehip seed | 0.40 | KAS | 16973.640980 | -18.078235 | 1128.844130 | 12116.616701 | 21830.665259 |
| Rosehip seed | 0.50 | KAS | 17453.169654 | -18.058143 | 749.235658 | 14229.468805 | 20676.870502 |
| Rosehip seed | 0.60 | KAS | 17528.500550 | -17.639750 | 947.470351 | 13451.864658 | 21605.136441 |
| Rosehip seed | 0.70 | KAS | 17571.162152 | -16.987833 | 2926.804746 | 4978.137721 | 30164.186584 |
| Rosehip seed | 0.80 | KAS | 16425.566997 | -13.564479 | 4302.009039 | -2084.483937 | 34935.617930 |
| Rosehip seed | 0.90 | KAS | 9010.640562 | -1.032232 | 2413.123310 | -1372.191035 | 19393.472158 |
| Rosehip skin | 0.10 | KAS | 14960.497174 | -22.900271 | 2166.799240 | 5637.512509 | 24283.481838 |
| Rosehip skin | 0.20 | KAS | 17104.548666 | -25.725531 | 4209.838426 | -1008.924131 | 35218.021463 |
| Rosehip skin | 0.30 | KAS | 26929.197023 | -43.421129 | 6569.450724 | -1336.868065 | 55195.262111 |
| Rosehip skin | 0.40 | KAS | 33547.376730 | -52.305077 | 2927.187852 | 20952.703928 | 46142.049532 |
| Rosehip skin | 0.50 | KAS | 23003.824053 | -30.118597 | 3668.137150 | 7221.103731 | 38786.544374 |
| Rosehip skin | 0.60 | KAS | 18217.548682 | -20.177692 | 3787.506889 | 1921.221828 | 34513.875536 |
| Rosehip skin | 0.70 | KAS | 12207.427730 | -8.827518 | 4555.335739 | -7392.600022 | 31807.455482 |
| Rosehip skin | 0.80 | KAS | 9748.792853 | -3.231596 | 3145.780742 | -3786.409242 | 23283.994948 |
| Rosehip skin | 0.90 | KAS | 11955.651796 | -4.387607 | 3785.257316 | -4330.995927 | 28242.299520 |
| Whole-fruit rosehip | 0.10 | KAS | 12155.696771 | -15.844573 | 1982.855395 | 3624.158592 | 20687.234950 |
| Whole-fruit rosehip | 0.20 | KAS | 12842.263426 | -15.202248 | 3602.762428 | -2659.172168 | 28343.699021 |
| Whole-fruit rosehip | 0.30 | KAS | 13702.885942 | -15.002299 | 2898.313788 | 1232.448212 | 26173.323671 |
| Whole-fruit rosehip | 0.40 | KAS | 15001.728069 | -16.021602 | 2966.657338 | 2237.231777 | 27766.224360 |
| Whole-fruit rosehip | 0.50 | KAS | 15680.876186 | -16.045545 | 2798.264347 | 3640.916456 | 27720.835916 |
| Whole-fruit rosehip | 0.60 | KAS | 16593.933195 | -16.627844 | 2872.657967 | 4233.883550 | 28953.982839 |
| Whole-fruit rosehip | 0.70 | KAS | 12240.663749 | -8.795058 | 4027.791434 | -5089.524060 | 29570.851558 |
| Whole-fruit rosehip | 0.80 | KAS | 5764.976577 | 2.240404 | 4571.245888 | -13903.507022 | 25433.460176 |
| Whole-fruit rosehip | 0.90 | KAS | 4302.949795 | 5.233578 | 5315.474125 | -18567.689457 | 27173.589047 |
| Rosehip seed | 0.10 | FWO | -12834.495549 | 26.777608 | 1877.041726 | -20910.754253 | -4758.236845 |
| Rosehip seed | 0.20 | FWO | -15241.091699 | 29.984980 | 1319.481950 | -20918.364312 | -9563.819085 |
| Rosehip seed | 0.30 | FWO | -17156.113047 | 32.263099 | 1386.557547 | -23121.988660 | -11190.237433 |
| Rosehip seed | 0.40 | FWO | -18163.953029 | 32.856176 | 1123.163506 | -22996.535556 | -13331.370502 |
| Rosehip seed | 0.50 | FWO | -18674.998364 | 32.888354 | 743.545444 | -21874.216197 | -15475.780531 |
| Rosehip seed | 0.60 | FWO | -18772.486654 | 32.505912 | 942.214842 | -22826.509915 | -14718.463393 |
| Rosehip seed | 0.70 | FWO | -18848.047999 | 31.906195 | 2923.075446 | -31425.026544 | -6271.069454 |
| Rosehip seed | 0.80 | FWO | -17781.884449 | 28.603595 | 4299.528444 | -36281.262245 | 717.493346 |
| Rosehip seed | 0.90 | FWO | -10522.722687 | 16.289698 | 2406.996761 | -20879.193873 | -166.251502 |
| Rosehip skin | 0.10 | FWO | -15872.887375 | 37.146330 | 2164.103211 | -25184.271964 | -6561.502787 |
| Rosehip skin | 0.20 | FWO | -18062.893170 | 40.069825 | 4207.710992 | -36167.212357 | 41.426016 |
| Rosehip skin | 0.30 | FWO | -27935.034880 | 57.862068 | 6568.667980 | -56197.732096 | 327.662336 |
| Rosehip skin | 0.40 | FWO | -34618.254759 | 66.871323 | 2929.208332 | -47221.620986 | -22014.888532 |
| Rosehip skin | 0.50 | FWO | -24139.890160 | 44.803105 | 3666.799281 | -39916.854097 | -8362.926222 |
| Rosehip skin | 0.60 | FWO | -19407.410078 | 34.954824 | 3785.168349 | -35693.675006 | -3121.145151 |
| Rosehip skin | 0.70 | FWO | -13469.876878 | 23.723327 | 4549.685786 | -33045.594843 | 6105.841087 |
| Rosehip skin | 0.80 | FWO | -11145.933194 | 18.330590 | 3139.590652 | -24654.501483 | 2362.635095 |
| Rosehip skin | 0.90 | FWO | -13515.905729 | 19.707303 | 3780.164577 | -29780.641165 | 2748.829708 |
| Whole-fruit rosehip | 0.10 | FWO | -13099.626369 | 30.158718 | 1978.932827 | -21614.287098 | -4584.965639 |
| Whole-fruit rosehip | 0.20 | FWO | -13861.164306 | 29.669239 | 3599.870877 | -29350.158562 | 1627.829949 |
| Whole-fruit rosehip | 0.30 | FWO | -14791.738228 | 29.602105 | 2894.155490 | -27244.284249 | -2339.192207 |
| Whole-fruit rosehip | 0.40 | FWO | -16143.120480 | 30.715634 | 2962.087394 | -28887.953889 | -3398.287070 |
| Whole-fruit rosehip | 0.50 | FWO | -16868.821038 | 30.819529 | 2793.135536 | -28886.713275 | -4850.928801 |
| Whole-fruit rosehip | 0.60 | FWO | -17821.049177 | 31.466700 | 2866.988974 | -30156.707114 | -5485.391241 |
| Whole-fruit rosehip | 0.70 | FWO | -13517.058900 | 23.712902 | 4019.588577 | -30811.952661 | 3777.834861 |
| Whole-fruit rosehip | 0.80 | FWO | -7163.214152 | 12.861061 | 4566.263838 | -26810.261719 | 12483.833415 |
| Whole-fruit rosehip | 0.90 | FWO | -5860.915645 | 10.084613 | 5319.809287 | -28750.207597 | 17028.376307 |
| Rosehip seed | 0.10 | Starink | -11829.713143 | 12.837569 | 1875.956192 | -19901.301174 | -3758.125113 |
| Rosehip seed | 0.20 | Starink | -14183.437666 | 15.946587 | 1322.357462 | -19873.082609 | -8493.792723 |
| Rosehip seed | 0.30 | Starink | -16055.118530 | 18.147652 | 1391.575722 | -22042.585610 | -10067.651450 |
| Rosehip seed | 0.40 | Starink | -17021.253462 | 18.669352 | 1128.616853 | -21877.299844 | -12165.207081 |
| Rosehip seed | 0.50 | Starink | -17502.042802 | 18.651351 | 749.008034 | -20724.764264 | -14279.321340 |
| Rosehip seed | 0.60 | Starink | -17578.259994 | 18.234397 | 947.260020 | -21653.990904 | -13502.529083 |
| Rosehip seed | 0.70 | Starink | -17622.237586 | 17.584567 | 2926.655509 | -30214.619900 | -5029.855273 |
| Rosehip seed | 0.80 | Starink | -16479.819695 | 14.166043 | 4301.909788 | -34989.443587 | 2029.804198 |
| Rosehip seed | 0.90 | Starink | -9071.123847 | 1.642531 | 2412.878022 | -19452.900052 | 1310.652359 |
| Rosehip skin | 0.10 | Starink | -14996.992782 | 23.470113 | 2166.691285 | -24319.512952 | -5674.472612 |
| Rosehip skin | 0.20 | Starink | -17142.882446 | 26.299303 | 4209.753261 | -35255.988805 | 970.223912 |
| Rosehip skin | 0.30 | Starink | -26969.430537 | 43.998766 | 6569.419389 | -55235.360806 | 1296.499731 |
| Rosehip skin | 0.40 | Starink | -33590.211851 | 52.887727 | 2927.268671 | -46185.232390 | -20995.191313 |
| Rosehip skin | 0.50 | Starink | -23049.266697 | 30.705977 | 3668.083602 | -38831.756620 | -7266.776774 |
| Rosehip skin | 0.60 | Starink | -18265.143138 | 20.768777 | 3787.413313 | -34561.067366 | -1969.218910 |
| Rosehip skin | 0.70 | Starink | -12257.925696 | 9.423350 | 4555.109722 | -31856.980975 | 7341.129583 |
| Rosehip skin | 0.80 | Starink | -9804.678467 | 3.835555 | 3145.533086 | -23338.814987 | 3729.458054 |
| Rosehip skin | 0.90 | Starink | -12018.061953 | 5.000395 | 3785.053571 | -28303.833033 | 4267.709126 |
| Whole-fruit rosehip | 0.10 | Starink | -12193.453955 | 16.417139 | 1982.698454 | -20724.316869 | -3662.591042 |
| Whole-fruit rosehip | 0.20 | Starink | -12883.019461 | 15.780927 | 3602.646723 | -28383.957218 | 2617.918295 |
| Whole-fruit rosehip | 0.30 | Starink | -13746.440033 | 15.586291 | 2898.147411 | -26216.161902 | -1276.718164 |
| Whole-fruit rosehip | 0.40 | Starink | -15047.383765 | 16.609363 | 2966.474503 | -27811.093383 | -2283.674147 |
| Whole-fruit rosehip | 0.50 | Starink | -15728.393980 | 16.636504 | 2798.059162 | -27767.470871 | -3689.317089 |
| Whole-fruit rosehip | 0.60 | Starink | -16643.017834 | 17.221398 | 2872.431190 | -29002.091732 | -4283.943936 |
| Whole-fruit rosehip | 0.70 | Starink | -12291.719555 | 9.391772 | 4027.463320 | -29620.495601 | 5037.056491 |
| Whole-fruit rosehip | 0.80 | Starink | -5820.906080 | -1.636345 | 4571.046503 | -25488.531794 | 13846.719634 |
| Whole-fruit rosehip | 0.90 | Starink | -4365.268429 | -4.620850 | 5315.647103 | -27236.651947 | 18506.115090 |

Table S19. Inter-method agreement and Friedman cross-check

| Sample | α | Integral mean Eₐ | Integral SD | Integral CV (%) | Min integral R² | Friedman Eₐ | Friedman R² | Friedman deviation (%) | Four-method mean Eₐ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 0.10 | 99.248 | 1.917 | 1.932 | 0.9518 | 109.029 | 0.9918 | 9.854 | 101.693 |
| Rosehip seed | 0.20 | 118.625 | 1.613 | 1.359 | 0.9828 | 132.803 | 0.9850 | 11.952 | 122.170 |
| Rosehip seed | 0.30 | 134.037 | 1.377 | 1.027 | 0.9851 | 144.057 | 0.9874 | 7.476 | 136.542 |
| Rosehip seed | 0.40 | 142.041 | 1.346 | 0.947 | 0.9912 | 145.623 | 0.9976 | 2.522 | 142.936 |
| Rosehip seed | 0.50 | 146.048 | 1.374 | 0.941 | 0.9963 | 149.539 | 0.9952 | 2.391 | 146.921 |
| Rosehip seed | 0.60 | 146.724 | 1.455 | 0.992 | 0.9942 | 143.537 | 0.9734 | 2.172 | 145.928 |
| Rosehip seed | 0.70 | 147.164 | 1.592 | 1.082 | 0.9474 | 150.079 | 0.7655 | 1.981 | 147.892 |
| Rosehip seed | 0.80 | 138.016 | 2.215 | 1.605 | 0.8794 | 131.633 | 0.8685 | 4.624 | 136.420 |
| Rosehip seed | 0.90 | 77.821 | 4.648 | 5.973 | 0.8746 | 71.281 | 0.9389 | 8.404 | 76.186 |
| Rosehip skin | 0.10 | 124.819 | 0.577 | 0.463 | 0.9597 | 109.634 | 0.9337 | 12.166 | 121.022 |
| Rosehip skin | 0.20 | 142.474 | 0.290 | 0.203 | 0.8919 | 172.276 | 0.9009 | 20.917 | 149.924 |
| Rosehip skin | 0.30 | 222.928 | 1.822 | 0.817 | 0.8936 | 273.722 | 0.9179 | 22.785 | 235.627 |
| Rosehip skin | 0.40 | 277.215 | 3.083 | 1.112 | 0.9850 | 257.429 | 0.9800 | 7.138 | 272.269 |
| Rosehip skin | 0.50 | 191.193 | 0.338 | 0.177 | 0.9516 | 166.495 | 0.9113 | 12.918 | 185.018 |
| Rosehip skin | 0.60 | 152.209 | 1.053 | 0.692 | 0.9204 | 134.549 | 0.8559 | 11.603 | 147.794 |
| Rosehip skin | 0.70 | 103.271 | 2.783 | 2.695 | 0.7822 | 65.254 | 0.6088 | 36.813 | 93.767 |
| Rosehip skin | 0.80 | 83.540 | 3.961 | 4.742 | 0.8276 | 90.591 | 0.9137 | 8.440 | 85.303 |
| Rosehip skin | 0.90 | 102.031 | 4.173 | 4.090 | 0.8330 | 103.227 | 0.8157 | 1.173 | 102.330 |
| Whole-fruit rosehip | 0.10 | 101.974 | 1.372 | 1.345 | 0.9495 | 108.305 | 0.9246 | 6.208 | 103.557 |
| Whole-fruit rosehip | 0.20 | 107.793 | 1.546 | 1.434 | 0.8640 | 104.636 | 0.8239 | 2.928 | 107.004 |
| Whole-fruit rosehip | 0.30 | 115.021 | 1.657 | 1.441 | 0.9179 | 122.912 | 0.9436 | 6.860 | 116.994 |
| Whole-fruit rosehip | 0.40 | 125.784 | 1.588 | 1.263 | 0.9275 | 130.299 | 0.9355 | 3.589 | 126.913 |
| Whole-fruit rosehip | 0.50 | 131.465 | 1.637 | 1.245 | 0.9401 | 136.177 | 0.9629 | 3.584 | 132.643 |
| Whole-fruit rosehip | 0.60 | 139.037 | 1.599 | 1.150 | 0.9435 | 143.603 | 0.9324 | 3.284 | 140.179 |
| Whole-fruit rosehip | 0.70 | 103.581 | 2.838 | 2.740 | 0.8220 | 41.660 | 0.1531 | 59.781 | 88.101 |
| Whole-fruit rosehip | 0.80 | 50.972 | 4.900 | 9.613 | 0.4430 | 35.132 | 0.2427 | 31.076 | 47.012 |
| Whole-fruit rosehip | 0.90 | 39.458 | 5.957 | 15.097 | 0.2468 | 41.674 | 0.2450 | 5.617 | 40.012 |

Table S20. Conversion-level correspondence with TG–DTG stages and domains.

| Sample | α | Mean Tα (°C) | Min Tα | Max Tα | Median rel. DTG | TG–DTG stage | DTG domain |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 0.10 | 253.650 | 236.424 | 266.478 | 0.2736 | Stage II: 150-400 °C | Moderate-rate DTG domain |
| Rosehip seed | 0.20 | 279.636 | 264.247 | 292.691 | 0.4993 | Stage II: 150-400 °C | Moderate-rate DTG domain |
| Rosehip seed | 0.30 | 301.590 | 287.213 | 314.406 | 0.4706 | Stage II: 150-400 °C | Moderate-rate DTG domain |
| Rosehip seed | 0.40 | 323.496 | 308.805 | 336.247 | 0.5291 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip seed | 0.50 | 339.635 | 324.397 | 352.183 | 0.8316 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip seed | 0.60 | 350.665 | 335.109 | 363.524 | 0.9682 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip seed | 0.70 | 365.908 | 351.456 | 379.507 | 0.4136 | Stage II: 150-400 °C | Moderate-rate DTG domain |
| Rosehip seed | 0.80 | 405.797 | 389.913 | 419.453 | 0.2134 | Stage III: 400-700 °C | Moderate-rate DTG domain |
| Rosehip seed | 0.90 | 485.618 | 450.971 | 512.880 | 0.0818 | Stage III: 400-700 °C | DTG shoulder/transition |
| Rosehip skin | 0.10 | 183.823 | 174.024 | 192.777 | 0.8928 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip skin | 0.20 | 206.488 | 197.392 | 215.164 | 0.9649 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip skin | 0.30 | 230.604 | 223.805 | 236.366 | 0.7564 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip skin | 0.40 | 263.873 | 256.989 | 268.200 | 0.6099 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip skin | 0.50 | 296.280 | 286.386 | 302.762 | 0.7144 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip skin | 0.60 | 323.209 | 310.452 | 332.042 | 0.8812 | Stage II: 150-400 °C | High-rate DTG core |
| Rosehip skin | 0.70 | 358.970 | 340.727 | 376.000 | 0.4165 | Stage II: 150-400 °C | Moderate-rate DTG domain |
| Rosehip skin | 0.80 | 427.323 | 400.067 | 450.400 | 0.2856 | Stage III: 400-700 °C | Moderate-rate DTG domain |
| Rosehip skin | 0.90 | 508.575 | 481.383 | 531.000 | 0.1852 | Stage III: 400-700 °C | DTG shoulder/transition |
| Whole-fruit rosehip | 0.10 | 198.986 | 187.659 | 210.370 | 0.5046 | Stage II: 150-400 °C | High-rate DTG core |
| Whole-fruit rosehip | 0.20 | 235.395 | 224.963 | 248.165 | 0.4449 | Stage II: 150-400 °C | Moderate-rate DTG domain |
| Whole-fruit rosehip | 0.30 | 270.886 | 258.417 | 284.546 | 0.6335 | Stage II: 150-400 °C | High-rate DTG core |
| Whole-fruit rosehip | 0.40 | 297.224 | 284.415 | 311.057 | 0.7263 | Stage II: 150-400 °C | High-rate DTG core |
| Whole-fruit rosehip | 0.50 | 320.686 | 307.032 | 335.029 | 0.8523 | Stage II: 150-400 °C | High-rate DTG core |
| Whole-fruit rosehip | 0.60 | 340.230 | 326.300 | 354.905 | 0.9683 | Stage II: 150-400 °C | High-rate DTG core |
| Whole-fruit rosehip | 0.70 | 361.513 | 345.015 | 384.138 | 0.7340 | Stage II: 150-400 °C | High-rate DTG core |
| Whole-fruit rosehip | 0.80 | 412.835 | 391.509 | 458.022 | 0.2879 | Stage III: 400-700 °C | Moderate-rate DTG domain |
| Whole-fruit rosehip | 0.90 | 491.305 | 463.743 | 547.086 | 0.1707 | Stage III: 400-700 °C | DTG shoulder/transition |

Table S21. Principal reliable conversion intervals, reliable isolated points, and low-confidence conversion regions.

| Sample | Principal reliable interval | Reliable isolated point(s) | Low-confidence conversion region(s) |
| --- | --- | --- | --- |
| Rosehip seed | α = 0.10–0.70 |  | α = 0.80–0.90 |
| Rosehip skin | α = 0.40–0.60 | α = 0.10 | α = 0.20–0.30; α = 0.70–0.90 |
| Whole-fruit rosehip | α = 0.30–0.60 | α = 0.10 | α = 0.20; α = 0.70–0.90 |

Table S22. Principal-reliable-interval means, ranges, conversion dependence, and uncertainty.

| Sample | Series | Principal reliable interval | nα | Mean Eₐ | Min Eₐ | Max Eₐ | Range | SD across α | SE of mean | CI low | CI high | RMS pointwise SE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | Friedman | α = 0.10–0.70 | 7 | 139.238 | 109.029 | 150.079 | 41.050 | 14.492 | 5.478 | 125.835 | 152.641 | 24.280 |
| Rosehip seed | KAS | α = 0.10–0.70 | 7 | 132.393 | 98.010 | 146.095 | 48.085 | 18.288 | 6.912 | 115.480 | 149.307 | 13.518 |
| Rosehip seed | FWO | α = 0.10–0.70 | 7 | 135.165 | 101.456 | 148.994 | 47.537 | 18.003 | 6.804 | 118.516 | 151.815 | 12.828 |
| Rosehip seed | Starink | α = 0.10–0.70 | 7 | 132.678 | 98.279 | 146.402 | 48.123 | 18.299 | 6.916 | 115.754 | 149.602 | 13.507 |
| Rosehip seed | Integral-method mean | α = 0.10–0.70 | 7 | 133.412 | 99.248 | 147.164 | 47.915 | 18.196 | 6.878 | 116.584 | 150.241 |  |
| Rosehip seed | Four-method descriptive mean | α = 0.10–0.70 | 7 | 134.869 | 101.693 | 147.892 | 46.199 | 17.161 | 6.486 | 118.998 | 150.740 |  |
| Rosehip skin | Friedman | α = 0.40–0.60 | 3 | 186.157 | 134.549 | 257.429 | 122.880 | 63.756 | 36.810 | 27.778 | 344.536 | 34.401 |
| Rosehip skin | KAS | α = 0.40–0.60 | 3 | 207.221 | 151.469 | 278.928 | 127.459 | 65.211 | 37.649 | 45.229 | 369.213 | 28.949 |
| Rosehip skin | FWO | α = 0.40–0.60 | 3 | 205.966 | 153.415 | 273.657 | 120.241 | 61.534 | 35.527 | 53.107 | 358.825 | 27.518 |
| Rosehip skin | Starink | α = 0.40–0.60 | 3 | 207.431 | 151.743 | 279.061 | 127.318 | 65.139 | 37.608 | 45.617 | 369.245 | 28.926 |
| Rosehip skin | Integral-method mean | α = 0.40–0.60 | 3 | 206.873 | 152.209 | 277.215 | 125.006 | 63.961 | 36.928 | 47.984 | 365.761 |  |
| Rosehip skin | Four-method descriptive mean | α = 0.40–0.60 | 3 | 201.694 | 147.794 | 272.269 | 124.475 | 63.891 | 36.887 | 42.980 | 360.407 |  |
| Whole-fruit rosehip | Friedman | α = 0.30–0.60 | 4 | 133.248 | 122.912 | 143.603 | 20.692 | 8.781 | 4.391 | 119.274 | 147.221 | 23.138 |
| Whole-fruit rosehip | KAS | α = 0.30–0.60 | 4 | 126.753 | 113.932 | 137.970 | 24.038 | 10.123 | 5.061 | 110.645 | 142.860 | 23.984 |
| Whole-fruit rosehip | FWO | α = 0.30–0.60 | 4 | 129.691 | 116.928 | 140.875 | 23.947 | 10.094 | 5.047 | 113.629 | 145.752 | 22.764 |
| Whole-fruit rosehip | Starink | α = 0.30–0.60 | 4 | 127.037 | 114.203 | 138.267 | 24.064 | 10.134 | 5.067 | 110.911 | 143.164 | 23.963 |
| Whole-fruit rosehip | Integral-method mean | α = 0.30–0.60 | 4 | 127.827 | 115.021 | 139.037 | 24.016 | 10.117 | 5.059 | 111.728 | 143.925 |  |
| Whole-fruit rosehip | Four-method descriptive mean | α = 0.30–0.60 | 4 | 129.182 | 116.994 | 140.179 | 23.185 | 9.774 | 4.887 | 113.629 | 144.735 |  |

# S9. Supplementary information for Section 3.2: Distributed-reactivity modelling and model-adequacy assessment

## S9.1. Data audit and validation eligibility

Table S23. Preflight data audit and calibration/prediction eligibility for the curves used in the DAEM assessment.

| Feedstock | β (°C min⁻¹) | Rows | TG at 150 °C (%) | TG at 700 °C (%) | Normalized mass at 700 °C | Upward steps > 0.05 pp |
| --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 551 | 96.20 | 25.98 | 0.270062 | 0 |
| Rosehip seed | 10 | 551 | 96.84 | 26.82 | 0.276952 | 0 |
| Rosehip seed | 15 | 551 | 97.45 | 24.78 | 0.254284 | 0 |
| Rosehip seed | 20 | 551 | 97.97 | 22.91 | 0.233847 | 0 |
| Rosehip skin | 5 | 551 | 93.67 | 33.40 | 0.356571 | 0 |
| Rosehip skin | 10 | 551 | 95.63 | 33.50 | 0.350308 | 0 |
| Rosehip skin | 15 | 551 | 96.89 | 29.39 | 0.303334 | 0 |
| Rosehip skin | 20 | 551 | 98.32 | 30.00 | 0.305126 | 0 |
| Whole-fruit rosehip | 5 | 551 | 92.11 | 29.19 | 0.316904 | 0 |
| Whole-fruit rosehip | 10 | 551 | 93.64 | 29.47 | 0.314716 | 0 |
| Whole-fruit rosehip | 15 | 551 | 94.12 | 28.74 | 0.305355 | 0 |

Note. Every included curve contains 551 points over 150–700 °C and no upward TG step larger than 0.05 percentage points. The whole-fruit 20 °C min⁻¹ curve was excluded from DAEM calibration because it exceeded the predefined R₇₀₀ consistency criterion. Consequently, the whole-fruit K = 3 result is a three-rate calibration model and does not support a four-rate leave-one-heating-rate-out validation design.

## S9.2. Candidate-model comparison and stability evidence

Table S24. Calibration and information-criterion comparison for K = 1–3 candidate DAEMs.

| Feedstock | K | Rates | Parameters | Global RMSE (pp) | Pooled SSE (fraction) | Max. stage ¦bias¦ (pp) | AICc | ΔAICc | BIC | ΔBIC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 1 | 4 | 4 | 2.025 | 0.903893 | 0.334 | -17181.14 | 3312.18 | -17158.37 | 3266.72 |
| Rosehip seed | 2 | 4 | 8 | 1.095 | 0.264493 | 0.198 | -19881.58 | 611.75 | -19836.06 | 589.03 |
| Rosehip seed | 3 | 4 | 12 | 0.952 | 0.199655 | 0.072 | -20493.32 | 0.00 | -20425.09 | 0.00 |
| Rosehip skin | 1 | 4 | 4 | 2.693 | 1.598097 | 0.579 | -15925.17 | 2828.53 | -15902.40 | 2783.07 |
| Rosehip skin | 2 | 4 | 8 | 1.713 | 0.646460 | 0.183 | -17911.87 | 841.83 | -17866.35 | 819.11 |
| Rosehip skin | 3 | 4 | 12 | 1.412 | 0.439612 | 0.046 | -18753.70 | 0.00 | -18685.47 | 0.00 |
| Whole-fruit rosehip | 1 | 3 | 4 | 1.604 | 0.425375 | 0.408 | -13654.24 | 2118.25 | -13632.62 | 2075.14 |
| Whole-fruit rosehip | 2 | 3 | 8 | 1.173 | 0.227343 | 0.075 | -14681.80 | 1090.69 | -14638.60 | 1069.15 |
| Whole-fruit rosehip | 3 | 3 | 12 | 0.841 | 0.116948 | 0.151 | -15772.49 | 0.00 | -15707.76 | 0.00 |

Table S25. Multistart convergence, bound-contact diagnostics, and candidate-level LOO performance.

| Feedstock | K | Near-optimal n | μ spread (kJ mol⁻¹) | σ spread (kJ mol⁻¹) | log₁₀A spread | Capacity spread | LOO mean RMSE (pp) | LOO max RMSE (pp) | LOO max ¦error¦ (pp) | Mean ¦T₅₀ error¦ (K) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 1 | 12 | 0.000 | 0.000 | 0.000 | 0.0000 | 2.133 | 2.663 | 6.557 | 4.035 |
| Rosehip seed | 2 | 6 | 0.022 | 0.003 | 0.002 | 0.0000 | 1.315 | 2.003 | 3.694 | 3.270 |
| Rosehip seed | 3 | 4 | 61.248 | 7.155 | 5.311 | 0.2755 | 1.219 | 1.962 | 3.912 | 3.151 |
| Rosehip skin | 1 | 12 | 0.002 | 0.000 | 0.000 | 0.0000 | 3.228 | 3.821 | 6.613 | 6.222 |
| Rosehip skin | 2 | 2 | 0.001 | 0.000 | 0.000 | 0.0000 | 2.126 | 2.436 | 4.630 | 7.399 |
| Rosehip skin | 3 | 4 | 0.742 | 7.771 | 0.655 | 0.0409 | 1.846 | 2.140 | 4.576 | 3.638 |
| Whole-fruit rosehip | 1 | 12 | 0.001 | 0.000 | 0.000 | 0.0000 | — | — | — | — |
| Whole-fruit rosehip | 2 | 5 | 77.901 | 27.936 | 3.151 | 0.0432 | — | — | — | — |
| Whole-fruit rosehip | 3 | 4 | 13.755 | 11.963 | 0.812 | 0.0597 | — | — | — | — |

## S9.3. Higher-complexity K = 3 diagnostic candidates and performance metrics

Table S26. Parameters of the higher-complexity K = 3 diagnostic candidates for seed and skin and the retained three-rate K = 3 calibration model for whole-fruit rosehip.

| Feedstock | Domain | μ (kJ mol⁻¹) | σ (kJ mol⁻¹) | log₁₀(A/min⁻¹) | A (min⁻¹) | Capacity cⱼ | Residual fraction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 1 | 146.052731 | 14.299999 | 12.536741 | 3.4414e+12 | 0.495801 | 0.259750 |
| Rosehip seed | 2 | 244.526103 | 2.000000 | 20.421389 | 2.6387e+20 | 0.099353 | — |
| Rosehip seed | 3 | 330.000000 | 49.423038 | 22.328305 | 2.1296e+22 | 0.145096 | — |
| Rosehip skin | 1 | 227.967045 | 14.047582 | 25.000000 | 1.0000e+25 | 0.230758 | 0.339013 |
| Rosehip skin | 2 | 278.334496 | 18.762328 | 25.000000 | 1.0000e+25 | 0.223264 | — |
| Rosehip skin | 3 | 330.000000 | 45.739326 | 23.468449 | 2.9407e+23 | 0.206965 | — |
| Whole-fruit rosehip | 1 | 128.496524 | 8.506626 | 13.599298 | 3.9746e+13 | 0.157778 | 0.317601 |
| Whole-fruit rosehip | 2 | 195.692746 | 12.667979 | 16.856814 | 7.1914e+16 | 0.311480 | — |
| Whole-fruit rosehip | 3 | 316.244920 | 54.297238 | 23.659394 | 4.5645e+23 | 0.213142 | — |

Note. Domains are ordered by μ and are formal kinetic reactivity domains. Capacity plus residual fraction closes to unity for each feedstock. Boundary contacts are evident for seed (μ₃ and σ₂) and skin (μ₃ and log₁₀A₁/log₁₀A₂).

Table S27. Heating-rate-specific calibration metrics for the K = 3 diagnostic candidates and whole-fruit calibration model.

| Feedstock | β (°C min⁻¹) | RMSE (pp) | MAE (pp) | Maximum ¦error¦ (pp) | Mean signed error (pp) |
| --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 0.694 | 0.577 | 1.811 | 0.155 |
| Rosehip seed | 10 | 0.914 | 0.754 | 1.626 | 0.684 |
| Rosehip seed | 15 | 0.560 | 0.469 | 1.106 | 0.067 |
| Rosehip seed | 20 | 1.411 | 1.178 | 3.202 | -0.964 |
| Rosehip skin | 5 | 1.590 | 1.342 | 3.019 | 1.246 |
| Rosehip skin | 10 | 1.120 | 0.969 | 2.132 | 0.969 |
| Rosehip skin | 15 | 1.403 | 1.092 | 3.753 | -1.038 |
| Rosehip skin | 20 | 1.493 | 1.317 | 3.591 | -1.156 |
| Whole-fruit rosehip | 5 | 0.681 | 0.549 | 1.628 | 0.475 |
| Whole-fruit rosehip | 10 | 0.678 | 0.563 | 1.989 | 0.181 |
| Whole-fruit rosehip | 15 | 1.095 | 0.912 | 2.584 | -0.640 |

Note. Signed error is 100 × (observed − predicted). Positive values therefore indicate underprediction of normalized residual mass, while negative values indicate overprediction.

Table S28. Complete leave-one-heating-rate-out metrics for the higher-complexity K = 3 diagnostic candidates of rosehip seed and skin.

| Feedstock | Held β (°C min⁻¹) | RMSE (pp) | MAE (pp) | Maximum ¦error¦ (pp) | T₁₀ error (K) | T₅₀ error (K) | T₉₀ error (K) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 5 | 1.006 | 0.826 | 3.377 | 4.248 | 3.611 | 30.330 |
| Rosehip seed | 10 | 1.186 | 0.980 | 2.103 | -6.740 | -0.220 | 17.762 |
| Rosehip seed | 15 | 0.721 | 0.607 | 1.431 | -1.369 | -3.067 | -34.263 |
| Rosehip seed | 20 | 1.962 | 1.640 | 3.912 | -7.083 | -5.705 | -22.524 |
| Rosehip skin | 5 | 2.140 | 1.807 | 3.894 | 5.304 | 0.016 | 18.789 |
| Rosehip skin | 10 | 1.469 | 1.292 | 2.663 | 0.302 | -1.717 | 11.877 |
| Rosehip skin | 15 | 1.831 | 1.433 | 4.576 | 1.420 | -8.141 | -30.753 |
| Rosehip skin | 20 | 1.942 | 1.742 | 4.288 | -5.508 | -4.680 | -20.379 |

Note. Each held-rate curve was excluded in full from fitting and predicted from the remaining three rates. Temperature errors are predicted minus observed values as exported by the analysis. Whole-fruit rosehip has no corresponding LOO table because its 20 °C min⁻¹ curve was excluded before DAEM calibration.

## S9.4. Supplementary diagnostic figures S5–S15

Figures S5–S12 document the non-retained higher-complexity K = 3 candidates for seed and skin, whereas Figures S13–S15 document the retained three-rate K = 3 calibration model for whole-fruit rosehip. DTG panels are explicitly diagnostic because DTG was not included in the fitting objective.

Figure S5. TG calibration for the higher-complexity K = 3 rosehip-seed candidate over 150–700 °C. Solid lines are observed normalised residual mass and dashed lines are fitted values. Although the candidate reduced in-sample error, parameter instability and bound contact prevented its retention over the parsimonious K = 2 model.

Figure S6. Temperature-resolved signed residuals for the K = 3 rosehip-seed candidate, defined as 100 × (observed − predicted). The residual curves show heating-rate-dependent and structured departures rather than random scatter.

Figure S7. Complete leave-one-heating-rate-out prediction for the K = 3 rosehip-seed candidate. Each observed heating-rate curve is compared with a prediction obtained after excluding that complete rate from fitting. The modest numerical improvement relative to K = 2 did not offset the K = 3 parameter-instability evidence.

Figure S8. Regenerated observed DTG and model-derived DTG for the K = 3 rosehip-seed candidate. DTG was not fitted. Differences in peak magnitude and profile provide an independent diagnostic of derivative-shape inadequacy and must not be interpreted as validated component kinetics.

Figure S9. TG calibration for the higher-complexity K = 3 rosehip-skin candidate over 150–700 °C. Despite improved in-sample fit relative to lower-complexity candidates, bound contact, structured residuals, and derivative-shape mismatch prevented retention as the preferred representation.

Figure S10. Temperature-resolved signed residuals for the K = 3 rosehip-skin candidate. Persistent heating-rate-dependent sign changes and late-temperature drift show that the remaining errors are structured rather than numerical noise.

Figure S11. Complete leave-one-heating-rate-out prediction for the K = 3 rosehip-skin candidate. Prediction errors, together with bound contact in μ₃ and log₁₀A₁/log₁₀A₂ and loss of derivative-shape fidelity, prevented model retention.

Figure S12. Regenerated observed DTG and model-derived DTG for the K = 3 rosehip-skin candidate. DTG was not fitted. The substantial peak-shape and peak-intensity discrepancies provide independent evidence against mechanistic interpretation of the fitted domains.

Figure S13. TG calibration for the retained three-rate K = 3 whole-fruit rosehip model at 5, 10, and 15 °C min⁻¹. The 20 °C min⁻¹ curve was excluded from calibration under the predefined R₇₀₀ consistency criterion; therefore, this result is calibration-only and is not independently predictively validated.

Figure S14. Temperature-resolved signed residuals for the retained three-rate K = 3 whole-fruit rosehip calibration model. The structured residual patterns, particularly at 15 °C min⁻¹, limit interpretation beyond descriptive calibration.

Figure S15. Regenerated observed DTG and model-derived DTG for the retained three-rate K = 3 whole-fruit rosehip calibration model. DTG was not fitted, and the mismatch in peak structure demonstrates why a smooth TG calibration must not be treated as a validated or transferable kinetic representation.

# S10. Supplementary information for Section 3.1.4: Endpoint robustness of preparation-specific Eₐ(α) trajectories

## S10.1. Supplementary tables S29–S31

Table S29. Endpoint-conditioned fixed-conversion temperatures and Friedman rate inputs for rosehip seed, skin, and whole-fruit rosehip at terminal temperatures of 600, 650, and 700 °C.

| Feedstock | Tf (°C) | α | β (°C min⁻¹) | Tα (°C) | Tα (K) | dα/dt (min⁻¹) | Relative DTG |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 600 | 0.10 | 5 | 235.412 | 508.562 | 0.0126372 | 0.2406 |
| Rosehip seed | 600 | 0.10 | 10 | 253.710 | 526.860 | 0.0302431 | 0.2785 |
| Rosehip seed | 600 | 0.10 | 15 | 255.169 | 528.319 | 0.0369803 | 0.2495 |
| Rosehip seed | 600 | 0.10 | 20 | 264.553 | 537.703 | 0.0504973 | 0.2573 |
| Rosehip seed | 600 | 0.20 | 5 | 263.100 | 536.250 | 0.0231182 | 0.4401 |
| Rosehip seed | 600 | 0.20 | 10 | 277.071 | 550.221 | 0.0530261 | 0.4883 |
| Rosehip seed | 600 | 0.20 | 15 | 281.558 | 554.708 | 0.0752709 | 0.5079 |
| Rosehip seed | 600 | 0.20 | 20 | 290.729 | 563.879 | 0.1012942 | 0.5161 |
| Rosehip seed | 600 | 0.30 | 5 | 285.516 | 558.666 | 0.0234674 | 0.4468 |
| Rosehip seed | 600 | 0.30 | 10 | 297.352 | 570.502 | 0.0498353 | 0.4589 |
| Rosehip seed | 600 | 0.30 | 15 | 302.691 | 575.841 | 0.0722971 | 0.4878 |
| Rosehip seed | 600 | 0.30 | 20 | 311.359 | 584.509 | 0.0993504 | 0.5062 |
| Rosehip seed | 600 | 0.40 | 5 | 306.914 | 580.064 | 0.0270148 | 0.5143 |
| Rosehip seed | 600 | 0.40 | 10 | 318.465 | 591.615 | 0.0524403 | 0.4829 |
| Rosehip seed | 600 | 0.40 | 15 | 324.600 | 597.750 | 0.0743366 | 0.5016 |
| Rosehip seed | 600 | 0.40 | 20 | 332.517 | 605.667 | 0.1044833 | 0.5324 |
| Rosehip seed | 600 | 0.50 | 5 | 322.852 | 596.002 | 0.0413252 | 0.7868 |
| Rosehip seed | 600 | 0.50 | 10 | 334.865 | 608.015 | 0.0814564 | 0.7501 |
| Rosehip seed | 600 | 0.50 | 15 | 342.410 | 615.560 | 0.1113740 | 0.7515 |
| Rosehip seed | 600 | 0.50 | 20 | 349.182 | 622.332 | 0.1579742 | 0.8049 |
| Rosehip seed | 600 | 0.60 | 5 | 333.594 | 606.744 | 0.0520705 | 0.9913 |
| Rosehip seed | 600 | 0.60 | 10 | 345.544 | 618.694 | 0.1084741 | 0.9990 |
| Rosehip seed | 600 | 0.60 | 15 | 354.078 | 627.228 | 0.1481626 | 0.9998 |
| Rosehip seed | 600 | 0.60 | 20 | 360.476 | 633.626 | 0.1959265 | 0.9983 |
| Rosehip seed | 600 | 0.70 | 5 | 347.067 | 620.217 | 0.0238268 | 0.4536 |
| Rosehip seed | 600 | 0.70 | 10 | 356.443 | 629.593 | 0.0717705 | 0.6609 |
| Rosehip seed | 600 | 0.70 | 15 | 366.322 | 639.472 | 0.0918231 | 0.6196 |
| Rosehip seed | 600 | 0.70 | 20 | 372.723 | 645.873 | 0.1269481 | 0.6468 |
| Rosehip seed | 600 | 0.80 | 5 | 381.294 | 654.444 | 0.0123583 | 0.2353 |
| Rosehip seed | 600 | 0.80 | 10 | 386.440 | 659.590 | 0.0233124 | 0.2147 |
| Rosehip seed | 600 | 0.80 | 15 | 399.850 | 673.000 | 0.0338113 | 0.2282 |
| Rosehip seed | 600 | 0.80 | 20 | 402.659 | 675.809 | 0.0500588 | 0.2551 |
| Rosehip seed | 600 | 0.90 | 5 | 431.800 | 704.950 | 0.0072388 | 0.1378 |
| Rosehip seed | 600 | 0.90 | 10 | 441.867 | 715.017 | 0.0123286 | 0.1135 |
| Rosehip seed | 600 | 0.90 | 15 | 462.717 | 735.867 | 0.0147741 | 0.0997 |
| Rosehip seed | 600 | 0.90 | 20 | 459.700 | 732.850 | 0.0206240 | 0.1051 |
| Rosehip seed | 650 | 0.10 | 5 | 235.965 | 509.115 | 0.0126875 | 0.2449 |
| Rosehip seed | 650 | 0.10 | 10 | 254.270 | 527.420 | 0.0305381 | 0.2859 |
| Rosehip seed | 650 | 0.10 | 15 | 256.224 | 529.374 | 0.0376594 | 0.2604 |
| Rosehip seed | 650 | 0.10 | 20 | 265.617 | 538.767 | 0.0513635 | 0.2685 |
| Rosehip seed | 650 | 0.20 | 5 | 263.727 | 536.877 | 0.0228594 | 0.4412 |
| Rosehip seed | 650 | 0.20 | 10 | 277.729 | 550.879 | 0.0521874 | 0.4885 |
| Rosehip seed | 650 | 0.20 | 15 | 282.594 | 555.744 | 0.0736914 | 0.5095 |
| Rosehip seed | 650 | 0.20 | 20 | 291.806 | 564.956 | 0.0991861 | 0.5186 |
| Rosehip seed | 650 | 0.30 | 5 | 286.440 | 559.590 | 0.0231694 | 0.4472 |
| Rosehip seed | 650 | 0.30 | 10 | 298.381 | 571.531 | 0.0488673 | 0.4574 |
| Rosehip seed | 650 | 0.30 | 15 | 304.303 | 577.453 | 0.0702955 | 0.4860 |
| Rosehip seed | 650 | 0.30 | 20 | 313.009 | 586.159 | 0.0963795 | 0.5039 |
| Rosehip seed | 650 | 0.40 | 5 | 307.961 | 581.111 | 0.0271417 | 0.5238 |
| Rosehip seed | 650 | 0.40 | 10 | 319.760 | 592.910 | 0.0525933 | 0.4923 |
| Rosehip seed | 650 | 0.40 | 15 | 326.653 | 599.803 | 0.0742810 | 0.5136 |
| Rosehip seed | 650 | 0.40 | 20 | 334.578 | 607.728 | 0.1050216 | 0.5491 |
| Rosehip seed | 650 | 0.50 | 5 | 323.709 | 596.859 | 0.0420224 | 0.8110 |
| Rosehip seed | 650 | 0.50 | 10 | 335.891 | 609.041 | 0.0832850 | 0.7796 |
| Rosehip seed | 650 | 0.50 | 15 | 344.093 | 617.243 | 0.1152030 | 0.7965 |
| Rosehip seed | 650 | 0.50 | 20 | 350.851 | 624.001 | 0.1623565 | 0.8488 |
| Rosehip seed | 650 | 0.60 | 5 | 334.418 | 607.568 | 0.0506806 | 0.9781 |
| Rosehip seed | 650 | 0.60 | 10 | 346.485 | 619.635 | 0.1065584 | 0.9974 |
| Rosehip seed | 650 | 0.60 | 15 | 355.618 | 628.768 | 0.1434394 | 0.9917 |
| Rosehip seed | 650 | 0.60 | 20 | 362.128 | 635.278 | 0.1884686 | 0.9854 |
| Rosehip seed | 650 | 0.70 | 5 | 349.338 | 622.488 | 0.0204726 | 0.3951 |
| Rosehip seed | 650 | 0.70 | 10 | 358.220 | 631.370 | 0.0617426 | 0.5779 |
| Rosehip seed | 650 | 0.70 | 15 | 369.644 | 642.794 | 0.0708639 | 0.4899 |
| Rosehip seed | 650 | 0.70 | 20 | 376.079 | 649.229 | 0.1009481 | 0.5278 |
| Rosehip seed | 650 | 0.80 | 5 | 385.950 | 659.100 | 0.0119026 | 0.2297 |
| Rosehip seed | 650 | 0.80 | 10 | 392.413 | 665.563 | 0.0221300 | 0.2071 |
| Rosehip seed | 650 | 0.80 | 15 | 409.160 | 682.310 | 0.0313193 | 0.2165 |
| Rosehip seed | 650 | 0.80 | 20 | 411.613 | 684.763 | 0.0457802 | 0.2394 |
| Rosehip seed | 650 | 0.90 | 5 | 441.450 | 714.600 | 0.0059682 | 0.1152 |
| Rosehip seed | 650 | 0.90 | 10 | 455.933 | 729.083 | 0.0094972 | 0.0889 |
| Rosehip seed | 650 | 0.90 | 15 | 488.700 | 761.850 | 0.0119335 | 0.0825 |
| Rosehip seed | 650 | 0.90 | 20 | 486.983 | 760.133 | 0.0155088 | 0.0811 |
| Rosehip seed | 700 | 0.10 | 5 | 236.424 | 509.574 | 0.0127547 | 0.2489 |
| Rosehip seed | 700 | 0.10 | 10 | 254.660 | 527.810 | 0.0307438 | 0.2910 |
| Rosehip seed | 700 | 0.10 | 15 | 257.039 | 530.189 | 0.0381653 | 0.2690 |
| Rosehip seed | 700 | 0.10 | 20 | 266.478 | 539.628 | 0.0520976 | 0.2781 |
| Rosehip seed | 700 | 0.20 | 5 | 264.247 | 537.397 | 0.0226429 | 0.4419 |
| Rosehip seed | 700 | 0.20 | 10 | 278.183 | 551.333 | 0.0516086 | 0.4885 |
| Rosehip seed | 700 | 0.20 | 15 | 283.424 | 556.574 | 0.0723778 | 0.5102 |
| Rosehip seed | 700 | 0.20 | 20 | 292.691 | 565.841 | 0.0974090 | 0.5200 |
| Rosehip seed | 700 | 0.30 | 5 | 287.213 | 560.363 | 0.0229340 | 0.4476 |
| Rosehip seed | 700 | 0.30 | 10 | 299.113 | 572.263 | 0.0482257 | 0.4565 |
| Rosehip seed | 700 | 0.30 | 15 | 305.628 | 578.778 | 0.0687590 | 0.4846 |
| Rosehip seed | 700 | 0.30 | 20 | 314.406 | 587.556 | 0.0940969 | 0.5023 |
| Rosehip seed | 700 | 0.40 | 5 | 308.805 | 581.955 | 0.0272943 | 0.5327 |
| Rosehip seed | 700 | 0.40 | 10 | 320.651 | 593.801 | 0.0527762 | 0.4996 |
| Rosehip seed | 700 | 0.40 | 15 | 328.280 | 601.430 | 0.0745573 | 0.5255 |
| Rosehip seed | 700 | 0.40 | 20 | 336.247 | 609.397 | 0.1059165 | 0.5654 |
| Rosehip seed | 700 | 0.50 | 5 | 324.397 | 597.547 | 0.0425697 | 0.8308 |
| Rosehip seed | 700 | 0.50 | 10 | 336.589 | 609.739 | 0.0845506 | 0.8004 |
| Rosehip seed | 700 | 0.50 | 15 | 345.373 | 618.523 | 0.1180881 | 0.8323 |
| Rosehip seed | 700 | 0.50 | 20 | 352.183 | 625.333 | 0.1653899 | 0.8829 |
| Rosehip seed | 700 | 0.60 | 5 | 335.109 | 608.259 | 0.0493226 | 0.9626 |
| Rosehip seed | 700 | 0.60 | 10 | 347.144 | 620.294 | 0.1048976 | 0.9930 |
| Rosehip seed | 700 | 0.60 | 15 | 356.882 | 630.032 | 0.1381601 | 0.9738 |
| Rosehip seed | 700 | 0.60 | 20 | 363.524 | 636.674 | 0.1802287 | 0.9622 |
| Rosehip seed | 700 | 0.70 | 5 | 351.456 | 624.606 | 0.0183245 | 0.3576 |
| Rosehip seed | 700 | 0.70 | 10 | 359.632 | 632.782 | 0.0546845 | 0.5176 |
| Rosehip seed | 700 | 0.70 | 15 | 373.036 | 646.186 | 0.0557005 | 0.3926 |
| Rosehip seed | 700 | 0.70 | 20 | 379.507 | 652.657 | 0.0814043 | 0.4346 |
| Rosehip seed | 700 | 0.80 | 5 | 389.913 | 663.062 | 0.0115689 | 0.2258 |
| Rosehip seed | 700 | 0.80 | 10 | 396.707 | 669.857 | 0.0213828 | 0.2024 |
| Rosehip seed | 700 | 0.80 | 15 | 417.114 | 690.264 | 0.0288742 | 0.2035 |
| Rosehip seed | 700 | 0.80 | 20 | 419.453 | 692.603 | 0.0418337 | 0.2233 |
| Rosehip seed | 700 | 0.90 | 5 | 450.971 | 724.121 | 0.0049189 | 0.0960 |
| Rosehip seed | 700 | 0.90 | 10 | 467.760 | 740.910 | 0.0082045 | 0.0777 |
| Rosehip seed | 700 | 0.90 | 15 | 510.860 | 784.010 | 0.0121967 | 0.0860 |
| Rosehip seed | 700 | 0.90 | 20 | 512.880 | 786.030 | 0.0145366 | 0.0776 |
| Rosehip skin | 600 | 0.10 | 5 | 173.328 | 446.478 | 0.0260033 | 0.9991 |
| Rosehip skin | 600 | 0.10 | 10 | 182.128 | 455.278 | 0.0435580 | 0.9205 |
| Rosehip skin | 600 | 0.10 | 15 | 184.304 | 457.454 | 0.0672377 | 0.8312 |
| Rosehip skin | 600 | 0.10 | 20 | 191.417 | 464.567 | 0.0794684 | 0.7394 |
| Rosehip skin | 600 | 0.20 | 5 | 195.708 | 468.858 | 0.0212411 | 0.8162 |
| Rosehip skin | 600 | 0.20 | 10 | 205.092 | 478.242 | 0.0462715 | 0.9779 |
| Rosehip skin | 600 | 0.20 | 15 | 204.561 | 477.711 | 0.0778500 | 0.9624 |
| Rosehip skin | 600 | 0.20 | 20 | 213.061 | 486.211 | 0.1054559 | 0.9812 |
| Rosehip skin | 600 | 0.30 | 5 | 220.775 | 493.925 | 0.0183185 | 0.7039 |
| Rosehip skin | 600 | 0.30 | 10 | 228.391 | 501.541 | 0.0388337 | 0.8207 |
| Rosehip skin | 600 | 0.30 | 15 | 226.831 | 499.981 | 0.0628895 | 0.7774 |
| Rosehip skin | 600 | 0.30 | 20 | 232.903 | 506.053 | 0.0993613 | 0.9245 |
| Rosehip skin | 600 | 0.40 | 5 | 252.611 | 525.761 | 0.0165082 | 0.6343 |
| Rosehip skin | 600 | 0.40 | 10 | 259.305 | 532.455 | 0.0333694 | 0.7052 |
| Rosehip skin | 600 | 0.40 | 15 | 258.822 | 531.972 | 0.0427196 | 0.5281 |
| Rosehip skin | 600 | 0.40 | 20 | 261.000 | 534.150 | 0.0617874 | 0.5749 |
| Rosehip skin | 600 | 0.50 | 5 | 281.881 | 555.031 | 0.0197605 | 0.7593 |
| Rosehip skin | 600 | 0.50 | 10 | 288.826 | 561.976 | 0.0386770 | 0.8174 |
| Rosehip skin | 600 | 0.50 | 15 | 294.053 | 567.203 | 0.0483304 | 0.5974 |
| Rosehip skin | 600 | 0.50 | 20 | 294.550 | 567.700 | 0.0657589 | 0.6119 |
| Rosehip skin | 600 | 0.60 | 5 | 306.037 | 579.187 | 0.0240769 | 0.9251 |
| Rosehip skin | 600 | 0.60 | 10 | 313.896 | 587.046 | 0.0463063 | 0.9786 |
| Rosehip skin | 600 | 0.60 | 15 | 323.842 | 596.992 | 0.0589312 | 0.7285 |
| Rosehip skin | 600 | 0.60 | 20 | 323.760 | 596.910 | 0.0785904 | 0.7313 |
| Rosehip skin | 600 | 0.70 | 5 | 331.719 | 604.869 | 0.0146003 | 0.5610 |
| Rosehip skin | 600 | 0.70 | 10 | 340.627 | 613.777 | 0.0266320 | 0.5628 |
| Rosehip skin | 600 | 0.70 | 15 | 355.743 | 628.893 | 0.0324699 | 0.4014 |
| Rosehip skin | 600 | 0.70 | 20 | 349.824 | 622.974 | 0.0541104 | 0.5035 |
| Rosehip skin | 600 | 0.80 | 5 | 381.667 | 654.817 | 0.0079914 | 0.3071 |
| Rosehip skin | 600 | 0.80 | 10 | 393.400 | 666.550 | 0.0154942 | 0.3275 |
| Rosehip skin | 600 | 0.80 | 15 | 419.844 | 692.994 | 0.0207184 | 0.2561 |
| Rosehip skin | 600 | 0.80 | 20 | 410.750 | 683.900 | 0.0262096 | 0.2439 |
| Rosehip skin | 600 | 0.90 | 5 | 452.929 | 726.079 | 0.0062834 | 0.2414 |
| Rosehip skin | 600 | 0.90 | 10 | 465.475 | 738.625 | 0.0133663 | 0.2825 |
| Rosehip skin | 600 | 0.90 | 15 | 490.422 | 763.572 | 0.0208318 | 0.2575 |
| Rosehip skin | 600 | 0.90 | 20 | 487.125 | 760.275 | 0.0267200 | 0.2486 |
| Rosehip skin | 650 | 0.10 | 5 | 173.710 | 446.860 | 0.0254912 | 0.9981 |
| Rosehip skin | 650 | 0.10 | 10 | 182.600 | 455.750 | 0.0429804 | 0.9262 |
| Rosehip skin | 650 | 0.10 | 15 | 184.944 | 458.094 | 0.0664735 | 0.8439 |
| Rosehip skin | 650 | 0.10 | 20 | 192.138 | 465.288 | 0.0788645 | 0.7537 |
| Rosehip skin | 650 | 0.20 | 5 | 196.633 | 469.783 | 0.0208496 | 0.8164 |
| Rosehip skin | 650 | 0.20 | 10 | 206.000 | 479.150 | 0.0457353 | 0.9856 |
| Rosehip skin | 650 | 0.20 | 15 | 205.656 | 478.806 | 0.0750465 | 0.9527 |
| Rosehip skin | 650 | 0.20 | 20 | 214.131 | 487.281 | 0.1029607 | 0.9840 |
| Rosehip skin | 650 | 0.30 | 5 | 222.411 | 495.561 | 0.0175344 | 0.6866 |
| Rosehip skin | 650 | 0.30 | 10 | 230.045 | 503.195 | 0.0370722 | 0.7989 |
| Rosehip skin | 650 | 0.30 | 15 | 228.900 | 502.050 | 0.0593364 | 0.7533 |
| Rosehip skin | 650 | 0.30 | 20 | 234.660 | 507.810 | 0.0935221 | 0.8938 |
| Rosehip skin | 650 | 0.40 | 5 | 255.022 | 528.172 | 0.0163585 | 0.6405 |
| Rosehip skin | 650 | 0.40 | 10 | 261.789 | 534.939 | 0.0329438 | 0.7099 |
| Rosehip skin | 650 | 0.40 | 15 | 262.778 | 535.928 | 0.0417548 | 0.5301 |
| Rosehip skin | 650 | 0.40 | 20 | 264.705 | 537.855 | 0.0599248 | 0.5727 |
| Rosehip skin | 650 | 0.50 | 5 | 284.364 | 557.514 | 0.0197654 | 0.7739 |
| Rosehip skin | 650 | 0.50 | 10 | 291.478 | 564.628 | 0.0385330 | 0.8304 |
| Rosehip skin | 650 | 0.50 | 15 | 298.405 | 571.555 | 0.0482442 | 0.6125 |
| Rosehip skin | 650 | 0.50 | 20 | 298.810 | 571.960 | 0.0657316 | 0.6282 |
| Rosehip skin | 650 | 0.60 | 5 | 308.486 | 581.636 | 0.0240966 | 0.9435 |
| Rosehip skin | 650 | 0.60 | 10 | 316.519 | 589.669 | 0.0461220 | 0.9939 |
| Rosehip skin | 650 | 0.60 | 15 | 328.040 | 601.190 | 0.0590439 | 0.7496 |
| Rosehip skin | 650 | 0.60 | 20 | 327.985 | 601.135 | 0.0806610 | 0.7709 |
| Rosehip skin | 650 | 0.70 | 5 | 336.575 | 609.725 | 0.0136965 | 0.5363 |
| Rosehip skin | 650 | 0.70 | 10 | 346.267 | 619.417 | 0.0242070 | 0.5216 |
| Rosehip skin | 650 | 0.70 | 15 | 365.500 | 638.650 | 0.0278740 | 0.3539 |
| Rosehip skin | 650 | 0.70 | 20 | 358.477 | 631.627 | 0.0402606 | 0.3848 |
| Rosehip skin | 650 | 0.80 | 5 | 391.644 | 664.794 | 0.0076883 | 0.3010 |
| Rosehip skin | 650 | 0.80 | 10 | 404.250 | 677.400 | 0.0141720 | 0.3054 |
| Rosehip skin | 650 | 0.80 | 15 | 436.111 | 709.261 | 0.0204083 | 0.2591 |
| Rosehip skin | 650 | 0.80 | 20 | 428.850 | 702.000 | 0.0234578 | 0.2242 |
| Rosehip skin | 650 | 0.90 | 5 | 467.629 | 740.779 | 0.0055353 | 0.2167 |
| Rosehip skin | 650 | 0.90 | 10 | 480.000 | 753.150 | 0.0114330 | 0.2464 |
| Rosehip skin | 650 | 0.90 | 15 | 509.071 | 782.221 | 0.0180627 | 0.2293 |
| Rosehip skin | 650 | 0.90 | 20 | 506.800 | 779.950 | 0.0231983 | 0.2217 |
| Rosehip skin | 700 | 0.10 | 5 | 174.024 | 447.174 | 0.0250851 | 0.9973 |
| Rosehip skin | 700 | 0.10 | 10 | 182.972 | 456.122 | 0.0425406 | 0.9307 |
| Rosehip skin | 700 | 0.10 | 15 | 185.517 | 458.667 | 0.0656996 | 0.8550 |
| Rosehip skin | 700 | 0.10 | 20 | 192.777 | 465.927 | 0.0782379 | 0.7663 |
| Rosehip skin | 700 | 0.20 | 5 | 197.392 | 470.542 | 0.0205344 | 0.8164 |
| Rosehip skin | 700 | 0.20 | 10 | 206.689 | 479.839 | 0.0452835 | 0.9907 |
| Rosehip skin | 700 | 0.20 | 15 | 206.710 | 479.860 | 0.0724836 | 0.9433 |
| Rosehip skin | 700 | 0.20 | 20 | 215.164 | 488.314 | 0.1007287 | 0.9866 |
| Rosehip skin | 700 | 0.30 | 5 | 223.805 | 496.955 | 0.0169226 | 0.6728 |
| Rosehip skin | 700 | 0.30 | 10 | 231.329 | 504.479 | 0.0359645 | 0.7868 |
| Rosehip skin | 700 | 0.30 | 15 | 230.917 | 504.067 | 0.0557864 | 0.7260 |
| Rosehip skin | 700 | 0.30 | 20 | 236.366 | 509.516 | 0.0879726 | 0.8617 |
| Rosehip skin | 700 | 0.40 | 5 | 256.989 | 530.139 | 0.0162744 | 0.6470 |
| Rosehip skin | 700 | 0.40 | 10 | 263.747 | 536.897 | 0.0326089 | 0.7134 |
| Rosehip skin | 700 | 0.40 | 15 | 266.556 | 539.706 | 0.0409284 | 0.5326 |
| Rosehip skin | 700 | 0.40 | 20 | 268.200 | 541.350 | 0.0584836 | 0.5728 |
| Rosehip skin | 700 | 0.50 | 5 | 286.386 | 559.536 | 0.0197294 | 0.7844 |
| Rosehip skin | 700 | 0.50 | 10 | 293.543 | 566.693 | 0.0383957 | 0.8400 |
| Rosehip skin | 700 | 0.50 | 15 | 302.429 | 575.579 | 0.0482116 | 0.6274 |
| Rosehip skin | 700 | 0.50 | 20 | 302.762 | 575.912 | 0.0657836 | 0.6443 |
| Rosehip skin | 700 | 0.60 | 5 | 310.452 | 583.602 | 0.0238625 | 0.9487 |
| Rosehip skin | 700 | 0.60 | 10 | 318.585 | 591.735 | 0.0455026 | 0.9954 |
| Rosehip skin | 700 | 0.60 | 15 | 332.042 | 605.192 | 0.0564920 | 0.7351 |
| Rosehip skin | 700 | 0.60 | 20 | 331.757 | 604.907 | 0.0830840 | 0.8138 |
| Rosehip skin | 700 | 0.70 | 5 | 340.727 | 613.877 | 0.0128746 | 0.5118 |
| Rosehip skin | 700 | 0.70 | 10 | 350.936 | 624.086 | 0.0227249 | 0.4971 |
| Rosehip skin | 700 | 0.70 | 15 | 376.000 | 649.150 | 0.0239407 | 0.3115 |
| Rosehip skin | 700 | 0.70 | 20 | 368.218 | 641.368 | 0.0342974 | 0.3359 |
| Rosehip skin | 700 | 0.80 | 5 | 400.067 | 673.217 | 0.0073565 | 0.2925 |
| Rosehip skin | 700 | 0.80 | 10 | 413.425 | 686.575 | 0.0132271 | 0.2894 |
| Rosehip skin | 700 | 0.80 | 15 | 450.400 | 723.550 | 0.0216563 | 0.2818 |
| Rosehip skin | 700 | 0.80 | 20 | 445.400 | 718.550 | 0.0264725 | 0.2593 |
| Rosehip skin | 700 | 0.90 | 5 | 481.383 | 754.533 | 0.0048102 | 0.1912 |
| Rosehip skin | 700 | 0.90 | 10 | 492.950 | 766.100 | 0.0102339 | 0.2239 |
| Rosehip skin | 700 | 0.90 | 15 | 531.000 | 804.150 | 0.0137615 | 0.1791 |
| Rosehip skin | 700 | 0.90 | 20 | 528.967 | 802.117 | 0.0179352 | 0.1757 |
| Whole-fruit rosehip | 600 | 0.10 | 5 | 186.506 | 459.656 | 0.0145884 | 0.4715 |
| Whole-fruit rosehip | 600 | 0.10 | 10 | 193.330 | 466.480 | 0.0318267 | 0.5435 |
| Whole-fruit rosehip | 600 | 0.10 | 15 | 202.679 | 475.829 | 0.0449080 | 0.4802 |
| Whole-fruit rosehip | 600 | 0.10 | 20 | 208.220 | 481.370 | 0.0596037 | 0.5243 |
| Whole-fruit rosehip | 600 | 0.20 | 5 | 222.480 | 495.630 | 0.0137118 | 0.4432 |
| Whole-fruit rosehip | 600 | 0.20 | 10 | 226.365 | 499.515 | 0.0294280 | 0.5025 |
| Whole-fruit rosehip | 600 | 0.20 | 15 | 237.800 | 510.950 | 0.0403950 | 0.4319 |
| Whole-fruit rosehip | 600 | 0.20 | 20 | 243.211 | 516.361 | 0.0535890 | 0.4714 |
| Whole-fruit rosehip | 600 | 0.30 | 5 | 255.809 | 528.959 | 0.0191780 | 0.6199 |
| Whole-fruit rosehip | 600 | 0.30 | 10 | 261.945 | 535.095 | 0.0360123 | 0.6150 |
| Whole-fruit rosehip | 600 | 0.30 | 15 | 273.770 | 546.920 | 0.0559868 | 0.5986 |
| Whole-fruit rosehip | 600 | 0.30 | 20 | 279.648 | 552.798 | 0.0749991 | 0.6598 |
| Whole-fruit rosehip | 600 | 0.40 | 5 | 281.336 | 554.486 | 0.0217249 | 0.7022 |
| Whole-fruit rosehip | 600 | 0.40 | 10 | 288.176 | 561.326 | 0.0427435 | 0.7299 |
| Whole-fruit rosehip | 600 | 0.40 | 15 | 299.624 | 572.774 | 0.0637069 | 0.6812 |
| Whole-fruit rosehip | 600 | 0.40 | 20 | 305.164 | 578.314 | 0.0856485 | 0.7534 |
| Whole-fruit rosehip | 600 | 0.50 | 5 | 303.833 | 576.983 | 0.0257245 | 0.8315 |
| Whole-fruit rosehip | 600 | 0.50 | 10 | 311.500 | 584.650 | 0.0477960 | 0.8162 |
| Whole-fruit rosehip | 600 | 0.50 | 15 | 323.083 | 596.233 | 0.0730535 | 0.7811 |
| Whole-fruit rosehip | 600 | 0.50 | 20 | 328.677 | 601.827 | 0.0964523 | 0.8485 |
| Whole-fruit rosehip | 600 | 0.60 | 5 | 322.776 | 595.926 | 0.0283134 | 0.9152 |
| Whole-fruit rosehip | 600 | 0.60 | 10 | 331.282 | 604.432 | 0.0566252 | 0.9670 |
| Whole-fruit rosehip | 600 | 0.60 | 15 | 342.233 | 615.383 | 0.0879547 | 0.9404 |
| Whole-fruit rosehip | 600 | 0.60 | 20 | 348.300 | 621.450 | 0.1132683 | 0.9964 |
| Whole-fruit rosehip | 600 | 0.70 | 5 | 340.319 | 613.469 | 0.0275741 | 0.8913 |
| Whole-fruit rosehip | 600 | 0.70 | 10 | 349.475 | 622.625 | 0.0529971 | 0.9050 |
| Whole-fruit rosehip | 600 | 0.70 | 15 | 359.323 | 632.473 | 0.0867842 | 0.9279 |
| Whole-fruit rosehip | 600 | 0.70 | 20 | 367.993 | 641.143 | 0.0794724 | 0.6991 |
| Whole-fruit rosehip | 600 | 0.80 | 5 | 377.164 | 650.314 | 0.0095208 | 0.3077 |
| Whole-fruit rosehip | 600 | 0.80 | 10 | 387.180 | 660.330 | 0.0177564 | 0.3032 |
| Whole-fruit rosehip | 600 | 0.80 | 15 | 388.862 | 662.012 | 0.0322693 | 0.3450 |
| Whole-fruit rosehip | 600 | 0.80 | 20 | 420.356 | 693.506 | 0.0282354 | 0.2484 |
| Whole-fruit rosehip | 600 | 0.90 | 5 | 441.914 | 715.064 | 0.0062292 | 0.2013 |
| Whole-fruit rosehip | 600 | 0.90 | 10 | 457.629 | 730.779 | 0.0120465 | 0.2057 |
| Whole-fruit rosehip | 600 | 0.90 | 15 | 446.233 | 719.383 | 0.0216596 | 0.2316 |
| Whole-fruit rosehip | 600 | 0.90 | 20 | 498.622 | 771.772 | 0.0260309 | 0.2290 |
| Whole-fruit rosehip | 650 | 0.10 | 5 | 187.129 | 460.279 | 0.0142690 | 0.4692 |
| Whole-fruit rosehip | 650 | 0.10 | 10 | 193.900 | 467.050 | 0.0312085 | 0.5427 |
| Whole-fruit rosehip | 650 | 0.10 | 15 | 203.150 | 476.300 | 0.0443018 | 0.4802 |
| Whole-fruit rosehip | 650 | 0.10 | 20 | 209.384 | 482.534 | 0.0579885 | 0.5270 |
| Whole-fruit rosehip | 650 | 0.20 | 5 | 223.838 | 496.988 | 0.0134162 | 0.4412 |
| Whole-fruit rosehip | 650 | 0.20 | 10 | 227.706 | 500.856 | 0.0284342 | 0.4945 |
| Whole-fruit rosehip | 650 | 0.20 | 15 | 238.847 | 511.997 | 0.0393604 | 0.4267 |
| Whole-fruit rosehip | 650 | 0.20 | 20 | 245.859 | 519.009 | 0.0503236 | 0.4573 |
| Whole-fruit rosehip | 650 | 0.30 | 5 | 257.209 | 530.359 | 0.0190531 | 0.6265 |
| Whole-fruit rosehip | 650 | 0.30 | 10 | 263.545 | 536.695 | 0.0360462 | 0.6268 |
| Whole-fruit rosehip | 650 | 0.30 | 15 | 274.959 | 548.109 | 0.0560118 | 0.6072 |
| Whole-fruit rosehip | 650 | 0.30 | 20 | 282.342 | 555.492 | 0.0748944 | 0.6806 |
| Whole-fruit rosehip | 650 | 0.40 | 5 | 283.031 | 556.181 | 0.0215449 | 0.7085 |
| Whole-fruit rosehip | 650 | 0.40 | 10 | 290.000 | 563.150 | 0.0422907 | 0.7354 |
| Whole-fruit rosehip | 650 | 0.40 | 15 | 300.992 | 574.142 | 0.0631034 | 0.6840 |
| Whole-fruit rosehip | 650 | 0.40 | 20 | 308.386 | 581.536 | 0.0835425 | 0.7592 |
| Whole-fruit rosehip | 650 | 0.50 | 5 | 305.567 | 578.717 | 0.0257670 | 0.8473 |
| Whole-fruit rosehip | 650 | 0.50 | 10 | 313.483 | 586.633 | 0.0477491 | 0.8304 |
| Whole-fruit rosehip | 650 | 0.50 | 15 | 324.532 | 597.682 | 0.0734426 | 0.7961 |
| Whole-fruit rosehip | 650 | 0.50 | 20 | 332.197 | 605.347 | 0.0971758 | 0.8831 |
| Whole-fruit rosehip | 650 | 0.60 | 5 | 324.682 | 597.832 | 0.0281662 | 0.9262 |
| Whole-fruit rosehip | 650 | 0.60 | 10 | 333.294 | 606.444 | 0.0559325 | 0.9727 |
| Whole-fruit rosehip | 650 | 0.60 | 15 | 343.700 | 616.850 | 0.0876403 | 0.9500 |
| Whole-fruit rosehip | 650 | 0.60 | 20 | 351.916 | 625.066 | 0.1099609 | 0.9993 |
| Whole-fruit rosehip | 650 | 0.70 | 5 | 342.713 | 615.863 | 0.0244978 | 0.8056 |
| Whole-fruit rosehip | 650 | 0.70 | 10 | 352.107 | 625.257 | 0.0475959 | 0.8277 |
| Whole-fruit rosehip | 650 | 0.70 | 15 | 361.088 | 634.238 | 0.0819504 | 0.8883 |
| Whole-fruit rosehip | 650 | 0.70 | 20 | 375.394 | 648.544 | 0.0541601 | 0.4922 |
| Whole-fruit rosehip | 650 | 0.80 | 5 | 384.873 | 658.023 | 0.0091259 | 0.3001 |
| Whole-fruit rosehip | 650 | 0.80 | 10 | 396.000 | 669.150 | 0.0167299 | 0.2909 |
| Whole-fruit rosehip | 650 | 0.80 | 15 | 394.354 | 667.504 | 0.0302884 | 0.3283 |
| Whole-fruit rosehip | 650 | 0.80 | 20 | 440.600 | 713.750 | 0.0259248 | 0.2356 |
| Whole-fruit rosehip | 650 | 0.90 | 5 | 455.686 | 728.836 | 0.0053040 | 0.1744 |
| Whole-fruit rosehip | 650 | 0.90 | 10 | 472.667 | 745.817 | 0.0104397 | 0.1815 |
| Whole-fruit rosehip | 650 | 0.90 | 15 | 455.538 | 728.688 | 0.0194754 | 0.2111 |
| Whole-fruit rosehip | 650 | 0.90 | 20 | 523.087 | 796.237 | 0.0221250 | 0.2011 |
| Whole-fruit rosehip | 700 | 0.10 | 5 | 187.659 | 460.809 | 0.0140169 | 0.4676 |
| Whole-fruit rosehip | 700 | 0.10 | 10 | 194.372 | 467.522 | 0.0307660 | 0.5424 |
| Whole-fruit rosehip | 700 | 0.10 | 15 | 203.544 | 476.694 | 0.0438207 | 0.4802 |
| Whole-fruit rosehip | 700 | 0.10 | 20 | 210.370 | 483.520 | 0.0567018 | 0.5291 |
| Whole-fruit rosehip | 700 | 0.20 | 5 | 224.962 | 498.113 | 0.0132199 | 0.4410 |
| Whole-fruit rosehip | 700 | 0.20 | 10 | 228.729 | 501.879 | 0.0277524 | 0.4893 |
| Whole-fruit rosehip | 700 | 0.20 | 15 | 239.725 | 512.875 | 0.0385546 | 0.4225 |
| Whole-fruit rosehip | 700 | 0.20 | 20 | 248.165 | 521.315 | 0.0480988 | 0.4488 |
| Whole-fruit rosehip | 700 | 0.30 | 5 | 258.417 | 531.567 | 0.0189276 | 0.6315 |
| Whole-fruit rosehip | 700 | 0.30 | 10 | 264.732 | 537.882 | 0.0360453 | 0.6354 |
| Whole-fruit rosehip | 700 | 0.30 | 15 | 275.850 | 549.000 | 0.0559368 | 0.6130 |
| Whole-fruit rosehip | 700 | 0.30 | 20 | 284.546 | 557.696 | 0.0744249 | 0.6944 |
| Whole-fruit rosehip | 700 | 0.40 | 5 | 284.415 | 557.565 | 0.0213842 | 0.7134 |
| Whole-fruit rosehip | 700 | 0.40 | 10 | 291.338 | 564.488 | 0.0419312 | 0.7392 |
| Whole-fruit rosehip | 700 | 0.40 | 15 | 302.085 | 575.235 | 0.0626022 | 0.6860 |
| Whole-fruit rosehip | 700 | 0.40 | 20 | 311.057 | 584.207 | 0.0819289 | 0.7644 |
| Whole-fruit rosehip | 700 | 0.50 | 5 | 307.032 | 580.182 | 0.0258362 | 0.8619 |
| Whole-fruit rosehip | 700 | 0.50 | 10 | 314.983 | 588.133 | 0.0478022 | 0.8427 |
| Whole-fruit rosehip | 700 | 0.50 | 15 | 325.700 | 598.850 | 0.0738024 | 0.8088 |
| Whole-fruit rosehip | 700 | 0.50 | 20 | 335.029 | 608.179 | 0.0976431 | 0.9111 |
| Whole-fruit rosehip | 700 | 0.60 | 5 | 326.300 | 599.450 | 0.0281691 | 0.9398 |
| Whole-fruit rosehip | 700 | 0.60 | 10 | 334.829 | 607.979 | 0.0554742 | 0.9780 |
| Whole-fruit rosehip | 700 | 0.60 | 15 | 344.883 | 618.033 | 0.0874847 | 0.9587 |
| Whole-fruit rosehip | 700 | 0.60 | 20 | 354.905 | 628.055 | 0.1057673 | 0.9869 |
| Whole-fruit rosehip | 700 | 0.70 | 5 | 345.015 | 618.165 | 0.0214859 | 0.7168 |
| Whole-fruit rosehip | 700 | 0.70 | 10 | 354.304 | 627.454 | 0.0426134 | 0.7512 |
| Whole-fruit rosehip | 700 | 0.70 | 15 | 362.594 | 635.744 | 0.0774821 | 0.8491 |
| Whole-fruit rosehip | 700 | 0.70 | 20 | 384.138 | 657.288 | 0.0388692 | 0.3627 |
| Whole-fruit rosehip | 700 | 0.80 | 5 | 391.509 | 664.659 | 0.0088126 | 0.2940 |
| Whole-fruit rosehip | 700 | 0.80 | 10 | 402.860 | 676.010 | 0.0159863 | 0.2818 |
| Whole-fruit rosehip | 700 | 0.80 | 15 | 398.950 | 672.100 | 0.0292819 | 0.3209 |
| Whole-fruit rosehip | 700 | 0.80 | 20 | 458.022 | 731.172 | 0.0245679 | 0.2292 |
| Whole-fruit rosehip | 700 | 0.90 | 5 | 469.133 | 742.283 | 0.0047213 | 0.1575 |
| Whole-fruit rosehip | 700 | 0.90 | 10 | 485.260 | 758.410 | 0.0094727 | 0.1670 |
| Whole-fruit rosehip | 700 | 0.90 | 15 | 463.743 | 736.893 | 0.0172587 | 0.1891 |
| Whole-fruit rosehip | 700 | 0.90 | 20 | 547.086 | 820.236 | 0.0187021 | 0.1745 |

Table S30. Endpoint-specific apparent activation-energy estimates and four-rate regression statistics for the Friedman, KAS, FWO, and Starink methods.

| Feedstock | Tf (°C) | α | Method | Eₐ (kJ mol⁻¹) | SE(Eₐ) | 95% CI (kJ mol⁻¹) | R² | Regression RMSE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 600 | 0.10 | Friedman | 109.818 | 8.038 | 75.232 to 144.405 | 0.9894 | 0.05297 |
| Rosehip seed | 600 | 0.10 | KAS | 100.057 | 18.197 | 21.761 to 178.353 | 0.9380 | 0.11990 |
| Rosehip seed | 600 | 0.10 | FWO | 103.380 | 17.309 | 28.905 to 177.855 | 0.9469 | 0.11996 |
| Rosehip seed | 600 | 0.10 | Starink | 100.324 | 18.183 | 22.089 to 178.559 | 0.9384 | 0.11990 |
| Rosehip seed | 600 | 0.20 | Friedman | 138.089 | 12.621 | 83.786 to 192.392 | 0.9836 | 0.07090 |
| Rosehip seed | 600 | 0.20 | KAS | 120.699 | 12.647 | 66.282 to 175.117 | 0.9785 | 0.07105 |
| Rosehip seed | 600 | 0.20 | FWO | 123.439 | 12.007 | 71.779 to 175.100 | 0.9814 | 0.07095 |
| Rosehip seed | 600 | 0.20 | Starink | 120.968 | 12.637 | 66.597 to 175.339 | 0.9786 | 0.07105 |
| Rosehip seed | 600 | 0.30 | Friedman | 154.793 | 12.687 | 100.208 to 209.379 | 0.9867 | 0.06194 |
| Rosehip seed | 600 | 0.30 | KAS | 140.104 | 13.453 | 82.220 to 197.988 | 0.9819 | 0.06568 |
| Rosehip seed | 600 | 0.30 | FWO | 142.232 | 12.757 | 87.343 to 197.122 | 0.9842 | 0.06551 |
| Rosehip seed | 600 | 0.30 | Starink | 140.372 | 13.441 | 82.540 to 198.203 | 0.9820 | 0.06568 |
| Rosehip seed | 600 | 0.40 | Friedman | 155.909 | 6.006 | 130.068 to 181.750 | 0.9970 | 0.02722 |
| Rosehip seed | 600 | 0.40 | KAS | 151.797 | 11.370 | 102.874 to 200.719 | 0.9889 | 0.05154 |
| Rosehip seed | 600 | 0.40 | FWO | 153.686 | 10.774 | 107.329 to 200.043 | 0.9903 | 0.05136 |
| Rosehip seed | 600 | 0.40 | Starink | 152.069 | 11.360 | 103.192 to 200.945 | 0.9890 | 0.05153 |
| Rosehip seed | 600 | 0.50 | Friedman | 155.439 | 5.618 | 131.266 to 179.612 | 0.9974 | 0.02520 |
| Rosehip seed | 600 | 0.50 | KAS | 153.800 | 5.769 | 128.979 to 178.621 | 0.9972 | 0.02588 |
| Rosehip seed | 600 | 0.50 | FWO | 155.847 | 5.443 | 132.427 to 179.268 | 0.9976 | 0.02568 |
| Rosehip seed | 600 | 0.50 | Starink | 154.081 | 5.762 | 129.287 to 178.875 | 0.9972 | 0.02587 |
| Rosehip seed | 600 | 0.60 | Friedman | 155.939 | 12.030 | 104.178 to 207.700 | 0.9882 | 0.05364 |
| Rosehip seed | 600 | 0.60 | KAS | 154.634 | 5.820 | 129.594 to 179.675 | 0.9972 | 0.02595 |
| Rosehip seed | 600 | 0.60 | FWO | 156.816 | 5.490 | 133.193 to 180.439 | 0.9976 | 0.02575 |
| Rosehip seed | 600 | 0.60 | Starink | 154.923 | 5.813 | 129.910 to 179.936 | 0.9972 | 0.02594 |
| Rosehip seed | 600 | 0.70 | Friedman | 204.355 | 46.702 | 3.415 to 405.296 | 0.9054 | 0.19336 |
| Rosehip seed | 600 | 0.70 | KAS | 165.687 | 16.958 | 92.723 to 238.652 | 0.9795 | 0.07021 |
| Rosehip seed | 600 | 0.70 | FWO | 167.532 | 16.087 | 98.313 to 236.750 | 0.9819 | 0.07006 |
| Rosehip seed | 600 | 0.70 | Starink | 165.976 | 16.943 | 93.076 to 238.875 | 0.9796 | 0.07021 |
| Rosehip seed | 600 | 0.80 | Friedman | 202.606 | 44.527 | 11.023 to 394.189 | 0.9119 | 0.15295 |
| Rosehip seed | 600 | 0.80 | KAS | 193.781 | 44.656 | 1.640 to 385.923 | 0.9040 | 0.15339 |
| Rosehip seed | 600 | 0.80 | FWO | 194.755 | 42.442 | 12.141 to 377.370 | 0.9133 | 0.15334 |
| Rosehip seed | 600 | 0.80 | Starink | 194.069 | 44.620 | 2.084 to 386.054 | 0.9044 | 0.15339 |
| Rosehip seed | 600 | 0.90 | Friedman | 114.442 | 41.233 | -62.968 to 291.852 | 0.7939 | 0.17203 |
| Rosehip seed | 600 | 0.90 | KAS | 154.038 | 42.333 | -28.107 to 336.182 | 0.8688 | 0.17662 |
| Rosehip seed | 600 | 0.90 | FWO | 157.845 | 40.217 | -15.195 to 330.885 | 0.8851 | 0.17649 |
| Rosehip seed | 600 | 0.90 | Starink | 154.393 | 42.298 | -27.600 to 336.387 | 0.8695 | 0.17662 |
| Rosehip seed | 650 | 0.10 | Friedman | 109.439 | 7.450 | 77.382 to 141.496 | 0.9908 | 0.04980 |
| Rosehip seed | 650 | 0.10 | KAS | 98.868 | 16.767 | 26.725 to 171.012 | 0.9456 | 0.11208 |
| Rosehip seed | 650 | 0.10 | FWO | 102.262 | 15.950 | 33.634 to 170.891 | 0.9536 | 0.11214 |
| Rosehip seed | 650 | 0.10 | Starink | 99.137 | 16.754 | 27.050 to 171.224 | 0.9460 | 0.11208 |
| Rosehip seed | 650 | 0.20 | Friedman | 135.206 | 11.879 | 84.095 to 186.318 | 0.9848 | 0.06771 |
| Rosehip seed | 650 | 0.20 | KAS | 118.954 | 11.726 | 68.503 to 169.405 | 0.9809 | 0.06684 |
| Rosehip seed | 650 | 0.20 | FWO | 121.794 | 11.128 | 73.915 to 169.673 | 0.9836 | 0.06672 |
| Rosehip seed | 650 | 0.20 | Starink | 119.225 | 11.715 | 68.818 to 169.632 | 0.9811 | 0.06683 |
| Rosehip seed | 650 | 0.30 | Friedman | 148.917 | 11.632 | 98.870 to 198.964 | 0.9879 | 0.05832 |
| Rosehip seed | 650 | 0.30 | KAS | 136.328 | 12.205 | 83.813 to 188.842 | 0.9842 | 0.06119 |
| Rosehip seed | 650 | 0.30 | FWO | 138.662 | 11.567 | 88.894 to 188.431 | 0.9863 | 0.06100 |
| Rosehip seed | 650 | 0.30 | Starink | 136.599 | 12.194 | 84.133 to 189.065 | 0.9843 | 0.06118 |
| Rosehip seed | 650 | 0.40 | Friedman | 149.966 | 4.796 | 129.331 to 170.602 | 0.9980 | 0.02259 |
| Rosehip seed | 650 | 0.40 | KAS | 145.828 | 9.863 | 103.391 to 188.266 | 0.9909 | 0.04646 |
| Rosehip seed | 650 | 0.40 | FWO | 148.036 | 9.337 | 107.863 to 188.208 | 0.9921 | 0.04625 |
| Rosehip seed | 650 | 0.40 | Starink | 146.106 | 9.854 | 103.710 to 188.503 | 0.9910 | 0.04645 |
| Rosehip seed | 650 | 0.50 | Friedman | 152.248 | 6.447 | 124.507 to 179.990 | 0.9964 | 0.02982 |
| Rosehip seed | 650 | 0.50 | KAS | 148.871 | 5.564 | 124.931 to 172.812 | 0.9972 | 0.02573 |
| Rosehip seed | 650 | 0.50 | FWO | 151.182 | 5.245 | 128.612 to 173.751 | 0.9976 | 0.02552 |
| Rosehip seed | 650 | 0.50 | Starink | 149.158 | 5.558 | 125.244 to 173.071 | 0.9972 | 0.02572 |
| Rosehip seed | 650 | 0.60 | Friedman | 149.745 | 14.077 | 89.178 to 210.311 | 0.9826 | 0.06462 |
| Rosehip seed | 650 | 0.60 | KAS | 149.780 | 6.642 | 121.203 to 178.357 | 0.9961 | 0.03049 |
| Rosehip seed | 650 | 0.60 | FWO | 152.220 | 6.272 | 125.232 to 179.208 | 0.9966 | 0.03029 |
| Rosehip seed | 650 | 0.60 | Starink | 150.073 | 6.635 | 121.526 to 178.620 | 0.9961 | 0.03048 |
| Rosehip seed | 650 | 0.70 | Friedman | 179.437 | 53.219 | -49.547 to 408.420 | 0.8504 | 0.23070 |
| Rosehip seed | 650 | 0.70 | KAS | 156.793 | 20.503 | 68.575 to 245.011 | 0.9669 | 0.08888 |
| Rosehip seed | 650 | 0.70 | FWO | 159.120 | 19.461 | 75.388 to 242.853 | 0.9710 | 0.08873 |
| Rosehip seed | 650 | 0.70 | Starink | 157.090 | 20.485 | 68.948 to 245.231 | 0.9671 | 0.08887 |
| Rosehip seed | 650 | 0.80 | Friedman | 162.317 | 38.275 | -2.369 to 327.004 | 0.8999 | 0.15675 |
| Rosehip seed | 650 | 0.80 | KAS | 160.383 | 38.017 | -3.191 to 323.958 | 0.8990 | 0.15570 |
| Rosehip seed | 650 | 0.80 | FWO | 163.112 | 36.127 | 7.671 to 318.552 | 0.9107 | 0.15562 |
| Rosehip seed | 650 | 0.80 | Starink | 160.702 | 37.986 | -2.738 to 324.142 | 0.8995 | 0.15569 |
| Rosehip seed | 650 | 0.90 | Friedman | 73.177 | 20.546 | -15.224 to 161.579 | 0.8638 | 0.12947 |
| Rosehip seed | 650 | 0.90 | KAS | 98.543 | 26.205 | -14.208 to 211.294 | 0.8761 | 0.16513 |
| Rosehip seed | 650 | 0.90 | FWO | 105.374 | 24.872 | -1.641 to 212.389 | 0.8997 | 0.16484 |
| Rosehip seed | 650 | 0.90 | Starink | 98.955 | 26.182 | -13.698 to 211.609 | 0.8772 | 0.16512 |
| Rosehip seed | 700 | 0.10 | Friedman | 109.029 | 7.007 | 78.879 to 139.179 | 0.9918 | 0.04735 |
| Rosehip seed | 700 | 0.10 | KAS | 98.010 | 15.597 | 30.900 to 165.119 | 0.9518 | 0.10538 |
| Rosehip seed | 700 | 0.10 | FWO | 101.456 | 14.838 | 37.614 to 165.299 | 0.9590 | 0.10545 |
| Rosehip seed | 700 | 0.10 | Starink | 98.279 | 15.585 | 31.222 to 165.336 | 0.9521 | 0.10539 |
| Rosehip seed | 700 | 0.20 | Friedman | 132.803 | 11.577 | 82.992 to 182.615 | 0.9850 | 0.06677 |
| Rosehip seed | 700 | 0.20 | KAS | 117.561 | 10.996 | 70.251 to 164.872 | 0.9828 | 0.06341 |
| Rosehip seed | 700 | 0.20 | FWO | 120.481 | 10.430 | 75.602 to 165.359 | 0.9852 | 0.06327 |
| Rosehip seed | 700 | 0.20 | Starink | 117.833 | 10.986 | 70.565 to 165.102 | 0.9829 | 0.06341 |
| Rosehip seed | 700 | 0.30 | Friedman | 144.057 | 11.517 | 94.505 to 193.610 | 0.9874 | 0.05906 |
| Rosehip seed | 700 | 0.30 | KAS | 133.108 | 11.572 | 83.318 to 182.898 | 0.9851 | 0.05934 |
| Rosehip seed | 700 | 0.30 | FWO | 135.619 | 10.961 | 88.459 to 182.779 | 0.9871 | 0.05912 |
| Rosehip seed | 700 | 0.30 | Starink | 133.383 | 11.561 | 83.640 to 183.126 | 0.9852 | 0.05933 |
| Rosehip seed | 700 | 0.40 | Friedman | 145.623 | 5.016 | 124.041 to 167.206 | 0.9976 | 0.02436 |
| Rosehip seed | 700 | 0.40 | KAS | 141.127 | 9.386 | 100.743 to 181.510 | 0.9912 | 0.04558 |
| Rosehip seed | 700 | 0.40 | FWO | 143.586 | 8.879 | 105.384 to 181.787 | 0.9924 | 0.04535 |
| Rosehip seed | 700 | 0.40 | Starink | 141.409 | 9.376 | 101.066 to 181.753 | 0.9913 | 0.04557 |
| Rosehip seed | 700 | 0.50 | Friedman | 149.539 | 7.331 | 117.997 to 181.082 | 0.9952 | 0.03471 |
| Rosehip seed | 700 | 0.50 | KAS | 145.114 | 6.229 | 118.310 to 171.917 | 0.9963 | 0.02949 |
| Rosehip seed | 700 | 0.50 | FWO | 147.626 | 5.878 | 122.336 to 172.915 | 0.9968 | 0.02927 |
| Rosehip seed | 700 | 0.50 | Starink | 145.404 | 6.223 | 118.630 to 172.178 | 0.9964 | 0.02948 |
| Rosehip seed | 700 | 0.60 | Friedman | 143.537 | 16.782 | 71.328 to 215.746 | 0.9734 | 0.07897 |
| Rosehip seed | 700 | 0.60 | KAS | 145.740 | 7.878 | 111.845 to 179.635 | 0.9942 | 0.03707 |
| Rosehip seed | 700 | 0.60 | FWO | 148.396 | 7.448 | 116.349 to 180.443 | 0.9950 | 0.03686 |
| Rosehip seed | 700 | 0.60 | Starink | 146.037 | 7.870 | 112.177 to 179.897 | 0.9942 | 0.03706 |
| Rosehip seed | 700 | 0.70 | Friedman | 150.079 | 58.740 | -102.657 to 402.815 | 0.7655 | 0.26957 |
| Rosehip seed | 700 | 0.70 | KAS | 146.095 | 24.335 | 41.391 to 250.799 | 0.9474 | 0.11168 |
| Rosehip seed | 700 | 0.70 | FWO | 148.994 | 23.107 | 49.573 to 248.414 | 0.9541 | 0.11154 |
| Rosehip seed | 700 | 0.70 | Starink | 146.402 | 24.314 | 41.787 to 251.018 | 0.9477 | 0.11167 |
| Rosehip seed | 700 | 0.80 | Friedman | 131.633 | 36.226 | -24.233 to 287.500 | 0.8685 | 0.17070 |
| Rosehip seed | 700 | 0.80 | KAS | 136.570 | 35.769 | -17.331 to 290.471 | 0.8794 | 0.16855 |
| Rosehip seed | 700 | 0.80 | FWO | 140.566 | 33.988 | -5.672 to 286.803 | 0.8953 | 0.16845 |
| Rosehip seed | 700 | 0.80 | Starink | 136.911 | 35.739 | -16.863 to 290.686 | 0.8801 | 0.16854 |
| Rosehip seed | 700 | 0.90 | Friedman | 71.281 | 12.862 | 15.939 to 126.622 | 0.9389 | 0.10299 |
| Rosehip seed | 700 | 0.90 | KAS | 74.919 | 20.064 | -11.409 to 161.246 | 0.8746 | 0.16065 |
| Rosehip seed | 700 | 0.90 | FWO | 83.182 | 19.027 | 1.314 to 165.050 | 0.9053 | 0.16024 |
| Rosehip seed | 700 | 0.90 | Starink | 75.361 | 20.046 | -10.889 to 161.611 | 0.8760 | 0.16063 |
| Rosehip skin | 600 | 0.10 | Friedman | 111.038 | 22.404 | 14.643 to 207.434 | 0.9247 | 0.11871 |
| Rosehip skin | 600 | 0.10 | KAS | 128.587 | 19.657 | 44.010 to 213.165 | 0.9553 | 0.10415 |
| Rosehip skin | 600 | 0.10 | FWO | 129.451 | 18.670 | 49.122 to 209.780 | 0.9601 | 0.10405 |
| Rosehip skin | 600 | 0.10 | Starink | 128.787 | 19.641 | 44.281 to 213.294 | 0.9556 | 0.10415 |
| Rosehip skin | 600 | 0.20 | Friedman | 174.504 | 48.176 | -32.782 to 381.791 | 0.8677 | 0.22089 |
| Rosehip skin | 600 | 0.20 | KAS | 143.165 | 38.453 | -22.285 to 308.615 | 0.8739 | 0.17631 |
| Rosehip skin | 600 | 0.20 | FWO | 143.660 | 36.544 | -13.577 to 300.896 | 0.8854 | 0.17624 |
| Rosehip skin | 600 | 0.20 | Starink | 143.368 | 38.422 | -21.947 to 308.682 | 0.8744 | 0.17631 |
| Rosehip skin | 600 | 0.30 | Friedman | 275.454 | 83.136 | -82.252 to 633.160 | 0.8459 | 0.24573 |
| Rosehip skin | 600 | 0.30 | KAS | 219.218 | 71.704 | -89.301 to 527.737 | 0.8237 | 0.21194 |
| Rosehip skin | 600 | 0.30 | FWO | 216.323 | 68.166 | -76.972 to 509.617 | 0.8343 | 0.21192 |
| Rosehip skin | 600 | 0.30 | Starink | 219.375 | 71.647 | -88.897 to 527.646 | 0.8242 | 0.21194 |
| Rosehip skin | 600 | 0.40 | Friedman | 337.723 | 74.349 | 17.824 to 657.621 | 0.9116 | 0.14330 |
| Rosehip skin | 600 | 0.40 | KAS | 347.446 | 97.509 | -72.102 to 766.995 | 0.8639 | 0.18794 |
| Rosehip skin | 600 | 0.40 | FWO | 338.708 | 92.710 | -60.191 to 737.607 | 0.8697 | 0.18794 |
| Rosehip skin | 600 | 0.40 | Starink | 347.521 | 97.432 | -71.693 to 766.735 | 0.8642 | 0.18794 |
| Rosehip skin | 600 | 0.50 | Friedman | 220.760 | 36.451 | 63.924 to 377.596 | 0.9483 | 0.10046 |
| Rosehip skin | 600 | 0.50 | KAS | 254.283 | 30.666 | 122.339 to 386.227 | 0.9717 | 0.08451 |
| Rosehip skin | 600 | 0.50 | FWO | 250.635 | 29.159 | 125.176 to 376.095 | 0.9736 | 0.08452 |
| Rosehip skin | 600 | 0.50 | Starink | 254.453 | 30.641 | 122.614 to 386.292 | 0.9718 | 0.08451 |
| Rosehip skin | 600 | 0.60 | Friedman | 161.006 | 35.490 | 8.305 to 313.707 | 0.9114 | 0.12992 |
| Rosehip skin | 600 | 0.60 | KAS | 186.099 | 32.269 | 47.258 to 324.940 | 0.9433 | 0.11813 |
| Rosehip skin | 600 | 0.60 | FWO | 186.238 | 30.667 | 54.287 to 318.188 | 0.9486 | 0.11808 |
| Rosehip skin | 600 | 0.60 | Starink | 186.341 | 32.242 | 47.614 to 325.069 | 0.9435 | 0.11813 |
| Rosehip skin | 600 | 0.70 | Friedman | 130.800 | 68.172 | -162.523 to 424.122 | 0.6480 | 0.27825 |
| Rosehip skin | 600 | 0.70 | KAS | 152.933 | 54.399 | -81.125 to 386.991 | 0.7981 | 0.22203 |
| Rosehip skin | 600 | 0.70 | FWO | 155.150 | 51.688 | -67.247 to 377.546 | 0.8183 | 0.22190 |
| Rosehip skin | 600 | 0.70 | Starink | 153.220 | 54.354 | -80.645 to 387.086 | 0.7989 | 0.22203 |
| Rosehip skin | 600 | 0.80 | Friedman | 100.678 | 36.611 | -56.848 to 258.203 | 0.7908 | 0.20366 |
| Rosehip skin | 600 | 0.80 | KAS | 107.917 | 40.861 | -67.893 to 283.726 | 0.7772 | 0.22729 |
| Rosehip skin | 600 | 0.80 | FWO | 113.251 | 38.798 | -53.685 to 280.186 | 0.8099 | 0.22700 |
| Rosehip skin | 600 | 0.80 | Starink | 108.278 | 40.826 | -67.382 to 283.938 | 0.7786 | 0.22728 |
| Rosehip skin | 600 | 0.90 | Friedman | 156.013 | 35.858 | 1.728 to 310.299 | 0.9044 | 0.17002 |
| Rosehip skin | 600 | 0.90 | KAS | 134.335 | 35.986 | -20.501 to 289.171 | 0.8745 | 0.17063 |
| Rosehip skin | 600 | 0.90 | FWO | 139.499 | 34.177 | -7.552 to 286.550 | 0.8928 | 0.17044 |
| Rosehip skin | 600 | 0.90 | Starink | 134.723 | 35.956 | -19.983 to 289.428 | 0.8753 | 0.17062 |
| Rosehip skin | 650 | 0.10 | Friedman | 110.509 | 21.640 | 17.398 to 203.621 | 0.9288 | 0.11662 |
| Rosehip skin | 650 | 0.10 | KAS | 126.426 | 18.855 | 45.299 to 207.552 | 0.9574 | 0.10161 |
| Rosehip skin | 650 | 0.10 | FWO | 127.405 | 17.906 | 50.360 to 204.449 | 0.9620 | 0.10150 |
| Rosehip skin | 650 | 0.10 | Starink | 126.628 | 18.839 | 45.570 to 207.686 | 0.9576 | 0.10161 |
| Rosehip skin | 650 | 0.20 | Friedman | 173.940 | 44.893 | -19.220 to 367.100 | 0.8824 | 0.20665 |
| Rosehip skin | 650 | 0.20 | KAS | 143.081 | 37.258 | -17.226 to 303.388 | 0.8806 | 0.17150 |
| Rosehip skin | 650 | 0.20 | FWO | 143.596 | 35.407 | -8.749 to 295.941 | 0.8916 | 0.17142 |
| Rosehip skin | 650 | 0.20 | Starink | 143.285 | 37.227 | -16.892 to 303.461 | 0.8811 | 0.17150 |
| Rosehip skin | 650 | 0.30 | Friedman | 276.439 | 74.528 | -44.230 to 597.109 | 0.8731 | 0.22058 |
| Rosehip skin | 650 | 0.30 | KAS | 222.572 | 65.462 | -59.089 to 504.233 | 0.8525 | 0.19374 |
| Rosehip skin | 650 | 0.30 | FWO | 219.538 | 62.232 | -48.224 to 487.300 | 0.8615 | 0.19372 |
| Rosehip skin | 650 | 0.30 | Starink | 222.727 | 65.410 | -58.708 to 504.162 | 0.8529 | 0.19374 |
| Rosehip skin | 650 | 0.40 | Friedman | 301.693 | 38.828 | 134.631 to 468.756 | 0.9679 | 0.08492 |
| Rosehip skin | 650 | 0.40 | KAS | 318.949 | 54.168 | 85.881 to 552.017 | 0.9455 | 0.11847 |
| Rosehip skin | 650 | 0.40 | FWO | 311.662 | 51.510 | 90.030 to 533.293 | 0.9482 | 0.11850 |
| Rosehip skin | 650 | 0.40 | Starink | 319.048 | 54.125 | 86.165 to 551.931 | 0.9456 | 0.11847 |
| Rosehip skin | 650 | 0.50 | Friedman | 191.359 | 36.673 | 33.569 to 349.148 | 0.9316 | 0.11546 |
| Rosehip skin | 650 | 0.50 | KAS | 220.630 | 29.791 | 92.448 to 348.812 | 0.9648 | 0.09380 |
| Rosehip skin | 650 | 0.50 | FWO | 218.695 | 28.320 | 96.845 to 340.546 | 0.9676 | 0.09378 |
| Rosehip skin | 650 | 0.50 | Starink | 220.829 | 29.767 | 92.751 to 348.908 | 0.9649 | 0.09380 |
| Rosehip skin | 650 | 0.60 | Friedman | 148.509 | 35.246 | -3.141 to 300.158 | 0.8988 | 0.14130 |
| Rosehip skin | 650 | 0.60 | KAS | 168.388 | 31.347 | 33.513 to 303.264 | 0.9352 | 0.12567 |
| Rosehip skin | 650 | 0.60 | FWO | 169.454 | 29.788 | 41.286 to 297.621 | 0.9418 | 0.12561 |
| Rosehip skin | 650 | 0.60 | Starink | 168.647 | 31.321 | 33.882 to 303.413 | 0.9355 | 0.12567 |
| Rosehip skin | 650 | 0.70 | Friedman | 92.404 | 45.847 | -104.862 to 289.669 | 0.6701 | 0.22272 |
| Rosehip skin | 650 | 0.70 | KAS | 126.318 | 46.334 | -73.042 to 325.678 | 0.7880 | 0.22509 |
| Rosehip skin | 650 | 0.70 | FWO | 129.961 | 44.014 | -59.416 to 319.338 | 0.8134 | 0.22489 |
| Rosehip skin | 650 | 0.70 | Starink | 126.632 | 46.296 | -72.562 to 325.825 | 0.7891 | 0.22508 |
| Rosehip skin | 650 | 0.80 | Friedman | 87.831 | 23.593 | -13.681 to 189.344 | 0.8739 | 0.15320 |
| Rosehip skin | 650 | 0.80 | KAS | 92.718 | 31.766 | -43.960 to 229.397 | 0.8099 | 0.20627 |
| Rosehip skin | 650 | 0.80 | FWO | 99.011 | 30.151 | -30.720 to 228.742 | 0.8435 | 0.20593 |
| Rosehip skin | 650 | 0.80 | Starink | 93.101 | 31.739 | -43.459 to 229.661 | 0.8114 | 0.20626 |
| Rosehip skin | 650 | 0.90 | Friedman | 142.060 | 32.473 | 2.341 to 281.779 | 0.9054 | 0.16761 |
| Rosehip skin | 650 | 0.90 | KAS | 121.910 | 33.473 | -22.111 to 265.931 | 0.8690 | 0.17277 |
| Rosehip skin | 650 | 0.90 | FWO | 127.955 | 31.786 | -8.808 to 264.718 | 0.8901 | 0.17256 |
| Rosehip skin | 650 | 0.90 | Starink | 122.319 | 33.444 | -21.579 to 266.218 | 0.8699 | 0.17276 |
| Rosehip skin | 700 | 0.10 | Friedman | 109.634 | 20.662 | 20.732 to 198.535 | 0.9337 | 0.11319 |
| Rosehip skin | 700 | 0.10 | KAS | 124.388 | 18.016 | 46.873 to 201.904 | 0.9597 | 0.09869 |
| Rosehip skin | 700 | 0.10 | FWO | 125.475 | 17.107 | 51.869 to 199.081 | 0.9642 | 0.09857 |
| Rosehip skin | 700 | 0.10 | Starink | 124.592 | 18.000 | 47.142 to 202.042 | 0.9599 | 0.09869 |
| Rosehip skin | 700 | 0.20 | Friedman | 172.276 | 40.400 | -1.549 to 346.101 | 0.9009 | 0.18811 |
| Rosehip skin | 700 | 0.20 | KAS | 142.215 | 35.003 | -8.389 to 292.819 | 0.8919 | 0.16298 |
| Rosehip skin | 700 | 0.20 | FWO | 142.787 | 33.262 | -0.327 to 285.901 | 0.9021 | 0.16290 |
| Rosehip skin | 700 | 0.20 | Starink | 142.420 | 34.974 | -8.060 to 292.900 | 0.8924 | 0.16298 |
| Rosehip skin | 700 | 0.30 | Friedman | 273.722 | 57.899 | 24.604 to 522.841 | 0.9179 | 0.17416 |
| Rosehip skin | 700 | 0.30 | KAS | 223.902 | 54.621 | -11.115 to 458.919 | 0.8936 | 0.16430 |
| Rosehip skin | 700 | 0.30 | FWO | 220.826 | 51.925 | -2.590 to 444.242 | 0.9004 | 0.16428 |
| Rosehip skin | 700 | 0.30 | Starink | 224.057 | 54.578 | -10.771 to 458.885 | 0.8939 | 0.16430 |
| Rosehip skin | 700 | 0.40 | Friedman | 257.429 | 26.034 | 145.413 to 369.445 | 0.9800 | 0.06612 |
| Rosehip skin | 700 | 0.40 | KAS | 278.928 | 24.338 | 174.210 to 383.646 | 0.9850 | 0.06182 |
| Rosehip skin | 700 | 0.40 | FWO | 273.657 | 23.155 | 174.027 to 373.286 | 0.9859 | 0.06186 |
| Rosehip skin | 700 | 0.40 | Starink | 279.061 | 24.319 | 174.424 to 383.698 | 0.9850 | 0.06182 |
| Rosehip skin | 700 | 0.50 | Friedman | 166.495 | 36.726 | 8.476 to 324.514 | 0.9113 | 0.13172 |
| Rosehip skin | 700 | 0.50 | KAS | 191.264 | 30.499 | 60.040 to 322.489 | 0.9516 | 0.10938 |
| Rosehip skin | 700 | 0.50 | FWO | 190.825 | 28.986 | 66.109 to 315.542 | 0.9559 | 0.10934 |
| Rosehip skin | 700 | 0.50 | Starink | 191.489 | 30.474 | 60.371 to 322.607 | 0.9518 | 0.10938 |
| Rosehip skin | 700 | 0.60 | Friedman | 134.549 | 39.034 | -33.402 to 302.499 | 0.8559 | 0.17170 |
| Rosehip skin | 700 | 0.60 | KAS | 151.469 | 31.491 | 15.974 to 286.964 | 0.9204 | 0.13852 |
| Rosehip skin | 700 | 0.60 | FWO | 153.415 | 29.922 | 24.673 to 282.158 | 0.9293 | 0.13843 |
| Rosehip skin | 700 | 0.60 | Starink | 151.743 | 31.465 | 16.360 to 287.127 | 0.9208 | 0.13851 |
| Rosehip skin | 700 | 0.70 | Friedman | 65.254 | 36.987 | -93.890 to 224.398 | 0.6088 | 0.21942 |
| Rosehip skin | 700 | 0.70 | KAS | 101.498 | 37.875 | -61.465 to 264.462 | 0.7822 | 0.22468 |
| Rosehip skin | 700 | 0.70 | FWO | 106.479 | 35.965 | -48.267 to 261.225 | 0.8142 | 0.22440 |
| Rosehip skin | 700 | 0.70 | Starink | 101.837 | 37.843 | -60.989 to 264.662 | 0.7836 | 0.22467 |
| Rosehip skin | 700 | 0.80 | Friedman | 90.591 | 19.691 | 5.869 to 175.312 | 0.9137 | 0.14533 |
| Rosehip skin | 700 | 0.80 | KAS | 81.056 | 26.155 | -31.482 to 193.594 | 0.8276 | 0.19305 |
| Rosehip skin | 700 | 0.80 | FWO | 88.108 | 24.818 | -18.677 to 194.893 | 0.8630 | 0.19267 |
| Rosehip skin | 700 | 0.80 | Starink | 81.455 | 26.133 | -30.984 to 193.895 | 0.8293 | 0.19303 |
| Rosehip skin | 700 | 0.90 | Friedman | 103.227 | 34.700 | -46.074 to 252.529 | 0.8157 | 0.21141 |
| Rosehip skin | 700 | 0.90 | KAS | 99.405 | 31.472 | -36.010 to 234.820 | 0.8330 | 0.19175 |
| Rosehip skin | 700 | 0.90 | FWO | 106.843 | 29.882 | -21.729 to 235.416 | 0.8647 | 0.19149 |
| Rosehip skin | 700 | 0.90 | Starink | 99.844 | 31.446 | -35.455 to 235.143 | 0.8345 | 0.19174 |
| Whole-fruit rosehip | 600 | 0.10 | Friedman | 112.367 | 20.181 | 25.535 to 199.199 | 0.9394 | 0.12977 |
| Whole-fruit rosehip | 600 | 0.10 | KAS | 104.652 | 15.196 | 39.268 to 170.037 | 0.9595 | 0.09772 |
| Whole-fruit rosehip | 600 | 0.10 | FWO | 106.934 | 14.421 | 44.885 to 168.983 | 0.9649 | 0.09754 |
| Whole-fruit rosehip | 600 | 0.10 | Starink | 104.881 | 15.183 | 39.554 to 170.208 | 0.9598 | 0.09771 |
| Whole-fruit rosehip | 600 | 0.20 | Friedman | 119.618 | 34.647 | -29.458 to 268.693 | 0.8563 | 0.19301 |
| Whole-fruit rosehip | 600 | 0.20 | KAS | 117.056 | 29.407 | -9.474 to 243.586 | 0.8879 | 0.16382 |
| Whole-fruit rosehip | 600 | 0.20 | FWO | 119.288 | 27.943 | -0.942 to 239.517 | 0.9011 | 0.16372 |
| Whole-fruit rosehip | 600 | 0.20 | Starink | 117.299 | 29.383 | -9.127 to 243.724 | 0.8885 | 0.16381 |
| Whole-fruit rosehip | 600 | 0.30 | Friedman | 129.900 | 19.385 | 46.493 to 213.307 | 0.9574 | 0.10609 |
| Whole-fruit rosehip | 600 | 0.30 | KAS | 121.381 | 23.520 | 20.182 to 222.581 | 0.9302 | 0.12872 |
| Whole-fruit rosehip | 600 | 0.30 | FWO | 123.952 | 22.338 | 27.838 to 220.066 | 0.9390 | 0.12858 |
| Whole-fruit rosehip | 600 | 0.30 | Starink | 121.644 | 23.501 | 20.529 to 222.758 | 0.9305 | 0.12871 |
| Whole-fruit rosehip | 600 | 0.40 | Friedman | 143.164 | 22.141 | 47.899 to 238.429 | 0.9543 | 0.10982 |
| Whole-fruit rosehip | 600 | 0.40 | KAS | 135.422 | 22.999 | 36.466 to 234.378 | 0.9455 | 0.11408 |
| Whole-fruit rosehip | 600 | 0.40 | FWO | 137.705 | 21.841 | 43.730 to 231.681 | 0.9521 | 0.11395 |
| Whole-fruit rosehip | 600 | 0.40 | Starink | 135.691 | 22.979 | 36.818 to 234.563 | 0.9458 | 0.11407 |
| Whole-fruit rosehip | 600 | 0.50 | Friedman | 146.199 | 17.124 | 72.519 to 219.879 | 0.9733 | 0.08140 |
| Whole-fruit rosehip | 600 | 0.50 | KAS | 142.032 | 21.680 | 48.750 to 235.314 | 0.9555 | 0.10306 |
| Whole-fruit rosehip | 600 | 0.50 | FWO | 144.353 | 20.586 | 55.780 to 232.926 | 0.9609 | 0.10293 |
| Whole-fruit rosehip | 600 | 0.50 | Starink | 142.310 | 21.662 | 49.107 to 235.513 | 0.9557 | 0.10305 |
| Whole-fruit rosehip | 600 | 0.60 | Friedman | 162.293 | 19.394 | 78.848 to 245.737 | 0.9722 | 0.08764 |
| Whole-fruit rosehip | 600 | 0.60 | KAS | 150.450 | 19.570 | 66.247 to 234.652 | 0.9673 | 0.08844 |
| Whole-fruit rosehip | 600 | 0.60 | FWO | 152.660 | 18.575 | 72.737 to 232.583 | 0.9712 | 0.08829 |
| Whole-fruit rosehip | 600 | 0.60 | Starink | 150.734 | 19.553 | 66.604 to 234.864 | 0.9674 | 0.08843 |
| Whole-fruit rosehip | 600 | 0.70 | Friedman | 130.803 | 39.958 | -41.122 to 302.728 | 0.8427 | 0.17948 |
| Whole-fruit rosehip | 600 | 0.70 | KAS | 150.825 | 20.853 | 61.102 to 240.548 | 0.9632 | 0.09367 |
| Whole-fruit rosehip | 600 | 0.70 | FWO | 153.311 | 19.783 | 68.193 to 238.429 | 0.9678 | 0.09346 |
| Whole-fruit rosehip | 600 | 0.70 | Starink | 151.121 | 20.834 | 61.478 to 240.765 | 0.9634 | 0.09366 |
| Whole-fruit rosehip | 600 | 0.80 | Friedman | 72.692 | 59.459 | -183.141 to 328.525 | 0.4277 | 0.36133 |
| Whole-fruit rosehip | 600 | 0.80 | KAS | 90.703 | 46.408 | -108.974 to 290.381 | 0.6564 | 0.28202 |
| Whole-fruit rosehip | 600 | 0.80 | FWO | 96.879 | 44.068 | -92.728 to 286.487 | 0.7073 | 0.28167 |
| Whole-fruit rosehip | 600 | 0.80 | Starink | 91.078 | 46.369 | -108.430 to 290.586 | 0.6586 | 0.28200 |
| Whole-fruit rosehip | 600 | 0.90 | Friedman | 75.516 | 61.266 | -188.091 to 339.124 | 0.4317 | 0.42161 |
| Whole-fruit rosehip | 600 | 0.90 | KAS | 64.217 | 52.843 | -163.149 to 291.583 | 0.4248 | 0.36365 |
| Whole-fruit rosehip | 600 | 0.90 | FWO | 72.818 | 50.224 | -143.280 to 288.915 | 0.5124 | 0.36353 |
| Whole-fruit rosehip | 600 | 0.90 | Starink | 64.660 | 52.800 | -162.521 to 291.841 | 0.4285 | 0.36364 |
| Whole-fruit rosehip | 650 | 0.10 | Friedman | 110.249 | 20.947 | 20.120 to 200.378 | 0.9327 | 0.13671 |
| Whole-fruit rosehip | 650 | 0.10 | KAS | 102.743 | 15.822 | 34.668 to 170.819 | 0.9547 | 0.10326 |
| Whole-fruit rosehip | 650 | 0.10 | FWO | 105.133 | 15.013 | 40.536 to 169.731 | 0.9608 | 0.10306 |
| Whole-fruit rosehip | 650 | 0.10 | Starink | 102.974 | 15.808 | 34.958 to 170.990 | 0.9550 | 0.10325 |
| Whole-fruit rosehip | 650 | 0.20 | Friedman | 111.866 | 34.033 | -34.565 to 258.296 | 0.8438 | 0.19668 |
| Whole-fruit rosehip | 650 | 0.20 | KAS | 111.992 | 29.409 | -14.544 to 238.528 | 0.8788 | 0.16995 |
| Whole-fruit rosehip | 650 | 0.20 | FWO | 114.504 | 27.941 | -5.715 to 234.723 | 0.8936 | 0.16983 |
| Whole-fruit rosehip | 650 | 0.20 | Starink | 112.240 | 29.384 | -14.191 to 238.672 | 0.8794 | 0.16995 |
| Whole-fruit rosehip | 650 | 0.30 | Friedman | 126.378 | 19.694 | 41.643 to 211.113 | 0.9537 | 0.11100 |
| Whole-fruit rosehip | 650 | 0.30 | KAS | 117.411 | 23.239 | 17.421 to 217.400 | 0.9273 | 0.13099 |
| Whole-fruit rosehip | 650 | 0.30 | FWO | 120.208 | 22.065 | 25.268 to 215.149 | 0.9369 | 0.13082 |
| Whole-fruit rosehip | 650 | 0.30 | Starink | 117.677 | 23.219 | 17.773 to 217.581 | 0.9278 | 0.13098 |
| Whole-fruit rosehip | 650 | 0.40 | Friedman | 136.499 | 22.451 | 39.900 to 233.099 | 0.9487 | 0.11536 |
| Whole-fruit rosehip | 650 | 0.40 | KAS | 130.022 | 23.237 | 30.041 to 230.003 | 0.9400 | 0.11940 |
| Whole-fruit rosehip | 650 | 0.40 | FWO | 132.609 | 22.061 | 37.688 to 227.530 | 0.9476 | 0.11923 |
| Whole-fruit rosehip | 650 | 0.40 | Starink | 130.296 | 23.217 | 30.401 to 230.191 | 0.9403 | 0.11940 |
| Whole-fruit rosehip | 650 | 0.50 | Friedman | 140.950 | 17.124 | 67.272 to 214.629 | 0.9713 | 0.08475 |
| Whole-fruit rosehip | 650 | 0.50 | KAS | 135.721 | 21.730 | 42.223 to 229.220 | 0.9512 | 0.10755 |
| Whole-fruit rosehip | 650 | 0.50 | FWO | 138.394 | 20.625 | 49.651 to 227.137 | 0.9575 | 0.10737 |
| Whole-fruit rosehip | 650 | 0.50 | Starink | 136.006 | 21.712 | 42.589 to 229.424 | 0.9515 | 0.10755 |
| Whole-fruit rosehip | 650 | 0.60 | Friedman | 153.389 | 22.620 | 56.061 to 250.717 | 0.9583 | 0.10614 |
| Whole-fruit rosehip | 650 | 0.60 | KAS | 143.896 | 21.118 | 53.033 to 234.759 | 0.9587 | 0.09909 |
| Whole-fruit rosehip | 650 | 0.60 | FWO | 146.472 | 20.038 | 60.255 to 232.690 | 0.9639 | 0.09889 |
| Whole-fruit rosehip | 650 | 0.60 | Starink | 144.187 | 21.099 | 53.404 to 234.970 | 0.9589 | 0.09908 |
| Whole-fruit rosehip | 650 | 0.70 | Friedman | 82.003 | 61.697 | -183.457 to 347.463 | 0.4690 | 0.31617 |
| Whole-fruit rosehip | 650 | 0.70 | KAS | 127.464 | 28.390 | 5.310 to 249.618 | 0.9097 | 0.14549 |
| Whole-fruit rosehip | 650 | 0.70 | FWO | 131.182 | 26.936 | 15.286 to 247.078 | 0.9222 | 0.14518 |
| Whole-fruit rosehip | 650 | 0.70 | Starink | 127.782 | 28.365 | 5.736 to 249.829 | 0.9103 | 0.14548 |
| Whole-fruit rosehip | 650 | 0.80 | Friedman | 47.062 | 49.849 | -167.422 to 261.546 | 0.3083 | 0.38704 |
| Whole-fruit rosehip | 650 | 0.80 | KAS | 62.609 | 41.949 | -117.883 to 243.100 | 0.5269 | 0.32570 |
| Whole-fruit rosehip | 650 | 0.80 | FWO | 70.392 | 39.835 | -101.004 to 241.787 | 0.6096 | 0.32531 |
| Whole-fruit rosehip | 650 | 0.80 | Starink | 63.015 | 41.913 | -117.323 to 243.354 | 0.5306 | 0.32568 |
| Whole-fruit rosehip | 650 | 0.90 | Friedman | 56.385 | 57.739 | -192.046 to 304.816 | 0.3229 | 0.46728 |
| Whole-fruit rosehip | 650 | 0.90 | KAS | 47.451 | 48.273 | -160.253 to 255.155 | 0.3257 | 0.39068 |
| Whole-fruit rosehip | 650 | 0.90 | FWO | 57.161 | 45.904 | -140.347 to 254.669 | 0.4367 | 0.39074 |
| Whole-fruit rosehip | 650 | 0.90 | Starink | 47.919 | 48.235 | -159.620 to 255.459 | 0.3304 | 0.39068 |
| Whole-fruit rosehip | 700 | 0.10 | Friedman | 108.305 | 21.876 | 14.181 to 202.429 | 0.9246 | 0.14462 |
| Whole-fruit rosehip | 700 | 0.10 | KAS | 101.068 | 16.486 | 30.133 to 172.003 | 0.9495 | 0.10899 |
| Whole-fruit rosehip | 700 | 0.10 | FWO | 103.552 | 15.643 | 36.244 to 170.861 | 0.9563 | 0.10877 |
| Whole-fruit rosehip | 700 | 0.10 | Starink | 101.301 | 16.472 | 30.428 to 172.174 | 0.9498 | 0.10898 |
| Whole-fruit rosehip | 700 | 0.20 | Friedman | 104.636 | 34.202 | -42.521 to 251.794 | 0.8239 | 0.20510 |
| Whole-fruit rosehip | 700 | 0.20 | KAS | 106.777 | 29.955 | -22.110 to 235.663 | 0.8640 | 0.17964 |
| Whole-fruit rosehip | 700 | 0.20 | FWO | 109.572 | 28.457 | -12.868 to 232.013 | 0.8811 | 0.17949 |
| Whole-fruit rosehip | 700 | 0.20 | Starink | 107.030 | 29.930 | -21.749 to 235.809 | 0.8648 | 0.17963 |
| Whole-fruit rosehip | 700 | 0.30 | Friedman | 122.912 | 21.249 | 31.484 to 214.339 | 0.9436 | 0.12261 |
| Whole-fruit rosehip | 700 | 0.30 | KAS | 113.932 | 24.098 | 10.247 to 217.617 | 0.9179 | 0.13904 |
| Whole-fruit rosehip | 700 | 0.30 | FWO | 116.928 | 22.878 | 18.491 to 215.366 | 0.9289 | 0.13884 |
| Whole-fruit rosehip | 700 | 0.30 | Starink | 114.203 | 24.077 | 10.607 to 217.799 | 0.9184 | 0.13903 |
| Whole-fruit rosehip | 700 | 0.40 | Friedman | 130.299 | 24.185 | 26.239 to 234.360 | 0.9355 | 0.12842 |
| Whole-fruit rosehip | 700 | 0.40 | KAS | 124.731 | 24.666 | 18.601 to 230.861 | 0.9275 | 0.13097 |
| Whole-fruit rosehip | 700 | 0.40 | FWO | 127.611 | 23.415 | 26.863 to 228.359 | 0.9369 | 0.13077 |
| Whole-fruit rosehip | 700 | 0.40 | Starink | 125.011 | 24.645 | 18.972 to 231.049 | 0.9279 | 0.13096 |
| Whole-fruit rosehip | 700 | 0.50 | Friedman | 136.177 | 18.903 | 54.845 to 217.508 | 0.9629 | 0.09662 |
| Whole-fruit rosehip | 700 | 0.50 | KAS | 130.378 | 23.266 | 30.272 to 230.484 | 0.9401 | 0.11892 |
| Whole-fruit rosehip | 700 | 0.50 | FWO | 133.348 | 22.080 | 38.347 to 228.349 | 0.9480 | 0.11870 |
| Whole-fruit rosehip | 700 | 0.50 | Starink | 130.669 | 23.246 | 30.650 to 230.687 | 0.9405 | 0.11891 |
| Whole-fruit rosehip | 700 | 0.60 | Friedman | 143.603 | 27.343 | 25.954 to 261.252 | 0.9324 | 0.13249 |
| Whole-fruit rosehip | 700 | 0.60 | KAS | 137.970 | 23.885 | 35.202 to 240.737 | 0.9435 | 0.11573 |
| Whole-fruit rosehip | 700 | 0.60 | FWO | 140.875 | 22.664 | 43.362 to 238.388 | 0.9508 | 0.11550 |
| Whole-fruit rosehip | 700 | 0.60 | Starink | 138.267 | 23.864 | 35.590 to 240.944 | 0.9438 | 0.11572 |
| Whole-fruit rosehip | 700 | 0.70 | Friedman | 41.660 | 69.284 | -256.445 to 339.765 | 0.1531 | 0.41840 |
| Whole-fruit rosehip | 700 | 0.70 | KAS | 101.775 | 33.489 | -42.317 to 245.866 | 0.8220 | 0.20224 |
| Whole-fruit rosehip | 700 | 0.70 | FWO | 106.852 | 31.775 | -29.864 to 243.568 | 0.8497 | 0.20183 |
| Whole-fruit rosehip | 700 | 0.70 | Starink | 102.117 | 33.459 | -41.847 to 246.082 | 0.8232 | 0.20222 |
| Whole-fruit rosehip | 700 | 0.80 | Friedman | 35.132 | 43.878 | -153.658 to 223.923 | 0.2427 | 0.40288 |
| Whole-fruit rosehip | 700 | 0.80 | KAS | 47.933 | 38.007 | -115.600 to 211.466 | 0.4430 | 0.34898 |
| Whole-fruit rosehip | 700 | 0.80 | FWO | 56.625 | 36.096 | -98.685 to 211.935 | 0.5517 | 0.34860 |
| Whole-fruit rosehip | 700 | 0.80 | Starink | 48.359 | 37.975 | -115.036 to 211.754 | 0.4478 | 0.34897 |
| Whole-fruit rosehip | 700 | 0.90 | Friedman | 41.674 | 51.726 | -180.886 to 264.235 | 0.2450 | 0.48030 |
| Whole-fruit rosehip | 700 | 0.90 | KAS | 35.777 | 44.195 | -154.380 to 225.934 | 0.2468 | 0.41037 |
| Whole-fruit rosehip | 700 | 0.90 | FWO | 46.330 | 42.053 | -134.609 to 227.270 | 0.3777 | 0.41071 |
| Whole-fruit rosehip | 700 | 0.90 | Starink | 36.266 | 44.161 | -153.745 to 226.277 | 0.2522 | 0.41039 |

Note. Each estimate is based on the four programmed heating rates. Confidence intervals are two-sided 95% t intervals for the regression slope with 2 degrees of freedom, transformed to Eₐ using the locked method-specific slope relations. KAS uses the manuscript sign convention ln(Tα²/β), giving Eₐ = Rsα.

Table S31. Endpoint-sensitivity diagnostics, reliability classifications, principal-reliable-interval summaries, endpoint mass retention, and robustness of preparation ordering.

Panel A. Pointwise reliability classification.

| Feedstock | Tf (°C) | α | Integral mean Eₐ | Integral CV (%) | Minimum integral R² | Friedman Eₐ | Friedman deviation (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 600 | 0.10 | 101.254 | 1.82 | 0.9380 | 109.818 | 8.46 |
| Rosehip seed | 600 | 0.20 | 121.702 | 1.24 | 0.9785 | 138.089 | 13.46 |
| Rosehip seed | 600 | 0.30 | 140.903 | 0.82 | 0.9819 | 154.793 | 9.86 |
| Rosehip seed | 600 | 0.40 | 152.517 | 0.67 | 0.9889 | 155.909 | 2.22 |
| Rosehip seed | 600 | 0.50 | 154.576 | 0.72 | 0.9972 | 155.439 | 0.56 |
| Rosehip seed | 600 | 0.60 | 155.458 | 0.76 | 0.9972 | 155.939 | 0.31 |
| Rosehip seed | 600 | 0.70 | 166.398 | 0.60 | 0.9795 | 204.355 | 22.81 |
| Rosehip seed | 600 | 0.80 | 194.202 | 0.26 | 0.9040 | 202.606 | 4.33 |
| Rosehip seed | 600 | 0.90 | 155.425 | 1.35 | 0.8688 | 114.442 | 26.37 |
| Rosehip seed | 650 | 0.10 | 100.089 | 1.89 | 0.9456 | 109.439 | 9.34 |
| Rosehip seed | 650 | 0.20 | 119.991 | 1.31 | 0.9809 | 135.206 | 12.68 |
| Rosehip seed | 650 | 0.30 | 137.196 | 0.93 | 0.9842 | 148.917 | 8.54 |
| Rosehip seed | 650 | 0.40 | 146.657 | 0.82 | 0.9909 | 149.966 | 2.26 |
| Rosehip seed | 650 | 0.50 | 149.737 | 0.84 | 0.9972 | 152.248 | 1.68 |
| Rosehip seed | 650 | 0.60 | 150.691 | 0.88 | 0.9961 | 149.745 | 0.63 |
| Rosehip seed | 650 | 0.70 | 157.668 | 0.80 | 0.9669 | 179.437 | 13.81 |
| Rosehip seed | 650 | 0.80 | 161.399 | 0.92 | 0.8990 | 162.317 | 0.57 |
| Rosehip seed | 650 | 0.90 | 100.957 | 3.79 | 0.8761 | 73.177 | 27.52 |
| Rosehip seed | 700 | 0.10 | 99.248 | 1.93 | 0.9518 | 109.029 | 9.85 |
| Rosehip seed | 700 | 0.20 | 118.625 | 1.36 | 0.9828 | 132.803 | 11.95 |
| Rosehip seed | 700 | 0.30 | 134.037 | 1.03 | 0.9851 | 144.057 | 7.48 |
| Rosehip seed | 700 | 0.40 | 142.041 | 0.95 | 0.9912 | 145.623 | 2.52 |
| Rosehip seed | 700 | 0.50 | 146.048 | 0.94 | 0.9963 | 149.539 | 2.39 |
| Rosehip seed | 700 | 0.60 | 146.724 | 0.99 | 0.9942 | 143.537 | 2.17 |
| Rosehip seed | 700 | 0.70 | 147.164 | 1.08 | 0.9474 | 150.079 | 1.98 |
| Rosehip seed | 700 | 0.80 | 138.016 | 1.60 | 0.8794 | 131.633 | 4.62 |
| Rosehip seed | 700 | 0.90 | 77.821 | 5.97 | 0.8746 | 71.281 | 8.40 |
| Rosehip skin | 600 | 0.10 | 128.942 | 0.35 | 0.9553 | 111.038 | 13.88 |
| Rosehip skin | 600 | 0.20 | 143.397 | 0.17 | 0.8739 | 174.504 | 21.69 |
| Rosehip skin | 600 | 0.30 | 218.305 | 0.79 | 0.8237 | 275.454 | 26.18 |
| Rosehip skin | 600 | 0.40 | 344.558 | 1.47 | 0.8639 | 337.723 | 1.98 |
| Rosehip skin | 600 | 0.50 | 253.124 | 0.85 | 0.9717 | 220.760 | 12.79 |
| Rosehip skin | 600 | 0.60 | 186.226 | 0.07 | 0.9433 | 161.006 | 13.54 |
| Rosehip skin | 600 | 0.70 | 153.768 | 0.78 | 0.7981 | 130.800 | 14.94 |
| Rosehip skin | 600 | 0.80 | 109.815 | 2.71 | 0.7772 | 100.678 | 8.32 |
| Rosehip skin | 600 | 0.90 | 136.186 | 2.11 | 0.8745 | 156.013 | 14.56 |
| Rosehip skin | 650 | 0.10 | 126.819 | 0.41 | 0.9574 | 110.509 | 12.86 |
| Rosehip skin | 650 | 0.20 | 143.321 | 0.18 | 0.8806 | 173.940 | 21.36 |
| Rosehip skin | 650 | 0.30 | 221.612 | 0.81 | 0.8525 | 276.439 | 24.74 |
| Rosehip skin | 650 | 0.40 | 316.553 | 1.34 | 0.9455 | 301.693 | 4.69 |
| Rosehip skin | 650 | 0.50 | 220.052 | 0.54 | 0.9648 | 191.359 | 13.04 |
| Rosehip skin | 650 | 0.60 | 168.830 | 0.33 | 0.9352 | 148.509 | 12.04 |
| Rosehip skin | 650 | 0.70 | 127.637 | 1.58 | 0.7880 | 92.404 | 27.60 |
| Rosehip skin | 650 | 0.80 | 94.943 | 3.72 | 0.8099 | 87.831 | 7.49 |
| Rosehip skin | 650 | 0.90 | 124.062 | 2.72 | 0.8690 | 142.060 | 14.51 |
| Rosehip skin | 700 | 0.10 | 124.819 | 0.46 | 0.9597 | 109.634 | 12.17 |
| Rosehip skin | 700 | 0.20 | 142.474 | 0.20 | 0.8919 | 172.276 | 20.92 |
| Rosehip skin | 700 | 0.30 | 222.928 | 0.82 | 0.8936 | 273.722 | 22.78 |
| Rosehip skin | 700 | 0.40 | 277.215 | 1.11 | 0.9850 | 257.429 | 7.14 |
| Rosehip skin | 700 | 0.50 | 191.193 | 0.18 | 0.9516 | 166.495 | 12.92 |
| Rosehip skin | 700 | 0.60 | 152.209 | 0.69 | 0.9204 | 134.549 | 11.60 |
| Rosehip skin | 700 | 0.70 | 103.271 | 2.70 | 0.7822 | 65.254 | 36.81 |
| Rosehip skin | 700 | 0.80 | 83.540 | 4.74 | 0.8276 | 90.591 | 8.44 |
| Rosehip skin | 700 | 0.90 | 102.031 | 4.09 | 0.8330 | 103.227 | 1.17 |
| Whole-fruit rosehip | 600 | 0.10 | 105.489 | 1.19 | 0.9595 | 112.367 | 6.52 |
| Whole-fruit rosehip | 600 | 0.20 | 117.881 | 1.04 | 0.8879 | 119.618 | 1.47 |
| Whole-fruit rosehip | 600 | 0.30 | 122.326 | 1.16 | 0.9302 | 129.900 | 6.19 |
| Whole-fruit rosehip | 600 | 0.40 | 136.273 | 0.92 | 0.9455 | 143.164 | 5.06 |
| Whole-fruit rosehip | 600 | 0.50 | 142.898 | 0.89 | 0.9555 | 146.199 | 2.31 |
| Whole-fruit rosehip | 600 | 0.60 | 151.281 | 0.79 | 0.9673 | 162.293 | 7.28 |
| Whole-fruit rosehip | 600 | 0.70 | 151.753 | 0.89 | 0.9632 | 130.803 | 13.81 |
| Whole-fruit rosehip | 600 | 0.80 | 92.887 | 3.73 | 0.6564 | 72.692 | 21.74 |
| Whole-fruit rosehip | 600 | 0.90 | 67.232 | 7.20 | 0.4248 | 75.516 | 12.32 |
| Whole-fruit rosehip | 650 | 0.10 | 103.617 | 1.27 | 0.9547 | 110.249 | 6.40 |
| Whole-fruit rosehip | 650 | 0.20 | 112.912 | 1.23 | 0.8788 | 111.866 | 0.93 |
| Whole-fruit rosehip | 650 | 0.30 | 118.432 | 1.30 | 0.9273 | 126.378 | 6.71 |
| Whole-fruit rosehip | 650 | 0.40 | 130.976 | 1.09 | 0.9400 | 136.499 | 4.22 |
| Whole-fruit rosehip | 650 | 0.50 | 136.707 | 1.07 | 0.9512 | 140.950 | 3.10 |
| Whole-fruit rosehip | 650 | 0.60 | 144.852 | 0.97 | 0.9587 | 153.389 | 5.89 |
| Whole-fruit rosehip | 650 | 0.70 | 128.809 | 1.60 | 0.9097 | 82.003 | 36.34 |
| Whole-fruit rosehip | 650 | 0.80 | 65.339 | 6.71 | 0.5269 | 47.062 | 27.97 |
| Whole-fruit rosehip | 650 | 0.90 | 50.844 | 10.77 | 0.3257 | 56.385 | 10.90 |
| Whole-fruit rosehip | 700 | 0.10 | 101.974 | 1.35 | 0.9495 | 108.305 | 6.21 |
| Whole-fruit rosehip | 700 | 0.20 | 107.793 | 1.43 | 0.8640 | 104.636 | 2.93 |
| Whole-fruit rosehip | 700 | 0.30 | 115.021 | 1.44 | 0.9179 | 122.912 | 6.86 |
| Whole-fruit rosehip | 700 | 0.40 | 125.784 | 1.26 | 0.9275 | 130.299 | 3.59 |
| Whole-fruit rosehip | 700 | 0.50 | 131.465 | 1.25 | 0.9401 | 136.177 | 3.58 |
| Whole-fruit rosehip | 700 | 0.60 | 139.037 | 1.15 | 0.9435 | 143.603 | 3.28 |
| Whole-fruit rosehip | 700 | 0.70 | 103.581 | 2.74 | 0.8220 | 41.660 | 59.78 |
| Whole-fruit rosehip | 700 | 0.80 | 50.972 | 9.61 | 0.4430 | 35.132 | 31.08 |
| Whole-fruit rosehip | 700 | 0.90 | 39.458 | 15.10 | 0.2468 | 41.674 | 5.62 |

Panel B. Pointwise endpoint sensitivity of the integral-method mean.

| Feedstock | α | Eₐ,600 | Eₐ,650 | Eₐ,700 | ΔEₐ,600 | ΔEₐ,650 | δEₐ,600 (%) | δEₐ,650 (%) | Endpoint range |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 0.10 | 101.254 | 100.089 | 99.248 | 2.005 | 0.841 | 2.02 | 0.85 | 2.005 |
| Rosehip seed | 0.20 | 121.702 | 119.991 | 118.625 | 3.077 | 1.366 | 2.59 | 1.15 | 3.077 |
| Rosehip seed | 0.30 | 140.903 | 137.196 | 134.037 | 6.866 | 3.160 | 5.12 | 2.36 | 6.866 |
| Rosehip seed | 0.40 | 152.517 | 146.657 | 142.041 | 10.476 | 4.616 | 7.38 | 3.25 | 10.476 |
| Rosehip seed | 0.50 | 154.576 | 149.737 | 146.048 | 8.528 | 3.689 | 5.84 | 2.53 | 8.528 |
| Rosehip seed | 0.60 | 155.458 | 150.691 | 146.724 | 8.733 | 3.967 | 5.95 | 2.70 | 8.733 |
| Rosehip seed | 0.70 | 166.398 | 157.668 | 147.164 | 19.235 | 10.504 | 13.07 | 7.14 | 19.235 |
| Rosehip seed | 0.80 | 194.202 | 161.399 | 138.016 | 56.186 | 23.383 | 40.71 | 16.94 | 56.186 |
| Rosehip seed | 0.90 | 155.425 | 100.957 | 77.821 | 77.605 | 23.137 | 99.72 | 29.73 | 77.605 |
| Rosehip skin | 0.10 | 128.942 | 126.819 | 124.819 | 4.123 | 2.001 | 3.30 | 1.60 | 4.123 |
| Rosehip skin | 0.20 | 143.397 | 143.321 | 142.474 | 0.923 | 0.847 | 0.65 | 0.59 | 0.923 |
| Rosehip skin | 0.30 | 218.305 | 221.612 | 222.928 | -4.623 | -1.316 | 2.07 | 0.59 | 4.623 |
| Rosehip skin | 0.40 | 344.558 | 316.553 | 277.215 | 67.343 | 39.337 | 24.29 | 14.19 | 67.343 |
| Rosehip skin | 0.50 | 253.124 | 220.052 | 191.193 | 61.931 | 28.859 | 32.39 | 15.09 | 61.931 |
| Rosehip skin | 0.60 | 186.226 | 168.830 | 152.209 | 34.017 | 16.620 | 22.35 | 10.92 | 34.017 |
| Rosehip skin | 0.70 | 153.768 | 127.637 | 103.271 | 50.496 | 24.366 | 48.90 | 23.59 | 50.496 |
| Rosehip skin | 0.80 | 109.815 | 94.943 | 83.540 | 26.275 | 11.403 | 31.45 | 13.65 | 26.275 |
| Rosehip skin | 0.90 | 136.186 | 124.062 | 102.031 | 34.155 | 22.031 | 33.48 | 21.59 | 34.155 |
| Whole-fruit rosehip | 0.10 | 105.489 | 103.617 | 101.974 | 3.515 | 1.643 | 3.45 | 1.61 | 3.515 |
| Whole-fruit rosehip | 0.20 | 117.881 | 112.912 | 107.793 | 10.088 | 5.119 | 9.36 | 4.75 | 10.088 |
| Whole-fruit rosehip | 0.30 | 122.326 | 118.432 | 115.021 | 7.304 | 3.411 | 6.35 | 2.97 | 7.304 |
| Whole-fruit rosehip | 0.40 | 136.273 | 130.976 | 125.784 | 10.488 | 5.191 | 8.34 | 4.13 | 10.488 |
| Whole-fruit rosehip | 0.50 | 142.898 | 136.707 | 131.465 | 11.433 | 5.242 | 8.70 | 3.99 | 11.433 |
| Whole-fruit rosehip | 0.60 | 151.281 | 144.852 | 139.037 | 12.244 | 5.814 | 8.81 | 4.18 | 12.244 |
| Whole-fruit rosehip | 0.70 | 151.753 | 128.809 | 103.581 | 48.171 | 25.228 | 46.51 | 24.36 | 48.171 |
| Whole-fruit rosehip | 0.80 | 92.887 | 65.339 | 50.972 | 41.915 | 14.366 | 82.23 | 28.18 | 41.915 |
| Whole-fruit rosehip | 0.90 | 67.232 | 50.844 | 39.458 | 27.774 | 11.386 | 70.39 | 28.86 | 27.774 |

Panel C. Principal-window endpoint summaries.

| Feedstock | T_f (°C) | Principal interval | Mean Eₐ | Range Eₐ | Minimum R² | Maximum CV (%) | Direction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 600 | α = 0.10–0.70 | 141.830 | 101.254–166.398 | 0.9380 | 1.82 | Increasing |
| Rosehip seed | 650 | α = 0.10–0.70 | 137.433 | 100.089–157.668 | 0.9456 | 1.89 | Increasing |
| Rosehip seed | 700 | α = 0.10–0.70 | 133.412 | 99.248–147.164 | 0.9474 | 1.93 | Increasing |
| Rosehip skin | 600 | α = 0.40–0.60 | 261.303 | 186.226–344.558 | 0.8639 | 1.47 | Decreasing |
| Rosehip skin | 650 | α = 0.40–0.60 | 235.145 | 168.830–316.553 | 0.9352 | 1.34 | Decreasing |
| Rosehip skin | 700 | α = 0.40–0.60 | 206.873 | 152.209–277.215 | 0.9204 | 1.11 | Decreasing |
| Whole-fruit rosehip | 600 | α = 0.30–0.60 | 138.194 | 122.326–151.281 | 0.9302 | 1.16 | Increasing |
| Whole-fruit rosehip | 650 | α = 0.30–0.60 | 132.742 | 118.432–144.852 | 0.9273 | 1.30 | Increasing |
| Whole-fruit rosehip | 700 | α = 0.30–0.60 | 127.827 | 115.021–139.037 | 0.9179 | 1.44 | Increasing |

Panel D. Endpoint mass retention and cross-heating-rate spread.

| Feedstock | T_f (°C) | R₅ | R₁₀ | R₁₅ | R₂₀ | Minimum | Maximum | Spread (pp) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rosehip seed | 600 | 27.700 | 28.691 | 27.852 | 26.330 | 26.330 | 28.691 | 2.361 |
| Rosehip seed | 650 | 26.760 | 27.572 | 26.144 | 24.460 | 24.460 | 27.572 | 3.112 |
| Rosehip seed | 700 | 25.980 | 26.793 | 24.755 | 22.910 | 22.910 | 26.793 | 3.883 |
| Rosehip skin | 600 | 35.385 | 35.574 | 32.737 | 33.387 | 32.737 | 35.574 | 2.837 |
| Rosehip skin | 650 | 34.276 | 34.396 | 31.009 | 31.628 | 31.009 | 34.396 | 3.387 |
| Rosehip skin | 700 | 33.367 | 33.467 | 29.361 | 29.970 | 29.361 | 33.467 | 4.106 |
| Whole-fruit rosehip | 600 | 31.150 | 31.449 | 30.330 | 25.125 | 25.125 | 31.449 | 6.324 |
| Whole-fruit rosehip | 650 | 30.090 | 30.310 | 29.450 | 22.837 | 22.837 | 30.310 | 7.473 |
| Whole-fruit rosehip | 700 | 29.190 | 29.441 | 28.740 | 20.929 | 20.929 | 29.441 | 8.511 |

Panel E. Shared-window preparation ordering and trajectory-direction robustness.

| T_f (°C) | Shared interval | Skin mean | Seed mean | Whole-fruit mean | Ordering |
| --- | --- | --- | --- | --- | --- |
| 600 | α = 0.40–0.60 | 261.303 | 154.184 | 143.484 | Rosehip skin > Rosehip seed > Whole-fruit rosehip |
| 650 | α = 0.40–0.60 | 235.145 | 149.028 | 137.511 | Rosehip skin > Rosehip seed > Whole-fruit rosehip |
| 700 | α = 0.40–0.60 | 206.873 | 144.938 | 132.096 | Rosehip skin > Rosehip seed > Whole-fruit rosehip |

## S10.2. Supplementary figure S16

Figure S16. Endpoint sensitivity of integral-method mean apparent activation-energy profiles for rosehip seed (a), rosehip skin (b), and whole-fruit rosehip (c). Open markers identify low-confidence points under the unchanged analytical-support rules; shaded regions denote the preparation-specific principal reliable conversion intervals on the adopted 700 °C basis.

# References

Burnham, K. P., & Anderson, D. R. (2004). Multimodel inference: Understanding AIC and BIC in model selection. Sociological Methods & Research, 33(2), 261–304. https://doi.org/10.1177/0049124104268644.

Conesa, J. A., & Domene, A. (2011). Biomasses pyrolysis and combustion kinetics through n-th order parallel reactions. Thermochimica Acta, 523, 176–181. https://doi.org/10.1016/j.tca.2011.05.021.

de Castro, L. L. R. L., de Sousa Neiva, L. G., Andrade, D. S. C., de Almeida, Y. M. B., de Sousa e Silva, H., Alves, T. S., & Barbosa, R. (2026). Sustainable hybrid films based on Ecovio, sodium nitrite, and babassu mesocarp starch for potential packaging applications. Polymers for Advanced Technologies, e70550. https://doi.org/10.1002/pat.70550.

Kim, H.-Y. (2019). Statistical notes for clinical researchers: Simple linear regression 3 - residual analysis. Restorative Dentistry & Endodontics, 44(1), e11. https://doi.org/10.5395/rde.2019.44.e11.

Kissinger, H. E. (1956). Variation of peak temperature with heating rate in differential thermal analysis. Journal of Research of the National Bureau of Standards, 57(4), 217–221. https://doi.org/10.6028/jres.057.026.

Li, J., Valentine, J. D., & Rana, A. E. (1999). The modified three point Gaussian method for determining Gaussian peak parameters. Nuclear Instruments and Methods in Physics Research Section A, 422(1–3), 438–443. https://doi.org/10.1016/S0168-9002(98)01113-9.

Liu, Y., Yang, L., Ma, C., & Zhang, Y. (2019). Thermal behavior of sweet potato starch by non-isothermal thermogravimetric analysis. Materials, 12(5), 699. https://doi.org/10.3390/ma12050699.

Mani, T., Murugan, P., & Mahinpey, N. (2009). Determination of distributed activation energy model kinetic parameters using simulated annealing optimization method for nonisothermal pyrolysis of lignin. Industrial & Engineering Chemistry Research, 48, 1464–1467. https://doi.org/10.1021/ie8013605.

Mishra, P., Pandey, C. M., Singh, U., Gupta, A., Sahu, C., & Keshri, A. (2019). Descriptive statistics and normality tests for statistical data. Annals of Cardiac Anaesthesia, 22(1), 67–72. https://doi.org/10.4103/aca.ACA_157_18.

Müller, L., Rubio-Pérez, G., Bach, A., Muñoz-Rujas, N., Aguilar, F., & Worlitschek, J. (2020). Consistent DSC and TGA methodology as basis for the measurement and comparison of thermo-physical properties of phase change materials. Materials, 13(20), 4486. https://doi.org/10.3390/ma13204486.

Singh, V. K., Musharraf, M., Swain, S. K., Dey, R. K., & Shekhar, S. (2023). Thermal kinetics of poly(amidoamine) functionalized silica gel. Bulgarian Chemical Communications, 55(Special Issue A), 225–231. https://doi.org/10.34049/bcc.55.A.0034.

Starink, M. J. (2003). The determination of activation energy from linear heating rate experiments: a comparison of the accuracy of isoconversion methods. Thermochimica Acta, 404, 163–176. https://doi.org/10.1016/S0040-6031(03)00144-8.

Soomro, A., Koçer, A. T., & Balkanlı, D. (2026). Fraction-resolved pyrolysis behavior, conversion-dependent kinetics, and predictive severity framework for engineered biochars from fig-processing residues. Biomass and Bioenergy, 214, 109556. https://doi.org/10.1016/j.biombioe.2026.109556.

Tu, R., Sun, Y., Wu, Y., Fan, X., Wang, J., Cheng, S., Jia, Z., Jiang, E., & Xu, X. (2019). Improvement of corn stover fuel properties via hydrothermal carbonization combined with surfactant. Biotechnology for Biofuels, 12, 249. https://doi.org/10.1186/s13068-019-1581-x.

Várhegyi, G., Bobály, B., Jakab, E., & Chen, H. (2011). Thermogravimetric study of biomass pyrolysis kinetics: A distributed activation energy model with prediction tests. Energy & Fuels, 25, 24–32. https://doi.org/10.1021/ef101079r.

Vyazovkin, S. (2020). Kissinger method in kinetics of materials: Things to beware and be aware of. Molecules, 25(12), 2813. https://doi.org/10.3390/molecules25122813.

Wang, H., Yan, X., Wang, Q., Wu, W., Tian, Y., & Xu, Q. (2026). Study on the modification mechanism and rheological properties of bio-oil-based composite-modified material for TOP-DOWN crack treatment in long-life pavement. Journal of Composites Science, 10(6), 298. https://doi.org/10.3390/jcs10060298.