# Supplementary Information for

# Diagnosing Spatial Disparities in Urban Flood Burden through Hydro-Environmental Normalization in Jakarta, Indonesia

This Supplementary Information contains only the technical tables needed to clarify the methods, support selected results, and justify the diagnostic interpretation in the main manuscript.

## S1. Data sources, indicators, transformations, and analytical use

### Table S1. Data sources, derived indicators, transformations, and analytical use

| Data component | Dataset or source | Native scale / period | Derived variable used in analysis | Transformation or harmonization | Analytical use | Main uncertainty |
| --- | --- | --- | --- | --- | --- | --- |
| Administrative boundary | Jakarta kelurahan boundary | Vector; static | Kelurahan analytical units | All raster and vector datasets harmonized and summarized to the common kelurahan analytical framework | Defines the common spatial unit for forcing, burden, expectation modelling, and spatial diagnostics | Boundary aggregation may smooth within-kelurahan variation (MAUP) |
| Terrain elevation | DEMNAS | ~8 m; static | Elevation | Subsidence-adjusted DEM summarized by zonal mean; dynamically normalized (5th–95th percentile); direction reversed so lower elevation represents stronger forcing | Inland and coastal hydro-environmental forcing | DEM timing and vertical uncertainty may differ from 2020–2024 terrain conditions |
| Land subsidence | InSAR-derived Jakarta subsidence surface | Gridded raster | Subsidence-adjusted elevation | Cumulative subsidence applied to the DEM before terrain derivation | Represents effective terrain conditions during the analytical period | Assumes subsidence representation is sufficiently stable throughout the study period |
| Terrain slope | DEMNAS-derived slope | ~8 m; static | Slope | Calculated from the subsidence-adjusted DEM; summarized by zonal mean and dynamically normalized | Inland and coastal hydro-environmental forcing | Local drainage micro-topography and engineered channels are not represented |
| Rainfall forcing | CHIRPS daily precipitation | ~0.05°; 2020–2024 | Maximum daily precipitation during the study period | Pixel-wise maximum extracted across 2020–2024, then summarized by zonal mean and dynamically normalized | Inland and coastal hydro-environmental forcing | Coarse spatial resolution relative to kelurahan scale; represents study-period maximum rather than formal climatological Rx1day |
| Hydrological connectivity | JRC Global Surface Water | ~30 m | River / permanent-water proximity | Euclidean distance raster generated from permanent water, summarized by zonal mean, dynamically normalized, and direction reversed so shorter distance represents stronger forcing | Inland and coastal hydro-environmental forcing | Small drains, canals, and engineered flow paths may not be represented |
| Coastal influence | Jakarta RBI coastline | Vector; static | Coastline proximity | Euclidean distance raster generated from the coastline, summarized by zonal mean, dynamically normalized, direction reversed, and combined with a linear distance-decay weighting | Coastal hydro-environmental forcing | Does not fully represent engineered shoreline modification, tidal hydraulics, or coastal structures |
| Built-up intensity | Sentinel-2 Harmonized Surface Reflectance | 10 m; 2020–2024 composite | Built-up intensity index | Spectral index derived from the study-period composite, summarized by zonal mean, and dynamically normalized | Inland and coastal hydro-environmental forcing | Spectral mixing and dense urban morphology may affect the built-up signal |
| Surface impermeability | ESA WorldCover | 10 m; static | Impervious land-cover proxy | Impervious land-cover class summarized by zonal mean; retained without percentile normalization | Inland and coastal hydro-environmental forcing | Mixed urban surfaces may be simplified by land-cover classification |
| Flood observation | Sentinel-1 GRD SAR | ~10 m; selected flood events during 2020–2024 | Core and extended flood masks | Multi-temporal SAR composites with permanent-water masking, terrain filtering, and connected-pixel filtering | Observed flood extent and flood fraction | Urban SAR mapping uncertainty from layover, radar shadow, double-bounce scattering, and threshold sensitivity |
| Population exposure | WorldPop gridded population | ~100 m | Population and population density | Population summarized by zonal sum; population density calculated using kelurahan area and combined with mapped flood extent | Observed flood-burden construction | Does not capture daytime mobility, temporary displacement, or event-time exposure |
| Population-exposure-weighted burden proxy | Derived from Sentinel-1 and WorldPop | Kelurahan-year; 2020–2024 | Flood fraction × population density | Annual burden calculated for each year and aggregated to a persistent burden using the median across annual observations | Response variable for expectation modelling | Represents relative flood burden rather than flood depth, economic loss, or comprehensive disaster impact |
| Hydro-environmental forcing | Derived composite | Kelurahan; 2020–2024 baseline | Inland, coastal, and compound forcing indices | Individual indicators dynamically normalized (5th–95th percentile), combined using literature-informed and equal-weight specifications, then summarized at kelurahan level | Environmental baseline for expected burden | Indicator selection, weighting strategy, and normalization choices influence the represented forcing surface |

Note: Higher normalized values consistently represent stronger flood-generating conditions. For distance, elevation, and slope variables, the normalized direction is reversed. Flood observation and population exposure are not part of the hydro-environmental baseline; they are used to construct observed burden.

## S2. Hydro-environmental weights, forcing statistics, and observed-burden construction

### Panel A. Indicator weights used for inland and coastal hydro-environmental forcing

| Indicator | Inland literature-informed weighting | Coastal literature-informed weighting | Equal-weight specification | Direction after normalization |
| --- | --- | --- | --- | --- |
| Maximum daily precipitation, 2020–2024 | 0.270 | 0.150 | Equal within component | Higher value = stronger forcing |
| Subsidence-adjusted elevation | 0.124 | 0.280 | Equal within component | Lower value = stronger forcing |
| Terrain slope | 0.164 | 0.050 | Equal within component | Lower value = stronger forcing |
| River or permanent-water proximity | 0.155 | 0.150 | Equal within component | Closer distance = stronger forcing |
| Coastline proximity | — | 0.220 | Equal within component | Closer distance = stronger forcing |
| Built-up intensity | 0.190 | 0.100 | Equal within component | Higher value = stronger forcing |
| Impervious surface | 0.097 | 0.050 | Equal within component | Higher value = stronger forcing |

Note: Inland indicator weights were adapted from the Analytic Hierarchy Process (AHP)-based flood hazard framework developed by Ariyani et al. (2024) [1], which assigned the highest relative importance to maximum daily precipitation (0.270), followed by built-up intensity (0.190), terrain slope (0.164), river proximity (0.155), subsidence-adjusted elevation (adapted from topography; 0.124), and impervious surface (adapted from soil-related infiltration control; 0.097). Coastal indicator weights were developed through a literature synthesis of coastal flood susceptibility and coastal vulnerability studies. Greater emphasis was assigned to subsidence-adjusted elevation (0.280) and coastline proximity (0.220), reflecting the dominant influence of low-lying coastal terrain and coastal connectivity consistently reported in previous AHP- and MCDA-based studies [2], [3]. Intermediate weights were assigned to precipitation (0.150) and river or estuarine connectivity (0.150), whereas built-up intensity (0.100), terrain slope (0.050), and impervious surface (0.050) received lower weights to represent secondary controls on coastal flood generation. This weighting scheme represents a literature-informed parameterization rather than a newly conducted AHP. Equal weighting was additionally evaluated as a sensitivity specification.

### Panel B. Summary statistics of hydro-environmental forcing components

| Forcing component | Mean | SD | Median | Minimum | Maximum |
| --- | --- | --- | --- | --- | --- |
| Inland forcing (literature-informed) | 0.6823 | 0.1364 | 0.708 | 0.1019 | 0.9799 |
| Coastal forcing (literature-informed) | 0.4122 | 0.2455 | 0 | 0.0000 | 0.9535 |
| Compound forcing, (literature-informed) | 0.733 | 0.102 | 0.725 | 0.42 | 0.945 |
| Compound forcing (equal-weight) | 0.758 | 0.101 | 0.757 | 0.472 | 0.959 |

### Panel C. Observed-burden construction and annual mapped flood extent

| Component | Definition or value | Analytical role |
| --- | --- | --- |
| Annual flood fraction | Fit=Aflood it/Ai | Proportion of kelurahan area mapped as flooded in selected event year t |
| Population density | Dit=Popit/Ai | Population exposure normalized by kelurahan area |
| Annual observed burden | Bit=Fit×Dit | Relative burden indicator combining mapped flood occurrence and population concentration |
| Persistent observed burden | Bi=medianBi,2020,...,Bi,2024 | Response variable used for environmental expectation modelling |
| 2020 mapped flood extent | 43.76 km² | Selected Sentinel-1 flood observation |
| 2021 mapped flood extent | 45.63 km² | Selected Sentinel-1 flood observation |
| 2022 mapped flood extent | 46.42 km² | Selected Sentinel-1 flood observation |
| 2023 mapped flood extent | 34.54 km² | Selected Sentinel-1 flood observation |
| 2024 mapped flood extent | 34.69 km² | Selected Sentinel-1 flood observation |
| Mean mapped flood extent | 41.01 km² | Mean across selected annual observations |

Note: The compound forcing score was calculated using a fuzzy algebraic sum of inland and coastal forcing. The observed-burden metric is a relative population-exposure-weighted indicator and not a measure of flood depth, flood duration, direct economic loss, household damage, or social vulnerability.

## S3. Sentinel-1 flood mapping parameters and expectation-model diagnostics

### Panel A. Selected Sentinel-1 flood-observation windows

| Year | Event label | Orbit | Pre-event window | Flood-event window | Post-event window | Analytical use |
| --- | --- | --- | --- | --- | --- | --- |
| 2020 | New Year Flood | Descending | 2019-09-01 to 2019-12-20 | 2019-12-29 to 2020-01-07 | 2020-01-15 to 2020-02-15 | Annual flood mask |
| 2021 | February Flood | Descending | 2020-10-01 to 2021-02-10 | 2021-02-11 to 2021-02-25 | 2021-03-01 to 2021-04-01 | Annual flood mask |
| 2022 | October Flood | Descending | 2022-06-01 to 2022-09-25 | 2022-09-28 to 2022-10-12 | 2022-10-20 to 2022-11-20 | Annual flood mask |
| 2023 | January Flood | Descending | 2022-08-01 to 2023-01-10 | 2023-01-18 to 2023-02-01 | 2023-02-10 to 2023-03-10 | Annual flood mask |
| 2024 | January–February Flood | Ascending | 2023-08-01 to 2024-01-10 | 2024-01-18 to 2024-02-07 | 2024-02-15 to 2024-03-15 | Annual flood mask |

### Panel B. Flood-mask extraction parameters

| Processing component | Core mask | Extended mask | Purpose |
| --- | --- | --- | --- |
| SAR input | Sentinel-1 GRD, VH polarization, IW mode | Sentinel-1 GRD, VH polarization, IW mode | Event-based flood observation |
| Non-urban flood rule | Ratio < 0.80 and dB difference < -1.5 | Ratio < 0.88 and dB difference < -0.8 | Detect open-surface inundation |
| Urban flood rule | dB difference > 1.8 or dB difference < -1.5 | dB difference > 1.2 or dB difference < -1.0 | Detect possible urban flood signal including double-bounce response |
| Non-urban terrain filter | Slope < 5° | Slope < 5° | Reduce implausible flood classification on steep terrain |
| Urban terrain filter | Slope < 3° | Slope < 3° | Reduce urban radar misclassification |
| Permanent-water mask | JRC Global Surface Water occurrence > 80% or seasonality > 10 | Same as core mask | Remove stable water surfaces from event flood extent |
| Land-cover mask | ESA WorldCover urban and forest classes | Same as core mask | Differentiate urban and non-urban detection logic |
| Connected-pixel filter | Minimum 8 connected pixels | Minimum 12 connected pixels | Remove isolated radar noise |
| Analytical role | Sensitivity specification | Primary flood-observation input | Test flood-mask uncertainty |

### Panel C. Environmental expectation model and residual diagnostics

| Model component or diagnostic | Value | Interpretation |
| --- | --- | --- |
| Compound forcing coefficient | 3.172 | Stronger compound forcing is associated with higher observed burden |
| Standard error | 0.162 | Precision of forcing coefficient estimate |
| t-statistic | 19.565 | Statistical strength of forcing association |
| p-value | < 0.001 | Association is statistically significant |
| R² | 0.228 | Forcing model explains part of observed-burden variation |
| Adjusted R² | 0.225 | Adjusted model fit |
| F-statistic | 76.93 | Overall model significance |
| Number of observations | 1305 | The environmental expectation model was estimated using 1,305 kelurahan-year observations, after which annual residuals were aggregated by their median to derive one HNFBDev value per kelurahan. |
| Breusch–Pagan test | < 0.001 | Heteroskedasticity detected |
| Jarque–Bera test | < 0.001 | Residual non-normality detected |
| Absolute residual versus expected burden | Positive association | Residual magnitude increases under higher expected burden |
| Recommended interpretation | — | HNFBDev should be interpreted as a relative diagnostic, not as a variance-independent performance score |

Note: The selected Sentinel-1 flood observations are event-based annual observations and should not be interpreted as a complete flood climatology. Model diagnostics justify cautious interpretation of HNFBDev as an observed-expected spatial diagnostic.

## S4. Spatial diagnostics, HNFBDev extremes, and robustness summary

### Panel A. Global and local spatial diagnostics

| Diagnostic component | Specification or class | Value | Interpretation |
| --- | --- | --- | --- |
| Spatial diagnostic variable | Standardized HNFBDev | — | Observed-expected flood-burden deviation |
| Spatial-weight matrix | k-nearest neighbours, k = 8 | — | Neighbourhood structure used in current spatial output |
| Global Moran’s I | HNFBDev spatial autocorrelation | 0.355 | Moderate positive spatial clustering |
| Permutation p-value | Global Moran’s I | 0.001 | Spatial clustering is statistically significant |
| Local Moran’s I | Not significant | 174 | Most kelurahan do not form significant local clusters |
| Local Moran’s I | Low-Low cluster | 44 | Neighbouring kelurahan with lower-than-expected burden |
| Local Moran’s I | High-High cluster | 29 | Neighbouring kelurahan with higher-than-expected burden |
| Local Moran’s I | Low-High outlier | 8 | Low-deviation kelurahan near higher-deviation neighbours |
| Local Moran’s I | High-Low outlier | 6 | High-deviation kelurahan near lower-deviation neighbours |
| Total analytical units | Kelurahan | 261 | Administrative units used in spatial diagnostics |

### Panel B. Highest positive HNFBDev locations

| Rank | Kelurahan | Residual | HNFBDev Z-score | Diagnostic meaning |
| --- | --- | --- | --- | --- |
| 1 | Tangki | 2.237 | 3.834 | Higher-than-expected burden |
| 2 | Kota Bambu Utara | 2.168 | 3.717 | Higher-than-expected burden |
| 3 | Jembatan Besi | 2.117 | 3.629 | Higher-than-expected burden |
| 4 | Duri Selatan | 2.079 | 3.564 | Higher-than-expected burden |
| 5 | Duri Utara | 2.077 | 3.56 | Higher-than-expected burden |
| 6 | Kali Anyar | 1.79 | 3.069 | Higher-than-expected burden |
| 7 | Marunda | 1.758 | 3.014 | Higher-than-expected burden |
| 8 | Jati Pulo | 1.719 | 2.947 | Higher-than-expected burden |
| 9 | Petojo Utara | 1.676 | 2.874 | Higher-than-expected burden |
| 10 | Pasar Baru | 1.523 | 2.612 | Higher-than-expected burden |

### Panel C. Robustness of literature-informed and equal-weight HNFBDev

| Robustness metric | Value | Interpretation |
| --- | --- | --- |
| Pearson correlation | 0.99 | Very high numerical agreement |
| Spearman rank correlation | 0.97 | Very high rank-order stability |
| RMSE | 0.144 | Low average difference between specifications |
| Overall implication | — | Main HNFBDev pattern is not driven primarily by the literature-informed weighting scheme |

### Panel D. Interpretation boundary

| Result type | What it supports | What it does not prove |
| --- | --- | --- |
| Positive HNFBDev | Observed burden is higher than environmentally expected | Does not prove adaptation failure, infrastructure weakness, or poor governance |
| Negative HNFBDev | Observed burden is lower than environmentally expected | Does not prove resilience or successful adaptation |
| High-High cluster | Higher-than-expected burden is spatially organized | Does not identify the causal mechanism |
| Low-Low cluster | Lower-than-expected burden is spatially organized | Does not identify protection effectiveness |
| Literature-informed weight and equal-weight robustness | Pattern is stable under alternative weighting | Does not remove all uncertainty from flood mapping, burden definition, or model specification |

[1]	D. Ariyani, Muh. Y. J. Purwanto, E. Sunarti, P. Perdinan, and A. T. Juniati, “Integrated flood hazard assessment using multi-criteria analysis and geospatial modeling,” Journal of Degraded and Mining Lands Management, vol. 11, no. 4, pp. 6121–6134, Jul. 2024, doi: 10.15243/jdmlm.2024.114.6121.

[2]	I. Sekovski, L. Del Río, and C. Armaroli, “Development of a coastal vulnerability index using analytical hierarchy process and application to Ravenna province (Italy),” Ocean Coast. Manag., vol. 183, p. 104982, Jan. 2020, doi: 10.1016/j.ocecoaman.2019.104982.

[3]	V. Hadipour, F. Vafaie, and K. Deilami, “Coastal Flooding Risk Assessment Using a GIS-Based Spatial Multi-Criteria Decision Analysis Approach,” Water (Basel)., vol. 12, no. 9, p. 2379, Aug. 2020, doi: 10.3390/w12092379.