Supplementary information for:

Radiomics machine learning study with small sample size: single random training-test set split may result in unreliable results

Contents

Supplementary Appendix 1. Magnetic resonance imaging (MRI) acquisition protocols.

Supplementary Appendix 2. Imaging preprocessing and radiomic feature extraction.

Supplementary Table 1. List of extracted radiomics features.

Supplementary Table 2. List of extracted radiomics features.

Supplementary Figure 1. Nested cross validation.

Supplementary appendix.

Appendix 1. MRI acquisition protocols

Glioblastoma (GBM) vs. brain metastasis

MRI studies were performed on one of three 3-T MR scanners (Achieva, Philips, Best, the Netherlands; Ingenia, Philips; Discovery MR750, GE, Milwaukee, WI) with an eight-channel sensitivity-encoding head coil. Achieva, Ingenia and Discovery MR750 were used for 74, 32 and 3 patients with glioblastoma, and 31, 15 and 11 patients with metastasis, respectively. The protocol of T2-weighted (T2) imaging is as follows: repetition time /echo time [TR/TE], 3000-9000/80-120; field of view, 220-240 mm; section thickness, 5 mm; and matrix, 192 × 192 or 256 × 256. Three-dimensional contrast-enhanced T1-weighted images (T1C; TR/TE, 6.3-8.3/2.9-4; field of view, 224-240 mm; section thickness, 1 mm; matrix, 192 × 192 or 256 × 256) were acquired after administration of a gadolinium-based contrast agent (0.1 mL/kg gadobutrol, Gadovist; Bayer, Toronto, Ontario, Canada).

Low- vs. high-grade in meningioma

MRI studies were performed on one of three 3-T MR scanners (Achieva, Philips, Best, the Netherlands; Ingenia, Philips; Discovery MR750, GE, Milwaukee, WI) with an eight-channel sensitivity-encoding head coil. Achieva, Ingenia and Discovery MR750 were used. The T2 imaging was acquired as follows: TR/TE, 3000-9000/80-120; field of view, 220-240 mm; section thickness, 3-5 mm; and matrix, 192 × 192 or 256 ×256. Three-dimensional T1C (TR/TE, 6.3-8.3/2.9-4; field of view, 224-240 mm; section thickness, 1 mm; matrix, 192 × 192 or 256 × 256) were acquired after administration of a gadolinium-based contrast agent (0.1 mL/kg gadobutrol, Gadovist; Bayer, Toronto, Ontario, Canada).

# Appendix 2. Image processing and radiomics feature extraction

Preprocessing of T2 and T1C images was performed to standardize the data analysis among patients and resampling was performed to a 1 mm. Low-frequency intensity nonuniformity was corrected via N4 bias correction algorithm as implemented in the Advanced Normalization Tools (ANTs) (1). Signal intensity normalization was performed by applying the z-score transformation algorithm, which subtracts the mean intensity value from each voxel intensity and divides the result by the standard deviation of the image. T1C images were coregistered to T2 images by affine transformation with normalized mutual information as a cost function.

For GBM vs. brain metastasis, tumor segmentation was performed on (a) contrast-enhancing (CE) lesions (defined as enhancing areas on T1C) and (b) peritumoral T2 hyperintense (PT) lesions (defined as non-enhancing T2 hyperintense areas excluding CE or necrotic portions). For low- vs. high-grade in meningioma, tumor segmentation was performed on the total tumor portion (including the cystic/necrotic portion) in every tumor section on T1C images. The regions of interests (ROIs) were transferred to T2 images. The segmentation was performed semiautomatically using a software package (3D slicer, version 4.11.0) with an interactive level-set ROI, using edge-based and threshold-based algorithms.

For GBM vs. brain metastasis, 18 first-order features and 61 second order features (including gray level co-occurrence matrix, gray level run-length matrix, gray level size zone matrix, gray level dependence matrix, and neighboring gray tone difference matrix) were extracted using each of the following: CE mask on T1C, CE mask on T2, and PT mask on T2. Fourteen shape features were extracted using each of the CE mask and PT mask. Thus, in total, 265 radiomic features were extracted. For low- vs. high-grade in meningioma, 18 first-order features and 61 second order features were extracted using T2 and T1C sequences. Fourteen shape features were extracted. Thus, in total, 172 radiomic features were extracted. Radiomic features were calculated automatically with an open-source python-based module (PyRadiomics, version 2.0) (Supplementary Table 1) (2). The features were adhered to the standard sets by the Image Biomarker Standardization Initiative (3).

Reference

1.	Avants BB, Tustison N, Song G. Advanced normalization tools (ants). Insight j 2009;2:1-35

2.	van Griethuysen JJM, Fedorov A, Parmar C, Hosny A, Aucoin N, Narayan V et al. Computational radiomics system to decode the radiographic phenotype. Cancer Res 2017;77:e104-e107

3.	Zwanenburg A, Vallières M, Abdalah MA, Aerts H, Andrearczyk V, Apte A et al. The image biomarker standardization initiative: Standardized quantitative radiomics for high-throughput image-based phenotyping. Radiology 2020;295:328-338

Supplementary Table 1. List of extracted radiomics features.

| Feature category | Feature list |
| --- | --- |
| Shape (n=14) | Volume, surface area, surface area to volume ratio, sphericity, maximum 3D diameter, maximum 2D diameter (column), maximum 2D diameter (row), maximum 2D diameter (slice), major axis, minor axis, least axis, elongation, flatness, mesh volume |
| First-order statistics (n=18) | Energy, total energy, entropy, minimum, 10th percentile, 90th percentile, maximum, mean, median, interquartile range, range, mean absolute deviation, robust mean absolute deviation, root mean squared, skewness, kurtosis, variance, uniformity |
| GLCM (n=24) | Autocorrelation, cluster prominence, cluster shade, cluster tendency, contrast, correlation, difference average, difference entropy, difference variance, inverse difference, inverse difference moment, inverse difference moment normalized, inverse difference normalized, informal measure of correlation 1, informal measure of correlation 2, inverse variance, joint average, joint energy, joint entropy, maximal correlation coefficient, maximum probability, sum average, sum entropy, sum of squares |
| GLRLM (n=16) | Short-run emphasis, long-run emphasis, gray level nonuniformity, gray level nonuniformity normalized, run-length nonuniformity, run-length nonuniformity normalized, run percentage, gray level variance, run variance, run entropy, low gray level run emphasis, high gray level run emphasis, short-run low gray level emphasis, short-run high gray level emphasis, long-run low gray level emphasis, long-run high-gray level emphasis |
| GLSZM (n=16) | Small area emphasis, large area emphasis, gray level non-uniformity, gray level non-uniformity normalized, size-zone non-uniformity, size-zone non-uniformity normalized, zone percentage, gray level variance, zone variance, zone entropy, low gray level zone emphasis, high gray level zone emphasis, small area low gray level emphasis, small area high gray level emphasis, large area low gray level emphasis, large area high gray level emphasis |
| GLDM (n=14) | Small dependence emphasis, large dependence emphasis, gray level non-uniformity, dependence non-uniformity, dependence non-uniformity normalized, gray level variance, dependence variance, dependence entropy, low gray level emphasis, high gray level emphasis, small dependence low gray level emphasis, small dependence high gray level emphasis, large dependence low gray level emphasis, large dependence high gray level emphasis, |
| NTGDM (n=5) | Coarseness, complexity, strength, contrast, busyness |

GLCM = gray level co-occurrence matrix, GLDM = gray level dependence matrix, GLRLM = gray level run-length matrix, GLSZM = gray level size zone matrix, NTGDM = neighboring gray tone difference matrix

Supplementary Table 2. Comparison of four CV methods when there is a large discrepancy between mean CV AUC in the training set and AUC in the test set

| Task | CV method | Mean CV AUC (±SD) | RSD in CV | Test AUC (95% CI) |
| --- | --- | --- | --- | --- |
| GBM task (Trial No. 126) | (1) 5-fold CV without a repetition | 0.980 (±0.032) | 7.086 | 0.874 (0.776–0.971) |
|  | (2) CV with 100 repetitions | 0.972 (±0.045) | 8.191 | 0.870 (0.773–0.968) |
|  | (3) Nested CV without a repetition | 0.970 (±0.041) | 8.518 | 0.874 (0.776–0.971) |
|  | (4) Nested CV with 100 repetitions | 0.971 (±0.024) | 9.707 | 0.875 (0.778–0.971) |
| Meningioma task (Trial No. 518) | (1) 5-fold CV without a repetition | 0.840 (±0.023) | 2.784 | 0.650 (0.527–0.774) |
|  | (2) CV with 100 repetitions | 0.835 (±0.069) | 5.297 | 0.648 (0.526–0.775) |
|  | (3) Nested CV without a repetition | 0.827 (±0.046) | 5.611 | 0.646 (0.525–0.773) |
|  | (4) Nested CV with 100 repetitions | 0.826 (±0.038) | 7.779 | 0.646 (0.524–0.771) |

CV, cross validation; AUC, area under the curve; SD, standard deviation; RSD, relative standard deviation; CI, confidence interval; GBM, glioblastoma.

Supplementary Figure 1. Nested cross validation.