# Supporting Information

Figure S1: Cumulative Pore Size Distributions for ACC-ILL sample obtained from fitting the experimental isotherms to the theoretical kernels: H2 (blue), D2 (orange), and N2 (green).

Figure S1 shows the cumulative pore size distributions for the H₂, D₂, and N₂ kernels. All three probes converge to a comparable total pore volume of ~0.59 cm³/g, consistent with previous calculations,8 confirming that the overall micropore volume of ACC-ILL is consistently recovered regardless of the probe molecule used. The N₂-derived curve rises more steeply and plateaus by ~5 nm, while the H₂ and D₂ curves continue to accumulate volume gradually up to 20 nm, reflecting differences in how each kernel distributes pore volume across the ultramicropore-to-mesopore range.

Figure S2: Predicted D2 experimental isotherm from the PSD determined from the N2 isotherm in semilog (left) and linear (right) scales.

Figure S2 shows the D2 isotherm prediction obtained by convolving the N2-derived PSD with the D2 theoretical kernel, following the procedure used in Figure 5. The predicted isotherm consistently underestimates D₂ uptake over the full pressure range, in contrast to the excellent agreement observed with the H₂-derived PSD (Figure 5). This underprediction is consistent with the reduced resolution of the N₂-derived PSD in the sub-1 nm ultramicropore range (Figure 4), where N₂ fails to resolve several pore populations that contribute disproportionately to H₂/D₂ uptake due to strong confinement effects.