## Comparative Radiolabelling of PSMA I&T and Somatostatin Analogues with ¹⁷⁷Lu and ⁹⁰Y: The Impact of SPE Purification

## Martin Kraihammer1, Clemens Decristoforo1, Adriana Knoll1, Taraneh Sadat Zavvar1, Elisabeth von Guggenberg1, Christine Rangger1, Roland Haubner1, Emanuel Sporer1*

## 1 Department of Nuclear Medicine, Medical University Innsbruck, Innsbruck, Austria

## * Corresponding Author: emanuel.sporer@i-med.ac.at

# Supplementary material

Table S1. Results from RP-HPLC analysis of [177Lu]Lu-PSMA I&T. Analytical results and activity amounts used for synthesis are given as mean values ± standard deviation with n=5 batches for each row of data. RCP = radiochemical purity. tR = retention time.

| [177Lu]Lu-PSMA I&T | overall RCP | unbound Lu-177 | impurity A (tR around 6 min) | activity amounts used for synthesis |
| --- | --- | --- | --- | --- |
| with SPE purification | 97.27± 0.38 % | 0.03± 0.04 % | 1.00±0.16 % | 24.2±0.6 GBq |
|  | 97.06± 0.21 % | 0.05± 0.06 % | 1.16±0.06 % | 31.9±0.8 GBq |
|  | 96.73± 0.24 % | 0.02± 0.01 % | 1.18±0.11 % | 38.9±1.8 GBq |
| without SPE purification | 98.29± 0.31 % | 0.10± 0.07 % | 0.20±0.03 % | 24.5±0.4 GBq |
|  | 97.78± 0.29 % | 0.06± 0.08 % | 0.34±0.13 % | 31.9±0.6 GBq |
|  | 97.87± 0.32 % | 0.04± 0.04 % | 0.31±0.04 % | 38.9±1.2 GBq |

Table S2. Radiochemical impurities analysed for [177Lu]Lu-PSMA I&T batches according to iTLC with silica gel as stationary phase and either sodium citrate 0.1M (mobile phase A) or methanol/ammonium acetate 1M 1:1 v/v (mobile phase B) as mobile phase. Results are given as means ± standard deviation (sd) in percent. Number of batches are denoted in brackets (n). p-values denote evaluation of statistical significance between the two production methods for the same starting activity levels. Statistical significance is marked with an asterisk (*).

| [177Lu]Lu-PSMA I&T | Radiochemical impurities in % (mobile phase A) | p- value | Radiochemical impurities in % (mobile phase B) | p- value | activity amounts used for synthesis |
| --- | --- | --- | --- | --- | --- |
| with SPE purification (n =5 for each row) | 0.10±0.16 % | 0.111 | 0.09±0.04 % | 0.071 | 24.2±0.6 GBq |
|  | 0.06±0.04 % | 0.516 | 0.05±0.01 % | 0.183 | 31.9±0.8 GBq |
|  | 0.02±0.004 % | 0.016* | 0.09±0.04 % | 0.341 | 38.9±1.8 GBq |
| without SPE purification (n =5 for each row) | 0.11±0.07 % | 0.111 | 0.05±0.01 % | 0.071 | 24.5±0.4 GBq |
|  | 0.05±0.02 % | 0.516 | 0.08±0.10 % | 0.183 | 31.9±0.6 GBq |
|  | 0.05±0.01 % | 0.016* | 0.06±0.04 % | 0.341 | 38.9±1.2 GBq |

Table S3. Results from RP-HPLC analysis of [177Lu]Lu-DOTA-TATE. Analytical results and activity amounts used for synthesis are given as mean values ± standard deviation with n=5 batches for each row of data. RCP = radiochemical purity. tR = retention time.

| [177Lu]Lu-DOTA-TATE | overall RCP | unbound Lu-177 | main impurity (tR around 16 min) | activity amounts used for synthesis |
| --- | --- | --- | --- | --- |
| with SPE purification | 97.12± 1.10 % | 0.05± 0.01 % | 0.63±0.05 % | 15.0±2.4 GBq |
|  | 96.83± 0.20 % | 0.04± 0.004 % | 0.72±0.05 % | 24.5±0.2 GBq |
|  | 96.32± 0.26 % | 0.05± 0.01 % | 0.69±0.10 % | 32.0±1.0 GBq |
| without SPE purification | 96.57± 0.24 % | 0.46± 0.25 % | 0.66±0.03 % | 15.0±3.6 GBq |
|  | 96.47± 0.55 % | 0.20± 0.06 % | 0.56±0.08 % | 24.4±0.6 GBq |
|  | 95.99± 0.55 % | 0.18± 0.09 % | 0.57±0.06 % | 32.0±0.5 GBq |

Table S4. Radiochemical impurities analysed for [177Lu]Lu-DOTA-TATE batches according to iTLC with silica gel as stationary phase and either sodium citrate 0.1M (mobile phase A) or methanol/ammonium acetate 1M 1:1 v/v (mobile phase B) as mobile phase. Results are given as means ± standard deviation (sd) in percent. Number of batches are denoted in brackets (n). p-values denote evaluation of statistical significance between the two production methods for the same starting activity levels. Statistical significance is marked with an asterisk (*).

| [177Lu]Lu-DOTA-TATE | Radiochemical impurities in % (mobile phase A) | p-value | Radiochemical impurities in % (mobile phase B) | p-value | activity amounts used for synthesis |
| --- | --- | --- | --- | --- | --- |
| with SPE purification (n =5 for each row) | 0.01±0.01 % | 0.008* | 0.13±0.07 % | 0.151 | 15.0±2.4 GBq |
|  | 0.02±0.02 % | 0.008* | 0.12±0.08 % | 0.595 | 24.5±0.2 GBq |
|  | 0.05±0.09 % | 0.135 | 0.16±0.06 % | 0.246 | 32.0±1.0 GBq |
| without SPE purification (n =5 for each row) | 0.44±0.21 % | 0.008* | 0.08±0.03 % | 0.151 | 15.0±3.6 GBq |
|  | 0.18±0.09 % | 0.008* | 0.09±0.06 % | 0.595 | 24.4±0.6 GBq |
|  | 0.11±0.04 % | 0.135 | 0.13±0.02 % | 0.246 | 32.0±0.5 GBq |

Table S5. Results from RP-HPLC analysis of [90Y]Y-DOTA-TOC. Analytical results and activity amounts used for synthesis are given as mean values ± standard deviation with n=5 batches for each row of data. RCP = radiochemical purity. tR = retention time.

| [90Y]Y-DOTA-TOC | overall RCP | unbound Y-90 | activity amounts used for synthesis |
| --- | --- | --- | --- |
| with SPE purification | 96.93±0.77 % | 0.12±0.10 % | 4.1±0.02 GBq |
|  | 96.13±0.42 % | 0.13±0.08 % | 6.8±2.0 GBq |
| without SPE purification | 97.46±1.41 % | 0.25±0.09 % | 4.0±0.8 GBq |
|  | 96.71±1.17 % | 0.32±0.09 % | 7.9±1.0 GBq |

Table S6. Radiochemical impurities analysed for [90Y]Y-DOTA-TOC batches according to iTLC with silica gel as stationary phase and either sodium citrate 0.1M (mobile phase A) or methanol/ammonium acetate 1M 1:1 v/v (mobile phase B) as mobile phase. Results are given as means ± standard deviation (sd) in percent. Number of batches are denoted in brackets (n). p-values denote evaluation of statistical significance between the two production methods for the same starting activity levels. Statistical significance is marked with an asterisk (*).

| [90Y]Y- DOTA-TOC | Radiochemical impurities in % (mobile phase A) | p-value | Radiochemical impurities in % (mobile phase B) | p-value | activity amounts used for synthesis |
| --- | --- | --- | --- | --- | --- |
| with SPE purification (n =5 for each row) | 0.05±0.03 % | 0.008* | 0.19±0.10 % | 0.008* | 4.1±0.02 GBq |
|  | 0.04±0.03 % | 0.024* | 0.18±0.09 % | 0.802 | 6.8±2.0 GBq |
| without SPE purification (n =5 for each row) | 0.24±0.07 % | 0.008* | 0.09±0.02 % | 0.008* | 4.0±0.8 GBq |
|  | 0.21±0.13 % | 0.024* | 0.19±0.12 % | 0.802 | 7.9±1.0 GBq |

Figure S1. Radiochromatogram of a batch of [177Lu]Lu-PSMA I&T with prolonged residence time on the C-18 cartridge during SPE purification. A major increase of the main impurity peak with a retention time of 5.9 minutes amounting to 8.60 % of relaitve area is apparent in the radiotrace of RP-HPLC analysis, therefore not allowing to meet the product release parameter of RCP ≥95.0%. Upper panel: full radiochromatogram without zoom. Lower panel: zoom of signal axis of the radiotrace to give a better view of impurity peaks.