SUPPLEMENTARY MATERIAL

Functional Failure Mode, Effects and Criticality Analysis of
Spacecraft Autonomous Collision Avoidance Systems

Tomás Costa Pinho; Antonio Vinicius Diniz Merladet; Chiara Manfletti; Daniel Rondon Pleffken; Carlos Henrique Netto Lahoz

1 ISAE-SUPAERO, Université de Toulouse, Toulouse, France

2 Chair of Space Mobility and Propulsion, Department of Aerospace and Geodesy, TUM School of Engineering and Design, Technical University of Munich, Munich, Germany

3 School of Graduate Studies, College of Aviation, Embry-Riddle Aeronautical University, Daytona Beach, Florida, USA

4 Aeronautics Institute of Technology (ITA), São José dos Campos, São Paulo, Brazil

Corresponding author: Antonio Vinicius Diniz Merladet; diniz.merladet@tum.de

# S1. Functional decomposition

Table S1. Functional decomposition used to generate the failure modes.

| Functional block | Parent function | Failure modes identified |
| --- | --- | --- |
| Ground-based tracking | Track space objects | Loss of tracking data; Insufficient tracking data; Inaccurate tracking data |
| Ground-based tracking | Provide tracking data to conjunction assessment | No tracking data sent; Tracking data delivered late; Tracking data sent as invalid TDMs; Tracking data sent as mismatched TDMs |
| Conjunction assessment | Compute conjunction data | No conjunction data computed; Incorrect conjunction data computed; Inconsistent conjunction data computed; Incomplete conjunction data computed; Conjunction data computed late |
| Conjunction assessment | Provide conjunction data to ground control | No conjunction data sent; Conjunction data delivered late; Invalid CDM delivered; Mismatched CDM delivered |
| Ground control center | Monitor spacecraft status and radio-frequency interference | Undetected anomaly; False-positive anomaly detection; False-negative anomaly detection; RFI analysis not conducted (when necessary); False-negative RFI analysis; False-positive RFI analysis |
| Ground control center | Decide ground response and collision-avoidance maneuver | Failure to react upon anomaly; No CAM plan generated; Delayed CAM plan generated; Incomplete or incorrect CAM plan generated; CAM plan creates new conjunction risk; CAM plan violates mission objectives |
| Ground control center | Transmit commands to spacecraft | No contingency control order sent; No CAM command transmitted; Command sent too late; Invalid command sent |
| Ground control center | Relay coordination data to spacecraft | Command sent with the wrong target or context; Delayed data relay; Invalid coordination data relayed to spacecraft; Mismatched data relay |
| Ground control center | Relay spacecraft data to coordination services | No data relay; Delayed data relay; Invalid spacecraft data relayed to coordination services; Mismatched data relay |
| Onboard communications | Receive data from ground | No data received; Delayed data received; Invalid ground data received |
| Onboard communications | Transmit data to ground | No data transmitted; Delayed data transmitted; Invalid spacecraft data transmitted to ground |
| Onboard communications | Receive inter-spacecraft coordination data | No data received; Coordination data received late; Invalid inter-spacecraft coordination data received |
| Onboard communications | Transmit inter-spacecraft coordination data | No data transmitted; Coordination data transmitted late; Invalid inter-spacecraft coordination data transmitted |
| Onboard communications | Mitigate radio-frequency interference | Mitigation mechanism (change of frequency) not activated; Mitigation mechanism (change transmission time) not activated; Mitigation mechanism (change antenna pointing/beam steering) not activated; Mitigation introduces new interference; Onboard interference |
| Onboard communications | Operate in the space environment | Antenna degraded; Degraded onboard communication connector or interface |
| Onboard computer | Convert and regulate electrical power | No power output; Incorrect or out-of-range output; Unstable power |
| Onboard computer | Process telecommands | No decryption; Command not decoded; Command decoded incorrectly; Command decoded too late; False-positive validation; False-negative validation; Telecommand lost; Telecommand executed twice; Telecommand sent to the wrong subsystem |
| Onboard computer | Perform autonomous reconfiguration | False-negative autonomous reconfiguration; False-positive autonomous reconfiguration; Inadequate reconfiguration |
| Onboard computer | Format telemetry packets | Packet not generated; Temporally inconsistent packet generated; Invalid packet generated |
| Onboard sensors | Measure navigation and attitude state | Unusable GNSS measurement when service is available; Unusable range or Doppler measurement; Unusable IMU measurement; Unusable star-tracker measurement |
| Onboard sensors | Measure environmental conditions | No measurement; False-negative environmental anomaly; False-positive environmental anomaly; Late measurement |
| Onboard sensors | Measure spacecraft health | No measurement; False-negative health anomaly; False-positive health anomaly; Late measurement |
| Propulsion control unit | Receive and validate the CAM plan | Invalid CAM plan received; False-negative validation; False-positive validation |
| Propulsion control unit | Compute thrust vector and resource allocation | Wrong thrust vector computed; Wrong resource allocation computed |
| Propulsion control unit | Dispatch propulsion commands | No control sent; Invalid control sent |
| Fuel and power resources | Provide fuel and electrical power | Empty fuel tank; Fuel not supplied; No power available or stored; No power supplied; Loss of solar-array power generation, if used; Loss of radioisotope power generation, if used; Loss of fuel-cell power generation, if used |
| Fuel and power resources | Monitor fuel and power state | No resource level reported; Wrong resource level reported |
| Thrusters | Execute thrust commands | Required propulsion unavailable; Incorrect propulsion; Degraded or incomplete propulsion; Unstable propulsion |
| Thrusters | Report propulsion performance | No propulsion-performance report generated; Invalid/incorrect report sent |
| ACAS data processing | Gather external and spacecraft data | Missing spacecraft data; Invalid spacecraft data; Invalid TDM |
| ACAS data processing | Validate ACAS inputs | False-positive input rejection; False-negative validation |
| ACAS data processing | Compute collision probability | Delayed output; Incorrect collision-probability output |
| ACAS decision module | Decide whether a CAM is required | No decision within deadline; False-positive decision; False-negative decision; Repeated decision |
| ACAS maneuver planning | Generate a CAM plan | No CAM plan generated; Delayed CAM plan generated; Incorrect or suboptimal CAM plan generated; CAM plan violates constraints; Invalid CAM plan generated |

# S2. Complete FMECA worksheet

Tables S2a–S2d together constitute the complete FMECA dataset. The failure-mode ID is the common cross-reference key across all four tables. Evidence citation numbers correspond to the numbered reference list in the main manuscript.

Table S2a. Failure definitions, effects, and severity.

| ID | Functional block | Function | Failure mode | Effect | S |
| --- | --- | --- | --- | --- | --- |
| 1 | Ground-based tracking | Track space objects | Loss of tracking data | Tracking data are unavailable to conjunction assessment. ACAS and ground control can therefore remain unaware of the object, increasing collision risk. | 9 |
| 2 | Ground-based tracking | Track space objects | Insufficient tracking data | Inconsistent data regarding space objects lead to downstream degraded conjunction detection. Possible undetected conjunction. | 7 |
| 3 | Ground-based tracking | Track space objects | Inaccurate tracking data | Biased or erroneous tracking states and uncertainties propagate into conjunction assessment, producing false-negative or false-positive alerts. A hazardous encounter can be missed, or an unnecessary CAM can consume resources and create additional operational risk. | 8 |
| 4 | Ground-based tracking | Provide tracking data to conjunction assessment | No tracking data sent | Tracking data are not delivered to conjunction assessment, preventing the encounter from being evaluated. | 9 |
| 5 | Ground-based tracking | Provide tracking data to conjunction assessment | Tracking data delivered late | Reaction based on outdated data. Reduced maneuver effectiveness. | 8 |
| 6 | Ground-based tracking | Provide tracking data to conjunction assessment | Tracking data sent as invalid TDMs | Inaccurate space object data, leading to inaccurate conjunction assessment. Reduced system awareness. | 8 |
| 7 | Ground-based tracking | Provide tracking data to conjunction assessment | Tracking data sent as mismatched TDMs | The TDM is associated with the wrong object, epoch, or measurement context. It can be rejected, delaying orbit determination, or accepted and used to form an incorrect state estimate and conjunction assessment. | 7 |
| 8 | Conjunction assessment | Compute conjunction data | No conjunction data computed | A required conjunction-processing task produces no assessment output. Automatic recovery can restore the service, but if reprocessing exceeds the latest useful time, the encounter remains unassessed and a necessary CAM can be missed. | 10 |
| 9 | Conjunction assessment | Compute conjunction data | Incorrect conjunction data computed | The conjunction state or collision-risk estimate is incorrect, which can create false alarms, missed hazardous conjunctions, unnecessary maneuvers, or ineffective maneuvers. | 9 |
| 10 | Conjunction assessment | Compute conjunction data | Inconsistent conjunction data computed | Contradictory information is received. Inconsistent decisions. Reduced trust. Increased risk. | 8 |
| 11 | Conjunction assessment | Compute conjunction data | Incomplete conjunction data computed | Required conjunction information is missing, preventing a complete risk assessment or safe maneuver decision. | 9 |
| 12 | Conjunction assessment | Compute conjunction data | Conjunction data computed late | The CDM or equivalent assessment is completed too late to support a timely decision, which can close the available maneuver window. | 8 |
| 13 | Conjunction assessment | Provide conjunction data to ground control | No conjunction data sent | A generated conjunction assessment is not delivered to ground control. Unless an independent onboard or alternate service detects the encounter, the risk decision can be delayed or omitted and the conjunction can remain unmitigated. | 10 |
| 14 | Conjunction assessment | Provide conjunction data to ground control | Conjunction data delivered late | The CDM or equivalent assessment reaches ground control too late to support a timely decision, which can close the available maneuver window. | 8 |
| 15 | Conjunction assessment | Provide conjunction data to ground control | Invalid CDM delivered | Recipients cannot interpret data. Increased risk due to less informed decisions. | 8 |
| 16 | Conjunction assessment | Provide conjunction data to ground control | Mismatched CDM delivered | Inconsistent data received by decision systems. May lead to incorrect risk assessment and incoherent decisions. | 7 |
| 17 | Ground control center | Monitor spacecraft status and radio-frequency interference | Undetected anomaly | A spacecraft or communication anomaly is not recognized. The affected function continues in a degraded or unsafe state, preventing timely isolation, fallback, or operator intervention and potentially compromising collision-avoidance capability. | 9 |
| 18 | Ground control center | Monitor spacecraft status and radio-frequency interference | False-positive anomaly detection | Unnecessary mitigation or fallback. Wasted resources. Reduced operator trust. Potential interference with normal operation. | 6 |
| 19 | Ground control center | Monitor spacecraft status and radio-frequency interference | False-negative anomaly detection | An existing anomaly is classified as nominal. The fault remains active without containment or fallback, allowing degradation to propagate into ACAS data, decisions, communications, or maneuver execution. | 9 |
| 20 | Ground control center | Monitor spacecraft status and radio-frequency interference | RFI analysis not conducted (when necessary) | Interference not detected. Spacecraft may cause RFI and violate ITU regulations. Communication may be degraded. | 7 |
| 21 | Ground control center | Monitor spacecraft status and radio-frequency interference | False-negative RFI analysis | RFI continues without mitigation. Communication degrades. Mission risk increases. | 8 |
| 22 | Ground control center | Monitor spacecraft status and radio-frequency interference | False-positive RFI analysis | Valid communication is incorrectly classified as harmful interference and is suppressed or reconfigured unnecessarily. Critical data or commands can be delayed or lost, reducing the available conjunction-response time. | 5 |
| 23 | Ground control center | Decide ground response and collision-avoidance maneuver | Failure to react upon anomaly | A detected anomaly does not trigger the required ground response. The initiating condition persists and can remove a necessary barrier against incorrect ACAS data, decisions, or spacecraft behavior. | 8 |
| 24 | Ground control center | Decide ground response and collision-avoidance maneuver | No CAM plan generated | Spacecraft lacks collision avoidance guidance. Collision risk not mitigated. CAM window may be lost. | 10 |
| 25 | Ground control center | Decide ground response and collision-avoidance maneuver | Delayed CAM plan generated | Maneuver plan generated too late. CAM window may be missed. Increased collision risk. | 8 |
| 26 | Ground control center | Decide ground response and collision-avoidance maneuver | Incomplete or incorrect CAM plan generated | The ground-generated CAM plan is incomplete, internally inconsistent, or operationally incorrect. If rejected, it consumes decision time; if accepted, it can produce an ineffective maneuver, violate constraints, waste resources, or create a secondary conjunction. | 9 |
| 27 | Ground control center | Decide ground response and collision-avoidance maneuver | CAM plan creates new conjunction risk | Executing maneuver causes close approach with previously non-threatening object. Increased collision risk. | 9 |
| 28 | Ground control center | Decide ground response and collision-avoidance maneuver | CAM plan violates mission objectives | Deviation. Interference with mission schedule. Excessive fuel usage. | 9 |
| 29 | Ground control center | Transmit commands to spacecraft | No contingency control order sent | When ground intervention is required to contain an ACAS or spacecraft anomaly, no contingency or override command reaches the spacecraft. The initiating condition remains uncontrolled and collision-avoidance capability can be lost. | 10 |
| 30 | Ground control center | Transmit commands to spacecraft | No CAM command transmitted | A ground-authorized CAM command is not transmitted. The spacecraft may have to rely on onboard autonomy or may miss the maneuver window, degrading collision-avoidance performance. | 8 |
| 31 | Ground control center | Transmit commands to spacecraft | Command sent too late | A ground contingency or CAM command reaches the spacecraft after its latest useful time. The intended mitigation is unavailable, conflicts with a later autonomous state, or forces a less effective corrective response. | 9 |
| 32 | Ground control center | Transmit commands to spacecraft | Invalid command sent | Command logically invalid. Unsafe or unnecessary actions. Increased risk. | 8 |
| 33 | Ground control center | Relay coordination data to spacecraft | Command sent with the wrong target or context | A command is associated with the wrong spacecraft, subsystem, epoch, configuration, or operational context. The intended target remains uncontrolled while another target can execute an inappropriate action, potentially creating an unsafe spacecraft state. | 9 |
| 34 | Ground control center | Relay coordination data to spacecraft | Delayed data relay | ACAS receives outdated data. System falls back to passive execution of ground-generated CAM. | 8 |
| 35 | Ground control center | Relay coordination data to spacecraft | Invalid coordination data relayed to spacecraft | ACAS fails to interpret or accept data. Onboard situational awareness degraded. | 8 |
| 36 | Ground control center | Relay coordination data to spacecraft | Mismatched data relay | ACAS receives inconsistent input data. Onboard decision-making degraded. | 7 |
| 37 | Ground control center | Relay spacecraft data to coordination services | No data relay | STC lacks real-time data on spacecraft. conjunction assessment is inaccurate. | 7 |
| 38 | Ground control center | Relay spacecraft data to coordination services | Delayed data relay | STC receives outdated data. CAM planning confidence is reduced. | 6 |
| 39 | Ground control center | Relay spacecraft data to coordination services | Invalid spacecraft data relayed to coordination services | Relayed spacecraft or coordination data are corrupted, incomplete, stale, or associated with the wrong source. Ground or external assessment can reject the data and lose time or accept them and form an incorrect view of spacecraft state and maneuver intent. | 6 |
| 40 | Ground control center | Relay spacecraft data to coordination services | Mismatched data relay | Conflicting data degrades confidence in onboard state and conjunction assessment. | 5 |
| 41 | Onboard communications | Receive data from ground | No data received | Required ground information is not received by the spacecraft. ACAS loses external conjunction, orbit, coordination, or command data and must rely on a validated onboard degraded mode; otherwise a hazardous conjunction can remain unmitigated. | 10 |
| 42 | Onboard communications | Receive data from ground | Delayed data received | ACAS can make outdated decisions. Maneuver or mitigation may be too late and window missed. | 8 |
| 43 | Onboard communications | Receive data from ground | Invalid ground data received | ACAS may process incorrect data. Risk of invalid maneuver plan or failure to detect conjunction if unreadable. | 9 |
| 44 | Onboard communications | Transmit data to ground | No data transmitted | Ground does not receive maneuver confirmation, health status or telemetry. No ground control decision support. | 9 |
| 45 | Onboard communications | Transmit data to ground | Delayed data transmitted | Ground receives telemetry late. conjunction assessment and response planning are degraded. | 8 |
| 46 | Onboard communications | Transmit data to ground | Invalid spacecraft data transmitted to ground | Ground receives corrupted or non-compliant data and rejects it. This degrades conjunction assessment and ground decision-making. | 8 |
| 47 | Onboard communications | Receive inter-spacecraft coordination data | No data received | The spacecraft does not receive peer coordination or planned-maneuver information. ACAS cannot account for cooperative intent, increasing the risk of uncoordinated maneuvers. | 9 |
| 48 | Onboard communications | Receive inter-spacecraft coordination data | Coordination data received late | Peer coordination or planned-maneuver information arrives after the useful coordination window and may no longer support safe planning. | 8 |
| 49 | Onboard communications | Receive inter-spacecraft coordination data | Invalid inter-spacecraft coordination data received | The received coordination data cannot be validated or are internally inconsistent. ACAS may reject the information or form an incorrect assumption about peer intent. | 8 |
| 50 | Onboard communications | Transmit inter-spacecraft coordination data | No data transmitted | The peer spacecraft does not receive coordination or planned-maneuver information, preventing cooperative deconfliction. | 9 |
| 51 | Onboard communications | Transmit inter-spacecraft coordination data | Coordination data transmitted late | The peer receives coordination or planned-maneuver information too late to incorporate it safely. | 8 |
| 52 | Onboard communications | Transmit inter-spacecraft coordination data | Invalid inter-spacecraft coordination data transmitted | The peer receives invalid or inconsistent coordination data, which can prevent coordination or create an incorrect assumption about the planned maneuver. | 8 |
| 53 | Onboard communications | Mitigate radio-frequency interference | Mitigation mechanism (change of frequency) not activated | Interference remains. System violates constraints. Coordination with STC or other spacecraft is degraded. | 8 |
| 54 | Onboard communications | Mitigate radio-frequency interference | Mitigation mechanism (change transmission time) not activated | Interference remains. System violates constraints. Coordination with STC or other spacecraft is degraded. | 7 |
| 55 | Onboard communications | Mitigate radio-frequency interference | Mitigation mechanism (change antenna pointing/beam steering) not activated | Interference remains. System violates constraints. Coordination with STC or other spacecraft is degraded. | 8 |
| 56 | Onboard communications | Mitigate radio-frequency interference | Mitigation introduces new interference | The selected RFI response shifts interference to another channel, user, or operational period, or disrupts a required ACAS link. Communication performance degrades and regulatory or coordination constraints can be violated. | 8 |
| 57 | Onboard communications | Mitigate radio-frequency interference | Onboard interference | Antenna system performance degraded (reduced signal strength, increased error rates, instability). Mission operations degradation due to unreliable communications. | 7 |
| 58 | Onboard communications | Operate in the space environment | Antenna degraded | Antenna system performance degraded or lost (reduced signal strength, increased error rates, reduced range). Mission operations and avoidance degradation. | 9 |
| 59 | Onboard communications | Operate in the space environment | Degraded onboard communication connector or interface | Signal delivered to the antenna is attenuated through RF lines. Intermittent link and degraded communication with ground. Increased collision risk. | 9 |
| 60 | Onboard computer | Convert and regulate electrical power | No power output | All subsystems powered by converter lose power entirely, leading to a total shutdown of operations. | 10 |
| 61 | Onboard computer | Convert and regulate electrical power | Incorrect or out-of-range output | Subsystems powered receive incorrect voltage, too high may lead to damage and too low to resets and operation failure. | 7 |
| 62 | Onboard computer | Convert and regulate electrical power | Unstable power | Excessive ripple or oscillations may cause sensitive electronics to reset or malfunction, in addition to coupling with signal lines causing communication errors (bit flips). | 6 |
| 63 | Onboard computer | Process telecommands | No decryption | Command is not executed, leading to loss of control actions. | 9 |
| 64 | Onboard computer | Process telecommands | Command not decoded | The telecommand cannot be decoded and is not executed. Retransmission consumes response time, and a time-critical ground control or CAM action can be lost, forcing reliance on autonomous fallback. | 9 |
| 65 | Onboard computer | Process telecommands | Command decoded incorrectly | The telecommand is decoded into the wrong instruction, parameter, address, or execution time. The spacecraft can enter an unsafe configuration, execute an incorrect maneuver-related action, or fail to perform the intended control, increasing collision and mission risk. | 9 |
| 66 | Onboard computer | Process telecommands | Command decoded too late | The telecommand is decoded after its execution deadline. The intended control action is no longer valid or effective, and delayed execution can conflict with the current autonomous state or require corrective action. | 8 |
| 67 | Onboard computer | Process telecommands | False-positive validation | A valid telecommand is rejected. The intended control or safety action is withheld, retransmission consumes time, and a time-critical intervention can become unavailable. | 9 |
| 68 | Onboard computer | Process telecommands | False-negative validation | An invalid or unauthorized telecommand is accepted. The spacecraft can execute an unintended action, change a safety-relevant configuration, or interfere with ACAS and propulsion behavior. | 8 |
| 69 | Onboard computer | Process telecommands | Telecommand lost | A successfully decoded telecommand is lost during onboard routing. The intended subsystem receives no action, ground intervention is delayed, and the spacecraft can be forced to rely on autonomous fallback. | 7 |
| 70 | Onboard computer | Process telecommands | Telecommand executed twice | A non-idempotent telecommand is executed more than once. Duplicate switching, configuration, or actuation can consume resources, violate sequencing assumptions, or create an unsafe state. | 7 |
| 71 | Onboard computer | Process telecommands | Telecommand sent to the wrong subsystem | The telecommand is routed to the wrong subsystem or function. The intended action is not performed, while the unintended recipient can change state or actuate incorrectly, causing delay, resource use, or an unsafe configuration. | 8 |
| 72 | Onboard computer | Perform autonomous reconfiguration | False-negative autonomous reconfiguration | A required reconfiguration is not initiated, allowing the initiating fault or degraded condition to persist and potentially compromise collision-avoidance capability. | 9 |
| 73 | Onboard computer | Perform autonomous reconfiguration | False-positive autonomous reconfiguration | Autonomous reconfiguration is initiated although the active configuration remains healthy. Useful capability can be removed, redundancy can be unnecessarily consumed, and mission or collision-avoidance performance can be degraded. | 7 |
| 74 | Onboard computer | Perform autonomous reconfiguration | Inadequate reconfiguration | The selected reconfiguration does not isolate the fault or restore the required function. The original degradation remains, and the transition can consume redundancy, resources, or time while leaving the spacecraft in an unsafe state. | 9 |
| 75 | Onboard computer | Format telemetry packets | Packet not generated | The onboard computer does not generate required telemetry packets. Ground control loses timely evidence of spacecraft state, ACAS status, resource margins, or maneuver execution, reducing its ability to diagnose and intervene. | 8 |
| 76 | Onboard computer | Format telemetry packets | Temporally inconsistent packet generated | Telemetry arrives with the wrong timestamp or is compromised by delays. Event correlation and analysis compromised, or operational decisions degraded. | 7 |
| 77 | Onboard computer | Format telemetry packets | Invalid packet generated | The telemetry packet has an invalid structure, identifier, length, integrity field, or metadata. Ground systems can reject it and lose information or misassociate it with the wrong source or parameter, degrading operational decisions. | 8 |
| 78 | Onboard sensors | Measure navigation and attitude state | Unusable GNSS measurement when service is available | ACAS plans maneuver based on invalid positional data. Maneuver is misaimed, decreasing safety margin and increasing collision risk. Additionally, velocity measurements (TDCP) degrade severely. | 9 |
| 79 | Onboard sensors | Measure navigation and attitude state | Unusable range or Doppler measurement | Velocity and distance/position measurements are degraded (velocity measurement is more robust). Orbit and status awareness is degraded and thus so the maneuver planning. Increased collision risk. | 8 |
| 80 | Onboard sensors | Measure navigation and attitude state | Unusable IMU measurement | Attitude measurements degrade and propagation drifts fast (trajectory and velocity measurements, coupled, can deviate). Degraded situational awareness and maneuver planning. Collision risk increases. | 9 |
| 81 | Onboard sensors | Measure navigation and attitude state | Unusable star-tracker measurement | Maneuver is planned based on incorrect attitude. Maneuver is misaimed, safety margins decrease and collision risk increases. | 9 |
| 82 | Onboard sensors | Measure environmental conditions | No measurement | Critical exposure (radiation spikes, thermal extremes) goes unnoticed, leading to data corruption and component degradation/failure. Degraded situational awareness and decisions. | 8 |
| 83 | Onboard sensors | Measure environmental conditions | False-negative environmental anomaly | Critical exposure goes unnoticed, leading to data corruption and component degradation/failure. Degraded situational awareness and decisions. | 9 |
| 84 | Onboard sensors | Measure environmental conditions | False-positive environmental anomaly | A nominal environmental condition is reported as hazardous. Protective modes, duty-cycle reductions, or sensor inhibitions can be activated unnecessarily, consuming resources and reducing mission or ACAS availability; repeated false alarms can also impair operator response. | 6 |
| 85 | Onboard sensors | Measure environmental conditions | Late measurement | Environmental information becomes available after its latest useful time, delaying protective action and degrading the validity of state estimation or autonomous decisions. | 8 |
| 86 | Onboard sensors | Measure spacecraft health | No measurement | Safety-relevant health parameters are unavailable. Fault detection and isolation are delayed or prevented, so degraded equipment can continue to supply invalid data or lose collision-avoidance capability without timely recovery. | 9 |
| 87 | Onboard sensors | Measure spacecraft health | False-negative health anomaly | A real health anomaly is not declared. The affected component continues to operate or supply data without containment, causing progressive degradation, invalid state information, reduced margins, or loss of a required ACAS function. | 9 |
| 88 | Onboard sensors | Measure spacecraft health | False-positive health anomaly | A healthy component is reported as failed. Unnecessary isolation, reconfiguration, or protective action can remove useful capability, consume redundancy and resources, and reduce ACAS availability; repeated false alarms can impair operator response. | 6 |
| 89 | Onboard sensors | Measure spacecraft health | Late measurement | Spacecraft-health information becomes available after its latest useful time, delaying fault isolation or protective action and degrading autonomous decisions. | 8 |
| 90 | Propulsion control unit | Receive and validate the CAM plan | Invalid CAM plan received | The system receives a plan that is not fit for use. Causing, at least (if rejected) delays due to rerequests and possible missed windows, increasing collision risk. If undetected, the invalid plan propagates downstream as an accepted input. | 8 |
| 91 | Propulsion control unit | Receive and validate the CAM plan | False-negative validation | An unsafe, infeasible, stale, or unnecessary CAM plan is accepted by the propulsion-control function. Execution can violate trajectory, resource, attitude, or mission constraints and can increase collision or secondary-conjunction risk. | 9 |
| 92 | Propulsion control unit | Receive and validate the CAM plan | False-positive validation | Valid CAM is discarded, causing delays due to rerequests and possible missed windows, increasing collision risk. | 7 |
| 93 | Propulsion control unit | Compute thrust vector and resource allocation | Wrong thrust vector computed | The CAM plan is converted into an incorrect thrust direction, magnitude, timing, or duration. The maneuver can be ineffective or destabilizing, deviate from the planned trajectory, waste resources, and leave or increase the conjunction risk. | 9 |
| 94 | Propulsion control unit | Compute thrust vector and resource allocation | Wrong resource allocation computed | Fuel, power, thermal, or actuator demand is allocated incorrectly. The maneuver can fail to start, terminate early, overdrive equipment, exhaust reserves, or damage the propulsion chain, producing an ineffective or unsafe trajectory change. | 8 |
| 95 | Propulsion control unit | Dispatch propulsion commands | No control sent | If a command does not leave the CU, the propulsive subsystem does not act. Missed windows and increased collision risk. | 9 |
| 96 | Propulsion control unit | Dispatch propulsion commands | Invalid control sent | The dispatched propulsion command is corrupted, incomplete, late, or addressed incorrectly. The commanded burn can be absent, partial, mistimed, or applied to the wrong actuator, increasing collision and spacecraft risk. | 9 |
| 97 | Fuel and power resources | Provide fuel and electrical power | Empty fuel tank | No propellant available. No thrust and thus no maneuverability. Increased collision risk and mission end. | 10 |
| 98 | Fuel and power resources | Provide fuel and electrical power | Fuel not supplied | Thrusters receive no propellant despite it being available. No ignition or thrust. No maneuverability, leading to missed CAMs and increased collision risk and possible end of mission. | 9 |
| 99 | Fuel and power resources | Provide fuel and electrical power | No power available or stored | Relating to the inability to store power, leading the propulsive system to experience electrical power loss. Loss of heaters, actuators, ignition. No thrust and thus no maneuverability. Increased collision risk and possible end of mission | 10 |
| 100 | Fuel and power resources | Provide fuel and electrical power | No power supplied | Power is available, but electricity does not reach the propulsive system. No ignition, valve actuation, or thrust. No maneuverability, increased collision risk and possible end of mission. | 9 |
| 101 | Fuel and power resources | Provide fuel and electrical power | Loss of solar-array power generation, if used | No electricity is generated by the (primary) power source. Powered subsystems run on reserves, until depletion, degrading performance. Once depleted, no maneuverability is possible, increasing collision risk and leading to mission end. | 9 |
| 102 | Fuel and power resources | Provide fuel and electrical power | Loss of radioisotope power generation, if used | No electricity is generated by the (primary) power source. Powered subsystems run on reserves, until depletion, degrading performance. Once depleted, no maneuverability is possible, increasing collision risk and leading to mission end. Radioactivity adds disposal complications. | 9 |
| 103 | Fuel and power resources | Provide fuel and electrical power | Loss of fuel-cell power generation, if used | No electricity is generated by the (primary) power source. Powered subsystems run on reserves, until depletion, degrading performance. Once depleted, no maneuverability is possible, increasing collision risk and leading to mission end. For hydrogen fuel cells, hydrogen can be dangerously released, creating additional safety hazards. | 9 |
| 104 | Fuel and power resources | Monitor fuel and power state | No resource level reported | The propulsion-control function receives no current fuel or power state. It can reject a feasible maneuver, authorize an infeasible one, or consume reserves below the required contingency margin. | 8 |
| 105 | Fuel and power resources | Monitor fuel and power state | Wrong resource level reported | The propulsion control unit uses an incorrect fuel or power state, which can reject a feasible CAM, authorize an infeasible plan, exhaust reserves, or shorten mission life. | 9 |
| 106 | Thrusters | Execute thrust commands | Required propulsion unavailable | The propulsion authority required for the planned CAM is unavailable. The spacecraft may become underactuated or unable to achieve the commanded trajectory change, leaving the conjunction unmitigated and potentially causing loss of mission. | 10 |
| 107 | Thrusters | Execute thrust commands | Incorrect propulsion | Thrust is produced with an incorrect direction, magnitude, timing, or duration. The achieved trajectory deviates from the CAM plan, can require corrective burns, and can leave the original conjunction unmitigated or create a secondary conjunction. | 9 |
| 108 | Thrusters | Execute thrust commands | Degraded or incomplete propulsion | Thrusters fire, but producing less thrust than commanded/the burn ends prematurely. CAMs are only partially executed, leaving residual collision risks. Can cause mission degradation and resource waste for corrective maneuvers. | 8 |
| 109 | Thrusters | Execute thrust commands | Unstable propulsion | Thrust oscillates during the burn, causing attitude disturbance, maneuver error, additional resource use, and increased structural loading. Collision-avoidance effectiveness can be reduced and a corrective maneuver may be required. | 9 |
| 110 | Thrusters | Report propulsion performance | No propulsion-performance report generated | System receives no feedback on thruster performance. CAM real-time execution verification is degraded, leading to possible undetected underperformance. Fault mitigation is degraded and, depending on fault severity, can lead to mission degradation. Increased collision risk. | 7 |
| 111 | Thrusters | Report propulsion performance | Invalid/incorrect report sent | If invalid and thus rejected, failure effects are similar to “No report sent”. If incorrect but plausible, and thus accepted, can lead to over or undercompensation, reducing CAM effectiveness, resource waste, missed faults or incorrect diagnosis. Increased collision risk and degrading the mission. | 8 |
| 112 | ACAS data processing | Gather external and spacecraft data | Missing spacecraft data | Collision probability computation is degraded or not possible, depending on extent of missing data. Either inflated uncertainty, degrading CAM or unsafe CAM, increasing collision risk or no CAM at all. | 9 |
| 113 | ACAS data processing | Gather external and spacecraft data | Invalid spacecraft data | Spacecraft data are corrupted, stale, incomplete, or associated with the wrong unit, time, source, or reference frame. Rejection reduces data availability and time margin; acceptance can corrupt collision-risk estimation and maneuver decisions. | 7 |
| 114 | ACAS data processing | Gather external and spacecraft data | Invalid TDM | If invalid tracking data are accepted, collision-risk estimates can produce false positives or false negatives, resulting in unsafe or unnecessary maneuvers. Rejection can delay the assessment beyond the useful decision window. | 7 |
| 115 | ACAS data processing | Validate ACAS inputs | False-positive input rejection | Valid input data are rejected. ACAS therefore operates with reduced information or waits for retransmission, which can produce an unnecessarily conservative decision or delay the response until a safe CAM is no longer feasible. | 8 |
| 116 | ACAS data processing | Validate ACAS inputs | False-negative validation | Invalid or inconsistent data are accepted as usable, contaminating collision-probability estimation and downstream decisions. The result can be a missed hazardous conjunction, an unnecessary CAM, or an unsafe maneuver plan. | 9 |
| 117 | ACAS data processing | Compute collision probability | Delayed output | The collision probabilities are produced after deadline, reducing the time left for decision and planning. Can lead to push CAM past ISO standard intervals, missed CAMs or deteriorate decision and planning, shrinking safety margins and increasing collision risk. | 10 |
| 118 | ACAS data processing | Compute collision probability | Incorrect collision-probability output | The collision-probability estimate is incorrect. Underestimation can leave a hazardous conjunction unmitigated, while overestimation can trigger an unnecessary maneuver, consume resources, or create a secondary conjunction. | 9 |
| 119 | ACAS decision module | Decide whether a CAM is required | No decision within deadline | A go/no-go decision is not produced within the applicable decision window. CAM planning cannot proceed in time, and a necessary maneuver can become infeasible or be missed, increasing collision risk. | 10 |
| 120 | ACAS decision module | Decide whether a CAM is required | False-positive decision | ACAS authorizes an unnecessary CAM. This leads to resource waste, reducing spacecraft lifetime and leading to unnecessary orbital changes (which can even lead to new conjunctions). Also undermines confidence in autonomy. | 8 |
| 121 | ACAS decision module | Decide whether a CAM is required | False-negative decision | ACAS does not authorize a necessary CAM. The conjunction remains unmitigated, creating the possibility of collision, loss of mission, and loss of confidence in autonomy. | 10 |
| 122 | ACAS decision module | Decide whether a CAM is required | Repeated decision | More than one decision is issued for the same conjunction. Can lead to double or conflicting CAMs, shrinking time margins and unnecessary waste of other resources. Also decreases the trust in autonomy. | 6 |
| 123 | ACAS maneuver planning | Generate a CAM plan | No CAM plan generated | No plan is generated for the respective conjunction, so CAM cannot be executed. Time margins shrink or are violated. CAM is missed or executed too late/unsafely. Increases collision risk, decreases trust in autonomy and degrades mission. | 10 |
| 124 | ACAS maneuver planning | Generate a CAM plan | Delayed CAM plan generated | A CAM plan is produced, but after the respective window, shortening (or eliminating) time for subsequent tasks. Maneuver may be unsafe or missed, producing similar effects as the aforementioned failure mode of “no plan generated”. | 10 |
| 125 | ACAS maneuver planning | Generate a CAM plan | Incorrect or suboptimal CAM plan generated | A plan is produced but it is valid for execution but does not achieve the necessary conjunction mitigation or uses more resources than necessary. Can result in high collision probability remaining, unsafe CAMs or avoidable deviations, resource waste and decreased trust in autonomy. | 9 |
| 126 | ACAS maneuver planning | Generate a CAM plan | CAM plan violates constraints | CAM plan generated violates one or more constraints, not being feasible (safely, at least). Can be rejected downstream, resulting in delays and possible missed CAMs. If executed, it can be unsafe or mission-degrading. | 8 |
| 127 | ACAS maneuver planning | Generate a CAM plan | Invalid CAM plan generated | The generated CAM plan cannot be parsed, validated, or executed because its format, version, units, timing, or internal content are invalid. Rejection consumes planning time; acceptance can produce an unsafe or ineffective maneuver. | 10 |

Table S2b. Causes and relative occurrence.

| ID | Cause | O |
| --- | --- | --- |
| 1 | Power loss. Hardware failure. Damaged data channel. Deployment/pointing faults. | 3 |
| 2 | Insufficient coverage. Observation frequency. Visibility gaps. Geometric limitations. Sensor maintenance. Weather. | 5 |
| 3 | Data fusion errors. Sensor noise. Calibration drift. Atmospheric interference. Sensor aging. Thermal effects. | 4 |
| 4 | Network or server failure. Queue overflow. Software crash. Security issues (tampering). | 3 |
| 5 | Insufficient observation frequency. Network congestion. Processing bottlenecks. Retransmission delays. | 5 |
| 6 | Data corruption in transmission. Network issues. Hardware malfunction. non-compliant CCSDS. Software bug. Message tampering. | 3 |
| 7 | Clock desynchronization. Software bug. Data fusion bug. | 3 |
| 8 | Software failure (bug/numerical instability). Invalid or loss of inputs. Internal processing fault. | 3 |
| 9 | Invalid or inconsistent inputs, propagation-model error, reference-frame or time mismatch, covariance error, numerical instability, or software defect. | 4 |
| 10 | Inconsistent TDMs. Data fusion error. Synchronization error. Incapability to detect ground-based tracking or ground control inconsistencies. | 4 |
| 11 | Missing tracking inputs, incomplete object or covariance data, processing interruption, or message-generation and export defects. | 4 |
| 12 | Processing overload. Inefficient algorithm. Poor priority handling. Old or limited hardware. | 4 |
| 13 | Network or server failure between conjunction assessment and ground control. Software crash or scheduling conflict. Data pipeline blockage (queue overflow). Security issues (tampering). Failure in conjunction data generation. | 3 |
| 14 | Poor prioritization of messages. Network congestion (server or buffer bottlenecks). Retransmission delays. | 4 |
| 15 | Data corruption during transmission (packet loss or bit errors). Non-compliant CCSDS formatting (protocol mismatch, outdated implementation). Software bugs. Message tampering. | 3 |
| 16 | Time desynchronization. Data fusion errors. | 3 |
| 17 | Monitoring gaps. Insufficient coverage. | 3 |
| 18 | Too sensitive thresholds. Sensor noise. Insufficient filtering. Incorrect detection logic. Corrupt input. | 4 |
| 19 | Too strict thresholds. Incomplete detection logic. Corrupt input. | 3 |
| 20 | Low frequency of scheduled runs. No spectral capability. Analysis not triggered at critical moments. | 4 |
| 21 | Limited sensor coverage. Miscalibration. Poorly configured thresholds. | 3 |
| 22 | Miscalibration. Logic bug. Valid communication mistaken for interference. | 4 |
| 23 | Operator inaction. Alert fatigue. Missing procedures. Fault in response logic. | 4 |
| 24 | Algorithm error. Software crash. Incomplete input data. Human inaction. | 3 |
| 25 | Inefficient algorithm. Processing overload. Suboptimal task prioritization. Slow hardware. | 4 |
| 26 | Incomplete input data. Outdated ephemerides. Missing mission constraints. Incorrect planning algorithm. | 3 |
| 27 | Small screening volume. Outdated ephemerides. Maneuver intention not shared. Poor communication. | 3 |
| 28 | Incorrect, incomplete or old constraints that do not reflect reality. Configuration errors (on constraint priority or safety margins). Coverage gaps. | 3 |
| 29 | Operator omission, command-generation or release failure, unavailable communication service, antenna misalignment, interference, software failure, or power loss. | 2 |
| 30 | Operator omission, command-generation or release failure, communication-service unavailability, or command inhibition caused by configuration or authorization logic. | 3 |
| 31 | Human input error. EMI. Transmission errors. Format violation. | 3 |
| 32 | Time desynchronization. Conflicting inputs. Fusion error. Human error. | 3 |
| 33 | Antenna misaligned. Interference. Transmission failure. Software crash. | 2 |
| 34 | Retransmissions. Communication congestion. Scheduling delay. | 4 |
| 35 | EMI/RFI. Transmission errors. Format violation. | 3 |
| 36 | Time desynchronization. Conflicting CDMs. Fusion error. | 3 |
| 37 | Network or server failure. Software crash. Queue overflow. Security issues. | 3 |
| 38 | Poor prioritization of messages. Network congestion. Retransmission delays. | 3 |
| 39 | Data corruption. Protocol mismatch. Software bugs. Message tampering. | 3 |
| 40 | Time desynchronization. Erroneous onboard state. Fusion errors. | 2 |
| 41 | Frequency drift or offset (oscillator aging or miscalibration). Misalignment. RFI mitigation problems. Signal loss or attenuation (incorrect gain, aging or hardware fault). | 3 |
| 42 | RFI mitigation problems. Onboard processing delay (congestion). Re-requests. | 4 |
| 43 | Frequency or timing offset, radio-frequency interference or electromagnetic interference, protocol mismatch, outdated interface implementation, or radiation-induced upset in onboard electronics. | 3 |
| 44 | Frequency drift or offset. Misalignment. RFI mitigation problems. Signal loss or attenuation (incorrect gain, aging or hardware fault). | 3 |
| 45 | RFI mitigation problems. On-board processing delay (congestion). Re-requests. | 4 |
| 46 | Upstream data error, protocol mismatch, outdated interface implementation, radio-frequency or electromagnetic interference, or radiation-induced upset in onboard electronics. | 3 |
| 47 | Frequency mismatch. Peer spacecraft failure or noncompliance with rules of the road. Policy restrictions (crewed). RFI mitigation problems. Signal loss or attenuation. | 3 |
| 48 | Peer transmitted coordination data too late. RFI mitigation problems. Re-requests (signal problems). Onboard processing delay (high latency). | 3 |
| 49 | Unsuccessful RFI mitigation. Outdated or mismatched protocol. Encoding failure. | 3 |
| 50 | Frequency drift or offset. Misalignment. Unsuccessful RFI mitigation. Signal loss. Policy restriction (crewed). | 3 |
| 51 | RFI mitigation problems. Re-requests (signal problems). Onboard processing delay. | 4 |
| 52 | Unsuccessful RFI mitigation. Outdated or mismatched protocol. Encoding failure. | 3 |
| 53 | Logic not triggered. No frequency for reallocation or (re)tuning. Actuator failure. | 3 |
| 54 | Logic not triggered. Unable to store data (buffer overflow). No available schedules (delay can cause missed maneuver opportunity). | 3 |
| 55 | Logic not triggered. RFI direction unknown. Beam steering actuator failure or misalignment. No available angle change for antenna. | 3 |
| 56 | Frequency, schedule or beam overlap due to outdated data. Poor ITU rules integration. Static fallback table. Inter-entity communication errors. Sensor or actuator drift and miscalibration. | 3 |
| 57 | Electronics generate EMI. Poor isolation or shielding. Degradation of these materials. Antennas too close or poor design. | 3 |
| 58 | Thermal stress (fluctuations). Surface wear or erosion. Space radiation. LEO atmospheric drag. Mechanical stress and aging. | 3 |
| 59 | Manufacturing or assembly defects. Thermal stress (fluctuations). Surface wear or erosion. Space radiation. LEO atmospheric drag. Mechanical stress and aging. | 3 |
| 60 | Component (MOSFETs and capacitors) failure. Circuit issues (unreasonable design, incorrect component connection and component quality). External factors (unstable power supply voltage, sudden changes in load, high ambient temperature, excessive dust, mechanical stress, and EMI) | 5 |
| 61 | Component degradation or aging. Faulty feedback control (component damage or miscalibration). | 5 |
| 62 | EMI. Component (LC filter) fault or degradation. Load characteristics, parasitic components and interaction (design componentmismatch). | 4 |
| 63 | Wrong/expired key. Data corruption in transmission. Cryptographic module fault (component failure, software bug/crash). | 3 |
| 64 | Synchronization or channel-decoding errors caused by noise, interference, an upstream decryption failure, or a received-error rate beyond the selected coding capability. | 4 |
| 65 | Channel introduces errors superior to code’s correction capability. Decoder failure (misinterpretation due to algorithm bug or limitation). Format mismatch. | 3 |
| 66 | High processing load or queue congestion on onboard computer. Low priority scheduling for critical commands. Time synchronization drift. Retransmission delays due to decryption or decoding failure. | 4 |
| 67 | Validation software bug. Time synchronization drift. Protocol mismatch. Overly strict validation rules. Upstream failure (decoding). | 3 |
| 68 | Validation software bug. Overly permissive validation rules or incomplete validation process. | 3 |
| 69 | Undetected upstream failure. Routing failure due to software bug or memory fault (buffer overflow). Hardware failure (damage to cable). | 4 |
| 70 | Ground operator error. ACK error (no ACK or delayed to after retransmission of command). Routing software bug. | 3 |
| 71 | Routing software bug. Configuration (human) error (wrong address). Cable crosstalk. Packet address corruption (EMI). | 3 |
| 72 | Upstream failure (sensor fault/coverage gaps). Configuration errors (ambiguous thresholds or incorrect/incomplete trigger logic). Memory (lookup table) corruption. Hardware (lines) failure. | 4 |
| 73 | Upstream failure (sensor fault/noise). Configuration errors (incorrect/oversensitive threshold or incorrect trigger logic). Software bug. | 3 |
| 74 | Configuration error (incorrect/incomplete mapping between failure and action, outdated parameters). Memory corruption. Incomplete reconfiguration (faulty redundant system or damaged connections). | 3 |
| 75 | Upstream failure (source/sensors). Software crash/deadlock. A watchdog, if present, may also interrupt the task due to delay. Hardware failure. | 4 |
| 76 | Delay due to resource exhaustion (insufficient memory, processing power or bandwidth) or watchdog resets. Wrong time stamp due to clock drift or time desynchronization. | 3 |
| 77 | Format errors (incorrect packet length, APID) due to memory corruption, software bug, radiation-induced bitflips or outdated software version. | 3 |
| 78 | Upstream failure (GNSS satellites clock error/drift and orbit error/variation). Atmospheric delays. Jamming, spoofing, multipath interference. (Receiver) clock drift. Improper antenna positioning and phase center error. (Receiver) noise or hardware problems (overheating, aging, loose connections). | 4 |
| 79 | Aliasing (sampling error) and Doppler ambiguity. Clock errors (drift) and oscillator instabilities (short and long term). Quantization error. Signal errors (low SNR, multipath and NLOS, atmospheric effects, interference/jamming or communication chain fault). Algorithm and processing limitations (emphasis on error models). | 4 |
| 80 | Bias/offset, (high-frequency) noise, scale factors, misalignment, hardware faults (stuck axis) emphasized by vibrations, mechanical and thermal stress, radiation and power supply. Saturated outputs during slews. | 4 |
| 81 | Sensor misalignment. Sensor noise, dark current (overtemperature, insufficient exposure time). Straylight saturation, atmospheric turbulence/jitter, environmental exposure. Lens aberrations. Algorithms (and processing) limitations. | 4 |
| 82 | Duty cycle too low. Sensor off due to self-test or watchdog fault, or protective measurements. Upstream (DC/DC, fuse, link) failure. | 4 |
| 83 | Saturation that looks normal. Low sensitivity (aging, aggressive filtering). Blind spots (poor orientation/- placement). Overly strict thresholds. Calibration drift. Alarm fatigue. | 3 |
| 84 | Overly sensitive thresholds coupled with noise or EMI. Algorithmic or logic bugs. Miscalibration. | 4 |
| 85 | Insufficient sampling rate, long filtering or debounce delay, data-conversion latency, processing overload, clock error, power instability, or sensor response limitations. | 4 |
| 86 | Duty cycle too low. Sensor off due to self-test or watchdog fault, or protective measurements. Upstream (DC/DC, fuse, link) failure. | 4 |
| 87 | Saturation that looks normal. Blind spots. Overly strict thresholds coupled with low sensitivity (aging, aggressive filtering). Calibration drift. Alarm fatigue. | 3 |
| 88 | Overly sensitive or poor thresholds and hysteresis coupled with noise, EMI and cross-talk. Algorithmic or logic bugs. Miscalibration. Sensor fault mimics component failure. | 4 |
| 89 | Insufficient sampling rate, long filtering or debounce delay, data-conversion latency, processing overload, clock error, power instability, or sensor response limitations. | 4 |
| 90 | Spoofing. Transmission corruption (EMI). Protocol mismatch. Clock drift. Upstream communication failure (corruption). | 4 |
| 91 | Overly permissive thresholds, incomplete scope (like no unit labelling) or bugs on validation. Emphasized by upstream failure on reception of invalid plan. | 3 |
| 92 | Overly strict thresholds. Protocol mismatch. Clock drift. Emphasized by upstream failure of delay on communication. | 3 |
| 93 | Modelling errors (unit mismatch, physical phenomena inaccuracies or unaccounted downstream variations) and environmental uncertainties. Numerical instability or bugs. Missing/inaccurate (upstream failure of sensors) or invalid (corruption) inputs. Clock drift. | 4 |
| 94 | Upstream failure for inputs (corruption, sensor failure) or downstream failure (wrong fuel level, fuel contamination/degradation), emphasized by model inaccuracies (unit mismatch, unaccounted thruster faults). Bugs. | 4 |
| 95 | Software crashes or bugs, emphasized by upstream data corruption. Memory corruption or memory overflow. Hardware or power failure (connector fault). Watchdog failure. | 3 |
| 96 | Data and memory corruption (EMI, bit flips). Internal congestion, coupled with poor prioritization. Protocol mismatch. High latency, emphasized by ageing or degradation. Clock drift. | 3 |
| 97 | Propellant leakage, venting, contamination, thermal-management failure, or incorrect resource accounting and reserve management. | 2 |
| 98 | Blockage of frozen propellant on line, due to heater failure, heater power loss or poor design (fuel lines too close to cold pressurization lines). Valve stuck closed (actuator failure, jam). Valve actuator power loss. | 3 |
| 99 | Battery electrical issues (overcharging/overdischarging, SC, plasma discharges, circulating currents), degradation (radiation, temperature) and poor resource management. PPU failure. No power generation. | 2 |
| 100 | Connector breakage/damage due to mechanical stress (launch vibrations), thermal stress or poor assembly. Protection faults (fuse, overtemperature, SEU) and component failure (switch, DC/DC) leading to PPU shutdown. | 3 |
| 101 | Deployment failure (tribology, shield interference). Mispointing (upstream sensor failure or actuator failure). Hot spots, cracks, component defects and electrical failures such as arcing, ESD, OC or SC. Emphasized by the failure of the thrusters due to physical, mechanical and thermal effects from exhaust gases. | 3 |
| 102 | Thermoelectric-converter degradation, electrical interconnect failure, thermal-path degradation, structural damage, or progressive loss of conversion efficiency. | 1 |
| 103 | Fuel depletion (leak) or feeding issues (blockage). Humidity exposure or fuel contamination. Catalyst degradation due to operational stress. Membrane failure (thinning or cracking) or due to cycling. Cooling system failure. Component failure (high pressure component cracking) emphasized by redox and thermal cycling. | 2 |
| 104 | Sensor failure (open circuit due to vibrations or degradation, noise due to EMI or poor shielding). Telemetry corruption or packet loss, software bug. Communication link failure or upstream power failure. | 4 |
| 105 | Sensor drift, bias, saturation, calibration error, acquisition-electronics failure, telemetry corruption, estimation-model error, or time misalignment. | 4 |
| 106 | Loss of required propellant feed, ignition, valve actuation, electrical power, thermal conditioning, or a sufficient set of healthy thrusters; upstream resource failure; or hardware damage and blockage. | 3 |
| 107 | Thruster misalignment (mount shift assembly, thermal bowing). Wrong thruster fired (configuration errors, or misinterpretation due to corruption). Asymmetrical propellant delivery. Upstream failure in sensors or CU (sensor drift, contamination from exhaust). Component (nozzle) damage or degradation (thermal expansion). | 3 |
| 108 | Partial blockage in propellant lines due to ice or contamination. Valve actuators fault (degraded response, only opening partially). Fuel leakage or insufficient upstream pressure. Thruster wear or damage (erosion, nozzle cracks). Premature cutoff due to faulty sensor signal (false alarm), component fault or software fault. | 4 |
| 109 | Gas entrainment, pressure-control fluctuation, propellant slosh, valve or regulator cycling, combustion or discharge instability, component wear, control-loop instability, or coupling with mission-specific structural modes. | 3 |
| 110 | Sensor failure (damage, aging, data acquisition electronics fault). Telemetry packet software fault. Communication link failure (open circuit, EMI). Upstream failure on onboard computer or power supply. | 4 |
| 111 | Sensor bias or drift, stuck value/saturation (thermal/radiation aging). Unmitigated data corruption (insufficient CRC/ECC). Configuration errors (units, signs, routing). Protocol mismatch. Clock desynchronization. | 4 |
| 112 | Sensor upstream failure (power loss, saturation, connector faults). Time desynchronization. Communication failure (EMI, bus congestion/overflow, software crash). Operational failure (duty cycle or routing misconfiguration). | 5 |
| 113 | Format or unit mismatch. Corruption (SEU/EMI). Upstream sensor failure (saturation, drift). Configuration error (routing, software version). Clock desynchronization. | 5 |
| 114 | Message-format or unit mismatch, CCSDS nonconformance, transmission corruption, timestamp error, upstream generation error, spoofing, or incomplete reception. | 4 |
| 115 | Validation thresholds are too restrictive; reference-frame, unit, identity, or time alignment is incorrect; data-fusion configuration is inconsistent; or valid data fall outside an inadequately tested acceptance envelope. | 2 |
| 116 | Validation thresholds are too permissive, required checks are bypassed, units or reference metadata are missing, duplicate or stale messages are accepted, or configuration and covariance logic are incorrect. | 3 |
| 117 | Heavy computation (model too detailed). Operational delay due to re-requests (data rejection) or waiting on data. Software bugs or queue. Poor task prioritization (CPU on other tasks) or resource allocation. Clock drift. Configuration errors (such as on deadlines). | 5 |
| 118 | Invalid or inconsistent inputs, incorrect assumptions, reference-frame or time mismatch, covariance error, unsuitable probability model, numerical instability, or implementation defect. | 3 |
| 119 | Missing or late inputs, unresolved rule or threshold conflicts, inadequate task prioritization, unbounded waiting for confirmation, software failure, clock error, communication delay or corruption, or hardware degradation. | 4 |
| 120 | Incorrect, incomplete, outdated, or noisy input data; an upstream computation error; overly conservative decision thresholds; or timing and synchronization errors. | 3 |
| 121 | Incorrect or incomplete inputs and upstream module computation failure (underestimation). Decision logic faults (such as overly loose thresholds or coverage gaps). Timing issues (delays, clock drift). | 3 |
| 122 | Duplicate or late inputs retrigger decision. Threshold oscillation (decision jitter at boundary). No commitment on decision. | 4 |
| 123 | Unfeasible (because overconstrained) CAM. Hardware failure or resource failure (such as OOM). Upstream failure on inputs (hardware or communications failure). Software failure (algorithm crash, nonconvergence). | 4 |
| 124 | Delay on inputs due to upstream fault. Heavy computation (for available time) exacerbated by unsuitable hardware and poor resource/priority allocation. Clock drift. Re-planning close to the deadline. | 4 |
| 125 | Planning limitations under time pressure. Parameter and thresholds mistuning (not reflecting conditions). Old, incorrect or misaligned inputs. Incomplete or incorrect model (such as improper balance between conjunction mitigation and resource usage). Suboptimal human override. | 2 |
| 126 | Incorrect, incomplete or old constraints that do not reflect reality. Configuration errors (on constraint priority or safety margins). Coverage gaps. | 3 |
| 127 | Memory corruption (EMI). Version/format misalignment or exporting faults in scheme. Inconsistent or no unit-tagging. Software faults (logic configuration error, bug). Validation steps failed or disabled (time-pressure). | 4 |

Table S2c. Baseline detection provisions and detectability.

| ID | Baseline detection provisions | D |
| --- | --- | --- |
| 1 | Signal timeout. Timestamp (continuity) verification. | 2 |
| 2 | CDM generation rate analysis. Observation gap analysis. Parameter monitoring. | 4 |
| 3 | Cross-check with external sources. Fusion residuals. Plausibility check. | 4 |
| 4 | ACK checks. Missing data alarms. | 2 |
| 5 | Timestamp verification. Latency monitoring and watchdogs on tasks. | 4 |
| 6 | CRC/checksum check. CCSDS format validation. Plausibility check. | 5 |
| 7 | Timestamp check. Plausibility and consistency check across systems. | 4 |
| 8 | Processing-job completion watchdog, expected-output count for known screening requests, task-health monitoring, and automatic reprocessing alarm. | 1 |
| 9 | Apply range, consistency, covariance, and physical-plausibility checks. Compare with an independent implementation or external assessment where available. | 4 |
| 10 | Output discrepancy. Plausibility and timestamp check. | 3 |
| 11 | Check mandatory fields, object identity, covariance completeness, timestamps, units, and message conformance before use. | 4 |
| 12 | Processing time monitoring. Output latency checks. Deadline alarms. | 5 |
| 13 | ACK checks. Timeout alarms. | 4 |
| 14 | Delivery timestamp check. Performance (buffer, latency) monitoring. | 5 |
| 15 | CRC/checksum checks. CCSDS format validation. Plausibility check. | 4 |
| 16 | Monitor CDM divergence (data consistency). Timestamp validation. | 4 |
| 17 | Watchdog timers. Heartbeat signal monitoring. | 5 |
| 18 | Alarm filter verification. False-positive log analysis. | 4 |
| 19 | Inconsistency tracking. Plausibility checks. | 5 |
| 20 | RFI analysis logs. RFI interference complaints. | 4 |
| 21 | Interference reports from recipients. RFI analysis logs. | 5 |
| 22 | Cross-check with external source. Discrepancy between expected and actual impact. | 4 |
| 23 | Alarm acknowledgement monitoring. Operator interaction logs. | 4 |
| 24 | Input verification. Plan generation monitoring. Timeout alert. | 3 |
| 25 | Plan generation latency monitoring. Timeout alarms. Output timestamp verification. | 4 |
| 26 | Plausibility check. Simulation of maneuver results. CDM context review. | 4 |
| 27 | Post-maneuver risk evaluation. | 3 |
| 28 | Simulate maneuver beforehand. Check constraints after generation. | 3 |
| 29 | Command-release log, transmission status, acknowledgment timeout, execution confirmation, and comparison between required and dispatched contingency actions. | 3 |
| 30 | Command-release log, transmission status, acknowledgment timeout, and comparison between planned and dispatched command sets. | 4 |
| 31 | Format compliance check. Checksum and plausibility check. | 3 |
| 32 | Timestamp validation. Cross-checks (including ACAS). | 4 |
| 33 | ACK. No reception alarm. Signal monitoring. | 3 |
| 34 | Latency monitoring. Timestamp verification. | 4 |
| 35 | ACAS rejection (NACK). Format integrity check. | 4 |
| 36 | Timestamp and plausibility check. | 4 |
| 37 | ACK. No reception alarm. System parameter monitoring. | 3 |
| 38 | Timestamp analysis. Latency monitors. | 4 |
| 39 | Format compliance checks. CRC/checksum verification. | 4 |
| 40 | Timestamp validation. Consistency cross-verification. | 4 |
| 41 | Watchdog. Timeout. Heartbeat. Channel/link status monitoring. | 3 |
| 42 | Timestamp verification. Delta-t monitoring. Channel/link status monitoring. | 4 |
| 43 | Check message format, integrity, sequence, timestamp, and physical plausibility. Monitor link quality and radio-frequency interference. | 4 |
| 44 | Watchdog. Timeout. Heartbeat. Channel/link status monitoring. | 3 |
| 45 | Timestamp verification. Delta-t monitoring. Channel/link status monitoring. | 4 |
| 46 | Check message format, integrity, sequence, timestamp, and physical plausibility. Monitor link quality and radio-frequency interference. | 4 |
| 47 | Known object in CDM but silent. coordination data timeout. | 4 |
| 48 | coordination data timeout. coordination data timestamp check. | 4 |
| 49 | coordination data fails format, checksum or plausibility check. | 4 |
| 50 | Watchdog timeout. No ACK from other spacecraft. | 3 |
| 51 | coordination data timestamp and transmission delay log. Peer response inconsistent. | 4 |
| 52 | coordination data rejected by peer (if feedback exists). | 4 |
| 53 | RFI and frequency measurement remains. Peer RFI reports. | 4 |
| 54 | RFI measurement remains. Peer RFI reports. | 4 |
| 55 | RFI and angle measurement remains. Peer feedback. | 4 |
| 56 | Peer feedback. post-mitigation monitoring with alarm. | 4 |
| 57 | System self-monitoring (high communication error rate or drop in signal strength). Feedback from STC. | 4 |
| 58 | System self-monitoring (high communication error rate or drop in signal strength). Feedback from STC. | 4 |
| 59 | System self-monitoring (abnormal VSWR values). Feedback from STC. | 4 |
| 60 | Voltage monitoring and alarms. | 2 |
| 61 | Voltage monitoring and alarms. | 2 |
| 62 | Ripple monitoring. System diagnostics (sensor noise and communication errors). | 5 |
| 63 | Timeout on module output. HMAC check. Cryptographic module error alarms. | 4 |
| 64 | Timeout on expected effect from ground. Decoder error alarm. | 4 |
| 65 | Unexpected spacecraft behavior observed in telemetry. Parameter consistency (range) check on onboard computer. | 5 |
| 66 | Delayed evidence of command execution in telemetry. Latency monitoring. task deadlines. | 4 |
| 67 | Command rejection log (history analysis for patterns). No effect from telemetry. Possible logs for each step (decryption, validation) for problem isolation. | 5 |
| 68 | Unexpected behaviour in telemetry. Command log analysis. | 5 |
| 69 | Timeout on expected effect from ground (telemetry). Possible command delivery ACK. | 4 |
| 70 | Unexpected behavior in telemetry. Incorporation of execution log comparison for duplicates. | 4 |
| 71 | Unexpected effect in telemetry or missing effect from intended subsystem. Routing log review. Incorporation of delivery acknowledgment check. | 5 |
| 72 | Monitor expected reconfiguration effects, watchdog status, command and state transitions, and consistency between health indicators and the active configuration. | 4 |
| 73 | Telemetry analysis after fallback activation. Fallback logs for pattern analysis. | 4 |
| 74 | Fault indicators/alarms remain after reconfiguration. Telemetry data analysis. | 4 |
| 75 | Missing sequence numbers (on application process ID of telemetry) by ground. Real-time monitoring of telemetry data streams. | 4 |
| 76 | Operational parameters (latency) monitoring. Timestamp verification. | 3 |
| 77 | Ground validation detects format inconsistencies (CRC discrepancy). | 3 |
| 78 | Signal strength monitoring and DOP analysis. Consistency (with other sensors and filter residuals) and plausibility (range) checks. Timestamp verification for clockdrift. | 4 |
| 79 | Cross-check with other sensors and residuals. Ionosphere/troposphere monitoring with space weather alarms. Timestamp verification. Communication chain health monitoring (cable temperature, etc.). | 4 |
| 80 | Apply range, rate, and consistency checks; compare with star-tracker, GNSS, or other navigation estimates; monitor filter residuals, temperature, vibration, saturation, and sensor health flags. | 4 |
| 81 | Real time consistency (with other sensors and filter residuals) and plausibility (range) checks with blinding/saturation alarms. Self-test (starcount, pixels, noise, delays). | 3 |
| 82 | Monitor data freshness and sensor health, and compare measurements with independent onboard indicators and external space-weather or environmental information where available. | 4 |
| 83 | Cross-check independent environmental sensors and compare with external space-weather or environmental information where available. | 5 |
| 84 | Cross-check between independent sensors or modalities. False-positive logs for trend analysis. | 4 |
| 85 | Monitor sample age and end-to-end latency through acquisition, conversion, filtering, and delivery. Check sensor health and time synchronization. | 5 |
| 86 | Implement indirect symptoms logic (coupled physical quantities). Watchdog on data stream. | 4 |
| 87 | Cross-check between independent sensors and indirect symptoms. Periodically apply stimulus to sensors to check response. Signal behaviour monitoring: rate of change (to avoid plausible but stuck values) and compare against model predictions. | 5 |
| 88 | Cross-check between independent sensors or modalities. False-positive logs for trend analysis. | 4 |
| 89 | Monitor sample age and end-to-end latency through acquisition, conversion, filtering, and delivery. Compare with independent subsystem indicators and expected behavior. | 5 |
| 90 | Strong format and integrity checks (error detection codes). Timestamp verification. Plausibility verification (cross-check with CDM or ephemerides). | 3 |
| 91 | Complete plausibility check (format, integrity, range, timestamp, mission constraints). Consider pre-execution simulation. | 4 |
| 92 | Logs of discards for posterior analysis. Timestamp verification. If applicable, cross-check with redundancy on validation with voting. | 5 |
| 93 | Vector plausibility checks. Timestamp verification. Consider performing pre-execution simulation or redundant computation (if enough resources). | 4 |
| 94 | Cross-check (available vs. requested) and plausibility (estimations). Pre-execution simulation (depending on resources, different model complexity can also be available). | 4 |
| 95 | Timeout alarms and watchdog monitoring. Health monitoring. Telemetry confirmation of no state change. | 3 |
| 96 | Watchdog on maximum delay. EDC (CRC or checksums). Timestamp verification. | 4 |
| 97 | Leak tests. Low level or pressure readings. Expected consumption vs. tank level change verifications. | 3 |
| 98 | No change in reading (pressure, fuel level) after command. | 3 |
| 99 | Voltage (and other parameters) monitoring at source with undervoltage alarms. | 3 |
| 100 | Voltage (and other parameters) monitoring at subsystem entry. Timeout on actuator response. | 3 |
| 101 | Telemetry on electrical data, deployment status (consider camera) and sun alignment (temperature) with alarms on discrepancy from expected value. | 2 |
| 102 | Alarms on discrepancy from expected value in electrical output, internal parameters (internal resistance, temperature) with discriminatory logic for cause (electric, thermal). | 2 |
| 103 | Telemetry on electric output, as well as thermal data, tank pressure and water balance with alarms on discrepancy from expected value. Computer models fed by sensor networks for predictive detection and maintenance. | 2 |
| 104 | Missing telemetry (fields). Timeout on sensor data. cross-check with other sensors (to detect stale data). | 3 |
| 105 | Compare independent measurements with consumption and state-estimation models. Apply range, rate, conservation, and temporal-consistency checks. | 4 |
| 106 | Compare commanded and achieved acceleration or velocity change; monitor ignition, pressure, flow, temperature, valve state, and power; use start-timeout alarms and pre-burn readiness checks. | 3 |
| 107 | Discrepancy in expected values and real values from navigation sensors and thruster sensors. | 4 |
| 108 | Discrepancy between commanded thrust and thruster performance. Discrepancy between commands and actual navigation data. Health monitoring alarms. | 4 |
| 109 | Fluctuations, oscillations or cycling in readings like velocity/acceleration, pressure in lines/chamber, temperature in thruster. GNC alarms. Perform short pre-maneuver verifications if resources allow. | 4 |
| 110 | Timeout alarm on expected report. Health monitoring (heartbeat loss). Cross-check with other sources (IMUs detected thrust). | 3 |
| 111 | Integrity and conformance checks (CRC/ECC, format validation). Plausibility checks (range, rate of change). Timestamp validation. Cross-check with other sources. Redundant sensors disagreement. | 4 |
| 112 | Completeness checks on all required parameters. Timeout monitoring against expected rates. Timestamp verification (incl. last timestamp alarm on staleness). Sensors/onboard computer health flags or watchdogs. | 2 |
| 113 | ACAS verification on inputs for format (CRC, schema, units), timestamp (continuity), value plausibility (range, rate). Additionally, cross-check with other sensors can be performed. | 2 |
| 114 | ACAS verification on inputs for format (CCSDS compliance, units), integrity (CRC, checksum), authenticity, timestamp (continuity), value plausibility (range, rate). Consider applying source quality scoring for sources with poor history. | 2 |
| 115 | Record rejection reason codes, monitor acceptance and rejection rates, compare with independent sources, and review rejected inputs through shadow processing where resources permit. | 5 |
| 116 | Monitor acceptance and rejection statistics, audit validation logs, check data provenance and age, and compare accepted data against independent sources or prediction residuals. | 6 |
| 117 | Timestamp verification. Parameter monitoring (such as latency) per stage (such as queue, CPU) with prediction on deadline alarms. | 2 |
| 118 | Apply output plausibility and sensitivity checks, compare with an independent or simplified implementation, and review marginal or safety-significant cases when time permits. | 6 |
| 119 | Use a deadline timer on the decision output, verify input arrival and data age, and monitor task execution, processor health, and time synchronization. | 3 |
| 120 | Cross-check (against past decisions, simpler models, other sources) and plausibility check. Verify input completeness and quality. | 6 |
| 121 | Cross-check (against past decisions, simpler models, other sources) and plausibility check. Verify input completeness and quality | 6 |
| 122 | Event identification with same-event and postwindow flags. Monitor decision variation. | 4 |
| 123 | Perform early feasibility check. Deadline timer on output with countdown on ISO standard. Input arrival verification and respective monitoring. Watchdog and parameter/ health monitoring. | 2 |
| 124 | Verification of plan timestamp against ISOstandardized deadline. Input arrival verification and respective monitoring. Watchdog and parameter/ health monitoring on subsystem. | 3 |
| 125 | CAM logging with post-execution reviews. Input arrival verification and respective monitoring. Plan quality check (such as comparing against an estimation or perform pre-simulation) with quality score and respective alarm. | 5 |
| 126 | Plan verification against constraints. Alarms for constraint change after planning. Perform dryruns. | 6 |
| 127 | Format and version verification. Integrity verification (such as checksum). Consistency verification (on units, time, etc.). | 2 |

Table S2d. Baseline RPN, recommended risk controls, and evidence citations.

| ID | RPN | Recommended risk-control measures | Evidence citations |
| --- | --- | --- | --- |
| 1 | 54 | Design for redundant trackers and data channels (with different modalities and common cause avoidance) with voting or switchover logic, through inter-agency coordination. Routine maintenance. | [12, 18, 32, 33, 34] |
| 2 | 140 | Parameter tuning. Object tracking priority. Inter-agency cooperation (in downstream conjunction assessment). Additional sources and redundancy. | [12, 18, 32, 33, 34] |
| 3 | 128 | Calibration routines. Robust sensor fusion with weighting/voting. Shielding. Drift estimators. | [12, 18, 32, 33, 34] |
| 4 | 54 | Provide redundant data paths and automatic retry or service-restart logic. Use the last valid tracking state only within a defined age limit and with increasing uncertainty. | [12, 18, 32, 33, 34] |
| 5 | 160 | Monitor end-to-end latency, prioritize time-critical data, and provide redundant network paths. Define a latest-useful time and a conservative response when it is exceeded. | [12, 18, 32, 33, 34] |
| 6 | 120 | Validate the CCSDS message structure, integrity, units, identity, and timestamp. Request retransmission when possible and use the last valid state only within a defined age limit. Apply authentication where required by the operational concept. | [12, 18, 32, 33, 34] |
| 7 | 84 | Use traceable time synchronization, such as PTP or NTP on ground networks where appropriate, together with cross-source consistency checks and robust data-fusion logic. | [12, 18, 32, 33, 34] |
| 8 | 30 | Backup processing unit. Software testing for critical scenarios. System parameter monitoring (RAM, response time). | [13, 14, 17, 21, 22, 23, 34] |
| 9 | 144 | Validate inputs and reference data, test the propagation and probability models across representative geometries and uncertainty conditions, and use independent cross-checks for safety-significant cases. | [13, 14, 17, 21, 22, 23, 34] |
| 10 | 96 | Synchronization protocols. Redundancy of inputs with robust data fusion methods and testing. Source grading and prioritization. | [13, 14, 17, 21, 22, 23, 34] |
| 11 | 144 | Request or obtain an independent data source when time permits. Use a documented conservative degraded mode only when its applicability and uncertainty bounds have been validated. | [13, 14, 17, 21, 22, 23, 34] |
| 12 | 160 | Use deadline-aware scheduling, adequate computing resources, performance monitoring, and optimized algorithms. Prioritize conjunctions according to documented operational criteria. | [13, 14, 17, 21, 22, 23, 34] |
| 13 | 120 | Redundant communication paths. Automatic retries. Latency monitoring. Fallback to last valid CDMs or ephemerides. | [13, 14, 17, 21, 22, 23, 34] |
| 14 | 160 | Monitor delivery latency, prioritize time-critical conjunction data, and provide redundant communication paths. Define a latest-useful delivery time and an escalation response. | [13, 14, 17, 21, 22, 23, 34] |
| 15 | 96 | Error-correction protocols (automatic retransmissions) and error correction codes. Fallback to last valid data. Encryption and authentication. Protocol version alignment. | [13, 14, 17, 21, 22, 23, 34] |
| 16 | 84 | Use traceable time synchronization, consistent object identifiers and reference frames, and cross-source checks. Reject or quarantine mismatched messages and request clarification or retransmission. | [13, 14, 17, 21, 22, 23, 34] |
| 17 | 135 | Periodic reviews. Operator training. Coverage and thresholds tuning. | [14, 21, 22, 23] |
| 18 | 96 | Periodic reviews and audits for thresholds. Operator training. Detection logic tuning. Redundant sources and input validation. Alarm hysteresis. | [14, 21, 22, 23] |
| 19 | 135 | Periodic reviews and audits. Operator training. Improved failure simulation during testing. | [14, 21, 22, 23] |
| 20 | 112 | Event-triggered logic. Regular routine runs. Sensor redundancy. | [14, 21, 22, 23] |
| 21 | 120 | Expanded spectral monitoring. Threshold calibration. External feedback. | [14, 21, 22, 23] |
| 22 | 80 | Periodic testing. Detection logic tuning. Version control updates. | [14, 21, 22, 23] |
| 23 | 128 | Automated pre-defined fallback. Operator training. Clear procedures. Decisionsupport design. | [6, 13, 14, 21, 27] |
| 24 | 90 | Validate input. Fallback maneuvers. Operator training. Algorithm robustness testing. | [6, 13, 14, 21, 27] |
| 25 | 128 | Optimize the planning algorithm, allocate sufficient computing resources, use deadline-aware task scheduling, and maintain validated maneuver templates for time-critical cases. | [6, 13, 14, 21, 27] |
| 26 | 108 | Input and constraint validation. Algorithm and model testing. Operator validation routines. | [6, 13, 14, 21, 27] |
| 27 | 81 | Screening volume tuning. Update ephemerides. Share intentions in time. | [6, 13, 14, 21, 27] |
| 28 | 81 | Include constraints in planning logic. Operator review. Extensive planner testing. Schedule maneuvers to occur before mission assignment. | [6, 13, 14, 21, 27] |
| 29 | 60 | Use independently routed command paths where justified, explicit transmission acknowledgment and execution confirmation, preauthorized contingency commands, bounded retry logic, and periodic operator drills. | [14, 20, 21] |
| 30 | 96 | Use command-release checklists, independent confirmation of dispatch, bounded retries through alternate links, early transmission when operationally appropriate, and an agreed autonomous fallback. | [14, 20, 21] |
| 31 | 81 | RFI mitigation. error-correction protocols. Redundant data transmission. Operational training. | [14, 20, 21] |
| 32 | 96 | Integrate ACAS cross-verification. Command plausibility check. | [14, 20, 21] |
| 33 | 54 | Redundant paths. Alignment routines. Fallback to last known state. | [14, 15, 20, 21, 34] |
| 34 | 128 | Transmission monitoring. Channel redundancy. Early dispatches. | [14, 15, 20, 21, 34] |
| 35 | 96 | RFI mitigation. Error correction codes. Format compliance verification. | [14, 15, 20, 21, 34] |
| 36 | 84 | Cross-verification with onboard state. Source prioritization. | [14, 15, 20, 21, 34] |
| 37 | 63 | Redundant communication paths. Automatic retries. Fallback to last valid state. | [14, 15, 20, 21, 34] |
| 38 | 72 | Prioritize critical data. Redundant communication paths. Fallback to estimated state. | [14, 15, 20, 21, 34] |
| 39 | 72 | Error-correction protocols. Fallback to last valid telemetry. Encryption. | [14, 15, 20, 21, 34] |
| 40 | 40 | Synchronization protocols. Fallback to ground-based tracking estimations. Source prioritization. | [14, 15, 20, 21, 34] |
| 41 | 90 | Redundancy on paths (with common cause avoidance and switchover logic). Channel/link status monitoring. Alignment and health checks. Inter-entity communication (anticipate RFI). | [15, 20, 35, 36] |
| 42 | 128 | Historical telemetry review. Prioritization logic. Channel/link status monitoring. Inter-entity communication. | [15, 20, 35, 36] |
| 43 | 108 | Use suitable error-detection and correction coding, interface conformance testing, software configuration control, and redundant transmission of safety-critical data. | [15, 20, 35, 36] |
| 44 | 81 | Redundancy on paths. Channel/link status monitoring. Alignment and health checks. Inter-entity communication (anticipate RFI). | [15, 20, 35, 36] |
| 45 | 128 | Historical telemetry review. Prioritization logic. Channel/link status monitoring. Inter-entity communication. | [15, 20, 35, 36] |
| 46 | 96 | Use suitable error-detection and correction coding, interface conformance testing, software configuration control, and redundant transmission of safety-critical data. | [15, 20, 35, 36] |
| 47 | 108 | Inter-entity communication (for pre-coordination and fallback channel). STC fallback and passive avoidance. Reception redundancy. Frequency alignment verification. | [15, 20, 35, 36] |
| 48 | 96 | Prioritization logic for coordination data. Fallback to STC. Inter-entity communication and time synchronization. | [15, 20, 35, 36] |
| 49 | 96 | Use defined coordination-message schemas, integrity and authenticity checks, timestamp and identity validation, software configuration control, and a conservative fallback when peer intent is uncertain. | [15, 20, 35, 36] |
| 50 | 81 | Retry logic. Health checks. Redundancy. Inter-entity communication. STC fallback. | [15, 20, 35, 36] |
| 51 | 128 | Priority logic. Channel status monitoring. Fallback to STC. Inter-entity coordination protocol (time synchronization). | [15, 20, 35, 36] |
| 52 | 96 | Use defined coordination-message schemas, integrity and authenticity checks, timestamp and identity validation, software configuration control, and redundant transmission where justified. | [15, 20, 35, 36] |
| 53 | 96 | RFI mitigation method redundancy with fallback logic and context-based selection. Pre-loaded frequency table of alternatives. Actuator self-test. | [15, 20, 35, 36] |
| 54 | 84 | RFI mitigation method redundancy with fallback logic and context-based selection. STC fallback. | [15, 20, 35, 36] |
| 55 | 96 | RFI mitigation method redundancy with fallback logic and context-based selection. Actuator self-test. STC fallback. | [15, 20, 35, 36] |
| 56 | 96 | RFI mitigation method redundancy with fallback logic and context-based selection. Updated frequency maps and ITU compliance checks. Regular calibration routines. | [15, 20, 35, 36] |
| 57 | 84 | Filtering. Improved shielding and separation. Routine tests. | [15, 20, 35, 36] |
| 58 | 108 | Redundancy and graceful degradation. Advanced space-qualified materials with protective coatings. Periodic performance monitoring. | [15, 20, 35, 36] |
| 59 | 108 | Redundancy and graceful degradation. Advanced space-qualified materials with protective coatings. Periodic performance monitoring. Thorough mechanical inspection. | [15, 20, 35, 36] |
| 60 | 100 | Component derating. Redundancy (backup power converter). Power supply filter. Heat dissipation methods. EMI and dust shielding. Design testing and optimization. | [11, 20, 35, 36, 37] |
| 61 | 70 | Over/under-voltage protection circuits. High-quality components. | [11, 20, 35, 36, 37] |
| 62 | 120 | EMI shielding and EMI filters. Component optimization and robust test of design. Heat dissipation methods. Power-up delay. | [11, 20, 35, 36, 37] |
| 63 | 108 | Key mismatch protocols. Redundancy on cryptographic hardware. Fallback to (unencrypted) retransmission. | [11, 20, 35, 36, 37] |
| 64 | 144 | Use link-appropriate error-detection and correction coding, adequate link margin, robust synchronization and decoding logic, and redundant receiver paths where justified. | [11, 20, 35, 36, 37] |
| 65 | 135 | Use realistic decoder and interface testing, configuration control, link-appropriate error-detection and correction coding, and an independent confirmation step for hazardous commands when operationally feasible. | [11, 20, 35, 36, 37] |
| 66 | 128 | Define telecommand-processing deadlines, monitor execution latency, prioritize safety-critical commands, and provide a safe response when a command cannot be completed within its useful time. | [11, 20, 35, 36, 37] |
| 67 | 135 | Tune validation criteria using representative data, maintain configuration control, cross-check command context and source, and permit an authorized override or recovery path when operationally appropriate. | [11, 20, 35, 36, 37] |
| 68 | 120 | Enforce source authentication, integrity, context, range, and authorization checks; test permissive-threshold cases; isolate rejected commands; and require an independent confirmation step for hazardous commands when time and architecture permit. | [11, 20, 35, 36, 37] |
| 69 | 112 | Use end-to-end command identifiers and delivery acknowledgments, protected routing tables, buffer and task watchdogs, redundant paths where justified, and bounded retransmission from a known command state. | [11, 20, 35, 36, 37] |
| 70 | 84 | Use unique command identifiers, duplicate suppression, idempotent command design where feasible, execution-state checks, and robust acknowledgment logic before retransmission. | [11, 20, 35, 36, 37] |
| 71 | 120 | Protect and verify destination identifiers, configuration and routing tables; perform end-to-end address and context checks; isolate communication paths where common-cause errors are credible; and confirm the intended effect for hazardous commands. | [11, 20, 35, 36, 37] |
| 72 | 144 | Validate trigger logic across representative faults and timing conditions, provide independent health indications where justified, and define a conservative safe configuration when trigger evidence is conflicting or incomplete. | [11, 20, 35, 36, 37] |
| 73 | 84 | Use multi-parameter confirmation, persistence or hysteresis, validated trigger logic, and explicit reconfiguration cost criteria. Human confirmation is appropriate only when the communication path, workload, and response time are adequate. | [11, 20, 35, 36, 37] |
| 74 | 108 | Validate fault-to-configuration mappings, verify the health and availability of the target configuration before transition, monitor whether the initiating fault clears, and define a safe response when reconfiguration is unsuccessful. | [11, 20, 35, 36, 37] |
| 75 | 128 | Maintain a bounded-latency minimum telemetry set in degraded modes, monitor packet-generation tasks and sequence continuity, prioritize safety-relevant telemetry, and provide restart or alternate reporting paths. | [11, 20, 35, 36, 37] |
| 76 | 63 | Time synchronization protocols with periodic checks. Priority scheduling for time-critical telemetry tasks. Realistic testing for adequacy of hardware. | [11, 20, 35, 36, 37] |
| 77 | 72 | Memory protection (ECC). Adherence to protocol, version control and software update validation. Onboard packet conformance check before transmission. | [11, 20, 35, 36, 37] |
| 78 | 144 | Use multi-constellation and, where applicable, multi-frequency GNSS with integrity monitoring. Cross-check against independent navigation sensors and fall back to a validated fused-state estimate when GNSS is unavailable or rejected. | [12, 32, 33] |
| 79 | 128 | High bit ADC (increase quantization levels). Multiple PRF techniques. anti-jamming, space filtering, robust atmospheric corrections (dual frequency and mathematical models) and adequate ground positioning (no reflective surfaces). Robust communication path design (redundancy, quality). Data fusion fallback. | [12, 32, 33] |
| 80 | 144 | Use in-flight bias and scale-factor estimation, thermal control, adequate dynamic range, radiation and vibration protection, and cross-checking with independent navigation sensors. Provide fault isolation and redundant IMUs where justified. | [12, 32, 33] |
| 81 | 108 | Periodic alignment calibration. Advanced sensor technology and algorithmic optimization. Shielding and sensor cooling. When blinded, fallback on remaining fused sensors. Exposure control (from detection and speed). | [12, 32, 33] |
| 82 | 128 | Use an adaptive sampling strategy, independent power and data paths, and appropriately diverse environmental sensors, such as radiation, temperature, and charging monitors. Provide shielding and conservative degraded-mode logic. | [12, 37, 39] |
| 83 | 135 | Perform periodic calibration and threshold reviews using representative environments. Use overlapping coverage and conservative conflict handling when independent sources disagree. | [12, 37, 39] |
| 84 | 96 | Use independent or diverse sensor confirmation, persistence and hysteresis, contextual thresholds, calibration and self-test, and operator confirmation only for costly protective actions when time and workload permit. | [12, 37, 39] |
| 85 | 160 | Define maximum data age, allocate time-critical processing priority, adapt sampling when conditions change, and bound filtering delay. Use independent environmental indications where justified. | [12, 37, 39] |
| 86 | 144 | Provide diverse or independently powered health sensing where justified, data-stream watchdogs, adaptive but bounded sampling, indirect symptom checks, and a predefined degraded mode when required health evidence is unavailable. | [12, 37, 39] |
| 87 | 135 | Use overlapping and diverse health indicators, periodic calibration and stimulus testing, stuck-value and rate-of-change checks, model residuals, and conservative conflict handling that marks the affected data or function as uncertain. | [12, 37, 39] |
| 88 | 96 | Use independent confirmation, persistence and hysteresis, calibrated contextual thresholds, sensor self-test, and an explicit rule for preserving healthy capability when the evidence is conflicting. | [12, 37, 39] |
| 89 | 160 | Define maximum data age, allocate time-critical processing priority, adapt sampling when conditions change, and provide a bounded-latency health-monitoring path for safety-critical parameters. | [12, 37, 39] |
| 90 | 96 | Encryption. Strong ECC. Automatic plan re-request. Fallback to last valid plan. Radiation hardening or shielding. Redundancy on paths. | [40, 41] |
| 91 | 108 | Apply independent plan validation where justified, including format, identity, freshness, units, dynamics, resource, attitude, mission-constraint, and secondary-conjunction checks. Operator review is conditional on available time and communications. | [40, 41] |
| 92 | 105 | Threshold and validation tuning with realistic testing. Human override for marginal scenarios. Consider dual validation on onboard computer and/or ACAS. Fallback on rejected plan, if within safety margins. | [40, 41] |
| 93 | 144 | Use independent computation or review when time and resources permit, traceable time synchronization, validated propulsion and spacecraft-dynamics models, conservative margins, and continuous model maintenance using current thruster and resource-state information. | [40, 41] |
| 94 | 128 | Use unit-aware and configuration-controlled resource models, compare requested demand with independently estimated availability, model degraded actuators, apply conservative margins, and use closed-loop performance feedback during execution when supported. | [40, 41] |
| 95 | 81 | Automatic retry logic. Redundancy on communication paths and CU with switchover logic. EDC and ECC. | [40, 41] |
| 96 | 108 | Use deadline-aware dispatch, protected destination and parameter fields, end-to-end integrity and freshness checks, command readback or echo where feasible, and execution inhibition when validation or timing requirements are not met. | [40, 41] |
| 97 | 60 | Use propellant-management measures appropriate to the selected propellant, leak isolation, conservative reserve thresholds, redundant level estimation, and periodic fuel-budget reviews. Select collision-avoidance maneuvers that preserve required reserves. | [42, 43, 44, 45, 46] |
| 98 | 81 | Redundancy on valves and lines (parallel) with isolation-oriented design. Redundancy on heaters and respective power supply. Pre-burn functional checks. | [42, 43, 44, 45, 46] |
| 99 | 60 | Redundancy of power sources. Allow load shedding and graceful degradation. Fallback on minimal consumption state. Emergency power reserves (for critical CAMs). Monitoring battery health, to model trends and anticipate failures. | [42, 43, 44, 45, 46] |
| 100 | 81 | Redundancy on key components with protective and fault-isolation design to avoid cascading failure. Alternative paths and power supplies with automatic switchover. Health monitoring to detect degradation with periodic functionality tests to avoid failures in CAM. | [42, 43, 44, 45, 46] |
| 101 | 54 | Provide robust deployment and pointing design, validated fault detection and isolation, battery reserves, load shedding, and conservative maneuver planning. Use telemetry and imagery where available to diagnose deployment, arcing, or structural damage. | [43] |
| 102 | 18 | Provide battery or alternative power reserves, redundant power distribution where practicable, conservative load shedding, robust thermal and mechanical design, containment, and end-of-life disposal planning. |  |
| 103 | 36 | Provide redundant or segmented stacks where justified, robust reactant and thermal management, leak detection and isolation, battery reserves, load shedding, and conservative maneuver planning. | [44, 45, 46] |
| 104 | 96 | Redundancy on sensors with different modalities, with periodic calibration and health monitoring. Conservative consumption margins and reserve resources. Fallback on conservative maneuver planning strategies and consider implementing consumption estimation models. | [42, 44, 45] |
| 105 | 144 | Use independent sensing or estimation where justified, periodic calibration and health monitoring, conservative reserve margins, and maneuver planning that accounts explicitly for resource uncertainty. | [42, 44, 45] |
| 106 | 90 | Provide fault-isolating thruster allocation and sufficient independent actuation where justified, bounded automatic retry, safe isolation of failed units, validated reconfiguration logic, and maneuver plans that preserve recovery authority. | [40, 41] |
| 107 | 108 | Use validated thruster-allocation and fault-management logic, current calibration and alignment data, independent navigation-based performance estimation, isolation and reassignment of faulty units, and reserve margins for corrective action. | [40, 41] |
| 108 | 128 | Thruster redundancy with reassignment to healthy units. Automatic retries (with bounded attempts) and fault isolation preventing further damage. Conservative maneuver planning (particularly after anomalies). Extra delta v margins in CAM planning. Closed-loop feedback logic for burn extension with real-time navigation data. | [40, 41] |
| 109 | 108 | Apply propellant-management measures, monitor pressure and thrust stability, tune control and valve logic, permit safe throttle reduction or termination, reassign thrust to healthy units where available, and qualify the integrated spacecraft-propulsion system against relevant structural and control modes. | [40, 41] |
| 110 | 84 | Use navigation-based thrust estimation as a conservative fallback, independent performance sensors where justified, redundant reporting paths, health monitoring, stream watchdogs, and priority handling for maneuver-execution telemetry. | [40, 41] |
| 111 | 128 | Robust validation protocols on CRC/ECC, format and unit tagging. Redundancy and diversity on sensors with voting logic. Periodic self-test and routine calibration. Upon detection, fault containment (watchdog, channel isolation) and fallback on navigation data estimates with conservative logic. | [40, 41] |
| 112 | 90 | Automatic alert of operators. Pre-agreed degraded mode on simplified model, CAM templates or last state with inflated covariance. Redundancy on data and paths with voting logic and different modalities, or channel substitution. Adaptive thresholds based on data completeness. Self-testing and bounded re-request/restart logic. | [6, 13, 15, 18, 27, 28, 34, 35] |
| 113 | 70 | Enforce format, unit, identity, reference-frame, timestamp, covariance, completeness, integrity, authenticity, and plausibility checks. Isolate or down-weight suspect data and use a bounded-age validated fallback with explicit uncertainty. | [6, 13, 15, 18, 27, 28, 34, 35] |
| 114 | 56 | Validate format, units, integrity, authenticity, timestamp, object identity, and physical plausibility. Use source-quality management and a conservative degraded mode based on a bounded-age last valid state or validated propagation model. | [6, 13, 15, 18, 27, 28, 34, 35] |
| 115 | 80 | Tune and configuration-control validation thresholds, use bounded context-aware adaptation, down-weight uncertain sources instead of rejecting them unconditionally where justified, and alert operators when rejection rates or data completeness threaten the decision window. | [6, 13, 15, 18, 27, 28, 34, 35] |
| 116 | 162 | Enforce mandatory integrity, identity, unit, timestamp, and plausibility checks. Isolate or down-weight suspect sources according to a controlled source-quality policy, and require an independent review or confirmation path for marginal safety-significant cases. | [6, 13, 15, 18, 27, 28, 34, 35] |
| 117 | 100 | Incorporate ISO standards in deadline alarms and enforcement. Early exit fallback on approximated but timely result (vs. better but late). Boundness on rerequests. Memory and queue control. Pre-run simulation. Priority management (w.r.t. TCA deadline). | [6, 13, 15, 18, 27, 28, 34, 35] |
| 118 | 162 | Strengthen upstream data-quality controls, test and validate models across representative geometries and uncertainty conditions, control configuration and numerical tolerances, and use conservative uncertainty treatment when model validity is limited. | [6, 13, 15, 18, 27, 28, 34, 35] |
| 119 | 120 | Reserve processing resources according to time to closest approach, bound confirmation waits, define deterministic tie-breaking and timeout logic, test deadline behavior in representative scenarios, and enter a predefined degraded decision mode that still enforces maneuver feasibility, mission constraints, and secondary-conjunction checks. | [6, 13, 27, 28] |
| 120 | 144 | Use input-quality and confidence checks, independent decision cross-checks, threshold hysteresis where appropriate, and human confirmation for marginal cases when time and communications permit. Verify maneuver feasibility and secondary-conjunction risk before authorization. | [6, 13, 27, 28] |
| 121 | 180 | Use input-quality and confidence checks, independent decision cross-checks, and conservative uncertainty treatment. Human confirmation can support marginal cases when time and communications permit. Any maneuver authorization must still pass feasibility, mission-constraint, and secondary-conjunction checks. | [6, 13, 27, 28] |
| 122 | 96 | Keep decision history per event with deduplication logic. Pre-defined window for commitment to freeze (after ACK from downstream module with a pre-defined waiting window), changes requiring override. Hysteresis on decision logic. Decision and override logs with periodic review. | [6, 13, 27, 28] |
| 123 | 80 | After the deadline, issue predefined conservative CAM templates. Under time pressure, consider simpler planning models. Allow humanin-the-loop (such as for critical scenarios) and operator assistance with respective checklists and periodic drills. Post-event reviews. Priority management and resource reservation. Robust design and communications, with backups and watchdog on tasks. | [6, 13, 27, 28] |
| 124 | 120 | Perform periodic drills with operator checklists. post-event reviews. Pre-defined deadlines and fallbacks such as human intervention and CAM template As deadline approaches, consider simpler planning models. Priority management (higher as TCA approaches) and resource reservation. | [6, 13, 27, 28] |
| 125 | 90 | CAM logging with post-execution reviews and tuning, as well as periodic drills. Extensive model testing against realistic scenarios with policy-aligned objectives. Adaptive model w.r.t. time constraint. Operator guardrails and decision support on overriding. Fallback on pre-defined conservative CAMs. | [6, 13, 27, 28] |
| 126 | 144 | Maintain a controlled and traceable constraint set, apply explicit margins at constraint boundaries, verify every generated plan, and provide validated maneuver templates for representative scenarios. Permit operator intervention when time and communications allow. | [6, 13, 27, 28] |
| 127 | 80 | Enforce version, schema, unit, identity, timestamp, integrity, dynamics, resource, mission-constraint, and secondary-conjunction validation before authorization. Preserve time for regeneration and use only configuration-controlled templates or operator intervention when applicable. | [6, 13, 27, 28] |

# S3. Rating scales and application rules

Table S3. Severity, relative-occurrence, and detectability scales.

| Dimension | Rank | Anchor | Application rule |
| --- | --- | --- | --- |
| Severity | 1 | No relevant effect | No effect on the ACAS function, safety, or mission. Assign the worst credible end effect within the defined ACAS boundary and operational assumptions. |
| Severity | 2 | Negligible local effect | Small local disturbance with no meaningful operational consequence. |
| Severity | 3 | Minor recoverable degradation | Function degrades but normal collision-avoidance capability and margins are preserved. |
| Severity | 4 | Limited operational effect | Temporary degradation or rework is required; no loss of essential ACAS capability. |
| Severity | 5 | Moderate degradation | Reduced performance, increased workload, or limited resource loss; CAM capability remains available. |
| Severity | 6 | Significant degradation | A supporting function is impaired and margins or availability are materially reduced. |
| Severity | 7 | Major degradation | Partial loss of an ACAS function, reduced maneuver options, or substantial mission impact; recovery remains credible. |
| Severity | 8 | Serious safety-relevant effect | A key output can be lost or corrupted, producing an unnecessary or ineffective response or a serious unsafe state. |
| Severity | 9 | Critical effect | Credible loss of a required barrier or function can leave a hazardous conjunction insufficiently controlled or cause major mission degradation. |
| Severity | 10 | Catastrophic end effect | The failure can directly leave a credible collision unmitigated or contribute to loss of spacecraft or mission; for crewed applications, potential loss of life. |
| Relative occurrence | 1 | Exceptional | Requires exceptional conditions or has exceptionally strong demonstrated reliability within the stated assumptions. This is a comparative engineering rank, not a probability. Use causal exposure, architecture, environment, and assumed baseline controls. |
| Relative occurrence | 2 | Very remote | Credible but expected only under rare initiating conditions or multiple adverse conditions. |
| Relative occurrence | 3 | Remote | A credible single failure or combination that is not expected frequently during the mission lifecycle. |
| Relative occurrence | 4 | Occasional | A plausible failure mechanism with meaningful lifecycle exposure or several credible initiating causes. |
| Relative occurrence | 5 | Repeated under some conditions | Can recur in relevant operational or environmental conditions without additional controls. |
| Relative occurrence | 6 | Moderately frequent | Expected to occur more than occasionally in the conceptual lifecycle or operating environment. |
| Relative occurrence | 7 | Frequent | Likely to recur across missions or extended operation without additional controls. |
| Relative occurrence | 8 | Very frequent | Expected repeatedly in common operating conditions. |
| Relative occurrence | 9 | Near-certain | Highly likely during the relevant operational period without effective control. |
| Relative occurrence | 10 | Expected/continuous | Expected to occur or persist as a normal consequence of the baseline design or environment. |
| Detectability | 1 | Almost certain and immediate | Independent automatic indication is expected before the unsafe effect develops. Rank the probability of timely detection by the baseline provisions listed in the row. Higher D means harder to detect before the effect becomes irreversible. |
| Detectability | 2 | Very high timely detectability | Direct automatic monitoring or clear missing-output logic is expected to detect the condition promptly. |
| Detectability | 3 | High timely detectability | One or more reliable checks should detect the condition within the response window. |
| Detectability | 4 | Moderate-high detectability | Detection is likely but depends on data availability, timing, correlation, or a single monitoring path. |
| Detectability | 5 | Moderate detectability | Detection is possible but may be indirect, delayed, or dependent on cross-source analysis. |
| Detectability | 6 | Moderate-low detectability | The failure can remain hidden through a significant part of the decision window or resemble valid behavior. |
| Detectability | 7 | Low detectability | Only weak, delayed, or non-independent indications are available before the end effect. |
| Detectability | 8 | Very low detectability | Detection generally relies on secondary consequences or exceptional review. |
| Detectability | 9 | Post-event detectability | The condition is unlikely to be identified before the unsafe effect; evidence is mainly retrospective. |
| Detectability | 10 | No credible timely detection | No credible provision is available to detect the condition before the end effect. |

# S4. Block-level summary

Table S4. RPN and severity summary by functional block.

| Functional block | Count | Median RPN | Q1 | Q3 | Maximum RPN | RPN ≥ 120 | Severity ≥ 9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Onboard sensors | 12 | 135 | 123 | 144 | 160 | 9 | 6 |
| ACAS decision module | 4 | 132 | 114 | 153 | 180 | 3 | 2 |
| Conjunction assessment | 9 | 120 | 96 | 144 | 160 | 5 | 4 |
| Ground-based tracking | 7 | 120 | 69 | 134 | 160 | 4 | 2 |
| Onboard computer | 18 | 116 | 88 | 128 | 144 | 9 | 7 |
| Propulsion control unit | 7 | 108 | 100.5 | 118 | 144 | 2 | 4 |
| Thrusters | 6 | 108 | 94.5 | 123 | 128 | 2 | 3 |
| Onboard communications | 19 | 96 | 93 | 108 | 128 | 3 | 7 |
| Ground control center | 24 | 93 | 78 | 114 | 135 | 6 | 9 |
| ACAS data processing | 7 | 90 | 75 | 131 | 162 | 2 | 4 |
| ACAS maneuver planning | 5 | 90 | 80 | 120 | 144 | 2 | 4 |
| Fuel and power resources | 9 | 60 | 54 | 81 | 144 | 1 | 8 |

# S5. Twenty highest baseline RPN values

Table S5. Twenty highest RPN values. The ranking is descriptive and is not a risk-acceptance threshold.

| Rank | ID | Functional block | Function | Failure mode | S | O | D | RPN | Risk-control measures |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 121 | ACAS decision module | Decide whether a CAM is required | False-negative decision | 10 | 3 | 6 | 180 | Use input-quality and confidence checks, independent decision cross-checks, and conservative uncertainty treatment. Human confirmation can support marginal cases when time and communications permit. Any maneuver authorization must still pass feasibility, mission-constraint, and secondary-conjunction checks. |
| 2 | 116 | ACAS data processing | Validate ACAS inputs | False-negative validation | 9 | 3 | 6 | 162 | Enforce mandatory integrity, identity, unit, timestamp, and plausibility checks. Isolate or down-weight suspect sources according to a controlled source-quality policy, and require an independent review or confirmation path for marginal safety-significant cases. |
| 3 | 118 | ACAS data processing | Compute collision probability | Incorrect collision-probability output | 9 | 3 | 6 | 162 | Strengthen upstream data-quality controls, test and validate models across representative geometries and uncertainty conditions, control configuration and numerical tolerances, and use conservative uncertainty treatment when model validity is limited. |
| 4 | 12 | Conjunction assessment | Compute conjunction data | Conjunction data computed late | 8 | 4 | 5 | 160 | Use deadline-aware scheduling, adequate computing resources, performance monitoring, and optimized algorithms. Prioritize conjunctions according to documented operational criteria. |
| 5 | 14 | Conjunction assessment | Provide conjunction data to ground control | Conjunction data delivered late | 8 | 4 | 5 | 160 | Monitor delivery latency, prioritize time-critical conjunction data, and provide redundant communication paths. Define a latest-useful delivery time and an escalation response. |
| 6 | 85 | Onboard sensors | Measure environmental conditions | Late measurement | 8 | 4 | 5 | 160 | Define maximum data age, allocate time-critical processing priority, adapt sampling when conditions change, and bound filtering delay. Use independent environmental indications where justified. |
| 7 | 89 | Onboard sensors | Measure spacecraft health | Late measurement | 8 | 4 | 5 | 160 | Define maximum data age, allocate time-critical processing priority, adapt sampling when conditions change, and provide a bounded-latency health-monitoring path for safety-critical parameters. |
| 8 | 5 | Ground-based tracking | Provide tracking data to conjunction assessment | Tracking data delivered late | 8 | 5 | 4 | 160 | Monitor end-to-end latency, prioritize time-critical data, and provide redundant network paths. Define a latest-useful time and a conservative response when it is exceeded. |
| 9 | 9 | Conjunction assessment | Compute conjunction data | Incorrect conjunction data computed | 9 | 4 | 4 | 144 | Validate inputs and reference data, test the propagation and probability models across representative geometries and uncertainty conditions, and use independent cross-checks for safety-significant cases. |
| 10 | 11 | Conjunction assessment | Compute conjunction data | Incomplete conjunction data computed | 9 | 4 | 4 | 144 | Request or obtain an independent data source when time permits. Use a documented conservative degraded mode only when its applicability and uncertainty bounds have been validated. |
| 11 | 64 | Onboard computer | Process telecommands | Command not decoded | 9 | 4 | 4 | 144 | Use link-appropriate error-detection and correction coding, adequate link margin, robust synchronization and decoding logic, and redundant receiver paths where justified. |
| 12 | 72 | Onboard computer | Perform autonomous reconfiguration | False-negative autonomous reconfiguration | 9 | 4 | 4 | 144 | Validate trigger logic across representative faults and timing conditions, provide independent health indications where justified, and define a conservative safe configuration when trigger evidence is conflicting or incomplete. |
| 13 | 78 | Onboard sensors | Measure navigation and attitude state | Unusable GNSS measurement when service is available | 9 | 4 | 4 | 144 | Use multi-constellation and, where applicable, multi-frequency GNSS with integrity monitoring. Cross-check against independent navigation sensors and fall back to a validated fused-state estimate when GNSS is unavailable or rejected. |
| 14 | 80 | Onboard sensors | Measure navigation and attitude state | Unusable IMU measurement | 9 | 4 | 4 | 144 | Use in-flight bias and scale-factor estimation, thermal control, adequate dynamic range, radiation and vibration protection, and cross-checking with independent navigation sensors. Provide fault isolation and redundant IMUs where justified. |
| 15 | 86 | Onboard sensors | Measure spacecraft health | No measurement | 9 | 4 | 4 | 144 | Provide diverse or independently powered health sensing where justified, data-stream watchdogs, adaptive but bounded sampling, indirect symptom checks, and a predefined degraded mode when required health evidence is unavailable. |
| 16 | 93 | Propulsion control unit | Compute thrust vector and resource allocation | Wrong thrust vector computed | 9 | 4 | 4 | 144 | Use independent computation or review when time and resources permit, traceable time synchronization, validated propulsion and spacecraft-dynamics models, conservative margins, and continuous model maintenance using current thruster and resource-state information. |
| 17 | 105 | Fuel and power resources | Monitor fuel and power state | Wrong resource level reported | 9 | 4 | 4 | 144 | Use independent sensing or estimation where justified, periodic calibration and health monitoring, conservative reserve margins, and maneuver planning that accounts explicitly for resource uncertainty. |
| 18 | 120 | ACAS decision module | Decide whether a CAM is required | False-positive decision | 8 | 3 | 6 | 144 | Use input-quality and confidence checks, independent decision cross-checks, threshold hysteresis where appropriate, and human confirmation for marginal cases when time and communications permit. Verify maneuver feasibility and secondary-conjunction risk before authorization. |
| 19 | 126 | ACAS maneuver planning | Generate a CAM plan | CAM plan violates constraints | 8 | 3 | 6 | 144 | Maintain a controlled and traceable constraint set, apply explicit margins at constraint boundaries, verify every generated plan, and provide validated maneuver templates for representative scenarios. Permit operator intervention when time and communications allow. |
| 20 | 2 | Ground-based tracking | Track space objects | Insufficient tracking data | 7 | 5 | 4 | 140 | Parameter tuning. Object tracking priority. Inter-agency cooperation (in downstream conjunction assessment). Additional sources and redundancy. |

# S6. Severity-led and RPN-based priority review

Table S6. Failure modes selected through severity-led screening and upper-RPN grouping.

| ID | Functional block | Function | Failure mode | S | O | D | RPN | Priority basis | Interpretation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 121 | ACAS decision module | Decide whether a CAM is required | False-negative decision | 10 | 3 | 6 | 180 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 13 | Conjunction assessment | Provide conjunction data to ground control | No conjunction data sent | 10 | 3 | 4 | 120 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 119 | ACAS decision module | Decide whether a CAM is required | No decision within deadline | 10 | 4 | 3 | 120 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 124 | ACAS maneuver planning | Generate a CAM plan | Delayed CAM plan generated | 10 | 4 | 3 | 120 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 60 | Onboard computer | Convert and regulate electrical power | No power output | 10 | 5 | 2 | 100 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 117 | ACAS data processing | Compute collision probability | Delayed output | 10 | 5 | 2 | 100 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 24 | Ground control center | Decide ground response and collision-avoidance maneuver | No CAM plan generated | 10 | 3 | 3 | 90 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 41 | Onboard communications | Receive data from ground | No data received | 10 | 3 | 3 | 90 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 106 | Thrusters | Execute thrust commands | Required propulsion unavailable | 10 | 3 | 3 | 90 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 123 | ACAS maneuver planning | Generate a CAM plan | No CAM plan generated | 10 | 4 | 2 | 80 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 127 | ACAS maneuver planning | Generate a CAM plan | Invalid CAM plan generated | 10 | 4 | 2 | 80 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 29 | Ground control center | Transmit commands to spacecraft | No contingency control order sent | 10 | 2 | 3 | 60 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 97 | Fuel and power resources | Provide fuel and electrical power | Empty fuel tank | 10 | 2 | 3 | 60 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 99 | Fuel and power resources | Provide fuel and electrical power | No power available or stored | 10 | 2 | 3 | 60 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 8 | Conjunction assessment | Compute conjunction data | No conjunction data computed | 10 | 3 | 1 | 30 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 116 | ACAS data processing | Validate ACAS inputs | False-negative validation | 9 | 3 | 6 | 162 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 118 | ACAS data processing | Compute collision probability | Incorrect collision-probability output | 9 | 3 | 6 | 162 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 9 | Conjunction assessment | Compute conjunction data | Incorrect conjunction data computed | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 11 | Conjunction assessment | Compute conjunction data | Incomplete conjunction data computed | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 64 | Onboard computer | Process telecommands | Command not decoded | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 72 | Onboard computer | Perform autonomous reconfiguration | False-negative autonomous reconfiguration | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 78 | Onboard sensors | Measure navigation and attitude state | Unusable GNSS measurement when service is available | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 80 | Onboard sensors | Measure navigation and attitude state | Unusable IMU measurement | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 86 | Onboard sensors | Measure spacecraft health | No measurement | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 93 | Propulsion control unit | Compute thrust vector and resource allocation | Wrong thrust vector computed | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 105 | Fuel and power resources | Monitor fuel and power state | Wrong resource level reported | 9 | 4 | 4 | 144 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 17 | Ground control center | Monitor spacecraft status and radio-frequency interference | Undetected anomaly | 9 | 3 | 5 | 135 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 19 | Ground control center | Monitor spacecraft status and radio-frequency interference | False-negative anomaly detection | 9 | 3 | 5 | 135 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 65 | Onboard computer | Process telecommands | Command decoded incorrectly | 9 | 3 | 5 | 135 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 67 | Onboard computer | Process telecommands | False-positive validation | 9 | 3 | 5 | 135 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 83 | Onboard sensors | Measure environmental conditions | False-negative environmental anomaly | 9 | 3 | 5 | 135 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 87 | Onboard sensors | Measure spacecraft health | False-negative health anomaly | 9 | 3 | 5 | 135 | Severity-led and upper RPN | Review irrespective of RPN because S ≥ 9. |
| 26 | Ground control center | Decide ground response and collision-avoidance maneuver | Incomplete or incorrect CAM plan generated | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 43 | Onboard communications | Receive data from ground | Invalid ground data received | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 47 | Onboard communications | Receive inter-spacecraft coordination data | No data received | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 58 | Onboard communications | Operate in the space environment | Antenna degraded | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 59 | Onboard communications | Operate in the space environment | Degraded onboard communication connector or interface | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 63 | Onboard computer | Process telecommands | No decryption | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 74 | Onboard computer | Perform autonomous reconfiguration | Inadequate reconfiguration | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 91 | Propulsion control unit | Receive and validate the CAM plan | False-negative validation | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 96 | Propulsion control unit | Dispatch propulsion commands | Invalid control sent | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 107 | Thrusters | Execute thrust commands | Incorrect propulsion | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 109 | Thrusters | Execute thrust commands | Unstable propulsion | 9 | 3 | 4 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 81 | Onboard sensors | Measure navigation and attitude state | Unusable star-tracker measurement | 9 | 4 | 3 | 108 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 125 | ACAS maneuver planning | Generate a CAM plan | Incorrect or suboptimal CAM plan generated | 9 | 2 | 5 | 90 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 112 | ACAS data processing | Gather external and spacecraft data | Missing spacecraft data | 9 | 5 | 2 | 90 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 27 | Ground control center | Decide ground response and collision-avoidance maneuver | CAM plan creates new conjunction risk | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 28 | Ground control center | Decide ground response and collision-avoidance maneuver | CAM plan violates mission objectives | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 31 | Ground control center | Transmit commands to spacecraft | Command sent too late | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 44 | Onboard communications | Transmit data to ground | No data transmitted | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 50 | Onboard communications | Transmit inter-spacecraft coordination data | No data transmitted | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 95 | Propulsion control unit | Dispatch propulsion commands | No control sent | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 98 | Fuel and power resources | Provide fuel and electrical power | Fuel not supplied | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 100 | Fuel and power resources | Provide fuel and electrical power | No power supplied | 9 | 3 | 3 | 81 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 33 | Ground control center | Relay coordination data to spacecraft | Command sent with the wrong target or context | 9 | 2 | 3 | 54 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 1 | Ground-based tracking | Track space objects | Loss of tracking data | 9 | 3 | 2 | 54 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 4 | Ground-based tracking | Provide tracking data to conjunction assessment | No tracking data sent | 9 | 3 | 2 | 54 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 101 | Fuel and power resources | Provide fuel and electrical power | Loss of solar-array power generation, if used | 9 | 3 | 2 | 54 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 103 | Fuel and power resources | Provide fuel and electrical power | Loss of fuel-cell power generation, if used | 9 | 2 | 2 | 36 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 102 | Fuel and power resources | Provide fuel and electrical power | Loss of radioisotope power generation, if used | 9 | 1 | 2 | 18 | Severity-led candidate | Review irrespective of RPN because S ≥ 9. |
| 12 | Conjunction assessment | Compute conjunction data | Conjunction data computed late | 8 | 4 | 5 | 160 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 14 | Conjunction assessment | Provide conjunction data to ground control | Conjunction data delivered late | 8 | 4 | 5 | 160 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 85 | Onboard sensors | Measure environmental conditions | Late measurement | 8 | 4 | 5 | 160 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 89 | Onboard sensors | Measure spacecraft health | Late measurement | 8 | 4 | 5 | 160 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 5 | Ground-based tracking | Provide tracking data to conjunction assessment | Tracking data delivered late | 8 | 5 | 4 | 160 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 120 | ACAS decision module | Decide whether a CAM is required | False-positive decision | 8 | 3 | 6 | 144 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 126 | ACAS maneuver planning | Generate a CAM plan | CAM plan violates constraints | 8 | 3 | 6 | 144 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 3 | Ground-based tracking | Track space objects | Inaccurate tracking data | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 23 | Ground control center | Decide ground response and collision-avoidance maneuver | Failure to react upon anomaly | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 25 | Ground control center | Decide ground response and collision-avoidance maneuver | Delayed CAM plan generated | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 34 | Ground control center | Relay coordination data to spacecraft | Delayed data relay | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 42 | Onboard communications | Receive data from ground | Delayed data received | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 45 | Onboard communications | Transmit data to ground | Delayed data transmitted | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 51 | Onboard communications | Transmit inter-spacecraft coordination data | Coordination data transmitted late | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 66 | Onboard computer | Process telecommands | Command decoded too late | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 75 | Onboard computer | Format telemetry packets | Packet not generated | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 79 | Onboard sensors | Measure navigation and attitude state | Unusable range or Doppler measurement | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 82 | Onboard sensors | Measure environmental conditions | No measurement | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 94 | Propulsion control unit | Compute thrust vector and resource allocation | Wrong resource allocation computed | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 108 | Thrusters | Execute thrust commands | Degraded or incomplete propulsion | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 111 | Thrusters | Report propulsion performance | Invalid/incorrect report sent | 8 | 4 | 4 | 128 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 6 | Ground-based tracking | Provide tracking data to conjunction assessment | Tracking data sent as invalid TDMs | 8 | 3 | 5 | 120 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 21 | Ground control center | Monitor spacecraft status and radio-frequency interference | False-negative RFI analysis | 8 | 3 | 5 | 120 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 68 | Onboard computer | Process telecommands | False-negative validation | 8 | 3 | 5 | 120 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 71 | Onboard computer | Process telecommands | Telecommand sent to the wrong subsystem | 8 | 3 | 5 | 120 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 2 | Ground-based tracking | Track space objects | Insufficient tracking data | 7 | 5 | 4 | 140 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |
| 62 | Onboard computer | Convert and regulate electrical power | Unstable power | 6 | 4 | 5 | 120 | Upper RPN group | Review because the descriptive RPN is in the upper group; this is not an acceptance threshold. |

# S7. FMECA framework conformance and tailoring

Table S7. Framework conformance, tailoring, and mission-specific completion needs.

| Framework element | Implementation in this study | Mission-specific completion needed |
| --- | --- | --- |
| Purpose and decision use | The purpose is failure identification, traceability, and comparative design-assurance prioritization. | Define program acceptance criteria, owners, and closure evidence. |
| System boundary and functional hierarchy | A distributed ACAS operational boundary and 34 parent functions are defined. | Tailor to the actual spacecraft, operator, services, interfaces, mission phases, and modes. |
| Architecture and interfaces | The author-generated architecture traces information, authority, resource, actuation, and feedback paths. | Replace with controlled design and interface baselines. |
| Failure-mode identification | 127 functional deviations cover omission, delay, corruption, unintended action, intermittency, and degraded output. | Perform multidisciplinary completeness review and add implementation-specific modes. |
| Effects analysis | A consolidated effects field records functional and end effects. | Separate local, next-higher-level, system, and mission effects where required by the program. |
| Cause analysis | Hardware, software, human, environmental, timing, communication, data, and configuration causes are considered. | Develop item-level causes, dependent failures, and verified failure mechanisms. |
| Baseline detection and compensating provisions | D is based only on the baseline detection provisions listed in each row and on timely detection. | Verify diagnostic coverage, latency, independence, false-alarm behavior, and recovery. |
| Severity and occurrence | Severity uses the worst credible end effect; O is a relative engineering rank, not a probability. | Replace or calibrate O with mission evidence and approved program categories where available. |
| Criticality and RPN | RPN = S x O x D supports comparison; S ≥ 9 is screened independently of RPN. | Use approved criticality numbers or matrices if quantitative rates and program criteria exist. |
| Single-point and common-cause analysis | Potential common causes are discussed but not modeled systematically. | Perform single-point-failure, common-cause, dependent-failure, and interface analyses. |
| Recommended controls and residual risk | Preventive, detective, compensating, and mitigating controls are proposed. | Assign owners, verification methods, status, and residual rankings only after implementation evidence. |
| Iteration and configuration control | Recommendations are treated as configuration-controlled changes requiring iterative maintenance and controlled updates. | Integrate change-impact reassessment with project dependability, STPA or equivalent interaction analysis, hazard analysis, change control, verification, validation, and operations processes. |
| Post-implementation change-impact reassessment | Every implemented recommendation or recommendation set requires reassessment before closure or residual-risk acceptance. | Reassess hazards, unsafe control actions, loss scenarios, interfaces, common-cause and dependent failures, cybersecurity, degraded modes, recommendation conflicts, affected rankings, and verification evidence. |