Supplementary material

Tables

Table S1: Functional electrical stimulation force model – Ding et al. (2007)

| Symbol | Unit | Definition |
| --- | --- | --- |
| cn | - | Representation of the Ca2+-troponin complex |
| fm | N | Muscle force |
| a | N/s | Force scaling factor related to pulse width |
| km | - | Sensitivity between strongly bound cross-bridges and cn |
| τ1 | s | Time constant force decline due to strongly bound cross-bridges absence |
| τ2 | s | Time constant force decline due to the extra friction between actin and myosin |
| τc | s | Time constant controlling the rise and decay of cn |
| t | s | Time since the start |
| ti | s | Time at which the i-th stimulation is delivered |
| rj | - | Factor accounting for Ca2+ nonlinear summation of closely spaced stimulation |
| r0 | - | Term characterizing cn enhancement from following stimuli |
| a' | N/s | Muscle force factor |
| pw | s | Stimulation pulse width |
| pw0 | s | Minimum pulse width leading to force production |
| pwt | s | Time constant controlling the a-pw relationship |

Table S2: Functional electrical stimulation fatigue model – Ding et al. (2003)

| Symbol | Unit | Definition |
| --- | --- | --- |
| arest' | N/s | Rested muscle scaling factor a' |
| kmrest | - | Rested sensitivity km |
| τ1rest | s | Rested time constant τ1 |
| αa' | s-2 | Fatigue coefficient for a' |
| αkm | s-1N-1 | Fatigue coefficient for km |
| ατ1 | N-1 | Fatigue coefficient for τ1 |
| τfat | s | Time constant controlling the recovery of a', km and τ1 |

Table S3: Scaling method and FES models values

| Parameters | Method | Values |
| --- | --- | --- |
| PCSA (cm2) | Values from [1] and [2] | rf=10.88 |
|  |  | da=2.54 dp=2.73 bic=7.33 tri=15.56 |
| Fiber type I/II proportion | Values from [3] and [4] | rf=35/65 |
|  |  | da=47/53 dp=56/44 bic=38/62 tri=44/56 |
| arest' (N/s) | a'restm=a'restrf*PCSAmPCSArf | rf=4920 |
|  |  | da=1148.6 dp=1234.5 bic=3314.7 tri=7036.3 |
| fmaxm (N) | Maximum produced force for a 1 second 33 Hz pulse train with arest',m | rf=287 |
|  |  | da=48 dp=51 bic=149 tri=617 |
| αa' (N-2) | αa'm=αa'rf*fpmfprf*a'restrfa'restm | rf=-4.0*10-2 |
|  |  | da=-1.4*10-1 dp=-1.1*10-1 bic=-5.6*10-2 tri=-3.4*10-2 |
| τfat (s) | τfatm=τfatrf*fpmfprf*a'restrfa'restm | rf=127 |
|  |  | da=445.5 dp=342.7 bic=179.6 tri=109.1 |

rf: rectus femoris; da: anterior deltoid; dp: posterior deltoid; bic: biceps; tri: triceps; fp: fiber type proportion

[1]	Peterson SL, Rayan GM. Shoulder and Upper Arm Muscle Architecture. J Hand Surg Am 2011;36:881–9. https://doi.org/10.1016/j.jhsa.2011.01.008.

[2]	KUMAZAKI T, TAKAHASHI T, NAKANO T, SAKAI T. Action and Contribution of the Iliopsoas and Rectus Femoris as Hip Flexor Agonists Examined with Anatomical Analysis. Juntendo Medical Journal 2022;68:JMJ22-0009-OA. https://doi.org/10.14789/jmj.JMJ22-0009-OA.

[3]	Srinivasan RC, Lungren MP, Langenderfer JE, Hughes RE. Fiber type composition and maximum shortening velocity of muscles crossing the human shoulder. Clinical Anatomy 2007;20:144–9. https://doi.org/10.1002/ca.20349.

[4]	Kusumoto Y, Goto H, Chiba K, Oonishi S, Tsuchiya J. Characteristics of muscle contraction of the rectus femoris using tensiomyography by sex in healthy college students: a cross-sectional study. PeerJ 2023;11:e14732. https://doi.org/10.7717/peerj.14732.

Table S4: States and controls boundaries (min in red, max in blue) for the optimal control program.

|  |  | OCP |  |  |
| --- | --- | --- | --- | --- |
|  |  | Node |  |  |
| States |  | 0 | 1…N-1 | N |
| cn (-) | Anterior deltoid, Posterior deltoid, Biceps, Triceps | 0 10 |  |  |
| fm (N) | Anterior deltoid | 0 48 |  |  |
|  | Posterior deltoid | 0 51 |  |  |
|  | Biceps | 0 149 |  |  |
|  | Triceps | 0 262 |  |  |
| a' (N/s) | Anterior deltoid | 0 1148.6 |  |  |
|  | Posterior deltoid | 0 1234.5 |  |  |
|  | Biceps | 0 3314.7 |  |  |
|  | Triceps | 0 4915.5 |  |  |
| τ1 (s) | Anterior deltoid, Posterior deltoid, Biceps, Triceps | 0.060601 1 |  |  |
| km (-) | Anterior deltoid, Posterior deltoid, Biceps, Triceps | 0.137 1 |  |  |
| q (°) | Shoulder | 0 85.94 |  |  |
|  | Elbow | 28.65 143.24 |  |  |
|  | Hand/pedal | 0 0 | 0 363 | 357 363 |
| q (°/s) | Shoulder | -573 573 |  |  |
|  | Elbow | -573 573 |  |  |
|  | Hand/pedal | 187.93 531.70 |  |  |
| Controls |  | 0 | 1…N-2 | N-1 |
| pw (µs) | Anterior deltoid, Posterior deltoid, Biceps, Triceps | 131 600 |  |  |

Table 5: Cost function formulation

| Index | Objective function | Description |
| --- | --- | --- |
|  | Pulse width |  |
| ɸpw1 | 0T1nmm=1nmpwmt dt | Minimize the average pulse width activation |
| ɸpw2 | 0T1nmm=1nmpwmt212 dt | Minimize the quadratic average pulse width activation |
| ɸpw3 | 0T1nmm=1nmpwmt313 dt | Minimize the cubic average pulse width activation |
| ɸpw∞ | 0Tmaxm=1, ... ,nmpwmt dt | Minimize the peak pulse width activation |
|  | Force |  |
| ɸfm1 | 0T1nmm=1nmfmmt dt | Minimize the average muscle force |
| ɸfm2 | 0T1nmm=1nmfmmt212 dt | Minimize the quadratic average muscle force |
| ɸfm3 | 0T1nmm=1nmfmmt313 dt | Minimize the cubic average muscle force |
| ɸfm∞ | 0Tmaxm=1, ... , nmfmmt dt | Minimize the peak muscle force |
|  | Stress |  |
| ɸσ1 | 0T1nmm=1nmfmmtPCSAm dt | Minimize the average muscle stress |
| ɸσ2 | 0T1nmm=1nmfmmtPCSAm212 dt | Minimize the quadratic average muscle stress |
| ɸσ3 | 0T1nmm=1nmfmmtPCSAm313 dt | Minimize the cubic average muscle stress |
| ɸσ∞ | 0Tmaxm=1, ... , nmfmmtPCSAm dt | Minimize the peak muscle stress |

|  | Fatigue |  |
| --- | --- | --- |
| ɸa'1 | 0T1nmm=1nma'restm-a'mt dt | Minimize the average muscle fatigue |
| ɸa'2 | 0T1nmm=1nma'restm-a'mt212 dt | Minimize the quadratic average muscle fatigue |
| ɸa'3 | 0T1nmm=1nma'restm-a'mt313 dt | Minimize the cubic average muscle fatigue |
| ɸa'∞ | 0Tmaxm=1, ... , nma'restm-a'mt dt | Minimize the peak muscle fatigue |
|  | Power |  |
| ɸw2 | 0T1nmm=1nmfmmt×lCEm212 dt | Minimize the quadratic average muscle power |
|  | Bayesian optimization / Physiological weight |  |
| ɸpwω2 | 0T1nmm=1nmωm×pwmt212 dt | Minimize the weighted quadratic average pulse width activation |
| ɸfmω2 | 0T1nmm=1nmωm×fmmt212 dt | Minimize the weighted quadratic average muscle force |
| ɸσω2 | 0T1nmm=1nmωm×fmmtPCSAm212 dt | Minimize the weighted quadratic average muscle stress |
| ɸa'ω2 | 0T1nmm=1nmωm×a'restm-a'mt212 dt | Minimize the weighted quadratic average muscle fatigue |
| ɸwω2 | 0T1nmm=1nmωm×fmmt×lCEm212 dt | Minimize the weighted quadratic average muscle power |

Table S6: Equation table for physiological and mechanical weight calculation

| N° | Definition | Equation | Description |
| --- | --- | --- | --- |
| 1 | Muscle-generated tangential crank torque | τm(θ)=r(θ) (fmaxm(θ)⋅et(θ)) | Tangential (et) produced torque (τ), with r=0.1 the arm lever for each 360° pedal angle (θ) per muscle (m) at maximum force (fmaxm). |
| 2 | Positive task-supporting torque | τm+(θ)=max⁡(τm(θ),0) | Positive muscle torque τm+(θ) per angle. |
| 3 | Contribution task ratio | contribm=1360θ=13601[τm(θ)>0 ] | Based on the first-order Ding et al. (2003) fatigue dynamics model, the muscle fatigability metric (fatm) was calculated using the slope between the 1st and 1500th cycle of the remaining capacity ratios. Muscle fatigue (a'm) was estimated for each cycle (k) using a constant produced force (ρfmaxm) of 80% fmaxm, activated during contribution phases (tactm) and deactivated when not contributing to the task (trestm). This metric represents the probability of a muscle to fatigue |
|  | Standardized repeated demand | ρ=0.8 |  |
|  | Activation and rest time | tactm=contribm, trestm=1-contribm |  |
|  | Total cycles | k∈[1, 1500] |  |
|  | Muscle fatigue evolution during active phase | a'act, km=a'restm+αa'm*ρfmaxm*τfatm1-e-tactmτfatm+e-tactmτfatm*a' km |  |
|  | Muscle fatigue evolution per cycle | a'k+1m=a'restm1-e-trestmτfatm+e-trestmτfatm*a'act, km |  |
|  | Muscle fatigability metric | fatm=a'1500ma'restm-a'1ma'restm ÷ 1500 |  |
| 6 | Muscle redundancy | r(θ)=m=141τm+(θ)>0 | The redundancy level r(θ) is defined as the number of muscles producing positive torque at pedal angle θ. The unique contribution um of muscle m is computed by integrating its positive torque (τm+) only over the angular regions where it is the sole contributing muscle, i.e. where r(θ)=1. |
|  | Unique-support contribution | um=∫1r(θ)=1 τm+(θ) dθ |  |
| 7 | Support inside the pre-risk region | pm=Θpre-riskτm+(θ) dθ | Measures the positive torque produced by muscle m inside the pre-risk angular region (90° before the common negative torque production). |
| 8 | Mechanical contribution metric | mcm=um+pm | Mechanical contribution metric mcm is based on unique-support contribution and muscle support inside the pre-risk region. |
| 9 | Muscle weight | ωm=mcm*fatm2 | Defines the final muscle weight by multiplying mechanical contribution metric mcm and muscle fatigability metric fatm, then normalizing it between 0 and 1. |

Figures

Figure S1: All (left) and muscle specific (right) pulse width charge across cycles for five quadratic cost functions and rehab (section 3), bayesiant optimization (section 4), and weight calculation (section 5). The translucent curves are the unfiltered pulse width and the full ones are filtered with a Savitzky-Golay filter (window length: 31 points, 3rd polynomial order, interpolation mode).

Figure S2: Bayesian optimization progress: number of completed cycles before failure with repect to iterations and corresponding weight updates. Decrease (red), increase (yellow), near optimal solutions (>95% cycle improvement in green), and best performance (dark green) compared to respective unweighted cost function.

Figure S3: Mean and standard deviation times to solve an OCP on the left and number of completed cycles on the right cycles based on the quadratic cost functions (color-coded), four window size used (color gradient), and three muscle weights: uniform ω=1 (section 3), optimal endurance weights for the cost function obtained by Bayesian optimization (section 4), and weights calculated from physiological and mechanical properties (section 5).

Figure S4: Muscle-specific fatigability, mechanical criticality, and normed weights for an arm cycling task.