Integrating Stocks, Collection and Treatment Pathways in an Urban Material Flow Analysis of Electrical and Electronic Equipment

Ana Outeirinho Morgado a* & Lucia Corsini a

a. Circular Economy & Sustainability Lab, Department of Engineering Science, University of Oxford, Oxford, UK

* corresponding author (ana.outeirinhomorgado@eng.ox.ac.uk)

Supplementary Information

Electrical and electronic equipment placed on the London market in 2024

Table 1 – Percentage of EEE POM by free riders.

| Category | Assumption 1 |
| --- | --- |
| 1. LHA | 0.00% |
| 2. SMA | 3.00% |
| 3. IT & Telecoms | 3.00% |
| 4. Consumer equipment | 12.50% |
| 5. Lighting equipment | 5.00% |
| 6. EEE tools | 12.50% |
| 7. Toys, leisure & sports equipment | 5.00% |
| 8. Medical devices | 0.00% |
| 9. Monitoring & control instruments | 0.00% |
| 10. Automatic dispensers | 0.00% |
| 11. Display equipment | 0.00% |
| 12. Cooling appliances | 0.00% |
| 13. Gas discharge lamps & LEDs | 20.00% |
| 14. PVs | 0.00% |

Collection of WEEE generated

Table 2 - Correspondence between the reported Waste Data Flow and the 14 UK WEEE categories for WEEE formally collected by Local Authorities.

| Category | Large Domestic Appliances | Small Domestic Appliances | TVs & Monitors | Fridges & Freezers | Fluorescent tubes & other light sources |
| --- | --- | --- | --- | --- | --- |
| 1. LHA | X | - | - | X | - |
| 2. SMA | - | X | - | - | - |
| 3. IT & Telecoms | - | X | - | - | - |
| 4. Consumer equipment | - | - | - | - | X |
| 5. Lighting equipment | - | - | - | - | X |
| 6. EEE tools | - | X | - | - | - |
| 7. Toys, leisure & sports equipment | - | X | - | - | - |
| 8. Medical devices | - | X | - | - | - |
| 9. Monitoring & control instruments | - | X | - | - | - |
| 10. Automatic dispensers | - | - | - | - | - |
| 11. Display equipment | - | - | X | - | - |
| 12. Cooling appliances | - | - | - | - | - |
| 13. Gas discharge lamps & LEDs | - | - | - | - | - |
| 14. PVs | - | - | - | - | - |

Table 3 – Percentage off WEEE fly-tipped per UK WEEE category found in the compositional analysis performed by a London Waste Authority.

| Category | % WEEE fly-tipped |
| --- | --- |
| 1. LHA | 0.31 |
| 2. SMA | 0.17 |
| 3. IT & Telecoms | 0.15 |
| 4. Consumer equipment | 0.28 |
| 5. Lighting equipment | 0.00 |
| 6. EEE tools | 0.00 |
| 7. Toys, leisure & sports equipment | 0.09 |
| 8. Medical devices | 0.00 |
| 9. Monitoring & control instruments | 0.00 |
| 10. Automatic dispensers | 0.00 |
| 11. Display equipment | 0.00 |
| 12. Cooling appliances | 0.00 |
| 13. Gas discharge lamps & LEDs | 0.00 |
| 14. PVs | 0.00 |

End-of-life (EoL)

Table 4 – Management of local authority collected waste in London, 2023 – 2024. 13

| End-of-life pathways | % |
| --- | --- |
| Recycling | 28.8 |
| Landfill | 0.1 |
| Incineration with energy recovery | 65.7 |
| Incineration without energy recovery | 0.0 |
| Other | 5.4 |

Data quality assessment

Table 5 – Data Quality Indicator (DQI) matrix for the present study.

| Score | Very good | Good | Fair | Poor | Very poor |
| --- | --- | --- | --- | --- | --- |
| Reliability of the source | Verified data based on measurements | Verified data partly based on assumptions or unverified data based on measurements | Non-verified data partly based on assumptions | Qualified estimate (e.g., by industrial expert) | Non-qualified estimate |
| Representative | Representative data from sufficient sample of sites over an adequate period to even out normal fluctuations | Representative data from a smaller number of sites but for adequate periods | Representative data from an adequate number of sites but from shorter periods | Representative data but from a smaller number of sites and shorter periods or incomplete data from an adequate number of sites and periods | Representativeness unknown or incomplete data from a smaller number of sites and/or from shorter periods |
| Temporal correlation | Less than three years of difference to year of study | Less than six years of difference | Less than 10 years of difference | Less than 15 years of difference | Age of data unknown or more than 15 years of difference |
| Geographical correlation | Data from area under study | Average data from larger area in which the area under study is included | Data from area with similar production conditions | Data from area with slightly similar production conditions | Data from unknown area or area with very different production conditions |
| Technological correlation | Data from enterprises, processes and materials under study | Data from processes and materials under study but from different enterprises | Data from processes and materials under study but from different technology | Data on related processes or materials but same technology | Data on related processes or materials but different technology |