Supplementary Information

Coupled Homogeneous-Heterogeneous Oxidation of Tetracycline by Natural Pyrite and H2O2: The Roles of Fe(II) Release and Sulfur Transformation

Runchang Sua,b, Yuhang Liua,b, Mei Zhanga,b, Min Guoa,b,

aState Key Laboratory of Advanced Metallurgy, University of Science and Technology Beijing, Beijing 100083, P. R. China

bSchool of Metallurgical and Ecological Engineering, University of Science and Technology Beijing, Beijing 100083, China

1. Chemical Composition and Microstructure of Natural Pyrite

Table S1 Chemical composition and content of natural pyrite by XRF (wt %)

| element | Fe | S | Si | Al | Ca | Zn | Pb | Cu | K | Mn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| content | 47.93 | 45.80 | 2.63 | 1.14 | 0.99 | 0.49 | 0.40 | 0.23 | 0.16 | 0.12 |

2. H2O2 Concentration and Absorbance Fitting Curve

Figure. S1. H2O2 concentration and absorbance fitting curve.

The concentration of H2O2 in aqueous solution was determined by titanium potassium oxalate spectrophotometry. The reaction principle is as follows. In the medium acidity of Ti4+ salt solution, adding H2O2 will generate orange complex ( [TiO(H2O2)2+] ). 68 mL of concentrated sulfuric acid was slowly poured into 75 mL of deionized water several times. 8.85 g of potassium titanium oxalate dihydrate was added to the obtained sulfuric acid solution, stirred to dissolve, and then the solution was diluted to 250 mL to obtain potassium titanium oxalate solution. 0.25 mL of potassium titanyl oxalate solution and 0.25 mL of different concentrations of H2O2 (0 ~ 70 mM ) solution were mixed in a 25 mL colorimetric tube and diluted to 25 mL. After reacting for 10 min, the absorbance of the mixed solution at λ = 400 nm was measured, and the relationship curve between H2O2 concentration and absorbance was drawn.

3. SEM Images of Pyrite Catalysts

Figure. S2. (a) XRD pattern of pyrite before and after five degrading cycles; SEM images of pyrite catalysts before (b, c) and after (d, f) tetracycline degrading cycles.

4 The Ion Concentration and Solution Phenomenon After Adsorption

Figure. S3. (a) Ion concentrations in the solution after 20 min adsorption, (b)The color change of solution when Fe3+ is tested by potassium thiocyanate.

5. DFT Calculation Method

5.1 Computational details of Fukui Function

In this study, the Fukui functions (f+、f- and f0) employed to predict local reactivity are rooted in conceptual density functional theory (CDFT) and were calculated using the Multiwfn. To eliminate the effects of geometric relaxation, all calculations were performed within the framework of the vertical approximation. Specifically, the single-point energy and wavefunction calculations for the N-1, N and N+1 systems strictly utilized the optimized geometric coordinates of the neutral tetracycline molecule. The wavefunctions of these three states, generated by ORCA, were subsequently imported into Multiwfn to compute both the 3D real-space Fukui function distributions and the condensed Fukui functions. The mathematical definitions of the three Fukui functions corresponding to nucleophilic, electrophilic, and radical attacks are expressed as follows:

(S1)

(S2)

(S3)

Where ρN+1、ρN and ρN-1 represent the electron density distributions of the anionic, neutral, and cationic systems, respectively. For 3D isosurface visualization, the spatial grid spacing was set to approximately 0.10 Bohr and rendered within the software. To obtain quantitative condensed atomic Fukui function values, the Hirshfeld population analysis scheme was adopted in Multiwfn to determine the net atomic charges under each electronic state [70,71].

5.2 Computational details of the bond dual descriptor

To quantitatively evaluate the intrinsic reactivity of specific chemical bonds within the tetracycline molecular skeleton and their susceptibility to radical attacks, the bond dual descriptor (BDD) was calculated utilizing the Multiwfn. As a further extension of CDFT, this descriptor substitutes traditional atomic charges with interatomic bond orders, thereby providing a more direct reflection of the dynamic variations in bond strength during electron transfer processes [72]. Within the framework of the vertical approximation, the calculations strictly utilized the optimized geometric coordinates of the neutral molecule to extract the wavefunctions of the N-1, N and N+1 states. The condensed Fukui functions of the chemical bond between atoms h and k were determined via the finite difference approximation:

(S4)

(S5)

(S6)

Accordingly, the BDD () is defined as the difference between the nucleophilic and electrophilic bond Fukui functions:

(S7)

Chemical bonds with negative BDD values (<0) possess strong intrinsic nucleophilicity and are highly vulnerable to electrophilic addition (e.g., by·OH or 1O2）). On the contrary, those characterized by positive BDD values (>0) display significant electrophilic traits, serving as priority sites for attacks by nucleophilic radicals like ·O2-.

5.3 Computational Details of Molecular Electrostatic Potential Analysis

In order to elucidate the molecular charge distribution, intermolecular electrostatic interactions, and susceptible reactive sites of tetracycline, molecular ESP mapping was conducted via Multiwfn [62]. The electrostatic potential V(r) at any given spatial point r, arising from the collective contributions of both atomic nuclei and electrons, is mathematically expressed as follows:

(S8)

Where  represents the nuclear charge of atom A located at position  and  denotes the electron density function of the molecule. Following the internationally accepted standard proposed by Bader et al., the electrostatic potential in this work was mapped onto the molecular van der Waals (vdW) surface, which is defined by a total electron density isosurface of 0.001 a.u.

Table S2 The Fukui function of tetracycline

| Atom | q(N) | q(N+1) | q(N-1) | f- | f+ | f0 | CDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1(C ) | 0.1307 | 0.0814 | 0.1304 | -0.0003 | 0.0492 | 0.0245 | 0.0495 |
| 10(C ) | -0.0644 | -0.0813 | 0.0016 | 0.066 | 0.0169 | 0.0415 | -0.0491 |
| 11(C ) | -0.0307 | -0.0674 | 0.0018 | 0.0325 | 0.0367 | 0.0346 | 0.0042 |
| 12(C ) | -0.0632 | -0.0856 | -0.015 | 0.0482 | 0.0224 | 0.0353 | -0.0258 |
| 13(C ) | 0.0933 | 0.0753 | 0.1341 | 0.0408 | 0.0179 | 0.0294 | -0.0229 |
| 14(C ) | -0.0463 | -0.0492 | -0.0142 | 0.0322 | 0.0029 | 0.0175 | -0.0293 |
| 15(C ) | 0.1275 | 0.0811 | 0.1349 | 0.0074 | 0.0464 | 0.0269 | 0.0391 |
| 16(C ) | -0.056 | -0.0756 | -0.0277 | 0.0283 | 0.0196 | 0.0239 | -0.0087 |
| 17(C ) | 0.0908 | 0.0633 | 0.1188 | 0.028 | 0.0275 | 0.0278 | -0.0005 |
| 18(C ) | 0.0713 | 0.0594 | 0.0746 | 0.0033 | 0.0118 | 0.0076 | 0.0085 |
| 19(C ) | 0.1718 | 0.0934 | 0.1823 | 0.0105 | 0.0784 | 0.0445 | 0.068 |
| 2(C ) | -0.1006 | -0.115 | -0.0958 | 0.0048 | 0.0144 | 0.0096 | 0.0096 |
| 20(O ) | -0.2486 | -0.3319 | -0.2241 | 0.0245 | 0.0833 | 0.0539 | 0.0588 |
| 21(N ) | 0.0056 | -0.0172 | 0.0124 | 0.0068 | 0.0228 | 0.0148 | 0.016 |
| 22(O ) | -0.2406 | -0.2927 | -0.218 | 0.0226 | 0.0521 | 0.0374 | 0.0295 |
| 23(O ) | -0.2225 | -0.2736 | -0.2083 | 0.0142 | 0.0511 | 0.0326 | 0.0369 |
| 24(C ) | -0.0934 | -0.0978 | -0.0882 | 0.0051 | 0.0044 | 0.0048 | -0.0007 |
| 25(O ) | -0.2146 | -0.2437 | -0.205 | 0.0096 | 0.0291 | 0.0194 | 0.0195 |
| 26(O ) | -0.1813 | -0.2073 | -0.1433 | 0.038 | 0.026 | 0.032 | -0.0121 |
| 27(O ) | -0.2095 | -0.2356 | -0.124 | 0.0855 | 0.0261 | 0.0558 | -0.0594 |
| 28(O ) | -0.2213 | -0.2317 | -0.213 | 0.0084 | 0.0104 | 0.0094 | 0.0021 |
| 29(O ) | -0.2553 | -0.2937 | -0.2451 | 0.0103 | 0.0384 | 0.0243 | 0.0282 |
| 3(C ) | 0.1302 | 0.1101 | 0.1311 | 0.0009 | 0.0201 | 0.0105 | 0.0192 |
| 30(N ) | -0.095 | -0.0973 | -0.0133 | 0.0817 | 0.0023 | 0.042 | -0.0793 |
| 31(C ) | -0.0417 | -0.0449 | -0.0256 | 0.0161 | 0.0032 | 0.0097 | -0.0129 |
| 32(C ) | -0.0378 | -0.0429 | -0.0206 | 0.0172 | 0.0051 | 0.0111 | -0.0121 |
| 33(H ) | 0.0309 | 0.0255 | 0.0363 | 0.0053 | 0.0054 | 0.0054 | 0.0001 |
| 34(H ) | 0.0275 | 0.0194 | 0.0357 | 0.0082 | 0.0082 | 0.0082 | 0 |
| 35(H ) | 0.0413 | 0.0337 | 0.0503 | 0.009 | 0.0076 | 0.0083 | -0.0014 |
| 36(H ) | 0.025 | 0.0151 | 0.0284 | 0.0035 | 0.0099 | 0.0067 | 0.0065 |
| 37(H ) | 0.0266 | 0.0204 | 0.0329 | 0.0062 | 0.0062 | 0.0062 | 0 |
| 38(H ) | 0.0362 | 0.0232 | 0.0622 | 0.026 | 0.013 | 0.0195 | -0.013 |
| 39(H ) | 0.0455 | 0.0251 | 0.0701 | 0.0246 | 0.0203 | 0.0225 | -0.0043 |
| 4(C ) | 0.0187 | 0.0147 | 0.0266 | 0.0079 | 0.004 | 0.006 | -0.0039 |

| Atom | q(N) | q(N+1) | q(N-1) | f- | f+ | f0 | CDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 40(H ) | 0.0472 | 0.0303 | 0.0738 | 0.0266 | 0.0169 | 0.0217 | -0.0096 |
| 41(H ) | 0.1774 | 0.1499 | 0.1855 | 0.008 | 0.0275 | 0.0178 | 0.0195 |
| 42(H ) | 0.1781 | 0.1465 | 0.1869 | 0.0088 | 0.0316 | 0.0202 | 0.0228 |
| 43(H ) | 0.0363 | 0.0353 | 0.0402 | 0.004 | 0.001 | 0.0025 | -0.003 |
| 44(H ) | 0.0352 | 0.0242 | 0.0442 | 0.009 | 0.011 | 0.01 | 0.0021 |
| 45(H ) | 0.0273 | 0.0204 | 0.0381 | 0.0108 | 0.007 | 0.0089 | -0.0039 |
| 46(H ) | 0.1689 | 0.1567 | 0.1777 | 0.0088 | 0.0122 | 0.0105 | 0.0034 |
| 47(H ) | 0.125 | 0.115 | 0.136 | 0.011 | 0.01 | 0.0105 | -0.001 |
| 48(H ) | 0.128 | 0.1198 | 0.1458 | 0.0179 | 0.0081 | 0.013 | -0.0097 |
| 49(H ) | 0.1546 | 0.1472 | 0.1641 | 0.0094 | 0.0074 | 0.0084 | -0.002 |
| 5(C ) | -0.026 | -0.028 | -0.0199 | 0.0061 | 0.002 | 0.0041 | -0.0041 |
| 50(H ) | 0.121 | 0.1088 | 0.1254 | 0.0044 | 0.0121 | 0.0083 | 0.0078 |
| 51(H ) | 0.0354 | 0.0233 | 0.0554 | 0.02 | 0.0121 | 0.016 | -0.0079 |
| 52(H ) | 0.0192 | 0.0153 | 0.0399 | 0.0207 | 0.0039 | 0.0123 | -0.0168 |
| 53(H ) | 0.0302 | 0.0305 | 0.0446 | 0.0144 | -0.0003 | 0.0071 | -0.0146 |
| 54(H ) | 0.0158 | 0.011 | 0.0378 | 0.0221 | 0.0048 | 0.0134 | -0.0173 |
| 55(H ) | 0.033 | 0.021 | 0.0548 | 0.0218 | 0.0121 | 0.0169 | -0.0097 |
| 56(H ) | 0.0372 | 0.0325 | 0.0525 | 0.0154 | 0.0047 | 0.01 | -0.0107 |
| 6(C ) | -0.0575 | -0.0622 | -0.0553 | 0.0022 | 0.0047 | 0.0035 | 0.0025 |
| 7(C ) | -0.0265 | -0.0288 | -0.0227 | 0.0039 | 0.0023 | 0.0031 | -0.0016 |
| 8(C ) | 0.0876 | 0.0863 | 0.0889 | 0.0012 | 0.0014 | 0.0013 | 0.0001 |
| 9(C ) | 0.0026 | -0.0113 | 0.0228 | 0.0202 | 0.0139 | 0.0171 | -0.0063 |

5. Intermediate Product Summary

Table S3 Tetracycline identification of intermediate product summary

|  | Reaction Time (min) | Accurate Mass [M+H]+ (Da) | Theoretical mass [M+H]+ (Da) | Molecular Formula [M+H]+ (Da) | Proposed Structures |
| --- | --- | --- | --- | --- | --- |
| 1 | 6.76 | 445.1593 | 445.1533 | C22H25N2O8 |  |
| 2 | 6.65 | 431.1436 | 431.1376 | C21H23N2O8 |  |
| 3 | 6.76 8.11 | 427.1491 | 427.1427 | C22H23N2O7 |  |
| 4 | 7.86 | 433.1593 | 433.1533 | C21H25N2O8 |  |
| 5 | 6.65 | 413.1322 | 413.1271 | C21H21N2O7 |  |
| 6 | 6.62 | 392.1322 | 392.1634 | C20H26NO7 |  |
| 7 | 15.01 | 371.1009 | 371.1053 | C20H19O7 |  |
| 8 | 7.42 | 219.1223 | 219.1307 | C14H19O2 |  |
| 9 | 7.76 | 241.1353 | 241.0998 | C12H17O5 |  |
| 10 | 6.13 | 321.1304 | 321.126 | C17H21O6 |  |
| 11 | 9.22 | 165.0909 | 165.0837 | C10H13O2 |  |
| 12 | 3.14 | 322.1607 | 322.1212 | C16H20N1O6 |  |
| 13 | 7.76 | 175.096 | 175.0681 | C11H11O2 |  |

*Corresponding author: Tel/fax: +86 10 62334926, E-mail address: guomin@ustb.edu.cn