Table 1: Outdoor radiofrequency electromagnetic field (RF-EMF) exposure levels (µW/m²) measured at four building orientations around ten schools in Graz, Austria. Measurements were performed using a broadband field meter (NBM 550, Narda) with an isotropic EF-0691 probe. Values are reported as minimum (Min), average (Avg), and maximum (Max) power density.

| School | Date | Orientation | Min (µW/m²) | Avg (µW/m²) | Max (µW/m²) |
| --- | --- | --- | --- | --- | --- |
| Mittelschule St. Andrä | Dec 09, 2025 | North | <106 | 1,962 | 7,847 |
|  |  | East | 1,020 | 7,047 | 45,903 |
|  |  | South | 363 | 2,008 | 15,922 |
|  |  | West | 586 | 4,906 | 20,353 |
| Modellschule Graz | Dec 09, 2025 | North | 344 | 1,453 | 6,706 |
|  |  | East | 468 | 3,569 | 10,716 |
|  |  | South | 1,375 | 8,786 | 29,414 |
|  |  | West | 289 | 1,655 | 9,276 |
| BRG Körösi | Dec 10, 2025 | North | <106 | 117 | 1,053 |
|  |  | East | <106 | 307 | 2,054 |
|  |  | South | <106 | 255 | 1,263 |
|  |  | West | <106 | 383 | 2,008 |
| BRG Carneri | Dec 10, 2025 | North | 45 | 923 | 2,547 |
|  |  | East | <106 | 923 | 6,209 |
|  |  | South | 403 | 2,924 | 10,190 |
|  |  | West | <106 | 446 | 2,344 |
| Praxismittelschule PH Steiermark | Dec 10, 2025 | North | 1,155 | 9,576 | 55,640 |
|  |  | East | 1,300 | 8,883 | 58,594 |
|  |  | South | 307 | 4,346 | 27,162 |
|  |  | West | 955 | 3,631 | 25,327 |
| BRG Lichtenfels | Dec 11, 2025 | North | 307 | 8,594 | 33,996 |
|  |  | East | 1,492 | 5,889 | 13,428 |
|  |  | South | <106 | 1,053 | 2,760 |
|  |  | West | <106 | 802 | 4,414 |
| HLW Schrödinger | Dec 11, 2025 | North | 179 | 1,414 | 10,610 |
|  |  | East | 514 | 3,037 | 17,248 |
|  |  | South | <106 | 892 | 6,622 |
|  |  | West | 1,300 | 12,955 | 40,552 |
| BRG Seebacher | Dec 11, 2025 | North | <106 | 1,227 | 12,955 |
|  |  | East | 1,698 | 6,622 | 21,094 |
|  |  | South | 2,760 | 14,524 | 47,911 |
|  |  | West | <106 | 446 | 5,889 |
| VS St. Leonhard | Dec 11, 2025 | North | 60 | 1,121 | 3,387 |
|  |  | East | <106 | 773 | 10,823 |
|  |  | South | 955 | 9,276 | 27,845 |
|  |  | West | <106 | 2,101 | 10,823 |
| BRG Klusemannstraße | Dec 11, 2025 | North | <106 | 1,492 | 18,069 |
|  |  | East | 10 | 923 | 21,094 |
|  |  | South | <106 | 1,655 | 23,873 |
|  |  | West | 166 | 2,008 | 9,475 |