Supplementary Information

False-Alarm-Controlled Evaluation of Gas Indices and Deep Learning for Early Detection of Coal Spontaneous Combustion

Ulas CINAR

Canakkale Onsekiz Mart University, Department of Occupational Health and Safety, Canakkale, Turkey. ulas.cinar@comu.edu.tr

This file contains Supplementary Tables S1 to S5 and Supplementary Figures S1 to S7. The field monitoring records are supplied separately as Supplementary Data S1.

Supplementary Tables

Supplementary Table S1. Representative studies on coal-mine fire and gas prediction, and the evaluation quantity each reports.

| Study | Method | Reported evaluation | Limitation addressed here |
| --- | --- | --- | --- |
| Singh et al. (2007) | Classical gas-ratio indices (Graham, Trickett, Willett) | Retrospective interpretation of sealed-fire status | No false-alarm rate is quantified; thresholds are applied post hoc to confirmed events |
| Onifade and Genc (2020) | Review of spontaneous-combustion prediction | Narrative synthesis | Notes fragmentation of the field but does not propose a common evaluation protocol |
| Danish and Onder (2020) | Mamdani fuzzy inference on CO, O2, N2, temperature | Agreement with observed fire intensity at one mine | Single site; no false-alarm accounting on non-fire periods |
| Zhang et al. (2022) | Machine learning for self-ignition tendency | Regression accuracy (R2) on laboratory data | Laboratory endpoint, not a time-resolved alarm decision |
| Qi et al. (2023) | ML prediction of borehole spontaneous combustion | Classification accuracy | Operating point fixed; no trade-off curve reported |
| Wang et al. (2024) | SSA-optimised CNN for combustion temperature | R2 / RMSE on temperature | Forecast error, not detection timeliness |
| Yang et al. (2024) | PSO-tuned LSTM for gas concentration | Forecast error metrics | No alarm decision or false-alarm rate |
| Shao et al. (2025) | ITOC-optimised CNN-BiGRU-CBAM | R2 = 0.974 for temperature | High regression accuracy does not establish early-warning value |
| Ding et al. (2025) | Bayesian-optimised LSTM | Temperature prediction error | Same as above; alarm behaviour unquantified |
| Xing et al. (2025) | SSA-optimised CNN-BiLSTM with attention | Multivariate forecast accuracy | Attention improves fit but is not linked to alarm performance |
| Zhao et al. (2025) | Interpretable stacked ensemble with SHAP | Accuracy plus feature attribution | Interpretability addressed; false-alarm trade-off not |
| This study | Matched false-alarm-rate benchmark of indices and Attention-LSTM models | Detection rate and lead time at a common false-alarm rate, with bootstrap CIs | Provides the missing common operating point for comparing methods |

Supplementary Table S2. Sampled parameter ranges of the simulation environment.

| Quantity | Symbol / units | Sampled range | Basis |
| --- | --- | --- | --- |
| Heat-generation scale | q | 0.19-0.34 (supercritical); 0.05-0.185 (subcritical) | Chosen to straddle the thermal-runaway bifurcation |
| Activation energy, self-heating | Ea (kJ mol⁻¹) | 42-46 | Low-temperature coal oxidation |
| Ventilation cooling coefficient | h (h⁻¹) | 0.024-0.030 | Convective heat removal |
| Incubation delay | t0 (h) | 150-950 | Variable onset within the monitoring window |
| Activation energy, CO / CO2 / ΔO2 | Ea (kJ mol⁻¹) | 48-60 / 36-48 / 32-44 | Species-specific yields; makes index ratios evolve with T |
| Activation energy, H2 / C2H4 | Ea (kJ mol⁻¹) | 80–90 / 96–108 | High-temperature indicator gases |
| Sensor relative noise | n/a | 0.16-12 % by channel | Independent per channel |
| Sensor drift amplitude | n/a | 0.2-3 % by channel | Slow random-walk drift |

Supplementary Table S3. Dataset partition. Splitting is by episode to prevent leakage between adjacent windows.

| Partition | Episodes | Windows | Normal | Suspected / Active |
| --- | --- | --- | --- | --- |
| Training | 147 | 15,582 | n/a | n/a |
| Validation | 31 | 3,286 | n/a | n/a |
| Test | 32 | 3,392 | 2,785 | 58 / 549 |
| Total | 210 | 22,260 | n/a | n/a |

Supplementary Table S4. Window-level classification performance on held-out simulated test episodes.

| Model / Class | Precision | Recall | F1 | Support | Accuracy |
| --- | --- | --- | --- | --- | --- |
| Physics-informed – Normal | 0.997 | 0.985 | 0.991 | 2,785 |  |
| Physics-informed – Suspected | 0.531 | 0.879 | 0.662 | 58 |  |
| Physics-informed – Active fire | 1.000 | 0.995 | 0.997 | 549 |  |
| Physics-informed – overall |  |  | 0.884 (macro) | 3,392 | 0.985 |
| Raw gases only – Normal | 0.995 | 0.987 | 0.991 | 2,785 |  |
| Raw gases only – Suspected | 0.542 | 0.776 | 0.638 | 58 |  |
| Raw gases only – Active fire | 1.000 | 0.995 | 0.997 | 549 |  |
| Raw gases only – overall |  |  | 0.876 (macro) | 3,392 | 0.985 |

Supplementary Table S5. Permutation importance for the physics-informed model.

| Channel | Type | Δ macro-F1 when permuted | Rank |
| --- | --- | --- | --- |
| CO | Measured gas | 0.245 | 1 |
| ΔO2 | Derived index | 0.193 | 2 |
| CO2 | Measured gas | 0.150 | 3 |
| H2 | Measured gas | 0.150 | 4 |
| CO/CO2 | Derived index | 0.147 | 5 |
| O2 | Measured gas | 0.135 | 6 |
| C2H4 | Measured gas | 0.054 | 7 |
| G1 | Derived index | 0.000 | 8 |
| TR | Derived index | −0.002 | 9 |
| CH4 | Measured gas | −0.009 | 10 |

Supplementary Figures

Supplementary Figure S1. A representative simulated test episode: latent coal temperature, carbon monoxide and methane, and the oxygen-carbon dioxide response.

Supplementary Figure S2. Index trajectories for the episode of Supplementary Fig. S1. The Jones-Trickett panel is shaded by the conventional fuel-identification bands.

Supplementary Figure S3. Correlation structure of the simulated test set. Channels are physically coupled but not degenerate.

Supplementary Figure S4. Permutation importance. The two classical severity ratios contribute nothing measurable once their constituent gases are present.

Supplementary Figure S5. Learned temporal attention profile by true class, averaged over simulated test windows.

Supplementary Figure S6. Simulation: detection rate with 95 % bootstrap confidence intervals (a) and median lead time (b) at a matched false-alarm rate of 0.10.

Supplementary Figure S7. Field: detection rate with 95 % bootstrap confidence intervals (a) and median lead time (b) at a matched false-alarm rate of 0.10.