Supplementary Table S1. MRI acquisition parameters.

| MRI acquisition parameters |  |
| --- | --- |
| MRI system | 3.0-T Siemens Prisma (Erlangen, Germany) |
| Receive coil | 64-channel head/neck coil |
| Resting-state fMRI |  |
| Sequence | Blood oxygen level-dependent gradient-echo echo-planar imaging (EPI) |
| Repetition time (TR) | 2,000 ms |
| Echo time (TE) | 30 ms |
| Matrix | 64 × 64 |
| Field of view (FOV) | 230 × 230 mm² |
| Flip angle | 90° |
| Slice number | 37 |
| Slice thickness | 3.6 mm |
| Slice gap | 0.36 mm |
| Acquired voxel size | 3.6 × 3.6 × 3.6 mm³ |
| Diffusion MRI |  |
| Sequence | Diffusion tensor imaging echo-planar imaging (EPI) |
| Repetition time (TR) | 5,000 ms |
| Echo time (TE) | 69 ms |
| Matrix | 128 × 128 |
| Field of view (FOV) | 224 × 224 mm² |
| Flip angle | 90° |
| Slice number | 35 |
| Slice thickness | 3.5 mm |
| Slice gap | 0.70 mm |
| Acquired voxel size | 1.75 × 1.75 × 3.5 mm³ |
| b values | 0 and 1,000 s/mm² |
| Diffusion-weighted volumes | 90 at b = 1,000 s/mm² |
| b0 volumes | 12 |
| Structural MRI |  |
| Sequence | Three-dimensional T1-weighted magnetization-prepared rapid gradient-echo (MPRAGE) |
| Repetition time (TR) | 2,200 ms |
| Echo time (TE) | 2.48 ms |
| Inversion time (TI) | 900 ms |
| Matrix | 256 × 256 |
| Field of view (FOV) | 230 × 230 mm² |
| Flip angle | 8° |
| Slice thickness | 1.0 mm |
| Voxel size | 0.9 × 0.9 × 1.0 mm³ |