# Supplementary Information for

# Energy demand reduction: a key policy action to achieve rapid national decarbonisation while maintaining quality of life

Authors and affiliations

John Barrett, Steve Pye, Sam Betts-Davies, Oliver Broad, James Price, Nick Eyre, Jillian Anable, Christian Brand, George Bennett, Rachel Carr-Whitworth, Alice Garvey, Jannik Giesekam, Greg Marsden, Jonathan Norman, Tadj Oreszczyn, Paul Ruyssevelt, Kate Scott

Correspondence to: j.r.barrett@leeds.ac.uk

Table of Contents

SI 1 – UK Energy Demand	2

SI 2 - Relevant analysis of energy demand scenarios at the global and national level	4

SI 3 – Scenario narrative	8

SI 4 – Sectoral modelling assumptions	15

SI 5 – Carbon budget modelling assumptions	29

References	30

## SI 1 – UK Energy Demand

The UK Government currently has no comprehensive plan on how to reduce the UK’s energy demand. A comprehensive plan would include a clear projection of energy demand and a sense on how individual strategies and policies would reduce this demand over time. Without a reduction in energy demand, the burden is placed on an even more rapid roll out of renewables to meet high levels of energy demand as well as the reliance on technologies yet to be fully demonstrated e.g. carbon capture and storage.

| a) | b) |
| --- | --- |

Supplementary Figure 1: a) Total energy demand and b) by sector in the UK (1970-2018) (Source: BEIS, 2020)

In the last 50 years, the UK’s energy demand has reduced by 2%1. In reality, there has been virtually no reduction in energy demand in the UK for 50 years. As can be seen in Supplementary Figure 1, there have been increases and falls in the total energy demand over this time period. Energy demand was at its high point in 2001 (10% higher than 1971), since then there has been a decline with 2018 being 11% lower than in 2001. However, in recent years these reductions have ceased, with no notable reduction in UK energy demand for the past 6 years.

This has been driven by opposing effects in the demand for energy services and the efficiency with which they are delivered. In the last 30 years, energy efficiency improvements have made the largest contribution to reducing UK emissions, exceeding the combined effects of renewable energy and fuel switching from coal to gas 2.

The limited changes in total UK energy demand hide some significant changes in its sectoral composition. Energy demand for transport has doubled in the last 50 years while the population has only increased by 20%. Residential energy demand has increased by 11% over the same period while the number of households has increased by 47%, although this hides the fact that that residential energy demand has reduced by 16% from its highest point in 2004. However, the reductions in residential energy demand have also slowed in recent years as energy efficiency programmes have weakened.

Energy demand for industry has reduced by 64% over the past 50 years. While some of this reduction can be attributed to energy efficiency improvements, the key driving factor is the shift in the UK towards a service-based economy 3. Data provided by the University of Leeds to the Department of Environment, Food and Rural Affairs (Defra) demonstrates that the UK’s demand for energy to satisfy UK consumption has increased, not reduced. However, the energy demand now occurs predominately outside the UK and is imported in the form of materials and products.

Future projections of UK energy demand undertaken by BEIS suggest that energy demand will remain reasonably constant up to 2040, with their reference scenario showing a 2% decline over the next 20 years 4. The BEIS analysis also suggests that known policies will have no significant effect of the overall level of UK energy demand. The analysis by BEIS refers to know policies, rather than future policy aims or targets.

## SI 2 - Relevant analysis of energy demand scenarios at the global and national level

This section identifies studies that have explored the potential mitigation contribution of reducing energy demand. The following literature review highlights that whilst there is a good understanding of mechanisms through which energy demand can be reduced and the potential range of impact this could have upon future global cumulative GHG emissions, there remains a limited picture of how energy demand reduction could be implemented at the national level, or the contribution it could make towards meeting national climate targets.

### 2.1 Energy Efficiency-led scenarios

The LED scenario developed by Grubler et al.5 is a frequently cited study that explores the potential contribution of global reductions in energy demand to greenhouse gas mitigation. Operating at the global level, it projects that energy demand could be 40% lower than today by 2050, based on a bottom-up analyses of changes in activity levels, energy intensity and final demand of end-use energy services. Its inclusion in the IPCC’s Special Report on 1.5oC 6 offered a contrasting perspective to other 1.5oC aligned scenarios, as the only scenario that did not increase energy demand. By reducing global energy demand, the reliance on speculative carbon dioxide removal  measures to achieve ambitious carbon budgets was diminished7. Furthermore, Grubler et al.5 evidence that reducing the size of the global energy system makes the task of transitioning to a low-carbon supply much easier to achieve, as low carbon energy technologies can assume a larger share of the total energy mix.

The IEA’s Energy Technology Perspectives 20208 report, included energy demand reducing measures within its ‘Sustainable Development Scenario’, in the form of exploiting energy efficiency improvements present in technologies. Broadly speaking, the scenario covers two groups of efficiency improvements, both of which are included in our UK focused LED scenario, adopting technologies that carry with them reductions in energy intensity, and material efficiency that aims to reduce energy service demand by reducing the material inputs needed to produce goods and services. Similarly, to Grubler et al. 5, it is argued that energy efficiency measures that reduce overall energy use are crucial to decarbonisation of energy systems, as it reduces the ‘resource constraint’ problem with the roll out of low-carbon technology8.

Brugger et al.9 assess the potential for overarching societal trends to have an impact on energy efficiency and final energy demand at the EU level. They identify 12 societal trends that may have a future impact on the success of energy efficiency policies and future energy demand, both positive and negative. These societal trends relate to digitalisation, new social and economic models, industrial transformation, and changes in quality of life. In a best-case scenario, trends such as increased consumer awareness, urbanisation and smaller space living reduce energy demand by 67% compared with an EU baseline projection of final energy demand. Conversely, their worst-case scenario, sees new societal trends contributing to a 40% increase in energy consumption through increased efficiency rebounds. They suggest this wide range of impacts displays that energy efficiency gains do not by themselves lead to energy demand reduction, highlighting the need for strong policy frameworks that ensure energy efficiency gains have the desired effect 9.

### 2.2 Changes to social practices, lifestyles, and behaviour

The literature also includes scenarios that explore the extent that energy demand could be reduced through changes to how society consumes energy services. There are various terms used to describe this mechanism for demand reduction, such as ‘lifestyle change’10–13, ‘behaviour change’8,14  or ‘social change’ 5,15,16.

Van Vuuren et al.12 explore global scenarios consistent with limiting warming to 1.5oC that minimise the reliance on carbon dioxide removal strategies. Their scenario includes significant energy demand reduction through ‘lifestyle change’ including reduced meat consumption, modal shift in mobility service provision, changes in heating preferences and the avoidance of consumption of household appliances. Whilst on their own, these changes are found to be unable to match the pace of mitigation required, as part of a broader mitigation pathway including energy efficiency measures and shifts to low-carbon energy production, demand reduction measures significantly reduce the need for carbon dioxide removal in limiting warming to 1.5oC.

Eyre et al.10 develop a scenario of lifestyle change in the UK built upon increasing concerns about energy use and its environmental implications. They suggest that lifestyle change impacts service demands mostly in residential and transportation energy services and have the potential to reduce national energy use and carbon emissions by 35% and 30% respectively. They conclude that such a shift would reduce the cost of transitioning the energy supply to low carbon alternatives by £70 billion, concurring with others in the literature 5,8 that energy demand reduction helps to facilitate supply-side transitions.

The Social Transformation Scenario developed by Kuhnhenn et al.16 is the most transformative scenario in terms of reducing energy demand. It does so by reducing the demand for high-carbon goods and services, focussing on high-carbon freight and passenger transport, heating, energy consuming appliances, and food. Energy demand in countries in the global north is significantly reduced through a strong decline in overall consumption levels, creating room for increased necessary consumption in the global south, whilst reducing global energy demand and staying within a global carbon budget consistent with 1.5oC of warming. They argue that the global north has a responsibility to dramatically reduce their consumption of high-carbon goods and services, and abandon the pursuit of economic growth, to facilitate the dual aim of redistributing wealth within nations and internationally, whilst achieving mitigation rates consistent with limiting warming to 1.5oC.

Keyßer and Lenzen17 assess global 1.5 oC scenarios that include a general reduction in economic output in the global north, due to strong climate mitigation. They compare these ‘degrowth’ mitigation scenarios to IPCC ‘archetype scenarios’ that assume ever expanding growth in economic output and energy demand. Whilst the political feasibility of degrowth is questioned, they suggest a reduction in final demand levels reduces the reliance on decoupling GDP from energy use at unprecedented rates, the development of unpredictable  carbon dioxide removal technologies, and the unmatched speed of technological change necessary to decarbonise current the energy supply 17.

Ivanova et al.15 evaluate the extent that various sustainable energy service consumption options led to genuine GHG mitigation, across food, housing, transport, and other consumption. They find that their top 10 most effective consumption options have the potential to mitigate 9.2tCO2eq/cap, compared to the high-carbon alternative consumption option. They suggest this indicates an untapped area of mitigation that goes beyond discussions of the efficiency of production, to consider the nature of and scale of consumption in relation to planetary boundaries and satisfying human needs.

Modelling social change using integrated assessment models is a key challenge discussed widely in the literature, with various approaches being taken 11,14,18. Van Sluisveld et al.11 insert ‘lifestyle change’ measures into an integrated assessment model, by changing key model parameters based on estimates established in the literature. Whilst this simple method does not directly capture agent decision making in the model, it facilitates a quick estimation of the scale of reductions given lifestyle changes could facilitate. Niamir et al.14 address this flaw, integrating an agent-based modelling framework into a computable general equilibrium model. This facilitates the scaling up of agent-based models of heterogeneous individuals to a larger level, capturing regional, social-demographic, and structural differences in individual’s decision making when adopting low-carbon decision making. Whilst these modelling approaches and methods used throughout the literature represent significant improvements in the methodologies to model social change-led energy demand reduction and GHG mitigation, the need for further model development is a key suggestion for further research throughout the literature.

### 2.3 Sectoral energy demand reduction

Some of the literature explores energy demand reduction scenarios in individual sectors with significantly high embodied energy such as construction 19, agriculture 20,21, transport 22 and other hard to mitigate sectors such as aviation, shipping, freight, and industry 18. Mata et al.19 investigate energy demand reduction in the building sector, through an exploration of sector roadmaps for net-zero around the world. They find that most roadmaps are focused on efficiency measures and technology-based upgrades to the housing stock, with limited focus on ‘lifestyle’ or ‘behavioural’ changes 19. Brand et al.23 explore four contrasting futures that compare transport-related ‘lifestyle’ changes and socio-cultural factors against a transition pathway focussing on transport electrification and the phasing out of conventionally fuelled vehicles. They find that lifestyle change alone can have a comparable and earlier effect on transport carbon and air quality emissions than a transition to electric vehicles with no lifestyle change.

Sharmina et al.18 identify key demand reduction options in four critical ‘hard to reach’ sectors: aviation, shipping, road freight and industry. They highlight that currently, integrated assessment models are unable to capture many demand side mitigation opportunities within their price elasticity demand mechanisms, such as modal shift for aviation, slow stemming for shipping, localised production reducing demand for freight transport, and circular economy measures in industry. Khalili et al.22 investigate scenarios of future global transport energy demand, mapping the shift away from fossil fuelled transport. They suggest that the efficiency improvements accompanying low carbon fuel switching and electrification could offset the projected increase in global demand for passenger and freight transport, and this increased demand can be managed by a stable final energy demand in 2050, compared to 2015.

The literature that centres reductions in energy demand to support transitions to low-carbon energy supply offer a crucially important alternative to those dominant scenarios relying solely on a supply-side transition and carbon dioxide removal. Given the pace and urgency needed to successfully mitigate the worst impacts of the climate crisis and the uncertain capacity of carbon dioxide removal measures to mop up remaining emissions, reducing final energy demand in countries where it is high is likely to be crucial to achieve the goals of the Paris Agreement.

However, to date, energy demand reduction scenarios have operated at a multi-national or global level, whilst mitigation targets and climate policies are devised at a national level, through NDCs and net-zero commitments. This presents a gap in the energy demand reduction scenario literature that has direct policy relevance. This demand for tailored national-level evidence highlighting the capacity of energy demand reductions as a mitigation solution, is present in the Climate Change Committee’s (CCC) Sixth Carbon Budget24, including energy and material efficiency measures, as well as changes to social practices including diet switching and modal shifting in transport. To further support national demand side climate mitigation policy, research is needed to develop cohesive frameworks for nations to operationalise energy demand reduction measures given unique national contexts. This research aims to fill this gap by developing a tailor-made low energy demand scenario for the UK.

## SI 3 – Scenario narrative

Historically, strategies to reduce energy demand have focused largely on improving energy efficiency over options to reduce demand for energy services or to change the way that services are delivered. This unnecessarily limits options whilst also failing to consider the underlying drivers of energy demand and the connection between energy efficiency and the economy. Rebound effects created by energy efficiency improvements have been shown to occur a) directly, with lower prices allowing for increased energy consumption, b) indirectly, with increased consumption of other energy services, and c) on an economy-wide scale, by stimulating economic growth 25. As there are also limits to the technical potential of efficiency improvements, particularly in energy intensive industrial processes 26, it is important to consider other demand side strategies alongside energy efficiency.

Creutzig et al.'s "Demand-side assessment framework"27, adopted by the IPCC, can be used to identify a broader range of strategies to reduce energy demand. Through this framework, we can avoid unnecessary energy services (e.g. reducing the need to travel), shift to the lowest intensity mode to deliver a service (e.g. modal shift from car to public or active transport), and improve energy efficiency. Each of these strategies can be applied to a range of energy services.

Before quantifying the energy demand for the individual sectors, we developed a scenario narrative to ensure consistency across our two LED scenarios, similar to the approach adopted in Grubler et al.5. This shared vision is applied across all of the different energy using sectors. They include a number of principles that ensure that our scenario is both transformative but also within the realms of possibility. What is “possible” is, of course, very difficult if not impossible to define. Therefore, the set of principles are broad enough to take into account that predicting technologies, social practices and behaviours in 2050 is highly problematic. It is important to remember that we are creating possible futures in the form of scenarios and not a prediction of the future.

Whilst there are many observable societal, political, economic, scientific or philosophical trends that may impact transitions to low carbon societies, this section identifies seven observable underlying trends that have impacted upon energy demand, and/or are likely to continue to do so throughout the scenario timescale. This list is not exhaustive, as various other trends may impact on energy demand, such as demographic changes. These seven trends are captured in figure 2. We recognise that there is considerable crossover between the various trends. We also recognise that some of these trends have the potential to both increase and reduce further energy demand 9.

Supplementary Figure 2. Seven underlying trends impacting on energy demand

### 3.1 Digitalisation

In the context of our scenario, digitalisation is the integration of digital systems and ICT into the energy system. It is already driving significant changes in the energy sector and is likely to accelerate change in the future by promoting new energy business models through to changing how consumers interact with energy services 28. Integrating ICT into the household and workplace energy systems and supply infrastructures will improve the capturing and use of energy system data. This will enable energy service providers to better understand consumption patterns, and more efficiently meet user’s needs 5. The benefits of digitalisation also extend to consumers. Increased use of smartphones in managing energy services extend control and interactivity to the consumer, opening the potential for increased efficiency of consumption of energy in the household. Further, digitalisation has the potential to substitute material goods with digital equivalents, such as books or music 29.

However, it is worth noting that increased digitalisation will not necessarily bring about a reduction in energy demand. Lange et al.30 identify four impacts of digitalisation on energy demand. Whilst digitalisation was found to facilitate efficiency increases with the potential to reduce energy demand, they highlight the potential for rebound effects. These include the potential to foster economic growth and the embodied energy within the production of ICT, that may both bring about increases in demand 30. This potential rebound effect is reiterated by others in the literature 29,31. In the mobility services context, Noussan & Tagliapietra 31 suggest that avoiding increased energy demand due to rebound effects requires policies that ensure optimized and shared use of energy services and technologies. Thus, whilst digitalisation has the potential to improve user information and control, find efficiency savings in the supply and use of energy, support sustainable sharing business models, and improve quality of life, active policy is likely needed to ensure these amount to a reduction in energy demand.

In the LED scenarios developed in this study, digitalisation is applied to a range of energy services to reduce demand, such as energy management systems to extend control and avoid unnecessary consumption and mobility services to promote shared travel and reduce private consumption.

### 3.2 Sharing and circular economy

The ‘sharing economy’ is an approach that aims to decrease the number of under-utilised ‘owned’ assets in an economy, by creating new business models that offer a service in its place 5. Decreasing the overall demand for under-utilised products reduces the energy demanded for their production. A prominent example of the potential of the shared economy is in mobility. Shared vehicles such as car clubs or cycle hire offer the ability to rent vehicles as and when they are needed, replacing private ownership 32. For example, car clubs have been found to reduce the private ownership of vehicles by 10.5 cars for every car club car in use 33. A second strategy, shared trips, increases utilisation by filling empty seats with passengers who would otherwise have travelled alone (such as those facilitated by ride sharing websites such as Liftshare or BlaBlaCar) 32. Sharing or use-based strategies that increase utilisation can be applied to other areas such as use of office buildings or shared use of consumer goods 5. Mobility sharing platforms, office sharing & hot-desking and shared consumer electronics and appliances are all areas where shared economy approaches are applied in the LED scenarios developed in this study. It is worth nothing that in some sectors, there are trends towards less sharing, for example increased life expectancy and divorce rates have resulted in lower building occupancy, increasing heating and appliance use per person.

The sharing economy is closely linked to the concept of “circular economy”. Circular economy is a concept that explores resource efficiency strategies to extend the time that resources are retained in the economy, to reduce material throughput and environmental impacts 26,34,35. The term has become popular in policy discourses, with the European Commission and UK Government exploring legislation that promote circular economy strategies 36,37. Circular economy approaches aim to reduce the flows of virgin material entering the system. These include a broad scope of strategies including: eco-design and production (ensuring waste materials can be recovered throughout the lifecycle), circular consumption (repairing and reuse aimed at increasing the use life of products), and developing new business models that increase the utilisation of products5,38. These circular economy approaches have the potential to reduce the energy demanded to extract and produce virgin material, and lower the demand for new products through increased use-life.

### 3.3 Energy efficiency

Despite an improvement of 1% annually in energy efficiency over the last 30 years 2, there is no indication that the potential for energy efficiency is anywhere near saturated. Globally, the overall conversion efficiency of primary energy into useful services is estimated to be approximately 15% 39,40. Whilst the economic potential is lower, historically innovation has tended to increase the cost effective potential at a similar rate to its uptake, so that a 20-30% economic potential has existed for many decades 41.

The LED scenarios assume realistic levels of continued energy efficiency gains. For example, we assume further efficiency gains in many appliances, but also recognise this requires a substantial shift in technology in some cases. In other cases, notably building retrofit, the potential is large with existing technology. The principal constraints are usually supply chain practices and consumer engagement rather than technological innovation. We recognise the role of information technology as an enabler of reduced energy demand and the continuing innovation of such a fast-moving industry. The increased digitalisation of society offers new opportunities to control energy demand through increased functionality (sensors, wireless controls, etc.) in buildings, transport and industry.

The most important driver of future energy efficiency improvement is likely to be the wider energy transition. As energy supply shifts progressively from fossil fuels to primary electricity, not only are the conversion losses in thermal electricity generation avoided, but more efficient end use is enabled. In particular, there are huge potential benefits in the electrification of the two largest end uses of energy – building heating and light vehicles. In each case, a factor of three in energy efficiency improvement can be achieved relatively straightforwardly, through replacing boilers by heat pumps and internal combustion engine vehicles with electric vehicles. In the longer term, similar effects are likely in a number of industrial processes, including steel and ammonia. These effects alone can reduce UK final energy demand by 30%42 and constitute the main driver of energy demand reduction in our scenarios.

3.4 Healthy society

It has been estimated that a scenario which meets the Paris Agreement and explicitly takes steps to benefit health via reduced air pollution, improved diet and active travel can result in 144,312 avoided UK deaths in 2040 compared to existing NDCs43.

The scenarios developed in this study build upon an underlying trend of an increasing focus on health, wellbeing and quality of life, at the policy level and by individuals. The context of the COVID-19 pandemic brings this attention on healthy environments and lifestyles into sharp focus. As a result, both of the low energy demand scenarios developed ensure there is not a reduction in health, wellbeing or quality of life. This is possible because there is a significant overlap and interdependence between policy and behavioural trends that both seek to improve health and reduce demand. In the case of nutrition, almost half of those switching to less energy intensive, plant-based diets, cite health reasons as their primary motivation for doing so44. Similarly, reducing average calorific intake would also improve national health, given the high prevalence of being overweight and obesity amongst UK citizens45. With regards to mobility services, significantly increasing the amount of active travel (cycling, walking) used to make shorter journeys, both reduces energy demand and improves physical and mental health46. Moreover, shifting private car journeys to shared and active transport helps to reduce negative health impacts associated with inactive lifestyles and exposure to toxic air pollution in cities47–49. For homes, providing well-insulated homes reduce the costs for the millions of families living in the poverty in the UK to achieve an appropriate internal temperature. This is strongly linked to healthy living. Reduced NOx and particulate emissions associated with EVs and HPs also have health benefits50. The scenarios developed here thus assume that health and wellbeing will become increasingly important to both individuals and policy makers in light of the global pandemic. This societal priority is reflected in the significant wellbeing benefits of the scenarios developed.

### 3.5 Increasing environmental awareness

As climate and ecological breakdown accelerates and significant climate impacts of events such as wildfires and flooding gain global attention, public concern for the environment grows across the globe. In the UK context, in January 2020 (before the COVID19 Pandemic), the environment ranked in the top 3 issues facing the country for over 30% of the British public51. This high point of environmental awareness largely correlates with the notable rise in high profile climate activism during 201951. Whilst during the COVID-19 pandemic, public concern relative to other issues has fallen, a quarter of views captured still suggest the environment is a significant issue. As captured by opinion polling and academic studies, at the beginning of the last decade environmental awareness and concern about the climate was relatively low, suggesting increasing environmental concern is a significant trend51,52.

Increasing environmental awareness and concern about the climate crisis is already having impacts on the demand for energy services. For example, in nutrition, there is a significant trend towards reducing meat consumption in diets. Whilst the empirical data on national spread of diets is poor, several market surveys have suggested the number of vegetarians has vastly increased in the last 5 years44,53,54. Some estimate that up to 33.5% of the population are reducing or cutting out meat from their diets 44, although this has yet to materialise in reduced UK meat consumption55. Further, Waitrose44 report that for 38% of those going vegan or vegetarian, concern for the environment is the most significant motivation. There is further evidence that this trend may continue into the future. Bryant56 highlights that even amongst meat-eaters, over 70% have a positive view of the environmental benefits of a plant-based diet. This evidence suggests that increasing environmental concern can lead to significant changes in social practices and demand for energy services.

In our LED scenarios it is assumed that concern over the environment and promotion of appropriate actions to reduce energy demand, as well as concerns regarding health, quality of life and energy use continue to drive social change. Given the inevitable worsening impacts of the climate crisis, and the already increasing trend of environmental awareness, social norms are assumed to shift to favour sustainable consumption of energy services.

### 3.6 Globalisation

Increasing globalisation in the form of international production networks and global value chains has a wide range of impacts on national final energy demand in different countries 57. Studies focussing on the environmental impacts of globalisation have indicated that around 25% of global CO2 emissions are embodied in global trade flows 58. The energy intensity of globalised supply chains highlights a potential conflict between increased globalisation in this sense, and reductions in global energy demand. Of these emissions transferred between countries in trade flows, most are embodied in products consumed by ‘developed’ countries59–61. This indicates that most developed countries, such as the UK, are net-importers of emissions and embodied energy in the products they consume, a trend that is growing over time 60.

In the UK context, final energy consumption declined by 11% between 2001 and 2013 3. However, Hardt et al.3 contextualise this trend with respect to the ongoing structural change occurring in the UK economy. They suggest that the most significant contributor to energy savings made through structural change are as a result of offshoring energy intensive production, available due to cost-effective global production networks and value chains 3. This finding presents a distinction between reductions in domestic final energy demand (and territorial GHG emissions) that make a genuine contribution to mitigating climate change, and instances where reductions in domestic final energy demand (and territorial emissions) are caused by offshoring emissions to other regions, thus failing to contribute to climate change mitigation. Understanding how globalisation can impact upon domestic and global energy demand is therefore important to ensure energy demand and emissions are not exported elsewhere. While our analysis of energy demand is from a territorial perspective, the LED scenarios developed here reflect genuine energy demand reductions that do not increase reliance on increased imports of highly energy-intensive products from abroad.

### 3.7 Work and automation

There are many overlaps between trends involving the increased digitalisation, as discussed previously, and automation of social and economic activity, such as the integration of machine learning and artificial intelligence into energy service provision 28. However, this trend considers the distinct impact that automation may have on working patterns in the UK, and how this may change the demand for energy services. As Graeber (2013) recounts, in 1930, John Maynard Keynes predicted that early industrialised wealthy countries such as the UK or USA would be working 10 – 15 hours in an average working week by the beginning of the 21st century due to significant advances in the productive capacity of technology. Technologically speaking, this prediction is not too far removed from reality. Several studies have aimed to assess how many jobs could be replaced by automation. (Frey and Osborne, 2013) indicate that 47% of jobs in the US are at a high risk of being displaced by automation. In the UK context, several studies have suggested that 30% of UK jobs were vulnerable to automation, whilst the ONS suggests 7.4% of jobs are at high risk of automation, with 65% of jobs at a medium risk 64,65.

So whilst the productive capacity of autonomous technology can reduce the need for labour, average full time working week over the past 20 years remain stable at around 38 hours per week 66. The stability of working time suggests that increasing integration of autonomous technology is not having a significant impact on working time. Whilst the stability of average hours worked is the result of multiple factors, it is partially explained by the need for increasing labour productivity to underpin growth-based business and economic models 67. This indicates that if prioritised, it is feasible that working less could be supported by increasing automation 68.

## SI 4 – Sectoral modelling assumptions

### 4.1 Linking the scenario narrative to the sector modelling

See additional file of table 1:

Supplementary Table 1: Mapping of scenario drivers across sectors

### 4.2 Sectoral modelling of low energy demand scenarios

We outline in more detail how the above underlying trends translate into the two LED scenarios for the UK. These are labelled “Shift demand” and “Transform demand”. The Shift demand scenario describes a significant shift in the attention given to energy demand strategies providing an ambitious programme of interventions across all of our five sectors. It describes what could possibly be achieved with existing technologies and current social and political framings. At the same time the scenario still considers changing social-economic factors such as the changing nature of retail, businesses becoming increasingly accountable for their emissions, localisation and increased public acceptance to pay for environmental costs.

The Transform demand scenario reflects a much more transformative future, where more significant reductions are realised but under which quality of life is enhanced. To achieve substantial reductions in energy demand, decisions should be made by recognising the need for social-technical transitions. Energy efficiency alone is not the only driver available to affect energy demand. Our scenarios reinforce the CCC’s 6th carbon budget, that suggests ‘behavioural’ and ‘technical’ solutions are imperative to meeting the UK’s climate ambition (CCC, 2020). Relying on technical solutions alone is insufficiently rapid and risky, and that policies influencing the demand for energy services (e.g. transport) should have a more prominent role.

Energy demand is shaped by cultural norms, values, preferences and structural factors 27. These practices are malleable and change over time leading to very different outcomes in relation to the UK’s demand for energy.  Recent shifts in attitudes related to climate change and the declaration of climate emergencies demonstrate the possibility for rapid change meaning that there is a danger that scenarios constrained by current thinking can be outdated and irrelevant. To overcome this problem, this scenario attempts to consider transformative change in technologies, social practices, infrastructure and institutions to achieve a greater level of energy demand reduction beyond the “Shift Demand” scenario.

A particular focus of the two LED scenarios is to consider strategies that both reduce energy demand but also maximise other social and economic benefits. These include healthy diets, active living, improved local air quality, quality housing and lowering the costs of the net-zero transition. The scenario sets outs to demonstrate the benefits of recognising the many co-benefits associated with role of energy demand reduction.

In the rest of this section, we outline how we have considered the different narrative drivers in both LED scenarios. The modelling approaches used for each of the sector analyses are listed in Table 2.

Supplementary Table 2. Modelling approaches used for each of the sectors

| Sector | Modelling approach |
| --- | --- |
| Mobility | TEAM-UK (Transport Energy Air pollution Model for the UK) |
| Nutrition (incl. agriculture) | Hybrid UK MRIO (Multi-Regional Input Output Model) |
| Shelter (domestic buildings) | UK National Household Model (NHM) |
| Non-domestic buildings | Bespoke Model |
| Materials and products (industry) | Hybrid UK MRIO (Multi-Regional Input Output Model) |

### 4.3 Mobility

In the UK, road transport accounted for just under three quarters of transport energy consumption in the UK in 2019, with the remainder almost entirely from air travel (24%). Energy use in railways (2%) and shipping (2%) were relatively minor. Of the road component, energy use from cars accounts for more than half (61%), with most of the remainder coming from ‘light duty vehicles’ (vans) (17%), heavy goods vehicles (HGVs) (18%) and buses (3%). Energy use from transport has increased by 16% since 1990 (6% since 2013) against a UK economy-wide decrease of 4% and remains 98% dependent on fossil fuels.

There has been little to no focus on reducing distance travelled; instead the focus has been first to Ultra Low Emission Vehicles (ULEVs), and then to zero-emission vehicles (ZEVs), primarily through electrification. However, the energy efficiency improvements have been more than lost mainly due to a continued swing towards larger passenger cars. This almost universal focus on improving energy consumption per passenger-km or tonne-km travelled ignores the other two core elements of the Avoid-Shift-Improve hierarchy.

The focus of the LED scenarios is to reduce energy demand for transport. This will be achieved through the 3-pronged approach of reducing the need for energy services at the same time as improving efficiency (e.g. speed limits, EVs) and using efficient decarbonised supply of energy. The need to travel will be reduced through better land-use planning, restrictions on car use in central, residential, and environmentally sensitive locations, and facilitating transfer of car trips to public transport, walking and cycling by reallocation of expenditures, street design, pricing and regulation. This allows for a policy perspective where reduced energy use does not run counter to quality of life but arises from measures designed to enhance it. Evidence suggests a lower rate of demand for passenger mobility is a necessary and a credible future, but that this would require a different policy package to ‘scale up’ and ‘lock in’ the new demand patterns, alongside new vehicle technology (Brand et al., 2019).

Key assumptions for both LED scenarios include:

- No more substantial new road building or airport capacity expansion; some roads repurposed for shared, public and active mobility. No more development on greenfield sites

- Integrated transport authorities in all urban/city regions (One network; One timetable; One ticket)

- Doubling investment in public transport, walking and cycling with the construction of high-quality cycling networks of segregated cycleways in all urban areas

- Single occupancy car use becoming socially unacceptable and parking charges and infrastructure designed to encourage vehicle sharing. High taxation on more than one car per household

- Eco-levy applied to the whole system – the more you travel and the more polluting modes you use, the more you pay – includes air travel (frequent flier levy)

- Car fleet is reduced substantially as driving licence uptake is down with transition to ‘car usership’

- But taxi and shared fleets increase – all electric by 2030

- Increase in LCV (van) fleet due to more online shopping – electric only sold from 2030

- Large and heavy ICE, PHEV and HEV cars gradually phased out by 2030 and a substantially expanded bus fleet will be largely electric.  Big investment in and standardisation of charging infrastructure across the nation

- HGV – renewed push for consolidation centres around big cities and towns – reduced miles travelled

- Road freight – much improved logistics, vertical integration e.g. Amazon – improves load factors for long and medium distance freight

- No significant shift from road to rail freight as rail capacity is largely taken up by net passenger rail increases (leisure up more than commuting and business down)

Additional assumptions specific to the Transform demand scenario include:

- The phase out of ICE, PHEV and HEV cars is brought forward to 2025.

- Less demand for aviation driven by an increased public awareness of the environmental damage, higher costs for frequent flying, and increased fuel costs through taxation.

- Introduction of a four-day working week due to a greater focus on quality of life resulting in a 10% reduction in commuting trips per person by 2030 and further reductions by 2050

- Increased reduction in commuting due to working at home or teleworking where industrial restructuring allows greater flexibility

- Greater reliance on video-conferencing in businesses improving work-life balance

- Lower car ownership levels, particularly in urban areas, in line with the “mobility” assumptions reducing the need to travel and a shift towards public transport, e-micro mobility and shared mobility

### 4.4 Nutrition

The CCC estimates that 11% of UK GHG emissions are attributable to agriculture and land use, and predict that the sector will become a major emitter in 2050 70. The food and drink industry also represents 7% of the UK’s industrial GHG emissions 71. As the ultimate driver of agricultural and related emissions, we consider how changes to the quantity and type of food demand could contribute to a UK low energy demand pathway by 2050.

We consider 3 key options for constraining energy demand in the sector, which cover each stage of the UK food system, and address the major current sources of inefficiency and emissions-intensity. The food system presents particular challenges in considering energy demand, given that non-energy emissions relating to land use change (LUC) and livestock (ruminant enteric fermentation) are more significant in the sector.

We consider the effect of calorific intakes being brought in line with Government Dietary Recommendations (i.e. healthy levels), and in reducing supply chain food waste. Calorific intake is the determinant of the volume of food demand per capita. The UK is challenged by overconsumption, with approximately 65% of the population of England estimated to be in ‘overweight’ or ‘obese’ BMI groups as of 2017 72.

We also consider the role of reducing food waste and losses across the supply chain; this is a key public policy target, as cited in the recent Resources and Waste Strategy73. It is also a means of avoiding additional food demand, by making more efficient use of the food that is currently delivered through the UK food system. As indicated by Bajželj et al.74, food waste reduction is more effective downstream (i.e. closer to households) given the embodied energy demand of the product by that stage. This is therefore one of our core energy demand reduction strategies in the analysis.

We consider the role of dietary transitions to more sustainable food products. Approximately 18% of global GHG emissions are attributable to livestock production, with ruminants posing the largest single anthropogenic methane source, and occupying 25% of global land for grazing 75. This scenario envisages a shift to plant-based diets as an extension of the recent trend for reduced meat consumption. From 2014 to 2018, the number of vegans in the UK is reported to have increased by 450,00076. Of this number, 42% of vegans were reported to have switched to this diet during 2018 (ibid). Finally, we consider the potential for greater agricultural and industrial efficiency, primarily through new or best practice technologies.

- Our analysis is based on the detailed assessment provided by Garvey et al21.

Key assumptions for the Shift demand scenario include:

- Calorific intake reduces from the current average of 3,154 calories for a UK adult to 2686 calories by 2050 thus improving health related issues associated with obesity.

- A reduction in the number of omnivores in the UK population from 66.5% today to 17% by 2050

- An increase in the number of people adopting our “healthy diet”21 from 21% to 27% of the UK population

- Percentage of vegetarians increases from 9.5% today to 36% by 2050

- Percentage of the UK population adopting a plant-based diet increases from 3% to 20%

- An annual reduction in avoidable food waste of 2.5% a year up to 2050

Additional assumptions specific to the Transform demand scenario include:

- Further reduction in calorific intake to ensure that a healthy diet is taken up by all UK adults giving an average daily calorific intake of 2,500 calories as recommend by Public Health England

- An increase in vegetarians by 2050 where they represent 42% of the UK population.

- An increase in plant-based diets by 2050 where they represent 25% of the UK population

- An increase in the annual reduction of food waste to 3.33% per annum

Key drivers of these scenarios:

- New technologies would allow optimisation within the UK food supply chain to reduce food waste and deliver goods more efficiently. Trends towards centralised retail and distribution (i.e. warehouse retailing and home delivery, without supermarkets) could contribute to energy demand reduction in the food sector, whilst creating new business structures for food retail.

- Nutrition tracking technologies at the consumer end also have a role in moderating calorie intake and promoting healthier diets. Information provision around embodied energy/emissions in food could also stimulate behaviour change in food buying.

- Apps and other IT-enabled services currently allow redistribution of excess food, reducing food waste.

- Greater demand for plant-based goods and vegetable proteins would create a market space and potential lower price for such products

### 4.5 Heat in Domestic Buildings

Residential buildings in the UK rely predominantly on heat produced from burning natural gas in boiler-based systems that are popular because they are well-known, considered responsive, reliable quiet and relatively cheap. As a result, in 2019, this sector was responsible for 69.2 Mt of direct carbon dioxide emissions 77. This represented 15.2% of total national GHG emissions in that year, a share which has been increasing steadily since the 1990s – a relative increase due largely to the faster rates of decarbonisation seen in the power and industrial sectors as compared to residential buildings.

In parallel, the relatively poor thermal state of the housing stock in the UK has been the focus of frequent commentary with some stating that the country houses some of the worst performing buildings in Europe in efficiency terms 78. As a result, 84% of all the energy used in the residential sector goes to producing space and water heat, a share which has only decreased by two percent points since 1990 in part because the level and volume of homes heated has increased. Three quarters of this energy is used for space heating alone.

Reducing energy use in this sector has been the focus of national policy since 1970.  Successful strategies have included regulating the use of efficient boilers and windows as well as new buildings, incentivising cavity and roof insulation, and efficient appliances (particularly lighting) plus rolling out supplier obligations to reduce the carbon content of services delivered, particularly to vulnerable households. While these have incentivised energy efficiency, the rate has not always kept pace with increased demand for domestic energy services in part because this sector has been particularly impacted by stop-start policies which have demotivated the domestic energy efficiency refurbishment sector and not encouraged economies of scale. A recent report by the UK Energy Research Centre (UKERC) put this in starker terms noting that the UK needed to upgrade around 19 thousand homes per week, compared to 3.8 thousand seen in 2018/19; at current rates, this report suggests that the target, set out by the CCC, would take more than 700 years to reach. 79. The UK government has set a goal of installing 600,000 heat pumps per annum by 2028 from the current base of 30,00080. However, the policy instruments to deliver this have yet to be defined.

To include the residential sector, the LED scenarios use the UK National Household Model (NHM) to develop a detailed analysis of how the current and future housing stock can develop and what they can achieve. The NHM was developed and is currently still run by the Department for Business, Energy and Industrial Strategy (BEIS) and is an open- source housing stock model that simulates the energy performance of buildings under different sets of technical and building use assumptions.

The measures included in each of the LED scenarios explore incremental levels of technical ambition applied across the housing stock. They look at faster, deeper, and more widespread roll-out of building fabric retrofits; consider more ambitious heat pump, hybrid heat, and solar hot water programmes; and assume different requirement levels for new build housing apply as energy and infrastructure use in other sectors change.

By simulating these changes the NHM derives energy requirements across the building stock for space heating and hot water. These are aggregated to into building categories that fit the descriptions used in the whole systems model. Their change over time then form the basis for the annual growth rates that define the change in energy demand that the whole energy system model sees.

Key assumptions include:

- Fabric efficiency sees accelerated roll-out of building retrofits for existing stock reaching 250 k and 1 m measures per year in the HA and QL cases respectively.

- These changes are combined with increasing numbers of clean and highly efficient air and ground-source heat pumps. Hybrid systems forgo natural gas and rely instead on cleaner hydrogen or syngas. Gas boilers are phased out of existing buildings and excluded from new dwellings.

- As the scenarios explores deeper changes to society, the way in which we interact with and consider our residential buildings changes, for example

  - The share of households practicing homeworking increases at a low, but steady pace. The QL future goes further and considers a corresponding increase in a 4-day working week practice.

  - New build dwelling construction, while maintained in the HA future, is reduced and replaced by repurposing non-domestic building space freed up by lower retail and office space needs.

  - Finally, smart systems helps to change our building heating habits, maintaining quality of life while delivering heat only where and when it is needed.

### 4.6 Non-domestic buildings

The non-domestic sector includes a complex mix of buildings with a wide variety of different uses, from purpose built retail and commercial spaces, through to storage and refrigeration, hospitality health, education, or public services. Over time, this patchwork sector has shifted, in emissions terms, in both importance and composition. Total GHG emissions have grown from 26 Mt CO2e (3.2% of UK territorial emissions) in 1990 to 30 Mt CO2e (6.7% of total) in 2019 81. In composition terms, the share of HFC emissions from cooling and refrigeration has progressed to represent 36% of the sector total; while a 40% reduction in public sector building emissions has left commercial and business buildings responsible for about 74% of total emissions in 2019.

In both the LED scenarios, the energy demand reductions take a two-pronged approach. On the one hand, they result from varying levels of ambition in rolling out energy efficiency measures detailed across the existing and future sector building stock. On the other, they rely on changes to the overall growth in non-domestic stock itself. The latter results from changes in the wider economy, e.g. reduced retail and office space requirements due to a move to online retail and homeworking, and lead to changes in growth expectations in non-domestic floor space.

The energy efficiency measure role out is based on assessing overall technical potential, using data from Building Energy Efficiency Survey (BEES) 82. This approach considers sector requirements in both electrical and natural gas, and assesses the potential for 13 measures to reduce overall sector demand when applied across the relevant building stock at a sub-sector level. These measures include energy management systems, building retrofit, building system control, ventilation and cooling, and more efficient appliances.

On floor space, the Transform demand scenario sees growth in storage counteracted by a reduction in office space leading total floor space to remain constant. Some of the reduction in office space assumes that a proportion of commercial units are retrofitted to flats for residential occupancy. Most other non-domestic building types remain flat, apart from limited growth in education and health in line with population. The Shift scenario sees almost a 40% increase in floorspace; despite some reductions in office, storage growth is much more pronounced, leading to an overall increase. This compares to just over 50% growth in the Reference case.

Key assumptions include:

- Varying speeds in BEES measure implementation with rapid, and very rapid, uptakes for both LED scenarios, relative to slower adoption in the Reference case.

- This means full deployment of all efficiency potential across most measures by 2040, with rapid roll-out during the 2020s.

- Building fabric assumed to realise full potential by 2050, with stronger roll out across the 2020s and 2030s.

- Switching to improved energy using devices for heating means large-scale switching to electricity, with the use of heat pumps (determined in UKTM)

- Floor space increases by only 2% in 2050, relative to 2020 in the most ambitious low energy case. This is largely due to reduction in office space counteracting an increase in storage facilities.

### 4.7 Lighting and appliances in domestic and non-domestic settings

Electrical appliances, cooking and lighting account for 19% of energy consumption in UK homes 83. While efficiency improvements in lighting and appliances have led to a fall in electricity demand in recent years, this is expected to be counteracted by increasing electrification, particularly of heat 84. Most existing research focuses on efficiency improvements as the main method of energy demand reduction in appliances. Scenarios tend to assume that the variety and usage of appliances will increase to 2050, however, in our Transform scenario, we consider the possibilities to reduce energy service demands where there is no reduction in quality of life.

Therefore, as well as considering energy efficiency we also considered the impacts of behavioural change to use appliances more efficiently and less often, and the potential to reduce the number of appliances per household without compromising quality of life. In some areas like cooking and refrigeration there is limited capacity to avoid energy consumption as these appliances deliver an essential service and most households only own one of each type of appliance. However, in other areas like lighting and consumer electronics, it is possible to reduce the amount an appliance is used without impacting quality of life. New technologies such as smart meters and disaggregated electricity consumption data can also play a role in helping individuals reduce unnecessary consumption 85,86.

Energy efficiency improvements in appliances have had the largest impact on energy demand in recent decades. Without efficiency improvements, energy consumption in buildings would be 12% higher than current levels 87. However, there is still a large potential for further efficiency savings which have not been realised. Therefore, to ‘improve’ consumption, we considered the impact of widespread adoption of the best available technologies in new appliances, and the potential for future technological advancements.

Key assumptions include:

- Gas hobs and ovens are phased out by 2035 replaced with electrical appliances. 10% efficiency saving are available in electric hobs and ovens, which are achieved by 2030 88.

- In lighting, incandescent sales are phased out by 2025, and are out of use by 2027. Fluorescent sales are phased out by 2030, and are out of use by 2035. We also assume that there are 5% efficiency improvements still available in LED technology, which are achieved by 2025 89.

- Best available technologies are adopted across other appliances, resulting in efficiency savings of 45-60% in cold and wet appliances, 65% in consumer electronics, and 50% in air conditioning units 5,87.

Additional assumptions specific to the Transform demand scenario include:

- The widespread adoption of smart meters with disaggregated and normative electricity consumption feedback reduces household electricity consumption by 5%, excluding clod appliances.

- A 5% reduction is achieved in air-conditioning units from improved cleaning of coils 90.

- A 20% reduction in energy use from ICT and other appliances through replacing manual with automated building management controls 5.

- The total number of appliances per household is reduced where by 20% in lighting and 10% in ICT and consumer electronics due to demand saturation, cloud computing and increased sharing.

### 4.8 Materials and products

This energy service category refers to strategies that aim to reduce the energy demand embodied in the production of materials and products by UK industry. We focus on the material input and demand for clothing and textiles, packaging, vehicles, electronics, appliances and machinery, furniture and buildings and infrastructure. In 2019, emissions attributed to UK industry total 102 MtCO2e, representing 22% of UK GHG emissions 91. GHG emissions from UK industry have declined significantly, falling by 53% when compared with levels in 1990 91. Some of these savings can be attributed to reductions in industrial energy use resulting from energy efficiency savings, however, most of these reductions are explained by the offshoring of the production of goods previously made in the UK 3. The energy demand reductions in this scenario look to reduce industrial energy use embodied in the production of materials and goods without offshoring this to other countries. The analysis focuses on UK action. Due to the international trade of materials and products additional reductions would be seen if similar strategies were followed by overseas consumers of UK manufactured materials and products.

We consider three key options for reducing the energy demanded in the production of materials and goods: resource efficiency, energy efficiency, and changes to construction’s demand for materials.  Resource efficiency strategies reduce the required output of materials and products. Energy efficiency strategies reduce the energy demand to produce each unit of output. Additionally, changes in construction demand are an important driver of material production and associated energy demand.

Resource efficiency options were based on an existing review of opportunities for the UK 35. This is the widest existing assessment of such options for the UK and has been used extensively by (amongst others) the Climate Change Committee in informing the potential for resource efficiency within their carbon budget analysis 24 and the UK government’s recent Industrial Decarbonisation Strategy 92. The resource efficiency opportunities related to construction and nutrition from the earlier analysis of Scott et al.35 were not included in the current assessment as each is analysed independently here.

Energy efficiency measures in the LED scenarios are based on the existing technology options within the UK TM model (see section 3.6), which are based upon a review of technological options at the level of industrial subsectors in the UK Energy Research Centre Usable Energy Database 93. The sectors represented in UK TM, covered by this analysis are iron and steel, non-ferrous metals, cement, non-metallic minerals, chemicals (ammonia), chemicals (high value chemicals), chemicals (other), paper and others. UK TM was used to select efficiency measures for each sector in a cost optimal manner to reach the emissions targets of each scenario.

The construction sector is the largest user of materials and the largest producer of waste by tonnage in the UK 94. The sector is the principal consumer of a sizeable proportion of industrial output, particularly of key highly energy intensive materials, such as cement and steel. There is potential to reduce demand for production of these materials (and the corresponding energy demand) by changing national demand for new buildings and infrastructure; and how these assets are designed and delivered. In this analysis we derive future estimates of demand for key construction materials based upon scenarios of future demand for buildings and infrastructure; and the uptake of a range of mitigation measures that may reduce the quantity of new materials needed to service this demand. Whilst many of the measures undertaken to reduce demand in the construction sector fall into the categories of resource and energy efficiency, they were treated separately given the significant contribution of construction to total energy demand, and constructions interrelationships with other sectors. To do so, a literature review of over 60 academic publications and industrial case studies was undertaken to identify mitigation options, which were then analysed using a construction-specific dedicated model. The bespoke model considers 36 specific applications of key materials across 17 different built asset categories.

Key Assumptions include:

Energy Efficiency

- Improvements to current processes and replacement technologies (including fuel switching) are selected at the UK TM subsector level, to meet emissions targets in a cost optimal manner.

Resource Efficiency

- In the reference case industrial output was taken from UK TM (which is aligned with the BEIS Industrial Pathways Model, 2017).

- Forty four resource efficiency strategies were applied across clothing and textiles, packaging, vehicles, electronics, appliances and machinery and furniture. The Transform scenario sees the adoption of maximum technical potential of these strategies in 2032, the Shift scenario represents a 66% adoption over the same time period. This aligns with the medium and high scenarios of Scott et al.35

- The full supply chain impacts of these strategies, applied within the UK, on the industrial subsectors of UK TM was determined using a multi-regional input-output modelling approach. A logistic model was used to extrapolate these impacts to other time periods.

Construction

- Baseline future demand for new infrastructure construction to 2050 was taken from projections within the Green Construction Board’s Low Carbon Route map for the Built Environment

- Demand for new construction under the Shift and Transform scenarios was primarily determined by assumptions developed within the analyses of other energy service sectors (e.g. under the Transform scenario for Transport it is assumed that no new road building takes place, therefore output for the asset sub-category of ‘Roads’ is set to zero in the corresponding construction sector model scenario).

- 21 additional mitigation options to reduce material demand from construction were adopted across specific applications and asset categories based on the maximum practically achievable deployment in 2050, with a linear trajectory to this year.

### 4.9 Mapping dependencies between sectors

A critical part of the sectoral modelling process was to map the dependencies between sectors, to ensure consistency so that key aspects of the narrative represented in one sector were also reflected in others. Figure 2 shows the main dependencies between sectors.  Many flow from the shelter analysis (shown by the blue arrows), where shifts in working patterns fundamentally change type and patterns of mobility demand, and the use of non-domestic buildings. Differences in house build assumptions are also reflected in the demands for construction materials. Choices about transport infrastructure also impact on the demand for construction materials, while changes in vehicle sales impact on manufacturing. Changes to diet also impact the level of land take, which feeds into land availability for forestry. It also changes the level of output in specific food and drink subsectors, impacting energy demand in that sector.

Supplementary Figure 2. Sector analysis and cross-sectoral linkages

## SI 5 – Carbon budget modelling assumptions

As well as the “net-zero” target for GHG emissions by 2050, the UK Government also outlines a number of carbon budgets that represent the maximum level of GHG emissions over a given 5 year period. This is to recognise the fact that it is the total cumulative emissions that link to CO2 concentrations in the atmosphere. Therefore, the UK doe have a total defined carbon budget from 2021 to 2050 however the five yearly carbon budget have been defined up to 2037. The generate a total carbon budget up to 2050 we assume a linear pathway from 2037 to 2050 when net-zero GHG emissions is achieved.

In addition to this, the Steer demand and two LED cases, Shift and Transform, were also run with a cumulative carbon budget consistent with the targeted temperature rise and responsibility sharing justice principles of the Paris Agreement was developed. Determining a single national carbon budget that can be defined as the UK’s obligation to the Paris Agreement is not possible. When dividing a global carbon budget between countries there are a number of assumptions that could be taken based on historical contribution to GHG emissions, capacity to change, per capita allocations and other equitable consideration. Added to this, deriving Paris compliant national carbon budgets come with significant scientific uncertainty ranges.

This enables a wide range of UK carbon budgets to be considered ‘aligned’ with Paris. The carbon budget applied above of 4.95 GtCO2 was calculated by taking into account the UK’s carbon budgets which are currently defined up to 2042 followed by a linear trajectory to achieve zero emissions by 2050. However, as discussed, this is not the only possible method to derive the UK’s remaining carbon budget.

With this in mind, we present a different carbon budget for comparison. The value of this is to ascertain whether the two LED scenarios offer the opportunity to increase the climate ambition of the UK.

The additional carbon budget chosen in this study was derived using a resource sharing approach as developed by Anderson et al.95. It allocates larger shares in remaining carbon emissions to ‘developing’ countries to facilitate necessary economic development, before apportioning the remaining budget to developed nations using a grandfathering method. The UK’s budget, derived by this method is 3.87 GtCO2. Our additional carbon budget is 22% lower than the budget derived from the UK’s current planned budgets. This budget is an ambitious carbon budget for the UK. It was chosen based on its robust internalisation of the principle of common but differentiated responsibility and respective capacity at the heart of the Paris Agreement. This assigns the UK, and other wealthy early-industrialised emissions sooner and faster than those who have contributed less to the climate crisis. In the modelled scenarios, this budget was implemented from 2020-2050, with net-zero emissions also having to be met by 2050.

## References

1.	BEIS. Energy Consumption in the UK 1970 to 2019. https://www.gov.uk/government/collections/digest-of-uk-energy-statistics-dukes (2020).

2.	Lees, E. & Eyre, N. Thirty years of climate mitigation: lessons from the 1989 options appraisal for the UK. Energy Effic. 14, (2021).

3.	Hardt, L. et al. Untangling the drivers of energy reduction in the UK productive sectors: Efficiency or offshoring? Appl. Energy 223, 124–133 (2018).

4.	BEIS. Updated energy and emissions projections 2019. www.gov.uk/government/publications (2020).

5.	Grubler, A. et al. A low energy demand scenario for meeting the 1.5 °c target and sustainable development goals without negative emission technologies. Nat. Energy 3, 515–527 (2018).

6.	IPCC. Summary for Policymakers. in Global Warming of 1.5°C. An IPCC Special Report on the impacts of global warming of 1.5°C above pre-industrial levels and related global greenhouse gas emission pathways, in the context of strengthening the global response to the threat of climate change, 1–24 (In Press, 2018). doi:10.1016/j.oneear.2019.10.025.

7.	Rogelj, J. et al. Mitigation Pathways Compatible with 1.5°C in the Context of Sustainable Development. IPCC An IPCC Spec. Rep. impacts Glob. Warm. 1.5°C above pre-industrial levels Relat. Glob. Greenh. gas Emiss. pathways, Context Strength. Glob. response to Threat Clim. Chang. Sustain. develo 82 (2018).

8.	International Energy Agency. Energy Technology Perspectives 2020. Energy Technology Perspectives 2020 www.iea.org/t&c/ (2020) doi:10.1787/ab43a9a5-en.

9.	Brugger, H., Eichhammer, W., Mikova, N. & Dönitz, E. Energy Efficiency Vision 2050: How will new societal trends influence future energy demand in the European countries? Energy Policy 152, (2021).

10.	Eyre, N., Brand, C., Layberry, R., Anable, J. & Strachan, N. Energy Lifestyles. in ENERGY 2050: Making the transition to a secure and low-carbon energy system: Synthesis report (eds. Ekins, P. & Skea, J.) 103–118 (UKERC, 2009).

11.	van Sluisveld, M. A. E., Martínez, S. H., Daioglou, V. & van Vuuren, D. P. Exploring the implications of lifestyle change in 2°C mitigation scenarios using the IMAGE integrated assessment model. Technol. Forecast. Soc. Change 102, 309–319 (2016).

12.	Van Vuuren, D. P. et al. Alternative pathways to the 1.5 °c target reduce the need for negative emission technologies. Nat. Clim. Chang. 8, 391–397 (2018).

13.	Institute for Global Environmental Strategies, Aalto University,  and D. ltd. 2019. 1.5-Degree Lifestyles: Targets and options for reducing lifestyle carbon footprints. (2019).

14.	Niamir, L., Ivanova, O. & Filatova, T. Economy-wide impacts of behavioral climate change mitigation: Linking agent-based and computable general equilibrium models. Environ. Model. Softw. 134, 104839 (2020).

15.	Ivanova, D. et al. Quantifying the potential for climate change mitigation of consumption options. Environmental Research Letters vol. 15 093001 (2020).

16.	Kuhnhenn, K., Costa, L., Mahnke, E., Schneider, L. & Lange, S. A Societal Transformation Scenario for Staying Below 1.5°C. https://www.boell.de/en/2020/12/09/societal-transformation-scenario-staying-below-15degc (2020).

17.	Keyßer, L. T. & Lenzen, M. 1.5 °C degrowth scenarios suggest the need for new mitigation pathways. Nat. Commun. 12, (2021).

18.	Sharmina, M. et al. Decarbonising the critical sectors of aviation, shipping, road freight and industry to limit warming to 1.5–2°C. Clim. Policy 21, 455–474 (2021).

19.	Mata et al. A map of roadmaps for zero and low energy and carbon buildings worldwide. Environmental Research Letters vol. 15 113003 (2020).

20.	Poore, J. & Nemecek, T. Reducing food’s environmental impacts through producers and consumers. Science (80-. ). 360, 987–992 (2018).

21.	Garvey, A., Norman, J. B., Owen, A. & Barrett, J. Towards net zero nutrition: The contribution of demand-side change to mitigating UK food emissions. J. Clean. Prod. 290, 125672 (2021).

22.	Khalili, S., Rantanen, E., Bogdanov, D. & Breyer, C. Global Transportation Demand Development with Impacts on the Energy Demand and Greenhouse Gas Emissions in a Climate-Constrained World. Energies 12, 3870 (2019).

23.	Brand, C., Anable, J. & Morton, C. Lifestyle, efficiency and limits: modelling transport energy and emissions using a socio-technical approach. Energy Effic. 12, 187–207 (2019).

24.	CCC. Sixth Carbon Budget: The UK’s Path to Net Zero. https://www.theccc.org.uk/publication/sixth-carbon-budget/ (2020).

25.	Sakai, M., Brockway, P. E., Barrett, J. R. & Taylor, P. G. Thermodynamic Efficiency Gains and their Role as a Key ‘Engine of Economic Growth’. Supplementary Materials: The UK MAcroeconometric Resource COnsumption Model (MARCO-UK). Energies 12, (2019).

26.	Cooper, S. J. G. et al. Thermodynamic insights and assessment of the ‘circular economy’’’. J. Clean. Prod. 162, 1356–1367 (2017).

27.	Creutzig, F. et al. Towards demand-side solutions for mitigating climate change. Nature Climate Change vol. 8 268–271 (2018).

28.	Rhodes, A. Digitalisation of Energy: An Energy Futures Lab Briefling Paper. Imp. Coll. London (2020).

29.	Court, V. & Sorrell, S. Digitalisation of goods: A systematic review of the determinants and magnitude of the impacts on energy consumption. Environ. Res. Lett. 15, 043001 (2020).

30.	Lange, S., Pohl, J. & Santarius, T. Digitalization and energy consumption. Does ICT reduce energy demand? Ecol. Econ. 176, 106760 (2020).

31.	Noussan, M. & Tagliapietra, S. The effect of digitalization in the energy consumption of passenger transport: An analysis of future scenarios for Europe. J. Clean. Prod. 258, 120926 (2020).

32.	Marsden, G., Anable, J., Bray, J., Seagriff, E. & Spurling, N. Shared mobility-where now, where next? Second report of the Commission on Travel Demand About the Commission on Travel Demand Shared Mobility Inquiry. https://www.creds.ac.uk/wp-content/uploads/CREDS-Shared-mobility-comm-report-WEB.pdf (2019).

33.	Carplus. Carplus Annual Survey of Car Clubs 2016/17. https://como.org.uk/wp-content/uploads/2018/06/Carplus-Annual-Survey-of-Car-Clubs-2016-17-London.pdf (2016).

34.	Hahladakis, J. N., Iacovidou, E. & Gerassimidou, S. Plastic waste in a circular economy. in Plastic Waste and Recycling 481–512 (Elsevier, 2020). doi:10.1016/b978-0-12-817880-5.00019-0.

35.	Scott, K., Giesekam, J., Barrett, J. & Owen, A. Bridging the climate mitigation gap with economy-wide material productivity. J. Ind. Ecol. 23, 918–931 (2019).

36.	DEFRA. Prevention is better than cure The role of waste prevention in moving to a more resource efficient economy. www.nationalarchives.gov.uk/doc/open-government- (2013).

37.	European Commission. Closing the Loop - an EU action plan for the circular economy. Communication to the European Parliament (2015).

38.	Cooper, S. J. G. et al. Thermodynamic insights and assessment of the ‘circular economy’. J. Clean. Prod. 162, 1356–1367 (2017).

39.	Cullen, J. M., Allwood, J. M. & Borgstein, E. H. Reducing energy demand: What are the practical limits? Environ. Sci. Technol. 45, 1711–1718 (2011).

40.	TWI2050. Transformations to Achieve the Sustainable Development Goals - Report prepared by The World in 2050 initiative. International Institute for Applied Systems Analysis http://pure.iiasa.ac.at/15347 (2018) doi:10.22022/TNT/07-2018.15347.

41.	National Academy of Sciences, National Academy of Engineering & National Research Council. Real Prospects for Energy Efficiency in the United States. Real Prospects for Energy Efficiency in the United States https://doi.org/10.17226/12621 (2010) doi:10.17226/12621.

42.	Eyre, N. & Fawcett, T. Policy for Energy Demand Reduction. (2020).

43.	Hamilton, I. et al. The public health implications of the Paris Agreement: a modelling study. Lancet Planet. Heal. 5, e74–e83 (2021).

44.	Waitrose & Partners. Food and Drink Report 2018-2019. https://www.waitrose.com/content/dam/waitrose/Inspiration/Waitrose & Partners Food and Drink Report 2018.pdf (2019).

45.	NHS Digital. Part 3: Adult overweight and obesity - NHS Digital. https://digital.nhs.uk/data-and-information/publications/statistical/statistics-on-obesity-physical-activity-and-diet/england-2020/part-3-adult-obesity-copy (2019).

46.	Public Health England. Working Together to Promote Active Travel A briefing for local authorities. www.gov.uk/phe (2016).

47.	Bull, F. C. et al. World Health Organization 2020 guidelines on physical activity and sedentary behaviour. British Journal of Sports Medicine vol. 54 1451–1462 (2020).

48.	WHO. Global Action Plan on Physical Activity 2018-2030: More Active People for a Healthier World. https://apps.who.int/iris/bitstream/handle/10665/272722/9789241514187-eng.pdf?sequence=1&isAllowed=y (2018).

49.	WHO. Ambient air pollution: A global assessment of exposure and burden of disease. (2016).

50.	Watts, N. et al. The 2020 report of The Lancet Countdown on health and climate change: responding to converging crises. Lancet 397, 129–170 (2021).

51.	YouGov. The most important issues facing the country. https://yougov.co.uk/topics/politics/trackers/the-most-important-issues-facing-the-country (2021).

52.	Pidgeon, N. Public understanding of, and attitudes to, climate change: UK and international perspectives and policy. Clim. Policy 12, (2012).

53.	YouGov. YouGov / Eating Better Survey Results. https://www.eating-better.org/uploads/Documents/2017/YouGov survey for Eating Better 2017_EatingLessMeat_W_170424.pdf (2017).

54.	Finder UK. UK diet trends 2021 | Finder UK. https://www.finder.com/uk/uk-diet-trends (2021).

55.	Norton, E. UK meat consumption. Savils https://www.savills.co.uk/research_articles/229130/298951-0 (2020).

56.	Bryant, C. J. We Can’t Keep Meating Like This: Attitudes towards Vegetarian and Vegan Diets in the United Kingdom. Sustainability 11, 6844 (2019).

57.	Shahbaz, M., Shahzad, S. J. H., Alam, S. & Apergis, N. Globalisation, economic growth and energy consumption in the BRICS region: The importance of asymmetries. J. Int. Trade Econ. Dev. 27, 985–1009 (2018).

58.	Andrew, R. M. & Peters, G. P. A MULTI-REGION INPUT-OUTPUT TABLE BASED ON THE GLOBAL TRADE ANALYSIS PROJECT DATABASE (GTAP-MRIO). Econ. Syst. Res. 25, 99–121 (2013).

59.	Davis, S. J., Peters, G. P. & Caldeira, K. The supply chain of CO 2 emissions. Proc. Natl. Acad. Sci. U. S. A. 108, 18554–18559 (2011).

60.	Meng, B., Peters, G. P., Wang, Z. & Li, M. Tracing CO2 emissions in global value chains. Energy Econ. 73, 24–42 (2018).

61.	Davis, S. J. & Caldeira, K. Consumption-based accounting of CO2 emissions. Proc. Natl. Acad. Sci. U. S. A. 107, 5687–5692 (2010).

62.	Graeber, D. On the Phenomenon of Bullshit Jobs. Strike! Magazine (2013).

63.	Frey, C. B. & Osborne, M. A. The future of employment: How susceptible are jobs to computerisation? Technol. Forecast. Soc. Change 114, 254–280 (2017).

64.	ONS. The probability of automation in England . https://www.ons.gov.uk/employmentandlabourmarket/peopleinwork/employmentandemployeetypes/articles/theprobabilityofautomationinengland/2011and2017 (2017).

65.	BEIS Parliamentary Committee. Automation and the future of work Twenty-third Report of Session 2017-19 Report, together with formal minutes relating to the report. https://publications.parliament.uk/pa/cm201719/cmselect/cmbeis/1093/1093.pdf (2019).

66.	ONS. Average actual weekly hours of work for full-time workers (seasonally adjusted) - Office for National Statistics. Labour Market Statistics Time Series https://www.ons.gov.uk/employmentandlabourmarket/peopleinwork/earningsandworkinghours/timeseries/ybuy/lms (2021).

67.	Schor, J. B. The Overworked American: The Unexpected Decline of Leisure. (Basic Books, 1992).

68.	Stronge, W. & Harper, A. The Shorter Working Week. https://autonomy.work/wp-content/uploads/2019/03/Shorter-working-week-docV6.pdf (2019).

69.	Committee on Climate Change. The Sixth Carbon Budget: The UK’s Path to Net Zero. www.theccc.org.uk/publications (2020).

70.	CCC. Land use: Reducing emissions and preparing for climate change - Climate Change Committee. https://www.theccc.org.uk/publication/land-use-reducing-emissions-and-preparing-for-climate-change/ (2018).

71.	Hammond, G. Sector analysis of Industrial Energy. (2018).

72.	NHS Digital. Health Survey for England 2017. https://digital.nhs.uk/data-and-information/publications/statistical/health-survey-for-england/2017 (2017).

73.	HM Government. OUR WASTE, OUR RESOURCES: A STRATEGY FOR ENGLAND OUR RESOURCES: A STRATEGY FOR ENGLAND OUR WASTE, OUR RESOURCES: A STRATEGY FOR ENGLAND 2. www.nationalarchives.gov.uk/doc/open-government-licence/version/3/oremailPSI@nationalarchives.gsi.gov.uk (2018).

74.	Bajželj, B. et al. Importance of food-demand management for climate mitigation. Nat. Clim. Chang. 4, 924–929 (2014).

75.	Stehfest, E. et al. Climate benefits of changing diet. Clim. Chang. 2009 951 95, 83–102 (2009).

76.	Society, T. V. Statistics - The Vegan Society. https://www.vegansociety.com/news/media/statistics. (2019).

77.	BEIS. DIGEST OF UNITED KINGDOM ENERGY STATISTICS 2020. www.gov.uk/government/organisations/department-for-business-energy-and- (2020).

78.	Maclean, K., Sansom, R., Watson, T. & Gross, R. Managing Heat System Decarbonisation: Comparing the impacts and costs of transitions in heat infrastructure Final Report Acknowledgment. https://www.imperial.ac.uk/media/imperial-college/research-centres-and-groups/icept/Heat-infrastructure-paper.pdf (2016).

79.	Rosenow, J. et al. The pathway to net zero heating in the UK. A UKERC policy Br.  1–11 (2019) doi:10.5286/ukerc.edc.000941.

80.	HM Government. Net Zero Strategy: Build Back Greener - October 2021. www.gov.uk/official-documents. (2021).

81.	BEIS. Final UK greenhouse gas emissions national statistics: 1990 to 2019 - GOV.UK. https://www.gov.uk/government/statistics/final-uk-greenhouse-gas-emissions-national-statistics-1990-to-2019 (2021).

82.	BEIS. Building Energy Efficiency Survey (BEES) - GOV.UK. Building Energy Efficiency Servey https://www.gov.uk/government/publications/building-energy-efficiency-survey-bees (2016).

83.	CCC. UK housing: Fit for the future? https://www.theccc.org.uk/publication/uk-housing-fit-for-the-future/ (2019).

84.	CCC. Net Zero - Technical Report. https://www.theccc.org.uk/publication/net-zero-technical-report/ (2019).

85.	BEIS. SMART METERING IMPLEMENTATION PROGRAMME A report on progress of the realisation of smart meter consumer benefits. https://assets.publishing.service.gov.uk/government/uploads/system/uploads/attachment_data/file/830668/smart-meters-benefits-realisation.pdf (2019).

86.	Kelly, J. & Knottenbelt, W. Does disaggregated electricity feedback reduce domestic electricity consumption? A systematic review of the literature. Conf. 3rd Int. NILM Work. (2016).

87.	IEA. Energy Efficiency 2018: analysis and outlooks to 2040. https://iea.blob.core.windows.net/assets/d0f81f5f-8f87-487e-a56b-8e0167d18c56/Market_Report_Series_Energy_Efficiency_2018.pdf (2018).

88.	IEA. Technology Brief: Cooking Appliances. www.etsap.org (2012).

89.	Paoli, L. & Cullen, J. Technical limits for energy conversion efficiency. Energy 192, 116228 (2020).

90.	McKinsey Global Institute. Resource Revolution: Meeting the world’s energy, materials, food, and water needs. https://www.mckinsey.com/~/media/mckinsey/business functions/sustainability/our insights/resource revolution/mgi_resource_revolution_full_report.pdf (2011).

91.	CCC. Reducing UK emissions: 2020 Progress Report to Parliament . https://www.theccc.org.uk/publication/reducing-uk-emissions-2020-progress-report-to-parliament/ (2020).

92.	BEIS. Industrial Decarbonisation Strategy. https://www.gov.uk/government/publications/industrial-decarbonisation-strategy (2021).

93.	Griffin, P., Hammond, G. & Norman, J. Industrial Energy Use from a Bottom-Up Perspective: Developing the Usable Energy Database . https://data.ukedc.rl.ac.uk/browse/edc/efficiency/industry/EnergyConsumption/UED_Documentation.pdf (2013).

94.	CLC & GCB. Zero Avoidable Waste in Construction. https://www.constructionleadershipcouncil.co.uk/wp-content/uploads/2016/05/ZAW-Report-Final-Draft-25-February-2020.pdf (2020).

95.	Anderson, K., Broderick, J. F. & Stoddard, I. A factor of two: how the mitigation plans of ‘climate progressive’ nations fall far short of Paris-compliant pathways. Clim. Policy 1–15 (2020) doi:10.1080/14693062.2020.1728209.

Data available from www.gov.uk/government/statistics/uks-carbon-footprint

For lists of modelling assumptions in each sector refer to the appendix.

Descriptions of sector level modelling approaches are available in the sector level evidence reports that accompany this report.

For a detailed description of the method used to derive this UK remaining carbon budget see Anderson et al. (2020). The budget here differs slightly to the one presented by Anderson et al. (2020), as the UK’s share of global cement process emissions are included.