# Online Resource 1

Article title: Stability of Lead Immobilization by Hydroxyapatite and Biochar in a Typical Brown Soil (Haplic Cambisol)

Journal: Water, Air, & Soil Pollution

Authors: Andrei M. Egorin, Svetlana A. Novikova, Anastasia V. Brikmans, Daria Kh. Shlyk, Andrei M. Gilev, Svetlana Yu. Bratskaya, Olga V. Nesterova

Corresponding author: Andrei M. Egorin, e-mail: andrey.egorin@gmail.com

S1. Full uncertainty budget for Pb determination

The Pb concentration in the extracts was determined potentiometrically (ELIT-231 ion-selective electrode). Calibration demonstrated that a linear Nernstian response (slope of 28.0 mV/decade, R² = 0.999) is maintained at Pb concentrations ≥ 10 mg/L. The total relative uncertainty in this range, calculated by taking into account the instrumental error (via the differential of the Nernst equation) and the contributions of dispensing devices, was u(C) ≈ 6.5% (1σ). This calculated estimate was verified by independent comparison with atomic absorption spectrometry (AAS): the average discrepancy between the methods for samples of 20–30 mg/L was ≈5.2%.

At concentrations < 10 mg/L, the electrode response becomes sub-Nernstian, and the discrepancy with AAS increases to 23–25%. In this regard, for the "low contamination level" treatments (load of 5 mg/g), where the equilibrium lead concentrations are minimal, a conservative empirical uncertainty estimate of 25% was applied in all tables and calculations. For the moderate and high contamination levels, the baseline estimate of 6.5% was used.

S2. Uncertainty budget for pH measurements

The equilibrium pH of the extracts was measured using a Hanna Instruments HI2215 pH meter (HI1131B combined glass electrode, specified accuracy of ±0.01 pH) with a preliminary two-point calibration using standard buffer solutions (pH 4.01 and 6.86). Since a two-point calibration does not allow for the calculation of the statistical regression error, the total uncertainty was evaluated as Type B. The budget includes instrumental accuracy (±0.01 pH), the standard error of the buffer solutions (±0.015 pH), and a conservative estimate of the liquid junction potential effect in the soil extract matrix with moderate ionic strength (±0.04 pH). The total standard uncertainty was u(pH) ≈ ±0.05. Thus, the dominant contribution to the error is made by the specific operation of the electrode in the actual extract matrix, rather than by the hardware characteristics.

S3. Statistical significance criterion and comprehensive revision

The statistical significance of the observed effects was determined by comparing the difference between the control and experimental treatments with the combined measurement uncertainty. For two independent quantities, the combined uncertainty of their difference was calculated according to the standard rule of error propagation: : u(A−B) = √(u(A)2 + u(B)2). The difference was considered statistically significant if it exceeded the calculated threshold at a coverage factor of k = 1. To evaluate the immobilization efficiency (reduction in the proportion of mobile Pb) at moderate and high contamination levels, the baseline relative uncertainty of each measurement was 6.5%, yielding a combined detection threshold of ≈9.2%. For the low contamination level, characterized by elevated analytical error, the significance threshold was increased to ≈35.4%. To identify a statistically significant pH shift, the critical difference was set at ≈0.07 units. Individual efficiency values that fell on the boundary of the detection threshold (e.g., 8.3% and 8.4% with a calculated threshold of ≈8.4%) are marked in Tables S2 and S4 with a "†" symbol. These results are not interpreted as confidently established independent facts; however, they are taken into account for the sake of completeness when defining the overall efficiency ranges of the ameliorants.

S4. Scientific Justification of Methodological Approaches

S4.1. Concept of Mobile and Bioavailable Forms

The separation of lead compounds by their degree of mobility was performed within the framework of a multilevel approach regulated by the ISO 17402:2008 standard [1]. The applied sequential two-step extraction (distilled water, followed by an unbuffered Ca2+ solution) corresponds to the conservative levels of assessing pollutant migration capacity. The extracted fractions are interpreted strictly geochemically—as water-soluble and exchangeable forms. In the present study, the term "bioavailability" is not used for the direct characterization of chemical extracts, since biological uptake is a physiological characteristic of a specific cultivated crop rather than exclusively a property of the medium. The exchangeable form is considered a labile pool of the pollutant, which can only potentially become bioavailable upon changes in soil and ecological conditions.

S4.2. Justification of the Water Extract

Extraction with distilled water is a standard approach for assessing the most mobile metal fraction, which includes free ions and forms weakly bound to soil colloids [2]. The concentration of lead in the water extract serves as a sensitive indicator of its mobility, regardless of the type of applied ameliorants. Unlike the application of acidic or chelating reagents, aqueous extraction does not induce an artificial shift in the acid-base equilibrium during sample preparation. This methodological condition guarantees that the recorded changes in the equilibrium pH of the extracts reflect exclusively native physicochemical interactions in the "soil–ameliorant–Pb" system, precluding artifacts from the chemical impact of the extractant itself.

S4.3. Justification of Extraction with a Dilute Ca2+ Solution

Extraction with dilute unbuffered solutions of neutral salts serves as a standardized method for recovering the exchangeable fraction of heavy metals. Although the most common extractant of this type is a 0.01 M CaCl2 solution [3], a justified alternative—a 0.01 M Ca(NO3)2 solution—was used in the present study [2, 4]. The use of a nitrate salt instead of a chloride one is aimed at eliminating the side effect of complexation. In chloride extracts, the proportion of the soluble PbCl+ complex can reach 31% of the total amount of extracted lead [5]. This leads to an artificial overestimation of the metal mobility indices, especially in alkaline and carbonate media [6]. The application of Ca(NO3)2 preserves the high displacing ability of the Ca2+ cation, yet acts as an inert background, ensuring a correct assessment exclusively of the exchangeable lead pool.

S4.4. Justification of Liquid-to-Solid (LS) Ratios

During the incubation stage, the LS ratio (1.5:1 mL/g) was selected to simulate a moisture regime approximating natural field conditions. During the chemical extraction stage, the LS ratio was set at 20:1 (20 L/kg). This parameter strictly corresponds to the US EPA 1311/1312 methodology [6, 7] and falls within the regulated ranges of the BCR sequential extraction scheme (up to 40 mL/g) [8, 9] and the ISO 21268 standard [10, 11].

S4.5. Mathematical Framework of Sorption Models

The parameters of the approximation models were calculated using the non-linear least squares method (Levenberg–Marquardt algorithm) based on the raw experimental data. The use of non-linearized equation forms eliminates the systematic bias of estimates and the distortion of the variance structure characteristic of mathematical transformations during linearization. The equilibrium sorption isotherms were described by the following equations:

Langmuir model (1):

(1)

Sips model (2):

(2)

where qe is the equilibrium specific sorption (mg/g); Ce is the equilibrium pollutant concentration in the solution (mg/L); qm is the maximum capacity (mg/g); Kl and Ks are the Langmuir (L/mg) and Sips affinity constants, respectively; n is the surface heterogeneity parameter.

The kinetics of the sorption process were analyzed using the following equations:

Pseudo-first-order model (Lagergren) (3):

(3)

Pseudo-second-order model (Ho–McKay) (4):

(4)

Elovich model (5):

(5)

where qt is the specific sorption at time t (mg/g); k1 is the pseudo-first-order rate constant (1/min); k2 is the pseudo-second-order rate constant (g/(mg·min)); α is the initial sorption rate (mg/(g·min)); β is a constant characterizing the chemisorption activation energy and the degree of surface coverage (g/mg).

## S5. Full Numerical Data

Table S1. Content of water-soluble lead, mg/kg of dry soil (± u(C))

| Treatment | Time, days | Low | Moderate | High |
| --- | --- | --- | --- | --- |
| Control | 8 | 41.48 ± 10.37 | 1630.88 ± 106.01 | 4144.90 ± 269.42 |
| Biochar | 8 | 30.32 ± 7.58 | 1317.60 ± 85.64 | 3592.20 ± 233.49 |
| HAP | 8 | 20.98 ± 5.25 | 1024.80 ± 66.61 | 2798.84 ± 181.93 |
| Biochar + HAP | 8 | 23.66 ± 5.92 | 973.56 ± 63.28 | 2806.00 ± 182.39 |
| Control | 46 | 10.80 ± 2.70 | 908.16 ± 59.03 | 2860.42 ± 185.93 |
| Biochar | 46 | 17.88 ± 4.47 | 824.96 ± 53.62 | 2790.88 ± 181.41 |
| HAP | 46 | 11.92 ± 2.98 | 728.56 ± 47.36 | 2331.56 ± 151.55 |
| Biochar + HAP | 46 | 14.42 ± 3.61 | 602.70 ± 39.18 | 2243.26 ± 145.81 |
| Control | 90 | 11.16 ± 2.79 | 1122.11 ± 72.94 | 2982.24 ± 193.85 |
| Biochar | 90 | 19.41 ± 4.85 | 851.43 ± 55.34 | 2731.29 ± 177.53 |
| HAP | 90 | 17.77 ± 4.44 | 458.61 ± 29.81 | 1721.06 ± 111.87 |
| Biochar + HAP | 90 | 16.15 ± 4.04 | 546.61 ± 35.53 | 1792.89 ± 116.54 |

Table S2. Reduction of water-soluble lead content relative to the control, %

| Treatment | Time, days | Low* | Moderate | High |
| --- | --- | --- | --- | --- |
| Biochar | 8 | 26.9 | 19.2 | 13.3 |
| HAP | 8 | 49.4 | 37.2 | 32.5 |
| Biochar + HAP | 8 | 42.9 | 40.3 | 32.3 |
| Biochar | 46 | −65.6* | 9.2 | 2.4 |
| HAP | 46 | −10.4* | 19.8 | 18.5 |
| Biochar + HAP | 46 | −33.6* | 33.6 | 21.6 |
| Biochar | 90 | −73.8* | 24.1 | 8.4† |
| HAP | 90 | −59.2* | 59.1 | 42.3 |
| Biochar + HAP | 90 | −44.6* | 51.3 | 39.9 |

* For the low contamination level, the elevated analytical error (±25%) makes these values statistically indistinguishable from zero.

Table S3. Content of exchangeable lead, mg/kg of dry soil (± u(C))

| Treatment | Time, days | Low | Moderate | High |
| --- | --- | --- | --- | --- |
| Control | 8 | 57.44 ± 14.36 | 1691.22 ± 109.93 | 2572.26 ± 167.20 |
| Biochar | 8 | 53.24 ± 13.31 | 1483.10 ± 96.40 | 2586.00 ± 168.09 |
| HAP | 8 | 44.68 ± 11.17 | 1447.64 ± 94.10 | 2350.96 ± 152.81 |
| Biochar + HAP | 8 | 21.76 ± 5.44 | 1310.88 ± 85.21 | 2285.00 ± 148.53 |
| Control | 46 | 20.72 ± 5.18 | 1070.30 ± 69.57 | 1960.16 ± 127.41 |
| Biochar | 46 | 24.06 ± 6.01 | 1085.06 ± 70.53 | 1925.74 ± 125.17 |
| HAP | 46 | 16.30 ± 4.08 | 917.44 ± 59.63 | 1797.00 ± 116.81 |
| Biochar + HAP | 46 | 15.78 ± 3.94 | 870.72 ± 56.60 | 1770.50 ± 115.08 |
| Control | 90 | 28.08 ± 7.02 | 1407.51 ± 91.49 | 1912.61 ± 124.32 |
| Biochar | 90 | 27.97 ± 6.99 | 1112.01 ± 72.28 | 1829.15 ± 118.89 |
| HAP | 90 | 15.95 ± 3.99 | 754.65 ± 49.05 | 1360.48 ± 88.43 |
| Biochar + HAP | 90 | 18.45 ± 4.61 | 723.03 ± 47.00 | 1469.59 ± 95.52 |

Table S4. Reduction of exchangeable lead content relative to the control, %

| Treatment | Time, days | Low* | Moderate | High |
| --- | --- | --- | --- | --- |
| Biochar | 8 | 7.3 | 12.3 | −0.5 |
| HAP | 8 | 22.2 | 14.4 | 8.6 |
| Biochar + HAP | 8 | 62.1 | 22.5 | 11.2 |
| Biochar | 46 | −16.1* | −1.4* | 1.8 |
| HAP | 46 | 21.3 | 14.3 | 8.3† |
| Biochar + HAP | 46 | 23.8 | 18.7 | 9.7 |
| Biochar | 90 | 0.4* | 21.0 | 4.4 |
| HAP | 90 | 43.2 | 46.4 | 28.9 |
| Biochar + HAP | 90 | 34.3 | 48.6 | 23.2 |

* For the low contamination level, the elevated analytical error (±25%) makes these values statistically indistinguishable from zero.

† The value is at the boundary of the detection threshold (for details, see Section S3).

Table S5. Measured pH of the extracts

| Level | Time, days | Fraction | Control | Biochar | HAP | Biochar + HAP |
| --- | --- | --- | --- | --- | --- | --- |
| Low | 8 | water-soluble | 4.99 | 5.20 | 5.38 | 5.63 |
| Low | 8 | exchangeable | 5.35 | 5.39 | 5.46 | 5.61 |
| Low | 46 | water-soluble | 5.23 | 5.65 | 5.34 | 5.60 |
| Low | 46 | exchangeable | 5.29 | 5.20 | 4.99 | 5.03 |
| Low | 90 | water-soluble | 5.64 | 5.40 | 5.66 | 5.34 |
| Low | 90 | exchangeable | 4.89 | 5.24 | 5.26 | 5.23 |
| Moderate | 8 | water-soluble | 4.25 | 4.41 | 4.53 | 4.48 |
| Moderate | 8 | exchangeable | 4.32 | 4.44 | 4.46 | 4.46 |
| Moderate | 46 | water-soluble | 4.33 | 4.28 | 4.33 | 4.45 |
| Moderate | 46 | exchangeable | 4.23 | 4.12 | 4.25 | 4.30 |
| Moderate | 90 | water-soluble | 4.19 | 4.28 | 4.27 | 4.30 |
| Moderate | 90 | exchangeable | 3.79 | 3.89 | 3.76 | 3.74 |
| High | 8 | water-soluble | 4.21 | 4.29 | 4.38 | 4.33 |
| High | 8 | exchangeable | 3.98 | 3.87 | 4.08 | 4.08 |
| High | 46 | water-soluble | 3.70 | 3.97 | 4.07 | 4.09 |
| High | 46 | exchangeable | 4.35 | 4.30 | 4.32 | 4.25 |
| High | 90 | water-soluble | 3.90 | 3.94 | 4.02 | 3.90 |
| High | 90 | exchangeable | 3.72 | 3.71 | 3.86 | 3.89 |

Table S6. Average pH shift relative to the control (ΔpH), n=9 per treatment, all levels and durations

| Treatment | Δ pH, water-soluble | Δ pH, exchangeable |
| --- | --- | --- |
| Biochar | +0.11 | +0.03 |
| HAP | +0.17 | +0.06 |
| Biochar + HAP | +0.19 | +0.07 |

S6. Conditions and goodness-of-fit criteria for full-profile Rietveld refinement (XRD)

S6.1. Refinement conditions

Quantitative phase analysis of the control sample was performed using the full-profile Rietveld refinement method in the Profex/BGMN software (BGMN version 4.2.23). The unit cell and profile parameters of defect-free framework minerals (quartz, plagioclase) were fixed at the values obtained for the control sample, whereas the parameters of labile phyllosilicates (illite, vermiculite) were refined freely.

S6.2. Goodness-of-fit criteria

Table S7. Goodness-of-fit criteria for the Rietveld refinement

| Parameter | Value |
| --- | --- |
| Rp | 3.25 % |
| Rpb | 40.11 % |
| R | 4.32 % |
| Rwp | 4.45 % |
| Rexp | 2.29 % |
| χ² = (Rwp/Rexp)² | ≈ 3.78 |
| Durbin–Watson d | 0.22 |
| 1-ρ | 8.00 % |

The obtained value of χ² ≈ 3.78 is acceptable for the quantitative phase analysis of multiphase natural soil systems containing structurally disordered clay minerals (illite, vermiculite). For such systems, achieving χ² < 2, which is typical for single-phase reference samples, is hindered by the deviation of the actual structure of phyllosilicates from idealized structural models.

S6.3. Quantitative phase analysis results (GOALs)

Table S8. Quantitative phase analysis results (Rietveld method)

| Phase | Mass fraction | Uncertainty |
| --- | --- | --- |
| Andesine | 0.4408 | ± 0.0050 |
| Quartz | 0.2635 | ± 0.0035 |
| Vermiculite | 0.00948 | ± 0.00080 |
| Illite | 0.2862 | ± 0.0054 |

S7. Full Data of the Model "Biochar + HAP" Binary System

Experimental conditions: Model solution — 0.01 M KNO3 (background electrolyte), initial pH 5.0; solution volume (V) — 50 mL; biochar dosage fixed at 0.200 g, mass of added hydroxyapatite (HAP) varied (total sorbent mass in the system — from 0.200 to 0.240 g); initial lead concentrations (C₀) were 10 and 100 mg/L; the experiment was performed in triplicate (n = 3).

Table S9. Residual Pb concentration in solution, mg/L (mean ± SD, n=3)

| Total dosage, g | HAP proportion, % | Cₑ (C₀=10 mg/L) | Cₑ (C₀=100 mg/L) |
| --- | --- | --- | --- |
| 0.200 (biochar only) | 0.00 | 0.487 ± 0.158 | 32.26 ± 12.07 |
| 0.202 | 0.99 | 0.315 ± 0.163 | 19.87 ± 7.79 |
| 0.205 | 2.44 | 0.298 ± 0.119 | 0.78 ± 0.56 |
| 0.210 | 4.76 | 0.246 ± 0.078 | 0.32 ± 0.09 |
| 0.222 | 9.91 | 0.264 ± 0.014 | 0.22 ± 0.13 |
| 0.240 | 16.67 | 0.321 ± 0.159 | 0.28 ± 0.14 |

Table S10. Equilibrium capacity of the mixture qₑ (mg Pb/g of total dosage) and removal efficiency, %

| Total dosage, g | HAP proportion, % | qₑ, C₀=10 | Removed, % (C₀=10) | qₑ, C₀=100 | Removed, % (C₀=100) |
| --- | --- | --- | --- | --- | --- |
| 0.200 | 0.00 | 2.378 | 95.13 | 16.94 | 67.74 |
| 0.202 | 0.99 | 2.397 | 96.85 | 19.83 | 80.12 |
| 0.205 | 2.44 | 2.366 | 97.02 | 24.20 | 99.22 |
| 0.210 | 4.76 | 2.322 | 97.54 | 23.73 | 99.68 |
| 0.222 | 9.91 | 2.193 | 97.36 | 22.47 | 99.78 |
| 0.240 | 16.67 | 2.016 | 96.79 | 20.78 | 99.72 |

Table S11. Boundaries of the specific capacity attributed directly to HAP under two limiting assumptions regarding the partitioning of bound Pb between the components

| m(HAP), g | HAP proportion, % | Lower boundary, mg/g (C₀=100) | Upper boundary, mg/g (C₀=100) | Upper to lower ratio |
| --- | --- | --- | --- | --- |
| 0.002 | 0.99 | 309.8 | 2003.3 | 6.5 |
| 0.005 | 2.44 | 314.8 | 992.2 | 3.2 |
| 0.010 | 4.76 | 159.7 | 498.4 | 3.1 |
| 0.022 | 9.91 | 72.8 | 226.8 | 3.1 |
| 0.040 | 16.67 | 40.0 | 124.7 | 3.1 |

The lower boundary was calculated based on the assumption of complete additivity (biochar in the mixture retains the same capacity as in the isolated system) (6):

The upper boundary was calculated based on the assumption of complete kinetic suppression of biochar (all Pb bound by the mixture is attributed to HAP) (7):

Where C₀ is the initial Pb concentration in the solution (mg/L); Ce,mix is the equilibrium (residual) Pb concentration in the solution for the biochar + HAP mixture (mg/L); Ce,ctrl is the equilibrium Pb concentration in the solution for the control (biochar only, mg/L); V is the solution volume (L); mHAP is the mass of added HAP (g).

Both estimates were performed using the mean Cₑ values from Table S9.

For the initial concentration of C₀ = 10 mg/L, the estimation of the limiting boundaries is not provided, as the absolute capacity values are small, and the structural uncertainty of the contribution partitioning method is commensurate with the determined values. The total amount of Pb in the system at C₀ = 100 mg/L and V = 50 mL is 5.0 mg. Starting from an HAP proportion of 2.44% of the total sorbent mass, 4.96 to 4.99 mg out of 5.0 mg (99.2–99.8%) is bound. Thus, the calculated decrease in the specific capacity of HAP with a further increase in its dose (across both estimation boundaries) is caused by the approach toward the complete exhaustion of the available lead pool in the solution, rather than by an antagonistic interaction between the components of the binary mixture. The interpretation of these results and the justification for the choice between the boundary estimates are presented in the main text of the article.

References

- ISO 17402:2008. Soil quality — Requirements and guidance for the selection and application of methods for the assessment of bioavailability of contaminants in soil and soil materials : published 2008-04. – Geneva : International Organization for Standardization, 2008. – 27 p. – URL: https://www.iso.org/standard/38349.html (accessed: 20.07.2026).

- Wang Z., Tang W., Ding X., Chen Z., He L., Yang L., Lin C. Different extractable pools of Cd and Pb in agricultural soil under amendments: Water-soluble concentration sensitively indicates metal availability // Journal of Environmental Sciences. – 2025. – Vol. 150. – P. 297–308. DOI: 10.1016/j.jes.2024.03.015.

- Houba V. J. G., Temminghoff E. J. M., Gaikhorst G. A., Van Vark W. Soil analysis procedures using 0.01 M calcium chloride as extraction reagent // Communications in Soil Science and Plant Analysis. – 2000. – Vol. 31, Iss. 9-10. – P. 1299–1396. DOI: 10.1080/00103620009370514.

- Meers E., Du Laing G., Tack F. M. G., Verloo M. G. Heavy metal displacement by exchangeable bases (Ca, Mg, K, Na) in soils and sediments // Soil Science. – 2009. – Vol. 174, Iss. 4. – P. 202–209. DOI: 10.1097/SS.0b013e3181a1795c.

- Du L., Cuss C. W., Dyck M., Shotyk W. Size fractionation of dissolved (< 0.45 µm) trace elements from extracted soil with water and CaCl2 using AF4-UV-ICPMS to predict their bioavailability // Geoderma. – 2023. – Vol. 440. – P. 116686. DOI: 10.1016/j.geoderma.2023.116686.

- Wasay S. A., Parker W. J., VanGeel P. J. Removal of lead from a calcareous soil by chloride complexation // Soil and Sediment Contamination. – 2002. – Vol. 11, Iss. 6. – P. 841–859. DOI: 10.1080/20025891107087.

- US EPA Method 1311. Toxicity Characteristic Leaching Procedure (TCLP). Part of Test Methods for Evaluating Solid Waste, Physical/Chemical Methods (SW-846). – Washington, DC : U.S. Environmental Protection Agency, 1992. – 35 p. – URL: https://www.epa.gov/hw-sw846/sw-846-test-method-1311-toxicity-characteristic-leaching-procedure (accessed: 20.07.2026).

- Rauret G., López-Sánchez J. F., Sahuquillo A., Rubio R., Davidson C., Ure A., Quevauviller Ph. Improvement of the BCR three step sequential extraction procedure prior to the certification of new sediment and soil reference materials // Journal of Environmental Monitoring. – 1999. – Vol. 1, Iss. 1. – P. 57–61. DOI: 10.1039/A807854E.

- Ure A. M., Quevauviller Ph., Muntau H., Griepink B. Speciation of heavy metals in soils and sediments. An account of the improvement and harmonization of extraction techniques undertaken under the auspices of the BCR of the Commission of the European Communities // International Journal of Environmental Analytical Chemistry. – 1993. – Vol. 51, Iss. 1-4. – P. 135–151. DOI: 10.1080/03067319308027619.

- ISO 21268-1:2019. Soil quality — Leaching procedures for subsequent chemical and ecotoxicological testing of soil and soil-like materials — Part 1: Batch test using a liquid to solid ratio of 2 l/kg dry matter : published 2019-10. – Geneva : International Organization for Standardization, 2019. – 32 p. – URL: https://www.iso.org/standard/68250.html (accessed: 20.07.2026).

- ISO 21268-2:2019. Soil quality — Leaching procedures for subsequent chemical and ecotoxicological testing of soil and soil-like materials — Part 2: Batch test using a liquid to solid ratio of 10 l/kg dry matter : published 2019-10. – Geneva : International Organization for Standardization, 2019. – 32 p. – URL: https://www.iso.org/standard/68251.html (accessed: 20.07.2026).