# Supplementary Information: Evaluation of A Conceptual Autonomous System for Early-Stage Wildfire Hotspot Suppression: A Comparative Analysis of Performance

Supplementary Information

Osman Acar*1 , Abdulkadir Saday2 , Eija Honkavaara3 , Aki Mikkola4 , Juha Plosila5

References in this Supplementary Information use the numbering of the main article. Equations, tables, and figures prefixed with S are supplementary items.

# Scope and reading guide

This Supplementary Information documents the complete assumption ledger, phase sequence, equations, uncertainty implementation, external rotor-power benchmarks, numerical verification, and evidence boundary used in the accompanying article. It is supplied to make the computational screen auditable without increasing the main-text display-item count. The inputs are screening assumptions unless a row explicitly identifies published measurements.

The wildfire operation is a conceptual early-intervention case study. It reduces a proposed sortie to installed mass, distance, duration, electrical load, fuel use, battery state, and a 5 kg payload-release event. It does not simulate fire spread, suppressant deposition, safe standoff, containment, extinguishment, or responder arrival. The proposed system is a pre-positioned autonomous support concept intended to slow localized early-stage fire propagation while conventional resources mobilize; it is not intended to replace firefighters, airtankers, helicopters, or incident-command operations.

The probability reported in the article is the probability of closing a declared computational mass-energy-electrical ledger under four bounded input families. It is not aircraft reliability, mission-success probability, fleet reliability, or wildfire-suppression probability. Supplementary Fig. S1 separates the quantities evaluated in that ledger from the physical and operational evidence gates that remain outside it.

# Supplementary Methods

## S1. Design representation and mission boundary

One representative vehicle and one outbound-service-return sortie were evaluated. Mobile stations are assumed to have been positioned before an alert. The calculation begins only after verified detection, authorized dispatch, and airspace clearance. It ends after return landing. Station-location optimization, dispatch latency, target allocation, multi-vehicle coordination, communications, fire behavior, suppression effect, and conventional-resource arrival are not state variables.

The common energy-architecture vector is

x=Pg,r,mf,mb,πg,  Eb=mbeb  (S1)

Here, Pg,r is rated installed generator output, mf is nominal initial fuel mass, mb is complete installed battery-pack mass, and πg is the hybrid dispatch policy. A candidate with Pg,r>0 and mf>0 is series hybrid-electric. The pure-electric boundary is Pg,r=mf=0, for which πg is inactive. Battery mass and specific energy refer to a complete usable installed pack, including enclosure, interconnects, battery-management and protective hardware, and wiring. Generator mass likewise denotes the intended complete installation rather than an engine-only mass. No commercial component is claimed to meet these assumed values.

The internal wet-mass limit of 25 kg is an engineering requirement used consistently in every architecture. It is not asserted to establish a regulatory category or authorization. The nominal payload is 5 kg. Treating 5 L as 5 kg is a sizing convention only; suppressant formulation and density are not selected.

## S2. Installed-mass and phase-state ledger

For one realization, wet take-off mass is

MTO=mNE+mgPg,r+mb+mfλf+msλs  (S2)

where tildes denote realized installed hardware-group masses, ms=5 kg, and λf and λs are fuel- and solution-fill multipliers. The reference non-energy-system modes sum to 11.85 kg. The central installed generator map is 3.10 kg at 3.0 kW, 3.35 kg at 3.5 kW, and 3.70 kg at 4.0 kW; the pure-electric boundary has mg0=0. Fuel and solution fill tolerances are independent triangular distributions with supports 0.95/1.00/1.05 and 0.98/1.00/1.02, respectively.

Physical mass changes phase by phase according to

Mp+1=Mp-Δmf,p-Δms,p  (S3)

Fuel burn is always removed from the physical ledger. The complete payload is removed only at the end of the application/observation phase. The primary aerodynamic sizing convention is conservative: Mppow=MTO before payload release and Mppow=MTO-msλs afterward. A paired diagnostic instead sets Mppow=Mp to credit progressive fuel burn. That replay increased the frozen LF and ES radii by only 0.225 km (1.6%) and 0.195 km (2.3%), respectively, and was not used for selection.

Supplementary Table S2 gives every mission phase, duration, load, generator state, and mass-state change. The outbound and return cruise durations are

toutD=D3.6 va-w∥ηr,  tretD=D3.6 va+w∥ηr.

The factor 3.6 converts kilometres and metres per second to hours. The same sampled along-track wind component is treated as a headwind outbound and a tailwind on return. The commanded airspeed remains 12 m s⁻¹; wind alters travel time rather than the aerodynamic evaluation speed. Ground speed is limited to at least 6 m s⁻¹ within the declared scenario to prevent a singular duration. This is a steady route-energy screen, not a time-resolved wind or gust model.

## S3. Flight-power implementation

For Nr=4 identical rotors of radius R, total disk area is

Adisk=NrπR2  (S4)

At air-relative forward speed Vp, equivalent parasite drag and required steady level-flight thrust are

Dpar,p=12ρaCDArefVp2,  Tp=Mppowg2+Dpar,p2  (S5)

The induced velocity is the positive solution of the forward-flight momentum relation and the induced shaft-power term is

vi,p=Tp2ρaAdiskVp2+vi,p2,  Pi,p=10-3κTpvi,p  (S6)

With cp=Tp/2ρaAdisk, the implementation evaluates the numerically stable rationalized root

vi,p=2cp2Vp4+4cp2+Vp21/2  (S7)

For rotor advance ratio μp=Vp/ΩR, the low-order profile term is

Ppro,p=10-3σCd08ρaAdiskΩR31+cμμp2  (S8)

The parasite term is

Ppar,p=10-3Dpar,pVp  (S9)

The electrical flight load is

Pflight,p=ξPPi,p+Ppro,p+Ppar,p+Pclimb,pηdrv+Paux  (S10)

where Pclimb,p=10-3Mppowgvz,p for positive climb rate. Factors 10-3 convert watts to kilowatts. The climb term is additive and does not solve a unified oblique-inflow problem. Negative climb power is not credited. The descent phases instead multiply induced, profile, and parasite power by kD=0.65, leaving the 0.12 kW auxiliary load unchanged.

The central flight inputs are R=0.30 m, Adisk=1.131 m², ρa=1.225 kg m⁻³, σ=0.09, Cd0=0.012, ΩR=180 m s⁻¹, cμ=4.65, κ=1.15, CDAref=0.35 m², ηdrv=0.82, and Paux=0.12 kW. Rotor radius and drag area were crossed over R=0.28/0.30/0.32 m and CDAref=0.25/0.35/0.45 m² in a deterministic cellwise re-optimization. Every coefficient is an author-declared screening input; none is claimed as a MORPHFIRE calibration.

The model does not resolve blade geometry, rotor-speed limits, thrust reserve, rotor-rotor interference, wake skew, ground effect, control power, motor/ESC loss maps, or transient response. The scalar descent factor cannot locate vortex-ring, turbulent-wake, or windmill-brake regimes [38-40]. Consequently, these equations support comparative screening, not a safe climb/descent envelope or validated range.

## S4. External rotor-power benchmarks

The external comparisons were performed without refitting κ, σ, or Cd0. The UIUC and NASA measured-data comparisons are deliberately described as a partial external benchmark, not validation of MORPHFIRE. The published-model comparison below is contextual only.

### S4.1 Contextual published-model comparison

Bauersfeld and Scaramuzza [22] reported a blade-element/momentum and motor model that achieved 2.5% power root-mean-square error on their own approximately 0.752 kg quadrotor with 5.1-inch propellers and gave a normalized endurance-optimal power ratio of Pe/Ph=0.914±0.0323 across simulated designs. At matched mass, rotor count, radius, and air density, their generic momentum estimator with a figure of merit of 0.60 gives 3.503 kW mechanical hover power. The present induced-plus-profile expression gives 3.508 kW, a 0.14% cross-model difference. The present central curve reaches Pmin/Phover=0.898 at 11.24 m s⁻¹, 1.8% below the published mean and within one reported standard deviation. These comparisons provide trend context only: the 2.5% result validates their aircraft-specific model, not the MORPHFIRE equations or a 23.5 kg vehicle.

### S4.2 UIUC isolated-propeller static benchmark

Measured static data were taken from the UIUC Volume 4 propeller database for APC Electric 18x8, 20x10, and 21x13 propellers [53]. The database defines

CT=Tρan2Dprop4,  CP=Pρan3Dprop5,

where n is revolutions per second and Dprop is propeller diameter. The hover specialization of Supplementary Equations S6 and S8 reduces, in the UIUC convention, to

CP,pred=κCT2CTπ+σCd0π432.

Using the manuscript values κ=1.15, σ=0.09, and Cd0=0.012 without refitting, 26 measurements gave a combined mean absolute percentage error (MAPE) of 5.41%, normalized root-mean-square error of 6.27%, signed mean percentage bias of +2.17%, and maximum absolute percentage error of 16.54% (Supplementary Table S6). Positive bias denotes model overprediction. This comparison validates only the shaft-power coefficient conditional on measured thrust for isolated propellers in static conditions. The calculation does not predict thrust from blade geometry, and propeller solidity was not supplied by the source; the manuscript’s assumed solidity was therefore retained rather than fitted.

### S4.3 NASA installed-quadrotor hover benchmark

NASA’s second Multirotor Test Bed (MTB2) provides a useful installed-system stress case because it uses four 24.5-inch rotors, close to the assumed 0.60 m MORPHFIRE diameter [54,55]. For the NASA rotor, R=0.3110 m and reported solidity is σ=0.08. In the total-disk-area convention,

CT=TρaAtotΩR2,  CP=PρaAtotΩR3,

with a torque-derived rotor-system mechanical-power proxy

Pproxy=j=14QjΩj,  CP,pred=κCT3/22+σCd08.

Two equal-height, four-rotor nominal-hover points at 3000 rpm were extracted from NASA run 136. At CT=0.005647 with 0-degree phase arrangement, the torque-derived proxy was 801.1 W and the unretuned model predicted 633.0 W, an error of -21.0%. At CT=0.005541 with the 0/-90-degree phase arrangement, the proxy was 789.4 W and the prediction was 619.7 W, an error of -21.5%. The model’s 25 kg design-limit hover loading corresponds to CT≈0.00546; the two NASA coefficients are 1.5-3.4% higher and therefore closely load-matched, but they do not bracket that value. The measured-to-predicted multiplier is 1.266-1.274.

The NASA comparison is a torque-derived rotor-system mechanical-power-proxy benchmark, not an electrical-power validation. The discrepancy may combine rotor-airfoil and Reynolds-number effects, rotor interaction, hubs and drivetrain mechanical loss represented in the torque data, and wind-tunnel installation effects. It must not be transferred as a universal calibration. The current upper residual power factor ξP=1.15 does not span the approximately 1.27 NASA discrepancy; therefore, existing architecture radii are not relabeled as physically validated. A future registered sensitivity should rerun the complete allocation and holdout process with ξP=1.27 only as an explicit model-form stress case.

## S5. Generator, fuel, and dispatch implementation

Available installed generator output is Pg,a=ξgPg,r. For requested on-state output Pg,pon>0 and load fraction λp=Pg,pon/Pg,a, the author-defined normalized chemical-input scenario is

Pf,pon=Pg,aa0+a1λp+a2λp2ξη,  mf,pon=Pf,ponLHV,  ηgPg,pon=λpξηa0+a1λp+a2λp2  (S11)

All three quantities are zero when the generator is off. If remaining dispatchable fuel cannot sustain the requested point throughout phase p, the generator retains that on-state point for a duty fraction

dp=min1,mf,premLHVPf,pontp,  Pg,p=dpPg,pon,  mf,p=dpmf,pon  (S12)

The on-state output must satisfy

0≤Pg,pon≤Pg,a  (S13)

and total usable fuel must satisfy

p​mf,ptp≤1-rfmfλfϕf  (S14)

The central coefficients are a0,a1,a2=0.15,4.3869,0.234375, selected so that the assumed central curve reaches ηg=0.21 at λ*=0.80. This 21% peak is a favorable author technology target rather than a measured or typical value. The deterministic technology screen therefore repeats the architecture allocation at peak efficiencies of 10.5%, 15.75%, and 21%, crossed with pack specific energies of 0.25, 0.35, and 0.45 kWh kg⁻¹.

Under load-following dispatch,

Pg,pon,LF=minPL,p,Pg,a  (S15)

Under efficiency-seeking dispatch,

λ*=argmax0<λ≤1 ηgλ,  Pgon,*=λ*Pg,a  (S16)

The ES policy requests 80% on-state output in generator-enabled fixed-duration phases, subject to available output, remaining fuel, instantaneous load, charge-power limit, charge efficiency, and energy headroom. The battery supplies any deficit and absorbs admissible surplus. During distance-varying cruise, requested output is curtailed to the instantaneous load so that increasing distance cannot create an artificial recharge benefit. The generator is disabled during preflight, both transformations, both ground legs, and application/observation.

Start-up fuel, start/stop limits, ramp rate, exhaust, vibration, cooling parasitics, heat soak, and temperature-dependent derating are not modeled. A future lumped generator thermal state could be written as

Cth,gTg=Ploss,gPg,pon-UgAgTg-Ta,  Tg≤Tg,max  (S17)

but the required heat capacity, installed heat-rejection conductance, imposed heat flux, allowable temperature, and cooling load have not been measured. Supplementary Equation S17 is therefore not activated in the closure indicator.

## S6. Battery-energy and static electrical implementation

With discharge-positive convention, generator duty cycling keeps discharge and charge separate:

Pb,p+=dpmaxPL,p-Pg,pon,0+1-dpPL,p,  Pb,p-=dpmaxPg,pon-PL,p,0  (S18)

Thus Pb,p+-Pb,p-=PL,p-Pg,p. Battery usable-energy state evolves as

Eb,p+1state=Eb,pstate-Pb,p+tpηd+ηcPb,p-tp  (S19)

The activated ledger uses usable DC-bus energy with ηd=1 and applies ηc=0.92 only to stored charge. Realized usable capacity is Eb,cap=Ebξb, the initial state is Eb,0state=Eb,cap, and zb,p=Eb,pstate/Eb,cap. Minimum and final SOC must remain at or above 0.20.

For every discharge phase, the static Thevenin screen uses the lower of phase-start and phase-end states,

zb,pscr=minEb,pstate,Eb,p+1stateEb,cap  (S20)

The maximum discharge represented inside the phase/duty-cycle abstraction is

Pb,ppk=PL,p,dp<1,maxPL,p-Pg,pon,0,dp=1.  (S21)

For zb=zb,pscr, terminal voltage and current satisfy

Vt=Voczb-IbR0,  1000Pb,ppk=VtIb  (S22)

where power is in kilowatts and the factor 1000 converts it to watts. The physically relevant lower-current discharge root is

Ib=Voc-Voc2-4000R0Pb,ppk2R0  (S23)

Feasibility requires a nonnegative discriminant, Vt≥Vmin, Ib≤Imax,dis, and zmin≤zb≤zmax. The screen uses Vnom=96 V, Vocz=960.90+0.20z V, Vmin=76.8 V, R0=0.18/Eb,cap ohm when Eb,cap is in kWh, Qb=1000Eb,cap/Vnom Ah, and Imax,dis=CIQb with CI=4/5/6 h⁻¹ across the bounded input screen. Charging under ES is limited to Pc,max=Eb,cap h⁻¹, ηc=0.92, and energy headroom.

The superscript “pk” does not denote a measured millisecond- or second-scale peak. The implementation omits polarization branches, relaxation, temperature, aging, cell imbalance, pack topology, charge-side voltage/current constraints, BMS/protection behavior, wiring loss, motor/ESC transients, and thermal accumulation [43-46]. The former power-to-energy condition is retained only as a diagnostic; it is not part of the activated closure event.

## S7. Joint closure and the excluded physical mission event

For one realization, the mass indicator is

IM=1MTO<25  (S24)

The battery energy and SOC indicator is

IE=1zmin≤zb,p≤zmax ∀p, zb,final≥zreserve  (S25)

The activated static voltage/current indicator is

IV=1Vt,p≥Vmin, Ib,p≤Imax,dis, Voc,p2-4000R0,pPb,ppk≥0 ∀p:Pb,ppk>0  (S26)

Fuel and generator-output indicators are

IF=1​p​mf,ptp≤1-rfmfλfϕf,  IG=1Pg,pon≤ξgPg,r ∀p  (S27)

The reported joint computational event is

ICD;x=IMIEIVIFIG  (S28)

All indicators are evaluated on the same mass, fuel, generator, and battery trajectory. Marginal probabilities are not multiplied. Fuel and generator limits are enforced inside dispatch, so Equations S27 also provide explicit ledger invariants.

A physically admissible mission would require a substantially larger event,

Imission=ICIflightIelectricalIthermalICGIstructuralIcontrolItransitionIoperation  (S29)

where the additional terms represent aircraft-specific thrust and rotor-speed margin; component-resolved bus, motor, ESC, protection, and transient limits; temperatures; center of gravity; structural loads; control authority; morphing and ground-mode success; and operational completion. None of these additional indicators is evaluated. In particular, Ioperation would have to include authorization, navigation, landing-site suitability, communications, safe standoff, suppressant deposition, handover, and professional response. A sample with IC=1 is therefore “model closed,” not flight capable, mission successful, or suppression effective.

## S8. Bounded uncertainty families

Inputs without empirical distributions were represented by bounded screening distributions rather than claimed fleet-frequency laws. Let a bounded quantity have minimum a, mode m, and maximum b. The triangular family uses the standard triangular inverse CDF. The beta-PERT family uses X=a+b-aY with Y∼Betaα,β and shape parameter λPERT=4,

α=1+λPERTm-ab-a,  β=1+λPERTb-mb-a.

Each bounded marginal is combined with either independent installed hardware-group masses or an equicorrelated Gaussian copula with ρ=0.30 for those mass groups. For the correlated alternative, a multivariate standard-normal vector Z with off-diagonal correlation 0.30 is transformed to uniforms Ui=ΦZi and then through the selected marginal inverse CDF. The value 0.30 is an illustrative common-cause integration-growth case, not an estimate from a prototype population. Fuel and solution fills remain independent triangular process-tolerance models in all four families. Unless otherwise stated, nonmass factors are sampled independently within their declared marginals.

The declared family set is

Θ={Tri​-​Ind,PERT​-​Ind,Tri​-​ρ0.30,PERT​-​ρ0.30}  (S30)

| Family | Marginal distribution | Hardware-mass dependence |
| --- | --- | --- |
| Tri–Ind | Triangular | Independent |
| PERT–Ind | Beta-PERT | Independent |
| Tri​-​ρ0.30 | Triangular | Positively correlated |
| PERT​-​ρ0.30 | Beta-PERT | Positively correlated |

At distance D, option x, and family θ, the empirical closure fraction is

PCD∣x,θ=1Nn=1NICnD;x,θ  (S31)

The assumption-bounded point estimate is the minimum across the four declared families,

PCΘD∣x=minθ∈ΘPCD∣x,θ  (S32)

For k successes in N realizations, the exact one-sided Clopper-Pearson lower limit is

LCPk;N,αq=0,k=0,B-1αq;k,N-k+1,k>0,  (S33)

where B-1 is the beta-distribution quantile. Three frozen options under four input families give J=12 predeclared holdout curves. A Bonferroni allocation sets αq=0.05/12. The confidence-qualified station screening radius is

D90,qΘ,CPx=supD:minθ∈ΘLCPKDx,θ;N,αq≥0.90  (S34)

The confidence procedure controls Monte Carlo sampling error within the 12 declared option-family holdout curves. It does not validate the assumed marginal families, the copula, the choice of ρ, or any physical submodel. Aerodynamic, technology, dispatch, fuel-credit, external-benchmark, and break-even calculations are deterministic or diagnostic sensitivities and carry no probability interpretation.

## S9. Discovery, freezing, holdout, and selection

The central hybrid design grid used generator ratings 3.0, 3.5, and 4.0 kW; fuel 0.50-2.00 kg in 0.25 kg increments; installed pack mass 1.20-3.20 kg in 0.20 kg increments; and LF or ES dispatch. Pure-electric candidates used installed pack masses of 3.00-7.00 kg in 0.25 kg increments. The central usable installed-pack specific energy was 0.35 kWh kg⁻¹. Candidate mass qualification required nominal mass below 25 kg and a multiplicity-adjusted exact 95% lower mass-compliance bound of at least 0.90.

Every mass-qualified point entered stochastic discovery. Discovery used 10,000 realizations per family, seed 20270815, common random numbers for option comparisons, a 40 km bracket, and ten bisections (0.0391 km resolution). It evaluated 123 LF hybrids, 123 ES hybrids, and 16 pure-electric candidates. Within each option, candidates were ranked by confidence-qualified radius, exact lower closure bound at 3.5 km, point-estimate radius, lower nominal wet mass, lower generator rating, lower fuel mass, lower pack mass, and finally lexical option label. The better hybrid policy was chosen from discovery only.

One LF design, one ES design, and one pure-electric design were frozen before holdout. Holdout used an independent 30,000-realization bank per family, seed 20270816, 12 bisections (0.00977 km resolution), and no reselection. At least 27,137 of 30,000 realizations had to close in every reported family for the adjusted exact lower bound to reach 0.90. The limiting holdout family was triangular with hardware-mass ρ=0.30 for all three options.

Increasing distance only increases cruise duration and source demand; it does not repair mass or electrical feasibility. The implementation verified that pass sets were nested in distance. Equation S34 is therefore an order-statistic inversion of one exact binomial acceptance rule rather than a collection of independent tests at each bisection point.

The frozen allocations were: LF hybrid, 4.0 kW generator, 1.75 kg fuel, and 1.20 kg/0.42 kWh pack at 23.50 kg nominal mass; ES hybrid, 4.0 kW, 1.00 kg fuel, and 2.00 kg/0.70 kWh pack at 23.55 kg; and pure electric, 6.75 kg/2.3625 kWh pack at 23.60 kg. Their holdout screening radii were 14.072, 8.467, and 7.207 km, respectively. These values are conditional screening boundaries and not validated range.

## S10. Restricted aerial-terrestrial break-even calculation

The comparator asks only whether replacing target-adjacent hover with a 200 m ground approach, 5 min application/observation interval, and 200 m withdrawal saves modeled energy after accounting for the morphing system. It does not compare operational risk or suppression effectiveness.

One 200 m ground leg at vg=1.5 m s⁻¹ lasts

tg=dg3.6vg=0.23.61.5=0.0370 h  (S35)

The complete target interval is 2tg+tapp=0.1574 h. Because the payload is released at the end of service, avoided hover energy respects the mass-state change:

Ehover,avoided=PHMloadedtg+tapp+PHMposttg  (S36)

Ground traction is Etraction=2Pgroundtg. At the central Pground=0.35 kW, traction requires 0.0259 kWh. Two 0.75 min transitions at 0.25 kW require 0.00625 kWh. The 0.35 kW application/observation debit is common to the two alternatives and cancels. Terrestrial service is energetically favorable only when

Ehover,avoided>ΔEcommon,morphD,Δmmorph,ΔCDAref+ETLR+Etr+Etraction  (S37)

Roots were solved over station distance from 0 to 40 km, morphing-mass penalty, drag-area increment 0/0.05/0.10 m², and ground power 0.35/0.75/1.00/1.50 kW. The energetic root was calculated independently of the separate mass-admissible ceiling. Relative to a 22.50 kg aerial-only comparator, the 25 kg engineering requirement permits at most a 2.50 kg morphing penalty. The 0.70-1.40 kg band shown in Supplementary Fig. S2 is an illustrative 20-40% share of the combined frame/morphing/wheel/lock group, not a measured mechanism mass.

At ΔCDAref=0.05 m² and 0.35 kW ground power, the unconstrained energetic mass roots are 9.73, 5.64, 3.89, and 1.87 kg at 5, 10, 15, and 30 km. Roots exceeding 2.50 kg are not mass admissible. The continuous zero-margin station-distance boundary at the 2.50 kg ceiling is 23.149 km. Increasing ground power to 1.50 kW reduces this boundary to 19.104 km and reduces the 30 km energetic root from 1.865 to 1.484 kg.

This sensitivity is not a terrain-capability model. Soil, ash, debris, slope, slip, turning, obstacles, route availability, clearance, tip-over stability, tire-soil interaction, landing, relaunch, transition success, heat, and smoke remain unmodeled [36,37]. Representative ground and transition trials are required before any operational benefit is claimed.

# Supplementary Results and Verification

## S11. Detailed holdout diagnostics

At the reported screening radii, the limiting triangular, ρ=0.30 family produced 27,159 closed realizations for LF, 27,151 for ES, and 27,137 for pure electric, each from 30,000 independent holdout realizations. The corresponding empirical fractions were 0.9053, 0.9050, and 0.9046, and the adjusted exact one-sided lower bounds were 0.900756, 0.900484, and 0.900007. The worst mass-compliance point estimates/exact lower bounds for the same frozen designs were 0.9634/0.9604, 0.9488/0.9454, and 0.9366/0.9328, respectively.

The battery-condition diagnostics were evaluated at each option’s common robust radius in the limiting family. LF showed marginal failure probabilities of 0.04047 for energy/SOC, 0.00220 for a negative current-solution discriminant, 0.02310 for terminal voltage, and 0.03553 for current. ES showed 0.05090, 0, 0.00027, and 0.00287. Pure electric showed 0.04010 energy/SOC failure and zero modeled discriminant, voltage, or current failures. The legacy CbEb condition produced only 0.00087 failure for LF and zero for ES and pure electric. These marginal events overlap and must not be added. The joint counts above, not any one marginal constraint, determine qualification.

At a common 3.5 km central mission, the separately sized LF hardware used 4.029 kWh chemical fuel input and 0.077 kWh battery-bus discharge, with no charge. The separately sized ES hardware used 3.608 kWh fuel input, 0.175 kWh discharge, and 0.008 kWh charge. On the same frozen LF hardware, ES used 3.597 kWh fuel input, 0.173 kWh discharge, and 0.006 kWh charge, compared with LF’s 4.029 and 0.077 kWh. Modeled mission-average generator efficiency was 0.20999 for ES and 0.20978 for LF. This replay indicates a fuel-battery substitution within the assumed map, not an independently demonstrated efficiency gain.

## S12. Deterministic architecture-stability diagnostics

The aerodynamic map re-optimized LF, ES, and pure electric at each combination of R=0.28/0.30/0.32 m and CDAref=0.25/0.35/0.45 m². LF hybrid had the larger deterministic station radius in all nine cells, with hybrid-minus-pure-electric differences from 10.112 to 11.885 km. In the central 0.30 m/0.35 m² cell, re-optimized deterministic radii were 20.649 km for hybrid and 8.970 km for pure electric. These values exceed the confidence-qualified holdout radii because they use central inputs and have no probability or confidence meaning.

The technology map re-optimized every architecture at installed usable-pack specific energies of 0.25/0.35/0.45 kWh kg⁻¹ and assumed generator peak efficiencies of 10.5/15.75/21%. Hybrid led in eight of nine cells. Pure electric led only at 0.45 kWh kg⁻¹ and 10.5% generator efficiency, by 0.645 km (12.036 versus 11.392 km). At 0.35 kWh kg⁻¹ and 10.5%, hybrid still led (11.128 versus 8.970 km). The cellwise reversal is an assumption boundary; the nine cells are not samples from a forecast technology distribution.

All 18 deterministic hybrid winners across the two maps used the 2.00 kg maximum tested fuel allocation. Only one winner in each map used the 4.0 kW maximum generator rating, while three technology-map winners used the 3.0 kW minimum tested rating. Six aerodynamic-map and five technology-map winners used the 1.20 kg minimum hybrid pack. The results therefore contain allocation-domain censoring and cannot be presented as continuous or global optima.

## S13. Numerical verification procedure

Verification tested whether the implementation followed the declared equations and bookkeeping. It did not test whether a physical vehicle follows those equations. The reference implementation read versioned inputs and exported candidate inventories, mass qualification, discovery and holdout outputs, constraint partitions, phase ledgers, power components, dispatch traces, deterministic sensitivity maps, break-even roots, an equation-code crosswalk, and a machine-readable run manifest.

Scalar and vector evaluations were compared for the aerodynamic model. The induced-velocity root was substituted back into Supplementary Equation S6 over masses of 18.5-25 kg and airspeeds of 0-16 m s⁻¹. Hover parasite power and power-component summation were checked as exact limiting identities. Generator tests reproduced the specified peak location, on-state and phase-average identities, available-output limits, and fuel budget. Battery tests substituted Equation S23 into Equation S22 and independently recomputed each phase energy state. Phase-by-phase mass, payload, fuel, source split, and final reserve were reconciled. Realization pass/fail status was checked for nesting across 81 distances on all 12 option-family holdout curves.

The exact-binomial implementation was tested at the threshold: 27,137/30,000 successes produced the adjusted lower bound 0.900007, while 27,136 produced 0.899973. Search coverage confirmed 123 LF, 123 ES, and 16 pure-electric mass-qualified candidates; each deterministic 3×3 map contained all nine cells. Every reported break-even root was substituted into the energy margin, and monotonicity was verified.

Maximum reported residuals were 5.33×10-15 m s⁻¹ for the induced relation, 1.44×10-15 kWh for generator energy bookkeeping, 4.55×10-13 W for the static battery circuit, 9.99×10-16 kWh for battery-state propagation, 1.42×10-14 kg for mass conservation, and 1.41×10-14 kWh for aerial-terrestrial roots. No distance-nesting or root-monotonicity violation was found. Supplementary Table S7 records the acceptance conditions and results.

## S14. Reproducibility status

This supplementary provides everything to replicate the study.

## S15. Interpretation of the external benchmarks

The three comparisons answer different questions. The Bauersfeld calculation supplies published-model context only; its measured-data error belongs to its own small quadrotor and cannot be inherited by MORPHFIRE. The UIUC comparison shows that the unretuned hover specialization reproduces independent isolated-propeller static power coefficients with 5.4% MAPE across 26 points. The NASA comparison shows that the same low-order expression underpredicts an installed four-rotor torque-derived rotor-system mechanical-power proxy by about 21% at a closely matched nondimensional loading. Taken together, the two measured-data benchmarks support using the equation structure as a screening decomposition while exposing substantial installation/model-form discrepancy.

The NASA factor is neither a correction to MORPHFIRE nor evidence that 1.27 is conservative for all conditions. Applying it after selection would be statistically and physically inconsistent. A future analysis should predeclare any discrepancy family, rerun mass/source allocation, and repeat independent holdout evaluation. Aircraft-specific validation still requires measured thrust-power-airspeed maps, motor/ESC electrical input, rotor interaction, configuration effects, climb and descent envelopes, and temperature.

Supplementary Tables

Supplementary Table S1. Symbols, definitions, and units. Grouped entries retain the notation needed to reproduce Equations S1-S37.

| Symbol | Definition | Unit |
| --- | --- | --- |
| Adisk, Aref | Total rotor-disk area; vehicle reference area | m² |
| CDAref | Equivalent vehicle drag area | m² |
| Cb, CI | Legacy power-to-energy ratio; current-rate screen | h⁻¹ |
| Cd0, cμ, kD | Mean blade-section profile-drag coefficient; profile advance-ratio multiplier; descent power factor | - |
| Cth,g; Tg,Ta,Tg,max | Generator heat capacity; generator, ambient, and allowable temperatures | kJ K⁻¹; °C |
| Ploss,g, UgAg | Generator loss power; effective heat-rejection conductance | kW; kW K⁻¹ |
| cp, g | Momentum-relation auxiliary quantity; gravitational acceleration | m² s⁻²; m s⁻² |
| D | One-way station distance | km |
| dg, dp | One-way ground distance; generator on-time duty fraction | km; - |
| D90,qΘ,CP | Exact-binomial confidence-qualified screening radius | km |
| Dpar,p, Tp | Parasite drag; steady flight thrust in phase p | N |
| Eb, Eb,cap | Nominal and realized installed usable battery energy | kWh |
| Eb,pstate | Battery usable-energy state in phase p | kWh |
| Etr, Etraction, ETLR, Ehover,avoided | Transformation, ground-traction, target landing/relaunch, and avoided-hover energy | kWh |
| eb | Usable installed-pack specific energy | kWh kg⁻¹ |
| IC, IM, IE, IV, IF, IG | Joint computational closure and its activated indicators | - |
| Imission | Expanded physical/operational mission indicator, not evaluated | - |
| J, N, KD, αq, LCP | Claim count, sample size, successes, allocated tail probability, exact lower limit | - |
| tp, tapp | Phase and application/observation durations | h |
| LHV | Fuel lower heating value | kWh kg⁻¹ |
| mb, mf, ms | Nominal battery, initial-fuel, and solution allocations | kg |
| mf,prem | Fuel remaining before phase p | kg |
| Δmf,p, Δms,p | Fuel burned and solution released in phase p | kg |
| mg, mNE, mNE,0 | Generator-system mass; realized and reference non-energy-system mass | kg |
| Δmmorph | Morphing-system mass penalty relative to aerial-only comparator | kg |
| ΔCDAref, ΔEcommon,morph | Morphing drag-area increment; common-flight energy penalty | m²; kWh |
| mi | Realized hardware-group mass | kg |
| Mloaded, Mpost | Loaded and post-release comparator masses | kg |
| Mp, Mppow | Physical and conservative power-sizing mass in phase p | kg |
| MTO, Mlim | Wet take-off mass; internal engineering mass limit | kg |
| P0, M0 | Legacy reference power and mass used only as a cross-check | kW; kg |
| PH, PC, PU, PD | Hover, cruise, climb, and descent phase-load functions | kW |
| Pflight,p | Modeled electrical flight power in phase p | kW |
| Pclimb,p, Paux, Pground | Additive climb, auxiliary, and ground-traction power | kW |
| Pi,p, Ppro,p, Ppar,p | Induced, profile, and parasite mechanical power | kW |
| Pg,a | Sampled available generator output, ξgPg,r | kW |
| Pg,r, Pg,pon, Pg,p | Rated, requested on-state, and phase-average generator power | kW |
| Pf,pon, mf,pon, mf,p | Chemical-input power, on-state fuel rate, and phase-average fuel rate | kW; kg h⁻¹ |
| Pb,p+, Pb,p-, Pb,ppk | Average discharge, average charge, and screened maximum discharge | kW |
| PL,p, Pc,max | Electrical load; pack charge-power limit | kW |
| Qb, R0, R1, C1 | Pack capacity, screening resistance, future polarization resistance/capacitance | Ah; ohm; ohm; F |
| rf, zreserve | Fuel reserve fraction; minimum final battery SOC | - |
| tout, tret, tg | Outbound/return cruise and one-way ground-travel time | h |
| Vp, va, vg, vi,p, vz,p, w∥ | Phase airspeed, cruise/ground speed, induced/vertical speed, along-track wind | m s⁻¹ |
| Vnom, Voc, Vt, Vmin | Nominal, open-circuit, terminal, and minimum pack voltage | V |
| x | Energy-allocation design vector | - |
| PC, B-1 | Empirical closure fraction; beta-distribution quantile | - |
| zb, Ib | Battery state of charge; pack current | -; A |
| zb,pscr, zmin, zmax, Imax,dis | Screened SOC, SOC bounds, and maximum discharge current | -; A |
| a0,a1,a2 | Normalized generator chemical-input coefficients | - |
| ηdrv, ηc, ηd | Propulsion-chain, charge, and discharge efficiency | - |
| ηg, λp, λ* | Generator efficiency, on-state load fraction, preferred fraction | - |
| ηr | Route-efficiency factor | - |
| ϕf | Usable-fuel fraction | - |
| πg | LF or ES generator-dispatch policy | - |
| λf, λs | Fuel- and solution-fill multipliers | - |
| ξb, ξg, ξη, ξP | Battery energy, generator output, generator map, and flight-power factors | - |
| κ, μp, σ | Induced-power factor, rotor advance ratio, rotor solidity | - |
| Ω, R, Nr | Rotor angular speed, radius, and rotor count | rad s⁻¹; m; - |
| ρ | Hardware-mass equicorrelation in dependence case | - |
| ρa | Air density | kg m⁻³ |
| Θ | Declared set of four screening families | - |
| Iflight, Ielectrical, Ithermal, ICG | Excluded flight, expanded-electrical, thermal, and centre-of-gravity indicators | - |
| Istructural, Icontrol, Itransition, Ioperation | Excluded structural, control, transition, and operational indicators | - |

Supplementary Table S2. Phase-resolved reference mission. The generator is enabled only for hybrid candidates in the marked phases. Payload release occurs at the end of application/observation.

| Phase | Reference duration or distance | Electrical-load model | Generator | State change |
| --- | --- | --- | --- | --- |
| Post-authorization preflight checks | 2.0 min | 0.12 kW auxiliary | Off | Computed sortie begins after authorization |
| Outbound climb | 1.0 min | PUMppow | On | Fuel burn if dispatched |
| Outbound cruise | toutD | PCMppow at 12 m s⁻¹ | On | Fuel burn if dispatched |
| Descent and landing | 0.5 min | PDMppow | On | Fuel burn if dispatched |
| Flight-to-ground transition | 0.75 min | 0.25 kW | Off | No mass change |
| Ground approach | 0.2 km at 1.5 m s⁻¹ | 0.35 kW central | Off | No mass change |
| Application and observation | 5.0 min | 0.35 kW | Off | 5 kg nominal solution released at phase end |
| Ground withdrawal | 0.2 km at 1.5 m s⁻¹ | 0.35 kW central | Off | Post-release mass |
| Ground-to-flight transition | 0.75 min | 0.25 kW | Off | No mass change |
| Return climb | 1.0 min | PUMppow | On | Fuel burn if dispatched |
| Return cruise | tretD | PCMppow at 12 m s⁻¹ | On | Fuel burn if dispatched |
| Positioning contingency | 1.0 min | PHMppow | On | Fuel burn if dispatched |
| Return descent and landing | 0.5 min | PDMppow | On | Fuel burn if dispatched |

Supplementary Table S3a. Principal system, mission, and design-space inputs. All values are analytical assumptions unless the evidence-status column states otherwise.

| Category | Parameter | Value used | Evidence status |
| --- | --- | --- | --- |
| Scope | Operational role | Authorized, pre-positioned conceptual early-intervention support; one vehicle/one sortie | No suppression, fleet, or readiness claim |
| Architecture | Mobility modes | Quadrotor flight and wheeled ground travel | Mechanism concept in [16]; no integrated prototype used here |
| Architecture | Rotor count, Nr | 4 | Concept fixed; hardware not selected |
| Mass | Internal wet-mass limit, Mlim | 25 kg | Engineering requirement, not regulatory classification |
| Mass | Reference non-energy-system mode, mNE,0 | 11.85 kg | Sum of screening modes, not weighed BOM |
| Payload | Premixed solution, ms | 5.00 kg | 5 L represented as 5 kg; formulation/density unselected |
| Flight | Cruise airspeed, va | 12 m s⁻¹ | Mission assumption |
| Flight | Climb translation/vertical speed | 6/2 m s⁻¹ | Mission assumption; trajectory not optimized |
| Flight | Descent power factor, kD | 0.65 | Unvalidated scalar assumption |
| Flight | Rotor radius, R | 0.30 m central; 0.28/0.30/0.32 m map | Parametric geometry |
| Flight | Total disk area, Adisk | 1.131 m² at R=0.30 m | Calculated |
| Flight | Air density, ρa | 1.225 kg m⁻³ | Fixed reference value |
| Flight | Solidity, σ | 0.09 | Author screening input |
| Flight | Section profile-drag coefficient, Cd0 | 0.012 | Author screening input |
| Flight | Rotor tip speed, ΩR | 180 m s⁻¹ | Author screening input |
| Flight | Profile advance-ratio coefficient, cμ | 4.65 | Author screening input |
| Flight | Induced-loss factor, κ | 1.15 | Author screening input |
| Flight | Equivalent drag area, CDAref | 0.35 m² central; 0.25/0.35/0.45 m² map | Parametric, not measured |
| Flight | Drivetrain efficiency, ηdrv | 0.82 | Author screening input |
| Flight | Auxiliary power, Paux | 0.12 kW | Author screening input |
| Flight | Legacy cross-check anchor P0,M0 | (6.0 kW, 24.8 kg) | Not used in present flight-power law |
| Route | Route-efficiency factor, ηr | 0.90/0.95/1.00 | Bounded scenario, no route dataset |
| Route | Along-track wind, w∥ | -4/0/+4 m s⁻¹ | Bounded steady scenario, not site climatology |
| Ground | One-way distance, dg | 0.2 km | Mission assumption |
| Ground | Speed, vg | 1.5 m s⁻¹ | Mission assumption |
| Ground | Traction power, Pground | 0.35 kW central; 0.35/0.75/1.00/1.50 kW sensitivity | No terrain calibration |
| Ground | Transition demand | 0.25 kW for 0.75 min each direction | No actuator/lock test |
| Service | Application/observation | 5 min at 0.35 kW | Duration/load assumption; effect unmodeled |
| Service | Suppression standoff | Operational parameter not resolved | No trajectory, deposition, or safe-distance inference |
| Reserve | Battery/fuel reserve | 20%/10% | Design assumptions |
| Hybrid grid | Generator rating | 3.0/3.5/4.0 kW | Discrete installed-output assumptions |
| Hybrid grid | Fuel allocation | 0.50-2.00 kg in 0.25 kg increments | Bounds of search, not tank certification |
| Hybrid grid | Installed pack mass | 1.20-3.20 kg in 0.20 kg increments | Complete pack assumption |
| Pure-electric grid | Installed pack mass | 3.00-7.00 kg in 0.25 kg increments | Complete pack assumption |
| Technology | Usable installed-pack specific energy, eb | 0.35 kWh kg⁻¹ central; 0.25/0.35/0.45 map | Technology scenarios, not selected pack |
| Technology | Generator peak efficiency | 21% central; 10.5/15.75/21% map | Author-defined scenarios, not measured map |

Supplementary Table S3b. Reference installed subsystem-mass basis. Lower and upper fractions define bounded marginal support about each listed mode; they are not confidence intervals. Generator, fuel, battery, and payload entries are replaced by architecture variables and fill models in Equation S2.

| Installed subsystem group | Mode (kg) | Lower fraction | Upper fraction | Treatment |
| --- | --- | --- | --- | --- |
| Frame, arms, landing load path, morphing mechanism, wheels, and locks | 3.50 | 0.10 | 0.20 | Non-energy basis; combined group only |
| Motors, propellers, and electronic speed controllers | 4.25 | 0.05 | 0.10 | Non-energy basis |
| Generator module at 3.0 kW reference | 3.10 | 0.08 | 0.15 | Replaced by rating-to-mass map |
| Fuel reference allocation | 2.00 | 0.05 | 0.05 | Replaced by mfλf |
| Buffer battery reference | 1.55 | 0.08 | 0.12 | Replaced by realized mb |
| Premixed solution | 5.00 | 0.02 | 0.02 | Fixed nominal payload and fill tolerance |
| Tank, pump, nozzle, and plumbing | 1.60 | 0.10 | 0.20 | Non-energy basis |
| Avionics, sensors, and communications | 0.90 | 0.10 | 0.15 | Non-energy basis |
| Integration and contingency | 1.60 | 0.15 | 0.30 | Screening basis combining hardware allowance and margin |

The combined frame/morphing/wheel/lock group is 29.5% of the non-energy-system mode and 14.0% of the 25 kg limit. The available evidence does not identify a mechanism-only mass. The 0.70-1.40 kg visualization band is therefore illustrative; it must not be cited as a measured morphing allocation. A future BOM should separate frame, landing path, links, joints, actuators, wheel hubs, locks, fasteners, and design contingency.

Supplementary Table S4. Generator, fuel, battery, and electrical-screen inputs. Values are activated unless marked “future” or “diagnostic.”

| Subsystem | Parameter | Value or model | Status/interpretation |
| --- | --- | --- | --- |
| Generator | Installed rating-to-mass map | 3.0 kW/3.10 kg; 3.5 kW/3.35 kg; 4.0 kW/3.70 kg | Screening map; no installed system weighed |
| Generator | Available output | Pg,a=ξgPg,r | Activated |
| Generator | Output factor, ξg | 0.90/1.00/1.05 | Bounded epistemic screen |
| Generator | Chemical-input coefficients | a0=0.15, a1=4.3869, a2=0.234375 | Assumed normalized scenario |
| Generator | Map scale, ξη | 0.90/1.00/1.10 | Bounded epistemic screen |
| Generator | Preferred load fraction, λ* | 0.80 | Mathematical optimum of assumed map |
| Generator | Peak efficiency | 0.21 central | Favorable unvalidated target |
| Generator | Dispatch | LF and ES | Both evaluated and separately sized |
| Generator | Charge-eligible phases | Climb, descent/landing, positioning | Cruise surplus curtailed; nonflight phases disabled |
| Generator | Thermal state | Equation S17 | Future; excluded from IC |
| Fuel | Lower heating value, LHV | 11.94 kWh kg⁻¹ | 43 MJ kg⁻¹ screening input; grade unselected |
| Fuel | Usable fraction, ϕf | 0.75/0.80/0.85 | Bounded screen |
| Fuel | Fill multiplier, λf | Triangular 0.95/1.00/1.05 | Independent process tolerance |
| Fuel | Reserve, rf | 0.10 | Activated |
| Battery | Central installed usable specific energy, eb | 0.35 kWh kg⁻¹ | Technology target; includes complete pack |
| Battery | Energy factor, ξb | 0.90/1.00/1.10 | Bounded epistemic screen |
| Battery | Initial usable energy | Eb,0state=Ebξb | Activated |
| Battery | Discharge efficiency, ηd | 1.00 at usable DC-bus boundary | Activated definition |
| Battery | Charge efficiency, ηc | 0.92 | Activated |
| Battery | Final/minimum SOC reserve | 0.20 | Activated |
| Battery | Nominal voltage, Vnom | 96 V | Static screen |
| Battery | Open-circuit voltage | Vocz=960.90+0.20z V | Static screen |
| Battery | Minimum terminal voltage, Vmin | 76.8 V | 0.80Vnom |
| Battery | Resistance | R0=0.18/Eb,cap ohm | Central screen; capacity in kWh |
| Battery | Resistance factor | 0.80/1.00/1.30 | Bounded epistemic screen |
| Battery | Capacity conversion | Qb=1000Eb,cap/Vnom Ah | Activated |
| Battery | Current-rate factor, CI | 4/5/6 h⁻¹ | Bounded epistemic screen |
| Battery | Maximum discharge current | Imax,dis=CIQb | Activated |
| Battery | Maximum charge power | Pc,max=Eb,cap h⁻¹ | Activated for ES surplus |
| Battery | Legacy power ratio, Cb | 8/12/20 h⁻¹ | Diagnostic only; not closure criterion |
| Battery | Polarization/thermal model | R1C1, temperature, aging, imbalance | Future; excluded from IC |
| Payload | Fill multiplier, λs | Triangular 0.98/1.00/1.02 | Independent process tolerance |

Supplementary Table S5. Uncertainty and deterministic-sensitivity ledger. Probabilistic labels describe sampling inside the computational model, not empirical fleet frequencies.

| Quantity | Values or implementation | Classification | Data status |
| --- | --- | --- | --- |
| Hardware-group mass | Table S3b supports; triangular or beta-PERT marginals | Epistemic screening distribution | BOM not frozen or weighed |
| Hardware-mass dependence | Independent or Gaussian copula with ρ=0.30 | Epistemic dependence alternative | Illustrative; not estimated |
| Rotor radius | 0.28/0.30/0.32 m | Deterministic model-form/geometry cells | Hardware not frozen |
| Equivalent drag area | 0.25/0.35/0.45 m² | Deterministic model-form cells | Not measured |
| Residual flight-power factor, ξP | 0.92/1.00/1.15 | Epistemic/model-discrepancy screen | Vehicle map absent; does not cover NASA 1.27 case |
| Generator-output factor, ξg | 0.90/1.00/1.05 | Epistemic screen | Installed bench map absent |
| Generator map factor, ξη | 0.90/1.00/1.10 | Epistemic screen | Generic map not calibrated |
| Generator dispatch | LF or ES | Policy alternative | No controller test |
| Battery-energy factor, ξb | 0.90/1.00/1.10 | Epistemic screen | Pack test absent |
| Battery resistance factor | 0.80/1.00/1.30 | Epistemic static-circuit screen | Pack identification absent |
| Battery current factor, CI | 4/5/6 h⁻¹ | Epistemic electrical screen | Cell/topology unselected |
| Along-track wind | -4/0/+4 m s⁻¹ | Environmental route scenario | Site distribution not inferred |
| Route efficiency, ηr | 0.90/0.95/1.00 | Operational route scenario | Route dataset absent |
| Usable-fuel fraction, ϕf | 0.75/0.80/0.85 | Epistemic screen | Fuel-system test absent |
| Fuel fill | Independent triangular ±5% | Controlled-process tolerance model | Metering data absent |
| Solution fill | Independent triangular ±2% | Controlled-process tolerance model | Metering data absent |
| Technology map | eb=0.25/0.35/0.45 kWh kg⁻¹; generator peak 0.105/0.1575/0.210 | Deterministic technology cells | Author scenarios; no occurrence weights |
| Ground-power map | 0.35/0.75/1.00/1.50 kW | Deterministic terrain-demand screen | Ground testing absent |
| Morphing drag increment | 0/0.05/0.10 m² | Deterministic break-even screen | Configuration drag absent |

Supplementary Table S6a. External rotor-power benchmark summary without coefficient refitting. MAPE is mean absolute percentage error. UIUC NRMSE is root-mean-square CP error normalized by the mean measured CP. A positive signed bias denotes prediction above measurement.

| Evidence level/data source | Points | Measured or published quantity | Present result | Interpretation |
| --- | --- | --- | --- | --- |
| Published multicopter model [22] | Contextual model ensemble/aircraft-specific flight validation | Pe/Ph=0.914±0.0323; its own model power RMSE 2.5% | Pmin/Phover=0.898 at 11.24 m s⁻¹ | Context only; its validation does not transfer to MORPHFIRE |
| Published momentum hover estimator [22] | Contextual matched calculation | Figure of merit 0.60 | 3.503 kW benchmark versus 3.508 kW present rotor power | Cross-model context only; not a measurement benchmark |
| UIUC APC-E 18x8 [53] | 11 | Static isolated-propeller CT,CP | MAPE 8.01%; signed bias +4.27% | Coefficient-domain partial validation |
| UIUC APC-E 20x10 [53] | 8 | Static isolated-propeller CT,CP | MAPE 3.89%; signed bias +2.45% | Coefficient-domain partial validation |
| UIUC APC-E 21x13 [53] | 7 | Static isolated-propeller CT,CP | MAPE 3.04%; signed bias -1.45% | Coefficient-domain partial validation |
| UIUC combined [53] | 26 | Static isolated-propeller CT,CP | MAPE 5.41%; signed bias +2.17%; NRMSE 6.27%; max absolute error 16.54% | Supports isolated-rotor hover decomposition conditional on measured thrust |
| NASA MTB2 run 136 [54,55] | 2 | Installed four-rotor torque-derived rotor-system mechanical-power proxy at 3000 rpm | Prediction errors -21.0% and -21.5% | Closely matched CT exposes installed-system/model discrepancy |

For the UIUC calculation, di=CP,pred,i-CP,meas,i and ei=100di/CP,meas,i. Signed bias is N-1i​ei, MAPE is N-1i​ei, RMSECP=N-1i​di21/2, and NRMSE=100RMSECP/CP,meas. Across all 26 points, RMSECP=0.00190568.

Supplementary Table S6b. All 26 UIUC static coefficient points used in the benchmark [53]. Source files were apce_18x8_static_2184od.txt, apce_20x10_static_2210od.txt, and apce_21x13_static_2218od.txt. Dimensional power is not reconstructed because the published coefficient files do not supply a per-run air density and exact as-tested diameter.

| APC-E dataset | Speed (rpm) | Measured CT | Measured CP | Predicted CP | Error (%) |
| --- | --- | --- | --- | --- | --- |
| 18x8 | 973.333 | 0.075677 | 0.026827 | 0.02238977 | -16.5402 |
| 18x8 | 1493.333 | 0.082373 | 0.026029 | 0.02498034 | -4.0288 |
| 18x8 | 1953.333 | 0.085557 | 0.025885 | 0.02625017 | +1.4107 |
| 18x8 | 2446.667 | 0.088769 | 0.025939 | 0.02755533 | +6.2313 |
| 18x8 | 2960.000 | 0.088479 | 0.025679 | 0.02743650 | +6.8441 |
| 18x8 | 3460.000 | 0.088198 | 0.025643 | 0.02732155 | +6.5459 |
| 18x8 | 3960.000 | 0.089937 | 0.025967 | 0.02803586 | +7.9673 |
| 18x8 | 4446.667 | 0.091241 | 0.026248 | 0.02857605 | +8.8694 |
| 18x8 | 4953.333 | 0.092181 | 0.026452 | 0.02896785 | +9.5110 |
| 18x8 | 5446.667 | 0.093322 | 0.026812 | 0.02944612 | +9.8244 |
| 18x8 | 5946.667 | 0.094663 | 0.027197 | 0.03001198 | +10.3503 |
| 20x10 | 1060.000 | 0.091802 | 0.030574 | 0.02880964 | -5.7708 |
| 20x10 | 1466.667 | 0.094813 | 0.029984 | 0.03007552 | +0.3052 |
| 20x10 | 1986.667 | 0.098835 | 0.030593 | 0.03179801 | +3.9388 |
| 20x10 | 2460.000 | 0.097484 | 0.030566 | 0.03121543 | +2.1247 |
| 20x10 | 2946.667 | 0.099369 | 0.030984 | 0.03202938 | +3.3739 |
| 20x10 | 3453.333 | 0.100422 | 0.031109 | 0.03248745 | +4.4310 |
| 20x10 | 3960.000 | 0.103124 | 0.031918 | 0.03367384 | +5.5011 |
| 20x10 | 4473.333 | 0.104020 | 0.032237 | 0.03407072 | +5.6882 |
| 21x13 | 1053.333 | 0.101138 | 0.036871 | 0.03280029 | -11.0404 |
| 21x13 | 1500.000 | 0.105801 | 0.036032 | 0.03486468 | -3.2397 |
| 21x13 | 1966.667 | 0.108120 | 0.036218 | 0.03590853 | -0.8545 |
| 21x13 | 2460.000 | 0.108242 | 0.036183 | 0.03596376 | -0.6059 |
| 21x13 | 2960.000 | 0.108776 | 0.035902 | 0.03620587 | +0.8464 |
| 21x13 | 3466.667 | 0.110343 | 0.036162 | 0.03691974 | +2.0954 |
| 21x13 | 3940.000 | 0.111286 | 0.036397 | 0.03735180 | +2.6233 |

Supplementary Table S6c. NASA MTB2 installed-quadrotor hover points [54,55]. Both points are equal-height, four-rotor nominal-hover measurements from run 136 at 3000 rpm. The mechanical-power proxy is derived from measured hub-frame reaction torque and angular speed; error is Ppred-Pproxy/Pproxy.

| Point | Rotor phase arrangement | CT | Torque-derived power proxy (W) | Predicted power (W) | Error (%) | Proxy/predicted |
| --- | --- | --- | --- | --- | --- | --- |
| 18 | 0° | 0.005647 | 801.1 | 633.0 | -21.0 | 1.266 |
| 47 | 0/-90° | 0.005541 | 789.4 | 619.7 | -21.5 | 1.274 |

Supplementary Table S7. Numerical verification matrix. “Passed” establishes agreement between code and equations, not physical validation.

| Check | Acceptance condition | Result |
| --- | --- | --- |
| Aerodynamic equations | Scalar/vector agreement; induced-root substitution; zero hover parasite power; component sum | Passed; maximum induced residual 5.33×10-15 m s⁻¹; other listed residuals zero |
| Generator map/dispatch | Reproduce λ*=0.80; output/fuel limits; duty-cycle energy identity | Passed; peak error 1.11×10-16; max energy residual 1.44×10-15 kWh |
| Battery circuit/state | Equation S22 identity and independent phase-state propagation | Passed; maximum circuit residual 4.55×10-13 W; state residual 9.99×10-16 kWh |
| Mass/source conservation | Phase mass, fuel, load split, requested/average output | Passed; mass residual 1.42×10-14 kg; no rating excess |
| Distance nesting | No probability increase or realization-level pass recovery as distance increases | Passed on 12 curves over 81 distances; zero violations |
| Exact-binomial threshold | 27,137/30,000 passes; one fewer success fails | Passed; lower bounds 0.900007 and 0.899973 |
| Exhaustive central grid | Every mass-qualified candidate enters discovery | Passed; 123 LF, 123 ES, 16 pure electric |
| Deterministic maps | All 3×3 aerodynamic and technology cells searched and re-optimized | Passed; nine cells each; no architecture search reached 40 km bracket |
| Aerial-terrestrial roots | Direct energy difference closes at each root and remains monotone | Passed; maximum residual 1.41×10-14 kWh; zero monotonicity violations |
| Frozen package | Source/input/output hashes match manifest | Passed; overall invariant status PASS; 34 result files listed |

Supplementary Table S8. Physical-validation gates required before stronger claims. The external rotor benchmarks reduce a limited part of gate V2 model-form uncertainty but do not complete any gate.

| Gate | Required evidence | Present quantity replaced or added | Claim enabled after successful validation |
| --- | --- | --- | --- |
| V1: installed mass | Named installed BOM; calibrated weighing of components, fluids, wiring, fasteners, enclosure, cooling, and protection | Table S3 mass modes/bounds and generator mass map | Component-backed mass feasibility |
| V2: flight power | Selected rotor/motor/ESC geometry; electrical and mechanical hover/cruise/oblique-climb/descent maps; interaction, VRS/turbulent-wake envelope; current/temperature | Equations S4-S10, additive climb, scalar descent factor, and ξP | Aircraft-specific power, thrust margin, and safe flight envelope within tests |
| V3: battery | Pack usable energy, Voc, R0+R1C1, mission pulses, charge limits, temperature, aging, enclosure/BMS mass | Equations S18-S23, eb, ξb, CI, reserve | Pack-level energy, transient electrical, and thermal closure |
| V4: generator | Installed continuous/part-load output and fuel after parasitics; transients, cooling, exhaust, vibration, rectification, derating, elevated ambient/heat flux | Equations S11-S17, installed mass map, ξg, ϕf, LF/ES rules | Physically comparable hybrid source and thermal-dispatch model |
| V5: morphing and ground | Mechanism mass, actuator torque/energy, lock loads, transition success, traction, slope, obstacles, representative surfaces | Equation S37, illustrative mass band, fixed transition/ground loads | Measured aerial-terrestrial break-even and ground/transition feasibility |
| V6: integrated mission | Synchronized mass, power, SOC, fuel, wind, temperature, configuration, and position over representative outbound-service-return trials | Cross-domain interactions and residual discrepancy | Validated single-vehicle performance within tested envelope |
| V7: operational/fire-effect study | Agency-approved concept of operations; detection/authorization timing; safe-site/standoff rules; deposition; controlled-burn effect; responder/airspace handover; fleet trials | Excluded Ioperation, fire effect, station/fleet model | Only the precisely tested operational and fire-effect claim; never replacement of professional response |

Supplementary Table S9. Prior-publication extension and claim boundary. The related conference paper [16] should be supplied with the submission. This table is a transparency aid and does not replace journal disclosure of related manuscripts.

| Aspect | Prior MorphoCopter paper [16] | Present article and supplement | Claim boundary |
| --- | --- | --- | --- |
| Mechanism | Inverted slider-crank transformation geometry and conceptual operating principle | Used as physical motivation; combined mass group and break-even penalty are parameterized | No measured mechanism mass, strength, or transition reliability |
| Mission | Conceptual bimodal wildfire-suppression motivation | Explicit authorized, pre-positioned early-intervention case; one phase-resolved sortie | No demonstrated suppression, containment, or operational readiness |
| Installed mass | Conceptual vehicle description | Joint sampled installed-mass ledger under 25 kg engineering limit | No weighed BOM or regulatory conclusion |
| Energy architecture | Not closed | Exhaustive series-hybrid LF/ES and pure-electric allocation screen | Best tested discrete allocations, not global optima |
| Flight power | Conceptual performance relation | Induced/profile/parasite decomposition, external trend and measurement benchmarks | Partial model-form evidence only; no matched-aircraft validation |
| Generator/fuel | Not coupled to complete mission allocation | Part-load scenario, LF/ES dispatch, fuel duty and reserve | Author-defined map; no installed thermal/fuel bench evidence |
| Battery | Mission-energy abstraction | Usable-energy state plus static terminal-voltage/current screen | No dynamic/electrothermal pack validation |
| Uncertainty | Not confidence-qualified | Four bounded families; independent holdout; adjusted exact lower bounds | Model-conditional sampling guarantee, not fleet reliability |
| Bimodal comparison | Qualitative premise | Mass/drag/ground-power break-even roots and mass ceiling | Energetic screen only; no terrain or transition demonstration |
| Reproducibility | Not equivalent to present package | Fixed seeds, hashes, verification matrix, archived-release requirement | Repository DOI must be completed before submission |

# Supplementary Figures

Supplementary Figure S1. Scope-controlled computational workflow and evidence boundary. The upper lane propagates declared installed mass, mission, aerodynamic, generator, dispatch, and static battery inputs through one phase-resolved trajectory and joint closure event before multiplicity-adjusted exact one-sided distance qualification. The lower lane contains unmodeled physical and operational gates, including authorization, response-system integration, fire-spread effect, and professional-resource handover. These gates are excluded from the closure probability. A screening radius cannot establish aircraft reliability, validated range, fleet reliability, transition success, safety, propagation delay, wildfire-treatment effectiveness, or replacement of conventional response resources.

Supplementary Figure S2. Restricted hover-versus-ground break-even boundary. Curves are unconstrained energetic roots for a 22.50 kg aerial-only comparator. The 2.50 kg horizontal line is the separate mass-admissible ceiling under the 25 kg engineering requirement. The shaded 0.70-1.40 kg band is illustrative and is not a measured mechanism allocation. Hover avoidance is evaluated at loaded mass through approach/service and at post-release mass during withdrawal. The curves are assumption-conditional energy boundaries, not measured ground performance, safety evidence, or wildfire-suppression performance.

# Supplementary declaration and completion notes

No human participants, human data, animals, or biological materials were involved. Ethics approval and consent were not applicable. No generative-image model or generative AI was used except for language polishing and conceptual diagram designing.

# Supplementary reference note

All citations refer to the numbered reference list in the main article. In particular, the external measurement benchmarks are Dantsker et al. and the UIUC Propeller Data Site [53], the NASA MTB2 data report and official dataset [54], and the associated installed-quadrotor experimental study [55].

# Supplementary Software

Quantitative figures were generated under Ubuntu 24.04.3 LTS using Python 3.12.13, Matplotlib 3.10.8, NumPy 2.3.5, SciPy 1.17.0 and Pillow 12.3.0. No generative-image model or generative AI was used except for language polishing and conceptual diagram designing.