Supplementary Information for

Comparing geospatial interpolation techniques for estimating ambient PM2.5 mass concentrations from sugarcane burning in South Florida

Anne Mielnik,1 Olivia Sablan,2 Jeffrey R. Pierce,2 Lilah M. Besser,3 Juyoung Park,4 Diana Mitsova,5 Diane Cook,6 Sherry WeMott,1 Christine L. Williams,7 Lisa A. K. Wiese,7 & Sheryl Magzamen1*

- Department of Environmental and Radiological Health Sciences, Colorado State University, Fort Collins, Colorado, 80523-1019, United States

- Department of Atmospheric Science, Colorado State University, Fort Collins, Colorado, 80523-1371, United States

- Comprehensive Center for Brain Health, University of Miami School of Medicine, Boca Raton, Florida, 33433, United States

- College of Nursing, University of Arizona, Tucson, Arizona 85721-0203, United States

- Department of Urban and Regional Planning, Florida Atlantic University, Boca Raton, Florida, 33431-0991, United States

- School of Electrical Engineering and Computer Science, Washington State University, Pullman, Washington, 99164-2752, United States

- C.E. Lynn College of Nursing, Florida Atlantic University, Boca Raton, Florida, 33431- 0991, United States

*Corresponding Author:

Sheryl Magzamen

Department of Environmental and Radiological Health Sciences

Colorado State University

1618 Campus Delivery

Fort Collins, CO 80523-1618

sheryl.magzamen@colostate.edu

Supplementary Information (19 pages) includes 10 tables and 10 figures.

Figure S1. Measured hourly maximum PM2.5 mass concentrations for each PurpleAir sensor and EPA AQS FRM monitor (listed by ID/site number) with dashed lines at 20 g/m3 and gray shaded areas on days NOAA HMS smoke plumes overlap PM2.5 measurements (i.e., illustrating smoke-impacted day criteria “C1”).

Table S1. Summary statistics for estimated 24-hour average PM2.5 concentrations from OK models with a Bessel fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 5.77 | 9.07 | 0.048 | 4.18 | 5.26 | 6.93 | 1.65x104 |
| Summer (JJA) | 133 | 3.46 | 1.58 | 0 | 2.51 | 3.25 | 3.94 | 14.0 |
| Fall (SON) | 134 | 3.66 | 1.80 | 0.058 | 2.46 | 3.13 | 4.45 | 14.7 |
| Winter (DJF) | 86 | 3.93x107 | 7.09x109 | 0 | 3.51 | 4.46 | 5.19 | 1.28x1012 |
| Smoke (C1) | 169 | 5.28 | 6.86 | 0 | 3.70 | 4.90 | 6.51 | 1.65x103 |
| No smoke (C1) | 276 | 1.22x107 | 3.96x109 | 0 | 2.53 | 3.40 | 4.28 | 1.28x1012 |
| Smoke (C2) | 247 | 1.36x107 | 4.19x109 | 0 | 3.41 | 4.53 | 6.18 | 1.28x1012 |
| No smoke (C2) | 198 | 3.37 | 1.55 | 0 | 2.34 | 3.11 | 4.02 | 14.0 |
| Overall | 445 | 7.59x106 | 3.12x109 | 0 | 2.84 | 3.83 | 5.08 | 1.28x1012 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). Note: Models predicted values well below 0 g/m3, which are represented as “0” in table. “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S2. Summary statistics for estimated 24-hour average PM2.5 concentrations from UK models with a Bessel fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 6.33 | 124 | 0 | 4.09 | 5.13 | 6.90 | 2.31x104 |
| Summer (JJA) | 133 | 3.49 | 1.69 | 0 | 2.51 | 3.27 | 3.98 | 17.5 |
| Fall (SON) | 134 | 3.73 | 2.06 | 0.051 | 2.36 | 3.23 | 4.62 | 23.8 |
| Winter (DJF) | 86 | 3.36x107 | 6.08x109 | 0 | 3.38 | 4.47 | 5.39 | 1.10x1012 |
| Smoke (C1) | 169 | 5.65 | 91.5 | 0 | 3.64 | 4.81 | 6.49 | 2.31x104 |
| No smoke (C1) | 276 | 1.04x107 | 3.39x109 | 0 | 2.43 | 3.39 | 4.34 | 1.09x1012 |
| Smoke (C2) | 247 | 1.17x107 | 3.59x109 | 0 | 3.34 | 4.50 | 6.11 | 1.10x1012 |
| No smoke (C2) | 198 | 3.39 | 1.65 | 0 | 2.34 | 3.20 | 4.05 | 17.5 |
| Overall | 445 | 6.50x106 | 2.67x107 | 0 | 2.77 | 3.82 | 5.13 | 1.10x1012 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). Note: Models predicted values well below 0 g/m3, which are represented as “0” in table. “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S3. Summary statistics for estimated 24-hour average PM2.5 concentrations from OK models with an exponential fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n |  | Mean |  | SD |  | Min. |  | Q1 |  | Median |  | Q3 |  | Max. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) |  | 92 |  | 5.72 |  | 2.01 |  | 1.34 |  | 4.19 |  | 5.25 |  | 6.87 |  | 13.3 |
| Summer (JJA) |  | 133 |  | 3.46 |  | 1.57 |  | 0.001 |  | 2.55 |  | 3.25 |  | 3.95 |  | 12.0 |
| Fall (SON) |  | 134 |  | 3.66 |  | 1.80 |  | 0.084 |  | 2.45 |  | 3.16 |  | 4.45 |  | 11.1 |
| Winter (DJF) |  | 86 |  | 4.82 |  | 2.30 |  | 0.050 |  | 3.49 |  | 4.44 |  | 5.20 |  | 16.2 |
| Smoke (C1) |  | 169 |  | 5.25 |  | 2.17 |  | 0.083 |  | 3.72 |  | 4.90 |  | 6.43 |  | 14.6 |
| No smoke (C1) |  | 276 |  | 3.63 |  | 1.77 |  | 0.001 |  | 2.51 |  | 3.40 |  | 4.32 |  | 16.2 |
| Smoke (C2) |  | 247 |  | 4.96 |  | 2.20 |  | 0.050 |  | 3.41 |  | 4.51 |  | 6.16 |  | 16.2 |
| No smoke (C2) |  | 198 |  | 3.37 |  | 1.54 |  | 0.001 |  | 2.36 |  | 3.09 |  | 4.01 |  | 12.0 |
| Overall |  | 445 |  | 4.25 |  | 2.09 |  | 0.001 |  | 2.83 |  | 3.85 |  | 5.10 |  | 16.2 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S4. Summary statistics for estimated 24-hour average PM2.5 concentrations from UK models with an exponential fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 5.66 | 2.10 | 1.33 | 4.10 | 5.14 | 6.90 | 14.1 |
| Summer (JJA) | 133 | 3.47 | 1.69 | 0.001 | 2.49 | 3.27 | 3.97 | 17.9 |
| Fall (SON) | 134 | 3.72 | 2.05 | 0.052 | 2.35 | 3.23 | 4.62 | 23.8 |
| Winter (DJF) | 86 | 4.77 | 2.39 | 0.051 | 3.36 | 4.46 | 5.39 | 17.1 |
| Smoke (C1) | 169 | 5.28 | 2.35 | 0.081 | 3.64 | 4.80 | 6.51 | 23.1 |
| No smoke (C1) | 276 | 3.61 | 1.83 | 0.001 | 2.42 | 3.38 | 4.34 | 17.0 |
| Smoke (C2) | 247 | 4.94 | 2.35 | 0.051 | 3.33 | 4.49 | 6.11 | 23.8 |
| No smoke (C2) | 198 | 3.39 | 1.64 | 0.001 | 2.33 | 3.20 | 4.05 | 17.9 |
| Overall | 445 | 4.25 | 2.20 | 0.001 | 2.76 | 3.81 | 5.13 | 23.8 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S5. Summary statistics for estimated 24-hour average PM2.5 concentrations from OK models with a Gaussian fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 6.80 | 197 | 0.063 | 4.16 | 5.33 | 6.61 | 3.68x104 |
| Summer (JJA) | 133 | 1.42x1012 | 2.74x1014 | 0 | 2.57 | 3.31 | 3.96 | 6.06x1016 |
| Fall (SON) | 134 | 1.47x105 | 3.11x107 | 0.008 | 2.52 | 3.17 | 4.42 | 7.02x109 |
| Winter (DJF) | 86 | 1.78x1034 | 3.21x1036 | 0 | 3.52 | 4.44 | 5.19 | 5.80x1038 |
| Smoke (C1) | 169 | 5.26 | 2.70 | 0 | 3.68 | 4.91 | 6.49 | 197 |
| No smoke (C1) | 276 | 5.53x1033 | 1.79x1036 | 0 | 2.56 | 3.41 | 4.34 | 5.80x1038 |
| Smoke (C2) | 247 | 6.18x1033 | 1.89x1036 | 0 | 3.45 | 4.52 | 6.17 | 5.80x1038 |
| No smoke (C2) | 198 | 9.56x1011 | 2.25x1014 | 0 | 2.38 | 3.16 | 4.04 | 6.06x1016 |
| Overall | 445 | 3.43x1033 | 1.41x1036 | 0 | 2.87 | 3.81 | 5.12 | 5.80x1038 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). Note: Models predicted values well below 0 g/m3, which are represented as “0” in table. “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S6. Summary statistics for estimated 24-hour average PM2.5 concentrations from UK models with a Gaussian fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 7.34 | 311 | 0.027 | 4.09 | 5.14 | 6.83 | 5.81x104 |
| Summer (JJA) | 133 | 3.22x1014 | 6.10x1016 | 0 | 2.52 | 3.31 | 4.03 | 1.34x1019 |
| Fall (SON) | 134 | 3.81 | 5.56 | 0.004 | 2.38 | 3.26 | 4.63 | 1.12x103 |
| Winter (DJF) | 86 | 5.48x1029 | 9.91x1031 | 0 | 3.39 | 4.44 | 5.39 | 1.79x1034 |
| Smoke (C1) | 169 | 5.32 | 3.24 | 0.004 | 3.63 | 4.81 | 6.51 | 273 |
| No smoke (C1) | 276 | 1.70x1029 | 5.52x1031 | 0 | 2.45 | 3.41 | 4.39 | 1.97x1034 |
| Smoke (C2) | 247 | 1.91x1029 | 5.85x1031 | 0 | 3.36 | 4.49 | 6.15 | 1.79x1034 |
| No smoke (C2) | 198 | 2.16x1014 | 5.00x1016 | 0 | 2.35 | 3.25 | 4.09 | 1.34x1019 |
| Overall | 445 | 1.06x1029 | 4.36x1031 | 0 | 2.80 | 3.85 | 5.15 | 1.79x1034 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). Note: Models predicted values well below 0 g/m3, which are represented as “0” in table. “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S7. Summary statistics for estimated 24-hour average PM2.5 concentrations from OK models with a linear fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 5.72 | 2.00 | 1.30 | 4.18 | 5.30 | 6.93 | 14.0 |
| Summer (JJA) | 133 | 3.45 | 1.56 | 0 | 2.53 | 3.21 | 3.92 | 22.7 |
| Fall (SON) | 134 | 3.65 | 1.80 | 0.070 | 2.40 | 3.13 | 4.44 | 10.4 |
| Winter (DJF) | 86 | 4.81 | 2.29 | 0.748 | 3.55 | 4.48 | 5.16 | 17.8 |
| Smoke (C1) | 169 | 5.24 | 2.17 | 0.070 | 3.70 | 4.87 | 6.47 | 14.7 |
| No smoke (C1) | 276 | 3.62 | 1.77 | 0 | 2.53 | 3.38 | 4.29 | 22.7 |
| Smoke (C2) | 247 | 4.96 | 2.20 | 0.070 | 3.38 | 4.56 | 6.18 | 17.8 |
| No smoke (C2) | 198 | 3.35 | 1.53 | 0 | 2.36 | 3.10 | 4.01 | 22.7 |
| Overall | 445 | 4.24 | 2.09 | 0 | 2.83 | 3.84 | 5.07 | 22.7 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S8. Summary statistics for estimated 24-hour average PM2.5 concentrations from UK models with a linear fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 5.66 | 2.10 | 1.27 | 4.10 | 5.13 | 6.92 | 14.1 |
| Summer (JJA) | 133 | 3.49 | 1.70 | 0 | 2.51 | 3.28 | 3.96 | 21.0 |
| Fall (SON) | 134 | 3.73 | 2.06 | 0.050 | 2.35 | 3.24 | 4.63 | 23.6 |
| Winter (DJF) | 86 | 4.78 | 2.39 | 0.359 | 3.37 | 4.47 | 5.41 | 18.2 |
| Smoke (C1) | 169 | 5.29 | 2.34 | 0.125 | 3.65 | 4.81 | 6.53 | 23.5 |
| No smoke (C1) | 276 | 3.62 | 1.84 | 0 | 2.43 | 3.38 | 4.35 | 20.9 |
| Smoke (C2) | 247 | 4.95 | 2.35 | 0.126 | 3.34 | 4.50 | 6.14 | 23.6 |
| No smoke (C2) | 198 | 3.39 | 1.65 | 0 | 2.34 | 3.21 | 4.04 | 21.0 |
| Overall | 445 | 4.26 | 2.21 | 0 | 2.77 | 3.82 | 5.13 | 23.6 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S9. Summary statistics for estimated 24-hour average PM2.5 concentrations from OK models with a nugget fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 5.74 | 2.01 | 1.75 | 4.30 | 5.25 | 6.72 | 13.2 |
| Summer (JJA) | 133 | 3.47 | 1.55 | 1.06 | 2.56 | 3.28 | 3.94 | 10.2 |
| Fall (SON) | 125 | 3.53 | 1.70 | 0.273 | 2.39 | 3.06 | 4.26 | 9.79 |
| Winter (DJF) | 86 | 4.80 | 2.30 | 0.683 | 3.51 | 4.44 | 5.17 | 13.9 |
| Smoke (C1) | 164 | 5.22 | 2.17 | 1.20 | 3.73 | 4.87 | 6.46 | 13.3 |
| No smoke (C1) | 272 | 3.63 | 1.77 | 0.272 | 2.52 | 3.38 | 4.30 | 13.9 |
| Smoke (C2) | 238 | 4.94 | 2.21 | 0.683 | 3.39 | 4.51 | 6.04 | 13.9 |
| No smoke (C2) | 198 | 3.37 | 1.53 | 0.273 | 2.38 | 3.10 | 4.04 | 10.2 |
| Overall | 436 | 4.23 | 2.08 | 0.273 | 2.87 | 3.83 | 5.09 | 13.9 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Table S10. Summary statistics for estimated 24-hour average PM2.5 concentrations from UK models with a nugget fit by meteorological season, by smoke-impacted vs. non-smoke-impacted days, and overall, for the entire study period.

|  | n | Mean | SD | Min. | Q1 | Median | Q3 | Max. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spring (MAM) | 92 | 5.61 | 2.11 | 1.24 | 4.05 | 5.08 | 6.90 | 14.1 |
| Summer (JJA) | 133 | 3.49 | 1.67 | 0.698 | 2.51 | 3.28 | 3.96 | 17.9 |
| Fall (SON) | 125 | 3.53 | 1.84 | 0.054 | 2.29 | 3.12 | 4.34 | 16.8 |
| Winter (DJF) | 86 | 4.72 | 2.38 | 0.430 | 3.33 | 4.40 | 5.33 | 17.2 |
| Smoke (C1) | 164 | 5.19 | 2.28 | 0.820 | 3.59 | 4.70 | 6.40 | 17.9 |
| No smoke (C1) | 272 | 3.58 | 1.82 | 0.054 | 2.40 | 3.35 | 4.27 | 17.2 |
| Smoke (C2) | 238 | 4.86 | 2.30 | 0.430 | 3.28 | 4.41 | 5.98 | 17.2 |
| No smoke (C2) | 198 | 3.39 | 1.63 | 0.054 | 2.33 | 3.21 | 4.04 | 17.9 |
| Overall | 436 | 4.19 | 2.15 | 0.054 | 2.75 | 3.77 | 5.02 | 17.9 |

All values are reported in g/m3 except for n (the number of estimated, gridded/area-weighted 24-hour average PM2.5 surfaces generated during the measurement period [i.e., of 541 days total]). “C1” and “C2” denote different criteria for identifying smoke-impacted days: “C1” indicates [PM2.5] ≥ 20 g/m3 for at least one hour within a 24-hour period or a NOAA HMS smoke plume overlapped a PurpleAir sensor or EPA monitor; “C2” shows an OBA for sugarcane crop management was administered within the EAA. Spatial resolution is 2 km per grid cell.

Figure S2. Estimated 24-hour average PM2.5 concentrations using OK on smoke-impacted days. November 25th, 2025 represents the “smoke-impacted” designation under both “C1” and “C2” criteria.

Figure S3. Estimated 24-hour average PM2.5 concentrations using UK on smoke-impacted days. November 25th, 2025 represents the “smoke-impacted” designation under both “C1” and “C2” criteria.

Figure S4. Estimated 24-hour average PM2.5 concentrations using OK on non-smoke-impacted days. June 30th, 2024 represents the “non-smoke-impacted” designation both “C1” and “C2” criteria.

Figure S5. Estimated 24-hour average PM2.5 concentrations using UK on non-smoke-impacted days. June 30th, 2024 represents the “non-smoke-impacted” designation both “C1” and “C2” criteria.

Figure S6. Variograms for OK models by fit.  Select days (with high LOOCV R2 and low LOOCV RMSE and MAE values) were chosen to represent models/fits.

Figure S7. Variograms for UK models by fit.  Select days (with high LOOCV R2 and low LOOCV RMSE and MAE values) were chosen to represent models/fits.

Figure S8. Measured vs. predicted PM2.5 concentrations for the entire study period by model and fit with 95% confidence intervals. X- and Y- axes were scaled for readability and exclude values over 40 g/m3.

Figure S9. Distributions of LOOCV R2 values for IDW and OK models by fit function shown as density plots (in pink) and by date (below) for the entire measurement period with blue trendline.

Figure S10. Distributions of LOOCV R2 values for IDW and UK models by fit function shown as density plots (in pink) and by date (below) for the entire measurement period with blue trendline.