Supplementary Materials for

Measurement of GABA and glutamate-glutamine levels with 1H-magnetic resonance spectroscopy in the neocortex of healthy subjects: the influence of age, sex, and education.

by Z. Kürtös, G. Orsi, G. Perlaki, B. Czéh, and S. A. Nagy

- Supplementary Materials and Methods

Supplementary Table 1 provides a detailed overview of the acquisition and processing parameters according to the expert consensus recommendations on minimum reporting standards for in vivo Magnetic Resonance Spectroscopy (Lin et al. 2021).

| Supplementary Table 1. Minimum Reporting Standards in Magnetic Resonance Spectroscopy checklist. |  |
| --- | --- |
| Hardware |  |
| a. Field strength | 3T |
| b. Manufacturer | Siemens |
| c. Model (software version if available) | Prisma Fit, VE11E |
| d. RF coils: nuclei (transmit/receive), number of channels, type, body part | 1H (receive), 64 channels, head/neck, Siemens |
| e. Additional hardware | None |
| Acquisition |  |
| a. Pulse sequence | MEshcher-GArwood Point RESolved Spectroscopy (MEGA-PRESS) to measure macromolecular-contaminated gamma Aminobutyric acid (GABA+) and Glutamine plus Glutamate (Glx) levels |
| b. Volume of Interest (VOI) locations | Anterior cingulate cortex, occipital cortex, Precuneus |
| c. Nominal VOI size | 30 × 30 × 25 mm³ |
| d. Repetition Time (TR), Echo Time (TE) | TR/TE=1500/68 ms |
| e. Total number of Excitations or acquisitions per spectrum | 128 averages |
| f. Additional sequence parameters (spectral width in Hz, number of spectral points, frequency offsets); If STEAM: Mixing Time TM; If MRSI: 2D or 3D, FOV in all directions, matrix size, acceleration factors | MEGA-PRESS sequence was used with spectral bandwidth = 1200 Hz, and 512 data points. Editing pulse (duration: 13.8 ms; bandwidth: 49 Hz; delta frequency: -2.8ppm) was applied at 1.9 ppm for the edit-ON condition and at 7.5 ppm for the edit-OFF condition |
| g. Water Suppression Method | A modified Water suppression Enhanced through T₁ effects (WET) approach was applied, using a CHEmical Shift Selective (CHESS) pulse. |
| h. Shimming Method, reference peak, and thresholds for “acceptance of shim” chosen | Vendor-provided automated shimming followed by manual shimming to achieve < 24 Hz Full Width at Half Maximum (FWHM) for the water peak |
| i. Triggering or motion correction method (respiratory, peripheral, cardiac triggering, incl. device used and delays) | None |
| Data analysis methods and outputs |  |
| a. Analysis software | Gannet GABA analysis toolkit v. 3.1.5 |
| b. Processing steps deviating from quoted reference or product | None |
| c. Output measure (eg, absolute concentration, institutional units, ratio) | Water-referenced measurements were reported in institutional units (with the exception of the uncorrected and cerebrospinal-fluid–corrected GABA+/Water and Glx/Water signal-integral ratios), along with signal-integral ratios referenced to creatine (Cr) |
| d. Quantification references and assumptions, fitting model assumptions | Water referencing was performed by assuming a pure-water molecular concentration of 55.51 mol/kg (see Section 2.4 of the main text for details). |
| Data Quality |  |
| a. Reported variables (SNR, Linewidth (with reference peaks)) | SNR for GABA+ and Glx, and linewidths as FWHM of Cr and water |
| b. Data exclusion criteria | Percentage fit error of GABA+ or Glx > 20% and linewidth of Cr > 20 Hz and linewidth of water > 25 Hz |
| c. Quality measures of postprocessing Model fitting (eg, CRLB, goodness of fit, SD of residual) | Percentage fit error and linewidth of Cr and water |
| d. Sample Spectrum See | Figure 1. |

- Supplementary results

  - Spectral quality control

Quality control parameters were comparable across the test and retest sessions, except for the Glx fit error in the PrC, where both water- and Cr-referenced values showed session-related differences. The detailed results are provided in Supplementary Table 2.

| Supplementary Table 2. Spectral quality control measures in the spectroscopic voxels for test and retest measures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACC |  |  | OCC |  |  | PrC |  |  |  |  |  |
|  | Test (n=6) | Retest (n=6) | Difference* | Test (n=10) | Retest (n=10) | Difference* | Test (n=10) | Retest (n=10) |  | Difference* |  |  |
| GM fraction | 0.54 (0.52-0.57) | 0.55 (0.52-0.57) | 0.6875 | 0.61 (0.60-0.63) | 0.62 (0.60-0.63) | 0.8457 | 0.54 (0.48-0.57) | 0.54 (0.48-0.57) |  | 0.4316 |  |  |
| WM fraction | 0.30 (0.27-0.35) | 0.29 (0.27-0.33) | >0.9999 | 0.29 (0.27-0.30) | 0.28 (0.26-0.29) | 0.8457 | 0.32 (0.29-0.37) | 0.32 (0.29-0.38) |  | 0.6250 |  |  |
| CSF fraction | 0.15 (0.13-0.16) | 0.15 (0.14-0.16) | 0.8125 | 0.10 (0.10-0.13) | 0.10 (0.09-0.12) | 0.9395 | 0.14 (0.12-0.17) | 0.12 (0.10-0.14) |  | 0.4453 |  |  |
| FWHM of water during manual shimming (Hz) | 17.90 (17.25-20.30) | 17.75 (16.28-20.78) | 0.4375 | 13.45 (13.13-14.53) | 13.65 (12.98-14.65) | 0.8359 | 12.30 (12.08-12.73) | 12.45 (12.15-12.88) |  | 0.8516 |  |  |
| LW Water (Hz) | 12.85 (11.89-13.81) | 12.63 (11.11-14.30) | 0.6562 | 9.55 (9.23-9.78) | 9.25 (8.87-9.51) | 0.3223 | 9.16 (8.96-9.34) | 9.06 (8.85-9.26) |  | 0.6797 |  |  |
| LW Cr (Hz) | 12.12 (10.73-13.66) | 11.72 (10.72-13.38) | 0.6875 | 8.73 (8.50-9.12) | 8.73 (8.62-9.14) | 0.2324 | 8.29 (8.07-8.38) | 8.40 (8.09-8.72) |  | 0.1309 |  |  |
| Fit error GABA+,Water (%) | 6.47 (4.43-7.18) | 4.76 (4.07-5.92) | 0.4375 | 4.93 (3.80-7.06) | 4.16 (3.61-6.37) | 0.7695 | 5.47 (3.90-9.05) | 4.82 (4.33-7.38) |  | 0.3750 |  |  |
| Fit error GABA+,Cr (%) | 6.49 (4.43-7.31) | 4.81 (4.22-5.94) | 0.4375 | 5.02 (3.96-7.15) | 4.32 (3.78-6.45) | 0.6953 | 5.59 (4.08-9.11) | 4.99 (4.51-7.47) |  | 0.3750 |  |  |
| Fit error Glx,Water (%) | 8.14 (5.72-11.87) | 7.54 (3.89-8.63) | 0.2188 | 6.55 (5.66-10.09) | 5.85 (4.81-10.30) | 0.6953 | 5.69 (4.67-15.06) | 4.68 (4.18-7.81) |  | 0.0371 |  |  |
| Fit error Glx,Cr (%) | 8.19 (5.87-11.89) | 7.58 (4.11-8.65) | 0.2188 | 6.63 (5.78-10.14) | 5.94 (4.94-10.35) | 0.6953 | 5.82 (4.82-15.11) | 4.83 (4.36-7.90) |  | 0.0371 |  |  |
| SNR GABA+ | 24.0 (20.0-31.5) | 25.5 (20.3-33.3) | 0.5625 | 22.0 (20.0-25.0) | 22.5 (19.8-27.0) | 0.9883 | 22.5 (19.0-25.5) | 17.5 (16.0-24.3) |  | 0.0625 |  |  |
| SNR Glx | 16.0 (11.5-31.0) | 19.5 (12.8-33.8) | 0.1250 | 15.0 (13.8-16.5) | 15.5 (13.0-18.3) | 0.7500 | 19.5 (13.3-24.8) | 17.5 (15.5-26.3) |  | 0.8359 |  |  |
| *P-values from Wilcoxon matched-pairs signed rank test. Data is presented in median (interquartile range) Statistically significant p-values are indicated in bold. Abbreviations: ACC: Anterior cingulate cortex; OCC: Occipital cortex; PrC: Precuneus; GM: Gray matter; WM: White matter; CSF: Cerebrospinal fluid; FWHM: Full width at half maximum; LW: linewidth; Cr: Creatine; GABA+: gamma Aminobutyric acid plus co-edited macromolecules; Glx: combined glutamate and glutamine signal SNR: Signal-to-Noise ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Test-retest reliability

Detailed test and retest data are presented in Supplementary Table 3.

| Supplementary Table 3. Water- and creatine-referenced test–retest GABA+ and Glx data in the ACC, OCC, and PrC regions, presented with and without corrections for tissue composition, water and metabolite partial volume effects. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACC |  | OCC |  | PrC |  |
| Metabolite | test (n=6) | retest (n=6) | test (n=10) | retest (n=10) | test (n=10) | retest (n=10) |
| GABA+/Water uncorrected | 0.8135 (0.6744–1.0867) | 0.8453 (0.6609–1.2040) | 0.6068 (0.5608–0.7097) | 0.6162 (0.4827–0.7424) | 0.4627 (0.3861–0.6743) | 0.5559 (0.3882–0.6669) |
| GABA+/Water a | 0.9178 (0.7975–1.2927) | 0.9870 (0.7885–1.4226) | 0.7211 (0.6143–0.7882) | 0.7000 (0.5468–0.8227) | 0.5331 (0.4407–0.7696) | 0.6311 (0.4301–0.7700) |
| GABA+/Water b | 2.8407 (2.4729–4.0076) | 3.0591 (2.4445–4.4101) | 2.2331 (1.9000–2.4393) | 2.1393 (1.6928–2.5492) | 1.6525 (1.3632–2.3836) | 1.9539 (1.3341–2.3852) |
| GABA+/Water c | 2.5775 (2.2224–3.5187) | 2.7781 (2.1344–3.9016) | 1.9053 (1.6776–2.1249) | 1.8900 (1.4870–2.2409) | 1.5542 (1.2307–2.0912) | 1.7323 (1.2327–2.1480) |
| GABA+/Water d | 2.1262 (1.8333–2.9026) | 2.2916 (1.7606–3.2184) | 1.5717 (1.3668–1.7528) | 1.5635 (1.2266–1.8509) | 1.2456 (1.0136–1.7250) | 1.4290 (1.0168–1.7719) |
| GABA+/Cr uncorrected | 0.1171 (0.1042–0.1539) | 0.1180 (0.0972–0.1447) | 0.1176 (0.1079–0.1268) | 0.1160 (0.0944–0.1237) | 0.1107 (0.0812–0.1237) | 0.1163 (0.1091–0.1195) |
| GABA+/Cr % | 0.1387 (0.1152–0.1812) | 0.1382 (0.1159–0.1705) | 0.1320 (0.1196–0.1423) | 0.1284 (0.1050–0.1451) | 0.1303 (0.0988–0.1423) | 0.1322 (0.1264–0.1366) |
| Glx/Water uncorrected | 1.3742 (1.2080–2.0780) | 1.5043 (1.3026–2.3032) | 1.1580 (1.0416–1.5426) | 1.2101 (1.1243–1.8262) | 1.4681 (0.9337–1.9904) | 1.4563 (1.0718–1.6248) |
| Glx/Water a | 1.5525 (1.4240–2.4770) | 1.7635 (1.5300–2.7420) | 1.3654 (1.1799–1.6793) | 1.3442 (1.2556–2.0243) | 1.6615 (1.1408–2.2892) | 1.6851 (1.2359–1.8542) |
| Glx/Water b | 4.8050 (4.4146–7.6797) | 5.4661 (4.7421–8.5003) | 4.2269 (3.6500–5.1980) | 4.1324 (3.8450–6.2639) | 5.1441 (3.5442–7.0905) | 5.2177 (3.8287–5.7410) |
| Glx/Water c | 4.3770 (3.9145–6.7396) | 4.9336 (4.2653–7.4275) | 3.6014 (3.2209–4.5703) | 3.6170 (3.3513–5.4735) | 4.5540 (3.2574–6.3585) | 4.6267 (3.4598–5.2567) |
| Glx/Water d | 3.6106 (2.3848–5.5594) | 4.0697 (3.5185–6.1269) | 2.9708 (2.6422–3.7000) | 2.9836 (2.7645–4.6038) | 3.7566 (2.6516–5.2451) | 3.8165 (2.8540–4.3362) |
| Glx/Cr uncorrected | 0.0568 (0.0494–0.0718) | 0.0671 (0.0576–0.0737) | 0.0673 (0.0596–0.0688) | 0.0690 (0.0614–0.0716) | 0.0807 (0.0777–0.0894) | 0.0799 (0.0750–0.0862) |
| Glx/Cr a | 0.0668 (0.0544–0.0856) | 0.0788 (0.0675–0.0874) | 0.0742 (0.0675–0.0790) | 0.0779 (0.0691–0.0820) | 0.0947 (0.0911–0.1018) | 0.0920 (0.0819–0.1002) |
| Data is presented in median (interquartile range). Water-referenced metabolite measurements (except uncorrected- and cerebrospinal fluid corrected GABA+/Water and Glx/Water signal integral ratios) are in institutional units, while creatine-referenced values represent signal integral ratios. a Estimated metabolite ratio corrected for cerebrospinal fluid fraction. b Estimated metabolite concentration corrected for partial volume effects of water. c Estimated metabolite concentration corrected for partial volume effects of water and metabolite. d Estimated metabolite concentration corrected for partial volume effects of water and metabolite and adjusted to the average voxel gray matter and white matter composition across our final study sample of 59 heathy subjects. Abbreviations: ACC: Anterior cingulate cortex; OCC: Occipital cortex; PrC: Precuneus; GABA+: gamma Aminobutyric acid plus co-edited macromolecules; Cr: Creatine; Glx: combined glutamate and glutamine signal |  |  |  |  |  |  |

References

Lin A, Andronesi O, Bogner W, Choi I Y, Coello E, Cudalbu C, Juchem C, Kemp G J, Kreis R, Krššák M, Lee P, Maudsley A A, Meyerspeer M, Mlynarik V, Near J, Öz G, Peek A L, Puts N A, Ratai E M, Mullins P G (2021). Minimum Reporting Standards for in vivo Magnetic Resonance Spectroscopy (MRSinMRS): Experts’ consensus recommendations. NMR in Biomedicine, 34(5). https://doi.org/10.1002/NBM.4484