Supplementary Table 1 Optical and Treatment Parameters of the Two LED-PBM Regimens

| Parameter | Sinusoidally modulated LED-PBM | Continuous LED-PBM |
| --- | --- | --- |
| Light source | 650-nm LED | 650-nm LED |
| Temporal output profile | Sinusoidally modulated, time-varying | Continuous, constant |
| Peak output power | 1.39 ± 0.10 mW | 1.39 ± 0.10 mW |
| Average output power | 0.70 ± 0.10 mW | 1.39 ± 0.10 mW |
| Irradiance | 1.85 mW/cm² | 3.70 mW/cm² |
| Estimated optical power entering a 4-mm pupil | 0.405 mW | 0.810 mW |
| Session duration | 3 minutes | 3 minutes |
| Treatment frequency | Twice daily | Twice daily |
| Minimum intersession interval | 4 hours | 4 hours |
| Device safety classification under tested conditions | IEC 60825-1:2014 Class 1 | IEC 60825-1:2014 Class 1 |

Supplementary Table 2 Linear mixed-effects model analysis of changes in ocular parameters at 1 and 3 months

| Indicator | Time_Point | Sinusoidal_Change | Sinusoidal_P | Continuous_Change | Continuous_P | DID_val | P_value | Cohens_d |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CFT | 1 Month Change | -5.741 ± 3.762 | 0.227 | 2.534 ± 3.443 | 0.676 | -8.275 (-18.330, 1.779) | 0.106 | -0.446 |
|  | 3 Months Change | -4.766 ± 3.753 | 0.348 | -0.631 ± 3.531 | 0.968 | -4.134 (-14.292, 6.023) | 0.423 | -0.223 |
| OD-PD, % | 1 Month Change | 0.105 ± 0.170 | 0.753 | 0.560 ± 0.159 | 0.001 | -0.455 (-0.914, 0.004) | 0.052 | -0.563 |
|  | 3 Months Change | 0.016 ± 0.172 | 0.989 | 0.325 ± 0.156 | 0.073 | -0.309 (-0.768, 0.150) | 0.186 | -0.383 |
| pRNFL-T, μm | 1 Month Change | -2.115 ± 1.146 | 0.122 | 0.963 ± 1.101 | 0.586 | -3.078 (-6.210, 0.054) | 0.054 | -0.527 |
|  | 3 Months Change | 1.333 ± 1.162 | 0.416 | 0.625 ± 1.093 | 0.780 | 0.708 (-2.437, 3.854) | 0.658 | 0.121 |
| pRNFL-S, μm | 1 Month Change | 0.288 ± 0.978 | 0.926 | -0.999 ± 0.940 | 0.466 | 1.288 (-1.386, 3.961) | 0.344 | 0.258 |
|  | 3 Months Change | 0.750 ± 0.992 | 0.663 | 0.300 ± 0.933 | 0.914 | 0.450 (-2.235, 3.135) | 0.741 | 0.090 |
| pRNFL-N, μm | 1 Month Change | 0.596 ± 1.165 | 0.815 | -0.943 ± 1.118 | 0.606 | 1.539 (-1.644, 4.722) | 0.342 | 0.259 |
|  | 3 Months Change | 0.266 ± 1.181 | 0.953 | 0.217 ± 1.110 | 0.963 | 0.049 (-3.147, 3.245) | 0.976 | 0.008 |
| pRNFL-I, μm | 1 Month Change | -0.904 ± 1.025 | 0.581 | 0.049 ± 0.985 | 0.996 | -0.953 (-3.755, 1.848) | 0.503 | -0.182 |
|  | 3 Months Change | -1.727 ± 1.040 | 0.177 | -1.623 ± 0.978 | 0.177 | -0.104 (-2.917, 2.710) | 0.942 | -0.020 |
| FAZ, um² | 1 Month Change | -0.107 ± 0.043 | 0.027 | -0.022 ± 0.039 | 0.784 | -0.084 (-0.199, 0.031) | 0.149 | -0.401 |
|  | 3 Months Change | -0.080 ± 0.043 | 0.119 | -0.040 ± 0.040 | 0.499 | -0.040 (-0.156, 0.075) | 0.493 | -0.191 |
| F-CCVD, % | 1 Month Change | 1.073 ± 0.389 | 0.012 | 0.261 ± 0.356 | 0.678 | 0.812 (-0.226, 1.851) | 0.125 | 0.426 |
|  | 3 Months Change | 1.136 ± 0.389 | 0.008 | 0.218 ± 0.358 | 0.757 | 0.918 (-0.124, 1.959) | 0.084 | 0.482 |
| ParaF-CCVD, % | 1 Month Change | 0.661 ± 0.232 | 0.010 | 0.289 ± 0.213 | 0.301 | 0.372 (-0.249, 0.993) | 0.239 | 0.326 |
|  | 3 Months Change | 0.495 ± 0.232 | 0.065 | 0.450 ± 0.214 | 0.070 | 0.045 (-0.578, 0.668) | 0.887 | 0.039 |
| PeriF-CCVD, % | 1 Month Change | 0.431 ± 0.162 | 0.016 | 0.340 ± 0.148 | 0.043 | 0.091 (-0.342, 0.524) | 0.680 | 0.114 |
|  | 3 Months Change | -0.200 ± 0.162 | 0.366 | 0.304 ± 0.149 | 0.080 | -0.504 (-0.938, -0.070) | 0.023 | -0.635 |
| F-CVI, % | 1 Month Change | -2.009 ± 1.076 | 0.117 | -0.593 ± 0.984 | 0.761 | -1.416 (-4.292, 1.459) | 0.333 | -0.269 |
|  | 3 Months Change | -1.079 ± 1.076 | 0.504 | -0.309 ± 0.991 | 0.918 | -0.769 (-3.653, 2.115) | 0.600 | -0.146 |
| ParaF-CVI, % | 1 Month Change | -0.826 ± 0.840 | 0.516 | -0.225 ± 0.768 | 0.927 | -0.602 (-2.847, 1.643) | 0.598 | -0.146 |
|  | 3 Months Change | -0.253 ± 0.840 | 0.923 | 0.163 ± 0.774 | 0.958 | -0.416 (-2.667, 1.836) | 0.716 | -0.101 |
| PeriF-CVI, % | 1 Month Change | -0.434 ± 0.629 | 0.706 | -0.568 ± 0.575 | 0.513 | 0.135 (-1.546, 1.815) | 0.875 | 0.044 |
|  | 3 Months Change | 0.106 ± 0.629 | 0.971 | 0.099 ± 0.579 | 0.970 | 0.006 (-1.680, 1.692) | 0.994 | 0.002 |

Data are presented as estimated mean changes from baseline ± standard error. CCVD, choriocapillaris vessel density; CI, confidence interval; CFT, central foveal thickness; CON, control; CVI, choroidal vascularity index; FAZ, foveal avascular zone; OD-PD, optic disc perfusion density; pRNFL, peripapillary retinal nerve fiber layer.