# Supplementary materials

# Appendix 1

# Lab analysis

## A.1 Determination of extractable bases with ammonium lactate

For the preparation of the extracting solution, several solutions were prepared: lactic acid solution, concentrated acetic acid solution and concentrated ammonia solution.

The lactic acid solution was prepared by adding 250 mL of lactic acid and 500 mL of water together in a volumetric flask. This mixture was then placed in an incubator for 48 hours at a temperature of 95 °C for the lactic acid to hydrolyse. After cooling, the exact concentration of the solution was determined. 10.0 mL of the solution was pipetted in a volumetric flask of 100 mL and water was added until a volume of 100 mL was reached. 10.0 mL of this diluted solution was pipetted into an Erlenmeyer, a few drops of phenolphthalein indicator was added and titration was executed with 0.1 mol/L NaOH. This was repeated three times and the concentration of the lactic acid solution was calculated (concentration A).

The concentrated acetic acid solution was prepared by pipetting 10.0 mL of the acid in a 500 mL volumetric flask, water was added until a volume of 500 mL was reached. 10.0 mL of this diluted solution was pipetted into an Erlenmeyer. A few drops of phenolphthalein indicator were added and a titration was executed with 0.1 mol/L NaOH. This was repeated three times and the concentration of the concentrated acetic acid was calculated (concentration B).

The concentrated ammonia solution was prepared by pipetting 10.0 mL of ammonia was pipetted in a 500 mL volumetric flask, water was added until a volume of 500 mL was reached. 10.0 mL of this diluted solution was pipetted in an Erlenmeyer. A few drops of methyl red indicator were added and a titration was executed with 0.1 mol/L HCl. This was repeated three times and the concentration of the concentrated ammonia is calculated (concentration C).

The concentrated extraction solution was prepared by adding 250 mL of water, 1000/A mL lactic acid solution, 4000/B mL concentrated acetic acid and 1000/C mL concentrated ammonia to a volumetric flask of 1 L. Ultrapure water was added until a volume of 1 L was reached. 200 mL of this concentrated extraction solution was subsequently diluted with 2 L ultrapure water. The pH was measured and acetic acid or ammonia were added until a value of 3.75 is reached.

A diluting solution was prepared by diluting 32 mL of nitric acid (HNO3, 70%) with 500 mL water.

Several standard solutions were prepared: stock solution, standard solutions 1, 2, 3 and 4. The stock solution was prepared by pipetting 20 mL P, 50 mL Ca, 5 mL Mg, 5 mL K and 1 mL Na from the corresponding 1000 mg/l stock solutions into a volumetric flask of 100 mL. HNO3 (1N) was added until a volume of 100 mL was reached. Standard solution 1 was prepared by pipetting 10 mL of the stock solution into a tube together with 10 mL ammonium lactate. Standard solution 2 was prepared by pipetting 10 mL of standard solution 1, 5 mL ammonium lactate and 5 mL HNO3 (1N) into a tube. Standard solution 3 was prepared by pipetting 10 mL standard solution 2, 5 mL ammonium lactate and 5 mL HNO3 (1N) into a tube. Standard solution 4 was prepared by pipetting 10 mL standard solution 3, 5 mL ammonium lactate and 5 mL HNO3 (1N) into a tube. One blank was prepared by adding 10 mL ammonium lactate and 10 mL HNO3 (1N) to a tube. The blank, standard solution 1, 2, 3 and 4 were measured to calibrate the ICP. The concentrations (in ppm) of the stock solution and the standard solutions are shown in table A.1.1.

Table A.1.1 Element concentrations in the solutions

| Element | Stock solution (mg/L) | Standard solution 1 (mg/L) | Standard solution 2 (mg/L) | Standard solution 3 (mg/L) | Standard solution 4 (mg/L) |
| --- | --- | --- | --- | --- | --- |
| P | 200 | 100 | 50 | 25 | 12.5 |
| Ca | 500 | 250 | 125 | 62.5 | 31.25 |
| Mg | 50 | 25 | 12.5 | 6.25 | 3.125 |
| K | 50 | 25 | 12.5 | 6.25 | 3.125 |
| Na | 10 | 5 | 2.5 | 1.25 | 0.625 |

Of each soil sample 1.5 g dried soil was added to a tube and 30.0 mL of the diluted extracting solution was added. Three internal standards were included. This was shaken for 4 hours and centrifuged for 15 min at 3000 G afterwards. The extracts were diluted by pipetting 5 mL of the clear supernatant and 5 mL HNO3 (1N) into tubes. These were then measured with the ICP.

The concentration of each soil element in each soil sample were calculated by using the following equation:

c=cextract- cblank* Vs*df

With   c = concentration in the soil (mg/kg)

cextract = concentration in the extract (mg/L)

cblank = concentration in the blank (mg/L)

V = volume extracting solution (30.0 mL)

s = sample weight (g)

df = diluting factor = 2

## A.2 Mineral determination with ICP-OES by dry ashing

Several reagents were prepared: nitric acid (6M), nitric acid (3M), nitric acid (1%) and nitric acid (10%). The nitric acid (6M) was prepared by diluting 38.4 mL of concentrated nitric acid (70%) with 100 mL ultrapure water. Nitric acid 3M was prepared by diluting 25 mL of nitric acid of 6M with 50 mL ultrapure water. Nitric acid (1%) was prepared by diluting 7 mL of the concentrated nitric acid (70%) with 500 mL ultrapure water. Nitric acid (10%) was prepared by diluting 154 mL of concentrated nitric acid (70%) diluted with 1000 mL ultrapure water.

1.000 g of each plant material sample was added to an individual porcelain crubicle, the crubicles were subsequently precalcinated with a Bunsen burner until no smoke was observed anymore. The crubicles were further calcinated in a muffle furnace at 550 °C for 2 hours until the samples were completely mineralized. The cubicles were afterwards cooled down in a desiccator and subsequently weighted.

The ashes were transferred into small beakers and masses was noted down. 10 mL of 6M HNO3 was added to each beaker and digestion was started by gently boiling the mixtures until approximately 1 mL remained. 5 mL of 3M HNO3 was added and the mixtures were reheated for a few minutes. The solutions were then filtered into a 50 mL volumetric flask. The small beakers were rinsed with 1% HNO3 and this was then poured over the filters. The extracts were then poured into falcon tubes of 50 mL. A 10-fold dilution of each sample was prepared by pipetting 1 mL of the extract and 9 mL 1% HNO3 in a falcon tube of 15 mL.

Several calibration solutions were prepared from 1000 ppm standard solutions. In these standard solutions, the elements are separately dissolved in 2 to 5% HNO3. To prepare diluted calibration solutions, it was necessary to first prepare stock solutions from the standard solutions from 1000 ppm. Stock solution Na – K – Ca – Mg (200 ppm) was prepared by pipetting 20.00 mL of each standard solution into a volumetric flask of 100.0 mL, the volume was reached by adding HNO3 (1%). Stock solution Zn – Fe (10 ppm) was prepared by pipetting 2.50 mL of each standard solution into a volumetric flask of 250.0 mL, the volume was reached by adding HNO3 (1%). Stock solution P (200 ppm) was prepared by pipetting 20.00 mL of the standard solution into a volumetric flask of 100.0 mL, the volume was reached by adding HNO3 (1%). Stock solution Se (0.1 ppm) was prepared by pipetting 1.00 mL of the standard solution into a volumetric flask of 100.0 mL, the volume was reached by adding HNO3 (1%). This solution was homogenized, 1.00 mL of this solution was then pipetted into a volumetric flask of 100.0 mL and the volume was reached by adding HNO3 (1%). Stock solution Cu – Mn (1 ppm) was prepared by pipetting 1.00 mL of each standard solution into a volumetric flask of 100.0 mL, the volume was reached by adding HNO3 (1%). The solution was homogenized and 10.00 mL of this solution was then pipetted into a volumetric flask of 100.0 mL, the volume was reached by adding HNO3 (1%).

Five calibration solutions were prepared. Calibration solution 5 were the stock solutions as mentioned above. Calibration solution 4 was prepared by pipetting 37.50 mL of each stock solution into a volumetric flask of 50.0 mL, the volume was reached by adding HNO3 (1%). Calibration solution 3 was prepared by pipetting 25.00 mL of each stock solution into a volumetric flask of 50.0 mL, the volume was reached by adding HNO3 (1%). Calibration solution 2 was prepared by pipetting 25.00 mL of calibration solution 3 into a volumetric flask of 50.0 mL, the volume was reached by adding HNO3 (1%). Calibration solution 1 was prepared by pipetting 25.00 mL of calibration solution 2 into a volumetric flask of 50.0 mL, the volume was reached by adding HNO3 (1%).

Table A.2.1 Mineral concentrations in the solutions

| Solution | Mineral | Standard solutions | Calibration sol. 5 | Calibration sol. 4 | Calibration sol. 3 | Calibration sol. 2 | Calibration sol. 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Na K Ca Mg | 998 1000 1002 1000 | 199.6 200.0 200.4 200.0 | 149.7 150.0 150.3 150.0 | 99.80 100.0 100.2 100.0 | 49.90 50.00 50.10 50.00 | 24.95 25.00 25.05 25.00 |
| B | Zn Fe | 998 1000 | 9.98 10.00 | 7.49 7.50 | 4.99 5.00 | 2.50 2.50 | 1.25 1.25 |
| C | P | 1000 | 200.00 | 150.0 | 100.0 | 50.00 | 25.00 |
| D | Se | 1000 | 0.100 | 0.0750 | 0.0500 | 0.0250 | 0.0125 |
| E | Cu Mn | 1001 1002 | 1.001 1.002 | 0.7508 0.7515 | 0.5005 0.5010 | 0.2503 0.2505 | 0.1251 0.1253 |

The calibration solutions were analysed with ICP-OES. A 1% HNO3 solution was used as a blank.

Ash content %= weightcrubicle with ash- weightcrubicleweightdry sample*100%

Mineral fraction in dry sample mgkg= csample- cblank*0.050 Lweightash *ash content*10 000

With cmineral = concentration of the mineral in the sample solution (mg/L)

cblank = concentration of the mineral in the blank solution (mg/L)

wash = weight of the ash in the beaker (mg)

Table S1. Average percentage of litter mass loss and corresponding standard deviation for five monospecific litter types and the two standard Green and Rooibos tea in Gerese highland Afromontane moist evergreen fragmented forest SW Ethiopia. Where M1= first month, M3= third month, M6= six month and one-year decomposition rate observational periods.

|  | Litter types | Months | n | Mean | Sd |
| --- | --- | --- | --- | --- | --- |
| 1 | Croton macrostachyus | M1 | 15 | 33.630 | 3.309 |
| 2 | Croton macrostachyus | M3 | 15 | 50.934 | 2.841 |
| 3 | Croton macrostachyus | M6 | 14 | 59.711 | 4.890 |
| 4 | Croton macrostachyus | M12 | 12 | 67.058 | 5.622 |
| 5 | Cupressus lusitanica | M1 | 15 | 17.619 | 5.316 |
| 6 | Cupressus lusitanica | M3 | 14 | 28.119 | 2.241 |
| 7 | Cupressus lusitanica | M6 | 15 | 36.207 | 3.801 |
| 8 | Cupressus lusitanica | M12 | 15 | 47.735 | 3.154 |
| 9 | Eucalyptus globulus | M1 | 15 | 29.514 | 5.840 |
| 10 | Eucalyptus globulus | M3 | 15 | 41.782 | 3.673 |
| 11 | Eucalyptus globulus | M6 | 13 | 51.673 | 3.519 |
| 12 | Eucalyptus globulus | M12 | 15 | 62.756 | 3.504 |
| 13 | Green tea | M1 | 15 | 60.589 | 3.143 |
| 14 | Green tea | M3 | 15 | 71.643 | 3.121 |
| 15 | Green tea | M6 | 13 | 77.207 | 3.275 |
| 16 | Green tea | M12 | 12 | 83.370 | 1.465 |
| 17 | Millettia ferruginea | M1 | 15 | 54.835 | 2.497 |
| 18 | Millettia ferruginea | M3 | 15 | 68.195 | 2.124 |
| 19 | Millettia ferruginea | M6 | 15 | 73.105 | 3.373 |
| 20 | Millettia ferruginea | M12 | 13 | 76.627 | 4.250 |
| 21 | Rooibos tea | M1 | 15 | 24.446 | 2.066 |
| 22 | Rooibos tea | M3 | 14 | 36.529 | 5.276 |
| 23 | Rooibos tea | M6 | 14 | 47.285 | 4.216 |
| 24 | Rooibos tea | M12 | 14 | 59.828 | 4.658 |
| 25 | Syzigium guineense | M1 | 15 | 11.707 | 1.853 |
| 26 | Syzigium guineense | M3 | 15 | 18.514 | 1.787 |
| 27 | Syzigium guineense | M6 | 15 | 25.537 | 2.674 |
| 28 | Syzigium guineense | M12 | 14 | 41.164 | 9.904 |

Table S2. Average percentage of litter mass loss and corresponding standard deviation of the eleven litter mixture of the three indigenous and two exotic trees leave in Gerese highland SW Ethiopia. Where M1= first month, M3= third month, M6= six month and Year= one-year decomposition rate observation period. Where A= C. macrostachyus, B= M. ferruginea, C= S. guineense , D= E. globulus and E= C. lusitanica.

|  | Mixture types | Months | n | Mean | sd |
| --- | --- | --- | --- | --- | --- |
| 1 | ABC | M1 | 15 | 35.324 | 2.160 |
| 2 | ABC | M3 | 15 | 50.627 | 1.860 |
| 3 | ABC | M6 | 15 | 57.477 | 3.765 |
| 4 | ABC | Year | 12 | 66.816 | 2.622 |
| 5 | ABCDE | M1 | 15 | 33.555 | 2.519 |
| 6 | ABCDE | M3 | 15 | 49.907 | 7.832 |
| 7 | ABCDE | M6 | 14 | 57.779 | 2.852 |
| 8 | ABCDE | Year | 15 | 65.908 | 2.176 |
| 9 | ABD | M1 | 14 | 42.379 | 2.267 |
| 10 | ABD | M3 | 15 | 58.029 | 1.955 |
| 11 | ABD | M6 | 15 | 66.897 | 7.431 |
| 12 | ABD | Year | 14 | 72.816 | 4.893 |
| 13 | ABE | M1 | 15 | 40.082 | 2.956 |
| 14 | ABE | M3 | 15 | 56.461 | 7.713 |
| 15 | ABE | M6 | 14 | 60.628 | 3.564 |
| 16 | ABE | Year | 13 | 68.069 | 4.543 |
| 17 | ACD | M1 | 10 | 22.433 | 3.174 |
| 18 | ACD | M3 | 10 | 35.755 | 2.213 |
| 19 | ACD | M6 | 10 | 46.738 | 3.563 |
| 20 | ACD | Year | 9 | 60.034 | 2.721 |
| 21 | ACE | M1 | 9 | 22.141 | 5.134 |
| 22 | ACE | M3 | 10 | 35.204 | 2.060 |
| 23 | ACE | M6 | 10 | 48.006 | 8.898 |
| 24 | ACE | Year | 9 | 56.011 | 2.597 |
| 25 | ADE | M1 | 15 | 28.385 | 3.159 |
| 26 | ADE | M3 | 15 | 40.416 | 2.436 |
| 27 | ADE | M6 | 14 | 52.290 | 3.288 |
| 28 | ADE | Year | 15 | 62.842 | 2.991 |
| 29 | BCD | M1 | 15 | 32.486 | 2.215 |
| 30 | BCD | M3 | 15 | 48.389 | 2.701 |
| 31 | BCD | M6 | 15 | 58.399 | 3.483 |
| 32 | BCD | Year | 14 | 67.793 | 2.612 |
| 33 | BCE | M1 | 13 | 31.022 | 2.196 |
| 34 | BCE | M3 | 15 | 44.553 | 2.912 |
| 35 | BCE | M6 | 14 | 52.205 | 2.902 |
| 36 | BCE | Year | 12 | 62.988 | 4.234 |
| 37 | BDE | M1 | 15 | 35.376 | 1.972 |
| 38 | BDE | M3 | 15 | 50.571 | 2.261 |
| 39 | BDE | M6 | 12 | 58.533 | 7.932 |
| 40 | BDE | Year | 14 | 68.889 | 2.092 |
| 41 | CDE | M1 | 15 | 16.958 | 2.689 |
| 42 | CDE | M3 | 15 | 28.049 | 2.924 |
| 43 | CDE | M6 | 15 | 38.234 | 2.446 |
| 44 | CDE | Year | 12 | 48.593 | 2.049 |

Table S3. Tukey multiple comparisons of means for the three indigenous and two exotic selected tree leaf litters together with the two standard green and rooibos teas in Gerese highland Afromontane moist evergreen fragmented forest SW Ethiopia.

|  | diff | lwr | upr | P adj |
| --- | --- | --- | --- | --- |
| Cupressus-Croton | 18.754 | 10.224 | 27.284 | 0.000 |
| Eucalyptus-Croton | 5.683 | -2.915 | 14.281 | 0.443 |
| Green tea-Croton | -18.751 | -27.538 | -9.965 | 0.000 |
| Millettia-Croton | -12.568 | -21.575 | -3.562 | 0.001 |
| Red tea-Croton | 10.031 | 1.397 | 18.665 | 0.011 |
| Syzygium-Croton | 27.655 | 19.057 | 36.253 | 0.000 |
| Eucalyptus-Cupressus | -13.071 | -21.566 | -4.577 | 0.000 |
| Green tea-Cupressus | -37.505 | -46.190 | -28.821 | 0.000 |
| Millettia-Cupressus | -31.322 | -40.230 | -22.415 | 0.000 |
| Red tea-Cupressus | -8.723 | -17.253 | -0.193 | 0.041 |
| Syzygium-Cupressus | 8.901 | 0.406 | 17.395 | 0.033 |
| Green tea-Eucalyptus | -24.434 | -33.186 | -15.683 | 0.000 |
| Millettia-Eucalyptus | -18.251 | -27.224 | -9.279 | 0.000 |
| Red tea-Eucalyptus | 4.348 | -4.250 | 12.946 | 0.746 |
| Syzygium-Eucalyptus | 21.972 | 13.409 | 30.534 | 0.000 |
| Millettia-Green tea | 6.183 | -2.970 | 15.336 | 0.415 |
| Red tea-Green tea | 28.782 | 19.996 | 37.569 | 0.000 |
| Syzygium-Green tea | 46.406 | 37.654 | 55.157 | 0.000 |
| Red tea-Millettia | 22.599 | 13.593 | 31.606 | 0.000 |
| Syzygium-Millettia | 40.223 | 31.250 | 49.195 | 0.000 |
| Syzygium-Red tea | 17.624 | 9.026 | 26.222 | 0.000 |

Fig. S1. Decomposition rate of eleven litter mixture of the three indigenous and two exotic trees leave of Afromontane moist evergreen fragmented forest in SW Ethiopia. Where litter code A= C. macrostachyus, B= M. ferruginea, C= S. guineense, D= E. globulus and E= C. lusitanica. Mean and standard deviation of litter mass remain located in the first month, third month, six month and twelve months.