Appendix A: Supplementary Data Architecture and Hyperparameter Specifications

Table A1: Secondary Spatial Data Processing Protocols and Spatial Resolutions

| Dataset Name | Source / Repository | Original Resolution | Extraction & Resampling Method | Model Pipeline Variable |
| --- | --- | --- | --- | --- |
| OpenStreetMap (OSM) | OpenStreetMap Foundation (2025) | Vector Line | 100m segment segmentation & spatial join | Road Network Geometry & Tier |
| SRTM DEM v3 | NASA Earthdata / USGS (2014) | 30m Grid | Bilinear interpolation to segment node | Surface Elevation (m), Slope (​∘​) |
| CHIRPS v2.0 | Climate Hazards Center (2014–2024) | 0.05° Grid | Spatial point-in-polygon extraction | 24-hr Max Rainfall Intensity (mm) |
| ESA WorldCover | European Space Agency (2021) | 10m Grid | 50m radial buffer land cover aggregation | Surface Impermeability Index (0-1) |
| Copernicus EMS / DFO | CEMS & Dartmouth Flood Obs. | Vector Polygons | Spatial intersection with road nodes | Binary Target (Y=1 flooded, Y=0 resilient) |

Table A2: Machine Learning Hyperparameter Tuning Configuration (XGBoost)

| Hyperparameter Name | Search Range / Options | Selected Optimal Value | Description / Function |
| --- | --- | --- | --- |
| n_estimators | 50 – 500 | 250 | Total number of gradient boosted trees |
| max_depth | 3 – 10 | 6 | Maximum tree depth to control model complexity |
| learning_rate (η) | 0.01 – 0.3 | 0.05 | Step size shrinkage to prevent overfitting |
| subsample | 0.5 – 1.0 | 0.8 | Subsample ratio of training instances |
| colsample_bytree | 0.5 – 1.0 | 0.7 | Subsample ratio of columns per tree |
| scale_pos_weight | 1.0 – 5.0 | 2.85 | Weight balance for SMOTE-adjusted class imbalance |

Appendix B: Model Sensitivity and Spatial Validation

Table B1: 5-Fold Cross-Validation Performance Across Iterations (XGBoost)

| Fold Iteration | ROC-AUC | F1-Score | Precision | Recall | Validation Accuracy |
| --- | --- | --- | --- | --- | --- |
| Fold 1 | 0.908 | 0.839 | 0.855 | 0.824 | 0.881 |
| Fold 2 | 0.915 | 0.851 | 0.868 | 0.835 | 0.892 |
| Fold 3 | 0.910 | 0.842 | 0.859 | 0.826 | 0.884 |
| Fold 4 | 0.918 | 0.854 | 0.871 | 0.838 | 0.895 |
| Fold 5 | 0.909 | 0.839 | 0.857 | 0.822 | 0.883 |
| Mean (± SD) | 0.912 (±0.004) | 0.845 (±0.007) | 0.862 (±0.007) | 0.829 (±0.007) | 0.887 (±0.006) |

Figure A1: End-to-End Data Fusion and Model Architecture

Figure A2: Conceptual SHAP Summary Plot (Global Feature Rankings)