| Table 7. Comparison of selected biodiesel-relevant characteristics of Camelina oil observed in the present study with relevant specifications for finished biodiesel under EN 14214 and ASTM D6751. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Present study: Camelina oil/feedstock | EN 14214:2012+A2:2019 – Finished FAME | ASTM D6751-24 – Finished B100 | Interpretation |
| Oxidative stability, induction period (h) | 7.3 | ≥ 8.0 h | 3.0≤ | Present value refers to Camelina oil, not finished biodiesel; therefore, it does not demonstrate fuel-standard compliance. |
| Cetane number | †48.2 | 51≤ | 47≤ | Estimated value for the present feedstock; not directly measured in biodiesel |
| Erucic acid (%) | 2.8–0.5 | — | — | Fatty-acid characteristic of Camelina oil; not a direct biodiesel compliance parameter |
| Total tocopherols (mg kg⁻¹ oil) | 768 | — | — | Oil antioxidant characteristic; no direct biodiesel specification |
| Oil content (%) | 39.0 | — | — | Seed/feedstock characteristic; not a finished-fuel specification |
| Specification limits apply to finished FAME/B100 biodiesel and are provided for contextual comparison only; they should not be interpreted as acceptance criteria for the Camelina oil analysed in the present study.†Cetane number was estimated from fatty-acid composition and was not measured directly in finished biodiesel. |  |  |  |  |